3GPP TSG-RAN WG2 Meeting #129bis	R2-2502821

Wuhan, China, 7th -11th April, 2025

Agenda Item:	8.2.4

Source:			ASUSTeK

Title:	Discussion on the assistance information from device

Document for: 	Discussion and Decision

- Introduction

In Rel-19 study item phase, assistance information for A-IoT transmission has been discussed in RAN2 meetings and following is captured in TR 38.769 [1]:

The segmentation study focuses on D2R direction (it can be further considered on R2D direction). For the candidate solution for segmentation: - The sequence number, the segment number and the number of segments are not supported; - An indication is used to indicate to the reader on whether the data is segmented and whether the MAC PDU is the last segment. It needs to be further discussed on the size of this indication (one or two bits) and the corresponding further details; - It is beneficial for the reader to be able to trigger a re-transmission of a segment; - It is assumed that the A-IoT device will not support AS layer buffering for A-IoT segmentation functionalities, i.e., all segment(s) are stored in upper layer(s). The use of the following assistance information is studied (the need of each is still to be decided): - The energy status report from the A-IoT device to the reader, which indicates that the A-IoT device, if the A-IoT device can determine this, does not have sufficient energy to perform the remaining/follow-up procedure. - The A-IoT device may report a 1-bit energy status indication to the reader in a D2R message. It needs to be further discussed if and how the reader control this indication of such report. - The reader may take this indication into account for the remaining/follow-up procedure (up to the reader implementation), e.g., the reader may choose not to transmit the subsequent messages for some time, or the reader may choose to not do anything in particular, etc. - It is beneficial for the reader to know an estimate of expected D2R message size. This D2R message size information would be beneficial but it is not essential. Two options are studied: - Option 1: The estimate of expected following D2R message size from the CN to the reader; - Advantages/disadvantages of option 1: If this information is available from CN, it saves the overhead/latency caused by Option 2 in AS layer. It can be also used by the reader as early as for the first D2R message, if needed. - Option 2: The simple message size indication from the device to the reader. - Advantages/disadvantages of option 2: It is useful in case the CN is not able to provide or does not provide the expected D2R message size. Limited granularity is supported considering the AS layer overhead. It requires D2R message to carry this information in advance, before the reader can actually use this size information for scheduling.

And in RAN2 #129 meeting, following agreements are made for segmentation:

Agreements on segmentation - To support segmentation, a 1 bit indication is introduced to indicate whether there is more data or not, if SA2 indicates that CN can provide an estimated expected D2R message size. If not possible, FFS if the 1 bit is sufficient. - Segment retransmission is supported. - For segment retransmission, reader explicitly indicates an offset in the MAC layer– e.g. number of bits successfully received so far (from the start). FFS This implies that unsegmented packet can also be retransmitted. FFS if this applies to msg3 - R2D segmentation is not supported for R19 A-IoT.

In this contribution, we would like to further discuss the assistance information provided by the device.

- Discussion

  - Indication of segmentation and D2R message size

It has been agreed that it is beneficial for the reader to know an estimate of expected D2R message size. And there are two options for the reader to know the expected D2R message size, one is received from CN and the other is received from device. RAN2 send an LS to SA2 and get reply [2] in RAN2 #129 meeting as following:

- Reply to LS on RAN2 agreements and assumptions for Ambient IoT SA2 thanks RAN2 for the LS (R2-2409412) on assistance information from the CN to the Reader for A-IoT. After discussion, SA2 would like to provide the answer to the questions in the LS as following: Q1: Can the CN provide, to the reader, an estimate of the expected size of the following D2R message(s) in response to the service request? SA2 answer: If provided by AF, AIOTF provides the reader with the approximate D2R message size based on AF request. Whether the CN can determine the approximate D2R message size if not provided by AF is under discussion in SA2. Q2: If such information (see Q1) can be provided by CN to the reader, can it be provided only in some cases or in all cases? SA2 answer: it can be provided in some cases. See answer to Q1.

Based on the answer from SA2, the AIOTF provides approximate D2R message size to the reader if the AIOTF received this information from the AF. In the last SA2 meeting (i.e., SA2 #167 meeting), it has been proposed, as following [3], that the AIOTF determines assistance information including approximate D2R message size based on configuration per AF and AIoT service type, if the AF does not provide the assistance information to the AIOTF.

Regarding the following EN in clause 5.4 of new TS 23.xyz entitled "Architecture support for Ambient power-enabled Internet of Things; Stage 2.", it is considered that 'approximate number of AIoT Devices' and 'approximate D2R message size' are useful for AIoT RAN in performing service operations, e.g. radio resource allocation. Editor's note: How the AIOTF determines the assistance information (e.g. approximate number of AIoT Devices, approximate D2R message size) without input from the AF is FFS. Therefore, it is proposed that the AIOTF determines these information based on configuration per AF and AIoT service type if not provided by the AF.

And this discussion was postponed in the meeting. The current text for the specification from SA2 is as following [4]:

## 5.4 Assistance information provided to AIoT RAN node The AIOTF provides the assistance information to the AIoT RAN together with the service operation requests. The AIOTF determines the assistance information provided to the AIoT RAN based on the information received from the AF. The Assistance information is used by the AIoT RAN for performing service operations, e.g. radio resource allocation. The following assistance information may be provided: - AIoT service type (e.g. Inventory, Command); - Optionally, approximate number of AIoT devices based on AF request; - Optionally, approximate D2R message size based on AF request. Editor's note: Other assistance information may be added later if necessary. Editor's note: How the AIOTF determines the assistance information (e.g. approximate number of AIoT Devices, approximate D2R message size) without input from the AF is FFS.

Currently, SA2 does not have conclusion on how the AIOTF determines the approximate D2R message size without input from the AF. However, based on the above documents, the CN could determine the approximate D2R message size based on AF request or configuration per AF and AIoT service type.

Observation 1: SA2 has discussed that the CN could determine the approximate D2R message size based on AF request or configuration per AF and AIoT service type.

As a result, the reader could know an estimated D2R message size from the CN. And on the device side, a 1-bit indication for segmentation could be enough.

Proposal 1: Confirm that the device provides 1-bit segmentation indication to indicate whether there is more data or not.

The segmentation indication could be either optionally or always present in D2R messages. To have a general format for D2R MAC PDU, we propose that the segmentation indication is always carried in each D2R MAC PDU. For example, if the size of available D2R data is larger than the resources for a D2R transmission, the device segments the D2R data and sets the segmentation indication to 1. The reader then schedules subsequent D2R transmission(s) to the device. If the size of available D2R data is not larger than the resources for the D2R transmission, the device sets the segmentation indication to 0. The reader then knows that this is the D2R transmission carrying complete D2R data or the last segment. The 1-bit segmentation indication from the device along with the estimated D2R message size form the CN could ensure efficient resources allocation on the reader.

Proposal 2: If the size of available D2R data is larger than the resources for a D2R transmission, the device segments the D2R data and sets the segmentation indication to 1.

Proposal 3: If the size of available D2R data is not larger than the resources for the D2R transmission, the device sets the segmentation indication to 0.

  - Indication of energy status

With limited energy capacity, the A-IoT device may not be always able to perform transmissions. If an A-IoT device does not have enough energy to receive/transmit the following transmission(s), the ongoing/follow-up procedure would fail. As captured in the TR [1] above, the device provides a 1-bit energy status indication to the reader in a D2R message, which indicates that the A-IoT device does not have sufficient energy to perform the remaining/follow-up procedure.

In RAN2 #127bis meeting, it has been agreed that the device may include energy status indication in D2R messages (e.g., MSG1, MSG3 and Command Response message). Similar to segmentation indication, the energy status indication could be either optionally or always present in D2R messages. To have a general format for D2R MAC PDU, we propose that the energy status indication could be always carried in each D2R MAC PDU. If the remaining energy of the device is sufficient for following transmission(s), the energy status indication indicates 0. If the remaining energy of the device is insufficient for following transmission(s), the energy status indication indicates 1. In addition, the device could stop the ongoing procedure or not initiate the follow-up transmission after providing the energy information indicating the remaining energy of the device being insufficient.

Proposal 4: For general format of D2R MAC PDU, the energy status indication could be always included, with value 1 or 0 to indicate whether the remaining energy of the device is insufficient

Proposal 5: The device stops the ongoing procedure after providing the energy information indicating the remaining energy of the device being insufficient.

- Conclusion

We have the following observation and proposals for assistance information form device:

Observation 1: SA2 has discussed that the CN could determine the approximate D2R message size based on AF request or configuration per AF and AIoT service type.

Proposal 1: Confirm that the device provides 1-bit segmentation indication to indicate whether there is more data or not.

Proposal 2: If the size of available D2R data is larger than the resources for a D2R transmission, the device segments the D2R data and sets the segmentation indication to 1.

Proposal 3: If the size of available D2R data is not larger than the resources for the D2R transmission, the device sets the segmentation indication to 0.

Proposal 4: For general format of D2R MAC PDU, the energy status indication could be always included, with value 1 or 0 to indicate whether the remaining energy of the device is insufficient

Proposal 5: The device stops the ongoing procedure after providing the energy information indicating the remaining energy of the device being insufficient.

- Reference

- 3GPP TR 38.769 V19.0.0, “Study on solutions for Ambient IoT in NR.”

- R2-2500052, “LS on A-IoT Conclusions in SA WG2.”

- S2-2501849, “Resolving EN about assistance information (Clause 5.4)”

- 3GPP TS 23.xyz V0.2.0, “Architecture support for Ambient power-enabled Internet of Things.”