3GPP TSG-RAN WG2 Meeting #129bis	R2-2502825

Wuhan, China, 7th -11th April, 2025 	Revision of R2-2500418

Agenda Item:	8.9.2

Source:			ASUSTeK

Title:	Discussion on time information for S&F

Document for: 	Discussion and Decision

- Introduction

In RAN2 #128 meeting, following agreements are made for Store & Forward operation:

Agreements:

1.	Time information can be broadcasted to indicate when the current satellite operation mode will transit from “S&F operation” mode to real-time/normal mode. RAN2 assumes that a R19 UE could use this information at least to delay some NAS procedures until the feeder link is resumed (FFS what we specify in RAN2 specs for this and if we differentiate the behaviour for R19 UEs supporting and not supporting S&F mode, also considering the relation to the Wait Timer). FFS which SIB is used (SIB1 or SIB31). FFS if absolute or relative time is used. FFS on the opposite transition (i.e. whether to indicate when the “S&F operation” will start)

2.	It is FFS whether we need to introduce a new release cause when the NW wants to release a R19 UE to RRC idle because the feeder link becomes unavailable

3.	“S&F operation” indication is provided in SIB1

4.	RAN2 assumes that AS signalling is not needed to indicate the architecture option (Split MME or Full CN).

5.	RAN2 waits for further progress in SA2, if any, before working on RAN2 aspects of a CIoT-UP solution for SF operation

6.	RAN2 understands that the UE configured with a satellite ID list by MME is not prevented to camp on a satellite operating in normal IoT NTN mode (i.e. with feeder-link connection), and perform subsequent access and data/signalling communication with that satellite

7.	RAN2 understands the UE configured with a satellite ID list by MME is not prevented to camp on, attempt to access to and communicate with a satellite which is not included in the MME-configured satellite list.

And in RAN2 #129 meeting, following agreements are made:

Agreements:

- RAN2 currently assumes that a S&F mode changes does not necessarily imply a MME change

- The already agreed time information on transition from S&F operation mode to normal mode is provided in SIB31 (can work on signalling optimizations in case the new time information and T-service would carry the same information)

- Use absolute UTC time to broadcast the (S&F-> normal mode) time information. When this information is broadcast and indicates a time in the future the UE can assume the NW is currently operating in S&F mode (until the indicated S&F-> normal mode transition time).

- A change to the (S&F-> normal mode) time indication shall not result in system information change notification. Legacy rules for acquiring SIB31 apply

In this contribution, we would like to further discuss the time information for S&F mode.

- Discussion

The Store and Forward (S&F) operation in IoT NTN is intended to provide some level of communication service for UEs under satellite coverage with intermittent/temporary satellite connectivity for delay-tolerant communication service. For example, when feeder link is unavailable for a satellite, the IoT NTN network would operate in S&F mode as shown in Figure 1. And when feeder link becomes available, the IoT NTN network would switch to real-time mode as shown in Figure 2.

Figure 1: An example for S&F mode in TR 22.865 [1]

Figure 2: An example for real-time mode in TR 22.865 [1]

In previous meetings, it has been agreed that the network broadcasts a “S&F operation indication” via SIB1. The indication would indicate that the NTN cell is operating in S&F operation mode. If the S&F operation indication is not provided in an NTN cell, the UE considers that the NTN cell is operating in a normal mode (i.e., real-time mode). And the network broadcasts a time information on transition from S&F operation mode to normal mode via SIB31. For the opposite transition (i.e., time information on transition from normal mode to S&F operation mode), it was discussed in the last meeting and marked as come back.

For a NTN cell supporting S&F operation mode, the satellite would switch between S&F operation mode and normal mode. That is to say, the two operation modes would work at a different time duration. For example, when the satellite currently works in S&F operation mode, the UE would receive the S&F operation indication in SIB1. If the S&F operation indication indicates “not barred”, the UE accesses the cell and receives the time information on transition from S&F operation mode to normal mode via SIB31. The UE could know when the S&F operation mode will end based on the time information.

For another example, when the satellite currently works in normal mode, the UE would not receive the S&F operation indication in SIB1. The UE accesses the cell if legacy barring bit is “not barred”. However, the satellite would switch to S&F operation mode afterward. Since the S&F operation indication can only indicate the current state, the UE would not have the knowledge about this mode switch. On the other hand, t-service indicates the time information on when the NTN cell is going to stop serving the currently covering area, which is irrelevant to satellite operation modes.

Observation 1: The current indication and time information are not enough for the UE to have the knowledge about the satellite operation mode switching.

The UE behaviour when the UE is in S&F operation mode would be limited compared to the UE behaviour in real-time mode. And there would be some drawback if the UE cannot know when to switch to S&F mode in advance. For example, without the time information on transition to S&F operation mode, when there is UL data arrival that cannot be operated in S&F operation mode at the very end of normal mode, the UE would initiate a scheduling request upon UL data arrival, which may cause useless signaling overhead and power consumption. It is likely that the UE receives a UL grant after switching to S&F operation mode, and the UL grant cannot be used due to the UL data cannot be operated in S&F operation mode.

Observation 2: The UE with time sensitive data would initiate a scheduling request after the satellite switches to S&F operation mode, which may cause useless signaling overhead and power consumption.

Introducing the time information on transition from normal mode to S&F operation mode could solve the about issue. With the time information on transition from/in S&F operation mode, the UE could know about when the S&F operation would be started and stopped on an NTN cell. As for how to indicate the time information on transition from normal mode to S&F operation mode, it could also be an absolute UTC time or be a time duration.

Proposal 1: In addition to time information about when to switch to normal mode, the network also provides the UE time information about when to switch to S&F mode.

Based on the time information, the UE could determine when to switch between normal mode and S&F operation mode. As agreed in previous meeting, the UE could delay some NAS procedures in the S&F operation mode. Moreover, the UE could determine whether to perform non-delay-tolerant transmission, which is not suitable in S&F operation mode, based on current satellite operation mode. The UE could also use the time information to determine whether to establish a RRC connection. For example, the UE with non-delay-tolerant data could prevent accessing or camping on a cell which is going to operate in S&F operation mode. In other hand, the UE with non-delay-tolerant data could access a cell which is going to operate in normal mode, e.g., after the time that the cell switches to normal mode.

Proposal 2: The UE could determine whether to request uplink resources and whether to perform RRC connection (re-)establishment/resume procedure based on the time information related to S&F mode.

- Conclusion

We have the following observations and proposals for S&F time indication:

Observation 1: The current indication and time information are not enough for the UE to have the knowledge about the satellite operation mode switching.

Observation 2: The UE with time sensitive data would initiate a scheduling request after the satellite switches to S&F operation mode, which may cause useless signaling overhead and power consumption.

Proposal 1: In addition to time information about when to switch to normal mode, the network also provides the UE time information about when to switch to S&F mode.

Proposal 2: The UE could determine whether to request uplink resources and whether to perform RRC connection (re-)establishment/resume procedure based on the time information related to S&F mode.

- Reference

- 3GPP TR 22.865 v19.2.0, “Study on satellite access Phase 3.”