3GPP TSG-RAN2 Meeting #129bis	R2-2502830

# Wuhan, China, 7-11 April, 2025

Agenda Item:	8.13.3

Source:	ASUSTeK

Title:	Issues on SRAP operations for supporting multi-hop L2 U2N Relay

Document for:	Discussion and Decision

1	Introduction

RAN2#128 made the following agreements on SRAP for multi-hop L2 U2N Relay:

Maintain the principle that the SRAP header enables mapping from the e2e bearer ID to the egress PC5 RLC channel.

Remote UE ID and BEARER ID are included in Rel-19 SRAP header for multi-hop L2 U2N relay.

…

The figure and description under P1 of R2-2410006 serves as a baseline connection establishment procedure for discussion for multi-hop U2N Relays if Approach 1 (all relay UEs must be in RRC_CONNECTED when the remote UE is in RRC_CONNECTED) is adopted.

This document discusses 3 issues on SRAP operations for supporting multi-hop L2 U2N Relay.

2	Discussion

2.1	SRAP header between relay UEs for SRB0 message forwarding during Remote UE connection establishment with gNB in multi-hop L2 U2N Relay

According to subclause 16.12.2.1 in TS 38.300 [1], for L2 U2N Remote UE's message on SRB0 in single-hop L2 U2N Relay, the SRAP header is not present over the PC5 hop between the Remote UE and the L2 U2N Relay UE, while the SRAP header is present over the Uu hop between the L2 U2N Relay UE and the gNB. In case of multi-hop L2 U2N Relay, the First Relay UE will forward the RRCSetupRequest message received from the Remote UE to the gNB via its parent Intermediate Relay UE and the Last Relay UE. It is not clear whether the SRAP header is present or not over the PC5 hop between relay UEs in the multi-hop relay path for RRCSetupRequest message forwarding.

Observation 1: It is not clear whether the SRAP header is present or not over the PC5 hop between Relay UEs in the multi-hop relay path for Remote UE’s SRB0 message forwarding.

According to the baseline connection establishment procedure specified in R2-2410006 [2], the L2 U2N Remote UE sends the first RRC message (i.e., RRCSetupRequest) for its connection establishment with gNB via the First Relay UE, using a specified PC5 Relay RLC channel configuration.  And, the first Relay UE sends the SidelinkUEInformationNR message to request for the dedicated configurations required to support the multi-hop relay operation for the U2N Remote UE. It is supposed that the dedicated configurations may include the SRAP configuration for the Remote UE and the SRAP configuration may include sl-LocalIdentity of the remote UE and sl-MappingToAddModList indicating RB-to-RLC channel mapping. With the SRAP configuration for the Remote UE, it is feasible for the First Relay UE to include SRAP header over the PC5 hop between the First Relay UE and its parent Intermediate Relay UE for Remote UE’s RRCSetupRequest message forwarding. Similarly, the Intermediate Relay UE may also be configured with the SRAP configuration for the Remote UE so as to include SRAP header over the PC5 hop between the Intermediate Relay UE and the Last Relay UE for RRCSetupRequest message forwarding.

Observation 2: Each Relay UE in the multi-hop relay path can be provided with the SRAP configuration for the Remote UE during Remote UE connection establishment with gNB.

Proposal 1:  SRAP header is present over the PC5 hop between Relay UEs in the multi-hop relay path for Remote UE’s SRB0 message forwarding.

2.2	SRAP configuration for supporting multi-hop L2 U2N Relay

According to the baseline connection establishment procedure specified in R2-2410006 [2], the gNB may provide SL-L2RelayUE-Config to each relay UE in the multi-hop relay path when a L2 U2N Remote UE is added. The SL-L2RelayUE-Config may include SL-SRAP-ConfigRelay indicating sl-LocalIdentity of the Remote UE and sl-MappingToAddModList indicating RB-to-RLC channel mapping.  When receiving a downlink SRAP PDU, each Relay UE in the relay path needs to determine the egress link for forwarding the SRAP PDU.

In R17 single-hop L2 U2N Relay, sl-L2IdentityRemote of the Remote UE is also included in the SL-L2RelayUE-Config to associate the SL-SRAP-ConfigRelay with the Remote UE so the L2 U2N Relay UE can determine the egress link for the downlink SRAP PDU based on the sl-L2IdentityRemote configured for the sl-LocalIdentity included in the SRAP header according to TS 38.351 [3]. However, for multi-hop L2 U2N Relay, if the Relay UE is not the First Relay UE, the egress link of a downlink SRAP PDU should be the child Relay UE of the Relay UE which received the downlink SRAP PDU. To enable the Relay UE to determine the egress link for the downlink SRAP PDU, a similar way as R17 single-hop L2 U2N Relay could be used i.e. the gNB includes the L2 ID of the child Relay UE in the SL-L2RelayUE-Config to associate the SL-SRAP-ConfigRelay of the Remote UE with the child Relay UE.

Observation 3: In multi-hop L2 U2N Relay, the egress link of a downlink SRAP PDU is the child Relay UE of a Relay UE (instead of the Remote UE) if the Relay UE is not the First Relay UE.

Proposal 2:  For supporting multi-hop L2 U2N Relay, the gNB provides the L2 ID of the child Relay UE of a Relay UE together with the SRAP Configuration of a Remote UE, when adding the Remote UE to the Relay UE, for the Relay UE to determine the egress link for downlink SRAP PDUs.

2.3	Uplink SRAP PDU examination for multi-hop L2 U2N Relay

If proposal 2 in the previous section is agreeable, there may be multiple sl-LocalIdentities configured to a Relay UE together with the L2 ID of the child Relay UE to associate the SL-SRAP-ConfigRelay of each Remote UE with the child Relay UE. In other words, there may be multiple sl-LocalIdentities associated with the L2 ID of the child Relay UE (or the L2 ID of the ingress link) of the Relay UE.

Observation 4: In multi-hop L2 U2N Relay, there may be multiple Remote UE Local IDs associated with the L2 ID of a child Relay UE (or the L2 ID of the ingress link) of a Relay UE.

In single-hop U2N Relay, handling of erroneous uplink protocol data in the L2 U2N Relay UE is quoted below:

For U2N Relay UE, when a SRAP Data PDU with SRAP header that contains a UE ID field or BEARER ID field which does not match sl-LocalIdentity or sl-RemoteUE-RB-Identity included in sl-SRAP-ConfigRelay is received except in the case where the SRAP Data PDU from SL-RLC1 as specified in TS 38.331 [3] is the first SRAP Data PDU received from a U2N Remote UE, or when a SRAP Data PDU that contains a UE ID which does not match the concerned sl-LocalIdentity corresponding to sl-L2IdentityRemote of the ingress link is received by U2N Relay UE, the SRAP entity shall:

-	Discard the received SRAP Data PDU.

As specified the paragraph above, the L2 U2N Relay UE in single-hop U2N Relay examines whether the UE ID included in an uplink SRAP PDU matches the sl-LocalIdentity corresponding to sl-L2IdentityRemote of the ingress link. In case of multi-hop L2 U2N Relay, if a Relay UE is not the First Relay UE, the ingress link of the uplink SRAP PDU is the child Relay UE of the Relay UE (instead of the Remote UE) and thus the legacy examination for R17 is not valid for the Relay UE. Instead, the Relay UE should examine whether the UE ID included in an uplink SRAP PDU matches one of the Local IDs associated with the L2 ID of the child Relay UE (or the L2 ID of the ingress link) of the Relay UE.

Observation 5: In multi-hop L2 U2N Relay, the legacy UE ID examination on an uplink SRAP PDU is not valid for a Relay UE other than the First Relay UE.

Proposal 3: In multi-hop L2 U2N Relay, a Relay UE other than the First Relay UE shall examine whether the UE ID included in an uplink SRAP PDU matches one of the Local IDs associated with the L2 ID of the child Relay UE (i.e. the L2 ID of the ingress link) of the Relay UE.

3 	Conclusion

In this contribution, we raise the following observations and proposals:

Observation 1: It is not clear whether the SRAP header is present or not over the PC5 hop between Relay UEs in the multi-hop relay path for Remote UE’s SRB0 message forwarding.

Observation 2: Each Relay UE in the multi-hop relay path can be provided with the SRAP configuration for the Remote UE during Remote UE connection establishment with gNB.

Observation 3: In multi-hop L2 U2N Relay, the egress link of a downlink SRAP PDU is the child Relay UE of the Relay UE (instead of the Remote UE) if the Relay UE is not the First Relay UE.

Observation 4: In multi-hop L2 U2N Relay, there may be multiple Remote UE Local IDs associated with the L2 ID of the child Relay UE (or the L2 ID of the ingress link) of a Relay UE.

Observation 5: In multi-hop L2 U2N Relay, the legacy UE ID examination on an uplink SRAP PDU is not valid for a Relay UE other than the First Relay UE.

Proposal 1:  SRAP header is present over the PC5 hop between Relay UEs in the multi-hop relay path for Remote UE’s SRB0 message forwarding.

Proposal 2:  For supporting multi-hop L2 U2N Relay, the gNB provides the L2 ID of the child Relay UE of a Relay UE together with the SRAP Configuration of a Remote UE, when adding the Remote UE to the Relay UE, for the Relay UE to determine the egress link for downlink SRAP PDUs.

Proposal 3: In multi-hop L2 U2N Relay, a Relay UE other than the First Relay UE shall examine whether the UE ID included in an uplink SRAP PDU matches one of the Local IDs associated with the L2 ID of the child Relay UE (i.e. the L2 ID of the ingress link) of the Relay UE.

4 	References

[1] TS 38.300 v18.4.0, NR; NR and NG-RAN Overall Description; Stage 2 (Release 18)

[2] R2-2410006, Report of [POST127][402][Relay] Multi-hop relay control plane

[3] TS 38.351 v18.2.0, NR; Sidelink Relay Adaptation Protocol (SRAP) Specification