3GPP TSG-RAN WG2 Meeting #129b	R2-2502871

Wuhan, China, 7th – 11th Apr, 2025

Agenda Item:	8.9.2

Source: 	TCL

Title:  	Discussion on Store & Forward satellite operation

Document for:	Discussion and Decision

- Introduction

In Rel-19 IoT NTN WID [1], Store & Forward satellite operation is one of the features to be supported.

- Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore: - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2] - At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3] Note: Strive to minimise UE impact. Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required

This contribution would like to provide our considerations on this objective of Rel-19 IoT NTN.

- Discussion on Store & Forward satellite operation

Below are the agreements related to the remainig issues made in the previous RAN2 meetings.

RAN2#128 Agreements:

1.	Time information can be broadcasted to indicate when the current satellite operation mode will transit from “S&F operation” mode to real-time/normal mode. RAN2 assumes that a R19 UE could use this information at least to delay some NAS procedures until the feeder link is resumed (FFS what we specify in RAN2 specs for this and if we differentiate the behaviour for R19 UEs supporting and not supporting S&F mode, also considering the relation to the Wait Timer). FFS which SIB is used (SIB1 or SIB31). FFS if absolute or relative time is used. FFS on the opposite transition (i.e. whether to indicate when the “S&F operation” will start)

2.	It is FFS whether we need to introduce a new release cause when the NW wants to release a R19 UE to RRC idle because the feeder link becomes unavailable

RAN2#129 Agreements:

Agreements:

- RAN2 currently assumes that a S&F mode changes does not necessarily imply a MME change

- The already agreed time information on transition from S&F operation mode to normal mode is provided in SIB31 (can work on signalling optimizations in case the new time information and T-service would carry the same information)

- Use absolute UTC time to broadcast the (S&F-> normal mode) time information. When this information is broadcast and indicates a time in the future the UE can assume the NW is currently operating in S&F mode (until the indicated S&F-> normal mode transition time).

- A change to the (S&F-> normal mode) time indication shall not result in system information change notification. Legacy rules for acquiring SIB31 apply

Additionally, regarding the issue whether “The time information can also be broadcasted to indicate when the current satellite operation mode will transit from normal mode to S&F operation mode”, the chair suggested to leave it to the next meeting for discussion.

We summarize the benefits and drawbacks of introducing the time information of normal mode to S&F operation mode.

Broadcasting of Normal-to-S&F mode transition time offers the following advantages:

1. Prioritized Real-Time Data Handling:

UEs can proactively send high-priority real-time data (e.g., emergency IoT sensor data) before the satellite enters S&F mode.

2. Dynamic Service Adjustment:

UEs can reduce service priority (e.g., switching video streams to text-based transmission) to adapt to potential delays in S&F mode.

3. Guided Cell Reselection:

By triggering UEs to reselect cells in advance, non-S&F-capable UEs can switch to other satellites in normal mode or terrestrial networks.

While some companies argue that cell reselection is unnecessary if the cell remains accessible, however theoretically, no existing mechanism prevents non-S&F UEs from accessing the cell in normal mode excepting for barring bit.

4. Network Load Optimization:

Satellites can guide non-essential UEs to leave before entering S&F mode, minimizing invalid access attempts during S&F operation.

Drawbacks of Introducing Dedicated Normal-to-S&F Time Indication:

1. Redundancy with Existing Parameters:

The `t-service` parameter already provides the satellite’s service termination time, enabling UEs to initiate measurements and handovers in advance.

2. Dynamicity and Accuracy Challenges:

Low Earth Orbit (LEO/MEO) satellites have fast-moving orbits, making it difficult to predict transition times precisely. Large deviations between broadcasted and actual times could lead to UE misjudgments (e.g., premature service termination or missed handover opportunities).

3. Increased Protocol Complexity:

Managing dual-directional transitions (Normal-to-S&F and S&F-to-Normal) requires additional signaling and storage resources, complicating satellite-UE coordination.

Based on the analysis above, we think the benefits of a dedicated Normal-to-S&F transition time indication can also be achieved through the existing t-Service parameter. Some companies emphasized that `t-service` is primarily designed for mobility management, while mode transitions are a distinct operational aspect, and suggested not to link these two parameters. However, network operators can achieve similar benefits by strategically configuring `t-service` values, allowing UEs to optimize their behavior autonomously.

Furthermore, introducing dual-directional transitions lead to overcomplicating the protocol and creating redundancy. Existing mechanisms, such as cell barring and `t-service`, already provide sufficient tools for UEs to handle mode transitions indirectly.

Therefore, we propose not to introduce dedicated Normal-to-S&F transition time indication. Instead, leverage `t-service` and existing cell selection/barring rules to achieve equivalent outcomes efficiently.

- No need to introduce the time information to indicate when the current satellite operation mode will transit from normal mode to S&F operation mode.

Another leftover issue is whether we need to introduce a new release cause when the NW wants to release a R19 UE to RRC_IDLE mode because the feeder link becomes unavailable. We believe that whether to introduce a new release cause depends on whether there is a corresponding specific behavior related to the new cause. If the UE is simply released to RRC_IDLE mode without any special actions, there is no need to introduce a new release cause specifically to inform the UE of the current state of the feeder link. Therefore we propose not to introduce a new release cause for feeder link becoming unavailable.

- No need to introduce a new release cause for feeder link becoming unavailable.

3. Conclusion

Based on the discussion above, we made the following observations and proposals:

- No need to introduce the time information to indicate when the current satellite operation mode will transit from normal mode to S&F operation mode.

- No need to introduce a new release cause for feeder link becoming unavailable.

4. Reference

[1] RP-242397, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, MediaTek