3GPP TSG-RAN2 Meeting #129-bis	R2-2502910

Wuhan, China, 7 - 11 April 2025 	Revision of R2-2501093

Agenda Item:	8.4.2 Procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE

Source: 	Ericsson

Title:  	LP-WUS in Idle and Inactive

Document for:	Discussion and Decision

# Introduction

In this contribution the open issues for LP-WUS in Idle and Inactive are discussed further.

For the RAN1 and RAN2 agreements in previous meetings the reader is referred to the Annex.

# Discussion

## LP-WUS and eDRX

LP-WUS can be supported with eDRX with limited specification impact, similar as PEI is supported with eDRX, i.e. during the PTW the same PO monitoring rules apply as for normal DRX.

The only change to the specification agreed for PEI with eDRX was to allow for a larger UE_ID value by using 15 bits of 5G-S-TMSI (5G-S-TMSI mod 32768) in 38.304. The UE_ID is further discussed in section 2.2 below.

Simulations results have shown that the use of LP-WUS with eDRX may not always be beneficial. For example when the eDRX cycle is very long (e.g. larger than 500 sec), then the MR ultra-deep sleep power consumption (Relative Power unit 0,015 in TR 38.869) becomes the determining factor for the battery lifetime and it does not matter whether LP-WUS is used or not. Therefore, over-optimization of the combination or LP-WUS and eDRX is not worth the effort.

LP-WUS is supported with eDRX without additional changes. I.e. the UE monitors LP-WUS during PTW instead of monitoring normal DRX during PTW as in regular eDRX.

## UE_ID for LP-WUS

RAN1 agreed that the maximum number of subgroups for LP-WUS is 31. Additionally, one codepoint, i.e. “subgroup”, is used to wake up all the UEs, e.g. to indicate systemInfoModification or etwsAndCmasIndication. This means that 5 more bits from the UE_ID are needed to uniformly distribute the UEs over the LP-WUS subgroups. In last meeting RAN2 briefly discussed which bits of the UE_ID to use for LP-WUS subgrouping but did not reach an agreement.

DRX

In NR the temporary identifier 5G-S-TMSI is used to avoid the privacy risks of using a permanent UE_ID (as the IMSI in LTE). The 5G-S-TMSI is updated for example during initial registration and service request procedures. Only the 5G-S-TMSI (48 bits) is included in the Paging message (or I-RNTI for RAN paging), i.e. the IMSI is not included.

The 48-bit 5G-S-TMSI is comprised of 10-bit AMF Set ID, 6-bit AMF Pointer and 4-octet 5G-TMSI. Therefore the 32-bit 5G-TMSI is unique within the AMF that allocated it and is suitable for paging.

To determine the PO of the UE with normal DRX 10 bits of the 5G-S-TMSI are used.

For RAN paging 10 bits of the 5G-S-TMSI are signalled in Core Network Assistance Information for RRC INACTIVE IE in INITIAL CONTEXT SETUP REQUEST message and stored in RAN to determine the PO of the UE. For CN paging the full 5G-S-TMSI is included in the PAGING message from the CN which is also included in the Paging message by gNB but is not stored in RAN.

eDRX ≤ 10,24 sec in RRC_INACTIVE

In Rel-17, if the UE is configured with eDRX via NAS signalling, see eDRX in RRC_IDLE below, then the RAN may configure an eDRX up to 10,24 sec (TeDRX, RAN up to 10,24 sec in ExtendedPagingCycle-r17 {rf256, rf512, rf1024}) provided that the eDRX in RRC_INACTIVE is smaller or equal to the eDRX negotiated via NAS.

The PO for eDRX up to 10,24 sec in RRC_INACTIVE is calculated with the DRX formula using T = 1024, in which case 12 bits of the 5G-S-TMSI are used. For these 12 bits of the 16-bit Extended UE Identity Index Value IE introduced in Rel-16 are used.

PEI

In Rel-17 up to 8 subgroups for UE-ID based subgrouping with PEI can be configured which require 3 bits of the 5G-S-TMSI (maxNrofPagingSubgroups-r17 = 8). Thus when PEI is configured 13 bits of the 5G-S-TMSI are needed when normal DRX is used and 15 bits are needed when eDRX up to 10,24 sec in RRC_INACTIVE is configured. For 13 or 15 bits the 16-bit Extended UE Identity Index Value IE introduced in Rel-16 is used.

eDRX > 10,24 sec in RRC_IDLE and RRC_INACTIVE

In Rel-17 the UE can request to be configured with eDRX in RRC_IDLE up to 2,91 h (TeDRX, CN) for CN paging via NAS protocol.

In Rel-18 the RAN may configure the UE with an eDRX up to 2,91 h (TeDRX, RAN) in RRC_INACTIVE provided that:

- TeDRX, RAN ≤ TeDRX, CN, and

- AMF confirmed to store DL signalling/data while the UE is in eDRX with TeDRX, RAN in RRC_INACTIVE

When there is DL data/signalling in the AMF then the AFM will request the gNB to start RAN paging just before the PTW is about to start using RAN PAGING REQUEST message. The gNB triggers RAN paging when there is DL data/signalling in the buffer.

10 bits of the 5G-S-TMSI could have been used to distribute the UEs over the 1024 Paging Hyperframes (PH). However, using the more random hashed 5G-S-TMSI provides a better uniform distribution over the PHs. Furthermore 3 bits are used to indicate the start of the PTW within the PF. During the PTW the formula for normal DRX with 5G-S-TMSI is used to determine the POs inside the PTW. The 13 bits of the hashed 5G-S-TMSI are signalled in the Core Network Assistance Information for RRC INACTIVE IE in INITIAL CONTEXT SETUP REQUEST message and stored in RAN to determine the PO for eDRX in RRC_INACTIVE.

Either 5 more bits of the 5G-S-TMSI or the hashed 5G-S-TMSI can be used for UE-ID based LP-WUS subgrouping.

Currently up to 15 bits of the 32-bit 5G-TMSI are used, i.e. when eDRX and PEI are configured. With 32 bits more than 4 billion values can be generated. In practice the operator is only using a fraction of the 5G-TMSI range and values are allocated incrementally. This means that the least significant bits are more random, and the most significant bits of the assigned 5G-TMSI are the same for different UEs in worse case. The hashed 5G-TMSI resolves this and achieves a more “random” UE_ID. Therefore, for eDRX, which uses the most significant bits, the hashed 5G-TMSI was used. A similar approach could be considered for LP-WUS.

Using 5 more bits of the hashed 5G-S-TMSI instead of 5G-S-TMSI ensures that UEs are uniformly distributed over all LP-WUS subgroups.

The Hashed UE Identity Index Value IE can be extended, i.e. no new IE is required:

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Extended UE Identity Index Value | M |  | BIT STRING (SIZE(16)) | Encoded as 5G-S-TMSI mod 65536. |

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Hashed UE Identity Index Value | M |  | BIT STRING (SIZE(13, …)) |  |

The Hashed UE Identity Index Value IE can be extended, i.e. no new IE is required for LP-WUS subgrouping.

Based on the above discussion and observations it is proposed:

Ask RAN3 to provide 18 bits of the Hashed UE Identity Index Value IE for UE-ID based LP-WUS subgrouping in Rel-19.

## LP-WUS and emergency PDU session

How to handle UE-ID based subgrouping during an emergency PDU session is FFS in RAN2:

- FFS whether/how to handle the case for UE-ID based subgrouping when the UE has an emergency PDU session.

In SA2 the use of LP-WUS during an emergency PDU session is also FFS (see section 5.4.12a.2 in 23.501):

Editor's note:	The use of LP-WUSPS for emergency sessions and emergency call back is FFS.

With LP-WUS the paging delay is increased significantly due to the large time offset between LP-WUS and the associated PO, i.e. high probability that the paging message is delayed with one DRX cycle.The LP-WUS time offset may even exceed the DRX cycle, e.g. DRX cycle is 640 ms and time offset is 900 ms. Which means that the paging message is always delayed due to the time offset.

When the UE has an emergency PDU session then eDRX is not used because eDRX may delay an (emergency) callback. Furthermore CN-assigned PEI is also not allowed when the UE has an emergency PDU session (see 24.501 section 5.3.25) for latency reasons. If proposal 3 is agreeable, then RAN2 should inform CT1, RAN3 and SA2. These WGs can then make the appropriate changes in their specification.

When the UE has an emergency PDU session LP-WUS is not used. Send an LS to CT1, RAN3 and SA2 to inform them.

If proposal 3 is agreeable, then RAN2 should inform CT1, RAN3 and SA2. These WGs can then make the appropriate changes in their specification.

## LP-WUS in lastUsedCellOnly

With PEI it is possible to limit the use of PEI to the cell where the UE was last in RRC_CONNECTED, i.e. when lastUsedCellOnly flag is set in PEI-Config in SIB1. In such case the UE only monitors PEI when it is (still) camped on the cell where it was released from RRC_CONNECTED the last time. Unless noLastCellUpdate was received in the last RRCRelease message, i.e. the NW can use this flag in the release when the last visited cell info is not sent/updated in the CN e.g. when there was SDT without UE context relocation. When lastUsedCellOnly is configured then the PEI transmissions are limited to the first paging attempt of the NW, i.e. PEI is not transmitted during paging escalation when there is paging to all the cells of the registration / RNA area. The procedure would be beneficial also for LP-WUS:

The LP-WUS transmissions can be limited to lastUsedCellOnly (similar to PEI).

Also, in case the LP-WUS transmissions are not limited to the last used cell, then power saving gain can be provided to other UEs by introducing a “mobility bit”. This can be used to prevent waking up “stationary” UEs when the NW is paging for “mobile” UEs:

- The gNB sets the mobility bit when it is paging a UE that has moved, i.e. when the UE is paged in another cell than it was last released.

- When a UE is monitoring paging in the same cell as it was released and the mobility bit is set in LP-WUS then the UE does not wake-up.

This approach has the benefit that UEs that have moved, can also use LP-WUS, but UEs that have not moved can avoid waking up when the NW is paging for UEs that have moved:

LP-WUS indicates when there is paging for a UE that has moved, i.e. the gNB is paging the UE in a cell different from the last used cell. A UE monitoring LP-WUS in its last used cell (i.e. has not moved) does not wake-up when this indication is received in LP-WUS.

## Radio paging capabilities

SA2 indicated in the LS [1] that there can be inter-operability issues when the UE performs initial registration with a legacy gNB that does not understand/support the paging capabilities signalled by the UE. It is the task of the gNB to copy the paging capabilities received from the UE in UECapabilityInformation message into the paging capability container on NGAP interface (i.e. UERadioPagingInformation IE).

In LTE the UE sends the paging capabilities separately in a paging capability container, and the eNB only have to copy the paging capability container received over Uu into the paging capability container sent over S1 interface. This ensures that when new paging capabilities are introduced for new features, since these new capabilities are correctly transferred to CN also upon initial registration in a legacy eNB which does not support the new feature.

In NR a paging capability container on Uu interface was introduced in Rel-17 and the UE-ID based PEI capability was added to the paging container. To avoid backwards compatibility issues the supported NR band list for paging and scheduling offset for PDSCH was not moved into the container. Unfortunately other Rel-17 and Rel-18 paging capabilities have been overlooked to be included in the Rel-17 paging container:

UERadioPagingInformation ::= SEQUENCE {

…

ueRadioPagingInformation            UERadioPagingInformation-IEs,

…

UERadioPagingInformation-IEs ::=    SEQUENCE {

supportedBandListNRForPaging        SEQUENCE (SIZE (1..maxBands)) OF FreqBandIndicatorNR   OPTIONAL,

…

UERadioPagingInformation-v15e0-IEs ::= SEQUENCE {

dl-SchedulingOffset-PDSCH-TypeA-FDD-FR1     ENUMERATED {supported}                         OPTIONAL,

…

dl-SchedulingOffset-PDSCH-TypeB-TDD-FR2     ENUMERATED {supported}                         OPTIONAL,

…

UERadioPagingInformation-v1700-IEs ::= SEQUENCE {

ue-RadioPagingInfo-r17                 OCTET STRING (CONTAINING UE-RadioPagingInfo-r17)    OPTIONAL,

inactiveStatePO-Determination-r17      ENUMERATED {supported}                               OPTIONAL,

numberOfRxRedCap-r17                   ENUMERATED {one, two}                                OPTIONAL,

halfDuplexFDD-TypeA-RedCap-r17         SEQUENCE (SIZE (1..maxBands)) OF FreqBandIndicatorNR OPTIONAL,

…

UERadioPagingInformation-v1800-IEs ::= SEQUENCE {

numberOfRxERedCap-r18                  ENUMERATED {one, two}                                OPTIONAL,

supportOf2RxXR-r18                     ENUMERATED {supported}                               OPTIONAL,

…

UERadioPagingInformation-v1840-IEs ::= SEQUENCE {

dl-SchedulingOffset-PDSCH-TypeA-FDD-FR2-NTN-r18    ENUMERATED {supported}                   OPTIONAL,

dl-SchedulingOffset-PDSCH-TypeB-FDD-FR2-NTN-r18    ENUMERATED {supported}                   OPTIONAL,

…

UE-RadioPagingInfo-r17 ::=            SEQUENCE {

-- R1 29-1: Paging enhancement

pei-SubgroupingSupportBandList-r17    SEQUENCE (SIZE (1..maxBands)) OF FreqBandIndicatorNR OPTIONAL,

...

}

The impact when paging capabilities not included in the paging container are missing in the PAGING message from the CN is briefly described in the table below:

| Paging capability | Impact when missing in the paging capabilities in the PAGING message from CN |
| --- | --- |
| supportedBandListNRForPaging halfDuplexFDD-TypeA-RedCap | gNB may page the (RedCap) UE where it does not support paging |
| dl-SchedulingOffset-PDSCH-TypeA and B (for NTN) | Cross-slot scheduling cannot be used to page the UE. However note that K0 = 0 has to be used for the other UEs that are paged in the PO that have not indicated to support cross-slot scheduling. |
| inactiveStatePO-Determination | In case the DRX cycle in idle and inactive is different the UE may calculate a different PO for monitoring CM-IDLE and RAN paging, in which case the UE would not detect RRC state mismatch when receiving paging in inactive. However the RRC state mismatch is resolved when the UE enters connected mode again. |
| numberOfRxERedCap numberOfRxRedCap supportOf2RxXR | The gNB does not know if the (e)RedCap UE or XR UE supports only 1 or 2 Rx, i.e. does not know when boosting is required. gNB may page the UE in a cell where it is not supported. |

From this table it can be concluded that there is no inter-operability issue with Paging when these paging capabilities are missing. But for example in case of (e)RedCap there can be some wasted system resources (e.g. paging in bands that the UE does not support, or boosting when it is not needed):

There are no inter-operability issues when the paging capabilities outside the paging container (i.e. ue-RadioPagingInfo-r17 IE) are missing the PAGING message from CN.

In case of missing (e)RedCap paging capabilities the gNB may use boosting when it is not needed or page in a band that the UE does not support.

ASN.1 corrections to the Rel-17 and Rel-18 paging capabilities have been discussed and not agreed in the past [5, 6]. These Rel-17/18 changes are not backwards compatible.

For LP-WUS, in case the gNB does not transfer the LP-WUS paging capability during initial registration to the CN, then there is an inter-operability issue with paging on a gNB that has LP-WUS configured. I.e., if initial registration was done in a legacy gNB which does not support or understand LP-WUS and the related signaling, later a gNB supporting LP-WUS would not realize the UE supports LP-WUS and the UE would become unreachable in that cell.

The LP-WUS UE-ID based subgrouping UE capability is included in the UE-RadioPagingInfo container.

## Minimum wake-up delay in UE capability

RAN1 is discussing that the gNB may configure a single or multiple time offset(s) in SIB. And that the UE signals the minimum wake-up delay it supports in its UE capability, from a set of 3 candidate values {[70ms], [500ms] and [900ms]} (for 20ms SSB periodicity). The minimum wake-up delay includes the synchronization delay using SSB(s). Furthermore, RAN1 has discussed that when the configured time offset is shorter than the minimum wake-up delay of the UE, then the UE falls back to legacy PO monitoring, or the UE receives the Paging message in the next PO when it is ready to receive paging.

Support of multiple time offsets per PO would lead to increased number of LP-WUS occasions and increased number of LP-WUS transmissions for paging UEs in PO and is therefore not desired.

Instead, if only the longer time offset is configured UEs supporting shorter wake-up could of course handle that, or if a shorter time offset is configured UEs only supporting a longer wake-up would either fall back to direct legacy PO monitoring, or associate the LP-WUS to the subsequent PO (whichever is later agreed).

When a single time offset is configured then there is one LP-WUS occasion associated with the PO. But without configuring more LP-WUS occasions, multiple LP-WUS occasions can be associated with a single PO.

A single time offset is configured per PO. FFS if UEs reporting a longer wake-up delay fall back to direct legacy PO monitoring or monitor an earlier LP-WUS occasion.

The FFS part of the proposal related to earlier LP-WUS occasion monitoring is explained with an example:

- UE signals its minimum time gap from a candidate set {[70ms], [500ms] and [900ms]} in UE capability.

- A single time offset (70 ms) is configured per PO.

- UEs that require 70 ms wake-up time use the configured LP-WUS occasion of the PO.

- UEs that requires a longer wake-up time use the LP-WUS occasion that is closest to the PO but that has a time offset that is larger or equal to the minimum wake-up latency of the UE.

- UE monitors legacy PO determined based on the UE identity, where the UE monitors LP-WUS or not.

In case the UE would monitor a different PO when it is using LP-WUS then this would increase the number of LP-WUS transmissions because the gNB does not know whether the UE is monitoring LP-WUS or not.

## Enabling/disabling LP-WUS operation in the UE

First there is a difference between enabling/disabling and activation/deactivation. Enabling/disabling LP-WUS operation in the UE is semi-static e.g. LP-WUS is disabled when statistics show that the UE is less reachable when using LP-WUS, e.g. due to poor UE implementation of LP-WUS. While activation/deactivation of LP-WUS is more dynamic and radio related e.g. when the UE moves in and out of LP-WUS coverage in the cell.

CN-assigned based subgrouping using LP-WUS can be disabled by the CN, i.e. the CN can decide not to configure a CN subgroup in the registration accept (assuming PEI baseline). However, the UE-ID based subgrouping is configured in the RAN and is transparent to the CN. But the NAS and CN signalling can easily be extended to enable the CN to enable/disable UE-ID based LP-WUS (and LP-WUS overall) for a specific UE:

- The UE can indicate in the registration request that it supports UE-ID based LP-WUS

- The CN can indicate in the registration accept whether the UE is allowed to use UE-ID based LP-WUS

- In case UE-ID based LP-WUS is disabled in the UE, the CN informs RAN:

  - Using the INITIAL CONTEXT SETUP REQUEST message for RAN paging

  - Using the PAGING message for CN paging

Given that the existing signalling already enables the CN to disable CN-assigned LP-WUS for a specific UE, it is easy to extend the existing signalling with UE-ID based LP-WUS. It is more complex using RRC signalling, and less motivated since the purpose is to e.g. disable LP-WUS for poor UE implementations, to disable UE-ID based subgrouping in RAN for a specific UE:

The CN has the ability to disable/enable LP-WUS in the UE during initial registration, but also has the opportunity to change this in the UE during registration triggered by mobility or periodic location update:

The existing signalling for CN assigned subgrouping is extended to enable the CN to enable/disable UE-ID based LP-WUS in the UE.

## LP-WUS entry/exit condition

To avoid ping pong with cell reselection the NW can configure a time to trigger:

In all cases, the UE shall reselect the new cell, only if the following conditions are met:

-	the new cell is better than the serving cell according to the cell reselection criteria specified above during a time interval TreselectionRAT;

-	more than 1 second has elapsed since the UE camped on the current serving cell.

The IE T-Reselection concerns the cell reselection timer TreselectionRAT for NR and E-UTRA. Value in seconds. For value 0, behaviour as specified in 7.1.2 applies.

T-Reselection ::=                   INTEGER (0..7)

To avoid ping pong with entry/exit for LP-WUS monitoring:

The NW can configure a time to trigger (seconds), i.e. minimum time during which the entry condition has to be fulfilled, before the UE may start LP-WUS monitoring.

## LP-WUS configuration

RAN2 agreed that the LP-WUS configuration can be provided in SIB. But RAN2 did not reach consensus whether LP-WUS configuration via dedicated RRC signalling is needed. We think that the LP-WUS configuration is common, i.e. cell specific, and there is no need for RRC signalling to enable UE specific LP-WUS configurations. In case LP-WUS configuration is provided in RRCRelease then there is complexity for how long the dedicated LP-WUS configuration remains valid, and in which cells/areas the LP-WUS configuration is valid. Furthermore LP-WUS configuration in dedicated signalling has been confused with enabling/disabling LP-WUS in a specific UE which is a separate discussion (see Section 2.7 above):

LP-WUS configuration is not provided in dedicated RRC signalling.

RAN2 agreed that separate thresholds can be configured for OFDM-based and OOK-based WUR. This implies that the presence of OFDM-based and/or OOK-based thresholds in SIB indicates whether the UE may use OFDM-based and/or OOK-based LP-WUS monitoring in the cell. E.g. when OOK-based threshold are not configured in the cell then the UE may not use OOK-based LP-WUS monitoring in the cell. When the UE uses OOK-based LP-WUS monitoring, the NW has to configure the LP-SS which increases the always-on broadcast and has a negative impact on NW energy efficiency. Further, less resources are required for the OFDM-based LP-WUS signal, and OFDM-based UE can use PSS/SSS for sync and serving cell RRM measurements:

gNB can configure in SIB whether only OFDM-based or both OFDM-based and OOK-based WUR are supported in the cell.

## UE_ID-based and CN-assigned subgroup signalling

RAN2 assumes that for LP-WUS the same signalling procedure and signalling messages as with PEI will be used. It is up to SA2/CT1/RAN3 to decide the signalling details, e.g. introduction of new IEs [1]:

Concerning LP-WUS subgrouping UE capability and LP-WUS configuration in SIB the following principles can be used:

The UE indicates LP-WUS support per band.

If the UE supports LP-WUS in a band then the UE also supports UE_ID-based subgrouping in that band. The UE may in addition also support CN-assigned subgrouping in bands where LP-WUS is supported.

The gNB can indicate in the LP-WUS configuration in SIB that only UE_ID-based subgrouping, only CN-assigned based subgrouping, or both are supported in the cell.

If the UE is configured with a CN-assigned subgroup ID and the gNB has configured both UE_ID-based and CN-assigned subgrouping then the UE uses the CN-assigned subgrouping ID.

The gNB does not use LP-WUS when the UE is paged in a band where the UE does not support LP-WUS, based on the radio paging capabilities in the PAGING message from CN for UEs in CM_IDLE and based on the UE capabilities for RAN paging for UEs in RRC_INACTIVE.

## LP-WUS monitoring rules

RAN1 agreed that when the UE does not detect LP-WUS or LP-WUS is received but the LP-WUS subgroup is not included then the UE is not required to monitor the following PO. RAN1 continues to discuss how to MOs can be grouped and used in case of multi beam operation e.g. MOs are repeated on a beam, or the UE only monitors specific MOs on a beam.

There can be cases where the UE is not able to monitor LP-WUS, e.g. during cell re-selection. In such case the UE should monitor the associated PO:

If the UE is not able to monitor the LP-WUS (FFS for which use cases) then the UE is required to monitor the following PO.

In case of multibeam operation each beam includes at least one LO and one MO:

Similar as with PEI/Paging it is up to UE implementation which beam(s) to monitor for Paging:

It is up to UE implementation on which beam(s) to monitor the LO.

# Summary

Observation 1	Either 5 more bits of the 5G-S-TMSI or the hashed 5G-S-TMSI can be used for UE-ID based LP-WUS subgrouping.

Observation 2	Using 5 more bits of the hashed 5G-S-TMSI instead of 5G-S-TMSI ensures that UEs are uniformly distributed over all LP-WUS subgroups.

Observation 3	The Hashed UE Identity Index Value IE can be extended, i.e. no new IE is required for LP-WUS subgrouping.

Observation 4	There are no inter-operability issues when the paging capabilities outside the paging container (i.e. ue-RadioPagingInfo-r17 IE) are missing the PAGING message from CN.

Observation 5	In case of missing (e)RedCap paging capabilities the gNB may use boosting when it is not needed or page in a band that the UE does not support.

Observation 6	When a single time offset is configured then there is one LP-WUS occasion associated with the PO. But without configuring more LP-WUS occasions, multiple LP-WUS occasions can be associated with a single PO.

Proposal 1	LP-WUS is supported with eDRX without additional changes. I.e. the UE monitors LP-WUS during PTW instead of monitoring normal DRX during PTW as in regular eDRX.

Proposal 2	Ask RAN3 to provide 18 bits of the Hashed UE Identity Index Value IE for UE-ID based LP-WUS subgrouping in Rel-19.

Proposal 3	When the UE has an emergency PDU session LP-WUS is not used. Send an LS to CT1, RAN3 and SA2 to inform them.

Proposal 4	The LP-WUS transmissions can be limited to lastUsedCellOnly (similar to PEI).

Proposal 5	LP-WUS indicates when there is paging for a UE that has moved, i.e. the gNB is paging the UE in a cell different from the last used cell. A UE monitoring LP-WUS in its last used cell (i.e. has not moved) does not wake-up when this indication is received in LP-WUS.

Proposal 6	The LP-WUS UE-ID based subgrouping UE capability is included in the UE-RadioPagingInfo container.

Proposal 7	A single time offset is configured per PO. FFS if UEs reporting a longer wake-up delay fall back to direct legacy PO monitoring or monitor an earlier LP-WUS occasion.

Proposal 8	The existing signalling for CN assigned subgrouping is extended to enable the CN to enable/disable UE-ID based LP-WUS in the UE.

Proposal 9	The NW can configure a time to trigger (seconds), i.e. minimum time during which the entry condition has to be fulfilled, before the UE may start LP-WUS monitoring.

Proposal 10	LP-WUS configuration is not provided in dedicated RRC signalling.

Proposal 11	gNB can configure in SIB whether only OFDM-based or both OFDM-based and OOK-based WUR are supported in the cell.

Proposal 12	The UE indicates LP-WUS support per band.

Proposal 13	If the UE supports LP-WUS in a band then the UE also supports UE_ID-based subgrouping in that band. The UE may in addition also support CN-assigned subgrouping in bands where LP-WUS is supported.

Proposal 14	The gNB can indicate in the LP-WUS configuration in SIB that only UE_ID-based subgrouping, only CN-assigned based subgrouping, or both are supported in the cell.

Proposal 15	If the UE is configured with a CN-assigned subgroup ID and the gNB has configured both UE_ID-based and CN-assigned subgrouping then the UE uses the CN-assigned subgrouping ID.

Proposal 16	If the UE is not able to monitor the LP-WUS (FFS for which use cases) then the UE is required to monitor the following PO.

Proposal 17	It is up to UE implementation on which beam(s) to monitor the LO.

# References

- S2-2412876, LS Reply on LP-WUS subgrouping, LS out, To: RAN2, RAN3, CT1, SA2#166

- S2-2412875, Support of LP-WUS Assistance, CR 23.501, vivo, Ericsson, Vodafone, Huawei, HiSilicon, SA2#166

- S2-2412877, LP-WUS subgroup support, CR 23.502, Ericsson, vivo, Huawei, HiSilicon, Vodafone, SA2#166

- R2-2411041, LS on emergency services and eDRX, LS out, To: SA2, CT1, CC: RAN3, RAN2#128

- R2-2208937, [AT119-e][013][NR17] RRC I, Ericsson, Report, RAN2#119-e

- R2-2208654, Correction on UERadioPagingInformation and UERadioPagingInfo container, Ericsson, CR 38.331, Rel-17, RAN2#119-e

# Annex: Agreements

## RAN1 agreements (selection)

R1-2500792	Summary #2 on LP-WUS operation in IDLE/INACTIVE mode	Moderator (Apple)

Agreement

For the LO to PO mapping from network perspective, support Option 2 (UEs corresponding to different POs monitor the same LO).

- This should not increase the maximum number of codepoints per LO/LP-WUS compared to Option 1.

- FFS conditions/restrictions for mapping multiple POs to one LO

- Down-select between 2 and 4 for max number of POs per LO.

R1-2500793	Summary #3 on LP-WUS operation in IDLE/INACTIVE mode	Moderator (Apple)

Agreement

For UE capability report on the wake-up delay:

- Alt 1: For the 3 candidate values for the wake-up delay capability report, support {[70ms], [500ms] and [900ms]}.

  - The reported values assume SSB periodicity of 20ms, where [70ms] assumes [3] SSBs needed for synchronization, [500ms] and [900ms] assume [5] SSBs needed for synchronization.

  - FFS: translation of wake-up delay for different SSB periodicities

  - FFS: different sets of 3 candidate value for different SSB periodicities

- Send an LS to RAN4 after the down-selection to confirm the number of SSBs. Final LS in R1-2501624.

Agreement

For the offset value(s) between an LO and a reference PO/PF, adopt Option 2B-1.

- gNB can configure 1 or 2 offset values.

  - FFS whether gNB can configure 3 offset values

- If multiple offset values are configured and if the gap between the LO associated with the largest offset value and the corresponding PO is no less than the wake-up delay a UE reports, the UE monitors the LO associated with the smallest offset value that has a gap between the LO and the PO no less than the wake-up delay.

  - Note: if a single offset value is configured, UE behaviour is according to Option 1-1.

- All the UEs supporting LP-WUS for idle/inactive mode supports the configuration of 2 offset values (FFS: 3 values).

R1-2500794	Summary #4 on LP-WUS operation in IDLE/INACTIVE mode	Moderator (Apple)

Agreement

Confirm the following working assumption with the modification:

Working Assumption

The maximum number of subgroups per PO supported in Rel-19 is 31 32.

## RAN2 agreements (selection)

On separate band issue

RAN2 understands that UE can report which band(s) is supported by LR to NW.

RAN2 understands that any potential overload issues could be addressed by current mechanism in spec.

Send LS to RAN1 and RAN4 to inform the agreements.

On sub-grouping

For UE_ID based subgrouping, similar formula defined for PEI subgrouping is reused for LP-WUS subgrouping, i.e.,

SubgroupID = (floor (UE_ID/(N*Ns*Np)) mod subgroupsNumForUEID) + (subgroupsNumPerPO – subgroupsNumForUEID), where

-	UE_ID is related to 5G-S-TMSI,

-	N is the number of total paging frames in one DRX cycle,

-	Ns is the number of the PO for a PF,

-	Np is the number of subgroupNumForUEID for PEI, if configured and UE supports PEI; otherwise, Np is 1,

-	subgroupsNumForUEID and subgroupsNumPerPO are the subgroup number for UE_ID based subgrouping for LP-WUS and the total subgroup number for LP-WUS, respectively.

On SA2 LS (R2-2500050/S2-2412876)

RAN2 sends reply LS to SA2/RAN3/CT1, and CC RAN1 and RAN4 to confirm the “LP-WUS” terminology.

Regarding the SA2 raised issue on UE Radio Capability for Paging Information, R2 understand that there is no issue for NW after Release 17 (in which case the LP-WUS UE-ID based subgrouping UE capability is included in the UE-RadioPagingInfo container). Whether there is issue for the other cases (for the features mentioned by SA2 LS R2-2500050) can be further discussed in the main session.