3GPP TSG RAN WG2 Meeting #129bis	R2-2502943

Wuhan, China, 7 - 11 April 2025

Agenda Item:	7.2

Source:	Huawei, HiSilicon

Title:	MAC CR rapporteur summary

Document for: Discussion and Decision

# 1	Introduction

This document is a summary of proposals on TS 38.321 for LTM.

# 2	Discussion

The following documents include proposals for, or related to TS 38.321:

| R2-2502169 | Discussion on indication from MAC to RRC layer in RACH-based LTM | vivo |
| --- | --- | --- |

The issue raised is the following text:

### 5.1.6 Completion of the Random Access procedure Upon successful completion of the Random Access procedure initiated for LTM cell switch, the MAC entity shall: 1> indicate the successful completion of the LTM cell switch to upper layers.

In case of RACH- based LTM cell switch, MAC sends:

-	an indication of RACH completion;

-	an indication of LTM cell switch completion.

Upon the RACH completion indication, RRC will:

-	stop T304

-	if there is an indication of LTM cell switch completion

-	release dedicated RACH resources

Upon the LTM cell switch completion indication, RRC will:

-	stop T304 and release dedicated RACH resources

The rapporteur observes that:

-	it is not needed to stop T304 twice or release dedicated RACH resources twice;

-	it also does not harm (and cases of double releasing exist and are considered ok).

Proposal: Discuss whether to remove the unnecessary indication to RRC in TS 38.321, to avoid stopping T304 twice and releasing twice dedicated RACH resources.