3GPP TSG-RAN WG2 Meeting #129bis	R2-2503944

Wuhan, China, 7 - 11 April 2025

Agenda Item:	8.6.2

Source:	Huawei, HiSilicon

Title:	Inter-CU LTM

Document for: Discussion and Decision

# 1	Introduction

LTM (L1/L2 Triggered Mobility) was introduced in Rel-18 for the intra-CU scenario. In Rel-19 [1], the scope is extended to inter-CU to support low latency and low interruption cell switch in broader areas. In this contribution, we discuss some remaining issues on inter-CU LTM.

# 2	Discussion

## 2.1	Inter-CU LTM in standalone

### 2.1.1	Security key update

In subsequent inter-CU LTM, how to update the security key is one issue to be considered when the UE switches among candidate cells. After several meetings discussion in both RAN2 and SA3, SA3 replied with an LS [2] to inform RAN2 that the NCC is transmitted via MAC CE.

SA3 thanks RAN2 for the LS on the security handling for inter-CU LTM in non-DC cases. Considering the RAN2’s request on avoiding RRC configuration between cell switches for subsequent inter-CU LTM in solution selection, SA3 reached a compromise that the NCC can be transmitted to the UE in plaintext via MAC CE message, even though the unprotected MAC leads to increased attack scenarios and probability of handover failure. In addition, SA3 will not consider the protection of MAC CE messages in this release, but may consider studying MAC CE protection in the future. The details on key distribution between the source gNB and the target gNB are left to RAN3 to decide and inform SA3. RAN3 should consider the scenario where the serving gNB utilizes the unused NH (if any is received from a path switch procedure at execution phase) to derive the KNG-RAN* for LTM handover.

In the existing inter-CU handover (e.g., L3 HO) procedure, the new serving gNB obtains a new (NCC, NH) from the AMF during path switch procedure. The procedure of (NCC, key) delivery between the AMF and the gNB can be re-used for inter-CU LTM: upon path switch completion, the new serving gNB gets a new (NCC, key). Then the serving gNB sends the obtained NCC to the UE via MAC CE for the next inter-CU LTM cell switch. After each inter-CU LTM cell switch, the serving gNB obtains a new (NCC, key) and can send  the new NCC to the UE, so there is no need to obtain multiple NCC values from the AMF at once.

Proposal 1: The NCC value sent from the serving gNB to the UE is obtained from the AMF in the path switch procedure. There is no need to enhance the path switch procedure to get multiple NCC values.

There are two options for when to send the MAC CE including the NCC to the UE.

Option 1: the serving gNB includes the NCC value in the LTM cell switch command MAC CE.

Option 2: the serving gNB sends the MAC CE before inter-CU LTM cell switch, in a specific (new) MAC CE.

Both option 1 and option 2 work for normal inter-CU LTM cell switch cases. However, for the LTM fast recovery procedure caused by RLF, it is better to adopt option 2 so that the UE can get an NCC value before RLF and use it to derive a new key when performing LTM recovery to an inter-CU candidate cell, which avoids RRC re-establishment.

In option 2, the UE stores the NCC value. When the UE receives an LTM cell switch command MAC CE and determines that a key change is needed by checking the Rel-19 L2 behaviour ID, the UE uses the NCC value to derive a new key.

Proposal 2: The NCC value is transmitted to the UE via a new MAC CE, which can be sent to the UE earlier than the LTM cell switch command MAC CE, and the UE stores the received NCC. The UE uses the stored NCC when receiving an LTM cell switch command MAC CE for which key change is needed.

Since the inter-CU LTM is a network triggered handover procedure, the network can ensure that this new NCC MAC CE is transmitted to the UE before/when triggering a cell switch that needs key change.

Proposal 3: The network ensures that an NCC is transmitted to the UE before/when triggering a cell switch that needs key change.

The procedure is shown in the figure below:

Figure 1. An example of key change for subsequent inter-CU LTM

### 2.1.2	Fast recovery

In RAN2 #127bis, RAN2 considered as FFS whether to support fast recovery when the selected cell by the UE is an inter-CU candidate cell.

RAN2 #127bis 1. The Rel18 handling on failure is reused in R19 if the UE selects an intra-CU LTM candidate cell after intra-CU LTM failure; for other cases, e.g. inter-CU LTM failure, the failure handling is FFS (related to SA3’s inputs).

According to the discussion above, before receiving an LTM cell switch MAC CE (e.g., from gNB1 to gNB2), the UE should already have an NCC value for the next inter-CU LTM cell switch. If an LTM cell switch fails and the UE selects an inter-CU candidate cell (e.g., in gNB3) for fast recovery, the UE can use this NCC to derive a new key. At the network side, there are two possible methods. One method is that the gNB1 sends new keys to all candidate gNBs when path switch is complete, including gNB2 and gNB3, so when the UE accesses a cell controlled by gNB3, gNB3 has the corresponding key. Another method is that gNB3 retrieves the new key from gNB1 upon receiving the msg3 from the UE, if gNB3 has not received the key from gNB1 already. By any of these methods, the UE can perform fast recovery to an inter-CU LTM candidate cell.

Proposal 4a:	Support recovery using an inter-CU LTM candidate cell.

Proposal 4b: The gNB controlling the by LTM candidate cell used for fast recovery retrieves the key from the source gNB if it has not received a new key.

The procedure is shown in the figure below. If step 4b is not performed, then the gNB3 uses step 7 and step 8 to retrieve a new key. In current RAN3 design, after each inter-gNB cell switch, the target gNB sends LTM configuration update message to other candidate gNBs, so the candidate gNBs know which gNB is this UE’s previous serving gNB. In the example shown in the following figure, gNB3 knows the previous serving gNB for the UE is gNB1, so gNB3 can initiate a key retrieve procedure to gNB1.

Figure 2. An example of LTM recovery procedure to an inter-CU candidate gNB

### 2.1.3	Co-existence with other features

In RAN2 #129 meeting, RAN2 considered as FFS whether the Rel-19 inter-CU LTM can co-exist with MIMO 2TA and NES features.

RAN2 #129 2. Same as Rel-18 intra-CU LTM, Rel-19 inter-CU LTM could coexist with CovE, (e)RedCap, while couldn’t coexist with NRU. FFS on MIMO 2TA and NES.

In Rel-18 LTM, when a candidate cell is an mTRP cell configured with 2TA, there was a discussion on whether the acquired early TA is still valid for cell switch, e.g., if the early TA is for TRP1, while the TCI state for cell switch is associated with TRP2, this case may cause RACH-less cell switch failure. The final decision was: 1) for early RACH based TA acquisition, the candidate DU sends the TAG ID pointer to the source DU along with the TA value, so the source DU knows for which TRP of the candidate cell the acquired early TA is, then the network ensures that the early TA matches with the cell switch TRP when the network intends to trigger RACH-less cell switch, 2) for UE based TA acquisition, the candidate cell cannot be configured with 2TA. This decision can be re-used for Rel-19 LTM.

Proposal 5: When an LTM candidate cell is configured with mTRP 2TA, the network ensures that the TA acquired by early RACH procedure matches with the TCI state (TRP) for RACH-less cell switch. UE-based TA measurement is not supported in this case.

In Rel-18 LTM and NES co-existence discussion, RAN2 discussed whether the UE still monitors PDCCH or sends UL message during RACH-less cell switch procedure when the cell is not in the DTX/DRX active period. The conclusion was to not introduce specific procedures for LTM RACH-less. This conclusion can be re-used for Rel-19 LTM.

RAN2 #126 R2-2405586 Coexistence of Cell DTXDRX and RACH-less LTM Sharp discussion Observation 1: There is NO “active time” definition for Cell DTX/DRX, and cell DTX/DRX “active period” doesn’t include exceptional cases. Proposal 1: Support coexistence of Cell DTX/DRX and RACH-less LTM cell switch. Proposal 2: If there is an ongoing RACH-less LTM cell switch or RACH-less handover, UE monitors PDCCH regardless of Cell DTX. Proposal 3: If there is an ongoing RACH-less LTM cell switch or RACH-less handover, UE can deliver configured uplink grant regardless of Cell DRX. => Noted Agreements Understanding is that an LTM target is not a serving cell and hence it will not apply CELL DTX/DRX until the switch is completed. Nothing is optimized or specified for RACHless

Proposal 6: An LTM target cell will not apply cell DTX/DRX until LTM cell switch is completed. RAN2 will not consider any specification impacts needed for LTM RACH-less cell switch to an NES candidate cell.

## 2.2	Inter-CU LTM in DC

### 2.2.1	MCG LTM

RAN2 agreed that MCG LTM cell switch can setup an SCG or modify the SCG, including PSCell change, as long as the SN is not changed. In Rel-18, upon MCG LTM cell switch, the target DU can be notified, but there is no notification of the SN. In Rel-19, as the SCG configuration can be changed, the SN needs to know which configuration to apply. RAN2 agreed that SCG release and addition will always be used for SCG reconfiguration. One solution could be that the UE is allocated different dedicated preambles for each candidate LTM configuration, but the SN would need to reserve more preambles for each UE, which is not convenient from a network perspective. Therefore, the MN should notify the SN of the target configuration.

Proposal 7: For Rel-19 MCG LTM, at LTM cell switch execution, the (target) MN indicates the target LTM candidate configurations ID to the SN.

In Rel-19 MCG LTM, if the MN key is changed, the SN key also needs to be changed, and it is derived by the target MN from the MN key. For MN key change, the UE derives the new MN key from the NH and from an NCC value, and derives the SN key from the MN key and the sk-counter, and the SN receives the SN key from the MN.

In Rel-15, the SN key is provided in the SgNB addition request, but for LTM, this message is likely to be used only at the preparation of an LTM candidate cell, not at LTM cell switch execution. As a notification from the MN to the SN is needed at LTM cell switch to indicate the candidate configuration ID, the same notification could include the SN key.

Depending on the exact scheme designed by SA3, it may be possible to provide the SN key earlier, so that operation on SN-terminated MCG bearers could start without waiting for the notification from the target MN, but this would require further specification work and the MN could send the notification early enough to the SN.

Proposal 8: For Rel-19 MCG LTM, if there are SN-terminated bearers in the target LTM candidate configuration, the target MN provides the SN key to the SN at LTM cell switch execution.

### 2.2.2	Coexistence of intra-CU and inter-CU SCG LTM

In Rel-18 SCG (intra-CU) LTM, LTM is configured by the SN without MN involvement. In Rel-19 SCG LTM, LTM is configured by the MN and the candidate LTM configuration is an MN RRC message that includes the SCG configuration, possibly with other changes by the MN. Besides, in order to support the co-existence of intra-SN candidate cells and inter-SN candidate cells, RAN2 agreed that intra-SN LTM candidate configuration can be configured by the MN as well, unlike the Rel-18 SCG LTM configuration.

RAN2 #128 8. RAN2 to support intra-CU SCG LTM in MN RRC message (i.e. MN RRCReconfiguration message), in addition to SN RRC message.

When there are intra-SN LTM candidate cells and inter-SN LTM candidate cells, to simplify the LTM preparation and configuration procedure, we can re-use a Rel-18 S-CPAC principle i.e., when there are inter-SN and intra-SN candidate cells, it is the MN that configures all the candidate configurations.

Proposal 9: When there are inter-SN and intra-SN LTM candidate cells for SCG LTM, it is the MN that configures all the LTM candidates (with a MN RRC message).

### 2.2.3	SCG LTM UL grant handling

In inter-CU SCG LTM, when SCG LTM is triggered, the UE applies the LTM candidate configuration, performs RACH-less or RACH-based LTM cell switch to the target PSCell and sends an RRCReconfigurationComplete message to the MN. Then, the MN transfers the SN RRCReconfigurationComplete message to the SN.

In Rel-18, if SRB3 is configured, the UE will transmit the RRCReconfigurationComplete message to the SN directly. However, in Rel-19, even if SRB3 is configured, the UE still transmits the RRCReconfigurationComplete message via SRB1.

Observation 1: In Rel-18 intra-CU LTM, when SRB3 is configured, the UE sends the RRCReconfigurationComplete message to the SN directly. However, in Rel-19 SCG LTM, the UE sends the RRCReconfigurationComplete to the MN, regardless whether SRB3 is configured.

In Rel-19 SCG LTM, if the SCG LTM cell switch is RACH-less, the UE may not have uplink data or signalling to transmit to the SCG. If the network configures UL grant skipping, the SCG may not receive any UE uplink transmission and consider RACH-less cell switch to be failed. To avoid this problem, the network should not configure the UE to skip UL grants in RACH-less SCG LTM procedure, e.g., not configure enhancedSkipUplinkTxDynamic, enhancedSkipUplinkTxConfigured and skipUplinkTxDynamic.

Proposal 10: For inter-CU SCG LTM, the network ensures that the UE does not skip the UL grant for RACH-less cell switch, regardless whether SRB3 is configured.

# 3	Conclusion

This contribution makes the following proposals:

Security key update

Proposal 1: The NCC value sent from the serving gNB to the UE is obtained from the AMF in the path switch procedure. There is no need to enhance the path switch procedure to get multiple NCC values.

Proposal 2: The NCC value is transmitted to the UE via a new MAC CE, which can be sent to the UE earlier than the LTM cell switch command MAC CE, and the UE stores the received NCC. The UE uses the stored NCC when receiving an LTM cell switch command MAC CE for which key change is needed.

Proposal 3: The network ensures that an NCC is transmitted to the UE before/when triggering a cell switch that needs key change.

Fast recovery

Proposal 4a:	Support recovery using an inter-CU LTM candidate cell.

Proposal 4b: The gNB controlling the by LTM candidate cell used for fast recovery retrieves the key from the source gNB if it has not received a new key.

Co-existence with other features

Proposal 5: When an LTM candidate cell is configured with mTRP 2TA, the network ensures that the TA acquired by early RACH procedure matches with the TCI state (TRP) for RACH-less cell switch. UE-based TA measurement is not supported in this case.

Proposal 6: An LTM target cell will not apply cell DTX/DRX until LTM cell switch is completed. RAN2 will not consider any specification impacts needed for LTM RACH-less cell switch to an NES candidate cell.

MCG LTM

Proposal 7: For Rel-19 MCG LTM, at LTM cell switch execution, the (target) MN indicates the target LTM candidate configurations ID to the SN.

Proposal 8: For Rel-19 MCG LTM, if there are SN-terminated bearers in the target LTM candidate configuration, the target MN provides the SN key to the SN at LTM cell switch execution.

Coexistence of intra-CU and inter-CU SCG LTM

Proposal 9: When there are inter-SN and intra-SN LTM candidate cells for SCG LTM, it is the MN that configures all the LTM candidates (with a MN RRC message).

SCG LTM UL grant handling

Proposal 10: For inter-CU SCG LTM, the network ensures that the UE does not skip the UL grant for RACH-less cell switch, regardless whether SRB3 is configured.

# 4	Reference

- RP-250339, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom.

- S3-251124, Reply LS on security handling for inter-CU LTM in non-DC cases, SA3.