3GPP TSG-RAN WG2 #129-bis	R2-2502952

Wuhan, China, 7th – 11st April, 2025

Agenda Item:	8.1.4

Source:	Ericsson

Title:	Remaining issues on UE-side data collection

Document for:	Discussion

# 1	Introduction

The topic of UE-side data collection was discussed at length during the Rel.18 SI, and several different options were discussed, some of them with impact in 3GPP and some not. RAN2 conclusions were eventually captured in the CR to TR 38.843 in [1]. Identified solutions can be to a large extent classified in UP- and CP-based solutions, and most of them have impacts outside RAN.

The topic was also discussed at RAN#106 where it was agreed to send an LS to SA2 and SA5 (Cc RAN2, SA3) asking SA2 and SA5 to start studying respectively the transfer of data over UP for solution 2 and the transfer of data over UP for solution 3 [2].
Further, it was agreed that RAN2 will continue to discuss and study this topic during Rel-19 time frame.

The agenda for RAN2#129-bis considers the following topics:

o	Study RAN aspects related to data transfer over UP

o	Discuss level of NG-RAN involvement in the control and configuration of UE side data collection.

o	Discuss NG-RAN involvement in the data transfer of UE side data collection (if any) (including visibility discussion).

o	Discuss aspects/solutions from RAN perspective that enable the data transfer to CN domain or OAM domain.

o	Discuss on the scalability aspects of CP

This paper focuses on those topics and proposes a way forward. Additionally, this paper addresses the LS on AI/ML UE sided data collection that RAN2 received from SA5 [3], and in particular the SA5 assumptions on the feasibility of option 3.

# 2	Discussion

Data collection is an essential step to allow UE-side model training. The UE needs to perform and collect radio measurements which will then be used to train UE-side models. Different options were captured in [1] related to how the UE-side collected data should be transferred from the UE to the data collection entity and finally to the UE-side training entity.

The first topic we discuss in this paper is the level of NG-RAN involvement. Some of the options defined by RAN2 in [1] are UP-based, and some others are CP based, hence the level of NG-RAN involvement may vary depending on that and also on the specific solution that is addressed, i.e. the impact on the NG-RAN is different depending on whether the CP data are transferred via NAS or RRC.

## 2.1	Control on radio resources to be configured for data collection

When it comes to the NG-RAN involvement in the control and configuration of the data collection (i.e. whether data collection should be enabled or not), we can identify a common aspect to both UP and CP based solution, which is the configuration of the radio resources for the purpose of UE-side data collection. Such configuration may be initiated by a UE request (e.g. via UEAssistanceInformation), and it may contain a set A/B of CSI-RS/SSBs resources in which the UE is allowed to perform its data collection. Further details around resource configuration procedures will be discussed in our companion in paper submitted in agenda item 8.1.2.2.

The NG-RAN involvement related to the configuration (via RRC signalling) of the radio resources in which the UE is allowed to perform the UE side data collection is common to all solutions, irrespective of whether the data transfer occurs via UP or CP.

## 2.2	Control on radio measurements/data to be collected

However, related to the radio measurements that should be collected by the UE, the NG-RAN impact may be different. If the collected data is transferred transparently to the NG-RAN (as it is in UP solutions, and in CP solution based on option 2), then the RAN does not need to provide any configuration related to the specific measurements that the UE should collect. The specific analytics information that should be exposed and made available at the data collection entity do not need to be part of RAN specifications, and the NG-RAN does not need to be updated. On the other hand, if the collected data are not transferred transparently with respect to the NG-RAN (as it is in option 3 with CP), then the data to be collected needs to be specified in RAN2 specifications, and the NG-RAN would then be impacted.

The NG-RAN involvement in the configuration of the radio measurements to be collected depends on whether the collected data is transmitted via NG-RAN transparently or not, i.e. no NG-RAN involvement is expected for UP-based solutions and option 2 with CP, whereas NG-RAN involvement is expected for option 3 with CP.

## 2.3	Control on whether the UE is allowed to transfer the collected data

When it comes to the NG-RAN involvement in the control and configuration of the data transfer, some NG-RAN involvement is always expected both in the UP- and CP-based solutions, however the implications from a specification point of view might be different.
For UP-based solutions, the procedure of transferring the AIML-related collected data can be handled as any other UP traffic injected by the UE in the 3GPP network. Since in all the options discussed (except option 1a), the data collection transfer is under the MNO control, then the MNO can configure the network nodes (gNB/UPF) to handle this specific type of traffic. For example, upon receiving a service request from the UE for data collection transfer, the network (based on the MNO configuration) can setup a specific PDU session and related transport channels and handle this traffic with specific QoS flow (and related QoS parameters). Accordingly, the MNO can, through the NG-RAN, have full control of the data collection transfer procedure, thereby controlling the related QoS, and more in general also whether to admit/reject/limit the data collection transfer, for the sake of minimizing the network overload. 
All the above has clearly NG-RAN impact, however no specific NG-RAN specification impact is expected. All the other aspects not impacting RAN will be studied by SA2/SA5 on the basis of the RAN LS [2] which tasks SA2/SA5 to study the UP-based solutions.

For UP-based solutions, the NG-RAN involvement in the data transfer is expected in the setting of PDU sessions and transport channels, as for any other UP service injected into the 3GPP network from the UE. No NG-RAN specification impact is expected.

For CP-based solutions, the NG-RAN impact depends on whether NAS signalling or RRC is used. 
Related to NAS, we observe that the NAS protocol might not be dimensioned to carry large amount of data, which could be the typical case for the transfer of data collected for AI/ML purposes. The usage of NAS for this purpose would be a completely new use case. First, a new NAS procedure may need to be specified (outside RAN2) in order to fetch the data from the UE, and second, since the amount of traffic involved may be large and impact the NG-RAN operations, the NG-RAN should also be involved in the setting of this fetching procedure. Further, still considering the amount of data to be transferred, segmentation of the ULInformationTransfer (considering today´s limitations of maximum 5 segments) should be taken into account for the transferring of NAS data. Additionally, mechanisms for failed delivery of UL NAS PDUs shall be looked into by RAN2 and SA2, as there is today no indication of an UL NAS PDU originated by a UE that failed to be delivered, and this could be a frequent case, when data delivered over NAS considerably increases in size.

Related to the transfer via RRC, a new procedure similar to the logged MDT need to be supported by the NG-RAN and also in the specification. For example, RAN2 may need to discuss procedures for the fetching from the UE of UE-side data. That can be based on UL RRC segmentation or request/response type of solutions (similar to what specified for the NW-side data collection). Hence RAN2 would need to discuss RRC procedures for the signalling of available UE-side data, and potentially further information from the UE, such as the UE memory/power status. 
However, such approach would imply rather major changes on the functionalities and use of the OAM system. Indeed, the OAM system is today not in charge of configuring the radio resources used for e.g. UE measurement collection. With this CP based approach, the OAM will have to support such functionality and provide radio resource configuration that are for specific UEs. The latter is a considerable functionality change with respect to legacy OAM. Since the consumer of UE-side data is not expected to be the NG-RAN, nor the OAM, this extra complexity appears to be unnecessary.

For CP-based solutions (both NAS-/RRC-based), the NG-RAN involvement in the data transfer is expected both in the implementation and in RAN2 specifications, in order to design procedures for fetching the data from the UE (e.g. based on RRC segmentation of request/response) and transmit them to the CN/OAM.

## 2.4	Scalability discussion

Related to CP solutions, especially for solutions based on RRC (e.g. as in option 3), there is an evident scalability problem in our view. Given the heavier NG-RAN involvement compared with UP-based solutions, an extensive update of the NG-RAN nodes might be needed in order support the data collection transfer. Due to the UE memory limitations, if the fetching of UE-side data is supported by only a few NG-RAN nodes in the network, then the ability of the UE to perform an efficient UE-side data collection will be quite limited. Further, from the point of view of system performances, it is desirable that the radio resource consumption due to the fetching of data is fairly shared across the network, but this would not be possible without a capillary update of the NG-RAN nodes to support this new feature. To further exacerbate the problem, many different RAN AIML functionalities/use cases are expected to be introduced by 3GPP in coming releases. Hence, whenever new measurements will need to be configured for data collection, and for fetching, the NG-RAN nodes would need to be updated to be compliant with the new releases. In summary, the UE's ability to train and execute AI/ML functions will rely on NG-RAN's support of UE data collection. This dependency creates artificial limitations that could restrict the use of AI/ML techniques to improve functionalities on the UE side. Again, as previously said, since the consumer of UE-side data is not expected to be the NG-RAN, this extra complexity appears to be unnecessary.

CP based solutions require a heavier update of the NG-RAN both for the configuration of the data to be collected and for the fetching (via CP) of the collected data. Without a capillary update of the NG-RAN nodes, the ability of the UE to perform an efficient UE-side data collection will be limited, as well as the possibility for the network to balance the radio overhead. NG-RAN nodes compliance with beyond Rel.19 AIML use cases need also to be considered.

On the other hand, as already outlined above, if a UP based solution is adopted, the NG-RAN complexity to handle the data collection configuration and the collected data transfer is much smaller. The data to be collected/exposed by the UE do not need to be specified (and hence handled) in RAN, whereas the collected data transfer can be handled by NG-RAN node as other UP services, using existing standardised methods to setup and manage UP data channels, e.g. by using priority mechanisms, admission control and QoS framework to manage the PDU Session and DRBs in charge of conveying the data collected by the UE. Such data can still be received at a logical node within the MNO network and therefore data can be made available to the MNO.

Since the data to be collected/exposed by the UE do not need to be specified/implemented in NG-RAN, and the transfer of collected data transfer can be handled by NG-RAN node as other UP services, no scalability problem are expected for UP-based solutions.

## 2.5	LS reply to SA5 on the feasibility of option 3

Recently, RAN2 (and RAN) received an LS reply from SA5 [3], related to the feasibility of option 3 for the UE-side data collection. From the LS, it turns out that SA5 has evaluated the feasibility the option 3 when the UP is used as the data transfer protocol. In particular:

From SA5 LS [3]: SA5 has made the following analysis about the termination points of the UP tunnel for this solution: 1. If the UP tunnel is between UE and Server, SA5 does not see any difference between solution 3 and solution 1a); 2. If the UP tunnel is between UE and 5GC, SA5 does not see any difference between solution 3 and solution 2. SA5 assumes that the UP tunnel associated with data transfer is between UE and gNB for the solution 3. SA5 would like to get confirmation of this assumption from and RAN2, as well as RAN’s perspective on the UP tunnel termination points.

From points 1. and 2., it is clear that SA5 has not considered the possibility that the UP is terminated in the OAM. From their point of view, the possibility for option 3 to rely on the UP is based on data collection from the UE via over the air UP channels (DRBs) and on the transfer of such data from the RAN to the management system (via management interfaces). 
On the other hand, they are asking RAN2 to confirm the assumption (highlighted above) that the UP is terminated in the gNB.

From RAN2 point of view the highlighted assumption is incorrect, as according to the current architecture, it is not possible to establish the UP channels only between the gNB and the UE, rather the UP traffic is always tunnelled via the gNB to the UPF, i.e. the gNB does not terminate the UP. Hence, it is not possible for the gNB to route the UP traffic received from the UE to the OAM.
This possibility was also discussed recently in the context of the two-sided dataset/model parameters transfer, and it turned out that RAN2 should avoid considering this option, given the high specification complexity.
We suggest the following reply to SA5.

According to the current architecture, the UP traffic is not terminated at the gNB. Hence, the gNB cannot transfer the UP data received from the UE to the OAM. RAN2 does not consider introducing this option.

On the other side, and keeping in mind that UP can only be established between the UE to the CN end to end, the statements made by SA5 in their point 1. and 2. are correct. It is therefore proposed to confirm to SA5 that their statements in point 1. and point 2. are correct.

Confirm in a reply LS to SA5 that the statements in point 1. and point 2. In the SA5 LS in S5-250828 are correct.

Send the reply LS in the Annex to SA5.

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	The NG-RAN involvement related to the configuration (via RRC signalling) of the radio resources in which the UE is allowed to perform the UE side data collection is common to all solutions, irrespective of whether the data transfer occurs via UP or CP.

Proposal 2	The NG-RAN involvement in the configuration of the radio measurements to be collected depends on whether the collected data is transmitted via NG-RAN transparently or not, i.e. no NG-RAN involvement is expected for UP-based solutions and option 2 with CP, whereas NG-RAN involvement is expected for option 3 with CP.

Proposal 3	For UP-based solutions, the NG-RAN involvement in the data transfer is expected in the setting of PDU sessions and transport channels, as for any other UP service injected into the 3GPP network from the UE. No NG-RAN specification impact is expected.

Proposal 4	For CP-based solutions (both NAS-/RRC-based), the NG-RAN involvement in the data transfer is expected both in the implementation and in RAN2 specifications, in order to design procedures for fetching the data from the UE (e.g. based on RRC segmentation of request/response) and transmit them to the CN/OAM.

Proposal 5	CP based solutions require a heavier update of the NG-RAN both for the configuration of the data to be collected and for the fetching (via CP) of the collected data. Without a capillary update of the NG-RAN nodes, the ability of the UE to perform an efficient UE-side data collection will be limited, as well as the possibility for the network to balance the radio overhead. NG-RAN nodes compliance with beyond Rel.19 AIML use cases need also to be considered.

Proposal 6	Since the data to be collected/exposed by the UE do not need to be specified/implemented in NG-RAN, and the transfer of collected data transfer can be handled by NG-RAN node as other UP services, no scalability problem are expected for UP-based solutions.

Proposal 7	According to the current architecture, the UP traffic is not terminated at the gNB. Hence, the gNB cannot transfer the UP data received from the UE to the OAM. RAN2 does not consider introducing this option.

Proposal 8	Confirm in a reply LS to SA5 that the statements in point 1. and point 2. In the SA5 LS in S5-250828 are correct.

Proposal 9	Send the reply LS in the Annex to SA5.

# References

R2-2407807, CR on RAN2 inputs to TR 38.843,” RAN2#127

RP-243316, LS on AI/ML UE sided data collection, RAN#106

S5-250828, Reply LS on AI/ML UE sided data collection, 3GPP SA WG5#159

# 4 Annex

3GPP TSG-RAN WG2 #129-bis	R2-2502952

Wuhan, China, 7th – 11st April, 2025

Title:	Reply LS on AI/ML UE sided data collection

Response to:	LS in S5-250828 on AI/ML UE sided data collection

Release:	Rel-19

Work Item:	FS_NR_AIML_air_Ph2

Source:	3GPP RAN WG2 #129-bis

To:	TSG SA5

Cc:	TSG RAN, SA, SA2, SA3

Contact person:

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

RAN2 would like to thank TSG SA5 for their LS in S5-250828 on AI/ML UE sided data collection.

Related to the SA5 assumption that “the UP tunnel associated with data transfer is between UE and gNB for the solution 3”, RAN2 understanding is that the UP traffic is not terminated at the gNB, given the current architecture. Hence, it is not possible for the gNB to transfer the UP traffic received from the UE to the OAM. RAN2 does not intend to introduce this functionality.

Concerning the statements made by SA5 in point 1. and point 2. of their LS, RAN2 confirms that these statements are correct.

# 2	Actions

To: TSG RAN, RAN2

ACTION: 	RAN2 kindly asks TSG SA5 to take into account the above reply.

# 3	Dates of next TSG RAN WG2 meetings

RAN2#130		19 May - 23 May 2025			Malta, MT