3GPP TSG RAN WG2#129bis   	R2-2502954

Wuhan, China, Apr. 7th – 11th, 2025

Agenda Item:	8.2.2

Source:	III

Title:	Discussion on paging procedure for Ambient-IoT

Document for:	Discussion

Introduction

In the previous RAN2#129 meeting, some agreements were achieved in the following:

Agreements <Simultaneous Paging Procedures> - Parallel service requests by the same reader is not supported. - The device is expected to only perform one procedure at a time. FFS device behaviour if multiple requests are received in parallel (if needed). < transaction ID > - The “transaction ID” can be generated by reader based on CN correlation ID. FFS how reader will generate “transaction ID”. FFS the size of transaction ID - 1 bit solution is excluded. FFS the size. Aim to have a reasonable size. - RAN2 acknowledges that multi-reader scenario may exist but we will not specify something specific for this purpose. We can rely on transaction ID and implementation to handle it. <Paging ID> - The “one identifier” in the paging message includes both the case of “one single device identifier” and “one group identifier”/”filtering criteria”, while the exact format of latter is supposed to be designed by SA2. - The current assumption is that the paging identifier is transparent to the A-IoT MAC Layer and carried by upper layer. FFS if there is really a need for visibility in the MAC layer < Resource Information > - The A-IoT paging message can include a number of msg1 resources < Scheduling of MSG1 Transmissions> - From RAN2 perspective, after initial paging message, the R2D transmission which determines the Msg1 resource(s), can be achieved by one of the below two ways, unless RAN1 concludes to use L1 signaling later: Way-1: introducing new R2D message other than the paging message, e.g., QueryRep-like; or Way-2: reusing the same paging message, using field(s) to indicate it is only to determine the Msg1 resource(s) and omitting the paging identifier (device ID/group ID) field < Service Type Indication> - The service type of A-IoT (e.g., inventory only, inventory + command) is not included in paging message.

Based on RAN2 progress, this contribution discusses different service request from different readers, transaction ID and device ID.

Discussion

Different service request is received from different reader

Different service request received from different readers was discussed before. In RAN2#129, some agreements were reached:

In RAN2#129 agreement: <Simultaneous Paging Procedures> - Parallel service requests by the same reader is not supported. - The device is expected to only perform one procedure at a time. FFS device behaviour if multiple requests are received in parallel (if needed).

In device perspective, the device may receive a new service request arriving from different readers while an ongoing procedure is still in progress. The following options can be considered as baseline:

Option 1: The device rejects new request(s), the device focuses on the current request in progress and

ignores any other requests from the different reader until the current request completes or fails.

Option 2: The device executes the new service request, the device skips old request process and always

executes the last received request.

Option 3: The device will process higher priority request, the device determines the execution order based on

the priority of the request, ensuring that high-priority requests are processed.

Proposal 1: A new service request is received from different reader(s) while an ongoing procedure

is still in progress. The following options can be considered as baseline:

Option 1: The device rejects new request(s).

Option 2: The device executes the new service request.

Option 3: The device will process higher priority request.

Transaction ID

In TR 38.769[1] describe, the reader transmits multiple (subsequent) A-IoT paging messages that are associated with the same service request from the CN. The duplicated response should be avoided for the same service request. In RAN2#128 discussion, session ID/transaction ID between CN and reader (i.e., based on the candidate solutions of SA2) can be used the ID to avoid duplicated response for same service request.

In previous RAN2#128 meetings, as following:

Agreements on paging - An ID in paging message is beneficial to avoid duplicated response for same service request. FFS whether ID is generated by the reader or CN. FFS on the ID size

In RAN2#129 agreement, the transaction ID can be generated by reader based on CN

Agreements - The “transaction ID” can be generated by reader based on CN correlation ID. FFS how reader will generate “transaction ID”. FFS the size of transaction ID - 1 bit solution is excluded. FFS the size. Aim to have a reasonable size. RAN2 acknowledges that multi-reader scenario may exist but we will not specify something specific for this purpose. We can rely on transaction ID and implementation to handle it.

When the CN correlation ID was clarified by SA2, the reader can generate a transaction ID based on the CN correlation ID.
We provides three options for further discussion.

Option 1: Use X bits (e.g. LSB) of the CN Correlation ID to generate the transaction ID (but make sure X is large enough to avoid collisions),

Option 2: A mapping mechanism based on CN correlation IDs,

Option 3: The reader selects a transaction ID that has not been used recently to avoid ID duplication.

Observation 1: When the CN correlation ID was clarified by SA2, the reader can generate a transaction ID based on the CN correlation ID. We provides three options for further discussion.

Option 1: Use X bits (e.g. LSB),
Option 2: A mapping mechanism,

Option 3: The reader selects a transaction ID that has not been used recently to avoid ID duplication.

Ambient IoT Device ID

In the RAN2#129, agreement about paging ID:

Agreements on paging ID - The “one identifier” in the paging message includes both the case of “one single device identifier” and “one group identifier”/”filtering criteria”, while the exact format of latter is supposed to be designed by SA2.

The Ambient IoT Device ID was defined by TR 23.700-13 [2], it includes multiple identifiers.

Based on the above consideration, following components are considered necessary to compose the Ambient IoT Device ID, which is defined by 3GPP: - Home Network Identifier: an identifier used to identify the home MNO; - Owner Identifier: an identifier used to identify a 3rd party who sends service requests to trigger 5GC to perform Ambient IoT service operation; NOTE 1: The Owner Identifier is allocated by the home MNO corresponding to the Home Network Identifier. Editor’s Note: The use of the Owner Identifier is FFS and may depend on the solution for Inventory, etc. - Instance Identifier: an identifier used to identify a specific Ambient IoT device owned by the 3rd party. NOTE 2: The Instance Identifier is allocated by the home MNO which may coordinate with the 3rd party. Figure 6.2.1-2: Structure of Ambient IoT Device ID and optional 3rd Party-defined Identifier

In TS 23.003[3], the SUPI is a globally unique 5G subscription permanent identifier allocated to each subscriber in the 5G System. It is feasible to choose IMSI as SUPI type for the Ambient-IoT Device ID.

- 2.2A Subscription Permanent Identifier (SUPI) The SUPI is a globally unique 5G Subscription Permanent Identifier allocated to each subscriber in the 5G System. It is defined in clause 5.9.2 of 3GPP TS 23.501 [119]. The SUPI is defined as: - a SUPI type: in this release of the specification, it may indicate an IMSI, a Network Specific Identifier (NSI), a Global Line Identifier (GLI) or a Global Cable Identifier (GCI); and - dependent on the value of the SUPI type: - an IMSI as defined in clause 2.1; - a Network Specific Identifier (NSI), taking the form of a Network Access Identifier (NAI) as defined in clause 28.7.2; - a Global Cable Identifier (GCI) taking the form of a NAI as defined in clause 28.15.2; - a Global Line Identifier (GLI) taking the form of an NAI as defined in clause 28.16.2. NOTE: Depending on the protocol used to convey the SUPI, the SUPI type can take different formats.

Base on above, the single device ID in paging message can be based on Ambient IoT Device ID /IMSI for uniqueness of device ID in single/multiple readers. This should be further discussed in RAN2.

Observation 2: The single device ID in paging message can be based on Ambient IoT Device ID /IMSI for uniqueness in single/multiple readers.

- Conclusion

We have the following proposal and observation:

Proposal 1: A new service request is received from different reader(s) while an ongoing procedure

is still in progress. The following options can be considered as baseline:

Option 1: The device rejects new request(s).

Option 2: The device executes the new service request.

Option 3: The device will process higher priority request.

Observation 1: When the CN correlation ID was clarified by SA2, the reader can generate a transaction ID based on the CN correlation ID. We provides three options for further discussion.

Option 1: Use X bits (e.g. LSB),
Option 2: A mapping mechanism,

Option 3: The reader selects a transaction ID that has not been used recently to avoid ID duplication.

Observation 2: The single device ID in paging message can be based on IMSI / Instance Identifier for uniqueness in single/multiple readers.

- Reference:

[1] TR 38.769, “Study on solutions for ambient IoT (Internet of Things)”.

[2] TR 23.700-13,” Study on Architecture support of Ambient power-enabled Internet of Things”.

[3] TS 23.003,” Numbering, addressing and identification;”.