3GPP TSG-RAN WG2 Meeting #129bis	R2-2502957

Wuhan, China, Apr. 7th – 11th, 2025

Agenda item:	8.7.6

Source:	China Telecom

Title:	Discussion on Rate Control for XR

Document for:	Discussion and Decision

1	Introduction

In the last RAN2#129 meeting, rate control for XR was widely discussed and the made the following agreements.

Agreements on XR rate control - RAN2 assumes for XR rate control, the gNB receives QoS flow information from the CN, specifying which QoS flows are subject to uplink rate control (i.e., Option 2). Send an LS to RAN3 and SA2. We may revisit UAI option based on SA2/RAN3 reply - Specify a new table for XR rate control. FFS distribution (exponential, linear), codepoints etc. - We will try to design a table first and check whether it is possible to meet the required range/granularity. Afterwards, we can check whether multipliers are needed - Working assumption: - Support rate query MAC CE with the target to use same design that we will agree for rate indication MAC CE. - The rate query MAC CE is configurable by the network, i.e. the network may turn it off completely (same as legacy). - Companies to check with their SA4 colleagues whether there are any issues with this

In this contribution, we focus on open issue on XR rate control and provide our views.

2	Discussion

In the previous meeting, RAN2 agreed to support rate indication at the per-QoS flow level. Regarding rate control, there are at least two options under consideration:

- Option 1: Flows are indicated by MAC CE.

- Option 2: Flows are indicated by RRC, while MAC CE is per DRB.

We advocate for Option 1, as it offers a straightforward approach to rate control. In contrast, Option 2 allows the network to configure which flows require rate control within the MAC CE. However, XR services encompass multiple distinct traffic flows, and congestion conditions for different flows may change dynamically. Consequently, the network would need to send an RRC Reconfiguration message each time flow configurations for MAC CE require modification.

Proposal 1: to agree flows are indicated by MAC CE.

Furthermore, whether a single or multiple flow indications should be included in one MAC CE remains an open discussion. The Release 15 recommended bit rate MAC CE only supports a single DRB and has a fixed size. The existing mechanism is designed solely for codec adaptation for MMTEL voice or MMTEL video. These traffic flows may be mapped to one or multiple DRBs, depending on the local RRM strategy. According to the agreement from the last meeting, the RAN side can estimate congestion information at both per-DRB and per-QoS flow levels. Moreover, multiple traffic flows may experience congestion simultaneously. If a single-flow indication or DRB/LCH in one MAC CE is adopted, the network would need to send multiple MAC CEs to the UE simultaneously, which is inefficient for XR services. Additionally, XR traffic flows exhibit significant differences in data rate requirements, meaning not all flows require rate control. Based on this analysis, one rate control MAC CE can encompass multiple flows. The network can also configure the maximum number of QoS flows per MAC CE via RRC.

Proposal 2: One rate control MAC CE can comprise multiple flows, and the network can configure the maximum number of QoS flows per MAC CE via RRC.

As illustrated in Figure 1, the current Recommended Bit Rate MAC CE has a fixed size and consists of two octets. The Bit Rate field is 6 bits, comprising 56 utilized code points to indicate values ranging from 0 to 8000 kbps, while the X Bit Rate Multiplier is 1 bit. Additionally, to support a recommended bit rate with per-QoS flow granularity, a new MAC CE should be specified in Release 19.

Figure 1 Recommended bit rate MAC CE

Proposal 3: New MAC CE should be specified in Rel-19 to support Per-QoS flow granularity recommended bit rate.

In the last meeting, SA4 has replied to RAN2 the range of bit rates [2]. SA4 indicated that the range of bit rates varies from 100kbps to 40Mbps. It can observed that the maximum value exceeds the minimum value by 400 times. SA4 has indicates that the desired granularity is as small as 1%, and the coarser granularity is 10%. Per TS38.321, the legacy recommend bit rates are encoded with a piecewise linear distribution. To achieve the required granularity of 10%, linear and piecewise linear distribution encoding would require larger number of code points.

In Rel-18 XR study, new BSR table is introduced to reduce the capacity impact caused by buffer size error. To make the balance between the signalling overhead and quantization error, exponential encoding is adopted in the new BSR design. Similarly, exponential encoding can also be used for XR rate control.

Proposal 4: Exponential distribution is adopted for bit rates encoding.

Per TS38.413 and TS23.501, the maximum number of QoS flows is 64. From the perspective of MAC CE, there is no need to retain the existing size of the QoS Flow ID within the MAC CE. To reduce overhead in the MAC CE, the QoS Flow ID can be mapped to a shorter ID configured by the RRC.

TS 38.413 9.3.1.51 QoS Flow Identifier This IE identifies a QoS flow within a PDU Session, or a MBS QoS flow within a MBS session. The definition and use of the QoS Flow Identifier is specified in TS 23.501 [9]. IE/Group Name; Presence; Range; IE type and reference; Semantics description. QoS Flow Identifier; M; INTEGER (0..63, …).

Proposal 5: To reduce overhead in the MAC CE, the QoS Flow ID can be mapped to a shorter ID configured by the RRC.

3 	Conclusion

Based on the above analysis, we have the following proposals:

Proposal 1: to agree that flows are indicated by MAC CE.

Proposal 2: One rate control MAC CE can comprise multiple flows, and the network can configure the maximum number of QoS flows per MAC CE via RRC.

Proposal 3: New MAC CE should be specified in Rel-19 to support Per-QoS flow granularity recommended bit rate.

Proposal 4: Exponential distribution is adopted for bit rates encoding.

Proposal 5: To reduce overhead in the MAC CE, the QoS Flow ID can be mapped to a shorter ID configured by the RRC.

4	References

[1]	TR 38.838, Study on XR (Extended Reality) evaluations for NR, V17.0.0, 2022-04

[2]   R2-2500056, Reply to LS on appropriate range and granularity of bit rate adaptation for XR applications