3GPP TSG-RAN WG2 Meeting #129bis	R2-2502965

Wuhan, China, 7th - 11th April 2025

Source:                    	DENSO CORPORATION

Title:  	Discussion on inter-CU LTM

Document for:        	Discussion and decision

Agenda Item:         	8.6.2	Inter-CU LTM

- Introduction

In RAN2-128 meeting [1], RAN2-127bis[2] and RAN2-127 meeting[3], RAN2 discussed the detail of inter-CU LTM procedure, and the following was agreed.

Agreements on inter-CU LTM in RAN2-128

Reply to SA3 that RAN2 think avoiding RRC configuration between cell switches for subsequent inter-CU LTM is important and it needs to be respected in solution selection.

Include the time plan information to indicate until when their feedback is required or until when RAN2 needs to down-select the options. We assume in RAN2#130, to complete WI.

Agreements on inter-CU LTM in RAN2-127bis

1.	The Rel18 handling on failure is reused in R19 if the UE selects an intra-CU LTM candidate cell after intra-CU LTM failure; for other cases, e.g. inter-CU LTM failure, the failure handling is FFS (related to SA3’s inputs).

2.	For non-DC case, if the new Rel-19 IDs of the serving cell and the target cell have same values, the UE compares the ltm-ServingCellNoResetID and ltm-NoResetID and performs the corresponding L2 reset operation as defined in Rel-18.

3.	The SCPAC-similar security update configuration is introduced for inter-CU SCG LTM, i.e. similar to IEs sk-CounterConfiguration, servingSecurityCellSetId and securityCellSetId. The names of the new IEs are to be discussed in stage3.

Agreements on inter-CU LTM in RAN2-127

RLC and PDCP re-establishment in inter-CU LTM:

- If the security key update is required, the UE shall perform MAC reset, RLC re-establishment and PDCP re-establishment. As baseline introduce a new Rel-19 ID in RRC: if the Rel-19 ID is different for the source cell and the target cell, the UE performs PDCP re-establishment, including security key update, however dependent on SA3 response, we can revisit it.

In this contribution, we provide our understanding for inter-CU LTM procedure, such as the procedure related fore security update.

- Discussion

In RAN2-128[1] meeting, RAN2 concluded that avoiding RRC configuration between cell switches for subsequent inter-CU LTM is important and it needs to be respected in solution selection. And in last SA3 meeting, SA3 reached a compromise that the NCC can be transmitted to the UE in plaintext via a MAC CE message. Therefore, RAN2 should discuss a procedure for the security update in the case that the NCC is included in a cell switch command MAC CE.

In the legacy HO procedure, the NCC value is mandatorily included in the HO command for inter-CU mobility. Upon the security key update procedure, the UE compares the received NCC value with the stored NCC value. If the received NCC value is different from the stored NCC value, the UE derives the new security key by a vertical key derivation. If the received NCC value is equal to the stored NCC value, the UE derives the new security key by a horizontal key derivation. Therefore, the NCC value is always needed upon the security key update.

In Rel-19 LTM, the UE checks the Rel-19 ID to determine whether the security key update procedure is necessary. If the Rel-19 ID is different for the source cell and the target cell, the UE performs the security key update. Upon the security key update procedure in Rel-19 LTM, the UE can also perform the security key update by using the NCC value included in the CSC MAC CE. In this case, the MAC layer of the UE needs to indicate the received NCC value to the RRC layer of the UE since the security key update procedure is performed on the RRC layer. However, it is not certain that the NCC value is always included in the CSC MAC CE. If the NCC value is not indicated to the RRC layer, the UE may not be able to update the security key even if the Rel-19 ID is different for the source cell and the target cell. To avoid this situation, we propose the UE performs the security key update only if the Rel-19 ID is different for the source cell and the target cell and the NCC value is indicated by the MAC layer.

Proposal 1: The UE performs the security key update only if the new Rel-19 ID is different for the source cell and the target cell and the NCC value is indicated by the MAC layer.

Furthermore, RAN2 can also discuss the LTM fast recovery procedure with the security key update. Currently, RAN2 agreed: “The Rel18 handling on failure is reused in R19 if the UE selects an intra-CU LTM candidate cell after intra-CU LTM failure; for other cases, e.g. inter-CU LTM failure, the failure handling is FFS (related to SA3’s inputs).”. RAN2 is considering the case of the fast recovery in Rel-19 LTM, as below.

- Intra-CU cell selection after intra-CU LTM failure

- Intra-CU cell selection after inter-CU LTM failure

- Inter-CU cell selection after intra-CU LTM failure

- Inter-CU cell selection after inter-CU LTM failure

In these scenarios, it is not clear what the definition of the term of “intra/inter-CU” is. Therefore, RAN2 should clarify the definition of “intra/inter-CU” in these scenarios. In the security update procedure for inter-CU LTM, the UE determines whether the target cell is inter-CU or intra-CU by comparing the Rel-19 ID for the target cell and the source cell. In the cell switch when the LTM failure occurred, the UE checks the Rel-19 ID for the target cell and the source cell. Upon the LTM failure, the UE reverts the Rel-19 ID back to the one for the initial source cell, which was connected to the UE before the LTM cell switch failure. Therefore, in the cell switch after the LTM failure, the UE checks the Rel-19 ID for the new target cell and the initial source cell. From these, the UE determines whether the target cell is inter-CU or intra-CU based on the initial source cell either before or after the LTM failure. Therefore, we propose RAN2 to clarify the initial source cell, which connected to the UE before the LTM cell switch failure, as the basis of “intra/inter-CU”.

Proposal 2: RAN2 to clarify the initial source cell, which connected to the UE before the LTM cell switch failure, as the basis of “intra/inter-CU”.

In these scenarios, we prefer to support only the intra-cell (of the initial source cell) selection upon the LTM fast recovery, i.e., scenario 1 and 2. In scenario 1, the UE performs the intra-CU LTM cell switch upon the fast recovery. Therefore, this scenario can reuse the Rel-18 LTM procedure. In scenario 2, the new target cell belongs to the same CU as the initial source cell. Therefore, these scenarios can also reuse Rel-18 LTM procedure upon the fast recovery.

However, in scenario 3, the UE cannot receive the NCC value if the NCC value is not included in the CSC MAC CE triggering the intra-CU LTM. As a result, the UE cannot derive the new security key for the inter-CU target cell. In scenario 4, under the current specification, the initial source cell only indicates the pair of the NCC value and the new security key to the initial target cell (which is the target cell of the UE before the LTM cell switch failure). If the same principle is applied to the LTM, the new target cell selected upon the cell selection cannot apply the same security key of the UE since the initial source cell does not indicate the new security key to the new target cell. For these reasons, we propose the UE only selects the intra-cell of the initial source cell upon the LTM fast recovery.

Proposal 3: The UE only selects the intra-cell of the initial source cell upon the LTM fast recovery.

- Summary and proposal

This paper discussed the procedure for mixture of inter-CU LTM and intra-CU LTM. In summary, the followings were proposed:

Proposal 1: The UE performs the security key update only if the new Rel-19 ID is different for the source cell and the target cell and the NCC value is indicated by the MAC layer.

Proposal 2: RAN2 to clarify the initial source cell, which connected to the UE before the LTM cell switch failure, as the basis of “intra/inter-CU”.

Proposal 3: The UE only selects the intra-cell of the initial source cell upon the LTM fast recovery.

- References

[1] R2-2500002, “RAN2#128 Meeting Report,” ETSI MCC.

[2] R2-2409502, “RAN2#127bis Meeting Report,” ETSI MCC.

[3] R2-2407902, “RAN2#127 Meeting Report,” ETSI MCC.