3GPP TSG-RAN WG2 #129bis	R2-2503171

# Wuhan, China, 7th – 11th April, 2025

Agenda Item:	2.5

Source:	Ericsson

Title:	RRC CR Guidelines

Document for:	Presentation

# 1	Introduction

This document is used at online presentation at RAN2#129bis.

# 2	RRC CR Guidelines

### More information

- Guidelines in TS 38.331 App A

- RAN2 Handbook

### Guidelines to get the CR “clean” and formally correct

- Use of full 38331 in WI CR? Consider this and select the proper size:

  - Typically, full ASN.1 is needed for the CR Rapp to ensure ASN.1 passes syntax check.

  - Easier to ensure changes/additions are consistent and complete.

  - Major drawback is file size and Word performance.

- Use correct MS Word Styles

  - Use a correct CR template (based on spec and latest CR Cover page).

  - Be careful when copy/paste text from e.g. emails while using tracked changes (font style, underline and text colour).

- Avoid changes on changes (at least in final CR)

- Avoid multiple tracking users (at least in final CR).

- Accept formatting comments added by MS Word (see to the right).

- If CR impacts an Introduction/Initiation section, good for reader to also include “parent” subclause heading, e.g. as

5.3.13          RRC connection resume

<Omitting unchanged sections>

5.3.13.2        Initiation

- See RAN2 Handbook for guidance/tools on CR cover page, MS Word style checking etc.

### Guidelines on procedure text

- Limit the “footprint” of new features in RRC “main” sections.

  - Add new subsections in logical order of existing subsections

  - Limit the amount of text, avoid to repeat descriptions available elsewhere

- Identify clashing WIs as early as possible (recall e.g. FeatureCombination, RACH-less HO)

- Sometimes good to capture Agreements in e.g. WI CR cover page, for reference during develpment.

### Guidelines on fields and field descriptions

- Use ASN.1 (SEQUENCE, CHOICE, ...) to express logic, to avoid to express restrictions in words.

- Do not add a field description that does not provide any information.

- Use one field description table per IE.

- Included vs configured

  - We use included to denote that a field is present in this message.

  - We use configured to denote that a field is present in this message or in a previous message.

- Avoid duplicate same requirement/description in multiple places of the

  - RRC spec (procedure text/field description, IE descriptions), and in

  - Other specs (RAN2 specs, RAN1/RAN4 specs).

    - Add reference to other specs, e.g RAN1/RAN4 specs and RRC subsections

- Nw is expected to respect UE capabilities, we need typically not add “if supported by the UE” for optional features with UE capability.

- Restriction on that both Feature A and Feature B cannot be configured with UE at the same time can be captured as restrictions in the feature capabilities in 36.306, rather than as network configuration restrictions in 38.331.

### Guidelines on IE and field naming

- IE names (ASN.1 types) are “global” to the ASN.1 module (“RRC”, “PC5-RRC”).

- Field names are local to the IE (ASN.1 type) where it is defined.

- IE that is used in multiple “parent” IEs (in multiple IE sections) should get an own IE section.

- Avoid encode the “full ASN.1 structure” in field names – avoid too long names.

- Can be sometimes good to give same prefix in names of related IEs, to get the IEs together in alphabetical order. Ex “MeasObjectId”, “PCI-RangeElement”, “NCR-PeriodicityAndOffset”.

- Do not use “EXT” etc in names of extended fields/IEs (exception for Lists, see RRC Guidelines.)

- Avoid the term “legacy” in names and specification texts.

- Hyphens and capitals:

Ex 1) SPS-PUCCH-AN-List-r16

SPSPUCCHANlist-r16 -> SPS-PUCCH-AN-list-r16 -> SPS-PUCCH-AN-List-r16 -> sps-PUCCH-AN-List-r16

-- TAG-SPSPUCCHANLIST-START

Ex2) NZP-CSI-RS-ResourceSetId

NZPCSIRSResourceSetId -> NZP-CSI-RS-ResourceSetId -> 	nzp-CSI-RS-ResourceSetId

-- TAG-NZPCSIRSRESOURCESETID-START

  - Can use R2-2502440 “R19 ASN.1 identifier naming conventions and check list” (Huawei, HiSilicon)

- There are many naming mistakes (not respecting Guidelines) in RRC from Rel-15/16/17/18. We leave them in Rel-19, and reuse existing names where needed (for consistency between releases and other specs).

- Ensure field/IE names are consistent throughout the specs (and in other specs)

  - RAN2 will provide final ASN.1 names to RAN1/RAN4 parameter lists at the end of Rel-19.

### Guidelines on ASN.1

newDraftField-r19              ENUMERATED {ffs};

newList-r19                     SEQUENCE (SIZE (1..maxSizeFFS)) OF  NewId;

maxSizeFFS = 9999;

Can use these in draft ASN.1, to pass ASN.1 syntax check:

- RAN2 is allowed to deviate from structures and names imposed by e.g. the RAN1/4 Parameter lists and Feature lists, if/where appropriate.

- Default values (in case an optional field is absent in the message)

  - Use DEFAULT

  - Often not really motivated to use default values for the sake of saving in signalling size

    - Better to require Nw to configure the field (Use Need R instead of Need S)

- No need to ensure the ASN.1 colouring is respected.

  - ASN.1 colouring is made for full spec before submission.

### Use of -v19xy and -r19 tags

A suffix of the form "-r19" is used for ASN.1 fields or IEs that are

- introduced as new in release Rel-19 (there no earlier version of the field/IE)

- replacing field/IE introduced in a previous release.

A suffix of the form "-v19xy" is used for ASN.1 fields or types that are an extension of a corresponding earlier field or IE.

Do not use the tags in procedure text or field description texts unless needed (to differentiate between versions of the field).

### Need codes

- The Need codes and Conditions are comments in the ASN.1 (--) and not part of the ASN.1 itself.

- The purpose is to avoid to

  - to shorten field description text, and

  - unify descriptive texts (use similar wording everywhere).

- They are used in downlink signalling only.

ServingCellConfig ::=               SEQUENCE {

tdd-UL-DL-ConfigurationDedicated    TDD-UL-DL-ConfigDedicated

OPTIONAL,   -- Cond TDD

initialDownlinkBWP                  BWP-DownlinkDedicated

OPTIONAL,   -- Need M

downlinkBWP-ToReleaseList           SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Id

OPTIONAL,   -- Need N

downlinkBWP-ToAddModList            SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Downlink

OPTIONAL,   -- Need N

firstActiveDownlinkBWP-Id           BWP-Id

OPTIONAL,   -- Cond SyncAndCellAdd

bwp-InactivityTimer                 ENUMERATED {ms2, ms3, ms4, ms5, ms6, ms8, ms10,

ms20, ms30,

ms40,ms50, ms60, ms80,ms100, ms200,

ms300, ms500,

ms750, ms1280, ms1920, ms2560

spare10, spare9, spare8,

spare7, spare6, spare5, spare4, spare3,

spare2, spare1 }    OPTIONAL,   --Need R

defaultDownlinkBWP-Id               BWP-Id

OPTIONAL,   -- Need S

uplinkConfig                        UplinkConfig

OPTIONAL,   -- Need M

supplementaryUplink                 UplinkConfig

OPTIONAL,    -- Need M

<cut>

From 38331:

| Abbreviation | Meaning |
| --- | --- |
| Need S | Specified Used for (configuration) fields, whose field description or procedure specifies the UE behavior performed upon receiving a message with the field absent (and not if field description or procedure specifies the UE behavior when field is not configured). |
| Need M | Maintain Used for (configuration) fields that are stored by the UE i.e. not one-shot. Upon receiving a message with the field absent, the UE maintains the current value. |
| Need N | No action (one-shot configuration that is not maintained) Used for (configuration) fields that are not stored and whose presence causes a one-time action by the UE. Upon receiving message with the field absent, the UE takes no action. |
| Need R | Release Used for (configuration) fields that are stored by the UE i.e. not one-shot. Upon receiving a message with the field absent, the UE releases the current value. |

The following rule provides guidance for determining need codes for optional downlink fields:

- if the field needs to be stored by the UE (i.e. maintained) when absent:

- use Need M (=Maintain);

- else, if the field needs to be released by the UE when absent:

- use Need R (=Release);

- else, if UE shall take no action when the field is absent (i.e. UE does not even need to maintain any existing value of the field):

- use Need N (=None);

- else (UE behaviour upon absence does not fit any of the above conditions):

- use Need S (=Specified);

- specify the UE behaviour upon absence of the field in the procedural text or in the field description table.

### Conditions

| Abbreviation | Meaning |
| --- | --- |
| Cond conditionTag | Conditionally present Presence of the field is specified in a tabular form following the ASN.1 segment. |

- Conditions are primarily used to specify network restrictions on how/when to include fields in a message:

- Main reason for introducing Cond was to formalize the descriptive text and force editors to use similar language.

  - Constraints on message contents of the same message, e.g. that a field B is mandatory present (or included) if the same message includes field A and when it is set value X.

  - Configuration Constraints, e.g. that a field D can only be signalled if field C is configured and set to value Y. (i.e. regardless of whether field C is present in the same message or previously configured (received in a previous message).

The use of these conditions is illustrated by an example.

-- /example/ ASN1START

RRCMessage-IEs ::= SEQUENCE {

fieldA                          FieldA                  OPTIONAL,   -- Need M

fieldB                          FieldB                  OPTIONAL,   -- Cond FieldAsetToX

fieldC                          FieldC                  OPTIONAL,   -- Need M

fieldD                          FieldD                  OPTIONAL,   -- Cond FieldCsetToY

nonCriticalExtension            SEQUENCE {}             OPTIONAL

}

-- /example/ ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| FieldAsetToX | The field is mandatory present if fieldA is included and set to valueX. Otherwise the field is optionally present, need R. |
| FieldCsetToY | The field is optionally present, need M, if fieldC is configured and set to valueY. Otherwise the field is absent and the UE does not maintain the value |

### Delta-signalling

- Paradigm/ambition that rules the RAN2 work on signalling design, but not so well documented what we mean.

- Delta-signalling allows the Nw to

  - include only those fields in a message that the Nw would like to modify in the UE (the UE configuration), and

  - omit the remaining fields of the message that are unchanged in the “UE configuration”.

  - Typically, a handover command should be a small message.

  - Relies on that the optional field has “Need M”, so that the field is maintained by UE if the field is absent in the message.

- Problem: It must be possible for a Nw to release the configuration of a feature of a new release in case UE is handed over to a gNB that does not support this feature. We have in principle two ways to somewhat “overcome” this problem:

  - SetupRelease of the field – Nw can set the fields of non-supported features to be released (but Nw need of course implement the Release).

  - The field is Need R (UE releases the field if not present), and the Nw must always include the field to keep the feature configured (no delta-signalling for this field).

  - In practice, full or partly support of delta-signalling is complex to achieve.

  - Use of Need R (and Need M only within SetupRelease structure) is usually recommended for new features.

- Full configuration - UE clears old config and applies the complete config received in the RRCReconfiguration message.

RRCReconfiguration-v1530-IEs ::=            SEQUENCE {

masterCellGroup OCTET STRING (CONTAINING CellGroupConfig) OPTIONAL, -- Need M

fullConfig      ENUMERATED {true}     OPTIONAL, -- Cond FullConfig

dedicatedNAS-MessageList                SEQUENCE (SIZE(1..maxDRB)) OF

DedicatedNAS-Message                     OPTIONAL, -- Cond nonHO

:

fullConfig Indicates that the full configuration option is applicable for the RRCReconfiguration message for intra-system intra-RAT HO. For inter-RAT HO from E-UTRA to NR, fullConfig indicates whether or not delta signalling of SDAP/PDCP from source RAT is applicable. This field is absent if any DAPS bearer is configured or when the RRCReconfiguration message is transmitted on SRB3, and in an RRCReconfiguration message for SCG contained in another RRCReconfiguration message (or RRCConnectionReconfiguration message, see TS 36.331 [10]) transmitted on SRB1.

| Conditional Presence | Explanation |
| --- | --- |
| FullConfig | The field is mandatory present in case of inter-system handover from E-UTRA/EPC to NR. It is optionally present, Need N, during a reconfiguration with sync which is not related to an LTM cell switch or subsequent CPAC, and also in first reconfiguration after reestablishment; or for intra-system handover from E-UTRA/5GC to NR. It is absent otherwise. |

### SetupRelease

This is the most advanced ASN.1 feature we use in RRC.

The usage of the parameterised SetupRelease type is like a function call in programming languages where the element type parameter is passed as a parameter. The parameterised type only implies a textual change in abstract syntax where all references to the parameterised type are replaced by the compiler with the release/setup choice.

MAC-CellGroupConfig ::=  SEQUENCE {

drx-Config               SetupRelease { DRX-Config }          OPTIONAL,   -- Need M

schedulingRequestConfig  SchedulingRequestConfig              OPTIONAL,   -- Need M

bsr-Config               BSR-Config                           OPTIONAL,   -- Need M

<cut>

expands to

MAC-CellGroupConfig ::=  SEQUENCE {

drx-Config        CHOICE {

release         NULL,

setup           DRX-Config

}

schedulingRequestConfig  SchedulingRequestConfig              OPTIONAL,   -- Need M

bsr-Config               BSR-Config                           OPTIONAL,   -- Need M

<cut>

This means in the PER-encoded bits, there is one choice-bit.

  - If the bit is 0, the following bits encodes the next field, and UE releases the drx-Config.

  - If this bit is 1, DRX-Config follows, and UE stores this (as specified for the DRX-Config).

### Lists

#### ToAddMod lists

sCellToAddModList   SEQUENCE (SIZE (1..10)) OF SCellConfig   OPTIONAL,   -- Need N

sCellToReleaseList  SEQUENCE (SIZE (1..10)) OF SCellIndex    OPTIONAL,   -- Need N

SCellConfig ::=     SEQUENCE {

sCellIndex             SCellIndex,

sCellConfigCommon      ServingCellConfigCommon       OPTIONAL,   -- Cond SCellAdd

sCellConfigDedicated   ServingCellConfig             OPTIONAL,   -- Cond SCellAddMod

...

}

SCellIndex ::= INTEGER (1..10)

- Used to configure a list stored by the UE

- New list elements can be added, existing list elements can be deleted.

- Each list item has a unique/configured id, each list item can be modified or released.

- Extension of lists

  - Increase list size (number of list elements), e.g. from 10 to 15

    - Introduce a new ToAddModListSizeExt to configure 5 additional list elements in the same message

    - Introduce a new ToReleaseListSizeExt that can release 5 additional list elements, in total 15 in the same message

    - Extend size of list element id – usually add a new parallel list of size 15

  - Add new fields to the list elements

    - With extension marker “…”.

      - Size cost

    - Without extension marker

      - Introduce new parallel list with the new field, same size, same list positions

  - Increase size of list and introduce new list elements

    - See e.g. pathlossReferenceRSToAddModList

#### Other lists

CellReselectionPriorities ::=       SEQUENCE {

freqPriorityListEUTRA  FreqPriorityListEUTRA                    OPTIONAL,       -- Need M

freqPriorityListNR     FreqPriorityListNR                       OPTIONAL,       -- Need M

t320                   ENUMERATED {min5, min10, min20, min30, min60, min120, min180,

spare1}                      OPTIONAL,     -- Need R

...

}

FreqPriorityListEUTRA ::=           SEQUENCE (SIZE (1..maxFreq)) OF FreqPriorityEUTRA

FreqPriorityListNR ::=              SEQUENCE (SIZE (1..maxFreq)) OF FreqPriorityNR

When UE receives the field/list in a message, it completely replaces an old list (received in a previous message).

To extend the size of the list (number of list elements), another list is introduced (xxxListExt). The List and the ListExt are together considered as one list.

See e.g. commonSearchSpaceList.

## Extensions

### Extension at the end of a message (non-critical extension)

- SEQUENCE {} is the “empty Sequence” or “empty object”.

- SEQUENCE {} makes an ASN.1 decoder at the receiver to ignore all bits that are received at the end of the message.

Rel-15, Rel-16

Paging ::=                          SEQUENCE {

pagingRecordList                    PagingRecordList          OPTIONAL, -- Need N

lateNonCriticalExtension            OCTET STRING              OPTIONAL,

nonCriticalExtension                SEQUENCE {}               OPTIONAL

}

Rel-17

Paging ::=                          SEQUENCE {

pagingRecordList                    PagingRecordList          OPTIONAL, -- Need N

lateNonCriticalExtension            OCTET STRING              OPTIONAL,

nonCriticalExtension                Paging-v1700-IEs          OPTIONAL

}

Paging-v1700-IEs ::=                SEQUENCE {

pagingRecordList-v1700              PagingRecordList-v1700    OPTIONAL, -- Need N

pagingGroupList-r17                 PagingGroupList-r17       OPTIONAL, -- Need N

nonCriticalExtension                SEQUENCE {}               OPTIONAL

}

### Late non-critical extension at the end of a message

- Extensions of an earlier release are added at the end of a message, but before extensions of a later release.

- OCTET STRING gets a length indicator in PER-encoded bits such that receiver understands where message continues.

Before UE-NR-Capability-v15c0 is introduced:

UE-NR-Capability ::=            SEQUENCE {

accessStratumRelease            AccessStratumRelease,

pdcp-Parameters                 PDCP-Parameters,

rlc-Parameters                  RLC-Parameters                                                        :

lateNonCriticalExtension        OCTET STRING                    OPTIONAL,

nonCriticalExtension            UE-NR-Capability-v1530          OPTIONAL

}

After UE-NR-Capability-v15c0 is introduced:

UE-NR-Capability ::=            SEQUENCE {

accessStratumRelease            AccessStratumRelease,

pdcp-Parameters                 PDCP-Parameters,

rlc-Parameters                  RLC-Parameters                                                        :

lateNonCriticalExtension        OCTET STRING (CONTAINING UE-NR-Capability-v15c0)

OPTIONAL,

nonCriticalExtension            UE-NR-Capability-v1530          OPTIONAL

}

-- Regular non-critical Rel-15 extensions:

UE-NR-Capability-v1530 ::=               SEQUENCE {

fdd-Add-UE-NR-Capabilities-v1530         UE-NR-CapabilityAddXDD-Mode-v1530

OPTIONAL,

tdd-Add-UE-NR-Capabilities-v1530         UE-NR-CapabilityAddXDD-Mode-v1530

OPTIONAL,

:

nonCriticalExtension                     UE-NR-Capability-v1540                                       OPTIONAL

}

:

-- Late non-critical Rel-15 extensions:

UE-NR-Capability-v15c0 ::=               SEQUENCE {

nrdc-Parameters-v15c0                    NRDC-Parameters-v15c0

OPTIONAL,

:

nonCriticalExtension                     UE-NR-Capability-v15g0

OPTIONAL

}

### Adding new fields in the middle of a message.

- Nice to get new fields in the correct context.

- Only possible if the IE is prepared with extension marker “…” when it was created.

- In the PER-encoded bits, a presence bit is added. If this bit is set (extension is present), also a length field is added in the encoded bits. This allows that a receiver (that support an earlier ASN.1 version) can skip the extension, and continue to decode after the extension.

- [[ ]] are used to group fields that are added at the same time, called “extension addition group” (EAC). One length indicator is added for the EAC.

- All new fields within [[ ]] are set as OPTIONAL.

- Overhead for “…” is one bit. Overhead for transmitting a field within [[ ]] is 2-3 octets (length indicator and optionality bits added by PER).

- We avoid using “…” in lists in SIBs and UE capabilities (reduce size).

- If the IE cannot be extended (“…” is missing), also the parent IE (or parent’s parent...) need to be “extended”.

### PER Examples

“Transfer syntax for RRC PDUs is derived from their ASN.1 definitions by use of Packed Encoding Rules, unaligned as specified in ITU-T Rec. X.691.”

Some examples on PER encoding from TS 38331:

all-Traffic-NumReq-r16          INTEGER (1..8)        OPTIONAL,

One bit indicates whether the field is present or not, 3 bits to encode the values 1..8.

cellBarred                          ENUMERATED {barred, notBarred},

One bit to encode the two vales.

preEmptiveBSR-r16                        ENUMERATED {supported}     OPTIONAL,

One optionality bit (only!).

ran-AreaCells                       SEQUENCE (SIZE (1..32)) OF  CellIdentity

CellIdentity ::=                         BIT STRING (SIZE (36))

Variable-sized list, 5 bits to indicate the number of cell identities in the list, 36 bits per cell identity.

periodicityAndOffset-r17                   CHOICE {

slots10                                    INTEGER (0..9),

slots20                                    INTEGER (0..19),

slots40                                    INTEGER (0..39),

slots80                                    INTEGER (0..79)

},

2 bits to encode the choice value. If “00”, 4 bits encodes the field. If “01”, 5 bits encodes the field etc.

condRRCReconfig-r16              OCTET STRING (CONTAINING RRCReconfiguration)

OPTIONAL,    -- Cond condReconfigAdd

One optionality bit, x bits (16?) PER-hidden length indicator for number of octets. 
Used for “containers”

supportedGapPattern-r16                 BIT STRING (SIZE (2))                   OPTIONAL

One optionality bit, 2-bits bit string.