3GPP TSG-RAN WG2 Meeting #129bis	R2-2502986

Wuhan, China, Apr. 7th – 11th , 2025

Source: 	Session chair (Ericsson)

Title:	Report from session on maintenance and SON/MDT

Title:	9.6

[AT129bis][600] Organizational – Maintenance and SON/MDT (Ericsson)

Scope:

    - Share plans for the meeting and list of ongoing email discussions

    - Share meetings notes and agreements for review and endorsement

    - Flag LSs and agreed CRs for discussion

Intended outcome:

    - General information sharing about the sessions

# 4	EUTRA Rel-17 and earlier

Only essential corrections. No documents should be submitted to 4. Please submit to 4.x

## 4.1	EUTRA corrections Rel-17 and earlier

(NB_IOTenh4_LTE_eMTC6-Core; leading WG: RAN1; REL-17; WID: RP-211340)

(UPIP_EN-DC_UE; leading WG: RAN3; REL-17; WID: RP-213669)

(LTE TEI17)

Essential corrections to LTE Rel-17 topics not covered by other agenda items.

(NB_IOTenh3-Core; leading WG: RAN1; REL-16; started: Jun 18; Completed: June 20; WID: RP-200293); REL-15 and Earlier NB-IoT WIs are in scope but not listed explicitly (long list).

(LTE_eMTC5-Core; LTE_eMTC5-Core; leading WG: RAN1; REL-16; started: Jun 18; Completed:  June 20; WID: RP-192875;), REL-15 and Earlier eMTC WIs are in scope but not listed explicitly (long list).

(LTE_feMob-Core; leading WG: RAN2; REL-16; started: Jun 18; Completed: June 20; WID: RP-190921);

(LTE_terr_bcast-Core, LTE_DL_MIMO_EE-Core, LTE_high_speed_enh2-Core; LTE TEI16 Non-positioning);

(LTE_NBIOT_eMTC_NTN; leading WG: RAN1; REL-17; WID: RP-211601)

REL-16 and Earlier EUTRA WIs are in scope but not listed explicitly (long list), Except Positioning WI, which is addressed by AIs below.

NOTE that LTE corrections related to NR WIs or Joint NR LTE WIs should be submitted to NR AIs below.

NOTE that LTE corrections which are the same as an NR correction should be submitted to the respective NR AI (so the NR CR and LTE CR can be treated together).

This Agenda Item is treated in the Maintenance Breakout session (Corrections for LTE_NBIOT_eMTC_NTN might be treated in the NTN breakout session)

Report on leave A4/A5

R2-2502330	Clarification on A4 A5 ReportOnLeave	ZTE Corporation, Sanechips	discussion	Rel-15	LTE_Aerial-Core

Proposal 1	To define A4/A5 ReportOnLeave as “optional feature without UE capability signalling”, agree the CRs in [1]~[4].

R2-2502331	Introducing UE capability for A4 A5 ReportOnLeave	ZTE Corporation, Sanechips	CR	Rel-15	36.306	15.12.0	1907	-	F	LTE_Aerial-Core

R2-2502332	Introducing UE capability for A4 A5 ReportOnLeave	ZTE Corporation, Sanechips	CR	Rel-16	36.306	16.13.0	1908	-	A	LTE_Aerial-Core

R2-2502333	Introducing UE capability for A4 A5 ReportOnLeave	ZTE Corporation, Sanechips	CR	Rel-17	36.306	17.8.0	1909	-	A	LTE_Aerial-Core

R2-2502334	Introducing UE capability for A4 A5 ReportOnLeave	ZTE Corporation, Sanechips	CR	Rel-18	36.306	18.4.0	1910	-	A	LTE_Aerial-Core

-	Xiaomi want a capa indication. MediaTek and Qualcomm agrees. ZTE think that if someone already implement but would then (of course) not have the capa indication. But if no one implemented then we can do the capa indication. Huawei also want a capa indication also.

-	Samsung agrees with the intention but need more time. ZTE think this is urgent and want a post meeting email disc.

We assume we will go with optional with UE capability indication, CRs to be seen next meeting.

timeUntilReconnection

R2-2502528	Correction on timeUntilReconnection in RLF report	ZTE Corporation, Sanechips	CR	Rel-17	36.331	17.12.0	5108	-	F	NR_ENDC_SON_MDT_enh-Core

R2-2502529	Correction on timeUntilReconnection in RLF report	ZTE Corporation, Sanechips	CR	Rel-18	36.331	18.5.0	5109	-	A	NR_ENDC_SON_MDT_enh-Core

-	Xiaomi think there are something missed when porting this change from NR specs. Ericsson and Qualcomm agrees. ZTEs intention was that the procedure should be the same.

-	Xiaomi think the failed PCell field wonder if it indeed can reflect the first failure.

[AT129bis][602][Maint] Correction on timeUntilReconnection in RLF report (ZTE)

Scope:

    - Produce agreeable CRs

Intended outcome:

    - In principle agreed CRs in R2-2503102 and R2-2503103

Deadline:

    - Wednesday 17:00. The intention is to agree over email.

R2-2503116 	Correction on timeUntilReconnection in RLF report	ZTE Corporation, Xiaomi, Sanechips	CR	Rel-16	36.331	16.X.Y	5113	-	F	NR_ENDC_SON_MDT_enh-Core

R2-2503102	Correction on timeUntilReconnection in RLF report	ZTE Corporation, Xiaomi, Sanechips	CR	Rel-17	36.331	17.12.0	5108	1	F	NR_ENDC_SON_MDT_enh-Core

R2-2503103	Correction on timeUntilReconnection in RLF report	ZTE Corporation, Xiaomi, Sanechips	CR	Rel-18	36.331	18.5.0	5109	1	A	NR_ENDC_SON_MDT_enh-Core

The above three are in-principle agreed

deprioritisationReq

R2-2502836	Correction on deprioritisationReq delete	Google	CR	Rel-18	36.331	18.5.0	5111	-	F	LTE-L23, TEI11

-	Docomo think the CR content is clear from 36.304. And if the timer is not stopped, there is no problem. Google agrees that 36.304 already has this info. Samsung agrees with Docomo and its clear that PLMN selection is perfomed the field is released. Huawei think the UE behaviour is clear from 304 but didn’t find any description of stopping the timer and think there is no need to capture this. Qualcomm wonders if the CR changes UE behaviour? If it changes UE behaviour Qualcomm is not OK with it.

Not pursued

NTN – to be handled in Sergios session

R2-2502561	Correction to explicit indication of epoch in SIB31	Nordic Semiconductor ASA	discussion	Rel-17

# 5	NR Rel-15 and Rel-16

Essential corrections only.

Tdoc Limitation: 2 tdocs in total for all sub agenda items

In case a correction need to be reflected in both NR TS and LTE TS, the corrections should be submitted under one single AI (so the NR and LTE correction can be treated together), the sub-Ais below this

## 5.1	Common

Includes the following WIs and input that doesn’t fit elsewhere.

(NR_newRAT-Core; leading WG: RAN1; REL-15; started: Mar. 17; closed: Jun. 19: WID: RP-191971)

(NR_IAB-Core; leading WG: RAN2; REL-16; started: Dec 18; target Aug 20; WID: RP-200840)

(NR_unlic-Core; leading WG: RAN1; REL-16; started: Dec 18; Closed June 20; WID: RP-192926).

(NR_IIOT-Core; leading WG: RAN2; REL-16; started: Mar 19; Completed: Jun 20; WID: RP-200797)

(NR_UE_pow_sav-Core; leading WG: RAN1; REL-16; started: Mar 19; Completed Jun 20; WID: RP-200494).

(NR_2step_RACH-Core; leading WG: RAN1; REL-16; started: Dec 18; Completed: June 20; WID: RP-200085).

(SRVCC_NR_to_UMTS-Core; leading WG: RAN2; REL-16; started: Dec 18; Completed; Mar 20; WID: RP-190713)

(RACS-RAN-Core, leading WG: RAN2; REL-16; started: Mar 19; completed: Jun 20; WID: RP-191088)

(NG_RAN_PRN-Core; leading WG: RAN3; REL-16; started: Mar 19; completed: June 20; WID: RP-200122)

(NR_eMIMO-Core, leading WG: RAN1; REL-16; started: Jun 18; target; Aug 20; WID: RP-200474)

(NR_CLI_RIM; leading WG: RAN1; REL-16; started: Dec 18; Completed: Jun 20; WID: RP-191997)

(NR_L1enh_URLLC-Core, leading WG: RAN1; REL-16; Completed: June 20; WID: RP-191584)

(LTE_NR_DC_CA_enh-Core; leading WG: RAN2; REL-16; started: Jun 18; Target Aug 20; WI RP-200791)

(NR_Mob_enh-Core; leading WG: RAN2; REL-16; started: Jun 18; Completed June 20; WID: RP-192277).

(NR_SON_MDT-Core; leading WG: RAN3; REL-16; started: Jun 19; Completed June 20; WID: RP-191776)

(5G_V2X_NRSL-Core; leading WG: RAN1; REL-16; started: Mar 19; completed; Aug 20; WID: RP-200129)

(NR_HST, NR_RRM_enh-Core, NR_RF_FR1, NR_RF_FR2_req_enh, NR_n66_BW, LTE_NR_B41_Bn41_PC29dBm-Core, NR_CSIRS_L3meas,)

(NR TEI16)

LTE mob enh corrections that are common with NR mobility enhancements should be submitted to this AI.

### 5.1.1	Stage 2 and Organisational

Incoming LSs, etc. You should discuss your stage 2 CRs with the specification rapporteurs before submission. Includes impact to 38.300, 36.300, 37.340

DCP

R2-2502473	Corrections on DCP	Huawei, HiSilicon, Nokia (Rapporteur)	CR	Rel-16	38.300	16.19.0	0983	-	F	NR_UE_pow_sav-Core

R2-2502474	Corrections on DCP	Huawei, HiSilicon, Nokia (Rapporteur)	CR	Rel-17	38.300	17.12.0	0984	-	A	NR_UE_pow_sav-Core

R2-2502475	Corrections on DCP	Huawei, HiSilicon, Nokia (Rapporteur)	CR	Rel-18	38.300	18.5.0	0985	-	A	NR_UE_pow_sav-Core

-	Xiaomi is OK with the first change. Samsung think this is clear from Stage-3. Vivo think the first change is OK. LG think the first change is not needed since its clear from Stage-3. ZTE think its clear from stage-3. Nokia supports the CR.

-	Xiaomi think that for the second change they want to clarify that its NR PCell/PSCell. Samsung is OK with the second change. Ericsson supports this change and if the second change is made then the first can be fixed at the same time.

The above three in-principle agreed

### 5.1.3	Control Plane corrections

#### 5.1.3.1	NR RRC

Corrections to 38331, and related change to other TS if applicable, e.g. 36331, Stage-2 etc.

SCG failure information

R2-2501906	Correction on Initiation of NR SCG failure information	CATT	CR	Rel-16	36.331	16.19.0	5102	-	F	LTE_NR_DC_CA_enh-Core

R2-2501907	Correction on Initiation of NR SCG failure information	CATT	CR	Rel-17	36.331	17.12.0	5103	-	F	LTE_NR_DC_CA_enh-Core

R2-2501908	Correction on Initiation of NR SCG failure information	CATT	CR	Rel-18	36.331	18.5.0	5104	-	F	LTE_NR_DC_CA_enh-Core

-	CATT suggests to merge to rapp CR next meeting. Qualcomm agrees but highlights its submitted to the wrong Agenda Item.

To be merged in rapp CR

URLLC

R2-2502271	Correction to nrofCandidates-CI for DCI format 2_4	Huawei, HiSilicon	CR	Rel-16	38.331	16.19.0	5295	-	F	NR_L1enh_URLLC-Core

R2-2502272	Correction to nrofCandidates-CI for DCI format 2_4	Huawei, HiSilicon	CR	Rel-17	38.331	17.12.0	5296	-	A	NR_L1enh_URLLC-Core

R2-2502273	Correction to nrofCandidates-CI for DCI format 2_4	Huawei, HiSilicon	CR	Rel-18	38.331	18.5.1	5297	-	A	NR_L1enh_URLLC-Core

-	Samsung supports the CR. Ericsson think the CR is NBC and this needs to be stated.

All in-principle agreed, but next version should not have curly quotes and next version of the CR should state on the cover page that NWs wanting to support this feature must implement this CR.

NZP CSI-RS resources

R2-2502344	Correction on number of NZP CSI-RS resources	Ericsson	CR	Rel-15	38.331	15.28.0	5298	-	F	NR_newRAT-Core

R2-2502389	Correction on number of NZP CSI-RS resources	Ericsson	CR	Rel-16	38.331	16.19.0	5299	-	A	NR_newRAT-Core

R2-2502390	Correction on number of NZP CSI-RS resources	Ericsson	CR	Rel-17	38.331	17.12.0	5300	-	A	NR_newRAT-Core

R2-2502391	Correction on number of NZP CSI-RS resources	Ericsson	CR	Rel-18	38.331	18.5.1	5301	-	A	NR_newRAT-Core

In-principle agreed or merged to rapp CR if there is one in the next meeting.

Spatial Relation Info

R2-2502884	Issues on PUCCH Spatial Relation Info Configuration	ZTE, Samsung	discussion	Rel-16

Proposal 1: RAN 2 is asked to select one of the following solutions:

-	Solution 1: Dummify size extension list, configuration extension release list, and the PUCCH-SpatialRelationInfoId-r16, and re-introduce a size extension ToAddMod and ToRelease list where the ToRelease list is the list of PUCCH-SpatialRelationInfoId-v1610.

-	Solution 2: Add a clarification in the field description to clarify that either PUCCH-SpatialRelatioinInfoId-Id or PUCCH-SpatialRelationInfo-Id-v1610 is considered as indicated if UE receiving the spatialRelationInfoToReleaseListExt-v1610

Proposal 2: The UE shall consider entries in spatialRelationInfoToAddModList and in spatialRelationInfoToAddModListSizeExt as a single list if the spatialRelationInfoToAddModListExt is not included, i.e. an entry created using spatialRelationInfoToAddModList can be modified using spatialRelationInfoToAddModListSizeExt (or deleted using spatialRelationInfoToReleaseListSizeExt) and vice-versa.’

On P1

-	Xiaomi prefers Solution 1. Huawei think the spec is clear and it works so no need to update the spec. MediaTek agrees with Huawei. CATT think there is an issue to be solved and prefers Solution 1. Ericsson are unsure what the issue would be, they think the current spec works but perhaps some polishing of a field description could be considered. ZTE think its unclear how to release the elements in the list. Samsung supports the proposal and think this list is not following the guidelines. Huawei think it doesn’t matter which release list sends an ID, the UE shall release it, UE behaviour is clear and this is already implemented in products and we cannot change it. Huawei finds Solution 1 NBC and not acceptable, and solution 2 is not needed. Perhaps some small polishing can be done, in Huaweis mind. ZTE think we can capture something in minutes.

On P2:

-	CATT think the spec is clear.

Postponed, but we will not consider any ASN.1 changes.

#### 5.1.3.2	UE capabilities

UE cap corrections 38306, 38331

interBandMRDC-WithOverlapDL-Bands

R2-2502505	Type clarification for interBandMRDC-WithOverlapDL-Bands-r16	Apple, Huawei, HiSilicon, xiaomi	CR	Rel-16	38.306	16.20.0	1251	-	F	NR_newRAT-Core, TEI16

R2-2502506	Type clarification for interBandMRDC-WithOverlapDL-Bands-r16	Apple, Huawei, HiSilicon, xiaomi	CR	Rel-17	38.306	17.12.0	1252	-	A	NR_newRAT-Core, TEI16

R2-2502507	Type clarification for interBandMRDC-WithOverlapDL-Bands-r16	Apple, Huawei, HiSilicon, xiaomi	CR	Rel-18	38.306	18.5.0	1253	-	A	NR_newRAT-Core, TEI16

In-principal agreed or merged to rapp CR if there is one in the next meeting.

IAB

R2-2502631	Correction to pdsch-256QAM-FR1 for IAB-MT	Nokia	CR	Rel-16	38.306	16.20.0	1259	-	F	NR_IAB-Core

R2-2502632	Correction to pdsch-256QAM-FR1 for IAB-MT	Nokia	CR	Rel-17	38.306	17.12.0	1260	-	A	NR_IAB-Core

R2-2502633	Correction to pdsch-256QAM-FR1 for IAB-MT	Nokia	CR	Rel-18	38.306	18.5.0	1261	-	A	NR_IAB-Core

-	Huawei think the first change is not needed since there is a table clarifying what IAB nodes support and not. But Huawei is OK to IPA now and then come back if needed.

In-principle agreed, but “pdsch-256QAM-FR1” should be italic.

SRS-only cells

R2-2502991	Consideration on Introduction of SRS Capability Reporting for SRS-only cell	ZTE Corporation	discussion	Rel-15	NR_newRAT-Core

Proposal 1: Introduce the below SRS capabilities on the FeatureSetDownlink from the R18 and allow the early implementation.

o	zeroSlotOffsetAperiodicSRS;

o	offsetSRS-CB-PUSCH-Ant-Switch-fr1-r16;

o	offsetSRS-CB-PUSCH-PDCCH-MonitorSingleOcc-fr1-r16;

o	offsetSRS-CB-PUSCH-PDCCH-MonitorAnyOccWithoutGap-fr1-r16;

o	offsetSRS-CB-PUSCH-PDCCH-MonitorAnyOccWithGap-fr1-r16;

o	offsetSRS-CB-PUSCH-PDCCH-MonitorAnyOccWithSpanGap-fr1-r16;

o	srs-AntennaSwitching2SP-1Periodic-r17;

o	srs-ExtensionAperiodicSRS-r17;

o	srs-OneAP-SRS-r17

Proposal 2: Add a precondition to the newly introduced capability in the FeatureSetDownlink, e.g. it is only present when the UE supports SRS carrier switching from this band.

On P1:

-	Huawei think its easier for implementation to go from R15. Nokia think R17 is fine. Ericsson think usually we change from R15 in these cases. Xiaomi think R15 is fine and better for implementations.

On P2:

-	Huawei do not think we need this. OPPO thought this proposal should be agreeable. Huawei think there is no need for this.

R2-2502891	Correction on SRS capability reporting	Huawei, HiSilicon, Ericsson, OPPO	CR	Rel-15	38.331	15.28.0	5324	-	F	NR_newRAT-Core

R2-2502892	Correction on SRS capability reporting	Huawei, HiSilicon, Ericsson, OPPO	CR	Rel-16	38.331	16.19.0	5325	-	A	NR_newRAT-Core

R2-2502893	Correction on SRS capability reporting	Huawei, HiSilicon, Ericsson, OPPO	CR	Rel-17	38.331	17.12.0	5326	-	A	NR_newRAT-Core

R2-2502894	Correction on SRS capability reporting	Huawei, HiSilicon, Ericsson, OPPO	CR	Rel-18	38.331	18.5.1	5327	-	A	NR_newRAT-Core

R2-2502895	Correction on SRS capability reporting	Huawei, HiSilicon, Ericsson, OPPO	CR	Rel-15	38.306	15.27.0	1268	-	F	NR_newRAT-Core

R2-2502896	Correction on SRS capability reporting	Huawei, HiSilicon, Ericsson, OPPO	CR	Rel-16	38.306	16.20.0	1269	-	A	NR_newRAT-Core

R2-2502897	Correction on SRS capability reporting	Huawei, HiSilicon, Ericsson, OPPO	CR	Rel-17	38.306	17.12.0	1270	-	A	NR_newRAT-Core

R2-2502898	Correction on SRS capability reporting	Huawei, HiSilicon, Ericsson, OPPO	CR	Rel-18	38.306	18.5.0	1271	-	A	NR_newRAT-Core

[AT129bis][603][Maint] Correction on SRS capability reporting (Huawei)

Scope:

    - Discuss offline (face-to-face preferred, if suitable)

    - Produce agreeable CRs

Intended outcome:

    - In principal agreed CRs in R2-2503104 - R2-2503111

Deadline:

    - Wednesday 17:00. The intention is to agree over email.

R2-2503104	Correction on SRS capability reporting	Huawei, HiSilicon, Ericsson, OPPO	CR	Rel-15	38.331	15.28.0	5324	1	F	NR_newRAT-Core

In-principle agreed

R2-2503105	Correction on SRS capability reporting	Huawei, HiSilicon, Ericsson, OPPO	CR	Rel-16	38.331	16.19.0	5325	1	A	NR_newRAT-Core

In-principle agreed, but should be Cat F

R2-2503106	Correction on SRS capability reporting	Huawei, HiSilicon, Ericsson, OPPO	CR	Rel-17	38.331	17.12.0	5326	1	A	NR_newRAT-Core

In-principle agreed, but should be Cat F

R2-2503107	Correction on SRS capability reporting	Huawei, HiSilicon, Ericsson, OPPO	CR	Rel-18	38.331	18.5.1	5327	1	A	NR_newRAT-Core

In-principle agreed, but should be Cat F

R2-2503108	Correction on SRS capability reporting	Huawei, HiSilicon, Ericsson, OPPO	CR	Rel-15	38.306	15.27.0	1268	1	F	NR_newRAT-Core

In-principle agreed

R2-2503109	Correction on SRS capability reporting	Huawei, HiSilicon, Ericsson, OPPO	CR	Rel-16	38.306	16.20.0	1269	1	A	NR_newRAT-Core

In-principle agreed, but should be Cat F

R2-2503110	Correction on SRS capability reporting	Huawei, HiSilicon, Ericsson, OPPO	CR	Rel-17	38.306	17.12.0	1270	1	A	NR_newRAT-Core

In-principle agreed, but should be Cat F

R2-2503111	Correction on SRS capability reporting	Huawei, HiSilicon, Ericsson, OPPO	CR	Rel-18	38.306	18.5.0	1271	1	A	NR_newRAT-Core

In-principle agreed, but should be Cat F

Old revisions

R2-2502865	Consideration on Introduction of SRS Capability Reporting for SRS-only cell	ZTE Corporation	discussion	Rel-15	NR_newRAT-Core

#### 5.1.3.3	Other

This agenda item addresses the idle and inactive behaviour specified in 38.304 or 36.304, LTE-specific changes for the applicable WIs, Other parts not covered elsewhere.

SUL

R2-2502011	Clarification on usage of q-RxLevMinSUL	Qualcomm Incorporated (Rapporteur)	CR	Rel-15	38.304	15.8.0	0429	-	F	NR_newRAT-Core

R2-2502012	Clarification on usage of q-RxLevMinSUL	Qualcomm Incorporated (Rapporteur)	CR	Rel-16	38.304	16.11.0	0430	-	A	NR_newRAT-Core

R2-2502013	Clarification on usage of q-RxLevMinSUL	Qualcomm Incorporated (Rapporteur)	CR	Rel-17	38.304	17.10.0	0431	-	A	NR_newRAT-Core

R2-2502015	Clarification on usage of q-RxLevMinSUL	Qualcomm Incorporated (Rapporteur)	CR	Rel-18	38.304	18.4.0	0432	-	A	NR_newRAT-Core

-	OPPO think the intention is OK but only want to remove “if present”, Samsung agrees. ZTE think no change is needed and do not think there should be a problem. Vivo and Xiaomi think no change is needed.

Postponed to next meeting

MDT

R2-2502404	Correction to Immediate MDT measurements	Nokia	CR	Rel-16	37.320	16.8.0	0137	-	F	NR_SON_MDT-Core

R2-2502405	Correction to Immediate MDT measurements	Nokia	CR	Rel-17	37.320	17.5.0	0138	-	A	NR_SON_MDT-Core

R2-2502406	Correction to Immediate MDT measurements	Nokia	CR	Rel-18	37.320	18.3.0	0139	-	A	NR_SON_MDT-Core

-	Qualcomm think that the text talks about what the standard supports, not what all UEs must support. Huawei is OK with the proposed CRs.

The above three are in-principle agreed

# 6	NR Rel-17

Essential corrections only.  Editorial/clarifications should be sent to be reviewed and approved by spec rapporteurs prior to submission.  Editorials should only be submitted by spec rapporteurs.

Tdoc limitation: 4 Tdocs

## 6.1	Common

(NR_MG_enh-Core; leading WG: RAN4; REL-17; WID: RP-211591)

(NR_UDC_enh-Core; leading WG: RAN2; REL-17; WID: RP-211203)

(NG_RAN_PRN_enh-Core; leading WG: RAN3; REL-17; WID: RP-202363)

(NR_IAB_enh-Core; leading WG: RAN2; REL-17; WID: RP-211548)

(NR_UE_pow_sav_enh-Core; leading WG: RAN2; REL-17; WID: RP-212630)

(LTE_NR_DC_enh2-Core; leading WG: RAN2; REL-17; WID: RP-201040)

(LTE_NR_MUSIM-Core; leading WG: RAN2; REL-17; WID: RP-212610)

(NR_Slice-Core; leading WG: RAN2; REL-17; WID: RP-212534)

(NR_QoE-Core; leading WG: RAN3; REL-17; WID: RP-211406)

(NR_ext_to_71GHz-Core; leading WG: RAN1; REL-17; WID: RP-212637)

(NR_cov_enh-Core; leading WG: RAN1; REL-17; WID: RP-211566): non-RACH-indication parts

(NR_redcap-Core; leading WG: RAN1; REL-17; WID: RP-211574)

(NR_feMIMO-Core; leading WG: RAN1; REL-17; WID: RP-212535)

(NR_SmallData_INACTIVE-Core, leading WG: RAN2; REL-17; WID: RP-212594)

(NR_IIOT_URLLC_enh-Core; leading WG: RAN2; REL-17; WID: RP-210854)

(NR_MBS-Core; leading WG: RAN2; REL-17; WID: RP-201038)

(NR_ENDC_SON_MDT_enh-Core; leading WG: RAN3; REL-17; WID: RP-201281)

(NR_NTN_solutions-Core; leading WG: RAN2; REL-17; WID: RP-211557)

(NR_SL_enh-Core; leading WG: RAN1; REL-17; WID: RP-202846)

(NR_SL_Relay-Core; leading WG: RAN2; REL-17; WID: RP-212601)

PRACH partitioning items

(NR TEI17)

Includes Rel-17 Work Items without specific R2 Agenda Item, e.g. RAN1 and RAN4 led items, SA2 and CT1 led items (was previously “Rel-17 Other”)

Includes aspects that does not fit under the more specific AIs, e.g. multi-WI aspects.

Corrections for NR_NTN_solutions-Core might be treated in the NTN breakout session.

### 6.1.1	Stage 2 and Organisational

Incoming LSs, etc. You should discuss your stage 2 CRs with the specification rapporteurs before submission. Includes impact to 38.300, 37.340, (36.300 if applicable)

HSDN

R2-2502441	Correction on cell type specific cell reselection prioritisation [NR_HSDN]	Huawei, HiSilicon, Nokia, Qualcomm, CMCC, Xiaomi, CATT, Apple	CR	Rel-17	38.300	17.12.0	0981	-	F	TEI17, NR_mobile_IAB-Core

R2-2502442	Correction on cell type specific cell reselection prioritisation [NR_HSDN]	Huawei, HiSilicon, Nokia, Qualcomm, CMCC, Xiaomi, CATT, Apple	CR	Rel-18	38.300	18.5.0	0982	-	A	TEI17, NR_mobile_IAB-Core

-	Samsung think only SA is impacted. Ericsson think the R17 CR should not have mobile IAB WI code. The date is also wrong. Docomo think we need separate CRs.

Both in-principle agreed, mobile IAB WI code should be removed for the R17 CR, and both should be cat F, and date should be correct. Or other approach if RAN2 secretary has good suggestions.

NTN – to be handled in Sergios session

R2-2501704	LS on stage 1 requirements for the support for PWS over satellite NGRAN in Rel-17 (C1-250715; contact: Qualcomm)	CT1	LS in	Rel-17	5GSAT	To:SA1	Cc:SA2, CT4, RAN2, RAN3

### 6.1.3	Control Plane corrections

Slot aggregation

R2-2501716	LS on Slot aggregation configuration with multi-PUSCH (R1-2501593; contact: Nokia)	RAN1	LS in	Rel-17	NR_ext_to_71GHz-Core	To:RAN2

Moved from 6.1.1

R2-2502920	Slot aggregation configuration with multi-PUSCH	Nokia	CR	Rel-17	38.331	17.12.0	5328	-	F	NR_ext_to_71GHz-Core

R2-2502921	Slot aggregation configuration with multi-PUSCH	Nokia	CR	Rel-18	38.331	18.5.1	5329	-	A	NR_ext_to_71GHz-Core

-	Nokia highlights that this is NBC.

Both above are in-principle agreed, but in the next version state on the cover page that UEs and NW supporting this feature must implement this CR.

#### 6.1.3.1	NR RRC

Corrections to 38331, and related change to other TS if applicable, except UE caps.

msg3-transmitPrecoder

R2-2502049	CR on the usage of msg3-transmitPrecoder	CATT, Huawei, HiSilicon	CR	Rel-17	38.331	17.12.0	5286	-	F	NR_SmallData_INACTIVE-Core, NR_cov_enh-Core, NR_redcap-Core, NR_slice-Core

R2-2502050	CR on the usage of msg3-transmitPrecoder	CATT, Huawei, HiSilicon	CR	Rel-18	38.331	18.5.1	5287	-	A	NR_SmallData_INACTIVE-Core, NR_cov_enh-Core, NR_redcap-Core, NR_slice-Core

R2-2502051	CR on the usage of RACH parameters in the case of fallbackRAR	CATT, Huawei, HiSilicon	CR	Rel-17	38.321	17.12.0	2055	-	F	NR_SmallData_INACTIVE-Core, NR_cov_enh-Core, NR_redcap-Core, NR_slice-Core

R2-2502052	CR on the usage of RACH parameters in the case of fallbackRAR	CATT, Huawei, HiSilicon	CR	Rel-18	38.321	18.5.0	2056	-	A	NR_SmallData_INACTIVE-Core, NR_cov_enh-Core, NR_redcap-Core, NR_slice-Core

-	Ericsson think there is no need to change the spec as we already told RAN1 in our LS to them, and then RAN1 said that they wont fix it either. CATT think RAN1 said they wont update their specs but we never said we wont update our spec. Xiaomi think we need these CRs, but the related MAC CRs are not needed. ZTE, LG and vivo agrees with Xiaomi.

-	Ericsson think the CR is NBC and the RRC CR says “if the field is not configured” and in other parts “if the field is absent”. Huawei think it makes sense in this case.

-	ZTE think the RRC spec already has similar wording covering this. Huawei want to have explicit wording on this particular field.

MAC CR is not pursued, the RRC CR can be revised offline.

[AT129bis][604][Maint] Usage of msg3-transmitPrecoder (CATT)

Scope:

    - Produce agreeable CRs

Intended outcome:

    - In principal agreed CRs in R2-2503112 and R2-2503113

Deadline:

    - Wednesday 17:00. The intention is to agree over email.

R2-2503112	CR on the usage of msg3-transmitPrecoder	CATT, Huawei, HiSilicon	CR	Rel-17	38.331	17.12.0	5286	1	F	NR_SmallData_INACTIVE-Core, NR_cov_enh-Core, NR_redcap-Core, NR_slice-Core

R2-2503113	CR on the usage of msg3-transmitPrecoder	CATT, Huawei, HiSilicon	CR	Rel-18	38.331	18.5.1	5287	1	A	NR_SmallData_INACTIVE-Core, NR_cov_enh-Core, NR_redcap-Core, NR_slice-Core

Both in-principle agreed

RedCap

R2-2502335	Correction on SCS and CP configuration in RedCap-specific initial BWP	ZTE Corporation, Sanechips, Vivo, Ericsson, MediaTek Inc, Qualcomm Incorporated, OPPO, Samsung	CR	Rel-17	38.331	17.12.0	5212	1	F	NR_redcap-Core	R2-2500439

R2-2502336	Correction on SCS and CP configuration in RedCap-specific initial BWP	ZTE Corporation, Sanechips, Vivo, Ericsson, MediaTek Inc, Qualcomm Incorporated, OPPO, Samsung	CR	Rel-18	38.331	18.5.1	5213	1	A	NR_redcap-Core	R2-2500440

-	Huawei and Nokia think RAN1 specs are already clear. ZTE said this was discussed offline but think there is a difference in RAN1 specs vs. this CR. Huawei need more time.

[AT129bis][605][Maint] SCS and CP configuration in RedCap-specific initial BWP (ZTE)

Scope:

    - Discuss offline to see if R2-2502335 and R2-2502336 should be in-principle agreed

Intended outcome:

    - Consensus of how to proceed with R2-2502335 and R2-2502336

Deadline:

    - Wednesday 17:00. The intention is to agree over email.

-	After the offline, ZTE reports that the original CRs are agreeable.

R2-2502335 and R2-2502336 are in-principle agreed

MIMO

R2-2502415	Correction on PUCCH spatial relation Activation/Deactivation for multiple TRP PUCCH repetition MAC CE	ZTE Corporation, Samsung	CR	Rel-17	38.321	17.12.0	2063	-	F	NR_FeMIMO-Core

R2-2502416	Correction on PUCCH spatial relation Activation/Deactivation for multiple TRP PUCCH repetition MAC CE	ZTE Corporation, Samsung	CR	Rel-18	38.321	18.5.0	2064	-	A	NR_FeMIMO-Core

-	Nokia think we should postpone.

Postponed

Preamble partitioning

R2-2502754	Correction on rsrp-ThresholdSSB-r17 in TS 38.331(R17)	Huawei, HiSilicon	CR	Rel-17	38.331	17.12.0	5316	-	F	NR_redcap-Core, NR_cov_enh-Core, NR_slice-Core, NR_SmallData_INACTIVE-Core

R2-2502755	Correction on rsrp-ThresholdSSB-r17 in TS 38.331(R18)	Huawei, HiSilicon	CR	Rel-18	38.331	18.5.1	5317	-	A	NR_redcap-Core, NR_cov_enh-Core, NR_slice-Core, NR_SmallData_INACTIVE-Core

-	Xiaomi think this is not needed. LG think its good to clarify this but think that this may be NBC.

-	Ericsson think there are other parameters where the same behaviour applies for so wonders why we have to clarify for this particular parameter?

-	MediaTek is not sure if this CR is correct. ZTE think this CR is not correct.

[AT129bis][606][Maint] Correction on rsrp-ThresholdSSB (Huawei)

Scope:

    - Discuss offline to see if/how the spec should be updated (refer to R2-2502754 and R2-2502755)

Intended outcome:

    - Consensus of how to proceed with R2-2502754 and R2-2502755, revisions in R2-2503114 and R2-2503115 if needed

Deadline:

    - Wednesday 17:00. The intention is to agree over email.

-	Huawei reports after the offline that it is better to postpone the issue to a later meeting.

Postponed

SCG activation

R2-2502940	Correction on CFRA for SCG activation	Huawei, HiSilicon	CR	Rel-17	38.321	17.12.0	2070	-	F	LTE_NR_DC_enh2-Core

R2-2502941	Correction on CFRA for SCG activation	Huawei, HiSilicon	CR	Rel-18	38.321	18.5.0	2071	-	A	LTE_NR_DC_enh2-Core

-	Huawei reports that these were treated in the main session

Misc

R2-2502573	Miscellaneous non-controversial corrections Set XXV	Ericsson	CR	Rel-17	38.331	17.12.0	5309	-	F	NR_newRAT-Core, TEI17

[Post129bis][601][Maint] Miscellaneous non-controversial corrections (Ericsson)

Scope:

    - Produce agreeable CRs

Intended outcome:

    - In principle agreed CRs

Deadline:

    - Short

NTN – to be handled in Sergios session

R2-2502249	Correction on carrier frequency information in RRCConnectionRelease.	CATT, Qualcomm Incorporated	CR	Rel-17	36.331	17.12.0	5106	-	F	NR_NTN_solutions-Core

R2-2502250	Correction on carrier frequency information in RRCConnectionRelease.	CATT, Qualcomm Incorporated	CR	Rel-18	36.331	18.5.0	5107	-	A	NR_NTN_solutions-Core

R2-2502519	Corrections to location-based measurement	ZTE Corporation, Ericsson, CATT, Sanechips	CR	Rel-17	38.331	17.12.0	5303	-	F	NR_NTN_solutions-Core

#### 6.1.3.2	UE capabilities

UE cap corrections 38306, 38331.

Paging Capability

R2-2502172	Discussion on capability mis-match issue for LP-WUS, PEI, (e)RedCap, 2Rx XR, NTN, inactiveStatePO-Determination	vivo	discussion	Rel-19	NR_LPWUS-Core, NR_UE_pow_sav_enh-Core, NR_redcap-Core, NR_redcap_enh-Core

Moved from 8.19

Proposal 1: It is up to network implementation to address the paging capability mis-match issue between the gNB and the UE for PEI/LP-WUS/inactiveStatePO-Determination-r17, e.g., all legacy gNBs are upgraded to support the latest ASN.1 for the UE capability on paging. No specification impact is foreseen.

R2-2501884	Discussion on Paging Capability Issue	OPPO	discussion	Rel-17	NR_newRAT-Core, TEI17

Proposal 1	R2 decide whether to rely on network implementation (no specified solution), or specified solution to solve the issues (for paging capability before R17, e.g, inactiveStatePO-Determination-r17).

Proposal 2	If R2 decides to adopt the specified solution, UE trigger a MRU (Mobility Registration Update) to update paging capability (if has not done since last NAS registration) upon camping on a cell indicating ranPagingInIdlePO.

R2-2502140	Discussion on UE Radio Capability for Paging Information	CATT	discussion	Rel-17	NR_UE_pow_sav_enh-Core, NR_redcap-Core, NR_NTN_enh-Core, NR_redcap_enh-Core, NR_XR_enh-Core

Moved from 6.1.3.1

Proposal 1: There are paging problems for PEI, LP-WUS and inactiveStatePO-Determination when Initial Registration is performed on a pre-R17 gNB.

Proposal 2: RAN2 discuss which option is adopted for the lost issue which leads paging problems.

-	Option 1: Up to network implementation to avoid lost issue.

-	Option 2: An indicator is sent to UE to indicate R17 related paging info has been sent to CN if UE capability transfer procedure (gNB->AMF) is completed and UE Radio Capability for Paging in the gNB includes R17 related paging info.

R2-2501953	Discussion on UE capabilities to be included in UE Radio Paging Information	ZTE Corporation, Sanechips	discussion	Rel-19

Moved from 8.19

Proposal 1: When gNB is upgraded to a certain release, it is able to decode the UE capabilities in the extended field for this release and include the corresponding UE capabilities in UE Radio Paging Information.

Proposal 2: For those capabilities not decoded and included in UE Radio Paging Information due to the release supported at source gNB side, it is up to the target gNB to decide whether to re-acquire UE capabilities after the UE Radio Paging Information is downloaded from CN or transferred from the source gNB. No spec impact is needed.

Proposal 3: Reply to SA2 with RAN3 and CT1 in CC as in the Annex with the above understanding shared.

R2-2502472	Discussion on the issue of UE radio paging capability loss	Huawei, HiSilicon	discussion	Rel-17	NR_newRAT-Core

Moved from 6.1.3.1

Proposal 1: RAN2 to confirm that UE will miss paging when camps in the cell with LP-WUS/PEI enabled, if the UE reports UE-RadioPagingInfo-r17 to a Rel-15/16 gNB that cannot forward this field to CN.

Proposal 2: RAN2 to confirm that RRC_INACTIVE UE will miss paging when camps in the cell with ranPagingInIdlePO-r17 indicated, if the UE reports inactiveStatePO-Determination-r17 to a Rel-15/16 gNB that cannot forward this field to CN.

Proposal 3: RAN2 to confirm that:

	For (e)RedCap, UE radio paging capability loss issue does not exist;

	For 2 Rx XR and DL scheduling offset for NTN, UE will not miss any paging but it’s not the best use of paging resources from NW perspective.

Proposal 4: RAN2 to discuss the following solutions:

	Solution 1: Up to deployment to upgrade the gNBs to support corresponding ASN.1 simultaneously

	Solution 2: UE gets information from the gNB whether corresponding capability can be maintained by the NW

All above noted. As agreed in the main session we will not change legacy UE capabilities and instead rely on network implementation

ul-GapFR2

R2-2502837	Correction to ul-GapFR2-r17	Huawei, HiSilicon, Apple, Nokia, Nokia Shanghai Bell, Ericsson, MediaTek Inc.	CR	Rel-17	38.306	17.12.0	1265	-	F	NR_RF_FR2_req_enh2

R2-2502838	Correction to ul-GapFR2-r17	Huawei, HiSilicon, Apple, Nokia, Nokia Shanghai Bell, Ericsson, MediaTek Inc.	CR	Rel-18	38.306	18.5.0	1266	-	A	NR_RF_FR2_req_enh2

Both are in-principal agreed

NTN – to be handled in Sergios session

R2-2502520	Corrections to location-based measurement initiation	ZTE Corporation, Ericsson, CATT, Sanechips	CR	Rel-17	38.306	17.12.0	1255	-	F	NR_NTN_solutions-Core

R2-2502521	Corrections to location-based measurement initiation	ZTE Corporation, Ericsson, CATT, Sanechips	CR	Rel-18	38.306	18.5.0	1256	-	A	NR_NTN_solutions-Core

#### 6.1.3.3	Other

Including idle and inactive behaviour specified in 38.304 or 36.304.

Emergency call back and paging

R2-2501752	Reply LS on emergency call back and paging (S2-2502427; contact: ZTE)	SA2	LS in	Rel-17	NR_newRAT-Core, NR_redcap-Core	To:RAN2	Cc:CT1, RAN3

Moved from 6.1.1

-	ZTE reports that RAN3 has agreed a stage-2 CR on this topic, and that CR is similar to what ZTE is proposing in R2-2502337. Nokia think that we may need to update UE behaviour. Samsung has the same understanding as ZTE, that the gNB should not release to INACTIVE with eDRX and Samsung wonders what UE behaviour is needed. Nokia think that the inactivity timer may cause the gNB to release the UE to INACTIVE. Ericsson think that the gNB should not use the inactivity timer for these scenarios. Qualcomm think NW can solve everything and its not reasonable to update UE behaviour, e.g. not release the UE. Huawei is not sure what is the status in other WGs yet, and suggest that we wait for LSs from RAN3. Ericsson think that in LTE there is already a solution, i.e. not release the UE to IDLE/INACTIVE with eDRX.

Noted

R2-2502337	On eDRX operation with emergency PDU session	ZTE Corporation, Sanechips	discussion	Rel-17	NR_newRAT-Core, NR_redcap-Core

Proposal 1: RAN2 to confirm that no new signaling from CN to RAN is needed to indicate the presence of emergency PDU session.

Proposal 2: RAN2 confirm that when UE is configured with emergency PDU session, the UE is not expected to be released to RRC_INACTIVE with eDRX configuration for RAN paging (to be captured in stage 2 spec).

Proposal 3: Send reply LS to SA2 about the decisions made in RAN2.

R2-2502504	Discussion on emergency services and eDRX	Nokia, Huawei, HiSilicon	discussion	Rel-17	NR_redcap-Core

Proposal 1:	RAN2 confirms and replies SA2 that the RAN node would release the UE to RRC_IDLE or RRC_INACTIVE state from RRC_CONNECTED when the UE has an emergency PDU session.

Proposal 2:   RAN2 should clarify the UE behaviour that eDRX should not be used when the UE has an emergency PDU session, to align with NAS specification.

Postponed

R2-2502338	Correction on eDRX configuration in RRC_INACTIVE	ZTE Corporation, Sanechips	CR	Rel-17	38.300	17.12.0	0979	-	F	NR_newRAT-Core, NR_redcap-Core

R2-2502339	Correction on eDRX configuration in RRC_INACTIVE	ZTE Corporation, Sanechips	CR	Rel-18	38.300	18.5.0	0980	-	A	NR_newRAT-Core, NR_redcap-Core

R2-2502962	(e)DRX usage clarification for UEs with emergency PDU Sessions	Nokia, Huawei, HiSilicon	CR	Rel-17	38.304	17.10.0	0433	-	F	NR_redcap-Core

R2-2502963	(e)DRX usage clarification for UEs with emergency PDU Sessions	Nokia, Huawei, HiSilicon	CR	Rel-18	38.304	18.4.0	0434	-	A	NR_redcap-Core

R2-2501975	On emergency services and eDRX	MediaTek Inc.	discussion	Rel-17

Proposal 1	RAN2 to agree the RAN node can be aware of an ongoing emergency PDU session by network implementation.

Proposal 2	To support keeping the UE with emergency PDU session to be in RRC_CONNECTED state, RAN2 to agree the network-based solution will be considered (No specs change is needed).

Proposal 3	RAN2 to agree on capturing that the UE shall not use eDRX if the UE has an emergency PDU session in RRC_INACTIVE state in TS 38.304.¨

R2-2502909	Emergency callback and paging	Ericsson	discussion

Proposal 1	RAN2 captures in a NOTE in 38.300 that using the ARP value reserved for emergency service the gNB can delay connection release to reduce the latency of a potential emergency callback.

Proposal 2	RAN2 informs SA2 and RAN3 about the RAN2 agreement.

Old revisions/withdrawn

R2-2502955	(e)DRX usage clarification for UEs with emergency PDU Sessions	Nokia, Huawei, HiSilicon	CR	Rel-17	38.804	14.0.0	0001	-	F	NR_redcap-Core

=> Withdrawn

R2-2502956	(e)DRX usage clarification for UEs with emergency PDU Sessions	Nokia, Huawei, HiSilicon	CR	Rel-18	38.804	14.0.0	0002	-	A	NR_redcap-Core

=> Withdrawn

# 7	Rel-18

## 7.0	Common

Rel-18 WIs not covered under an explicit AI in 7.x.  Multi-WI Rel-18 items, e.g. cross-WI-issues not handled under another WI. UE capabilities.

#### 7.0.2.11	Further enhancement of data collection for SON MDT in NR and EN-DC

(NR_ENDC_SON_MDT_enh2-Core; leading WG: RAN3; REL-18; WID: RP-221825)

SPR configuration

R2-2501909	Corrections on SPR configuration	CATT	CR	Rel-18	38.331	18.5.1	5283	-	F	NR_ENDC_SON_MDT_enh2-Core

-	Samsung think there are missing colons. Ericsson is the existing text is OK but we only need adding a clause mentioning the target config, further “configured by the target” wording is confusing and could be fixed.

Postponed

References

R2-2502407	Updating references to TS 28.558 from TS 28.552	Nokia	CR	Rel-18	37.320	18.3.0	0140	-	F	NR_ENDC_SON_MDT_enh2-Core

In-principle agreed

timeSinceSHR

R2-2502530	Correction on timeSinceSHR in SHR report	ZTE Corporation, Sanechips	CR	Rel-18	38.331	18.5.1	5304	-	F	NR_ENDC_SON_MDT_enh2-Core

In-principle agreed

RACH-less

R2-2502645	Correction on ra-InformationCommon logging in RLF-Report SHR and SPR	Ericsson	CR	Rel-18	38.331	18.5.1	5312	-	F	NR_ENDC_SON_MDT_enh2-Core

-	ZTE agrees with the intention but want to do it in R19. Ericsson and QC think we should do it in R18. Huawei support the CR. Samsung is OK with the intention but want more time.

Postponed

NPN

R2-2502798	Correction on SON for NPN in TS 38.300	Huawei, HiSilicon, CATT, Samsung, ZTE, CMCC	CR	Rel-18	38.300	18.5.0	0986	-	F	NR_ENDC_SON_MDT_enh2-Core

In-principle agreed

Intended SIBs

R2-2502844	Correction on logging intendedSIBs	Samsung	CR	Rel-18	38.331	18.5.1	5322	-	F	NR_ENDC_SON_MDT_enh2-Core

-	Ericsson think this is not strictly needed. Qualcomm is OK with this CR.

In-principle agreed

# 8	Rel-19

## 8.10	SON/MDT Ph4

(NR_ENDC_SON_MDT_Ph4-Core; leading WG: RAN3; REL-19; WID: RP-234038)

Time budget: 0.5 TU

Tdoc Limitation: 4 tdocs

### 8.10.1	Organizational

LS, Rapporteur input, including workplan, etc.

Capabilities

R2-2501912	Views on R19 SONMDT UE capabilities	CATT	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1: an optional UE capability with signalling is defined for RLF report for MCG LTM.

Proposal 2: an optional UE capability without signalling is defined for SHR for MCG LTM.

Proposal 3: an optional UE capability with signalling is defined for RLF report for CHO with candidate SCG.

Proposal 4: an optional UE capability without signalling is defined for SHR/SCGFailureInformation for CHO with candidate SCG.

Proposal 5: an optional UE capability without signalling is defined for RLF report for time/location based CHO.

Proposal 6: an optional UE capability without signalling is defined for RA-report for SDT.

Proposal 7: an optional UE capability without signalling is defined for MHI Enhancement for SCG Deactivation/Activation.

Proposal 8: an optional UE capability without signalling is defined for MRO for EN-DC SCG failure in LTE specification.

Noted, we will discuss capabilities in a later meeting.

Running CRs

R2-2501913	Introduction of SONMDT UE Capabilities	CATT	draftCR	Rel-19	38.331	18.5.1	B	NR_ENDC_SON_MDT_Ph4-Core

R2-2501914	Introduction of SONMDT UE Capabilities	CATT	draftCR	Rel-19	38.306	18.5.0	B	NR_ENDC_SON_MDT_Ph4-Core

R2-2501915	Introduction of SONMDT UE Capabilities	CATT	draftCR	Rel-19	36.306	18.4.0	B	NR_ENDC_SON_MDT_Ph4-Core

R2-2502649	Running CR for SONMDT features	Ericsson	draftCR	Rel-19	38.331	18.5.1	B	NR_ENDC_SON_MDT_Ph4-Core

R2-2502787	Running 36.331 CR for R19 SONMDT	Huawei, HiSilicon	draftCR	Rel-19	36.331	18.5.0	B	NR_ENDC_SON_MDT_Ph4-Core

The above are endorsed

[Post129bis][603][SONMDT] Running NR RRC CR for SONMDT (Ericsson)

Scope:

    - Update the running CRs based on the progress in the meeting

Intended outcome:

    - Updated running CR to be submitted to next meeting

Deadline:

    - Long

[Post129bis][604][SONMDT] Running LTE RRC CR for SONMDT (Huawei)

Scope:

    - Update the running CRs based on the progress in the meeting

Intended outcome:

    - Updated running CR to be submitted to next meeting

Deadline:

    - Long

R2-2502788	Open issue list for running 36.331 CR for R19 SONMDT	Huawei, HiSilicon	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Noted

Proposal 1: For ASN.1 definitions in TS 36.331, it is proposed to introduce the perRAInfoListNR IE, and it includes PerRAInfoList-r16, PerRAInfoList-v1660 and PerRAInfoList-v1800 (which are specified in TS 38.331).

Proposal 2: It is proposed to keep the condition for UE setting timeSCGFailure and previousPSCellId, i.e. 3>	if the last RRCReconfiguration message including the reconfigurationWithSync for the SCG was received to enter the PSCell in which the SCG failure was declared.

On P2:

-	Samsung wonders if it covers PSCell change and or PSCell addition? Huawei is OK to leave this as FFS.

-	Nokia wonders if we can instead have a container which carries all the fields (PerRAInfoList-r16, PerRAInfoList-v1660 and PerRAInfoList-v1800)? Samsung think we can use a container (OCTET STRING) and then in the field description we clarify the content.

Agreements For ASN.1 definitions in TS 36.331, it is proposed to introduce the perRAInfoListNR IE, and it includes PerRAInfoList-r16, PerRAInfoList-v1660 and PerRAInfoList-v1800 (which are specified in TS 38.331).

R2-2502974	Discussion on Rel-19 SONMDT open issues	Xiaomi	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Noted

[AT129bis][601][SONMDT] Address open issues for RRC (Ericsson)

Scope:

    - Discuss offline how to proceed with the proposals in R2-2502974 and R2-2502927

Intended outcome:

    - Report in R2-2503101 with agreeable proposals which later can be used to progress running CRs.

Deadline:

    - Wednesday 17:00. The intention is to agree over email.

R2-2503101	[AT129bis][601][SONMDT] Address open issues for RRC	Ericsson

Noted

Agreements - For CHO with candidate SCGs, RAN2 to reuse the current stage-3 description, i.e. include candidate PCell and candidate PSCell measurements results in neighbor measurements within RLF reports (or SCGFailureInformation). - For CHO with candidate SCGs, RAN2 to clarify that the time duration between two fulfilled events (i.e., timeBetweenFulfillmen) shall only be included when both CHO and CPAC conditions are satisfied for failure cases. - For CHO with candidate SCGs, RAN2 explicitly define a new lastHO-Type for CHO with candidate SCGs. - For CHO with candidate SCGs, logging of elapsed time between fulfilling the last triggering event and handover execution in SHR is not required when only one condition (CHO or CPAC) is fulfilled. - For CHO with candidate SCGs, RAN2 to agree to include “firstFulfilledConfig”, “timeBetweenFulfillment”, and “timeBetweenLastFulfillmentAndEvent” in the choWithCandidateSCGInfoList IE within the SCGFailureInformation message. Keep the current ASN.1 structure and no change are needed. - Network is informed in the next RRC complete message about the SHR availability in case the UE performs RACH-less LTM cell switch. No need to change the current formulation of determining the SHR in the current running CR. - Follow CHO like mechanism (i.e., a single rlf-Report for the RLF/HOF, including LTM recovery cell ID) for a consecutive LTM cell switch failure, i.e., an RLF/HOF at source or target cell followed by an HOF during LTM recovery. Relevant FFS can be removed from the running CR. No change is needed in the relevant text of running CR. - FFS whether it is needed to avoid duplication of information in case of two reports being generated CHO with candidate SCGs, any redundancy (e.g., measurements) are recorded in the reports for PCell (i.e., in SHR, SPR).

### 8.10.2	MRO enhancements for Rel-18 mobility features

LTM has 1st priority. CHO with candidate SCGs has 2nd priority

Subsequent CPAC is paused until if/when we get a RAN3 LS on the subject

R2-2502927	Remaining issues for MRO	ZTE Corporation, Sanechips	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

#### 8.10.2.1	LTM

R2-2502707	MRO enhancements for LTM	CMCC	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1: Include the unnecessary LTM configuration, e.g. LTM candidate cell(s) unused, in SON reports, for both failure and near failure cases.

Proposal 2: Include the following UE-based TA related information in RLF report:

－	Difference between the TA value received from the LTM cell switch command and TA value measured by the UE

－	Elapsed time between UE obtains the target cell TA and UE receives the cell switch command, or an indication to indicate whether the UE obtains TA before UE receives LTM cell switch command

－	Time between the PDCCH ordered TA acquisition and the reception of the LTM Cell Switch Command MAC CE that triggered the execution of the failed LTM cell switch

On P1:

-	vivo think the NW can have this info based on config and UE history. Samsung agrees with vivo, and this is not about failure and is out of scope. Nokia supports this proposal since it may be useful for the NW. Ericsson wonders how the NW based solution should work, does the NW have to remember? Vivo think its based on config and UE history. Xiao think this is not needed.

On P2:

-	vivo think this is not needed since if the UE got the LTM command the UE got a TA value. Samsung agrees with vivo about the first bullet and the second and third bullet the NW can know. Xiaomi think we discussed third bullet this already and agreed not to do it. NEC agrees with first bullet. Huawei supports the third bullet. ZTE want all bullets of P2.

R2-2502789	Discussion on MRO for LTM	Huawei, HiSilicon	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Focus on P2

Proposal 1: For RACH-less access based on network TA, include the following information in RLF report:

－	Time period between the reception of the PDCCH order and the reception of HO command MAC CE related to the failed LTM cell switch

Proposal 2: It is proposed RAN2 to agree to enhance the RLF report with the beam information contained in the LTM cell switch command.

On P2:

-	CATT agrees with P2. Ericsson prefers to have the beam info for the case when the UE recovered. vivo think we already indicated to RAN3 that they should solve this. LG think the previous discussion was for SHR, LG supports the proposal. Apple think we agreed the opposite earlier. Qualcomm think we discussed this already and sent to RAN3 that they should solve it. Samsung think that the target has this info. Ericsson want to log the beam info when doing recovery. Samsung think also for recovery its known.

R2-2502826	Discussion on random access report for LTM	ASUSTeK	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core	R2-2500419

Focus on P2-P5

Proposal 1:	RA reports for LTM shall include the following cases:

-	RACH for Early UL synchronization on a candidate cell.

-	NW-triggered RACH-based LTM towards a target cell.

-	UE-triggered RACH-based LTM towards a selected Cell after failed LTM execution.

Proposal 2:	Discuss whether to reuse current values of raPurpose or introduce new values of raPurpose for LTM-related RA reports.

Proposal 3:	For LTM-related RA reports, the UE sets the cellId based on ltm-CandidatePCI and ssb-Frequency in corresponding LTM-Candidate.

Proposal 4:	Discuss whether to set values of spCellID for LTM-related RA reports.

Proposal 5:      A single RA report is used to indicate random access information for all early UL synchronization on the same candidate cell.

On P2:

-	Ofinno want a new codepoint. Vivo want reuse reconfiWithSync.

On P5:

-	vivo think no reporting of early UL sync info is needed. ZTE agrees.

Agreements - We will not add support for including the unnecessary LTM configuration, e.g. LTM candidate cell(s) unused, in SON reports, for both failure and near failure cases. - Do not specify support for reporting difference between the TA value received from the LTM cell switch command and TA value measured by the UE - Do not specify support for reporting of elapsed time between UE obtains the target cell TA and UE receives the cell switch command, or an indication to indicate whether the UE obtains TA before UE receives LTM cell switch command - For LTM-related RA reports, the UE sets the cellId to the CGI if available, otherwise sets it based on ltm-CandidatePCI and ssb-Frequency in corresponding LTM-Candidate.

R2-2501916	MRO Enhancements for LTM	CATT	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1: MRO enhancement for SCG LTM is not considered in Rel-19 SON/MDT.

Enhance RLF report for LTM MRO

Proposal 2: Include TCI state ID from LTM cell switch command in RLF report for RACH-less LTM failure.

Proposal 3: The legacy SHR trigger conditions based on T310 threshold and T304 threshold can be applied to LTM.

Proposal 4: Include L3 measurement results of serving cell, target cell and other LTM candidate cell(s) in SHR for LTM MRO.

Proposal 5: Enhance raPurpose in RACH report to support successful RACH-based LTM execution. The detail can be discussed further, e.g., whether to reuse legacy raPurpose or introduce a new raPurpose.

Proposal 6: A new raPurpose in RACH report is introduced to indicate the network-based early TA acquisition.

Proposal 7: RAN2 to discuss how to log the RACH information of network-based early TA acquisition to avoid the impact on the legacy RACH information logging (e.g., RACH information for handover):

-	Option 1: one entry in RACH report to log the RACH information of all network-based early TA acquisition before one LTM execution;

-	Option 2: one entry in RACH report to log the RACH information of all network-based early TA acquisition before LTM configuration release;

-	Option 3: log the RACH information of network-based early TA acquisition per candidate cell in one or multiple entries.

Other issues

Proposal 8: To address the issue of frequent overriding RLF report/SHR for subsequent LTM, recording multiple reports can be considered as a candidate solution.

Proposal 9: Indication information is needed in RLF report/SHR to indicate whether it’s L1 measurement based LTM or L3 measurement based LTM.

R2-2502288	MRO enhancement for LTM	NEC	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

R2-2502347	Discussion on MRO enhancements for LTM	Lenovo	discussion	Rel-19

Proposal 1: The UE can report L1 measurement results during LTM procedure, and a new RA purpose to indicate that the random access procedure was triggered by a PDCCH order for early TA acquisition, in the RA report.

R2-2502408	MRO for LTM and near failure handling for CHO with candidate SCGs	Nokia	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1: Log additional information regarding TAs that were unnecessarily acquisitioned.

Proposal 2: RAN2 to discuss adding the following information into RLF report: TA difference between TA provided by network in Cell Switch Command for which the RACH-less LTM failed, and TA received in RAR for recovery/re-establishment.

Proposal 3: RAN2 to discuss signalling that the UE indicates that the use of the UE measurement-based TA is preferred.

Proposal 4: RAN2 to enhance SHR with new information and triggering criteria that allow the network to identify unnecessary LTM configurations.

Observation 2: The ACK for first UL transmission taking longer than expected during an LTM HO represents an undesired behaviour.

Proposal 5: RAN2 to define a threshold related to the number of RLC retransmissions of the first UL transmission in RLC AM for RACH-less LTM as a new SHR trigger.

Proposal 6: RAN2 to agree to also log near failure information for the secondary link in case of CHO with candidate SCG.

Proposal 7: RAN2 to agree to extend SPR with the same information agreed for SHR for CHO with candidate SCGs.

Proposal 8: RAN2 to discuss and decide on a correlation mechanism for SHR and SPR generated for the same CHO with candidate SCG execution, for example by introducing a coexistence flag in each report.

Proposal 9: RAN2 to agree on adding Target Pscell ID in SHR and Target Pcell ID in SPR to identify the cell pairs linked to an execution of CHO with candidate SCG.

Proposal 10: RAN2 to agree that there is a need to distinguish if the T310 started before or after fulfilment of the first execution conditions for CHO with Candidate SCG(s).

R2-2502450	SON and MDT for LTM	Qualcomm Incorporated	discussion	NR_ENDC_SON_MDT_Ph4-Core

R2-2502531	Discussion on MRO enhancements 	Samsung	discussion

Proposal 1: Define separate UE capability parameters for RLF report and SHR for LTM cell switch.

Proposal 2: No special handling is needed for reporting successHO-InfoAvailable in RACH less LTM cell switch.

Proposal 3: L1 measurements for LTM are logged only for the candidates for which LTM-SSB-Config-r18 is configured (i.e. only if configured for LTM based on L1 measurements).

Proposal 4: lastHO-Type is not extended with “CHO with Candidate SCG”.

R2-2502604	Discussion on MRO for LTM	Ofinno, LLC	discussion	Rel-18	NR_ENDC_SON_MDT_Ph4-Core

Measurement information in MRO

Proposal 1: Indicate in RLF report/ SHR report whether the LTM to the target cell is based on L3 measurement or L1 measurement.

Proposal 2: Indicate in RLF report/ SHR report whether the neighbor cell (i.e., LTM candidate cells) measurements is based on L3 measurement or L1 measurement.

RLF Report

Proposal 3: Indicate in RLF-report whether the UE received TA from cell switch command for the candidate cell.

Proposal 4: Introduce UE capability for indicating whether UE supports RLF report for LTM.

SHR Report

Proposal 5: Indicate whether RACH-less access is based on CG or DG based on one of:

-	Explicit indication whether RACH-less access is based on CG or DG

-	Explicit indication whether CG for RAHC-less access is valid or invalid

Proposal 6: Indicate index of SSB where the CG is determined to be valid/ invalid.

Proposal 7: Indicate whether a neighbour cell is an LTM candidate cell or not. Whether the indication is using explicit or implicit indication can be decided during stage-3 implementation.

TA information

Proposal 8: A UE logs information of whether early UL synchronization is performed for a neighbor cell (LTM candidate cell).

2.3 RA Report

Proposal 9: A UE logs in RA report, that the access (RACH-based/ RACH-less) is for LTM cell switch.

Proposal 10: In RA report, indicate whether TA of candidate cell was acquired for target cell for LTM cell switch, based on one of:

-	Indicate whether RACH-based or RACH-less access

-	Indicate TA availability

Proposal 10: A UE logs RA information for early uplink synchronization in raReport.

Proposal 11: Introduce new raPurpose or early uplink synchronization.

Proposal 12: A UE logs multiple early UL synchronization attempts to a candidate cell using a single entry in raReport. UE logs the number if early UL synchronization attempts(i.e., RACH attempts) made to the candidate cell. UE logs RA information corresponding to the last attempt.

Proposal 13: A UE logs the time elapsed between receiving PDCCH order for triggering RACH (for early UL synchronization) on a candidate cell, and receiving LTM cell switch command MAC CE for cell switch to the same candidate cell.

Proposal 14: RAN2 to discuss whether the first PDCCH order or the last PDCCH order for early UL synchronization on the candidate cell is considered for logging the time elapsed.

R2-2502646	Further considerations on Mobility Robustness Optimization	Ericsson	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1	Enhance the RLF report with beam information of.

a.	the beam toward which the UE failed to perform LTM cell switch,

b.	The beam indicated in the last successful LTM cell switch command, when failure occurs shortly after the LTM cell switch,

c.	The beam the UE succeeded to recover/re-establish or reconnect in.

Proposal 2	In case of RLF shortly after a successful LTM cell switch, the UE logs whether the UE has switched the beam by the target cell prior to the RLF. This enables the network to correctly classify Too Early HO failure from failure due to wrong beam configuration at the target cell.

Proposal 3	The UE logs and reports the time between the PDCCH ordered TA acquisition and the reception of the LTM Cell Switch Command MAC CE that triggered the execution of the failed LTM cell switch. For the UE-based TA estimation, there is no need to log this information.

Proposal 4	The UE logs the difference between a TA received in a RAR (or a MsgB) in the target cell and the TA received in the LTM Cell Switch Command MAC CE in the RLF report.

Proposal 5	UE logs available L1 measurement results for the serving cell, the target cell and other LTM candidate cells when the UE is LTM configured.

R2-2502806	MRO for LTM	LG Electronics Inc.	discussion	Rel-19	R2-2501212

Proposal 1:	In the RLF report for MCG LTM, RAN2 should revisit and decide whether the UE should explicitly log the access type.

Proposal 2:	In the RLF report for MCG LTM, the UE logs not only the access type but also the commanded access type which was indicated LTM CSC if the access type is different from the commanded access type.

Proposal 3:	In the SHR for MCG LTM, ltmCandidate is explicitly defined as the same enum type as choCandidate.

Proposal 4:	In the SHR for MCG LTM, RAN2 discuss that the UE log candidate cells not only for those candidate cells unused but also for those candidate cells selected yet not requiring actual cell switching.

R2-2502858	RA information enhancement for LTM	SHARP Corporation	discussion

Proposal 1: UE does not log RA information for early UL sync in RA report.

Proposal 2: multiple successive RA procedures for the same LTM candidate cell can be included in a single RA report.

Proposal 3: a new raPurpose in RA report can be used to differentiate the RA is for LTM early UL sync.

Proposal 4: PCell identity is included in RA report for LTM early UL sync on a LTM candidate cell.

Proposal 5: no enhancement is needed for RA report in case of RACH-based LTM cell switch.

#### 8.10.2.2	CHO with candidate SCGs

R2-2502861	SON enhancement for CHO with candidate SCG	SHARP Corporation	discussion

Proposal 1: new triggering conditions, e.g. time gap between the first met condition (CHO or CPAC) and the second met condition (CPAC or CHO) is above a threshold, can be considered for SHR/SPR procedure.

Proposal 2: Consider new triggering conditions for SHR/SPR procedure: 1) CPAC execution condition is fulfilled late than CHO execution condition can be considered; 2) CPAC execution condition is fulfilled early than CHO execution condition; but the time gap between the CHO condition fulfillment and CPAC condition fulfillment is above a threshold.

Proposal 3: UE includes the target PSCell ID in SHR for successful CHO with candidate SCGs.

Proposal 4: UE includes the target PCell ID in SPR for successful CHO with candidate SCG.

Proposal 5: in case of MCG RLF, introduce information in RLF-report to indicate whether condition evaluation for CHO with candidate SCGs is stopped due to an SCG failure.

On P1:

-	LG and Samsung think current conditions are enough. CATT wants new triggers. Nokia and Ericsson also think no new trigger condition is needed.

On P2:

-	 Samsung think same logic applies as for P1, i.e. not needed. LG agrees.

On P3 and P4

-	Ericsson thinik this is obvious and already in the running CR.

Agreements - No new triggering conditions such as time gap between the first met condition (CHO or CPAC) and the second met condition (CPAC or CHO) is above a threshold, can be considered for SHR/SPR procedure. - No new triggering conditions for SHR/SPR procedure for CHO with candidate SCG. - UE includes the target PSCell ID in SHR for successful CHO with candidate SCGs. - UE includes the target PCell ID in SPR for successful CHO with candidate SCG. - In general, and where applicable, agreements valid for SHR, RLF reports and SCG failure info applies also to SPR.

R2-2501917	New SHR Trigger for CHO with Candidate SCGs	CATT, Vivo, Lenovo, Huawei, HiSilicon, CMCC	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1: In order for the network not to miss abnormal time gap information, introduce a specific SHR trigger condition, i.e., the time gap between fulfilled CHO execution condition and fulfilled CPAC execution condition exceeds a configured threshold.

R2-2502348	Discussion on MRO enhancements for CHO with candidate SCGs	Lenovo	discussion	Rel-19

Proposal 1: Introduce new trigger condition(s) for SHR or SPR for a CHO with candidate SCG(s), e.g. a threshold related to time duration between fulfilment of CHO execution condition configured for the target PCell and fulfilment of CPAC execution condition configured for the associated target PSCell.

Proposal 2: The type of the first fulfilled execution condition (e.g. CHO execution condition or CPAC execution condition), and time from the last triggered event for the PCell (or PSCell) to the time to the last triggered event for the PSCell (or PCell), can be stored and reported by the UE in SHR.

Proposal 3: The type of the first fulfilled execution condition (e.g. CHO execution condition or CPAC execution condition), and time from the last triggered event for the PCell (or PSCell) to the time to the last triggered event for the PSCell (or PCell), can be stored and reported by the UE in SPR.

R2-2502451	SON and MDT for CHO with candidate SCGs	Qualcomm Incorporated	discussion	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1	Reuse the timer timeConnFailure in RLF Report to indicate time elapsed between the CHO with candidate SCG execution and RLF.

Proposal 2	Reuse the timer timeSCGFailure in SCGFailureInformation to indicate time elapsed between the CHO with candidate SCG execution and SCG failure.

Proposal 3	UE should log the CHO recovery cell ID in RLF Report when UE attempts CHO recovery after a CHO with candidate SCG execution failure.

Proposal 4	In case the UE attempts CHO recovery after a CHO with candidate SCG execution failure and observes that there are 2 or more suitable cells, UE may prioritize selecting a cell for CHO recovery based on whether its associated PSCell’s execution condition was met before or not.

R2-2502708	MRO enhancements for CHO with candidate SCGs	CMCC	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1: RAN2 discusses the fallback scenario of CHO with candidate SCGs, i.e. CHO with candidate SCGs and CHO-only are both configured.

Proposal 2: RAN2 discusses the following information to be included for the scenario of CHO-only and CHO with candidate SCGs are both configured:

－	An indication to indicate neither of the two conditions of CHO with candidate SCGs is fulfilled before received the CHO-only or CHO with SCG configuration.

－	The fulfilled CHO with candidate SCGs execution condition when CHO-only configuration is received.

－	The time difference between the first fulfilled execution condition and UE receives CHO-only configuration.

－	The time difference between UE receives CHO with candidate SCGs configuration and CHO-only configuration is fulfilled.

－	Indicate whether the CHO-only or CHO with SCG is configured in RLF report.

R2-2502790	Discussion on MRO for CHO with candidate SCGs	Huawei, HiSilicon	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1: It is proposed RAN2 to agreed that when CHO is fulfilled but CPC conditions are not fulfilled, in RLF report, UE reports identifier of candidate PCell(s) which met the configured CHO execution conditions when the RLF is encountered.

Proposal 2: It is proposed RAN2 to agreed that when CPC is fulfilled but CHO conditions are not fulfilled, in RLF report, UE reports identifier of candidate PSCell(s) which met the configured CPAC execution conditions when the RLF is encountered.

R2-2502807	MRO for CHO and Associated CPAC	LG Electronics Inc.	discussion	Rel-19	R2-2501213

Proposal 1: For CHO with associated CPA/CPC, the UE reports information for the near failure case to the network via SHR not SPR.

Proposal 2. For CHO with associated CPA/CPC, the UE should include the satisfaction time of the first execution condition in the SHR as part of the near failure information.

#### 8.10.2.3	Other

### 8.10.3	SON/MDT for Slicing

R2-2501734	LS on SON for Network Slicing (R3-250914; contact: ZTE)	RAN3	LS in	Rel-19	NR_ENDC_SON_MDT_Ph4-Core	To:RAN2

Moved from 8.10.1

R2-2502409	Enhancements for network slicing (LS R3-250914)	Nokia	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1: RAN2 should study how the UE should provide information that helps the network to identify the scenario when the UE did not manage to select a cell in the frequencies corresponding to the highest ranked NSAG(s).

Proposal 2: Enhance the Mobility History Information to enable the network to identify the scenario when the UE did not manage to select a cell in the frequencies corresponding to the highest ranked NSAG(s).

Proposal 3: Enhance the Mobility History Information with all the NSAG IDs that did not lead to successful cell reselection and the NSAG ID (or the lack of any NSAG ID) that was used to select a new cell.

Proposal 4: Enhance the Mobility History Information with information about the scanned frequencies (e.g., a list of scanned frequencies) used during cell reselection.

Proposal 5: The Mobility History Information can also include the used slice specific reselection parameters along with the NSAG IDs used during cell reselection.

On P1 and P2:

-	Samsung do not see the need for this, and instead agree with the Ericsson proposal below. CMCC see benefit of having slice related info reported from the UE. Huawei  supports the Nokia proposal. Qualcomm think that we can confirm the scenario in the RAN3 LS. Xiaomi agree with Samsung. Ericsson think there is no good justification for enhancing MHI and nothing is broken, the legacy MDT feature is enough. Samsung think that the info is not useful since there can be many reasons why the UE does not camp on the expected cell from a slice-prio p.o.v. Ericsson

FFS if/what RAN2 enhancements we will do to address the scenario when the UE did not manage to select a cell in the frequencies corresponding to the highest ranked NSAG(s).

R2-2502647	Discussion on slice base cell reselection information in Logged MDT	Ericsson	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1	The scenario described in the LS may occur only if there is a coverage hole in the frequencies corresponding to the highest ranked NSAG.

Proposal 2	Send LS reply to RAN3 mentioning the scenario identified in RAN3 can be supported using legacy logged MDT procedure, no further enhancements are needed.

R2-2502452	SON and MDT for slicing	Qualcomm Incorporated	discussion	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1	UE can indicate via a flag in logged MDT report that it collected logged MDT using the slice-based cell reselection parameters (and not general cell reselection parameters).

R2-2502518	Discussion on MDT for Slicing	Samsung	discussion

Proposal 1:RAN2 to inform RAN3 that it is possible to address the scenario of UE supporting slice-based cell reselection not finding any suitable cell in the frequencies corresponding to the highest ranked NSAG, without any enhancement of logged MDT.

R2-2502738	Discussion on SONMDT for network slicing	CMCC	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1: The NSAG(s) supported by the highest ranked cell or best cell, the NSAG(s) supported by the UE and the priority information can be reported to the network, which can be helpful for NW to optimize the sliceAllowedCellListNR and/or sliceExcludedCellListNR, or slice deployment, and can assist the UE to re-select a suitable cell for slice-based cell re-selection.

R2-2502791	Discussion on SONMDT for Slicing	Huawei, HiSilicon	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1: It is proposed RAN2 to agree on introducing the following report type in logged MDT configuration:

-	highest ranked cell or best cell does not support any or all of the intended NSAGs

Proposal 2: It is proposed RAN2 to agree on including the NSAG information received from NAS in logged MDT report.

Proposal 3: It is proposed RAN2 to discuss whether to address the case where the UE fall backs to legacy cell reselection procedure from slice-based cell reselection.

Proposal 4: It is proposed RAN2 to discuss possible solutions, including:

-	addition of a new report type related to P3 in logged MDT configuration

-	addition of a fallback indicator in logged MDT report

R2-2502928	SON/MDT for Slicing	ZTE Corporation, Sanechips	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1	RAN2 to discuss which option to take, logged MDT or RA report to optimize the slice based cell reselection.

Proposal 2	LS reply to RAN3 on RAN2 decision.

### 8.10.4	SON/MDT for NTN

R2-2502453	SON and MDT for NTN	Qualcomm Incorporated	discussion	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1	RAN2 assumes  that the signaling structure agreed for encoding geographical area in case of MBS NTN can be used as a reference for encoding geographical area based logged MDT for NTN.

Proposal 2	RAN2 should discuss whether to support reference location/radius or polygon-based or both.

Proposal 3	Use dedicated signalling to provide area scope configuration to the UE.

Proposal 4	Introduce a UE capability to support geographic area scope checking.

Proposal 5	gNB can only configure either mapped cell list or geographic area scope to the UE.

Proposal 6	Only if the UE can obtain location information in IDLE/Inactive state, the UE performs geographical area checking for logged MDT.

Proposal 7	RAN2 discusses which option is used in case that the area scope is not configured to the UE or UE cannot obtain location information in IDLE/Inactive state if geographic area scope is configured.

Option 1: The UE collects logged MDT data without checking area scope.

Option 2: UE does not perform logged MDT data collection.

On P1:

-	CATT think that we should “reuse” the area IEs and not “use as reference”. Nokia think that we should just reply to RAN3 and we should say it is feasible to reuse and we can discuss signalling structure later.

On P2:

-	Qualcomm want to support location/radius and FFS polygon-based. Xiaomi prefers only location/radius. CATT think we should reuse, i.e. both formats should be supported.

On P4:

-	CATT think this should be conditionally mandatory. Qualcomm think this should be a standalone capability since this is for IDLE/INACTIVE.

On P5:

-	Huawei think that RAN3 LS/procedures suggest that only area should be supported over Uu.

On P7:

-	Qualcomm think option 1 is simpler for the UE. Ericsson want to wait with this. Huawei also want to wait.

-	Nokia think UE always knows its location to some degree. Ericsson think its not mandatory for UE to always know.

Agreements: - RAN2 assumes that the signaling structure agreed for encoding geographical area in case of MBS NTN can be used as a reference for encoding geographical area based logged MDT for NTN. - Support reference location/radius and polygon-based area indication. - Use dedicated signalling to provide area scope configuration to the UE - Introduce a UE capability to support geographic area scope checking. FFS if with or without signalling. - Only geographic area scope is used to indicate applicable logging area to the UE. - Only if the UE can obtain location information in IDLE/Inactive state, the UE performs geographical area checking for logged MDT. FFS if, when the UE is in IDLE/INACTIVE, the UE logs or not if the UE cannot obtain its location.

-	Qualcomm thinks we need to send an LS to RAN3.

[Post129bis][605][SONMDT] LS to RAN3 on MDT for NTN (Qualcomm)

Scope:

    - Produce approvable LS

Intended outcome:

    - Approvable LS in R2-2503117

Deadline:

    - Short

R2-2501918	Consideration on SONMDT enhancements for intra-NTN mobility	CATT	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1: RAN2 to confirm that the geographical area defined for MBS NTN can be reused for Area Scope of logged MDT over Uu for NTN.

Proposal 2: Send a Reply LS to RAN3 with RAN2 agreements on the MDT area scope of NTN.

Proposal 3: The gNB uses the single format in Uu for configuring area scope of logged MDT, no matter it’s configured in the geographical area or mapped cell ID format in NGAP.

Proposal 4: RAN2 to consider how to set the TA information in the CGI report for NTN:

-	Option 1: It’s up to UE to select one TAC indicated in trackingAreaList and add it in CGI info IE.

-	Option 2: To include the whole trackingAreaList in CGI info IE for NTN.

R2-2502242	SON of intra-NTN mobility	China Telecom	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1: Regarding MRO for CHO with a time-based trigger condition, include relative time between the time RLF occurs and t1-Threshold in the RLF report.

Proposal 2: Regarding MRO for CHO with a time-based trigger condition, include relative time between CHO execution failure and t1-Threshold in the RLF report.

Proposal 3: Regarding MRO for CHO with a location-based trigger condition, include measured distance from UE to moving reference location of serving cell, and measured distance from UE to moving reference location of candidate cell when RLF occurred in the RLF report.

R2-2502349	Discussion on MRO for intra-NTN mobility	Lenovo	discussion	Rel-19

Proposal 1: RLF report can be enhanced to include measurement results of source cell, target cell and candidate cell(s) at the absolute time value (t1-Threshold + duration).

Proposal 2: RLF report can be enhanced to include measurement results of source cell, target cell and candidate cell(s) when location based CHO execution condition is fulfilled before RLF happens.

Proposal 3: RLF report can be enhanced to include the distance between UE and a reference location/moving reference location of source cell when measurement based CHO execution condition is fulfilled before RLF happens, or the distance between UE and a reference location/moving reference location of candidate cell(s) when measurement based CHO execution condition is fulfilled before RLF happens.

R2-2502439	SON/MDT for NTN	Samsung	discussion

Proposal 1: RAN2 to send LS to RAN3 informing that geographical area based Area Scope over Uu is not feasible for logged MDT.

Proposal 2: UE reports following additional information in RLF report for NTN related failures

a.	Timing Advance reported in the Timing Advance Report MAC CE, if it was triggered during the random access procedure during a handover.

b.	A flag indicating whether T430 had expired when RLF or HOF occurred.

c.	Information related to satellite position.

Proposal 3: UE may log tracking area list in CGI-Info-Logging in various SON/MDT reports.

R2-2502641	MRO for NTN RACH-less Access	Nokia	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1: RAN2 to consider enhancing SHR for RACH-less access for NTN.

Proposal 2: RAN2 to consider enhancing RLF Report for RACH-less access for NTN.

Proposal 3: Enhance the SHR Report message definition with NTN RACH-less specific reporting.

Proposal 4: Define a new SHR trigger for the UE to log the enhancements measurements specific to NTN RACH-less access.

Proposal 5: Enhance the RLF Report message definition with NTN RACH-less specific measurements and a new RLF Cause specific to NTN RACH-less access failure.

Proposal 6: RAN2 to send a reply LS for the incoming LS from RAN3, R2-2500026/R3-247908, and indicate it is technically feasible to use a similar Uu signalling structure as adopted for encoding geographical area in the case of MBS NTN, for geographical area based logged MDT for NTN.

R2-2502648	Discussion on the reply LS to RAN3 on SON-MDT enhancements for NTN	Ericsson	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1	RAN2 to confirm that the geographical area for MBS NTN can apply to MDT area scope, i.e. a geographical area represented by a (set of) referenceLocation and radius or by a (set of) polygon(s).

Proposal 2	RAN2 to discuss if and how to optimize location checking for logged MDT for the sake of UE energy saving. Detailed solutions (how frequent the UE should check the location) can be FFS.

R2-2502792	Discussion on SONMDT for NTN	Huawei, HiSilicon	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1: It is proposed RAN2 to agree on reusing the geographical area defined for MBS NTN for Area Scope of logged MDT over Uu for NTN.

Proposal 2: It is proposed RAN2 to discuss whether to introduce some means to minimize negative impacts at UE side, e.g. the UE performs geographical area checking only if it has available position information.

R2-2502808	NTN Logging for Unchanged PCI Mobility	LG Electronics Inc.	discussion	Rel-19	R2-2501214

Proposal 1: For unchanged PCI satellite mobility, the UE uses the RLF report, including a new cause to indicate a failure of the unchanged PCI satellite mobility if RLF is declared due to this mobility.

Proposal 2: For unchanged PCI satellite mobility, the UE reports the near failure information that out-of-sync indication(s) has been detected during the mobility if the UE detects the out-of-sync indications during the unchanged PCI satellite mobility but successfully complete this mobility.

Proposal 3: To determine a proper near failure of the unchanged PCI satellite mobility, a new logging condition is considered based on reception of a significant number of out-of-sync indications during the mobility.

R2-2502929	SON/MDT for NTN	ZTE Corporation, Sanechips	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1	RAN2 to discuss for the RACH-less HO, whether to report to network a supposedly RACH-less which falls back to RACH based, and in which report (e.g., Random Access report or rlf-Report.)

Proposal 2	Introduce absolute time, absolute location, and speed information in NTN RLF reports when UE triggers CHO and upon RLF.

Proposal 3	Adopt geographic area as the area scope information for NTN MDT.

Proposal 4	Reuse the geographic area defined in SIB25, for the absolute geographic area as the area scope information for NTN MDT. Can coordinate with Rel-19 NTN Working Item during stage 3 phase.

Proposal 5	LS back to RAN3 on above agreements.

R2-2502975	Discussion on SONMDT for NTN mobility	Xiaomi	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1: The geographical area (a reference location plus a radius) can be reused for area scope of logged MDT over Uu for NTN.

### 8.10.5	Leftovers from Rel-18

RACH optimization for SDT focus on RSRP and data volume in SON reports, and existing failure causes.

MHI Enhancement for SCG Deactivation/Activation.

R2-2502482	SON/MDT for Leftovers from Rel-18	Samsung	discussion

Proposal 1: Value range of SDT-UL-DataVolume is 0 to 96000. UE reports 96000 when the data volume is larger than 96000.

Proposal 2: UL data volume is reported only for MO SDT and only radio bearers (DRB and SRB2) configured for SDT are considered.

Proposal 3: perRA-InforList reported in SCGFailureInformationNR includes PerRAInfoList-r16, PerRAInfoList-v1660 and PerRAInfoList-v1800 as in SCGFailureInformation (same as in running LTE CR).

Proposal 4: UE reports the absolute time it has spend in PSCell with SCG activated in MHI.

On P1:

-	ZTE wants a ratio. Samsung clarifies its 96000 bits.

On P2:

-	Ericsson think that there is no condition in the current running CR, so no need to add SRB/DRB since the UE will only do SDT if configured to do SDT and on the configured bearers.

Agreements: - Value range of SDT-UL-DataVolume is 0 to 96000 bits. UE reports 96000 when the data volume is larger than 96000. This is encoded as ENUMERATED. - UL data volume is reported only for MO SDT and only radio bearers configured for SDT are considered. This has no specific specification impact. - perRA-InforList reported in SCGFailureInformationNR includes PerRAInfoList-r16, PerRAInfoList-v1660 and PerRAInfoList-v1800 as in SCGFailureInformation (same as in running LTE CR). - UE reports the absolute time it has spent in PSCell with SCG activated in MHI.

R2-2501919	MHI Enhancement for SCG Activation and Deactivation	CATT	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1: UE reports the absolute time the UE has spent in PSCell with SCG activated in MHI.

Proposal 2: RAN2 to discuss the following two options about logging the SCG activation/deactivation time for a PSCell in MHI:

-	Option 1: logging each SCG activation/deactivation time duration for a PSCell in chronological order;

-	Option 2: logging the accumulated time for SCG activation/deactivation for a PSCell;

R2-2502350	Discussion on RACH optimization for SDT	Lenovo	discussion	Rel-19

Proposal 1: The UE reports RSRP and data volume when triggered RA-SDT fails in the RA report for SDT optimization.

R2-2502410	MHI/UHI Enhancement for SCG Deactivation/Activation	Nokia	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1: It is proposed to include the information of SCG activation/deactivation, e.g., the ratio of time in SCG activated state together with some more assistance information related to the condition(s) for SCG activation/deactivation, in MHI.

R2-2502709	MHI/UHI Enhancement for SCG Deactivation/Activation	CMCC	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1: Include the accumulated time of UE spent in SCG activation state in MHI.

R2-2502859	RA report enhancements for SDT	SHARP Corporation	discussion

Proposal 1: UE does not log the failure cause for failed SDT in RA report for the case “upon the lower layers indicate that cg-SDT-TimeAlignmentTimer or the configuredGrantTimer expired before receiving network response for the UL CG-SDT transmission with CCCH message while SDT procedure is ongoing”.

Proposal 2: UE logs the DL RSRP and UL data volume in both RA-SDT initiation failure case and normal ongoing RA-SDT failure case.

Proposal 3: the elapsed time since the execution of RA-SDT is also included in RA report for successful RA-SDT case.

R2-2502862	MHI enhancement for SCG activation/deactivation	SHARP Corporation	discussion

Proposal 1: SCG activation related time is reported per PSCell in a PCell.

Proposal 2: percentage of time is used SCG activation time in MHI.

Proposal 3: percentage of time is specified as enumeration type with several typical values.

Proposal 4: UE only sets the SCG activation time for a PSCell entry only when the SCG has ever been in deactivation during the stay in the PSCell.

R2-2502930	Remaining issues for Rel-18 leftovers	ZTE Corporation, Sanechips	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1	RAN2 to discuss which option to define sdt-UL-DataVolume-r19:

(1) an absolute data volume, or

(2) a relative value (e.g., a ratio compared to the network configured threshold, from 1 to 10, in step of 0.1).

Proposal 2	RAN2 to discuss which option to take for timeSinceSdtExecution:

(1) if timeSinceSdtExecution reaches the maximum (i.e., 48 hours), network should not see it as a valid value, or

(2) UE should not report this value if it is equal or beyond the maximum value.

Proposal 3	UE includes traffic characteristics (e.g., average data rate during activation period) in the MHI together with the SCG activation/deactivation information.

R2-2502951	On remaining issues related to Rel.18 leftovers	Ericsson	discussion

Proposal 1	For each PSCell entry of the MHI, log the absolute, accumulated time with SCG state set to activated during the stay in the PSCell.

Proposal 2	Agree on the 38.331 TP implementing the inclusion of the absolute, accumulated time with SCG state set to activated during the stay in the PSCell in MHI provided in Appendix A.

Proposal 3	RA report enhancements for SDT is considered concluded in RAN2, no further enhancements are discussed in Rel-19.

# Summary

In-principle agreed NBC CRs:

R2-2502271	Correction to nrofCandidates-CI for DCI format 2_4	Huawei, HiSilicon	CR	Rel-16	38.331	16.19.0	5295	-	F	NR_L1enh_URLLC-Core

R2-2502272	Correction to nrofCandidates-CI for DCI format 2_4	Huawei, HiSilicon	CR	Rel-17	38.331	17.12.0	5296	-	A	NR_L1enh_URLLC-Core

R2-2502273	Correction to nrofCandidates-CI for DCI format 2_4	Huawei, HiSilicon	CR	Rel-18	38.331	18.5.1	5297	-	A	NR_L1enh_URLLC-Core

R2-2502920	Slot aggregation configuration with multi-PUSCH	Nokia	CR	Rel-17	38.331	17.12.0	5328	-	F	NR_ext_to_71GHz-Core

R2-2502921	Slot aggregation configuration with multi-PUSCH	Nokia	CR	Rel-18	38.331	18.5.1	5329	-	A	NR_ext_to_71GHz-Core

Post meeting email discussions:

[Post129bis][601][Maint] Miscellaneous non-controversial corrections (Ericsson)

Scope:

    - Produce agreeable CRs

Intended outcome:

    - In principle agreed CRs

Deadline:

    - Short

[Post129bis][603][SONMDT] Running NR RRC CR for SONMDT (Ericsson)

Scope:

    - Update the running CRs based on the progress in the meeting

Intended outcome:

    - Updated running CR to be submitted to next meeting

Deadline:

    - Long

[Post129bis][604][SONMDT] Running LTE RRC CR for SONMDT (Huawei)

Scope:

    - Update the running CRs based on the progress in the meeting

Intended outcome:

    - Updated running CR to be submitted to next meeting

Deadline:

    - Long

[Post129bis][605][SONMDT] LS to RAN3 on MDT for NTN (Qualcomm)

Scope:

    - Produce approvable LS

Intended outcome:

    - Approvable LS in R2-2503117

Deadline:

    - Short