3GPP TSG-RAN WG2 Meeting #129bis	R2-2503070

Wuhan, China, April 7 – 11, 2025

Title:	LS on S&F mode indications to NAS

Response to:	-

Release:	Rel-19

Work Item:	IoT_NTN_Ph3-Core

Source:	RAN2

To:	CT1

Cc:	-

Contact person:	Xiao XIAO

xiaoxiao@catt.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

# 1	Overall description

RAN2 agreed to support the time information for the transition between "store-and-forward (S&F) operation mode" and "normal mode" which can be provided by a Rel-19 IoT NTN satellite supporting S&F operation. Specifically:

- RAN2 agreed that time information for the transition from current "S&F operation mode" to "normal mode" is provided in system information (i.e. SIB31). For UEs supporting S&F operation, the UE AS indicates the information on transition time from current "S&F operation mode" to "normal mode" (if received) to the upper layers, e.g. to delay some NAS procedures till the feeder link is resumed.

- RAN2 also agreed to introduce an indication in system information for the "normal mode" to "S&F operation mode" transition, at least for NAS use. The information on transition time for the "normal mode" to "S&F operation mode" transition (if received) is sent from AS to NAS.

RAN2 assumed that whether/how the above transition time information is used by the upper layers is up to CT1.

# 2	Actions

ACTION: 	RAN2 respectfully asks CT1 to take above information into account, consider possible specification changes in NAS and provide feedback (if needed).

# 3	Dates of next TSG RAN WG2 meetings

TSG RAN WG2 Meeting #130	2025-05-19 ~ 2025-05-23 	Malta, MT

TSG RAN WG2 Meeting #131	2025-08-25 ~ 2025-08-29 	Bangalore, IN