3GPP TSG RAN WG2#129bis	R2-2501713

Wuhan, China, 7th - 11th April 2025

3GPP TSG RAN WG1 Meeting #120	R1-2501560

Athens, Greece, February 17th – 21st, 2025

Title:	Reply LS on CSI-RS measurement with SBFD operation

Response to:	R4-2420165

Release:	Rel-19

Work Item:	NR_duplex_evo-Core

Source:	RAN1

To:	RAN4

Cc:	RAN2

Contact Person:	Mohammed Al-Imari

mohammed.al-imari@mediatek.com

Send any reply LS to:	 3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# Overall description

RAN1 would like to thank RAN4 for their LS in R4-2420165. RAN1 would like to provide the following responses to RAN4 questions.

Issue#1: Collision Case 2 that involves CSI-RS resources used for L1-RSRP/SINR and RLM/BFD/CBD measurements

For collision Case 2 (semi-statically configured DL reception vs. dynamically scheduled UL transmission) that involves CSI-RS resources used for L1-RSRP/SINR and RLM/BFD/CBD measurements, RAN1 would like to provide the following response:

The intention of the mentioned RAN1 agreement is to follow the existing prioritization rules between semi-static DL channels/signals and dynamically scheduled UL transmission including semi-statically configured CSI-RS resources for L1-RSRP/SINR measurement and RLM/BFD/CBD measurement. The existing prioritization rules for serving-cell for collision Case 2 are as follows: - TS 38.213, Clause 11.1 “Slot configuration” “For operation on a single carrier in unpaired spectrum, if a UE is configured by higher layers to receive a PDCCH, or a PDSCH, or a CSI-RS, or a DL PRS in a set of symbols of a slot, the UE receives the PDCCH, the PDSCH, the CSI-RS, or the DL PRS if the UE does not detect a DCI format that indicates to the UE to transmit a PUSCH, a PUCCH, a PRACH, or a SRS in at least one symbol of the set of symbols of the slot; otherwise, the UE does not receive the PDCCH, or the PDSCH, or the CSI-RS, or the DL PRS in the set of symbols of the slot.” - TS 38.213, Clause 11.1.1 “UE procedure for determining slot format” “If a UE is configured by higher layers to receive a CSI-RS or a PDSCH in a set of symbols of a slot and the UE detects a DCI format 2_0 with a slot format value other than 255 that indicates a slot format with a subset of symbols from the set of symbols as uplink or flexible, or the UE detects a DCI format indicating to the UE to transmit PUSCH, PUCCH, SRS, or PRACH in at least one symbol in the set of the symbols, the UE cancels the CSI-RS reception in the set of symbols of the slot or cancels the PDSCH reception in the slot.”

Issue#2: Behaviour for CSI-RS based L1-RSRP/SINR and RLM/BFD/CBD measurements

For resource configuration and/or reporting configuration for CSI-RS based L1-RSRP/SINR measurement and RLM/BFD/CBD measurement, RAN1 would like to provide the following response:

- According to the following RAN1 agreements, Configuration 1 and Configuration 2 are not applicable to CSI-RS resources configuration and CSI-RS based RLM/BFD/CBD measurement.

Agreement (RAN1#120) The configuration of Configuration 1 or 2 for DL BWP only applies to PDSCH receptions within the DL BWP. Agreement (RAN1#117) For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) for an SBFD aware UE, the SBFD-aware UE is provided with one of the configurations. - Configuration 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only. - Configuration 2: The transmissions/receptions can be in SBFD symbols and non-SBFD symbols. - FFS granularity of the configuration, e.g. per UE, per channel/signal etc. FFS whether support of configuration 2 is subject to UE capability. Agreement (RAN1#118bis) For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) for an SBFD aware UE, - Support of configuration 2 is subject to UE capability. Configuration 1 is the default capability. - SBFD-aware UE can be configured with Configuration 2 on a per UL/DL BWP basis. - The configuration for DL BWP at least applies to PDSCH receptions within the DL BWP. - The configuration for UL BWP applies to PUCCH and PUSCH transmissions within the UL BWP. - For SRS, only Configuration 1 is applicable.

- The following RAN1 agreements for CSI reporting are applicable to CSI-RS based L1-RSRP/SINR measurement/reporting.

Agreement (RAN1#118bis) For a CSI report associated with periodic/semi-persistent CSI-RS, the valid symbol type for CSI derivation for periodic/semi-persistent CSI-RS resources for the CSI report is explicitly configured. Only CSI-RS transmission occasions within the valid symbol types are used for CSI derivation. Agreement (RAN1#120) For a CSI report associated with periodic/semi-persistent CSI-RS, the valid symbol type for CSI derivation is configured in CSI-ReportConfig.

# Actions

RAN1 respectfully asks RAN4 to take the above clarifications and replies into consideration in their future work.

# Dates of next TSG RAN WG1 meetings

TSG RAN WG1 Meeting #120-bis	April 7 – 11, 2025	Wuhan, CN

TSG RAN WG1 Meeting #121	May 19 – 23, 2025	Malta, MT