3GPP TSG RAN WG2#129bis	R2-2501721

Wuhan, China, 7th - 11th April 2025

3GPP TSG-RAN WG1 Meeting #120	R1- 2501628

Athens, Greece, February 17th – 21st, 2025

Title: 	LS on AI/ML Positioning Case 3b

Response to:

Release:	Rel-19

Work Item:	NR_AIML_Air

Source:	RAN1

To:	RAN3, RAN2

Cc:

Contact person

Name:	Yufei Blankenship

E-mail Address:	yufei.blankenship@ericsson.com

# Attachments:	None

# 1	Overall description

RAN1 would like to provide the following agreements, working assumption and conclusion related to Case 3b. RAN1 respectfully asks RAN3 and RAN2 to take them into account in their future work.

Related to measurement report, where the measurement report may be applicable to model inference and training data collection:

Agreement (RAN1#116) - For AI/ML based positioning case 3b, at least the following types of time domain channel measurements are supported for reporting: - timing information; - paired timing information and power information. Agreement (RAN1#116bis) For AI/ML based positioning Case 3b, for gNB channel measurements reported to LMF, the timing information is represented relative to the existing UL RTOA reference time T0+tSRS as defined in TS 38.215. FFS: whether it is applicable when Case 3b is used to support multi-RTT Agreement (RAN1#118) For AI/ML positioning Case 2b and 3b, regarding the power information for determining the model input, - For downlink power measurement, use DL PRS-RSRPP defined in TS 38.215 as a starting point. - For measurement report of DL PRS-RSRPP, use the existing measurement report mapping table for PRS-RSRPP in 38.133 as a starting point. - For uplink power measurement, use UL SRS-RSRPP defined in TS 38.215 as a starting point. - For measurement report of UL SRS-RSRPP, use the existing measurement report mapping table for SRS-RSRPP in 38.133 as a starting point. Agreement (RAN1#118bis) From RAN1 perspective, for model inference of AI/ML positioning Case 3b, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF: - (Mandatory) Channel measurement; - (Optional) Quality of the channel measurement; - FFS: details of the quality - (Mandatory) Time stamp of the channel measurement. Agreement (RAN1#119) For Rel-19 AI/ML based positioning, for Case 3b, in addition to path-based measurement that is referring to the measurement in the existing specifications (up to Rel-18), additionally support the following enhancement to the measurement, - The measurement is composed of Nt’ values of the estimated channel response in time domain. The Nt’ values are selected from a list of Nt consecutive channel response values, which have timing granularity T. - The timing information for the Nt’ values are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR. - The associated measurement (e.g., power if reported) corresponds to the measurement for the reported Nt’ values. - The timing information is defined relative to a reference time, same as the path-based measurement. - The Nt’ selected time domain channel measurement values are expected to be those with the highest power. - The starting time of the list of Nt consecutive values is determined as: starting time = first detected path rounded down with timing granularity T. - LMF can signal parameter values of Nt, Nt’, k to gNB via NRPPa. Candidate set values: - Nt’<=24. FFS: Nt’ values. - Nt = {32, 64, 128} - FFS: k - The gNB/TRP may use different Nt’, Nt and/or k values other than the signalled parameter for measurement reporting. In this case, it’s up to LMF implementation to process the reported measurement - FFS: whether transmit offset from gNB to LMF Note: measurement by UE is a separate discussion. Note: the purpose of the time domain channel measurements, such as for Rel-19 AI/ML based positioning, is not specified Agreement (RAN1#120) For Rel-19 AI/ML based positioning, for Case 3b, “FFS: k” in RAN1#119 agreement is resolved by supporting: - k = {0…5} Agreement (RAN1#120) For Rel-19 AI/ML based positioning, for Case 3b, “FFS: Nt’ values” in RAN1#119 agreement is resolved by supporting: - Nt’ = {8, 16, 24} Agreement (RAN1#120) For AI/ML based positioning Case 3b, with the existing definition of two channel measurement types (A) or type (B): (A) path-based measurement, i.e., measurement in the existing specifications (up to Rel-18), (B) Rel-19 enhanced measurement (see definition in RAN1#119 agreement for Case 3b). - Descriptive info of the channel measurement is included together with the channel measurement values, either explicitly or implicitly. - The descriptive info of the channel measurement includes at least: - measurement type (A) or type (B) and - measurement parameters. - Regarding measurement parameters, - For (A) path-based measurement, the measurement parameters include: number of additional paths, k. - For (B) Rel-19 enhanced measurement, the measurement parameters include at least: Nt', k. - FFS: Nt - Note: Type (A) and Type (B) are used for discuss purpose only. Conclusion (RAN1#120) For Rel-19 AI/ML based positioning Case 3b, - If the gNB cannot report measurements according to measurement type (A) or measurement type (B) requested by LMF, the gNB responds to LMF that: the gNB cannot provide measurement with the requested measurement type. - The gNB does not respond to LMF with a measurement report where the measurement type is different from requested. - It is up to RAN3 to decide the signaling details. - Note: It is RAN1 understanding that this is the same behavior as in legacy when the NG-RAN node cannot support the requested measurement.

Related to training data collection:

Agreement (RAN1#116bis) For training data generation of AI/ML based positioning Case 3a and 3b, the measurement and its related data (e.g., timestamp) are generated by TRP/gNB. Agreement (RAN1#116bis) For training data collection of AI/ML based positioning, the collected data sample can include the following components: Part A: - channel measurement - quality indicator of channel measurement - time stamp of channel measurement Part B: - ground truth label (or its approximation) - quality indicator of label - time stamp of label Note: “Part A” and “Part B” terminologies are only for RAN1 discussion purpose, and may not be used in specification. Note: contents in Part A and Part B may or may not be generated by different entities. Note: Part A and/or Part B, and their contents may or may not apply for each case FFS: detailed definition of channel measurement Working Assumption (RAN1#117) For training data generation of AI/ML based positioning Case 3b, the label and its related data (e.g., time stamp) can be generated by: - PRU - Non-PRU UE with estimated location - LMF Note: transfer of label and its related data is out of RAN1 scope. Note: It is assumed that user data privacy of non-PRU UE is preserved. Agreement (RAN1#117) For training data collection of AI/ML based positioning, if a training data sample contains both Part A and Part B, RAN1 assumes that Part A and Part B in one training data sample are: - for a same UE (PRU or Non-PRU UE), and - for a same location associated with Part B. Note: the association can be discussed Agreement (RAN1#118) For training data collection of AI/ML based positioning case 3b, for time stamp of channel measurement, - For channel measurement generated by TRP/gNB, existing IE “Time Stamp” in TS 38.455 can be reused from RAN1 perspective - Note: Purpose, such as above “training data collection", will not necessarily be specified in RAN 1 specifications Agreement (RAN1#118bis) For training data collection of AI/ML based positioning, the quality indicator of timing information in Part A when reported is: - When applicable, the existing IE for timing quality, i.e., NR-TimingQuality in 37.355 and IE “Timing Measurement Quality” in 38.455; - FFS: details on how to associate quality indicator to timing information Agreement (RAN1#120) For AI/ML based positioning, - When channel measurement is to be reported/sent, time stamp of channel measurement refers to the time instance when the channel measurement is performed. - When label is to be reported/sent, time stamp of label is the time instance for which the label is valid. Agreement (RAN1#120) - For training data collection of AI/ML based positioning 3a/3b, if Part A and Part B are generated by different entities, for pairing between a Part A entry and a Part B entry, the following is needed: - The time stamp of Part A (if Part A is transmitted) and the time stamp of Part B (if Part B is transmitted). - FFS: other information is not precluded (e.g., if Part B is valid for a duration) Note: Purpose such as “training data collection” will not necessarily be specified in RAN1 specifications.

# 2	Actions

To RAN3, RAN2:

ACTION:	RAN1 respectfully asks RAN3 and RAN2 to take the above into account in their future work.

# 3	Dates of next TSG RAN WG1 meetings

TSG RAN WG1 Meeting #120-bis	7th April– 11th April, 2025	China

TSG RAN WG1 Meeting #121	19th May– 23rd May, 2025	Malta