3GPP TSG RAN WG2#129bis	R2-2501744

Wuhan, China, 7th - 11th April 2025

3GPP TSG-RAN WG4 Meeting #114	R4-2503017

Athens, Greece, 17-21 February 2025

Title:	LS on Signalling for 7 MHz Channel Bandwidth

Response to:

Release:	Rel-19

Work Item:	NR_FR1_7MHz_BW-Core

Source:	RAN4

To:	RAN2

Cc:	RAN3

Contact Person:

#### Name:	Bill Shvodian

E-mail Address:	bill.shvodian@t-mobile.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:

1. Overall Description:

RAN4 would like to inform RAN2 on the progress made on the topic of release independence for the 7 MHz channel bandwidth.

RAN4 has noted that TS 38.307 says that “NR channel bandwidths, all unless otherwise stated” are release independent from Release 15. RAN4 understands that there were some unique characteristics of 3 MHz channel bandwidths including new RAN1 requirements and signalling related to support for the punctured SSB that prevented the 3 MHz channel bandwidth from being release independent from Rel-15. RAN4 has discussed release independence of the 7 MHz channel bandwidth and have decided that there is no reason from a RAN4 perspective that 7 MHz cannot be release independent from Release 15 for a band with a minimum channel bandwidth of 3MHz or 5MHz.

RAN4 also notes that 35 and 45 MHz channel bandwidths were introduced in a Release 17 Work Item, but are release independent from Release 15, including the use of spare bits in 38.306 Release 15. RAN4 believes that adding 7 MHz should be similar to adding 35 and 45 MHz from signaling design aspect.

RAN4 understands that maximum and minimum channel bandwidths can be reported for band combinations. For band combinations that include a band or bands that support 7 MHz, we think it is important to note that 7 MHz is unlikely to be the minimum or maximum channel bandwidth that a UE supports for a band in a given band combination. We think it would be fine to add support for 7 MHz as a possible maximum or minimum supported bandwidth in a release later than Rel-15, as 35 and 45 MHz were added in SupportedBandwidth-v1700, while the bandwidths themselves are release independent from Rel-15.

RAN4 respectfully requests that RAN2 consider the signalling aspects related to adding the 7 MHz channel bandwidth, and to inform RAN4 if RAN2 sees any reason from a signalling perspective why the 7 MHz channel bandwidth cannot be release independent from Release 15.

2. Actions:

To RAN2

ACTION: 	RAN4 respectfully requests that RAN2 consider the signalling aspects related to adding the 7 MHz channel bandwidth, and to inform RAN4 if RAN2 sees any reason from a signalling perspective why the 7 MHz channel bandwidth cannot be release independent from Release 15.

3. Date of Next TSG-RAN4 Meetings:

3GPP TSG RAN WG4#114bis	7 - 11 Apr 2025	Wuhan, China

3GPP TSG RAN WG4#115	19 - 23 May 2025	St. Julian’s, Malta