3GPP TSG-RAN WG2 meeting #130	R2-2503302

Agenda Item:	2.2

Source: 	ETSI MCC

Title:		Report of 3GPP TSG RAN WG2 meeting #129bis, Wuhan, China

Report of 3GPP TSG RAN WG2 meeting #129bis

Wuhan, China

7 - 11 April, 2025

Document for: Approval

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

# Contents

Organisation of the meeting	7

Statistics/Executive Summary	7

General	8

1	Opening of the meeting	8

1.1	Call for IPR	8

1.2	Network usage conditions	8

1.3	Other	8

General	8

2.1	Approval of the agenda	8

2.2	Approval of the report of the previous meeting	9

2.3	Reporting from other meetings	9

2.4	Instructions	9

2.5	Others	10

3	Incoming liaisons	11

4	EUTRA Rel-17 and earlier	11

4.1	EUTRA corrections Rel-17 and earlier	11

4.3	Positioning corrections Rel-16 and earlier	13

5	NR Rel-15 and Rel-16	13

5.1	Common	13

5.1.1	Stage 2 and Organisational	13

5.1.2	User Plane corrections	14

5.1.2.1	MAC	14

5.1.2.2	RLC PDCP SDAP BAP	14

5.1.3	Control Plane corrections	14

5.1.3.1	NR RRC	14

5.1.3.2	UE capabilities	16

5.1.3.3	Other	18

5.3	NR Positioning Support	18

6	NR Rel-17	19

6.1	Common	19

6.1.1	Stage 2 and Organisational	19

6.1.2	User Plane corrections	20

6.1.3	Control Plane corrections	21

6.1.3.1	NR RRC	21

6.1.3.2	UE capabilities	24

6.1.3.3	Other	25

6.3	NR positioning enhancements	26

7	Rel-18	26

7.0	Common	26

7.0.1	UE Capabilities	26

7.0.2	Rel-18 corrections	27

7.0.2.1	RACH-less HO	27

7.0.2.2	NR network-controlled repeaters	28

7.0.2.3	NR support for UAV	28

7.0.2.4 	Mobile Terminated Small Data Transmission	28

7.0.2.5	IDC enhancements for NR and MR-DC	28

7.0.2.6	Mobile IAB (Integrated Access and Backhaul) for NR	28

7.0.2.7	Timing Resiliency and URLLC Enh	28

7.0.2.8	Enhanced support of reduced capability NR devices	28

7.0.2.9	Further NR coverage enhancements	28

7.0.2.10	Network energy savings for NR	28

7.0.2.11	Further enhancement of data collection for SON MDT in NR and EN-DC	30

7.0.2.12	Dual Transmission/Reception (Tx/Rx) Multi-SIM for NR	30

7.0.2.13	NR MIMO evolution	31

7.0.2.14	Enhancements of NR Multicast and Broadcast Services	32

7.0.2.15	Enhancement on NR QoE management and optimizations for diverse services	32

7.0.2.16	XR Enhancements for NR	32

7.0.2.17	NR NTN enhancements	33

7.0.2.18	IoT NTN enhancements	36

7.0.2.19	Enhanced NR Sidelink Relay	37

7.0.2.20	TEI18	41

7.0.2.21	Others	43

7.1	Expanded and improved NR positioning	43

7.1.1	Organizational	43

7.1.2	Stage 2	44

7.1.3	SLPP corrections	44

7.1.4	LPP corrections	44

7.1.5	RRC corrections	45

7.1.6	MAC corrections	46

7.1.7	Corrections to other specifications	48

7.2	Further NR mobility enhancements	49

7.6	NR Sidelink evolution	53

7.8	R18 Other	55

7.8.1	RAN4 led items	55

7.8.2	RAN1 led items	55

7.8.3	Other	56

8	Rel-19	56

8.0	General	56

8.1	AI/ML for NR air interface	59

8.1.1	Organizational	59

8.1.2	Functionality based LCM	61

8.1.2.1	LCM for NW-sided model for Beam Management use case	61

8.1.2.2	LCM for UE-sided model  for Beam Management use case	61

8.1.2.3	LCM for Positioning use case	67

8.1.5	Model transfer/delivery	76

8.2	Ambient IoT	80

8.2.1	Organizational	80

8.2.2	A-IoT Paging	81

8.2.3	A-IoT Random Access	86

8.2.4	A-IoT Data Transmission and Other general aspects	90

8.3	AI/ML for Mobility	96

8.3.1	Organizational	96

8.3.2	RRM measurement prediction	97

8.3.2.1	Simulation results	97

8.3.2.2	Model Generalization	97

8.3.3	Measurement event predictions	98

8.3.4	RLF/HO failure prediction	100

8.3.5	LCM and spec impact for AI/ML mobility	100

8.4	Low-power wake-up signal and receiver for NR (LP-WUS/WUR)	106

8.4.1	Organizational	106

8.4.2	Procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE	108

8.4.3	RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE	112

8.4.4	Procedures for LP-WUS in RRC_CONNECTED	116

8.5	Network Energy Saving Enh.	120

8.5.1	Organizational	120

8.5.2	On-demand SSB SCell operation	122

8.5.3	On-demand SIB1	126

8.5.4	Adaptation of common signal/channel transmissions	131

8.6	Mobility Enhancement Ph4	134

8.6.1	Organizational	135

8.6.2	Inter-CU LTM	140

8.6.3	L1 event triggered measurement reporting	145

8.6.4	Conditional intra-CU LTM	151

8.7	XR Enhancements Ph3	154

8.7.1	Organizational	155

8.7.2	Multi-modality support	158

8.7.3	RRM measurement gaps/restrictions related enhancements	158

8.7.4	Scheduling enhancements	159

8.7.4.1	LCP enhancements	159

8.7.4.2	DSR enhancements	162

8.7.5	RLC enhancements	165

8.7.6	XR rate control	169

8.8	NTN for NR Ph3	172

8.8.1	Organizational	172

8.8.2	Downlink coverage enhancements	174

8.8.3	Uplink Capacity/Throughput Enhancement	178

8.8.4	Support of Broadcast service	179

8.8.5	Support of regenerative payload	182

8.8.6	LTE to NR NTN mobility	182

8.9	IoT NTN Ph3	183

8.9.1	Organizational	183

8.9.2	Support of Store & Forward	184

8.9.3	Uplink Capacity Enhancement	187

8.9.4	Support of PWS	196

8.10	SON/MDT Ph4	197

8.10.1	Organizational	197

8.10.2	MRO enhancements for Rel-18 mobility features	199

8.10.2.1	LTM	200

8.10.2.2	CHO with candidate SCGs	203

8.10.2.3	Other	205

8.10.3	SON/MDT for Slicing	205

8.10.4	SON/MDT for NTN	206

8.10.5	Leftovers from Rel-18	209

8.11	Evolution of NR duplex operation: Sub-band full duplex (SBFD)	211

8.11.1	Organizational	211

8.11.2	Random access in SBFD	214

8.11.3	Other aspects	217

8.12	NR MIMO Phase 5	219

8.12.1	Organizational	219

8.12.2	Asymmetric DL sTRP/UL mTRP	220

8.12.3	Others	224

8.13	NR sidelink multi-hop relay	228

8.13.1	Organizational	228

8.13.2	Relay discovery and (re)selection	232

8.13.3	Control Plane Procedures and SRAP impact	238

8.13.4	Service continuity	246

8.14	Additional topological enhancements	248

8.15	NavIC L1 SPS A-GNSS support	249

8.16	BDS B2b in A-GNSS	249

8.17	IoT-NTN TDD mode	249

8.18	TEI19	252

8.19	NR Others	259

9	Breakout session reports	263

9.1	Session on V2X/SL, R19 NES and MOB	263

9.2	Session on R18 MIMO, R19 MIMO, R19 LP-WUS, and SBFD	263

9.3	Session on NR NTN and IoT NTN	263

9.4	Session on positioning and sidelink relay	263

9.5	Session on R18 MBS, R18 QoE and R19 XR	263

9.6	Session on maintenance and SON/MDT	263

Closing of the meeting	263

Annex A:	List of participants	263

Annex B:	List of Tdocs	264

Annex C:	Incoming liaison statements	264

Annex D:	Outgoing liaison statements	269

Annex E:	List of agreed in principle CRs	270

Annex F:	List of email discussions during the meeting	274

Annex G:	Post-meeting email discussions after RAN2#129bis	275

Short email discussions, Deadline Apr. 18th, 10:00 UTC	276

Long email discussions, for R2-130, May.  2nd 10:00 UTC (unless otherwise stated)	278

Annex H:	History	285

# Organisation of the meeting

Meeting:	3GPP TSG RAN2#129bis

Meeting location:	Wuhan, China

Duration:	7 - 11.04.2025

Host:	CCSA

TSG RAN WG2 Chair	Diana Pani (InterDigital) (diana.pani@interdigital.com)

TSG RAN WG2 Vice chair:	Kyeongin Jeong (Samsung) (kyeongin.j@samsung.com)

TSG RAN WG2 Vice chair:	Erlin Zeng (CATT) (erlin.zeng@catt.cn)

TSG RAN WG2 MCC Support:	Juha Korhonen (ETSI MCC) (juha.korhonen@etsi.org)

Email reflector:	3GPP_TSG_RAN_WG2@LIST.ETSI.ORG

Technical documents:	ftp://ftp.3gpp.org/tsg_ran/WG2_RL2/TSGR2_129bis/Docs

Next meetings:	TSG RAN2#130	19 - 23.05.2025, St Julians, Malta

TSG RAN2#131	25 - 29.08.2025, Bangalore, India

# Statistics/Executive Summary

TSG RAN2#129bis was a normal face-to-face meeting, with a possibility for two-way remote access.

The topics discussed were:

-	NR Rel-15, Rel-16 and Rel-17 User Plane corrections, UE Capabilities, RACH-less HO, Network energy savings for NR, Dual Transmission/Reception (Tx/Rx) Multi-SIM for NR, NR MIMO evolution, Enhancement on NR QoE management and optimizations for diverse services, XR Enhancements for NR, TEI18, Rel-18 Other, Rel-19 General, AI/ML for NR air interface, Ambient IoT, AI/ML for Mobility, Additional topological enhancements, TEI19 - Diana Pani (Chair)

-	NR Sidelink evolution, Further NR mobility enhancements, Network Energy Saving Enh, Mobility Enhancement Ph4 - Kyeongin Jeong (VC)

-	NR MIMO evolution, Low-power wake-up signal and receiver for NR (LP-WUS/WUR), Evolution of NR duplex operation: Sub-band full duplex (SBFD), NR MIMO Phase 5, Number of UEs in RRC_INACTIVE state with data transmission, Support of 7Mhz channel bandwidth, CSSF optimization - Erlin Zeng (VC)

-	EUTRA corrections Rel-17 and earlier, Rel-17 NR stage 2, RRC and UE capabilities, IoT NTN enhancements, NR NTN enhancements, TEI18, NTN for NR Ph3, IoT NTN Ph3, IoT-NTN TDD mode - Sergio Parolari

-	Positioning corrections Rel-16 and earlier, Rel-15 and Rel-16 NR Positioning Support, NR positioning enhancements, Expanded and improved NR positioning, Enhanced NR Sidelink Relay, NR sidelink multi-hop relay, NavIC L1 SPS A-GNSS support, BDS B2b in A-GNSS, SRS frequency hopping, On-demand posSIB - Nathan Tenny

-	XR Enhancements Ph3 - Dawid Koziol

-	EUTRA corrections Rel-17 and earlier, NR Rel-15, Rel-16 and Rel-17 (except user plane), Further enhancement of data collection for SON MDT in NR and EN-DC, SON/MDT Ph4 - Mattias Bergström

The statistics from this meeting are:

-	385 f2f participants

-	1430 Tdoc numbers allocated with 1416 available contributions. (See the attached tdoc list)

-	58 incoming liaison statements, out of which 57 were noted. The remaining non-treated liaison will be treated in RAN2#129bis meeting.

-	24 outgoing liaison statements

-	52 at-meeting email discussions

-	78 email approvals/discussions scheduled after the RAN2#129bis meeting (17 short and 61 long), see Annex G for details.

Number of CRs or draftCRs submitted: 269. Out of these, 89 were agreed in principle. See Annex E for details.

# General

This meeting was an ordinary meeting and had full decision power, i.e. full decision power to make agreements and approvals according to RAN WG2 terms of reference, without any need to ratify decisions at a later RAN2 or other meeting.

# 1	Opening of the meeting

## 1.1	Call for IPR

The attention of the delegates of this Working Group is drawn to the fact that 3GPP Individual Members have the obligation under the IPR Policies of their respective Organizational Partners to inform their respective Organizational Partners of Essential IPRs they become aware of. The delegates were asked to take note that they were hereby invited: - to investigate whether their organization or any other organization owns IPRs which were, or were likely to become Essential in respect of the work of 3GPP. - to notify their respective Organizational Partners of all potential IPRs, e.g., for ETSI, by means of the IPR Statement and the Licensing declaration forms (https://www.etsi.org/images/files/IPR/etsi-ipr-form.doc)

NOTE:	IPRs may be declared to the Director-General or Chairman of the SDO, but not to the RAN WG2 Chairman.

## 1.2	Network usage conditions

1/ 	To avoid email system overload, please don’t attach files and documents to emails e.g. for offline email discussions, but instead use files placed on the meeting server instead. Inbox/Drafts folder is used for meeting offline discussions.

## 1.3	Other

In accordance with the Working Procedures it is reaffirmed that: (i) compliance with all applicable antitrust and competition laws is required; (ii) timely submissions of work items in advance of TSG or WG meetings are important to allow for full and fair consideration of such matters; and (iii) the chair will conduct the meeting with strict impartiality and in the interests of 3GPP

Note on (i): In case of question please contact your legal counsel.

Note on (ii): WIDs don’t need to be submitted to the RAN2 meeting and will typically not be discussed here either.

# General

## 2.1	Approval of the agenda

R2-2501701	Agenda for RAN2#129bis	Chairman	agenda

Approved

## 2.2	Approval of the report of the previous meeting

R2-2501702	RAN2#129 Meeting Report	MCC	report

Approved

## 2.3	Reporting from other meetings

## 2.4	Instructions

CRs

- Use latest CR template version 12.3 for all CRs submitted to RAN2 meeting

Rel-18 and earlier maintenance CRs

- Only essential/critical corrections are expected

- Editorial and clarification corrections should be sent to be reviewed and approved by spec rapporteurs prior to submission.

- Editorials corrections should be collected and submitted by spec rapporteurs.

- NOTE: the tdoc limit applies to all CRs (i.e. WI spec rapporteurs are NO longer expected to submit individual contributions).  They can submit a company CR where they also include miscellaneous corrections that have been sent to them.

Rel-18 UE capabilities

-	EUTRA UE capabilities corrections are covered by separate CRs

-	RAN1/RAN4 NR UE capabilities (new) and corrections are covered in Rel-18 common MegaCRs (38306 and 38331) covering all rel-18 WIs (end outcome).

-	UE capabilities in LPP 37355 and SLPP 38355 are covered in the main CRs for the Positioning WI.

During the work on NR UE caps:

-	In a Common Rel-18 Agenda Item (AI): RAN1 and RAN4 feature corrections are handled jointly under a common AI, with some explicit exceptions. Running UE cap MegaCRs are maintained for the parts handled in the common AI.

-	In WI-specific Rel-18 Agenda Items: RAN2 features/corrections are handled per WI and agreed as individual CRs

Tdoc limitations

Tdoc limitations doesn’t apply to Rapporteur Input, i.e.

-	Assigned summary rapporteur input of the summary.

-	Email / offline discussions outcomes by discussion rapporteur,

-	Limit of 1 WI/SI  rapporteurs input for WI planning.  The work plan is not expected to be updated/submitted every meeting, unless needed.   It can include progress of other WG groups in the same Tdoc (i.e. separate Tdocs on other WG agreements are not required).

-	TS rapporteur input for TS maintenance.

-	Contact Company of a LSin that triggers RAN2 action may submit one tdoc to facilitate the LS reply. This only applies to one of the contact companies in case there are several (default the first).

Tdoc limitations doesn’t apply to Input created at the meeting, revisions, assigned documents etc.

Tdoc limitations doesn’t apply to shadow / mirror CRs (Cat A), or In-Principle Agreed CRs.

Tdoc limitations applies to all other submitted tdocs (e.g. discussion tdoc and CR tdoc are counted as two).

Postponed CRs still count towards tdoc limit unless 3 or more companies are co-sourcing it.

Tdoc request/submission for RAN2#129bis deadlines:

- Tdoc Submission deadline: Mar. 28th, 1000 UTC

## 2.5	Others

Including new spec handling aspecs and output of [POST129][031][Git] Support email thread (Ericsson) and [POST129][032][ASN.1] ASN.1 review process (Nokia)

R2-2501703	RAN2 Handbook	MCC	discussion

Noted

Improvements to specification handling

R2-2502548	Further aspects on  improvements to specification handling	Ericsson	discussion

Observation 1	No major change to the current ways of working with CRs is needed if Gitlab is used.

Observation 2	All the currently used elements of 3GPP specifications can be handled within Gitlab, for some elements the handling will be simpler e.g. ASN.1 and advanced tables.

Observation 3	There are multiple ways of working with offline files for internal preparation and use in the Git universe.

Observation 4	If needed, it would be possible to create Word-documents of the Markdown specifications using open and free tools such as Pandoc.

-	Nokia notes that we have to still have further discussions on the repository and how we manage all the specs etc and we should continue discussions.  More companies are encouraged to participate.

Noted

R2-2501885	Discussion on Git-based specification handling approach	OPPO	discussion

Proposal 1	R2 summarize the discussion / evaluation on Git tool(s) (e.g., via Chairman Report or LS), and report to RAN plenary for further decision (e.g., trial period for other WGs, and finally whether / how to adopt the new Git tools).

Proposal 2	When R2 evaluate the new Git based framework, following aspects / questions have to be taken into account:

1) Accessibility of ETSI Forget Tools: Evaluate the accessibility and access-speed from all over the world, e.g., via a load testing when a large number of users access the platform;

2) Readability and Comprehensibility: Evaluate the readability and comprehensibility, including whether to limit Git to ASN.1 coding only or apply to CR and the whole specification covering procedural text as well (and if yes, how to handle CR specifically);

3) Impact analysis inside/outside R2: Collect view from companies regarding whether impact to the coming R2 (and other RAN WGs) work for 6G, and impact to product development / market needs outsides of 3GPP.

Noted

R2-2502174	On specification file formats and workflows	Apple	discussion

Observation 1: Markdown is a simple, clear and easy-to-use language which would be a major improvement compared to MS Word.

Observation 2: Markdown by itself is not sufficient for 3GPP needs; if we adopt Markdown we will also need to adopt LaTeX for formulas, CSS for styles, MSC/mermaid for figures and possibly additional languages.

Observation 3: a solution for tables in Markdown is yet to be found.

Observation 4: between MSC and Mermaid, Mermaid is preferred as it has built-in support for “packet figures” (besides sequence diagrams and flow charts) and because it can be embedded into a Markdown document; other formats e.g. PlantUML should also be considered.

Observation 5: some figures cannot be modeled even with Mermaid and so a vector graphics format (e.g. SVG) would also be needed at least in some cases.

Observation 6: LaTeX is more complex than Markdown and the “source” is less readable.

Observation 7: LaTeX is a self-sufficient solution which can address all our needs (including math, styles, figures, complex tables) without relying on supplementary tools and languages.

Observation 8: both approached of keeping ASN.1 embedded in the document and storing it in a separate files have pros and cons that need to be discussed further.

Observation 9: RAN2 trial indicates that we’ve largely figured out the spec maintenance and CR approval process with git and Gitlab.

Observation 10: we are yet to figure out a process of CR and handling during a meeting.

Observation 11: tools currently used to produce and maintain 3GPP specifications are not open and not multi-platform.

Noted

Discussions

-	CATT thinks that we can stop the RAN2 discussions and then rely on plenary discussions.  Xiaomi agrees with CATT

-	Xiaomi thinks that we are moving to fast as we didn’t even agree what issue we are addressing.

-	ZTE agrees but thinks that we should focus on what is different from other groups, like ASN.1 and we should separate the ASN.1 code and but the code directly in gitlab.

-	Oppo thinks that we should have a clear objective/target from RAN2 perspective.

-	Samsung thinks that we need more training and timing of these tools.

-	Qualcomm thinks after trying they are more open.  We should test in RAN2 and not have plenary mandate, the people that do the work should be testing and do exercise.

-	Huawei agrees that we haven’t agreed on what issues we are addressing and we don’t know the purpose of new things being tried.   We should consider what we are trying to achieve, for example if it is to make merge better then MCC should be looking into it.

# 3	Incoming liaisons

Note: LSs are moved to the respective agenda items if any.

# 4	EUTRA Rel-17 and earlier

Only essential corrections. No documents should be submitted to 4. Please submit to 4.x

## 4.1	EUTRA corrections Rel-17 and earlier

(NB_IOTenh4_LTE_eMTC6-Core; leading WG: RAN1; REL-17; WID: RP-211340)

(UPIP_EN-DC_UE; leading WG: RAN3; REL-17; WID: RP-213669)

(LTE TEI17)

Essential corrections to LTE Rel-17 topics not covered by other agenda items.

(NB_IOTenh3-Core; leading WG: RAN1; REL-16; started: Jun 18; Completed: June 20; WID: RP-200293); REL-15 and Earlier NB-IoT WIs are in scope but not listed explicitly (long list).

(LTE_eMTC5-Core; LTE_eMTC5-Core; leading WG: RAN1; REL-16; started: Jun 18; Completed:  June 20; WID: RP-192875;), REL-15 and Earlier eMTC WIs are in scope but not listed explicitly (long list).

(LTE_feMob-Core; leading WG: RAN2; REL-16; started: Jun 18; Completed: June 20; WID: RP-190921);

(LTE_terr_bcast-Core, LTE_DL_MIMO_EE-Core, LTE_high_speed_enh2-Core; LTE TEI16 Non-positioning);

(LTE_NBIOT_eMTC_NTN; leading WG: RAN1; REL-17; WID: RP-211601)

REL-16 and Earlier EUTRA WIs are in scope but not listed explicitly (long list), Except Positioning WI, which is addressed by AIs below.

NOTE that LTE corrections related to NR WIs or Joint NR LTE WIs should be submitted to NR AIs below.

NOTE that LTE corrections which are the same as an NR correction should be submitted to the respective NR AI (so the NR CR and LTE CR can be treated together).

This Agenda Item is treated in the Maintenance Breakout session (Corrections for LTE_NBIOT_eMTC_NTN might be treated in the NTN breakout session)

Report on leave A4/A5

R2-2502330	Clarification on A4 A5 ReportOnLeave	ZTE Corporation, Sanechips	discussion	Rel-15	LTE_Aerial-Core

Proposal 1	To define A4/A5 ReportOnLeave as “optional feature without UE capability signalling”, agree the CRs in [1]~[4].

R2-2502331	Introducing UE capability for A4 A5 ReportOnLeave	ZTE Corporation, Sanechips	CR	Rel-15	36.306	15.12.0	1907	-	F	LTE_Aerial-Core

R2-2502332	Introducing UE capability for A4 A5 ReportOnLeave	ZTE Corporation, Sanechips	CR	Rel-16	36.306	16.13.0	1908	-	A	LTE_Aerial-Core

R2-2502333	Introducing UE capability for A4 A5 ReportOnLeave	ZTE Corporation, Sanechips	CR	Rel-17	36.306	17.8.0	1909	-	A	LTE_Aerial-Core

R2-2502334	Introducing UE capability for A4 A5 ReportOnLeave	ZTE Corporation, Sanechips	CR	Rel-18	36.306	18.4.0	1910	-	A	LTE_Aerial-Core

-	Xiaomi want a capa indication. MediaTek and Qualcomm agrees. ZTE think that if someone already implement but would then (of course) not have the capa indication. But if no one implemented then we can do the capa indication. Huawei also want a capa indication also.

-	Samsung agrees with the intention but need more time. ZTE think this is urgent and want a post meeting email disc.

We assume we will go with optional with UE capability indication, CRs to be seen next meeting.

timeUntilReconnection

R2-2502528	Correction on timeUntilReconnection in RLF report	ZTE Corporation, Sanechips	CR	Rel-17	36.331	17.12.0	5108	-	F	NR_ENDC_SON_MDT_enh-Core

R2-2502529	Correction on timeUntilReconnection in RLF report	ZTE Corporation, Sanechips	CR	Rel-18	36.331	18.5.0	5109	-	A	NR_ENDC_SON_MDT_enh-Core

-	Xiaomi think there are something missed when porting this change from NR specs. Ericsson and Qualcomm agrees. ZTEs intention was that the procedure should be the same.

-	Xiaomi think the failed PCell field wonder if it indeed can reflect the first failure.

[AT129bis][602][Maint] Correction on timeUntilReconnection in RLF report (ZTE)

Scope:

    - Produce agreeable CRs

Intended outcome:

    - In principle agreed CRs in R2-2503102 and R2-2503103

Deadline:

    - Wednesday 17:00. The intention is to agree over email.

R2-2503116 	Correction on timeUntilReconnection in RLF report	ZTE Corporation, Xiaomi, Sanechips	CR	Rel-16	36.331	16.19.0	5113	-	F	NR_SON_MDT-Core

R2-2503102	Correction on timeUntilReconnection in RLF report	ZTE Corporation, Xiaomi, Sanechips	CR	Rel-17	36.331	17.12.0	5108	1	F	NR_ENDC_SON_MDT_enh-Core

R2-2503103	Correction on timeUntilReconnection in RLF report	ZTE Corporation, Xiaomi, Sanechips	CR	Rel-18	36.331	18.5.0	5109	1	A	NR_ENDC_SON_MDT_enh-Core

The above three are in-principle agreed

R2-2502561	Correction to explicit indication of epoch in SIB31	Nordic Semiconductor ASA	discussion	Rel-17

Alt1: the “next upcoming SFN” means any SFN after SFN n that is carrying last subframe of last repetition of message within SI window until SFN n+1023.

Alt2: the “next upcoming SFN” means any SFN after SFN n being the last subframe of SI window until SFN n+1023.

-	Xiaomi thinks a clarification is not needed

-	QC thinks interpretation 2 is correct but if we have a change we should go for a different formulation

-	MTK supports the intention and prefers to align to interpretation 2

-	Ericsson and Apple agree with Xiaomi that we don’t need further clarification. Apple thinks the same applies to NR as well.

-	Nordic suggests the following clarifications:

-	startSFN cannot be in the past from the UE point of view

-	startSFN can be in the future up to 1023 SFNs (according to E/// this was also RAN1 original intention)

-	proper network configuration solves the issue, i.e. startSFN is set to SFN of the last repetition within the SI window or a later SFN

-	QC thinks this could be solved by saying that the network never configures startSFN in the

Past

RAN2 understands that the network never configures startSFN in the past for the serving cell (can come back to check if a clarification in the spec is also useful)

deprioritisationReq

R2-2502836	Correction on deprioritisationReq delete	Google	CR	Rel-18	36.331	18.5.0	5111	-	F	LTE-L23, TEI11

-	Docomo think the CR content is clear from 36.304. And if the timer is not stopped, there is no problem. Google agrees that 36.304 already has this info. Samsung agrees with Docomo and its clear that PLMN selection is perfomed the field is released. Huawei think the UE behaviour is clear from 304 but didn’t find any description of stopping the timer and think there is no need to capture this. Qualcomm wonders if the CR changes UE behaviour? If it changes UE behaviour Qualcomm is not OK with it.

Not pursued

## 4.3	Positioning corrections Rel-16 and earlier

(LTE_NavIC-Core, LTE TEI16 Positioning), REL-15 and Earlier WIs related to positioning are in scope but not listed explicitly (long list).

Tdoc Limitation: 1 tdoc

# 5	NR Rel-15 and Rel-16

Essential corrections only.

Tdoc Limitation: 2 tdocs in total for all sub agenda items

In case a correction need to be reflected in both NR TS and LTE TS, the corrections should be submitted under one single AI (so the NR and LTE correction can be treated together), the sub-Ais below this

## 5.1	Common

Includes the following WIs and input that doesn’t fit elsewhere.

(NR_newRAT-Core; leading WG: RAN1; REL-15; started: Mar. 17; closed: Jun. 19: WID: RP-191971)

(NR_IAB-Core; leading WG: RAN2; REL-16; started: Dec 18; target Aug 20; WID: RP-200840)

(NR_unlic-Core; leading WG: RAN1; REL-16; started: Dec 18; Closed June 20; WID: RP-192926).

(NR_IIOT-Core; leading WG: RAN2; REL-16; started: Mar 19; Completed: Jun 20; WID: RP-200797)

(NR_UE_pow_sav-Core; leading WG: RAN1; REL-16; started: Mar 19; Completed Jun 20; WID: RP-200494).

(NR_2step_RACH-Core; leading WG: RAN1; REL-16; started: Dec 18; Completed: June 20; WID: RP-200085).

(SRVCC_NR_to_UMTS-Core; leading WG: RAN2; REL-16; started: Dec 18; Completed; Mar 20; WID: RP-190713)

(RACS-RAN-Core, leading WG: RAN2; REL-16; started: Mar 19; completed: Jun 20; WID: RP-191088)

(NG_RAN_PRN-Core; leading WG: RAN3; REL-16; started: Mar 19; completed: June 20; WID: RP-200122)

(NR_eMIMO-Core, leading WG: RAN1; REL-16; started: Jun 18; target; Aug 20; WID: RP-200474)

(NR_CLI_RIM; leading WG: RAN1; REL-16; started: Dec 18; Completed: Jun 20; WID: RP-191997)

(NR_L1enh_URLLC-Core, leading WG: RAN1; REL-16; Completed: June 20; WID: RP-191584)

(LTE_NR_DC_CA_enh-Core; leading WG: RAN2; REL-16; started: Jun 18; Target Aug 20; WI RP-200791)

(NR_Mob_enh-Core; leading WG: RAN2; REL-16; started: Jun 18; Completed June 20; WID: RP-192277).

(NR_SON_MDT-Core; leading WG: RAN3; REL-16; started: Jun 19; Completed June 20; WID: RP-191776)

(5G_V2X_NRSL-Core; leading WG: RAN1; REL-16; started: Mar 19; completed; Aug 20; WID: RP-200129)

(NR_HST, NR_RRM_enh-Core, NR_RF_FR1, NR_RF_FR2_req_enh, NR_n66_BW, LTE_NR_B41_Bn41_PC29dBm-Core, NR_CSIRS_L3meas,)

(NR TEI16)

LTE mob enh corrections that are common with NR mobility enhancements should be submitted to this AI.

### 5.1.1	Stage 2 and Organisational

Incoming LSs, etc. You should discuss your stage 2 CRs with the specification rapporteurs before submission. Includes impact to 38.300, 36.300, 37.340

DCP

R2-2502473	Corrections on DCP	Huawei, HiSilicon, Nokia (Rapporteur)	CR	Rel-16	38.300	16.19.0	0983	-	F	NR_UE_pow_sav-Core

R2-2502474	Corrections on DCP	Huawei, HiSilicon, Nokia (Rapporteur)	CR	Rel-17	38.300	17.12.0	0984	-	A	NR_UE_pow_sav-Core

R2-2502475	Corrections on DCP	Huawei, HiSilicon, Nokia (Rapporteur)	CR	Rel-18	38.300	18.5.0	0985	-	A	NR_UE_pow_sav-Core

-	Xiaomi is OK with the first change. Samsung think this is clear from Stage-3. Vivo think the first change is OK. LG think the first change is not needed since its clear from Stage-3. ZTE think its clear from stage-3. Nokia supports the CR.

-	Xiaomi think that for the second change they want to clarify that its NR PCell/PSCell. Samsung is OK with the second change. Ericsson supports this change and if the second change is made then the first can be fixed at the same time.

The above three in-principle agreed

### 5.1.2	User Plane corrections

User Plane corrections will be handled in the User Plane break out session

#### 5.1.2.1	MAC

R2-2502992	Clarification on the power ramping offset for 2-step RACH switching to 4-step RACH	OPPO	CR	Rel-16	38.321	16.19.0	2072	-	F	NR_2step_RACH-Core

-	Huawei doesn’t think this is needed and it is NBC.  Vivo agrees with Huawei.

-	Qualcomm thinks this CR is agreable.   MEdiatek agrees with Huawei that this has UE implementation impact.

-	ZTE thinks that we can just delete the whole bracket and reference to the RAN1 spec.  This is more of a clarification.  LG, Qualcomm, and Lenovo would agree with the deletion of the bracket.

The CR is postponed

R2-2502993	Clarification on the power ramping offset for 2-step RACH switching to 4-step RACH	OPPO	CR	Rel-17	38.321	17.12.0	2073	-	A	NR_2step_RACH-Core

R2-2502994	Clarification on the power ramping offset for 2-step RACH switching to 4-step RACH	OPPO	CR	Rel-18	38.321	18.5.0	2074	-	A	NR_2step_RACH-Core

#### 5.1.2.2	RLC PDCP SDAP BAP

### 5.1.3	Control Plane corrections

#### 5.1.3.1	NR RRC

Corrections to 38331, and related change to other TS if applicable, e.g. 36331, Stage-2 etc.

SCG failure information

R2-2501906	Correction on Initiation of NR SCG failure information	CATT	CR	Rel-16	36.331	16.19.0	5102	-	F	LTE_NR_DC_CA_enh-Core

R2-2501907	Correction on Initiation of NR SCG failure information	CATT	CR	Rel-17	36.331	17.12.0	5103	-	F	LTE_NR_DC_CA_enh-Core

R2-2501908	Correction on Initiation of NR SCG failure information	CATT	CR	Rel-18	36.331	18.5.0	5104	-	F	LTE_NR_DC_CA_enh-Core

-	CATT suggests to merge to rapp CR next meeting. Qualcomm agrees but highlights its submitted to the wrong Agenda Item.

To be merged in rapp CR

URLLC

R2-2502271	Correction to nrofCandidates-CI for DCI format 2_4	Huawei, HiSilicon	CR	Rel-16	38.331	16.19.0	5295	-	F	NR_L1enh_URLLC-Core

R2-2502272	Correction to nrofCandidates-CI for DCI format 2_4	Huawei, HiSilicon	CR	Rel-17	38.331	17.12.0	5296	-	A	NR_L1enh_URLLC-Core

R2-2502273	Correction to nrofCandidates-CI for DCI format 2_4	Huawei, HiSilicon	CR	Rel-18	38.331	18.5.1	5297	-	A	NR_L1enh_URLLC-Core

-	Samsung supports the CR. Ericsson think the CR is NBC and this needs to be stated.

All in-principle agreed, but next version should not have curly quotes and next version of the CR should state on the cover page that NWs wanting to support this feature must implement this CR.

NZP CSI-RS resources

R2-2502344	Correction on number of NZP CSI-RS resources	Ericsson	CR	Rel-15	38.331	15.28.0	5298	-	F	NR_newRAT-Core

R2-2502389	Correction on number of NZP CSI-RS resources	Ericsson	CR	Rel-16	38.331	16.19.0	5299	-	A	NR_newRAT-Core

R2-2502390	Correction on number of NZP CSI-RS resources	Ericsson	CR	Rel-17	38.331	17.12.0	5300	-	A	NR_newRAT-Core

R2-2502391	Correction on number of NZP CSI-RS resources	Ericsson	CR	Rel-18	38.331	18.5.1	5301	-	A	NR_newRAT-Core

In-principle agreed or merged to rapp CR if there is one in the next meeting.

Spatial Relation Info

R2-2502884	Issues on PUCCH Spatial Relation Info Configuration	ZTE, Samsung	discussion	Rel-16

Proposal 1: RAN 2 is asked to select one of the following solutions:

-	Solution 1: Dummify size extension list, configuration extension release list, and the PUCCH-SpatialRelationInfoId-r16, and re-introduce a size extension ToAddMod and ToRelease list where the ToRelease list is the list of PUCCH-SpatialRelationInfoId-v1610.

-	Solution 2: Add a clarification in the field description to clarify that either PUCCH-SpatialRelatioinInfoId-Id or PUCCH-SpatialRelationInfo-Id-v1610 is considered as indicated if UE receiving the spatialRelationInfoToReleaseListExt-v1610

Proposal 2: The UE shall consider entries in spatialRelationInfoToAddModList and in spatialRelationInfoToAddModListSizeExt as a single list if the spatialRelationInfoToAddModListExt is not included, i.e. an entry created using spatialRelationInfoToAddModList can be modified using spatialRelationInfoToAddModListSizeExt (or deleted using spatialRelationInfoToReleaseListSizeExt) and vice-versa.

On P1

-	Xiaomi prefers Solution 1. Huawei think the spec is clear and it works so no need to update the spec. MediaTek agrees with Huawei. CATT think there is an issue to be solved and prefers Solution 1. Ericsson are unsure what the issue would be, they think the current spec works but perhaps some polishing of a field description could be considered. ZTE think its unclear how to release the elements in the list. Samsung supports the proposal and think this list is not following the guidelines. Huawei think it doesn’t matter which release list sends an ID, the UE shall release it, UE behaviour is clear and this is already implemented in products and we cannot change it. Huawei finds Solution 1 NBC and not acceptable, and solution 2 is not needed. Perhaps some small polishing can be done, in Huaweis mind. ZTE think we can capture something in minutes.

On P2:

-	CATT think the spec is clear.

Postponed, but we will not consider any ASN.1 changes.

#### 5.1.3.2	UE capabilities

UE cap corrections 38306, 38331

interBandMRDC-WithOverlapDL-Bands

R2-2502505	Type clarification for interBandMRDC-WithOverlapDL-Bands-r16	Apple, Huawei, HiSilicon, xiaomi	CR	Rel-16	38.306	16.20.0	1251	-	F	NR_newRAT-Core, TEI16

R2-2502506	Type clarification for interBandMRDC-WithOverlapDL-Bands-r16	Apple, Huawei, HiSilicon, xiaomi	CR	Rel-17	38.306	17.12.0	1252	-	A	NR_newRAT-Core, TEI16

R2-2502507	Type clarification for interBandMRDC-WithOverlapDL-Bands-r16	Apple, Huawei, HiSilicon, xiaomi	CR	Rel-18	38.306	18.5.0	1253	-	A	NR_newRAT-Core, TEI16

In-principle agreed or merged to rapp CR if there is one in the next meeting.

IAB

R2-2502631	Correction to pdsch-256QAM-FR1 for IAB-MT	Nokia	CR	Rel-16	38.306	16.20.0	1259	-	F	NR_IAB-Core

R2-2502632	Correction to pdsch-256QAM-FR1 for IAB-MT	Nokia	CR	Rel-17	38.306	17.12.0	1260	-	A	NR_IAB-Core

R2-2502633	Correction to pdsch-256QAM-FR1 for IAB-MT	Nokia	CR	Rel-18	38.306	18.5.0	1261	-	A	NR_IAB-Core

-	Huawei think the first change is not needed since there is a table clarifying what IAB nodes support and not. But Huawei is OK to IPA now and then come back if needed.

In-principle agreed, but “pdsch-256QAM-FR1” should be italic.

SRS-only cells

R2-2502991	Consideration on Introduction of SRS Capability Reporting for SRS-only cell	ZTE Corporation	discussion	Rel-15	NR_newRAT-Core

Proposal 1: Introduce the below SRS capabilities on the FeatureSetDownlink from the R18 and allow the early implementation.

o	zeroSlotOffsetAperiodicSRS;

o	offsetSRS-CB-PUSCH-Ant-Switch-fr1-r16;

o	offsetSRS-CB-PUSCH-PDCCH-MonitorSingleOcc-fr1-r16;

o	offsetSRS-CB-PUSCH-PDCCH-MonitorAnyOccWithoutGap-fr1-r16;

o	offsetSRS-CB-PUSCH-PDCCH-MonitorAnyOccWithGap-fr1-r16;

o	offsetSRS-CB-PUSCH-PDCCH-MonitorAnyOccWithSpanGap-fr1-r16;

o	srs-AntennaSwitching2SP-1Periodic-r17;

o	srs-ExtensionAperiodicSRS-r17;

o	srs-OneAP-SRS-r17

Proposal 2: Add a precondition to the newly introduced capability in the FeatureSetDownlink, e.g. it is only present when the UE supports SRS carrier switching from this band.

On P1:

-	Huawei think its easier for implementation to go from R15. Nokia think R17 is fine. Ericsson think usually we change from R15 in these cases. Xiaomi think R15 is fine and better for implementations.

On P2:

-	Huawei do not think we need this. OPPO thought this proposal should be agreeable. Huawei think there is no need for this.

R2-2502891	Correction on SRS capability reporting	Huawei, HiSilicon, Ericsson, OPPO	CR	Rel-15	38.331	15.28.0	5324	-	F	NR_newRAT-Core

R2-2502892	Correction on SRS capability reporting	Huawei, HiSilicon, Ericsson, OPPO	CR	Rel-16	38.331	16.19.0	5325	-	A	NR_newRAT-Core

R2-2502893	Correction on SRS capability reporting	Huawei, HiSilicon, Ericsson, OPPO	CR	Rel-17	38.331	17.12.0	5326	-	A	NR_newRAT-Core

R2-2502894	Correction on SRS capability reporting	Huawei, HiSilicon, Ericsson, OPPO	CR	Rel-18	38.331	18.5.1	5327	-	A	NR_newRAT-Core

R2-2502895	Correction on SRS capability reporting	Huawei, HiSilicon, Ericsson, OPPO	CR	Rel-15	38.306	15.27.0	1268	-	F	NR_newRAT-Core

R2-2502896	Correction on SRS capability reporting	Huawei, HiSilicon, Ericsson, OPPO	CR	Rel-16	38.306	16.20.0	1269	-	A	NR_newRAT-Core

R2-2502897	Correction on SRS capability reporting	Huawei, HiSilicon, Ericsson, OPPO	CR	Rel-17	38.306	17.12.0	1270	-	A	NR_newRAT-Core

R2-2502898	Correction on SRS capability reporting	Huawei, HiSilicon, Ericsson, OPPO	CR	Rel-18	38.306	18.5.0	1271	-	A	NR_newRAT-Core

[AT129bis][603][Maint] Correction on SRS capability reporting (Huawei)

Scope:

    - Discuss offline (face-to-face preferred, if suitable)

    - Produce agreeable CRs

Intended outcome:

    - In principle agreed CRs in R2-2503104 - R2-2503111

Deadline:

    - Wednesday 17:00. The intention is to agree over email.

R2-2503104	Correction on SRS capability reporting	Huawei, HiSilicon, Ericsson, OPPO	CR	Rel-15	38.331	15.28.0	5324	1	F	NR_newRAT-Core

In-principle agreed

R2-2503105	Correction on SRS capability reporting	Huawei, HiSilicon, Ericsson, OPPO	CR	Rel-16	38.331	16.19.0	5325	1	A	NR_newRAT-Core

In-principle agreed, but should be Cat F

R2-2503106	Correction on SRS capability reporting	Huawei, HiSilicon, Ericsson, OPPO	CR	Rel-17	38.331	17.12.0	5326	1	A	NR_newRAT-Core

In-principle agreed, but should be Cat F

R2-2503107	Correction on SRS capability reporting	Huawei, HiSilicon, Ericsson, OPPO	CR	Rel-18	38.331	18.5.1	5327	1	A	NR_newRAT-Core

In-principle agreed, but should be Cat F

R2-2503108	Correction on SRS capability reporting	Huawei, HiSilicon, Ericsson, OPPO	CR	Rel-15	38.306	15.27.0	1268	1	F	NR_newRAT-Core

In-principle agreed

R2-2503109	Correction on SRS capability reporting	Huawei, HiSilicon, Ericsson, OPPO	CR	Rel-16	38.306	16.20.0	1269	1	A	NR_newRAT-Core

In-principle agreed, but should be Cat F

R2-2503110	Correction on SRS capability reporting	Huawei, HiSilicon, Ericsson, OPPO	CR	Rel-17	38.306	17.12.0	1270	1	A	NR_newRAT-Core

In-principle agreed, but should be Cat F

R2-2503111	Correction on SRS capability reporting	Huawei, HiSilicon, Ericsson, OPPO	CR	Rel-18	38.306	18.5.0	1271	1	A	NR_newRAT-Core

In-principle agreed, but should be Cat F

Old revisions

R2-2502865	Consideration on Introduction of SRS Capability Reporting for SRS-only cell	ZTE Corporation	discussion	Rel-15	NR_newRAT-Core

#### 5.1.3.3	Other

This agenda item addresses the idle and inactive behaviour specified in 38.304 or 36.304, LTE-specific changes for the applicable WIs, Other parts not covered elsewhere.

SUL

R2-2502011	Clarification on usage of q-RxLevMinSUL	Qualcomm Incorporated (Rapporteur)	CR	Rel-15	38.304	15.8.0	0429	-	F	NR_newRAT-Core

R2-2502012	Clarification on usage of q-RxLevMinSUL	Qualcomm Incorporated (Rapporteur)	CR	Rel-16	38.304	16.11.0	0430	-	A	NR_newRAT-Core

R2-2502013	Clarification on usage of q-RxLevMinSUL	Qualcomm Incorporated (Rapporteur)	CR	Rel-17	38.304	17.10.0	0431	-	A	NR_newRAT-Core

R2-2502015	Clarification on usage of q-RxLevMinSUL	Qualcomm Incorporated (Rapporteur)	CR	Rel-18	38.304	18.4.0	0432	-	A	NR_newRAT-Core

-	OPPO think the intention is OK but only want to remove “if present”, Samsung agrees. ZTE think no change is needed and do not think there should be a problem. Vivo and Xiaomi think no change is needed.

Postponed to next meeting

MDT

R2-2502404	Correction to Immediate MDT measurements	Nokia	CR	Rel-16	37.320	16.8.0	0137	-	F	NR_SON_MDT-Core

R2-2502405	Correction to Immediate MDT measurements	Nokia	CR	Rel-17	37.320	17.5.0	0138	-	A	NR_SON_MDT-Core

R2-2502406	Correction to Immediate MDT measurements	Nokia	CR	Rel-18	37.320	18.3.0	0139	-	A	NR_SON_MDT-Core

-	Qualcomm think that the text talks about what the standard supports, not what all UEs must support. Huawei is OK with the proposed CRs.

The above three are in-principle agreed

## 5.3	NR Positioning Support

(NR_newRAT-Core; leading WG: RAN1; REL-15; started: Mar. 17; closed: Jun. 19: WID: RP-191971)

(NR_pos-Core; leading WG: RAN1; REL-16; started: Mar 19; target; Jun 20; WID: RP-200218).

(NR TEI16 Positioning)

Stage 2 corrections shall be discussed with the specification rapporteur (Sven Fischer sfischer@qti.qualcomm.com) before submission. Stage 2 CRs not discussed with the specification rapporteur will not be treated.

R2-2502922	Correction for equalIntegerAmbiguity Request from UE	Ericsson	CR	Rel-16	37.355	16.14.0	0550	-	F	NR_pos-Core

Not pursued (can be discussed under TEI19 in future)

Discussion:

Samsung understand the motivation but wonder why there is no request for other information (antenna reference point uncertainty, etc.) and why this data point is special.

Xiaomi think it is an enhancement.

Nokia agree with Xiaomi and want to understand the consequences for the deployment if not approved.

CATT agree with Xiaomi and think not every field needs to be requested, but they would be open if there is a real requirement and it comes in TEI19.

Chair wonders what an updated network will do if a legacy UE does not send the request.

Ericsson are not sure of the deployment status, but they think some network logic could be implemented to distinguish legacy UEs.  On the consequence if not adopted, they understand that the server will always provide the information, resulting in unnecessary signalling.

Ericsson wonder if we could do it from Rel-18 as a correction.  Qualcomm think it is not a correction in Rel-18 either, and if we introduce it in a later release we create two different UE behaviours.  Qualcomm understand that if the information is not provided, it is not a catastrophe and the system does not break.

R2-2502923	Correction for equalIntegerAmbiguity Request from UE	Ericsson	CR	Rel-17	37.355	17.9.0	0551	-	A	NR_pos-Core

Not pursued (can be discussed under TEI19 in future)

R2-2502924	Correction for equalIntegerAmbiguity Request from UE	Ericsson	CR	Rel-18	37.355	18.4.0	0552	-	A	NR_pos-Core

Not pursued (can be discussed under TEI19 in future)

# 6	NR Rel-17

Essential corrections only.  Editorial/clarifications should be sent to be reviewed and approved by spec rapporteurs prior to submission.  Editorials should only be submitted by spec rapporteurs.

Tdoc limitation: 4 Tdocs

## 6.1	Common

(NR_MG_enh-Core; leading WG: RAN4; REL-17; WID: RP-211591)

(NR_UDC_enh-Core; leading WG: RAN2; REL-17; WID: RP-211203)

(NG_RAN_PRN_enh-Core; leading WG: RAN3; REL-17; WID: RP-202363)

(NR_IAB_enh-Core; leading WG: RAN2; REL-17; WID: RP-211548)

(NR_UE_pow_sav_enh-Core; leading WG: RAN2; REL-17; WID: RP-212630)

(LTE_NR_DC_enh2-Core; leading WG: RAN2; REL-17; WID: RP-201040)

(LTE_NR_MUSIM-Core; leading WG: RAN2; REL-17; WID: RP-212610)

(NR_Slice-Core; leading WG: RAN2; REL-17; WID: RP-212534)

(NR_QoE-Core; leading WG: RAN3; REL-17; WID: RP-211406)

(NR_ext_to_71GHz-Core; leading WG: RAN1; REL-17; WID: RP-212637)

(NR_cov_enh-Core; leading WG: RAN1; REL-17; WID: RP-211566): non-RACH-indication parts

(NR_redcap-Core; leading WG: RAN1; REL-17; WID: RP-211574)

(NR_feMIMO-Core; leading WG: RAN1; REL-17; WID: RP-212535)

(NR_SmallData_INACTIVE-Core, leading WG: RAN2; REL-17; WID: RP-212594)

(NR_IIOT_URLLC_enh-Core; leading WG: RAN2; REL-17; WID: RP-210854)

(NR_MBS-Core; leading WG: RAN2; REL-17; WID: RP-201038)

(NR_ENDC_SON_MDT_enh-Core; leading WG: RAN3; REL-17; WID: RP-201281)

(NR_NTN_solutions-Core; leading WG: RAN2; REL-17; WID: RP-211557)

(NR_SL_enh-Core; leading WG: RAN1; REL-17; WID: RP-202846)

(NR_SL_Relay-Core; leading WG: RAN2; REL-17; WID: RP-212601)

PRACH partitioning items

(NR TEI17)

Includes Rel-17 Work Items without specific R2 Agenda Item, e.g. RAN1 and RAN4 led items, SA2 and CT1 led items (was previously “Rel-17 Other”)

Includes aspects that does not fit under the more specific AIs, e.g. multi-WI aspects.

Corrections for NR_NTN_solutions-Core might be treated in the NTN breakout session.

### 6.1.1	Stage 2 and Organisational

Incoming LSs, etc. You should discuss your stage 2 CRs with the specification rapporteurs before submission. Includes impact to 38.300, 37.340, (36.300 if applicable)

R2-2501704	LS on stage 1 requirements for the support for PWS over satellite NGRAN in Rel-17 (C1-250715; contact: Qualcomm)	CT1	LS in	Rel-17	5GSAT	To:SA1	Cc:SA2, CT4, RAN2, RAN3

Noted

- QC thinks it would be good to clarify in Stage 2 that PWS is not supported in Rel-17 and Rel-18

We wait for SA1 feedback before deciding to have any clarification in Stage 2

R2-2501752	Reply LS on emergency call back and paging (S2-2502427; contact: ZTE)	SA2	LS in	Rel-17	NR_newRAT-Core, NR_redcap-Core	To:RAN2	Cc:CT1, RAN3

HSDN

R2-2502441	Correction on cell type specific cell reselection prioritisation [NR_HSDN]	Huawei, HiSilicon, Nokia, Qualcomm, CMCC, Xiaomi, CATT, Apple	CR	Rel-17	38.300	17.12.0	0981	-	F	TEI17, NR_mobile_IAB-Core

R2-2502442	Correction on cell type specific cell reselection prioritisation [NR_HSDN]	Huawei, HiSilicon, Nokia, Qualcomm, CMCC, Xiaomi, CATT, Apple	CR	Rel-18	38.300	18.5.0	0982	-	A	TEI17, NR_mobile_IAB-Core

-	Samsung think only SA is impacted. Ericsson think the R17 CR should not have mobile IAB WI code. The date is also wrong. Docomo think we need separate CRs.

Both in-principle agreed, mobile IAB WI code should be removed for the R17 CR, and both should be cat F, and date should be correct. Or other approach if RAN2 secretary has good suggestions.

### 6.1.2	User Plane corrections

User Plane Related aspects will be handled in the User Plane break out session. (exception: TEI new proposals if any).

To be treated in SL breakout (notify Kyeongin)

R2-2502305	Correction to MAC on IUC	Ericsson, Apple, ZTE Corporation, Sanechips	CR	Rel-17	38.321	17.12.0	2061	-	F	NR_SL_enh-Core

R2-2502306	Correction to MAC on IUC	Ericsson, Apple, ZTE Corporation, Sanechips	CR	Rel-18	38.321	18.5.0	2062	-	A	NR_SL_enh-Core

R2-2502614	Correction on IAB related MAC CEs	ZTE Corporation, Samsung, Sanechips	CR	Rel-17	38.321	17.12.0	2066	-	F	NR_IAB_enh-Core

-	Ericsson thinks that the text is clear as it is.  Huawei also thinks that this is editorial so we shouldn’t encourage such CRs.

-	Samsung explains that IAB node definition doesn’t include IAB donor, and this means this is not correct.

The CR is in principle agreed

R2-2502615	Correction on IAB related MAC CEs	ZTE Corporation, Samsung, Sanechips	CR	Rel-18	38.321	18.5.0	2067	-	A	NR_IAB_enh-Core

SCG activation

R2-2502940	Correction on CFRA for SCG activation	Huawei, HiSilicon	CR	Rel-17	38.321	17.12.0	2070	-	F	LTE_NR_DC_enh2-Core

-	Apple and Ericsson thinks the intention is correct by there is nothing really broken

The CR is not pursued

R2-2502941	Correction on CFRA for SCG activation	Huawei, HiSilicon	CR	Rel-18	38.321	18.5.0	2071	-	A	LTE_NR_DC_enh2-Core

Power saving features

R2-2502654	Corrections on Power saving features in NTN	Qualcomm Incorporated	discussion	Rel-17	NR_NTN_solutions-Core

Proposal 1	Add Rel-17 late noncritical extension to signal MAC-ParametersFRX-Diff-r16 in NTN. Text proposal is provided above.

- Vivo supports this

- Ericsson thinks we’d better say that we don’t support DCP in NTN. QC could agree but still thinks we should signal separately for NTN

- Huawei supports introducing mac-ParametersNTN

- Nokia agrees with Ericsson but could accept p1

DCP is not supported in NTN (at least up to Rel-19). FFS how/whether we clarify this in the specs

Proposal 2	Clarify that in NTN, if drx-onDurationTimer would not be running prior to symbol n (i.e., UE is in DRX sleep prior to symbol n), it is up to UE implementation whether to wakeup from C-DRX sleep prior to symbol n to report periodic CSI.

Can come back next time

### 6.1.3	Control Plane corrections

Slot aggregation

R2-2501716	LS on Slot aggregation configuration with multi-PUSCH (R1-2501593; contact: Nokia)	RAN1	LS in	Rel-17	NR_ext_to_71GHz-Core	To:RAN2

Moved from 6.1.1

R2-2502920	Slot aggregation configuration with multi-PUSCH	Nokia	CR	Rel-17	38.331	17.12.0	5328	-	F	NR_ext_to_71GHz-Core

R2-2502921	Slot aggregation configuration with multi-PUSCH	Nokia	CR	Rel-18	38.331	18.5.1	5329	-	A	NR_ext_to_71GHz-Core

-	Nokia highlights that this is NBC.

Both above are in-principle agreed, but in the next version state on the cover page that UEs and NW supporting this feature must implement this CR.

#### 6.1.3.1	NR RRC

Corrections to 38331, and related change to other TS if applicable, except UE caps.

msg3-transmitPrecoder

R2-2502049	CR on the usage of msg3-transmitPrecoder	CATT, Huawei, HiSilicon	CR	Rel-17	38.331	17.12.0	5286	-	F	NR_SmallData_INACTIVE-Core, NR_cov_enh-Core, NR_redcap-Core, NR_slice-Core

R2-2502050	CR on the usage of msg3-transmitPrecoder	CATT, Huawei, HiSilicon	CR	Rel-18	38.331	18.5.1	5287	-	A	NR_SmallData_INACTIVE-Core, NR_cov_enh-Core, NR_redcap-Core, NR_slice-Core

R2-2502051	CR on the usage of RACH parameters in the case of fallbackRAR	CATT, Huawei, HiSilicon	CR	Rel-17	38.321	17.12.0	2055	-	F	NR_SmallData_INACTIVE-Core, NR_cov_enh-Core, NR_redcap-Core, NR_slice-Core

R2-2502052	CR on the usage of RACH parameters in the case of fallbackRAR	CATT, Huawei, HiSilicon	CR	Rel-18	38.321	18.5.0	2056	-	A	NR_SmallData_INACTIVE-Core, NR_cov_enh-Core, NR_redcap-Core, NR_slice-Core

-	Ericsson think there is no need to change the spec as we already told RAN1 in our LS to them, and then RAN1 said that they wont fix it either. CATT think RAN1 said they wont update their specs but we never said we wont update our spec. Xiaomi think we need these CRs, but the related MAC CRs are not needed. ZTE, LG and vivo agrees with Xiaomi.

-	Ericsson think the CR is NBC and the RRC CR says “if the field is not configured” and in other parts “if the field is absent”. Huawei think it makes sense in this case.

-	ZTE think the RRC spec already has similar wording covering this. Huawei want to have explicit wording on this particular field.

MAC CR is not pursued, the RRC CR can be revised offline.

[AT129bis][604][Maint] Usage of msg3-transmitPrecoder (CATT)

Scope:

- Produce agreeable CRs

Intended outcome:

    - In principle agreed CRs in R2-2503112 and R2-2503113

Deadline:

    - Wednesday 17:00. The intention is to agree over email.

R2-2503112	CR on the usage of msg3-transmitPrecoder	CATT, Huawei, HiSilicon	CR	Rel-17	38.331	17.12.0	5286	1	F	NR_SmallData_INACTIVE-Core, NR_cov_enh-Core, NR_redcap-Core, NR_slice-Core

R2-2503113	CR on the usage of msg3-transmitPrecoder	CATT, Huawei, HiSilicon	CR	Rel-18	38.331	18.5.1	5287	1	A	NR_SmallData_INACTIVE-Core, NR_cov_enh-Core, NR_redcap-Core, NR_slice-Core

Both in-principle agreed

R2-2502249	Correction on carrier frequency information in RRCConnectionRelease.	CATT, Qualcomm Incorporated	CR	Rel-17	36.331	17.12.0	5106	-	F	NR_NTN_solutions-Core

- Samsung wonders if we need the first change. Ericsson agrees

- Samsung thinks we should rather have a clarification in Stage2. MTK and Oppo agree

- QC thinks a clarification is needed and having this in Stage 2 would also be ok

Not pursued

We introduce a clarification in Stage2

R2-2502250	Correction on carrier frequency information in RRCConnectionRelease.	CATT, Qualcomm Incorporated	CR	Rel-18	36.331	18.5.0	5107	-	A	NR_NTN_solutions-Core

Not pursued

RedCap

R2-2502335	Correction on SCS and CP configuration in RedCap-specific initial BWP	ZTE Corporation, Sanechips, Vivo, Ericsson, MediaTek Inc, Qualcomm Incorporated, OPPO, Samsung	CR	Rel-17	38.331	17.12.0	5212	1	F	NR_redcap-Core	R2-2500439

R2-2502336	Correction on SCS and CP configuration in RedCap-specific initial BWP	ZTE Corporation, Sanechips, Vivo, Ericsson, MediaTek Inc, Qualcomm Incorporated, OPPO, Samsung	CR	Rel-18	38.331	18.5.1	5213	1	A	NR_redcap-Core	R2-2500440

-	Huawei and Nokia think RAN1 specs are already clear. ZTE said this was discussed offline but think there is a difference in RAN1 specs vs. this CR. Huawei need more time.

[AT129bis][605][Maint] SCS and CP configuration in RedCap-specific initial BWP (ZTE)

Scope:

    - Discuss offline to see if R2-2502335 and R2-2502336 should be in-principle agreed

Intended outcome:

    - Consensus of how to proceed with R2-2502335 and R2-2502336

Deadline:

    - Wednesday 17:00. The intention is to agree over email.

-	After the offline, ZTE reports that the original CRs are agreeable.

R2-2502335 and R2-2502336 are in-principle agreed

MIMO

R2-2502415	Correction on PUCCH spatial relation Activation/Deactivation for multiple TRP PUCCH repetition MAC CE	ZTE Corporation, Samsung	CR	Rel-17	38.321	17.12.0	2063	-	F	NR_FeMIMO-Core

-	Nokia doesn’t think this is needed

-	Ericsson thinks the correction is correct but not strong opinion whether it is needed.

The CR is postponed

R2-2502416	Correction on PUCCH spatial relation Activation/Deactivation for multiple TRP PUCCH repetition MAC CE	ZTE Corporation, Samsung	CR	Rel-18	38.321	18.5.0	2064	-	A	NR_FeMIMO-Core

-	Nokia think we should postpone.

Postponed

R2-2502519	Corrections to location-based measurement	ZTE Corporation, Ericsson, CATT, Sanechips	CR	Rel-17	38.331	17.12.0	5303	-	F	NR_NTN_solutions-Core

- QC thinks this might also have RAN5 impacts. Also wonders if we need to update 304. ZTE thinks 304 is fine.

RAN2 confirms that location-based measurements for both quasi-earth fixed cell scenario and earth-fixed cell scenario is supported since R17

Agreed in principle (with the parenthesis after the dash: “(quasi-)”)

Preamble partitioning

R2-2502754	Correction on rsrp-ThresholdSSB-r17 in TS 38.331(R17)	Huawei, HiSilicon	CR	Rel-17	38.331	17.12.0	5316	-	F	NR_redcap-Core, NR_cov_enh-Core, NR_slice-Core, NR_SmallData_INACTIVE-Core

R2-2502755	Correction on rsrp-ThresholdSSB-r17 in TS 38.331(R18)	Huawei, HiSilicon	CR	Rel-18	38.331	18.5.1	5317	-	A	NR_redcap-Core, NR_cov_enh-Core, NR_slice-Core, NR_SmallData_INACTIVE-Core

-	Xiaomi think this is not needed. LG think its good to clarify this but think that this may be NBC.

-	Ericsson think there are other parameters where the same behaviour applies for so wonders why we have to clarify for this particular parameter?

-	MediaTek is not sure if this CR is correct. ZTE think this CR is not correct.

[AT129bis][606][Maint] Correction on rsrp-ThresholdSSB (Huawei)

Scope:

    - Discuss offline to see if/how the spec should be updated (refer to R2-2502754 and R2-2502755)

Intended outcome:

    - Consensus of how to proceed with R2-2502754 and R2-2502755, revisions in R2-2503114 and R2-2503115 if needed

Deadline:

    - Wednesday 17:00. The intention is to agree over email.

-	Huawei reports after the offline that it is better to postpone the issue to a later meeting.

Postponed

Misc

R2-2502573	Miscellaneous non-controversial corrections Set XXV	Ericsson	CR	Rel-17	38.331	17.12.0	5309	-	F	NR_newRAT-Core, TEI17

[Post129bis][601][Maint] Miscellaneous non-controversial corrections (Ericsson)

Scope:

    - Produce agreeable CRs

Intended outcome:

    - In principle agreed CRs

Deadline:

    - Short

#### 6.1.3.2	UE capabilities

UE cap corrections 38306, 38331.

Paging Capability

R2-2502172	Discussion on capability mis-match issue for LP-WUS, PEI, (e)RedCap, 2Rx XR, NTN, inactiveStatePO-Determination	vivo	discussion	Rel-19	NR_LPWUS-Core, NR_UE_pow_sav_enh-Core, NR_redcap-Core, NR_redcap_enh-Core

Moved from 8.19

Proposal 1: It is up to network implementation to address the paging capability mis-match issue between the gNB and the UE for PEI/LP-WUS/inactiveStatePO-Determination-r17, e.g., all legacy gNBs are upgraded to support the latest ASN.1 for the UE capability on paging. No specification impact is foreseen.

R2-2501884	Discussion on Paging Capability Issue	OPPO	discussion	Rel-17	NR_newRAT-Core, TEI17

Proposal 1	R2 decide whether to rely on network implementation (no specified solution), or specified solution to solve the issues (for paging capability before R17, e.g, inactiveStatePO-Determination-r17).

Proposal 2	If R2 decides to adopt the specified solution, UE trigger a MRU (Mobility Registration Update) to update paging capability (if has not done since last NAS registration) upon camping on a cell indicating ranPagingInIdlePO.

R2-2502140	Discussion on UE Radio Capability for Paging Information	CATT	discussion	Rel-17	NR_UE_pow_sav_enh-Core, NR_redcap-Core, NR_NTN_enh-Core, NR_redcap_enh-Core, NR_XR_enh-Core

Moved from 6.1.3.1

Proposal 1: There are paging problems for PEI, LP-WUS and inactiveStatePO-Determination when Initial Registration is performed on a pre-R17 gNB.

Proposal 2: RAN2 discuss which option is adopted for the lost issue which leads paging problems.

-	Option 1: Up to network implementation to avoid lost issue.

-	Option 2: An indicator is sent to UE to indicate R17 related paging info has been sent to CN if UE capability transfer procedure (gNB->AMF) is completed and UE Radio Capability for Paging in the gNB includes R17 related paging info.

R2-2501953	Discussion on UE capabilities to be included in UE Radio Paging Information	ZTE Corporation, Sanechips	discussion	Rel-19

Moved from 8.19

Proposal 1: When gNB is upgraded to a certain release, it is able to decode the UE capabilities in the extended field for this release and include the corresponding UE capabilities in UE Radio Paging Information.

Proposal 2: For those capabilities not decoded and included in UE Radio Paging Information due to the release supported at source gNB side, it is up to the target gNB to decide whether to re-acquire UE capabilities after the UE Radio Paging Information is downloaded from CN or transferred from the source gNB. No spec impact is needed.

Proposal 3: Reply to SA2 with RAN3 and CT1 in CC as in the Annex with the above understanding shared.

R2-2502472	Discussion on the issue of UE radio paging capability loss	Huawei, HiSilicon	discussion	Rel-17	NR_newRAT-Core

Moved from 6.1.3.1

Proposal 1: RAN2 to confirm that UE will miss paging when camps in the cell with LP-WUS/PEI enabled, if the UE reports UE-RadioPagingInfo-r17 to a Rel-15/16 gNB that cannot forward this field to CN.

Proposal 2: RAN2 to confirm that RRC_INACTIVE UE will miss paging when camps in the cell with ranPagingInIdlePO-r17 indicated, if the UE reports inactiveStatePO-Determination-r17 to a Rel-15/16 gNB that cannot forward this field to CN.

Proposal 3: RAN2 to confirm that:

-	For (e)RedCap, UE radio paging capability loss issue does not exist;

-	For 2 Rx XR and DL scheduling offset for NTN, UE will not miss any paging but it’s not the best use of paging resources from NW perspective.

Proposal 4: RAN2 to discuss the following solutions:

-	Solution 1: Up to deployment to upgrade the gNBs to support corresponding ASN.1 simultaneously

-	Solution 2: UE gets information from the gNB whether corresponding capability can be maintained by the NW

All above noted. As agreed in the main session we will not change legacy UE capabilities and instead rely on network implementation

NTN

R2-2502520	Corrections to location-based measurement initiation	ZTE Corporation, Ericsson, CATT, Sanechips	CR	Rel-17	38.306	17.12.0	1255	-	F	NR_NTN_solutions-Core

Agreed in principle (with the parenthesis after the dash: “(quasi-)”)

R2-2502521	Corrections to location-based measurement initiation	ZTE Corporation, Ericsson, CATT, Sanechips	CR	Rel-18	38.306	18.5.0	1256	-	A	NR_NTN_solutions-Core

Agreed in principle (with the parenthesis after the dash: “(quasi-)”)

ul-GapFR2

R2-2502837	Correction to ul-GapFR2-r17	Huawei, HiSilicon, Apple, Nokia, Nokia Shanghai Bell, Ericsson, MediaTek Inc.	CR	Rel-17	38.306	17.12.0	1265	-	F	NR_RF_FR2_req_enh2

R2-2502838	Correction to ul-GapFR2-r17	Huawei, HiSilicon, Apple, Nokia, Nokia Shanghai Bell, Ericsson, MediaTek Inc.	CR	Rel-18	38.306	18.5.0	1266	-	A	NR_RF_FR2_req_enh2

Both are in-principle agreed

#### 6.1.3.3	Other

Including idle and inactive behaviour specified in 38.304 or 36.304.

Emergency call back and paging

R2-2501752	Reply LS on emergency call back and paging (S2-2502427; contact: ZTE)	SA2	LS in	Rel-17	NR_newRAT-Core, NR_redcap-Core	To:RAN2	Cc:CT1, RAN3

Moved from 6.1.1

-	ZTE reports that RAN3 has agreed a stage-2 CR on this topic, and that CR is similar to what ZTE is proposing in R2-2502337. Nokia think that we may need to update UE behaviour. Samsung has the same understanding as ZTE, that the gNB should not release to INACTIVE with eDRX and Samsung wonders what UE behaviour is needed. Nokia think that the inactivity timer may cause the gNB to release the UE to INACTIVE. Ericsson think that the gNB should not use the inactivity timer for these scenarios. Qualcomm think NW can solve everything and its not reasonable to update UE behaviour, e.g. not release the UE. Huawei is not sure what is the status in other WGs yet, and suggest that we wait for LSs from RAN3. Ericsson think that in LTE there is already a solution, i.e. not release the UE to IDLE/INACTIVE with eDRX.

Noted

R2-2502337	On eDRX operation with emergency PDU session	ZTE Corporation, Sanechips	discussion	Rel-17	NR_newRAT-Core, NR_redcap-Core

Proposal 1: RAN2 to confirm that no new signaling from CN to RAN is needed to indicate the presence of emergency PDU session.

Proposal 2: RAN2 confirm that when UE is configured with emergency PDU session, the UE is not expected to be released to RRC_INACTIVE with eDRX configuration for RAN paging (to be captured in stage 2 spec).

Proposal 3: Send reply LS to SA2 about the decisions made in RAN2.

R2-2502504	Discussion on emergency services and eDRX	Nokia, Huawei, HiSilicon	discussion	Rel-17	NR_redcap-Core

Proposal 1:	RAN2 confirms and replies SA2 that the RAN node would release the UE to RRC_IDLE or RRC_INACTIVE state from RRC_CONNECTED when the UE has an emergency PDU session.

Proposal 2:   RAN2 should clarify the UE behaviour that eDRX should not be used when the UE has an emergency PDU session, to align with NAS specification.

Postponed

R2-2502338	Correction on eDRX configuration in RRC_INACTIVE	ZTE Corporation, Sanechips	CR	Rel-17	38.300	17.12.0	0979	-	F	NR_newRAT-Core, NR_redcap-Core

R2-2502339	Correction on eDRX configuration in RRC_INACTIVE	ZTE Corporation, Sanechips	CR	Rel-18	38.300	18.5.0	0980	-	A	NR_newRAT-Core, NR_redcap-Core

R2-2502962	(e)DRX usage clarification for UEs with emergency PDU Sessions	Nokia, Huawei, HiSilicon	CR	Rel-17	38.304	17.10.0	0433	-	F	NR_redcap-Core

R2-2502963	(e)DRX usage clarification for UEs with emergency PDU Sessions	Nokia, Huawei, HiSilicon	CR	Rel-18	38.304	18.4.0	0434	-	A	NR_redcap-Core

R2-2501975	On emergency services and eDRX	MediaTek Inc.	discussion	Rel-17

Proposal 1	RAN2 to agree the RAN node can be aware of an ongoing emergency PDU session by network implementation.

Proposal 2	To support keeping the UE with emergency PDU session to be in RRC_CONNECTED state, RAN2 to agree the network-based solution will be considered (No specs change is needed).

Proposal 3	RAN2 to agree on capturing that the UE shall not use eDRX if the UE has an emergency PDU session in RRC_INACTIVE state in TS 38.304.¨

R2-2502909	Emergency callback and paging	Ericsson	discussion

Proposal 1	RAN2 captures in a NOTE in 38.300 that using the ARP value reserved for emergency service the gNB can delay connection release to reduce the latency of a potential emergency callback.

Proposal 2	RAN2 informs SA2 and RAN3 about the RAN2 agreement.

Old revisions/withdrawn

R2-2502955	(e)DRX usage clarification for UEs with emergency PDU Sessions	Nokia, Huawei, HiSilicon	CR	Rel-17	38.804	14.0.0	0001	-	F	NR_redcap-Core

=> Withdrawn

R2-2502956	(e)DRX usage clarification for UEs with emergency PDU Sessions	Nokia, Huawei, HiSilicon	CR	Rel-18	38.804	14.0.0	0002	-	A	NR_redcap-Core

=> Withdrawn

## 6.3	NR positioning enhancements

(NR_pos_enh-Core; leading WG: RAN1; REL-17; WID: RP-210903)

# 7	Rel-18

## 7.0	Common

Rel-18 WIs not covered under an explicit AI in 7.x.  Multi-WI Rel-18 items, e.g. cross-WI-issues not handled under another WI. UE capabilities.

### 7.0.1	UE Capabilities

Multi-WI handling of Rel-18 feature lists and UE capability Mega CRs.

R2-2501706	LS on Rel-18 RAN1 UE features list for NR after RAN1#120 (R1-2501390; contact: NTT DOCOMO, AT&T)	RAN1	LS in	Rel-18	NR_MIMO_evo_DL_UL, NR_pos_enh2, Netw_Energy_NR, NR_netcon_repeater, NR_NTN_enh, NR_Mob_enh2, NR_SL_enh2, NR_redcap_enh, NR_MC_enh, NR_XR_enh, NR_FR1_lessthan_5MHz_BW, NR_DSS_enh, NR_BWP_wor, NR_cov_enh2, TEI18	To:RAN2	Cc:RAN4

Noted

### 7.0.2	Rel-18 corrections

Essential corrections only. For smaller corrections please contact CR editor / Rapporteur directly.  Coordinate with rapporteurs and chair if input above limit is required

Tdoc limitation: 6

R2-2502574	Miscellaneous non-controversial corrections Set XXV	Ericsson	CR	Rel-18	38.331	18.5.1	5310	-	F	NR_newRAT-Core, TEI18

=> Revised in R2-2503184

[Post129bis][601][Maint] Miscellaneous non-controversial corrections (Ericsson)

Scope:

    - Produce agreeable CRs

Intended outcome:

    - In principle agreed CRs

Deadline:

    - Short

R2-2503184	Miscellaneous non-controversial corrections Set XXV	Ericsson	CR	Rel-18	38.331	18.5.1	5310	1	F	NR_newRAT-Core, TEI18

=> Agreed in principle

#### 7.0.2.1	RACH-less HO

Corrections to generalized RACH-less HO procedure, including NTN, mIAB, and overlapping sections of the LTM cell switch procedure

R2-2501772	Discussion on RACH-less HO Use Case Capturing in Stage-2	vivo, CATT, Huawei, HiSilicon, Nokia, Nokia Shanghai Bell 	discussion	Rel-18	NR_NTN_enh-Core, TEI18, NR_mobile_IAB-Core

Proposal 1: For configuring RACH-less handover for a UE, RAN2 discusses whether the mobile IAB-DU migration procedure is regarded as intra-gNB HO procedure or inter-gNB HO procedure, or neither.

Proposal 2: RAN2 down-selects the following options as the text proposal for sub-clause 9.2.3.6 of 38.300 if it is confirmed that mobile IAB-DU migration procedure is not regarded as intra-gNB HO procedure:

Opt 1: A UE can be configured with RACH-less handover in case of intra-gNB HO procedure in TN and NTN, or inter-gNB HO procedure in NTN, or mobile IAB-DU migration procedure;

Opt 2: A UE can be configured with RACH-less handover in case of intra-gNB HO procedure in TN and NTN, or inter-gNB HO procedure in NTN.

-	Ericsson thinks that current specs limit only to intra case and this option 1 is changing to include inter case.

Noted

R2-2502532	Support of inter-gNB scenarios for RACH-less handover	Ericsson	discussion	Rel-18	TEI18

Noted

R2-2502605	Correction on RACH-less HO in NR-NTN	vivo, Ericsson, Nokia, Nokia Shanghai Bell, ZTE Corporation, Sanechips, THALES, Aalyria, Samsung	CR	Rel-18	38.300	18.5.0	0904	2	F	TEI18, NR_NTN_enh-Core	R2-2410642

Noted

[AT129bis][004][RACH-less] HO use case CR  (Vivo)

Intended outcome: agreable CR

Deadline:  Thursday

R2-2503167	Correction on RACH-less HO Use Case	vivo, Nokia, Nokia Shanghai Bell, Ericsson, Samsung	CR	Rel-18	38.300	18.5.0	0904	3	F	NR_NTN_enh-Core, NR_mobile_IAB-Core, TEI18

The CR is in principle agreed

R2-2502090	Rapporteur CR for RACH-less HO and LTM [RACH-lessHO]	Huawei, HiSilicon, CATT, Nokia, Qualcomm, Samsung	CR	Rel-18	38.321	18.5.0	2059	-	F	TEI18, NR_Mob_enh2-Core, NR_NTN_enh-Core, NR_mobile_IAB-Core

The CR is in principle agreed

#### 7.0.2.2	NR network-controlled repeaters

(NR_NetConRepeater; leading WG: RAN1; REL-18; WID: RP-230175)

#### 7.0.2.3	NR support for UAV

(NR_UAV-Core; leading WG: RAN2; REL-18; WID: RP-230782 and LTE WID: RP-230783 )

#### 7.0.2.4 	Mobile Terminated Small Data Transmission

(NR_MT_SDT-Core; leading WG: RAN2; REL-18; WID: RP-222993)

#### 7.0.2.5	IDC enhancements for NR and MR-DC

(NR_IDC_enh-Core; leading WG: RAN2; REL-18; WID: RP-221281)

#### 7.0.2.6	Mobile IAB (Integrated Access and Backhaul) for NR

( NR_mobile_IAB -Core; leading WG: RAN3; REL-18; WID: RP-232669)

#### 7.0.2.7	Timing Resiliency and URLLC Enh

(NR_TRS_URLLC; leading WG: RAN3; REL-18; WID: RP-230754)

#### 7.0.2.8	Enhanced support of reduced capability NR devices

(NR_redcap_enh-Core; leading WG: RAN1; REL-18; WID: RP-232671)

#### 7.0.2.9	Further NR coverage enhancements

(NR_cov_enh2-Core; leading WG: RAN1; REL-18; WID: RP-221858)

#### 7.0.2.10	Network energy savings for NR

(Netw_Energy_NR-Core; leading WG: RAN1; REL-18; WID: RP-223540)

R2-2501985	Correction for cell barring for NES CellDTX-DRX UEs	Huawei, HiSilicon	CR	Rel-18	38.331	18.5.1	5284	-	F	Netw_Energy_NR-Cor

This field indicates the cell barring status for UEs supporting nes-CellDTX-DRX as described in 5.2.2.4.2 on the selected frequency band.

The CR is in principle agreed with the change above in R2-2502998

R2-2502417	Correction on Semi-Persistent CSI Report SubConfiguration Activation/Deactivation MAC CE	ZTE, Apple, Qualcomm Incorporated	CR	Rel-18	38.321	18.5.0	2065	-	F	Netw_Energy_NR-Core

The change is editorial and it should be incorporated in the MAC rapporteur editorial CR

The CR is not pursued

SSB-less

R2-2501740	LS on neighboring cell measurement for Rel-18 SSB-less (R4-2502694; contact: ZTE)	RAN4	LS in	Rel-18	Netw_Energy_NR	To:RAN2

R2-2501952	Consideration on servingCellMO for SSB-less Scell	ZTE Corporation, Sanechips	discussion	Rel-18	Netw_Energy_NR-Core

Proposal 1: servingCellMO can be configured for SSB-less SCell to indicate the carrier frequency of the SSB-less SCell for UE to determine whether to perform intra-frequency or inter-frequency neighbor cell measurements

Also we understand the servingCellMO is configured for UE to determine to perform inter-frequency or intra-frequency neighbor cell measurements. For the SSB-less SCell itself, since UE is able to identify it is a SSB-less SCell, SSB measurements on SSB-less SCell can be skipped via UE implementation.

Proposal 2: For the case when servingCellMO is configured for SSB-less SCell, SSB measurements on SSB-less SCell can be skipped via UE implementation.

Noted

R2-2502843	Discussion on servingCellMO for SSB-less Scell	Huawei, HiSilicon, Apple, Qualcomm Incorporated, MediaTek Inc.	discussion	Rel-18	Netw_Energy_NR-Core

Proposal 1: In R18 spec, capture in the conditional presence of servingCellMO that “For SSB-less SCell(s), this field is not present.”

Proposal 2: In legacy releases, capture in the conditional presence of servingCellMO that “For intra-band SSB-less SCell(s), this field is not present.”

Noted

R2-2502919	SSBLess handling	Nokia	discussion	Rel-18	Netw_Energy_NR-Core

Proposal 1:In the MO indicated by ServingCellMO, network may configure ssbFrequency to indicate a carrier frequency for the SSB-less SCell so that UE can determine measurement as intra-frequency or inter-frequency measurement based on the ssbFrequency.

Noted

Discussion

-	Qualcomm thinks we shouldn’t do this just for purpose of nw flexibility if we don’t see benefits to do it.

-	Apple and Huawei indicate that we agreed few meetings ago that there is no benefit.

-	ZTE explains that it is for neighbor cell measurements.  Ericsson agrees with ZTE and it wouldn’t hurt to have it.  Huawei thinks that neighbor cells measurements it can be configured in a normal MO.   ZTE thinks that if we don’t change specs then it is always allowed but according to their proposal they have limited to the RAN4 scenarios.  LG agrees with ZTE.

-	Samsung agrees with Huawei, QC, Apple.   Qualcomm thinks that this is beneficial if all the neighbor cells have SSB.  ZTE thinks there are benefits even if some cells have SSBs in the same frequency.

The CR is postponed

R2-2503173	Summary of offline discussion on servingCellMO for SSB-less Scell	ZTE Corporation, Sanechips	discussion	Rel-18	Netw_Energy_NR-Core

For the serving SSB-less SCell, even if the servingCellMO is configured, the SSB measurements on the corresponding SSB-less SCell can be skipped via UE implementation.

-	Compromised solution: servingCellMO is optionally configured for SSB-less SCell and if it is configured UE will perform inter-frequency or intra-frequency neighbour cell measurements accordingly:

-	center frequency of the SSB of the neighbor cell is same as that of SSB frequency configured in the MO indicated by servingCellMO -> intra-frequency neighbor cell measurements

-	center frequency of the SSB of the neighbor cell is different from that of SSB frequency configured in the MO indicated by servingCellMO -> inter-frequency neighbor cell measurements

Noted

#### 7.0.2.11	Further enhancement of data collection for SON MDT in NR and EN-DC

(NR_ENDC_SON_MDT_enh2-Core; leading WG: RAN3; REL-18; WID: RP-221825)

SPR configuration

R2-2501909	Corrections on SPR configuration	CATT	CR	Rel-18	38.331	18.5.1	5283	-	F	NR_ENDC_SON_MDT_enh2-Core

-	Samsung think there are missing colons. Ericsson is the existing text is OK but we only need adding a clause mentioning the target config, further “configured by the target” wording is confusing and could be fixed.

Postponed

References

R2-2502407	Updating references to TS 28.558 from TS 28.552	Nokia	CR	Rel-18	37.320	18.3.0	0140	-	F	NR_ENDC_SON_MDT_enh2-Core

In-principle agreed

timeSinceSHR

R2-2502530	Correction on timeSinceSHR in SHR report	ZTE Corporation, Sanechips	CR	Rel-18	38.331	18.5.1	5304	-	F	NR_ENDC_SON_MDT_enh2-Core

In-principle agreed

RACH-less

R2-2502645	Correction on ra-InformationCommon logging in RLF-Report SHR and SPR	Ericsson	CR	Rel-18	38.331	18.5.1	5312	-	F	NR_ENDC_SON_MDT_enh2-Core

-	ZTE agrees with the intention but want to do it in R19. Ericsson and QC think we should do it in R18. Huawei support the CR. Samsung is OK with the intention but want more time.

Postponed

NPN

R2-2502798	Correction on SON for NPN in TS 38.300	Huawei, HiSilicon, CATT, Samsung, ZTE, CMCC	CR	Rel-18	38.300	18.5.0	0986	-	F	NR_ENDC_SON_MDT_enh2-Core

In-principle agreed

Intended SIBs

R2-2502844	Correction on logging intendedSIBs	Samsung	CR	Rel-18	38.331	18.5.1	5322	-	F	NR_ENDC_SON_MDT_enh2-Core

-	Ericsson think this is not strictly needed. Qualcomm is OK with this CR.

In-principle agreed

#### 7.0.2.12	Dual Transmission/Reception (Tx/Rx) Multi-SIM for NR

(NR_DualTxRx_MUSIM-Core; leading WG: RAN2; REL-18; WID: RP-233071)

R2-2502868	Clarification on the MUSIM Gap and Measurement Gap Collision	ZTE Corporation	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

Proposal 1: RAN2 to discuss which option is preferred for resolving the collision between a MUSIM gap without priority and a concurrent measurement gap with priority

	Option 1: Take the lowest priority for the MUSIM Gap.

	Option 2: Send a LS to ask RAN4 on how to solve this issue.

Proposal 1a: Agree with the TP in the Annex if the option 1 is selected.

-	CATT, Mediatek, Samsung thinks that this is a RAN4 issue.    Nokia agrees and this change is not right.

-	Qualcomm thinks that it is simple, that if the UE doesn’t signal any priority the NW can assign anything.

-	ZTE thinks that we need to have a very consistent behaviour.

-	Huawei thinks that RAN2 needs to decide the cases allowed and then we need to see if we need new requirement.

Noted

[AT129bis][005][MUSIM]  (ZTE)

- Intended outcome: discuss what RAN2 cases are allowed/valid and need for RAN4 LS.

- Deadline:  Thursdayd

R2-2503163	LS on Collision between MUSIM Gap and Measurement Gap	ZTE	LS out	Rel-18	NR_DualTxRx_MUSIM-Core	To:RAN4

The LS is approved

#### 7.0.2.13	NR MIMO evolution

(NR_MIMO_evo_DL_UL-Core; leading WG: RAN1; REL-18; WID: RP-233028)

R2-2501712	LS on RRC parameter for PRACH transmission in 2TA (R1-2501540; contact: CATT)	RAN1	LS in	Rel-18	NR_MIMO_evo_DL_UL-Core	To:RAN2

Noted

R2-2501719	Reply LS on differentiation of sDCI mTRP, mDCI mTRP and sTRP (R1- 2501611; contact: CATT)	RAN1	LS in	Rel-18	NR_MIMO_evo_DL_UL-Core	To:RAN2

Noted

R2-2501723	Reply LS on UL 8Tx (R1-2501636; contact: Samsung)	RAN1	LS in	Rel-18	NR_MIMO_evo_DL_UL-Core	To:RAN2

Noted

8Tx

R2-2502810	Discussion on supporting 8Tx	ASUSTeK	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

Noted

Confirm the assumption that for UL 8Tx 2TB transmission scheduled by one DCI, MAC entity considers two uplink grants for two TBs in two HARQ processes with one HARQ process ID and two HARQ buffers.

R2-2502811	Correction on supporting 8Tx in MAC specification	ASUSTeK, Samsung, ZTE	CR	Rel-18	38.321	18.5.0	2068	-	F	NR_MIMO_evo_DL_UL-Core

=> Withdrawn

R2-2502812	Stage-2 Correction on UL 8Tx	ASUSTeK	CR	Rel-18	38.300	18.5.0	0987	-	F	NR_MIMO_evo_DL_UL-Core

DISCUSSION

-	ZTE do not think stage 2 CR is needed, even though the texts are correct. Nokia, LG E, Ericsson agree.

The CR is not pursued.

R2-2502835	Correction on supporting 8Tx in MAC specification	ASUSTeK, Samsung, ZTE	CR	Rel-18	38.321	18.5.0	1990	2	F	NR_MIMO_evo_DL_UL-Core	R2-2500410

The CR is revised to R2-2503034, with impact to NW added in the cover page.

R2-2503034 is agreed in principle.

Other changes

R2-2502105	Corrections on simultaneousU-TCI-UpdateListx and RACH-ConfigTwoTA	CATT	CR	Rel-18	38.331	18.5.1	5291	-	F	NR_MIMO_evo_DL_UL-Core

The content of the CR is agreeable, will update with proper style and submitted to the next R2 meeting.

R2-2502855	Correction to 306 on PMI subband R value	Ericsson	CR	Rel-18	38.306	18.5.0	1267	-	F	NR_MIMO_evo_DL_UL-Core

DISCUSSION

-	Xiaomi think this is editorial change and think this can be taken into account in rapp’s CR. Nokia also OK with the intention.

Intention is agreeable, will be included in the next version of UE capability Rapp CR.

R2-2502987	Correction for UE capability on DMRS port	Huawei, HiSilicon	discussion	Rel-18	NR_MIMO_evo_DL_UL

DISCUSSION

-	ZTE think the intention is OK, since it is just to align with R1. But ZTE think it can be included in the Rapp’s CR.

-	HW think it is not editorial.

-	Xiaomi think it is OK to have a separate CR.

-	Xiaomi suggest companies to carefully check the mega CR.

Delete "at least" before "one port" in the description of ‘mappingTypeA-1SymbolFL-DMRS-Addition2Symbol-r18’. The changes will be included in the next version of UE capability Rapp CR.

#### 7.0.2.14	Enhancements of NR Multicast and Broadcast Services

(NR_MBS_enh-Core; leading WG: RAN2; REL-18; WID: RP-231829)

#### 7.0.2.15	Enhancement on NR QoE management and optimizations for diverse services

(NR_QoE_enh-Core; leading WG: RAN3; REL-18; WID: RP-223488)

R2-2501729	Reply LS on MBS Communication Service Type (R3-250858; contact: ZTE)	RAN3	LS in	Rel-18	NR_QoE_enh-Core	To:SA4	Cc:RAN2, SA5

Noted

#### 7.0.2.16	XR Enhancements for NR

(NR_XR_enh-Core; leading WG: RAN2; REL-18; WID: RP-230786)

R2-2502092	Discussion on issues with retx-less CG	Huawei, HiSIlicon, Apple, Futurewei, Qualcomm	discussion	Rel-18	NR_XR_enh-Core

-	Samsung, Xiaomi supports the change and it is aligning with intention of features.

-	LG doesn’t think the change is needed

-	Vivo prefers to change the capability but no stron view on RRC

-	Sony thinks current spec is correct.  Huawei doesn’t think that is the correct understanding, the UE just monitors the PDCCH and it doesn’t matter for which transmission.   Apple agrees

-	Sharp thinks that we should aling behavior with MAC spec and support the proposal.

-	Offino supports and we should do it in both specs.

Clarify in the RRC and UE capability spec that retransmission-less CG does not disable PDCCH monitoring for CG, but only disable the start of the HARQ RTT Timer after the CG transmission.

Noted

[AT129bis][006][XR] Re-tx less CG (Huawei)

Intended outcome: agree to CRs by email

Deadline:  Thursday

R2-2502999	Correction to UE capability for retx-less CG	Huawei, HiSilicon, Apple, Futurewei, Qualcomm	CR	Rel-18	38.306	18.5.0	1272	-	F	NR_XR_enh-Core

The CR is in principle agreed

R2-2503000	Correction to RRC spec for retx-less CG	Huawei, HiSilicon, Apple, Futurewei, Qualcomm	CR	Rel-18	38.331	18.5.1	5333	-	F	NR_XR_enh-Core

The CR is in principle agreed

R2-2502297	Discussing on issues on DSR and proposed TP to PDCP	Xiaomi Communications	discussion

Proposal 1.  A Delay-critical PDCP SDU is defined as whose remaining time till discardTimer expiry is less than the remainingTimeThreshold associated with the indicated MAC entity.

-	LG and Vivo doesn’t think we need to clarify anymore as it is clear that it is per MAC entity already.  If we change this then we have to clarify in other places whether it is per MAC entity.

No clarification needed

Noted

R2-2502857	Corrections on PDCP SN Gap Reporting	Samsung	discussion	Rel-18	38.323

Proposal 1: Specify that Discard Bitmap indicates which SDUs are reported as discarded and which SDUs are reported as not discarded in the transmitting PDCP entity. (Adopt TP1)

-	LG and Futurewei thinks that this is not needed as how to set the bitmap is already clear.

Not needed

Proposal 2: Specify the consideration for discard status at the receiving PDCP entity as: (Adopt TP2)

-	deliver to upper layers in ascending order of the associated COUNT value after performing header decompression, if not decompressed before;

o	all stored PDCP SDU(s) with consecutively associated COUNT value(s) starting from COUNT = RX_DELIV, where consecutively associated COUNT value(s) include COUNT value(s) of both the stored PDCP SDU(s) and PDCP SDU(s) which have been considered as discarded as specified in clause 5.16.2;

-	update RX_DELIV to the COUNT value of the first PDCP SDU which has not been delivered to upper layers and has not been considered as discarded as specified in clause 5.16.2, with COUNT value > RX_DELIV”.

-	LG and Futurewei don’t think this is needed.  There is only one rare case that this happens.

Not needed

Noted

R2-2502933	DSR inclusion	Ericsson	CR	Rel-18	38.321	18.5.0	2069	-	D	NR_XR_enh-Core

-	Nokia has a different understanding.   Vivo, LG, Xiaomi thinks that there is a condition already so the current text is fine.

The CR is not pursued

#### 7.0.2.17	NR NTN enhancements

(NR_NTN_enh-Core; leading WG: RAN1; REL-18; WID: RP-232669)

RACH-less HO

R2-2501727	Reply LS on inter-gNB RACH-less HO in NTN (R3-250762; contact: Nokia)	RAN3	LS in	Rel-18	NR_NTN_enh-Core	To:RAN2	Cc:RAN1

Noted

The related discussion is expected to happen in 7.0.2.1

R2-2502009	Correction to RACH-less handover for NTN	Xiaomi	CR	Rel-18	38.300	18.5.0	0977	-	F	NR_NTN_enh-Core

Not pursued

FR2-NTN UE capability

R2-2501747	Reply LS to RAN2 on UE capabilities for FR2-NTN (R4-2503039; contact: vivo)	RAN4	LS in	Rel-18	NR_NTN_enh-Core	To:RAN2	Cc:RAN1

Noted

R2-2501782	Correction on NTN in FR2-NTN bands	vivo, ZTE Corporation, Ericsson, Sanechips	CR	Rel-18	38.306	18.5.0	1200	1	F	NR_NTN_enh-Core	R2-2409544

Agreed in principle

Soft satellite switch with re-sync

R2-2501717	Reply LS on soft satellite switch with re-sync (R1-2501606; contact: Huawei)	RAN1	LS in	Rel-18	NR_NTN_enh-Core	To:RAN2

Noted

R2-2501741	reply LS on soft satellite switch with re-sync (R4-2502701; contact: Huawei)	RAN4	LS in	Rel-18	NR_NTN_enh-Core	To:RAN2, RAN1

Noted

For Rel-18 RAN2 will not specify a specific solution but leave to NW implementation to reconfigure SMTCs as needed

Indicate RAN2 decision in the reply LS to RAN1 and RAN4

R2-2502842	Discussion on satellite switch with resync	Huawei, HiSilicon	discussion	Rel-18	NR_NTN_enh-Core

Proposal 1: Add the following in the field description of ssb-TimeOffset: “It is given in number of subframes and the value can only be 5 or multiples of 5”.

- Ericsson thinks this is needed only if we need to shift back. Nokia agrees

- Oppo thinks that RAN2 can at least take an assumption

RAN2 understands that if SFN is aligned at the uplink reference points for source and target satellites, the ssb-TimeOffset is given in number of subframes and the value can only be 5 or multiples of 5.

Proposal 2: On Q1 from RAN1 LS, RAN2 answer is that: RAN2 is asking whether a Rel-18 UE (based on current Rel-18 RAN1 specifications) can maintain the pre-acquired DL synchronization started between [t-ServiceStart, t-Service] with target satellite if the SSB pattern (e.g., SSB offset) is changed (determined by ssb-TimeOffset) at t-Service. The ssb-TimeOffset will be configured in 5ms or multiples of 5ms, RAN2 does not intend to change the SSB pattern within half frame as defined in clause 4.1 of TS 38.213, the SSB configuration is only “implicitly reconfigured” from RAN2 perspective.

On Q1 from RAN1 LS, RAN2 answer is that: RAN2 is asking whether a Rel-18 UE (based on current Rel-18 RAN1 specifications) can maintain the pre-acquired DL synchronization started between [t-ServiceStart, t-Service] with target satellite if the SSB pattern (e.g., SSB offset) is changed (determined by ssb-TimeOffset) at t-Service. If SFN is aligned at the uplink reference points for source and target satellites, the ssb-TimeOffset is given in number of subframes and the value can only be 5 or multiples of 5. RAN2 does not intend to change the SSB pattern within half frame as defined in clause 4.1 of TS 38.213.

Proposal 3: On Q2 from RAN1 LS, RAN2 answer is that: RAN2 confirms the only change is SSB time offset (determined by ssb-TimeOffset).

Agreed

Proposal 4: On Q3 from RAN1 LS, RAN2 answer is that: Starting from R17, it is up to UE implementation to reacquire SIB 19 before the end of serving cell validity duration, and RAN2 assumes in the soft satellite switch case it is also up to UE implementation to maintain a valid ephemeris information of target satellite and ensures it is not outdated at satellite switch.

Agreed

Proposal 5: Regarding the two options of ssb-TimeOffset from RAN4 LS, RAN2 answer is that: Option 1 is aligned with RAN2 understanding.

Agreed

Proposal 6: Regarding RAN4 question on ssb-TimeOffset granularity and half-frame indication, RAN2 answer is that: ssb-TimeOffset can only be configured as 5ms or multiples of 5ms. RAN2 understanding is that SSB from the target satellite will indicate SFN and half frame bit corresponding to the actual time domain position as in legacy.

Regarding RAN4 question on ssb-TimeOffset granularity and half-frame indication, RAN2 answer is that: if SFN is aligned at the uplink reference points for source and target satellites, ssb-TimeOffset can only be configured as 5ms or multiples of 5ms. RAN2 understanding is that SSB from the target satellite will indicate SFN and half frame bit corresponding to the actual time domain position as in legacy.

Proposal 7: Option 3 is adopted, unless RAN1 raised further concern. The target satellite uses different SSB transmission pattern with source satellite during [t-Start, t-Service], and then change back to the source satellite SSB pattern after t-Service.

Proposal 8: Approve the reply LS in the Annex.

Draft a reply LS according to agreements above

[AT129bis][301][R18 NR NTN] Soft satellite switch with re-sync (Huawei)

Scope: Draft a reply LS to RAN1 and RAN4 and also a CR to capture the agreement on ssb-TimeOffset

Intended outcome: Draft reply LS and 38331 CR

Deadline for companies' feedback:  Wednesday 2025-04-09 20:00

Deadline for Draft reply LS and CR:  Thursday 2025-04-10 08:00

R2-2503051	Draft Reply LS on soft satellite switch with re-sync	Huawei	LS out	Rel-18	NR_NTN_enh-Core	To:RAN1, RAN4

Revise the last RAN2 answer as: “The starting boundaries of SFN0 of the source satellite and the target satellites are aligned at the uplink time synchronization reference point and ssb-TimeOffset can only be configured as 5ms or multiples of 5ms. RAN2 understanding is that SSB from the target satellite will indicate SFN and half frame bit corresponding to the actual time domain position as in legacy.”

Revised in R2-2503065

R2-2503065	Reply LS on soft satellite switch with re-sync	Huawei	LS out	Rel-18	NR_NTN_enh-Core	To:RAN1, RAN4

Change “is” to “was” in the first RAN2 answer

R2-2503069	Reply LS on soft satellite switch with re-sync	Huawei	LS out	Rel-18	NR_NTN_enh-Core	To:RAN1, RAN4

Approved

R2-2503052	Clarification on ssb-TimeOffset	Huawei	CR	Rel-18	38.331	18.5.0	XXXX	-	F	NR_NTN_enh-Core

Revised in R2-2503066 to clarify the description aligned to the response in the LS

R2-2503066	Clarification on ssb-TimeOffset	Huawei	CR	Rel-18	38.331	18.5.0	XXXX	1	F	NR_NTN_enh-Core

Agreed in principle

R2-2502681	LS response to RAN1 and RAN4 for soft satellite switch with re-sync	Ericsson	discussion	Rel-18	NR_NTN_enh-Core

Proposal 1	RAN2 to confirm there might be specification work in RAN1 when SSBs are shifted back upon t-Service for the soft satellite switch with resync.

Proposal 2	Confirm the second and the third assumptions from RAN1’s LS.

Proposal 3	UE initial access during soft satellite switch is not optimized in Rel-18 maintenance.

Proposal 4	Confirm that option 1 is the correct understanding of ssb-TimeOffset parameter.

Proposal 5	From RAN2 perspective, it is feasible to configure ssb-TimeOffset with values multiple of 5ms.

Proposal 6	The question on the half-frame indication and SFN indication bits touches upon a RAN1 procedure which restricts where an SSB can be placed within a half-frame. Thus, this question can be answered once RAN1 has finalized their discussions on this and other aspects, including the feasibility of supporting this feature accounting for the specification impacts.

R2-2502550	Further Considerations on SSBs from Source and Target Satellite in Soft Switching with Resynchronization	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_NTN_enh-Core

Proposal 1: RAN2 responds to RAN4 and states ssb-TimeOffset provided within SatSwitchWithReSync can be configured to other value than multiples of 5.

Proposal 2: RAN2 responds to RAN4 and indicates there is no separate half-frame or SFN indication for satellite switching with resynchronization which would apply during [t-ServiceStart, t-Service].

Proposal 3: RAN2 responds to RAN1 and indicates the intention was to check if the existing RAN1 specification supports such behaviour.

Proposal 4: RAN2 responds to RAN1 and indicates the “SSB pattern change” was supposed to refer to SSB shift in time by applying ssb-TimeOffset.

Proposal 5: RAN2 responds to RAN1 and indicates SIB19 comprises SatSwitchWithReSync, wherein ntn-Config associated with target satellite is conveyed. It is expected such information is also valid at the time of satellite switching.

Proposal 6: RAN2 considers a draft LS shown in Annex A when preparing a response LS to RAN1 and RAN4.

Proposal 7: RAN2 to consider relying on NCD-SSBs to mitigate the SSB overlapping issue for soft-satellite switching with resynchronization.

Proposal 8: RAN2 considers extending SatSwitchWithReSync in SIB19 by including NCD-SSB configuration which would be valid from t-ServiceStart to t-Service.

Proposal 9: RAN2 considers the NR RRC changes shown in Annex B.

R2-2501771	Discussion on RAN1 and RAN4 Reply LS on Soft Satellite Switch with Re-sync	vivo	discussion	Rel-18	NR_NTN_enh-Core

R2-2502294	Discussion on the reply LS from RAN1/RAN4 on soft satellite switch	CATT	discussion

R2-2502327	Discussion on the satellite switch with sync	OPPO	discussion	Rel-18	NR_NTN_enh-Core

R2-2502523	Discussion on ssb-TimeOffset for soft satellite switch with resync	ZTE Corporation, Sanechips	discussion	Rel-18	NR_NTN_enh-Core

R2-2502539	Discussion on the reply LSs on the soft satellite switch with resynchronization	Xiaomi	discussion

R2-2502653	Way forward for soft satellite switch with resync	Qualcomm Incorporated	discussion	Rel-18	NR_NTN_enh-Core

R2-2502740	Discussion on LS from RAN1,RAN4 on soft satellite switch with re-sync	CMCC	discussion	Rel-18	NR_NTN_enh-Core

Location/time-based CHO

R2-2502669	Correction to Rel-18 NR NTN CHO with only location/time-based trigger	Samsung	CR	Rel-18	38.331	18.5.1	5314	-	F	NR_NTN_enh-Core

- Xiaomi thinks there is no need for this as the NW can know the UE release and know whether the UE supports this. vivo also thinks this is not needed. Nokia agrees. Samsung this is not the general way we handle features

- QC agrees with Samsung. Ericsson agrees we should rely on capabilities and not UE releases. and supports the CR. CATT also agrees

Agreed in principle

R2-2502670	Correction to Rel-18 NR NTN CHO with only location/time-based trigger	Samsung	CR	Rel-18	38.306	18.5.0	1263	-	F	NR_NTN_enh-Core

Agreed in principle

#### 7.0.2.18	IoT NTN enhancements

(IoT_NTN_enh-Core; leading WG: RAN2; REL-18; WID: RP-223519)

R2-2501726	Reply LS on UE Location Information for NB-IoT NTN (R3-250761; contact Qualcomm)	RAN3	LS in	Rel-18	IoT_NTN_enh-Core	To:SA2	Cc:RAN2, CT1

RAN2 shares the same understanding as RAN3 (no need to further reply).

R2-2501968	Correction on SIB33 reception in RRC_CONNECTED	Huawei, HiSilicon	CR	Rel-18	36.331	18.5.0	5105	-	F	IoT_NTN_enh-Core

- ZTE partially agrees with this but suggests to clarify the sentence and the description in the reason for change

- Samsung supports this but suggest to revise as:

“… where the UE is required to acquire the SystemInformationBlockType31 (SystemInformationBlockType31-NB in NB-IoT) and may acquire the SystemInformationBlockType33 (SystemInformationBlockType33-NB in NB-IoT).”

- ZTE also suggests to clarify the summary of change as:

“In 5.2.1.3, clarify that BL UEs, UEs in CE, or NB-IoT UEs may acquire the SIB33(-NB) in RRC CONNECTED after the expiry of T317”

Revise the CR as above in R2-2503061

R2-2503061	Correction on SIB33 reception in RRC_CONNECTED	Huawei, HiSilicon	CR	Rel-18	36.331	18.5.0	5105	1	F	IoT_NTN_enh-Core

Agreed in principle

R2-2502522	Corrections to location-based measurement	ZTE Corporation, Ericsson, CATT, Sanechips	CR	Rel-18	36.300	18.4.0	1417	-	F	IoT_NTN_enh-Core

Remove the impacted architecture ffrom the cover sheet

Put the parenthesis after the –: “(quasi-)”

Revised in R2-2503062

R2-2503062	Corrections to location-based measurement	ZTE Corporation, Ericsson, CATT, Sanechips	CR	Rel-18	36.300	18.4.0	1417	1	F	IoT_NTN_enh-Core

Agreed in principle

R2-2502687	Correction on release procedures when HARQ feedback is not configured	Samsung	CR	Rel-18	36.331	18.5.0	5112	-	F	IoT_NTN_enh-Core

- Ericsson thinks the CR is not needed as the current text does not say how HARQ feedback is disabled. Xiaomi and Nokia agree with Ericsson. Vivo also agree

RAN2 understands that Note 0a also covers the case when HARQ feedback is disabled by DCI

Not pursued

#### 7.0.2.19	Enhanced NR Sidelink Relay

(NR_SL_relay_enh-Core; leading WG: RAN2; REL-18; WID: RP-223501)

Capability prerequisites

R2-2502779	Discussion on prerequisite of Rel-17/18 SL relay capability	Samsung, OPPO, Xiaomi, Huawei, HiSilicon	discussion	Rel-18	NR_SL_relay_enh-Core

Noted

Proposal 1: (FFS point) RAN2 is kindly asked to discuss whether to carry out specification change from Rel17.

Discussion:

Qualcomm wonder if it is critical to have the prerequisites; they think there are other features without them.

OPPO tend to agree with Qualcomm; it is almost the end of Rel-19, and it seems too much to change the Rel-17 capabilities.

Huawei think adding the prerequisites is logically and technically correct, but it would be OK to leave them as they are, although they think Rel-17 and Rel-18 should be consistent; if we do not do the general update, they think we should remove the existing prerequisites from Rel-18 in TR 38.822.

Ericsson have a similar understanding to Huawei; they think it helps the implementation to understand which features can be combined, and they would prefer to correct it from Rel-17.

OPPO think even if we change from Rel-17, we will not change 38.822, and then there would be a mismatch.  They understand that the burden of the change falls on the UE side and vendors may not want to accept the impact.

Ericsson note that 38.822 is not a TS and wonder if we can update it.

Huawei understand that normally the rapporteur can do updates, but they are infrequent.  They agree that the TR is not so critical for implementation, and it seems more important to make sure that 38.306 is correct.

Samsung think we could check for 38.822 with the rapporteur, but for 38.306 we should have some correction.

Nokia ask if 38.822 is normative or informative.  Samsung think we could confirm with the rapporteur, but for this discussion we should focus on 38.306.

LG think no TR is normative.

Xiaomi think if we do not change anything, there will be misalignment between releases, so we should either introduce the prerequisites in Rel-17 or remove them in Rel-18.

Samsung understand that the March plenary already updated TR 38.822 for Rel-18 capabilities.  They are willing to check with the 38.822 rapporteur for guidance.

[AT129bis][412][Relay] Relay capability prerequisites (Samsung)

Scope: Further discuss the issue of relay prerequisites in Rel-17/18, including TS 38.306 and TR 38.822, and determine a way forward.

Intended outcome: Report to Thursday CB session in R2-2503082

Deadline: Wednesday 2025-04-09 1900 CST

Proposal 2: RAN2 is kindly asked to agree that feature 31-1 can be considered as the prerequisite of 31-3/31-4/31-8/31-9.

Proposal 3: RAN2 is kindly asked to agree that feature 31-4 can be considered as the prerequisite of 31-5.

Proposal 4: (FFS point) RAN2 is kindly asked to discuss whether 31-6 is the prerequisite of 31-4/31-5.

Proposal 5: RAN2 is kindly asked to agree that 31-1/31-4/31-6 are prerequisite of 51-4.

Proposal 6: (FFS point) RAN2 is kindly asked to discuss whether 31-5 is the prerequisite of 51-4.

Proposal 7: RAN2 is kindly asked to agree that 31-1/31-4 are prerequisite of 51-5.

Proposal 8: (FFS point) RAN2 is kindly asked to discuss whether 31-6 is the prerequisite of 51-4/51-5.

Proposal 9: RAN2 is kindly asked to agree that 51-5 is prerequisite of 51-8/51-9/51-15/51-16/51-19.

Proposal 10: RAN2 is kindly asked to agree that 51-9 is the prerequisite of 51-13/14.

Proposal 11: RAN2 is kindly asked to agree that 51-1 are the prerequisite of 51-2/51-3/51-11/51-12.

Proposal 12: RAN2 is kindly asked to agree that 51-5 and 31-3 are the prerequisite of 51-18, respectively, for remote UE and relay UE.

R2-2503082	Summary of offline discussion on the prerequisite of Rel-17/18 SL relay capability	Samsung	discussion	Rel-18	NR_SL_relay_enh-Core

Postponed (coordination by interested companies encouraged towards next meeting)

Proposal 1: the following prerequisite can be agreeable (Rel-18 only):

	31-1/31-4: prerequisite of 51-4/51-5

	51-5: prerequisite of 51-8/9/15/16/19

	51-9: prerequisite of 51-13/14

	51-1: prerequisite of 51-2/3/11/12

	51-5(remote UE)/31-3(relay UE): prerequisite of 51-18

Proposal 2: RAN2 discusses whether there is any issue if no change is pursued considering that different companies have different understandings on the following prerequisite:

	(Rel17) 31-1 (BC for relay discovery): prerequisite of 31-3 (Basic L2 relay UE)/4(Basic L2 remote UE)/8 (Basic L3 relay UE)/9 (Basic L3 remote UE)

	(Rel17) 31-4(Basic L2 remote UE): prerequisite of 31-5(HO to idle/inactive relay UE)

	(Rel18) 31-5(HO to idle/inactive relay UE): prerequisite of 51-4(remote UE path switch)

	(Rel18) 31-6(simultaneous relay discovery and Uu UL/DL): prerequisite of 51-4(remote UE path switch)/5(multipath remote UE operation)

Proposal 3: If some issues are identified in P2, RAN2 discusses on whether we need some changes to TS38.306, including the specification release for changes.

Discussion:

Nokia wonder if it is OK if we change Rel-18 only for a Rel-17 feature; it may suggest that we are changing the legacy feature in Rel-18.  Samsung think we should have technical consensus about the capabilities first.

Ericsson are fine with the evaluation, but they agree that we should change from Rel-17 or not at all.

Huawei have the same understanding as Ericsson and Nokia: both releases or neither.

OPPO are fine with no change and think it is too late to change Rel-17.  They understand that there is no practical implementation problem.

Xiaomi think we could focus on P3 first; if there is no change we don’t need to analyse the other proposals.

Samsung think companies advocating no change feel there is no technical issue, but some divergence was found in the discussion, so there may be actual misalignment about some capabilities if we do not change anything.

Ericsson think we should reach a common understanding across companies, otherwise we may have divergent implementations.

OPPO understand that for the prerequisites in P2, companies may have different understandings and a change could be a functional NBC.

Qualcomm think for P2, we might discuss a bit and try to converge, then determine if a spec change is needed.

Ericsson think it should be OK to change from Rel-17 if we can align with what devices have actually done.  They think we could take some time and come back next meeting, but they do not see a problem if we take changes that do not force any actual change in implementations.

OPPO are OK to check further in next meeting, but they wonder what the problem is if we do not change anything and allow different implementations from different UEs.  Ericsson agree that the Rel-17 assumption will be that there are no prerequisites, but they think Rel-17 relay has dependencies on earlier features from sidelink.  OPPO understand it was an oversight in Rel-17.

Samsung wonder if there would be any NBC concern if there is no RRC change.  Chair understands the problem could be with the controversial prerequisites in P2; an existing implementation could be rendered non-compliant.

Huawei understand the intention here is to check for the essential functionality to support relay/remote behaviour and Rel-18 extension, and it does not make sense to have, e.g., a UE that declares support for relay but not for discovery.

Samsung indicate that the Rel-17 aspects are in P2.

Other contributions

R2-2502184	Correction to PC5 RLC Channel Release	Apple, ASUSTek	CR	Rel-18	38.331	18.5.0	5292	-	F	NR_SL_relay_enh-Core

Revised in R2-2503081

Discussion:

OPPO understand the intention but wonder if the existing text can already cover it.  The “discard the end-to-end communication configuration” bullet may already cause the release.  Apple think the difference is more of a UE internal behaviour, but we need the PC5-RRC procedure to release the channels.

OPPO understand that in the new text the UE is guided to trigger the channel release procedure, and they think this is already implied by the UE updating the RLC channel configuration; similarly, the UE should trigger SUI because the information has changed.

Apple indicate that if there is a QoS update between the UEs, OPPO are correct, but in this case with no QoS update the second hop still needs to be released.  OPPO think the discarding of the QoS information can be interpreted as a QoS update and will trigger the procedure anyway.  Apple think we discussed this last meeting and found cases where the link may be not released; there was a proposal to send a QoS update in that case, and companies did not want to, so they see that something is still needed.

Huawei wonder if we made this change for idle/inactive, whether we would also need the same change at the end of the clause where the UE is guided to release all the e2e DRB information, including QoS; so maybe a trigger is also needed there.

Apple clarify that the intention is to address the case where one hop has dropped and the second hop is still there, so the discard behaviour should be there for both cases.

Samsung understand the condition is that one link failed, and the intention is to release the other link.

Nokia think the CR is correct; they have not found where the second leg is released in the current spec.

[AT129bis][411][Relay] Release of PC5 RLC channels (Apple)

Scope: Check the CR in R2-2502184, determine if the change is necessary, and produce a final version.

Intended outcome: Agreeable CR in R2-2503081

Deadline: Wednesday 2025-04-09 1900 CST

R2-2503081	Correction to PC5 RLC Channel Release	Apple, ASUSTek	CR	Rel-18	38.331	18.5.0	5292	1	F	NR_SL_relay_enh-Core

Agreed in principle

R2-2502813	Correction on terminology of local ID pair list	ASUSTeK	CR	Rel-18	38.331	18.5.1	5319	-	F	NR_SL_relay_enh-Core

Revised in R2-2503083

Discussion:

Huawei are OK with the change of name, but they wonder if the procedural changes are needed.  ASUSTeK clarify that the current description is one entry at a time, and they understand that it needs to be treated as a whole list.  Huawei think in this light the level 4 bullets may need to be changed as well.

[AT129bis][413][Relay] Local ID pair list (ASUSTeK)

Scope: Check and finalize the CR in R2-2502813.

Intended outcome: Agreed in principle CR (without CB if possible) in R2-2503083

Deadline: Wednesday 2025-04-09 1900 CST

R2-2503083	Correction on terminology of local ID pair list	ASUSTeK	CR	Rel-18	38.331	18.5.1	5319	1	F	NR_SL_relay_enh-Core

Postponed (handled in long post-meeting email discussion)

Discussion:

ASUSTeK suggest postponing to coordinate with 38.351 and take into account other comments.

Apple are not sure about the structure of the requirements yet, in relation to a single entry vs. the list.  ASUSTeK indicate that the problem is in the procedure when one entry is updated, and all the other entries need to be included as well.  They think this can be further discussed among interested companies towards next meeting.

Nokia are concerned that it looks functionally NBC if the remote is updated and the relay is not; the relay will not populate the list fully when the remote is expecting a complete list.

Huawei indicate that the reason to have no release list is that there is no need for explicit release of the entries, because the local ID will be released when the end-to-end connection is released.  They wonder what it would mean if we still consider it as a ToAddMod list; the remote UE implemented according to the current text will consider that a new entry is added and the others are kept, which is not our normal “full list” behaviour.  Huawei can live with the current behaviour.

ASUSTeK think if we keep it as a ToAddMod list, there is a problem because there is no element ID, so there is no way to determine which element is updated or whether an entry is a completely new entry.

Huawei understand that there is no “modify” case and the list is only for additions.

[Post129bis][410][Relay] Local ID pair list (ASUSTeK)

Scope: Further check the CR in R2-2503083 and determine:

- if the CR is necessary or the list can be left as a “ToAdd” only;

- if the CR is functionally NBC;

- if something is needed, whether the procedural change is correct.

Intended outcome: CR to next meeting

Deadline: Long

R2-2502814	Correction on mapping bi-directional E2E sidelink DRB to two AM RLC entities for L2 U2U Relay	ASUSTeK	CR	Rel-18	38.331	18.5.1	5320	-	F	NR_SL_relay_enh-Core

Not pursued

Discussion:

Nokia understand that the CR optimizes the usage of the IDs, but everything works without it.

ASUSTeK confirm that it is to minimize the use of IDs.  In this light, Nokia think it is not an essential correction.

CATT agree with Nokia.

R2-2502936	Correction on sidelink relay RRC specification	ZTE Corporation, Sanechips	CR	Rel-18	38.331	18.5.1	5331	-	F	NR_SL_relay_enh-Core

Remove the change to SidelinkParameters description

Agreed in principle with this change, as R2-2503084

Discussion:

Huawei are OK with the change for RRC connection establishment triggers, but for the UE capability change they think it is editorial and should be from Rel-17 if at all.

#### 7.0.2.20	TEI18

R2-2502476	Correction on UE capability for SIB17bis	Huawei, HiSilicon	CR	Rel-18	38.306	18.5.0	1250	-	F	NR_UE_pow_sav_enh-Core, TEI18

-	Samsung agrees with CR but is not sure about NR-DC architecture as this is for idle/inactive

Delete the DC from architecture option

The CR is in principle agreed and DC architecture will be removed in the may submission

R2-2501746	LS on NR NB-IoT in-band operation (R4-2503035; contact: Ericsson)	RAN4	LS in	Rel-19	TEI18	To:RAN2	Cc:RAN1

Noted

R2-2502682	In-band operation for NB-IoT	Ericsson	discussion	Rel-18	TEI18

Observation 1	RAN4's assumption is that UEs only need to support the same operation mode for anchor and non-anchor carriers which in principle indicates that mixed operation mode is not applicable to NTN in Release 18. This does not have impact in RAN2 specifications.

Observation 2	RAN4's assumption that RRC should indicate guardband-r13 in operationModeInfo IE in MIB-NB does not have an impact in RAN2 specifications.

Observation 3	A NB-IoT UE should not be able to access a cell with an operation mode it does not support.

Observation 4	The network is not expected to broadcast inband-SamePCI or inband-DifferentPCI for NTN in Release 18.

Proposal 1	Update stage 2 and add a note to state which combinations are allowed in NB-IoT NTN. Take the TP as a baseline.

- QC agrees but also thinks we need a new UE capability

- ZTE wonders how this in-band operation is expected to work in a NR band

- Nokia supports p1 but thinks we need to discuss p2

- Samsung thinks we just need Stage 2 update and a capability

Update stage 2 and add a note to state which combinations are allowed in NB-IoT NTN (can discuss the actual TP in offline 302)

Proposal 2	RAN2 to discuss whether signalling enhancements are needed to prevent UEs from access a NB-IoT NTN cell that sets operationModeInfo to inband-SamePCI or inband-DifferentPCI.

Continue in offline 302

[AT129bis][302][TEI18] In-band operation for NB-IoT (Ericsson)

Scope: discuss the actual change for stage 2 as well as possible stage 3 changes and also the reply LS to RAN4

Intended outcome: summary of the offline discussion and Draft reply LS

Deadline for companies' feedback:  Thursday 2025-04-10 20:00

Deadline for offline discussion summary and Draft reply LS:  Friday 2025-04-11 08:00

R2-2503053	Report of [AT129bis][302][TEI18] in-band operation for NB-IoT	Ericsson	discussion	TEI18

- ZTE and Nokia thinks we need to clarify this is about inband operation with NR.

- ZTE thinks we should add a new table with a new description title. Mediatek agrees

- Ericsson thinks a note to the existing table is sufficient as the table is already referring to coexistence with NR

- Nokia is fine with adding a new note to the existing table

Come back Friday regarding how to clarify this in Stage 2, e.g. note to existing Table or new table

Proposal 2	Introduce a new UE capability without signalling to indicate the support of in-band in NB-IoT NTN.

- ZTE thinks we don’t know what’s the feature

- HW thinks we can wait for RAN4 to decide on this and we can ask about this in the LS we send to them

We wait for RAN4 indication whether a new capability is needed

Proposal 3	From RAN2 perspective, the introduction of in-band support in NB-IoT NTN does not have any other Stage 3 impact.

Further continue the discussion offline

R2-2503063	Report of [AT129bis][302][TEI18] in-band operation for NB-IoT – Ph2	Ericsson	discussion	TEI18

Withdrawn

- Ericsson indicates that some companies need more time to check potential Ran2 impacts before replying to RAN4

- QC and thinks we still need to provide a reply LS to RAN4

Come back in the next meeting on possible impacts on RAN2 specs

Attempt to send a reply LS to RAN4 from this meeting, confirming the feasibility and indicating that RAN2 will continue to assess if any clarification in RAN2 specs is needed in the next RAN2 meeting

R2-2503054	Draft Reply on NR NB-IoT in-band operation	Ericsson	LS out	Rel-19	TEI18	To:RAN4	Cc:RAN1

Revised in R2-2503064 to reflect meeting agreements

Discussed in [Post129bis][303]

[Post129bis][303][TEI18] LS to RAN4 on NR NB-IoT in-band operation (Ericsson)

Scope: Draft a reply LS to RAN4 on NR NB-IoT in-band operation

Intended outcome: Approved LS

Deadline: 1-week

R2-2503064	Reply LS on NR NB-IoT in-band operation	RAN2	LS out	Rel-19	TEI18	To:RAN4	Cc:RAN1

=> Approved (but then coversheet revised by MCC: this is a Rel-18 liaison)

R2-2503200	Reply LS on NR NB-IoT in-band operation	RAN2	LS out	Rel-18	TEI18	To:RAN4	Cc:RAN1

=> Approved

R2-2501773	Discussion on RAN4 LS on IoT NTN and NR NTN Inband Operation.	vivo	discussion	Rel-18	TEI18

Observation 1: For NB-IoT NTN inband operation in Rel-18, the operational mode of anchor and non-anchor carriers will be the same (i.e. guard-band or standalone) by network implementation based on the current RRC signaling. There is no RAN2 stage-3 impact foreseen.

Observation 2: For NB-IoT NTN inband operation in Rel-18, guardband-r13 should be always indicated in operationModeInfo-r13 in MasterInformationBlock-NB. It can be supported by the current specification. There is no RAN2 impact foreseen.

Observation 3: To make sure a UE will not access a cell indicating inband-SamePCI-r13 or inband-DifferentPCI-r13 in operationModeInfo-r13 in MasterInformationBlock-NB, cellBarred-NTN shall be set to 'barred' on that cell. There is no RAN2 impact foreseen as the existing cell bar mechanism can fulfill this goal.

Proposal 1: RAN2 to capture the operational mode of anchor and non-anchor carriers will be the same for NB-IoT NTN inband operation in clause 5.5a of Rel-18 36.300.

Proposal 2: For NB-IoT NTN inband operation in Rel-18, it is RAN2’s understanding that cellBarred-NTN shall be set to 'barred' if the cell indicating inband-SamePCI-r13 or inband-DifferentPCI-r13 in MasterInformationBlock-NB (No spec change is required).

Proposal 3: For NB-IoT NTN inband operation in Rel-18, RAN2 responds to RAN4 that the RAN4 assumptions are aligned with RAN2 specification and no RAN2 stage-3 impact is foreseen.

R2-2502047	RAN2 impacts for NB-IoT operation as NR Inband	Nokia , Nokia Shanghai Bells	discussion

Proposal 1: RAN2 to discuss need for new UE capability in Rel-18 (without signalling) for the proposed NB-IoT Guard band mode operation for NB-IoT NR Inband deployment.

Proposal 2: Network configuration for NB-IoT operating mode for NR Inband should be  captured in RAN2 specification.

Proposal 3: No UE behaviour changes needed related to Inband configuration parameters in Rel-18 as this parameter is not applicable for IoT-NTN in NR Inband.

#### 7.0.2.21	Others

Including Multi-WI Rel-18 items, e.g. cross-WI-issues not handled under another WI

R2-2502508	Type clarification for intraBandNR-CA-non-collocated-r18	Apple, Huawei, HiSilicon, xiaomi	CR	Rel-18	38.306	18.5.0	1254	-	F	NonCol_intraB_ENDC_NR_CA, TEI18

-	Samsung agrees but wonders if we have to make the same change in RRC as the same text appears there.   Apple will check offline if there is another change needed in RRC.

The CR is agreed in principle and if there is corresponding CR in RAN4 apple will add it to the may submission

Check offline if RRC CR is also needed and bring it to next meeting

## 7.1	Expanded and improved NR positioning

(NR_pos_enh2-Core; leading WG: RAN1; REL-18; WID: RP-232670)

Time budget: 0 TU

Tdoc Limitation: 1 tdoc

Minor and editorial issues should be coordinated with the appropriate spec rapporteur and submitted by rapporteur company together with any additional corrections the rapporteur company may have.    Larger issues can be discussed based on contributions/individual CRs.

### 7.1.1	Organizational

Including incoming LSs and rapporteur inputs.

R2-2501708	Reply LS on co-existence of SL-CA and SL PRS transmission/reception in Dedicated SL-PRS resource pool (R1-2501479; contact: vivo)	RAN1	LS in	Rel-18	NR_pos_enh2-Core	To:RAN2

Noted

R2-2502973	Correction on NW restriction for dedicated SL-PRS resource pool	vivo, Ericsson	CR	Rel-18	38.331	18.5.1	5104	1	F	NR_pos_enh2-Core	R2-2409639

“In this release” to be deleted

Agreed in principle with this change, as R2-2503079

Discussion:

Qualcomm wonder why the phrase “in this release” is needed.  vivo indicate other SL positioning fields have used this convention.  Huawei agree with Qualcomm’s comment.

vivo wonder if we should remove “in this release” everywhere.  Qualcomm think we do not have any special expectation of a change in this part.

### 7.1.2	Stage 2

Impact to 38.300, 37.340, and 38.305.

This agenda item may be handled at lower priority.

### 7.1.3	SLPP corrections

Impact to 38.355.

R2-2501896	Missing rate of change direction of azimuth/elevation for relative velocity	Xiaomi, Qualcomm, CATT, Huawei, MediaTek Inc.	CR	Rel-18	38.355	18.5.0	0015	-	F	NR_pos_enh2-Core

Agreed in principle

Discussion:

Ericsson think the reason for change could be clarified.  Xiaomi understand a reference to 23.032 should be all right.

Session chair will confirm with RAN2 chair whether this CR needs to be reported as functional NBC.

### 7.1.4	LPP corrections

Impact to 37.355.

R2-2501852	Corrections on the descriptions of nr-PeriodicOrOneShotTimeWindow and nr-StartSFN-TimeWindow	CATT, Qualcomm Incorporated	CR	Rel-18	37.355	18.4.0	0548	-	F	NR_pos_enh2-Core

“within indicated time window(s)” at end of the IE description to be deleted

Agreed in principle with this change, as R2-2503080

Discussion:

ZTE understand that for the one-shot window, there is no confusion, but for the periodic window there is the same ambiguity as with the UTW, whether the start is relative to the first SFN of the window or SFN#0.  They think we could include the question in the LS already allocated to RAN1 on the UTW.

Qualcomm think it is not the same issue as with the SRS window; this is a measurement window, and the startSFN tells where to start the measurement.  The PRU and the UE need to measure at the same time.

Huawei think the CR is needed because the current field descriptions are confusing, with two fields described together.

Nokia are OK with the CR but have some minor comments on the change from “indicated” to “configured”; they indicate there is another instance of “indicated time window” that should be cleaned up.  Chair thinks “within indicated time window(s)” at the end of the description could be deleted.  Qualcomm agree with the chair.

Ericsson think the CR is OK, but there are some editorial issues on the coversheet.

R2-2502274	Correction on the periodic AD of NR integrity service alert	Huawei, HiSilicon	CR	Rel-18	37.355	18.4.0	0549	-	F	NR_pos_enh2-Core

Postponed

Discussion:

CATT think we do not support periodic AD for integrity service alert intentionally; periodic assistance data is for PRU measurement information, and the integrity service alert is for the error source.

vivo support the intention of the CR, but they think the coversheet needs some changes.

Nokia think removing CPP from the text makes it more generic, and they are not sure how broad the impacts are.  They are also not sure if this is really a correction.

Xiaomi do not think the GNSS integrity functionality can be copied wholesale to RAT-dependent, because the base stations are not moving as the satellites are; if something changes, the network can trigger an update based on implementation.  They also think the change for DL-AoD is not correct.

CATT see it as an enhancement, not a correction.  In their view the CR expands the functionality of the periodic AD.

Qualcomm are not sure if it is a correction, but in any case they understand that the CPP AD are needed periodically because the UE needs multiple instances to obtain a single fix.  For integrity in GNSS it came automatically, and they agree that the logic applies also for NR positioning.  However, they note that we do not have a periodic procedure for DL-AoD, only DL-TDOA and RTK; this could be changed but they are not sure it is essential.

ZTE wonder if this is for periodic AD from the CN, which could be handled by implementation (sent several times with the same content).  They agree with Qualcomm that there should be separate signalling and they do not see the benefit of deleting CPP.

Huawei think ZTE’s comment is why we have periodic delivery: If the network finds it necessary, the network could use LPP for unsolicited AD, but the inefficiencies in this process are why we have periodic AD delivery.  On DL-AoD, they see that the description has some restrictions today and needs to be broadened if we want to support it.

Nokia see the technical motivation, but they think adding periodic AD for DL-AoD is a significant change that may not be appropriate for a frozen release.

Huawei understand that the TRP location is needed also for DL-AoD, so it makes sense.

ZTE would be willing to postpone the CR and work on the signalling details until next meeting.

Qualcomm think we do not need new signalling; it was introduced for CPP but can be expanded.  They agree that introducing periodic AD for non-CPP methods is a dramatic change, and we should think about whether we want to make this change.

### 7.1.5	RRC corrections

Impact to 38.331 and 38.306.

R2-2501793	Correction on condition-based IUC for shared SL-PRS resource pool	vivo, Ericsson	draftCR	Rel-18	38.331	18.5.0	F	NR_pos_enh2-Core

Agreed in principle

Discussion:

Xiaomi think it could be reworded in terms of “data” including SL-PRS transmission.  vivo indicate that the CR is only for the trigger condition, i.e., it is not about what IUC is used for but the condition by which IUC is triggered.

ZTE understand the CR is correct because IUC is only transmitted with data, not with standalone SL-PRS.

vivo understand Xiaomi’s suggestion would mean IUC can be transmitted with SL-PRS without data, which is not what RAN1 agreed.  Xiaomi will check offline.

Huawei agree with vivo’s original change.

R2-2502663	RRC Sidelink Positioning Correction	Ericsson, vivo	CR	Rel-18	38.331	18.5.0	5313	-	F	NR_pos_enh2-Core

Postponed (can be seen with the changes in a new field description for mNumberOfSymbols)

Discussion:

Qualcomm wonder about the language “in this release”; do we expect it to change in other releases?  vivo indicate it was used in existing related fields.

ZTE think it is not a big issue and the CR does not need a revision.

Huawei think the change for number of symbols is in the wrong place and it should be in a field description for mNumberOfSymbols.

Xiaomi support vivo’s change on the number of symbols.  They also find that there are some typos and missing hyphens according to the naming conventions.

R2-2502815	Correction on sl-PRS-ResourcePoolID for SL-PRS CG	ASUSTeK	CR	Rel-18	38.331	18.5.1	5321	-	F	NR_pos_enh2-Core

Revised in R2-2502990

R2-2502990	Correction on sl-PRS-ResourcePoolID for SL-PRS CG	ASUSTeK, Ericsson	CR	Rel-18	38.331	18.5.1	5321	1	F	NR_pos_enh2-Core

Postponed (can be seen next meeting with company comments addressed)

Discussion:

Huawei think the name is the same as the field being mapped, and it is not clear what it means.  They suggest we just clarify the correct range in the field description, e.g., the maximum value that can be configured is 4, and remove the duplicate sentence.

CATT think the issue is valid but the change is not correct and should clarify that the IE refers to an SL-PRS resource pool.  They also think the field description should clarify that only the three LSBs are used.

Samsung agree with CATT and think there should be some clarification on the unused bit.  They understand that the ID is applicable only to the type 1 CG and should be configured by the network only for that case, and this can be reflected in the field description.

vivo agree with Samsung that it should be configured only along with type 1 CG configuration.

Withdrawn/Not available

R2-2502079	Correction on SL positioning UE capability	Huawei, HiSilicon	CR	Rel-18	38.331	18.5.1	5289	-	F	NR_pos_enh2-Core

Withdrawn

R2-2502888	Corrections on RRC connection resume procedure initiated by activation or configuration of positioning SRS	CATT	CR	Rel-18	38.331	18.5.1	5323	-	F	NR_pos_enh2-Core

Withdrawn

### 7.1.6	MAC corrections

Impact to 38.321.

R2-2502081	Discussion and draft LS for startSFN in UTW	ZTE Corporation	discussion	Rel-18	NR_pos_enh2-Core

Noted

Proposal 1: Send LS asking RAN1 whether startSFN is needed, and what is the understanding of startSFN in UTW:

-	Option 1: The startSFN is the starting SFN of the UTW allocation determination, i.e., the periodicity and time offset of the UTW are regard to the configured slot#0 of startSFN;

-	Option 2: The startSFN is the starting SFN of the UTW activation, i.e., the periodicity and time offset of the UTW are regard to the SFN#0 slot#0.

Ask RAN1 whether any clarification of UTW’s staring SFN is needed in RAN2 specification or RAN1 specification.

Discussion:

Huawei think RAN1 are already aware of the issue; they understand that the startSFN parameter is not needed because of restrictions on the periodicity (always a divisor of the SFN length).

CATT wonder what the RAN2 spec impact would be; they note that neither of the related CRs includes startSFN.  They think no LS is needed unless there is some spec impact for us.

ZTE indicate that the impact would be dummifying an RRC parameter if the parameter is not needed, and implementing option 1 or option 2 in the RRC field description if it is needed.  They understand we could also let RAN1 clarify it themselves.  In any case they understand that there is nothing related to capture in the MAC.

vivo think the LS is beneficial and includes the guidance that from our perspective either RAN1 or RAN2 could capture it.

MediaTek think the LS is useful to guarantee that RAN1 give us a good reply.

Ericsson also support sending the LS.  ZTE think it is normal 3GPP procedure to notify them, and they consider that RAN1 may not discuss it by themselves since the parameter currently has no RAN1 spec impact.

Huawei think since it is a RAN1 parameter and we just copied it from the parameter list, they can discuss it directly.

Xiaomi support sending an LS; they think we need the clarification.

Samsung support sending an LS and understand that this is a case where we found an issue in the RAN1 parameter list.  They wonder if there is a discussion history in RAN1 already.  Huawei indicate that the discussion has been offline by email so far.

Huawei think the LS could be a bit more generic, asking what the purpose of startSFN is without giving them any options.

ZTE do not think a more generic question would be beneficial; they see that we have two well-defined options here along with the question of whether the parameter is needed.  Huawei think there is not consensus on the options.

CATT think there are two issues, whether the startSFN is required and if so, how to interpret it.  They see that we need to ask the first question, and the second one can be asked in an open way rather than fixing the two options.

Xiaomi wonder if we could say that some companies have two assumptions, so that we do not give an official RAN2 position.  ZTE note that the contents of the LS will be a RAN2 view anyway, but they consider that the two options are examples rather than a restriction on RAN2, so we could ask a generic question about how the parameter is used.

[AT129bis][410][POS] LS to RAN1 on startSFN parameter (ZTE)

Scope: Draft an LS to RAN1 asking if the startSFN parameter is needed, and if so how to interpret it.

Intended outcome: Approved LS (without CB if possible) in R2-2503087

Deadline: Wednesday 2025-04-09 1900 CST

R2-2503087	LS on the startSFN parameter for positioning SRS with TX frequency hopping	ZTE Corporation	LS out	Rel-18	NR_pos_enh2-Core	To:RAN1

Approved (email discussion [AT129bis][410])

R2-2502080	Correction on SRS hopping in positioning	ZTE Corporation, Ericsson, CATT, Samsung, vivo, Xiaomi, Qualcomm, Lenovo	CR	Rel-18	38.321	18.5.0	2031	2	F	NR_pos_enh2-Core	R2-2501428

Postponed (will be seen again related to agreement below)

Discussion:

Huawei think we should wait for a reply to the LS.  ZTE understand that there is no link between the startSFN issue and this CR, which just removes the UTW calculation.

CATT understand that there is no MAC spec impact from the LS, so they see no dependency for this CR.

Samsung agree with ZTE and CATT.  Xiaomi also agree and think the impact from the LS will just be to RRC.

Huawei think the formula for calculating the window will depend on RAN1’s answer, wherever it is captured.  ZTE think whether to capture the formula in MAC is quite independent of whether the startSFN is needed.

Ericsson think what is currently captured in the MAC does not work and it would be better to remove it.  Huawei indicate that it appears to be not aligned with the RRC because of the combined parameter structure, but it could be edited for clarity.

ZTE think we are repeating last meeting’s discussion.  They do not see that this formula is really control of PHY in the sense that we usually do it from MAC, and they note that the RAN1 spec quotes the RRC parameter, not anything from the MAC.

Huawei think that their point on the modelling is not answered by proponents of the CR; the boundaries of windows are always calculated in the MAC and used as control for the PHY, and they do not see why we change the modelling paradigm a long time after the freeze.

vivo agree that MAC can control PHY for specific features, e.g., when there is a timer, but in this case they do not see the need for MAC control; the PHY can send SRS under the control of RRC.

CATT agree with vivo, and they note that there is a time window in LPP as well with no MAC control.

Huawei agree that the window can be used directly without a timer, but they see that the split should be that the window is defined in MAC and the SRS behaviour within the window in PHY; they see LPP as a different case because there are multiple layers in between and the behaviour is not visible to the MAC layer.

Qualcomm note that the window does not control the SRS transmission but only tells where collisions may happen; they understand that this could be specified in RRC and it is not a “control” function of MAC.

ZTE indicate that this issue was overlooked in the running CR stage and there were no related comments at the time, which they consider a mistake.  They see that the point of the MAC control is to let it control timers, and here there is no timer.  Huawei think a timer is just one model of a way to specify the window, and they wonder if we could reach a middle ground without a major change to the spec.

Ericsson think we should take the CR.

ZTE think where we would capture the formula may depend on RAN1’s reply, and the only part that may need clarification is startSFN, not the whole formula.  They see that the formula is not really needed in the spec, but if RAN1 has a request to specify the startSFN behaviour, something could be needed.

Huawei disagree with ZTE’s comment, because they think the parameter does not mean anything the PHY without being translated to slots.

CATT understand that the RRC can specify where the window starts and its duration and periodicity, as we already do with LPP, and RAN1 specs already capture how the PHY signals are sent, so they see no role for the MAC.  Huawei understand that the start parameter is not there in RRC.

vivo think we could agree that RRC will capture how to calculate the timing of the window, and there are similar descriptions already (SMTC, measurement gap), so maybe we could agree now to capture it in RRC.

Huawei understand that the SMTC and measurement gap are specified in RRC because they control RRC functionality and there is no involvement of lower layers.

CATT understand that with startSFN in RRC, the start information is there and the periodicity and offset define where the window is after the start.  Huawei do not think the startSFN defines when the window starts; it is a reference SFN for the periodicity and offset, and here we do not need a reference SFN.

CATT wonder what the MAC behaviour for startSFN would be.  Huawei indicate that it would be part of the formula.

Qualcomm think this is just a window for collision; it does not control any behaviour in PHY or MAC, and they do not see the related MAC functionality in comparison to other examples where we have window calculations in MAC.

Samsung share Qualcomm’s view and think if we want to model the window boundaries in MAC, we would expect a more detailed indication from MAC to PHY with procedural text in PHY.

ZTE think if we take this in the MAC layer, we would expect PPW indication in the MAC layer as well, and here there is no control by the MAC, only information to PHY about the window boundaries.

Huawei indicate that there is no explicit procedural behaviour, only an indication of the start and end of the window, but they understand that it is implicit, and there are explicit PHY dependencies for the UE behaviour inside and outside the window.

Qualcomm understand that there is no “endSFN” defining the end of the window.  Huawei indicate that the duration defines the end, and it could be modelled as a timer or in other ways.

ZTE think we have discussed already for a long time and companies have had chances to take everything into account.

Huawei understand that the RRC description is incomplete without the formula, and it must be in upper layers, but they would be open to capturing it elsewhere.  ZTE see that the formula just translates the field description wording.

CATT agree that the RRC parameters can be clarified in RRC, including the formula if necessary.

Qualcomm understand that we will capture the meaning of the parameters in RRC once we have the RAN1 reply.  They see this as a modelling discussion.

Agreement:

The definition of the UTW boundaries is captured in RRC field descriptions, including the formula which will be added once we have the RAN1 reply on startSFN.  The CR in R2-2502080 will be implemented in the next meeting (removal of section 5.32 from MAC).

Not treated (over tdoc quota)

R2-2502078	Correction to UTW for positioning SRS frequency hopping	Huawei, HiSilicon	CR	Rel-18	38.321	18.5.0	2058	D	NR_pos_enh2-Core

### 7.1.7	Corrections to other specifications

Impact to any specifications not identified above.

## 7.2	Further NR mobility enhancements

(NR_Mob_enh2-Core; leading WG: RAN2; REL-18; WID:RP-233970)

Time budget: 0 TU)

Minor and editorial issues should be coordinated with the CR rapporteur. Note RRC CR and MAC CR rapporteurs’ summary and suggestion (based on the submitted contributions) may be provided.

Tdoc Limitation: 1 tdocs.

R2-2501730	Reply LS on L3 measurement based LTM support over F1 (R3-250879; contact: Vodafone)	RAN3	LS in	Rel-18	NR_Mob_enh2-Core	To:RAN2	Cc:RAN4, RAN

Noted.

R2-2501748	Reply LS on L3 measurement based LTM support over F1 (RP-250790; contact: Vodafone)	RAN	LS in	Rel-18	NR_Mob_enh2-Core	To:RAN3	Cc:RAN2, RAN4

Noted.

R2-2501736	Reply LS on the fast RRC processing for LTM (R4-2420218; contact: Ericsson)	RAN4	LS in	Rel-18	NR_Mob_enh2-Core	To:RAN2

Noted.

R2-2501737	LS on UE capability for L1-RSRP measurement in LTM (R4-2502619; contact: vivo)	RAN4	LS in	Rel-18	NR_Mob_enh2-Core	To:RAN2

Noted.

R2-2502170	Correction on UE capability for L1-RSRP measurement in LTM	vivo, Ericsson, Xiaomi	CR	Rel-18	38.306	18.5.0	1249	-	F	NR_Mob_enh2-Core

[Huawei]: Think “Both LTM candidate cell(s) and serving cells for SSB-based L1-RSRP measurement are counted” is not needed since the previous text already covers it. Also, the second text is not clear, it would be better to enhance the wordings. [Apple]: We should also consider that we have used “neighboring cells” instead of candidate cells.

Wordings can be further discussed offline.

Comeback in CB session (revised CR in R2-2503016 if needed).

R2-2503016	Correction on UE capability for L1-RSRP measurement in LTM	vivo, Ericsson, Xiaomi	CR	Rel-18	38.306	18.5.0	1249	1	F	NR_Mob_enh2-Core

Agreed in principle.

R2-2502171	Correction on the maximum number of SSB rsources for L1 maeasurement without gaps in LTM	vivo, Ericsson, Xiaomi	CR	Rel-18	38.331	18.5.1	5302	-	F	NR_Mob_enh2-Core

[Huawei]: Not sure whether it’s good to make dummy this level IE that is proposed by Vivo. [Apple]: The linkage with CA parameter IE is not clear. [Qualcomm]: Can we agree with general principle, NBC change is acceptable only when there is no other option.

IE level for dummy and the linkage with CA parameter IE can be further discussed offline.

NBC change is acceptable only when there is no other option.

Comeback in CB session (revised CR in R2-2503017 if needed).

R2-2503017	Correction on the maximum number of SSB rsources for L1 maeasurement without gaps in LTM	vivo, Ericsson, Xiaomi	CR	Rel-18	38.331	18.5.1	5302	1	F	NR_Mob_enh2-Core

[MediaTek]: Wonder if it is NBC change. [Vivo, Ericsson]: Understand it is BC change. Note UE behaviour is not changed. [Huawei]: Understand although it is BC change in ASN.1 point of view, it is still NBC change in the functional point of view.

Cover page needs to indicate functional NBC change.

With the change above, CR in R2-2503021 is agreed in principle.

R2-2502966	Discussion on eEMR/IMR and LTM L2 Reset/UE-based TA  measurement	Samsung Electronics Czech	discussion	Rel-18	38.331	NR_Mob_enh2-Core

Proposal 1: RAN2 to adopt the TP provided in Annex 1.

RAN2 to adopt the TP provided in Annex 1.

[Ericsson]: Think that the text proposal achieves the same as the current text, or at least fail to understand what the real difference is. [Nokia]: What is the problem with the current spec? [Samsung]: The current text does not cover when the timer was not configured. Proposed text is not new, just same as what is specified for inactive. [ZTE]: Agree with Samsung.

Proposal 2: RAN2 to adopt the TP provided in Annex 2.

RAN2 to adopt the TP provided in Annex 2.

[Ericsson]: Indeed seems this case was overlooked in the procedural text and needs to be added.

Proposal 3: RAN2 to adopt the TP provided in Annex 3.

RAN2 to adopt the TP provided in Annex 3.

[Ericsson]: The proposed text is related to SIB11 and for this case UE will do something according to what is support. The existing text seems to be fine.

Proposal 4: RAN2 to discuss whether to remove the measIdleValidityDuration in VarEnhMeasIdleConfig, if stored, after discarding the VarMeasIdleReport.

Remove the measIdleValidityDuration in VarEnhMeasIdleConfig, if stored, after discarding the VarMeasIdleReport.

[Samsung]: UE deletes all other related variables only except measIdleValidityDuration. Note this is configured only by RRC release or SIB. [Qualcomm]: Ok with this change.

Proposal 5: RAN2 to agree to change the serving cell IDs under VarLTM-ServingCellNoResetID and VarLTM-ServingCellUE-MeasuredTA-ID as “ID associated with the serving cell”.

Change the serving cell IDs under VarLTM-ServingCellNoResetID and VarLTM-ServingCellUE-MeasuredTA-ID as “ID associated with the serving cell”.

[Ericsson]: Is a small clarification but is good to have. Otherwise, a reader may think that is the serving cell ID, which is not.

CR in R2-2503007 will be provided in CB session.

R2-2503007	Corrections on eEMR/IMR and LTM L2 Reset/UE-based TA measurement	Samsung, Ericsson	CR	Rel-18	38.331	18.5.1	5334	-	F	NR_Mob_enh2-Core

Cover page needs to indicate functional NBC change.

With the change above, CR in R2-2503020 is agreed in principle.

R2-2502533	Capability coordination for LTM	Ericsson	CR	Rel-18	38.331	18.5.1	5305	-	F	NR_Mob_enh2-Core

[Ericsson]: We may need more parameters based on offline discussion (e.g. number of candidate per TCI state configuration). [ZTE]: Understand if UE capability is defined per band, then out of the current bands, the NW will assume the lowest number to be configured.

Offline discussion (including P3 and P4 in R2-2502553).

Comeback in CB session (revised CR in R2-2503018 if needed).

R2-2503018	Capability coordination for LTM	Ericsson	CR	Rel-18	38.331	18.5.1	5305	1	F	NR_Mob_enh2-Core

Agreed in principle.

R2-2501904	Handling of the default SRB configuration upon LTM execution	CATT	CR	Rel-18	38.331	18.5.1	5285	-	F	NR_Mob_enh2-Core

[Ericsson]: It is true that if the LTM is at the SCG the UE will apply the default SRB1 and SRB2 configuration, but this may be correct in case of split SRB. Also, applying the default is not harmful as the network does not really need to provide new SRB1 and SRB2 configurations in this case. So, what we have now it works, even if this implies that for SRB1 and SRB2 the default configuration is always re-applied…but maybe this is a safe approach. [Qualcomm]: Think technically correct, but we can survive without this change. [Ericsson]: We agreed that MCG LTM and SCG LTM are not executed at the same time, once LTM happens either MCG or SCG, the other note will know what happened for SRB (whether it is changed to default configuration or not). [Apple]: RAN2 has never considered like MSG control plane or SCG control plane up to now. [CATT]: To Ericsson, it requires additional coordination between nodes to have same understanding on SRBs, which needs more specification efforts. [Samsung]: Support the proposal. Technically correct and it will not bring any further network coordination. [Huawei, MediaTek]: Same view as Samsung. [MediaTek]: It is functional NBC change so it should be indicated by cover page.

Proposed change is agreed.

Cover page should indicate functional NBC change.

CR will be revised in R2-2503008.

Comeback in CB session.

R2-2503008	Handling of the default SRB configuration upon LTM execution	CATT	CR	Rel-18	38.331	18.5.1	5285	1	F	NR_Mob_enh2-Core

Agreed in principle.

R2-2501976	Clarification on LTM cell switch failure recovery	Google Korea LLC	discussion	Rel-18	38.331

Proposal: 	RAN2 is requested to discuss the two options and decide option 1 or option 2 is adopted.

1)	Modify Section 5.3.5.8.3: Add a NOTE clarifying that during T304 expiry in an LTM cell switch, the UE only reverts back to the configurations and variables retained as per section 5.3.5.18.6.

2)	Modify Section 5.3.5.18.6: Add a NOTE clarifying that the UE retains all current dedicated and common radio configurations for reverting back to the UE configuration upon T304 expiry as per section 5.3.5.8.3.

Noted.

[Ericsson]: This is exactly the same case when there is a reconfiguration with sync with full configuration. With full configuration UE delete all the dedicated and common configuration, but then if the reconfiguration with sync fails, the UE needs to revert back to the previous configuration. In this case, there is nothing really new and we should not pursue this change. [Nokia]: Agree with Ericsson. [Google]: UE behaviour for release is specified under LTM cell switch execution. [Ericsson]: Note it is not LTM specific issue since the same text is there for L3 HO (with full configuration). [Apple]: Share the view with Ericsson. Do not see a real problem.

R2-2502169	Discussion on indication from MAC to RRC layer in RACH-based LTM	vivo	discussion	Rel-18	NR_Mob_enh2-Core

Proposal 1: Remove the indication from MAC to RRC layer for the completion of RACH-based LTM.

Proposal 2: If P1 is agreeable, RAN2 to adopt the TP provided in Annex1.

Not pursued.

[Ericsson]: It seems that there is no issue in the procedural text point by vivo in RRC. The indentation 1> are all “or” and this means that the UE will only execute one of those. Also, even if the indication from lower layer is repeated in one indentation 2> we still think there is no issue and this does not mean the indication needs to be repeated twice. [Huawei]: Nothing is broken with this indication.

R2-2502553	Remaining Rel-18 LTM Corrections	Nokia	discussion	Rel-18	NR_Mob_enh2-Core

Proposal 1: Update the first sentence to "Indicates whether the UE supports early TA acquisition based on PDCCH ordered CFRA procedure before the cell switch command MAC CE”. Adopt the TP in Annex A for TS 38.306.

Noted.

[Ericsson]: We understand the limitation comes from WGs which are not RAN2 and for this reason we cannot simply modify the meaning of the capability as we want (description which we believe is actually correct). [MediaTek]: It is RAN1/4 issue, and to perform early UL synchronization, the UE needs to perform measurements on SSB and selection of SSB. Understand the maximum number of this IE considered this aspect.

Proposal 2: RAN2 to clarify the understanding on the timeframe associated with UE “maintaining” the UE-measured TA and to reflect this in 38.306. The following options can be considered as a starting point (other alternatives are not precluded):

1)	UE measures and maintains the TA value starting from the moment when it is configured to perform UE-based TA measurement.

2)	UE measures and maintains the TA value starting from the moment when it receives the CSC

Noted.

[Ericsson]: The UE-based TA is a UE-only feature and the intention is that the UE calculate itself the TA (if indicated by the network). When the UE calculate the TA and how to maintain it is up to the UE implementation and RAN4 did not specify any requirement for this feature.

Proposal 3: In addition to the number of LTM cells and L1 measurement configuration, MN informs SN about the number of cells or/and TCI states for which early DL synchronization procedures can be performed. Adopt the TP in Annex B in TS 38.331.

Proposal 4: MN informs SN about the number of cells which early UL synchronization procedures can be performed. Adopt the TP in Annex B in TS 38.331.

[Session chair]: P3 and P4 can be further discussed as part of R2-2502533 offline.

R2-2502600	Rel-18 LTM issues	Ofinno, LLC	discussion	Rel-18	NR_Mob_enh2-Core

Proposal: To avoid the mismatch of UE-based TA measurements between the UE and the network, RAN2 agreed one of the following alternatives:

1.	A UE determines to configure the lower layer for UE based TA measurements for each LTM candidate cell if the received LTM-Config includes ltm-ServingCellUE-MeasuredTA-ID but not include the ltm-CandidateToAddModList

2.	the ltm-CandidateToAddModList is mandatory present if the received LTM-Config includes ltm-ServingCellUE-MeasuredTA-ID

Not pursued.

[Ericsson]: The field description of ltm-UE-MeasuredTA-ID already handle the case described in the contribution: “the network configures also for all LTM candidate configurations within ltm-CandidateToAddModList in LTM-Config and ensures that the UE has stored a value for ltm-ServingCellUE-MeasuredTA-ID within VarLTM-ServingCellUE-MeasuredTA-ID” [Nokia]: NW will handle all cases that should be done correctly. [Apple]: Do not prefer changing the UE behaviour. Prefer NW handle it.

R2-2502942	Handling of PUCCH resources for L1 LTM reports at TAT expiry	Huawei, HiSilicon	CR	Rel-18	38.331	18.5.1	5332	-	F	NR_Mob_enh2-Core

R2-2502971	Handling of PUCCH resources for L1 LTM reports at TAT expiry	Huawei, HiSilicon	CR	Rel-18	38.331	18.5.1	5332	1	F	NR_Mob_enh2-Core	R2-2502942

[Ericsson]: We are wondering if this is really correct. For the CSI-ReportConfig of the serving cell this makes sense since the TAT is maintained towards the serving cell. However, LTM-CSI-ReportConfig provide resource for cells which are not “serving cell”, since these are candidate cells. So in principle the UE may still continue to use those PUCCH resources. [Qualcomm, Xiaomi, CATT, Nokia]: Agree with the proposal. [Ericsson]: It is functionally NBC change.

Cover page will indicate functionally NBC change.

CR with the change above in R2-2503009 is in principle agreed.

R2-2502943	MAC CR rapporteur summary	Huawei, HiSilicon	discussion	Rel-18	NR_Mob_enh2-Core

R2-2502979	Summary of RRC proposals for FeMob	Ericsson	discussion	Rel-18	NR_Mob_enh2-Core

Summaries were included as Ericsson comments in the session minutes.

R2-2501932	Correction on LTM UE Capability based on the RAN4 LS	MediaTek Inc.	discussion	Rel-18	NR_Mob_enh2-Core

The proposed change was already treated above.

## 7.6	NR Sidelink evolution

(NR_SL_enh2-Core; leading WG: RAN1; REL-18; WID: RP-230077)

Time budget: 0 TU

Tdoc Limitation: 1 tdocs

R2-2501735	LS on AdditionalSpectrumEmission in NR SL Pre-configuration (R4-2418075; contact: LGE)	RAN4	LS in	Rel-18	NR_SL_enh2-Core	To:RAN2	Cc:RAN1

Noted.

R2-2502061	Discussion on resource re-selection in case of LBT failure on MCSt	LG Electronics Inc.	discussion	Rel-18	38.321	NR_SL_enh2-Core

Proposal 1. RAN2 agrees that the RAN2 agreement (“Trigger resource (re)selection if all initial transmission and retransmission within MCSt fail due to LBT failure.”) for single TB MCSt based on multi-slot candidate resources can be applied to multi-TB MCSt (i.e., Single slot candidate resources based multi-TBs MCSt, Multi-slot candidate resources based multi-TBs MCSt) as well.

Proposal 2. Adopt the corresponding TP captured in Appendix.

[OPPO]: Understand the resource (re)selection should be triggered in a HARQ process point of view. Resource (re)selection is triggered if, from a HARQ process point of view, consecutive resources that were assigned have LBT failure indications. [Ericsson]: Prefer resource (re)selection is triggered for other MCSt cases just as non-MCSt case. [Session chair]: There would be various options to handle each MCSt case. Is there real technical blocking issue for any direction or is it a matter of taste? [Qualcomm]: One thing we should consider is related to the HARQ buffer refresh. Also, it is related to the efficiency of resource (re)selection. If HARQ retransmission is sent successfully later (within MCSt), the resource (re)selection does not need to be triggered earlier.

[Session chair]: Examples of MCSt TB patterns:

- Pattern#1: TB1-TB1-TB1-TB1

- Pattern#2: TB1-TB1-TB2-TB2

- Pattern#3: TB1-TB2-TB1-TB2

- Pattern#4: TB1-TB2-TB3-TB4

Question#1: To determine whether retransmission is needed or not:

- Option#A: When an LBT failure indication is received for any resource within a MCSt, which is assigned to a given TB.

- Option#B: When LBT failure indication(s) is/are received for all consecutive resource within a MCSt, which is assigned to a given TB.

- Option#C: When LBT failure indication(s) is/are received for all resources within a MCSt, which are assigned to a given TB.

Question#2: To determine when resource (re)selection for retransmission is triggered.

- Option#W: At each resource for a given TB within a MCSt

- Option#X: At the end of contiguous resource for a given TB within a MCSt

- Option#Y: At the end of resource for a given TB within a MCSt

- Option#Z: At the end of resource within a MCSt

[AT129b][101][SL] (LG)

Scope: Discuss multiple candidate options for SL resource (re)selection in MCSt, identify any real blocking issue for candidate options, and attempt to find a unified solution (if possible) that can be applicable to all MCSt cases.

Intended outcome: Discussion summary in R2-2503014.

Deadline: Comeback in CB session => Completed.

R2-2503014	Summary of [AT129b][101][V2X/SL] Discuss multiple candidate options for SL resource (re)selection in MCSt (LG)	LG Electronics Inc.	discussion	Rel-18	38.321	NR_SL_enh2-Core

[Session chair]: Based on that there is no technical blocking issue for any direction, would like to suggest A+W considering MCSt using single slot candidate resource is done as best effort manner.

Option (A + W) for MCSt using single slot candidate resources is agreed.

- To determine whether retransmission is needed or not:

- Option A: When an LBT failure indication is received for any resource within a MCSt, which is assigned to a given TB.

- To determine when resource (re)selection for retransmission is triggered:

- Option W: At each resource for a given TB within a MCSt.

We will see CR next meeting (LG).

R2-2502935	Correction on sidelink RRC specification	ZTE Corporation, Sanechips, Ericsson	CR	Rel-18	38.331	18.5.1	5330	-	F	NR_SL_enh2-Core

[Qualcomm]: For change 2, LTM was a parallel WI in Rel-18, and there was no explicit agreement on co-existence between SL and LTM. Prefer not adding this change. [Vivo]: Although we understand intention of change, also agree with Qualcomm. [Ericsson]: Ok with change 2. [NEC, Samsung]: Have similar view as Qualcomm.

[OPPO]: For change 3, although we agree with intention, think adding “for the NR sidelink unicast, groupcast and broadcast communication transmission” is not needed since TX profile is only applied to groupcast/broadcast. [Huawei, Qualcomm]: Agree with OPPO. [Xiaomi]: Agree with the change 3 for clearer understanding.

Change 2 and change 3 will be removed.

CR with the change above in R2-2503010 is in principle agreed.

R2-2502062	MAC correction on Release-18 Sidelink evolution	LG Electronics Inc.	CR	Rel-18	38.321	18.5.0	2057	-	F	NR_SL_enh2-Core

Agreed in principle.

R2-2502185	Correction on MCSt Restriction for Resource Selection	Apple	CR	Rel-18	38.321	18.5.0	2060	-	F	NR_SL_enh2-Core

Agreed in principle.

[OPPO]: With this change, it seems MCSt is not the reason for this note, it sounds MCSt is the result of this note. Propose to add “For SL unlicensed band operation” instead. [LG]: There are other notes and normative texts that are similar to this note. Do we apply “For SL unlicensed band operation” to only note? [Xiaomi]: For all NOTEs, it is related to multi-consecutive slots transmission. Then with this change, some note is related to multi-consecutive slots transmission and some note is related to “SL unlicensed band operation”. [OPPO]: Ok with the originally proposed text.

R2-2502322	Discussion on resource re-selection from SL LBT Failure indication	Ericsson, Apple, Qualcomm Incorporated, vivo	discussion	Rel-18	NR_SL_enh2-Core

R2-2502305	Correction to MAC on IUC	Ericsson, Apple, ZTE Corporation, Sanechips	CR	Rel-17	38.321	17.12.0	2061	-	F	NR_SL_enh-Core

R2-2502306	Correction to MAC on IUC	Ericsson, Apple, ZTE Corporation, Sanechips	CR	Rel-18	38.321	18.5.0	2062	-	A	NR_SL_enh-Core

Noted.

[Huawei]: Understand the second change is not correct since there is some conditions in 38.214.  Also understand the first change is not essential correction, which does not give any useful information. [OPPO]: Agree with Huawei on the second change.

## 7.8	R18 Other

Specific items may be allocated to a breakout session for treatment.

Impacts from Other RAN WGs and TSGs that has no separate TU budget in RAN2. LS ins for Rel-18 specific WIs/SIs that has no RAN WI.

Clarification CRs should be discussed with spec rapporteurs of the topic prior to submission.

Time budget: 1 TU

Tdoc Limitation: 2

### 7.8.1	RAN4 led items

R2-2501745	LS on clarification for  intraBandNR-CA-non-collocated-r18 and interBandMRDC-WithOverlapDL-Bands-r16 (R4-2503033; contact: Apple, Huawei)	RAN4	LS in	Rel-18	NonCol_intraB_ENDC_NR_CA	To:RAN2

Noted

R2-2502899	Clarification on capability for inter-frequency configuration for less than 5MHz	Huawei, HiSilicon, Xiaomi, Samsung, Qualcomm Incorporated	CR	Rel-18	38.306	18.5.0	1196	2	F	NR_FR1_lessthan_5MHz_BW	R2-2409385

-	Ericsson doesn’t think this is needed

-	Samsung indicates that this CR is capturing the agreement we made last meeting.  Apple thinks a note is a better way to go.   Huawei agrees with Samsung.

[AT129bis][007][less5MHz] CR (Huawei)

Intended outcome: agree to CR by email after taking into account wording suggestions

Deadline:  Thursday

R2-2503174	Clarification on capability for inter-frequency configuration for less than 5MHz	Huawei, HiSilicon, Xiaomi, Samsung, Qualcomm Incorporated	CR	Rel-18	38.306	18.5.0	1196	3	F	NR_FR1_lessthan_5MHz_BW

Delete “support of”

The CR is in principle agreed and the deletion will be done in submission to next meeting

### 7.8.2	RAN1 led items

R2-2502270	Adding enabledDefaultBeamFormultiCellScheduling	Huawei, HiSilicon, NTT DOCOMO INC.	CR	Rel-18	38.331	18.5.1	5294	-	F	NR_MC_enh-Core

-	Samsung thinks that we can make this change in better way with less overhead.

Check offline if there is a better way to do the change

[AT129bis][008][NR MC] CR (Huawei)

Intended outcome: agree to CR by email after check if there is a better way to make the change

Deadline:  Thursday

R2-2503164	Adding enabledDefaultBeamFormultiCellScheduling	Huawei, HiSilicon, NTT DOCOMO INC., Samsung, Ericsson	CR	Rel-18	38.331	18.5.1	5294	1	F	NR_MC_enh-Core

The CR is in principle agreed

R2-2502634	Correction to ULTxSwitchingBandPair-r18	Nokia	CR	Rel-18	38.306	18.5.0	1262	-	F	NR_MC_enh-Core

-	Huawei and Xiaomi don’t agree with the first change

-	Docomo thinks the intention of first change is agreable but needs time to review.

Correction to ULTxSwitchingBandPair-r18 addition for up to two TAGs is not needed

The following change will be merged with the rapporteur CR “Add the definition of uplinkTxSwitchingPeriodForBandPair-r18 within ULTxSwitchingBandPair-r18”

The CR is not pursued

### 7.8.3	Other

RAN3, SA2, SA3, CT1 led items and others, e.g. eNPN, Slicing, NTN self evaluation issues, etc.

# 8	Rel-19

## 8.0	General

This AI is reserved for Rel-19 LSs from other WGs.  No contributions are expected on these LSs for this meeting

R2-2501707	LS on Rel-19 RAN1 UE features list for NR after RAN1#120 (R1-2501399; contact: NTT DOCOMO, AT&T)	RAN1	LS in	Rel-19	NR_MIMO_Ph5, NR_duplex_evo, Netw_Energy_NR_enh, NR_LPWUS, NR_XR_Ph3	To:RAN2	Cc:RAN4

Noted

R2-2501724	LS on Rel-19 higher layers parameters list Post RAN1#120 (R1-2501644; contact: Ericsson)	RAN1	LS in	Rel-19	NR_MIMO_Ph5, NR_duplex_evo, Netw_Energy_NR_enh, NR_LPWUS, NR_XR_Ph3	To:RAN2, RAN3	Cc:RAN4

Noted

ASN.1 review/RRC guidelines

R2-2502093	Discussion on R19 ASN1 review	Huawei, HiSilicon	discussion	Rel-19

Timeline for ASN1 review

Proposal 1: Think about how to leave enough time for ASN1 review before RAN2#131bis meeting.

Optimized ASN.1 review process

Proposal 2: For the ASN1 review in R19, adopt the following procedure:

-	Each WI has a discussion RRC file (e.g., version 19.0.0 or approved WI CR) where WI-specific comments are made. Separate files collect cross-WI issues and miscellaneous small topics

-	Each WI RRC CR rapporteur marks the comments as ProgAgree/PropRej/PropDisc; WI RRC CR rapporteurs also implement the ProgAgree comments into the CR, same as today

-	RRC rapporteur uses the VBA tool to collects all the comments from the RRC file of each WI and merge it into a mega MS excel spreadsheet, same as today

-	Discuss during the meeting the PropDisc and flagged PropRej/PropAgree issues. Agree on the WI-specific CRs, same as today

Proposal 3: Start the WI-specific ASN1 review early before R2#131bis, before the publication of the first R19 RRC spec (v19.0.0) based on the approved WI CR. Then, start the cross-WI ASN1 and WI-specific review immediately based on the first RRC specification after it is published.

Better format for ASN1 review

Proposal 4: Optimize the ASN1 review format, by inputting the comments to another file or add the comment to the end of the file, and bubble comments only include issue numbers.

Discussion

-	Qualcomm thinks proposal 2 we don’t need to RIL based system as this will overload and complicate the system.   Huawei is open to this as it is just to facilitate the book keeping process.

-	Xiaomi agrees with Huawei.  And for proposal 4 with bubbles creates problem so we can go back to LTE system where every company maintains their comments and rapporteur merges them.

-	ZTE thinks that all RRC WI CRs should be compilable and we can do the merge.  Ericsson thinks that the compilability of the CRs is not the problem.

-	Vivo thinks that the bubble is the main issue, so we should separate.

-	Apple thinks we have a good process and we should make sure that the functional completeness of the WI is in good state.

-	Oppo agrees that we should follow existing process and we can optimize the comments with separate files.

-	Nokia agrees that we should have a common review of the full spec.

-	Ericsson thinks that companies should continue reviewing the WI CRs after the CR is reviewed.

-	ZTE thinks that we need to ensure we get stable input from RAN1/RAN4

Noted

Keep ASN.1 review (i.e. RIL based) on the common merged CR only.

Discuss and identify how to merge the CRs to enable earlier start of ASN.1 review

R2-2502764	Report on [POST129][ASN.1] ASN.1 Review Process Improvements in Release 19 (Nokia)	Nokia	discussion	Rel-19

Proposal 1: Split the review file into multiple parts, respecting section boundaries. The exact split and number of review files is FFS.

Proposal 2: Discuss the following alternatives to provide comments in the review files:

•	Reduce the text content of the RILs, and possible reduce the RIL template.

•	Use a separate review file per WI

•	Use the Word comment bubbles as pointers to a table of RIL comments, which could be stored

o	In a separate Excel file

o	In a separate Word file

o	In the review Word file in a table at the end

Proposal 3: Continue to use the manual file locking procedure as a baseline, but keep as FFS the possibility of automatically creating the lock file and providing for download the next version of the review file with the version number incremented.

Continue discussions

[POST129bis][002][ASN.1 review] Process improvements (Nokia)

Intended outcome:

Deadline:  Long

After offline Nokia explains that companies want more time to continue this until Malta

R2-2502643	RRC CR Guidelines	Ericsson	discussion

R2-2503171	RRC CR Guidelines	Ericsson	discussion

Add tdoc reference to the RAN2 handbook

Noted

R2-2502440	R19 ASN.1 identifier naming conventions and check list	Huawei, HiSilicon	discussion	Rel-19

Noted

UE capability handling

R2-2502767	Rel-19 UE capability handling	Xiaomi	discussion	Rel-19	NR_newRAT-Core

Proposal 1: Same as Rel-18, if UE capability is implemented during RAN1/4 related NR Others discussion, draft CRs (running CRs) for 38.331 and 38.306 should be produced and endorsed, then merged to mega CR.

Proposal 2: RAN1/4 related CR contributor(s) to ensure that:

-	Submitted endorsed CRs as draft CR is preferred.

-	The author identity of the endorsed CRs for RAN1/4 related NR Others capability is set to the WI-code for all the changes in the CRs.

-	For RAN1/4 related NR others, to add the Index and FG description on top of the UE capability in 38.331.

-	The drafting rules, including the correct use of word-styles, using latest specification version are to be followed.

-	Strictly following R2-2501458 ‘Guideline on writing a CR’ while writing the CR.

Proposal 3: Same as Rel-18, to session chairs of WI and RAN2 WI-specific UE capability running CR rapporteurs, please be reminded to ensure that:

-	RAN2 features and capabilities, that are developed only in RAN2, are developed individually per WI, under WI-specific agenda Items. Draft CRs (running CRs) for 38.331 and 38.306 are produced and endorsed.

-	The author identity of RAN2 capability CR is set to the WI-code for all the changes in the CRs.

-	The 306 CRs shall include an annex containing the RAN2 determined UE capabilities in the feature list format (similar to annex containing RAN2 agreements), for easy compilation into the TR38.822 in the later stage (as agreed in RAN2 #116-e). The annex of RAN2 determined UE capabilities feature list should align with field description.

-	The drafting rules, including the correct use of word-styles, using latest specification version are to be followed.

-	Strictly following R2-2501458 ‘Guideline on writing a CR’ while writing the CR.

-	Apple thinks that we would need to resolve the annex numbering and also cross feature dependencies we need to check.

Proposal 5: RAN2 to inform RAN1/4 on the following:

-	For RAN1/4 TEI19 UE features, RAN1/4 provide RAN2 with the unique TEI identifier for each feature group introduced in TEI19. The corresponding TEI identifier can be added as part of ‘Feature’ column.

Based on above discussion,

Proposal 6: Same as Rel-18, the final CRs from the mega rapporteur to have:

-	Implemented all feature groups based on RAN1/4 FL (author identify set to WI code for that WI), including RAN1/4 TEI19 feature list (author identify set to RAN1/4 defined TEI identifier).

o	For RAN1/4 UE feature list, the feature list Tdoc numbers to be provided in the coversheet

-	Merged RAN2 WI-specific UE capability endorsed CRs, RAN2 endorsed CRs in RAN1/4 related NR Others (if any) (author identify set to WI code for that WI).

o	RAN2 Tdoc number of the endorsed CRs (title and number) in the coversheet.

-	Note: Mega CR implementation timeline needs to be aligned with ASN.1 review

The proposals are endorsed and will be used as baseline for future UE capability work.

Discussion

-	Qualcomm thinks that we need to remind RAN4 that all UE capabilities have to be captured in the feature list and those will be the only capabilities that we capture.

[AT129bis][003][UE caps] LS to RAN1/RAN4 (Xiaomi)

Intended outcome: Send an LS indicating that RAN2 expects a full feature list from both RAN1 and RAN4 and will only implement UE capabilities that are in this UE feature list.   RAN4 can still send early LSs to explain the capabilities, but it is important that they capture all the feature lists at the end.  RAN2 also expects that for TEI specific UE capabilities they provide the TEI identifier for each feature.

Remind them the future guidance.

Remind them about RRC parameters and functional stability of features by May – given very short ASN.1 review time.   Also provide TEI identifier for RRC parameters for TEI19 features.

Deadline:  Friday

R2-2503166	LS on Rel-19 higher layer parameter and UE Feature	RAN2	LS out	Rel-19		To:RAN1, RAN4

Add To RAN1 and RAN4, that RAN2 would prefer that the higher layer parameter list is sent by the end of May and UE feature list latest by August.

The LS is approved in R2-2503180 with the change above

## 8.1	AI/ML for NR air interface

(NR_AIML_air-Core; leading WG: RAN1; REL-19; WID: RP-250792 and SID: RP-243245)

Time budget: 2.5 TU

Tdoc Limitation: 5 tdocs

### 8.1.1	Organizational

LS, Rapporteur input, including workplan.

Including outcome of [POST129][024][AI PHY] Stage 2 running CR and [POST129][025][AI PHY] RRC running CR (Ericsson)

LS

RAN2 in “To”

R2-2501721	LS on AI/ML Positioning Case 3b (R1- 2501628; contact: Ericsson)	RAN1	LS in	Rel-19	NR_AIML_air	To:RAN3, RAN2

Noted

R2-2502661	LS on AI/ML Positioning Case 3b	Ericsson	discussion	Rel-19

Not treated

R2-2501759	Reply LS on AI/ML UE sided data collection (S5-250828; contact: Intel, NEC, Huawei)	SA5	LS in	Rel-19	AIML_MGT_Ph2	To:RAN, RAN2	Cc:SA, SA2, SA3

From RAN2 point of view UP tunnel between UE and gNB is not considered for 5GA R19/20, as currently the UP tunnel doesn’t terminate in the gNB

Noted

[AT129bis][012][AI PHY] Reply LS to SA5 (Huawei)

Intended outcome: Agree to LS by email

Deadline:  Thursday

R2-2503168	Reply LS on AI/ML UE sided data collection	RAN2	LS out	Rel-19	AIML_MGT_Ph2	To:SA5	Cc:SA, SA2, SA3, RAN

The LS is approved

RAN2 in “CC”

R2-2501710	Reply LS on LMF-based AI/ML Positioning for Case 2b (R1-2501523; contact: vivo)	RAN1	LS in	Rel-19	AIML_CN	To:SA2	Cc:RAN2, RAN3

Noted

R2-2501711	Reply LS on LMF-based AI/ML Positioning for Case 3b (R1- 2501525; contact: Ericsson)	RAN1	LS in	Rel-19	AIML_CN	To:SA2	Cc:RAN2, RAN3

Noted

R2-2501728	Reply LS on LMF-based AI/ML Positioning for case 3b (R3-250796; contact: ZTE)	RAN3	LS in	Rel-19	AIML_CN, NR_AIML_air, NR_AIML_air-Core	To:SA2	Cc:RAN1, RAN2

Noted

Running CRs

R2-2501806	Introduction of AI for Air interface feature in 38300	vivo(Rapporteur)	draftCR	Rel-18	38.300	18.5.0	NR_AIML_air-Core

-	Ericsson thinks that we should add some diagrams to illustrate applicability reporting.  Nokia agrees.

Rapporteur will add back the bubbles for the next revision to be reviewed over email discussion

Add diagrams for the overall applicability reporting procedure

The CR will be used as a baseline for the next revision

[POST129bis][013][AI PHY] 38.300 Running CR (Vivo)

Intended outcome: Update 300 CR based on agreements from RAN2#129bis

Deadline:  Long

R2-2501920	38.305 running CR for AIML Positioning	CATT	draftCR	Rel-19	38.305	18.5.0	B	NR_AIML_air-Core

Continue discussion over email

[POST129bis][014][AI PHY] 38.305 Running CR (CATT)

Intended outcome: Update CR based on agreements from RAN2#129bis

Deadline:  Long

R2-2502123	Running MAC CR for AI/ML for Air Interface	Apple (Rapporteur)	draftCR	Rel-19	38.321	18.5.0	B	NR_AIML_air-Core

-	Samsung thinks that looking at the current CR there is no need for MAC specification.

Postpone and Revisit MAC CR next meeting and agree on need for CR based on agreements

R2-2502618	Running CR for AI/ML Positioning Accuracy Enhancements	Qualcomm Incorporated	draftCR	Rel-19	37.355	18.4.0	B	NR_AIML_air-Core

Continue discussion over email

[POST129bis][015][AI PHY] 37.355 Running CR (Qualcomm)

Intended outcome:

- Update CR based on agreements from RAN2#129bis

- List of remaining open issues

Deadline:  Long

R2-2502903	Draft RRC running CR for AIML PHY	Ericsson	draftCR	Rel-19	38.331	18.5.1	B	NR_AIML_air-Core

Use the CR as a baseline for further email discussion

[POST129bis][016][AI PHY] 38.331 Running CR (Ericsson)

Intended outcome:

- Update CR based on agreements from RAN2#129bis

- List of remaining open issues

Deadline:  Long

Other Documents

R2-2502793	Open issue list for running 37.320 CR for R19 AI for PHY	Huawei, HiSilicon, Ericsson	discussion	Rel-19	NR_AIML_air-Core

Proposal 1: for OAM centric, only adopt management based immediate MDT for NW-side data collection.

RAN2 assumes that for OAM centric, only adopt management based immediate MDT for NW-side data collection

Proposal 2: It is proposed to support immediate MDT for NW-side data collection for NR-DC, i.e. for the NW-side data collection in NR-DC scenario, the MN and SN can be configured independently by the OAM, and the MN and SN can independently configure and receive the reports from the UE (as in existing TS 37.320).

-	LG thinks that we cannot send this via SRB3.  Apple thinks that NR DC is not in the scope.

For NR-DC, the configuration and reporting of NW sided data collection can only be via MN

Proposal 3: It is proposed to send a LS to SA5 and RAN3 to check with them if there are any concerns with RAN2 agreements.

Noted

R2-2502794	Draft LS on OAM-centric solution for NW-side data collection	Huawei	LS out	Rel-19	NR_AIML_air-Core	To:SA5, RAN3

The LS is revised in R2-2503162 with current agreements

R2-2503162	Draft LS on OAM-centric solution for NW-side data collection	Huawei	LS out	Rel-19	NR_AIML_air-Core	To:SA5, RAN3

RAN2 respectfully asks SA5 and RAN3 to take this agreements into account for their work and notify us if there are any concerns to support this for Rel-19

The LS is approved in R2-2503170 with the action item above

R2-2503170	LS on OAM-centric solution for NW-side data collection	RAN2	LS out	Rel-19	NR_AIML_air-Core	To:SA5, RAN3

Approved

### 8.1.2	Functionality based LCM

Contributions should focus on general understanding of LCM procedure (except for data collection and model transfer/delivery), what is required to enable the UE to perform different steps of the LCM procedure, what is the granularity of functionality, dependencies with RAN1 and what is needed from RAN1 to progress in RAN2

Contributions should be submitted in 8.1.2.x and aspects related to data collections should be submitted in data collection section

Two-sided model discussions are out of scope of this AI

#### 8.1.2.1	LCM for NW-sided model for Beam Management use case

LCM related to NW-sided model for beam management use case only

No contributions expected for this meeting, waiting for further RAN1 progress

#### 8.1.2.2	LCM for UE-sided model  for Beam Management use case

Including functionality identification, additional conditions and further reporting of applicable functionalities, and any necessary signaling/protocol aspects. Contributions should attempt to take into consideration the reply LS from RAN1 (R1-2410898) on BM applicable functionality reporting.

Details related to Option B: Inference related parameters for applicability determination/reporting

R2-2501783	Discussion on Applicable Functionality Reporting Option B for BM	OPPO, Lenovo, ZTE Corporation, Apple, Huawei, HiSilicon, CATT, vivo, CMCC, NTT DOCOMO, Samsung, LG Electronics, Xiaomi, InterDigital	discussion	Rel-19	NR_AIML_air-Core

Proposal 1: Option B works like the following way:

    - Procedure 1: UE receives RRCReconfiguration message including one set or multiple sets of inference related parameters via OtherConfig.

    - Procedure 2: For initial reporting, UE sends RRCReconfigurationComplete message including applicability and/or inapplicability, e.g. by indicating whether one set or multiple sets of inference related parameters are applicable or inapplicable.

    - Procedure 3: For subsequent reporting upon change: UE sends UEAssistanceInformation message including applicability and/or inapplicability, e.g. by indicating whether one set or multiple sets of inference related parameters are applicable or inapplicable.

Proposal 2: For Option B, after reporting applicability, the UE waits NW to send RRCReconfiguration message with full inference configuration in CSI-ReportConfig.

Noted

Alternative view to Procedure 1:

R2-2502763	LCM for UE-side Beam Management	Nokia	discussion	Rel-19	NR_AIML_air-Core

Proposal 1: CSI-ReportConfig, included in CSI-MeasConfig -> csi-ReportConfigToAddModList, is used to transmit one or multiple sets inference-related parameters for beam prediction.

Proposal 2: Add a flag to CSI-ReportConfig to indicate that a configuration contains one or multiple sets of inference-related parameters instead of a full configuration. This flag could also be used  as an indication that the configuration is subject to the applicability determination procedure.

Proposal 3: As an alternative to providing inference-related parameters in CSI-MeasConfig -> csi-ReportConfigToAddModList, a new list of CSI-ReportConfigs containing inference-related parameters could be defined.

Noted

Discussion on partial configuration (Otherconfig or CSI csi-ReportConfigToAddModList

-	NEC and qualcomm support CSI-measconfig

-	Qualcomm has concerns with otherconfig as there may be ambiguity.   Ericsson agrees with other config as we don’t need to provide the full configuration.   Apple doesn’t understand ambiguity, RAN1 just indicates other config and we are working on signaling structure.

-	LG thinks that from signaling perspective other config is more efficient.

-	ZTE, LG, Apple, Xiaomi, Samsung agree with Otherconfig.  Samsung changed their mind as there are mandatory IEs for CSIconfig and we should make a generic framework and this is the advantage of the Otherconfig.   We can address the issue of other config being per cell and BWP

Proposal from Interdigital from unofficial offline discussion

RAN2 assumes UE receives RRCReconfiguration message including one set or multiple sets of inference related parameters via OtherConfig for option B. This assumption can be confirmed (i.e., whether to reconsider CSI-ReportConfig) after receiving Option B inference related parameters (e.g., in RAN1 RRC parameters list).

Potential aspects to consider if RAN2 revisit:

-	To reconsider CSI-ReportConfig for option B, for example, if the list of inference related parameters is fully contained within existing CSI-ReportConfig.

-	to take into accounts UE behaviour when confirming the assumption e.g., whether option A and option B result in different UE behavior

-	Oppo thinks we should send an LS to give them our details

Agreements on option B 1 RAN2 assumes UE receives RRCReconfiguration message including one set or multiple sets of inference related parameters via OtherConfig for option B. This assumption can be confirmed (i.e., whether to reconsider CSI-ReportConfig) after receiving Option B inference related parameters (e.g., in RAN1 RRC parameters list). Potential aspects to consider if RAN2 revisit: - To reconsider CSI-ReportConfig for option B, for example, if the list of inference related parameters is fully contained within existing CSI-ReportConfig. - to take into accounts UE behaviour when confirming the assumption e.g., whether option A and option B result in different UE behavior

Alternative view to Procedure 2:

R2-2501807	Discussion on LCM for UE-sided model for Beam Management	vivo	discussion	NR_AIML_air-Core

Proposal 5. For Option B, the UE feedbacks the applicability reporting via a UAI message upon receiving one set or multiple sets of inference related parameters.

Noted

Parameters included within Option B

R2-2501783	Discussion on Applicable Functionality Reporting Option B for BM	OPPO, Lenovo, ZTE Corporation, Apple, Huawei, HiSilicon, CATT, vivo, CMCC, NTT DOCOMO, Samsung, LG Electronics, Xiaomi, InterDigital	discussion	Rel-19	NR_AIML_air-Core

Proposal 3: For Option B for BM Case 1/2, one set or multiple sets of inference related parameters can be configured in OtherConfig, where each set in OtherConfig contains the following parameters indicated in RAN1 reply LS (R1-2410898) as baseline:

    - One or more associated ID(s).

    - Set A related information.

    - Set B related information.

    - Time instances related information for prediction (For BM Case 2 only).

FFS the parameter details for Set A/Set B.

FFS whether associated ID is mandatory or optional.

Other details of Option B parameters (if time allows)

R2-2502108	Further Discussion on LCM for UE-side Model for AI-BM	MediaTek Inc.	discussion

Proposal 2: Each set of inference parameters is identified by an index.

Not treated

R2-2502124	Remaining issues on LCM procedure of UE-sided model for AI/ML based beam management	Apple	discussion	Rel-19	NR_AIML_air-Core

Proposal 9: To avoid ambiguity of applicability reporting between Option A and B, introduce CSI-ReportConfigId under the set of inference related parameters as identifier of the set.

Proposal 10: RAN2 confirm that option A and option B can be configured in the same RRCReconfiguration message with the unified applicability report procedure. And a separate UE capability is introduced for option B to allow more flexibility.

Not treated

Applicability reporting

Cause of non-applicability

R2-2501921	Discussion on applicability reporting for BM use case	CATT	discussion	Rel-19	NR_AIML_air-Core

Proposal 7: No explicit cause is reported along with the non-applicable functionality reporting.

Noted

R2-2501940	Unified Signaling Structure and Further consideration on applicability reporting	Xiaomi	discussion	Rel-19	NR_AIML_air-Core

Proposal 6: Together with inapplicability reporting, UE further indicates cause value of inapplicability, i.e., 1) not applicable to NW-side additional condition, 2) not applicable to UE-side additional condition and/or 3) model is not available in device.

Noted

Discussion

-	Apple, LG agrees with Xiaomi and it is helpful to the network.

-	Qualcomm thins thinks that we should just report that a model is not available or applicable temporarily.   Nokia agrees we can simplify.   Xiaomi thinks that we can do a simple indication whether model is available or not.   ZTE also thinks it is useful and it can save RRC signaling.  Interdigital agrees and if the model is not available there is no reason to keep it.  Lenovo, Ericsson, Samsung agrees with Qualcomm but we have to further discuss the simple cause.

-	CMCC, Vivo, Huawei agrees with CATT and the network doesn’t care why I tis not applicable.

-	Oppo thinks that we should give a cause only if it can impact the behaviour otherwise a simple indication is not very useful.

Together with inapplicability reporting, UE further indicates a simple cause value of inapplicability FFS how to define this simple cause related to model availability and how we capture it in the spec

Inference configuration handling upon non-applicability

R2-2501784	LCM for UE-sided model  for Beam Management use case	OPPO	discussion	Rel-19	NR_AIML_air-Core

Proposal 2: Upon receiving one or more full inference configuration(s) via RRCReconfiguration message, UE shall maintain all the full inference configuration(s) no matter the full inference configuration is applicable or inapplicable until the network releases it explicitly.

Proposal 3: Upon receiving one or more full inference configuration(s) via RRCReconfiguration message, UE shall not activate the periodic CSI reporting for an inapplicable full inference configuration.

Noted

R2-2502902	LCM for UE-side models for beam management	Ericsson	discussion

Proposal 6: If the UE reports a periodic CSI report configuration (i.e. full inference configuration in CSI-ReportConfig) as non-applicable in the initial applicability report, the UE releases the configuration.

Noted

Discussion

Upon receiving one or more full inference configuration(s) via RRCReconfiguration message, UE shall maintain all the full inference configuration(s) no matter the full inference configuration is applicable or inapplicable until the network releases it explicitly.

Prohibit timer for applicability reporting

R2-2502411	Continuous Discussion On LCM for UE-sided model	ZTE Corporation	discussion	Rel-19	NR_AIML_air-Core

Proposal 3: The UE can be configured with a prohibit timer. This timer starts once the UE initiates applicability reporting for a specific functionality either via the UAI or RRCReconfigurationComplete. Before the prohibit timer expires, the UE will refrain from initiating another UAI to report applicability for the same functionality. Reporting of inapplicability is not restricted by this prohibit timer.

Noted

R2-2502599	Discussion on LCM for UE-sided model for Beam Management use case	Huawei, HiSilicon	discussion	Rel-19	NR_AIML_air-Core

Proposal 13: The NW can configure different values of the prohibit timer for “applicable to non-applicable” update and “non-applicable to applicable” update to be applied for functionalities that are not currently configured for inference operation. If a functionality that is currently configured for inference operation becomes non-applicable, the UE should immediately report the status change.

Noted

Discussion on prohibit timer

-	Qualcomm, Apple, Lenovo, Xiaomi, ZTE thinks it shouldn’t be based on functionality but just on applicability reporting and we should only have it from non-applicable to applicable.

-	Nokia, Ericsson, Samsung, Oppo, LG, vivo doesn’t see the reason as we shouldn’t have models that are fluctuating so much and if they are we should just deconfigure.  Ericsson doesn’t see the gain.

-	Interdigital thinks we don’t know how they fluctuate so we should have a simple mechanism that can be deconfigured by the network.

No prohibit timer is introduced

- Agreements on applicability reporting

- Together with inapplicability reporting, UE further indicates a simple cause value of inapplicability FFS how to define this simple cause related to model availability and how we capture it in the spec

- Upon receiving one or more full inference configuration(s) via RRCReconfiguration message, UE shall maintain all the full inference configuration(s) no matter the full inference configuration is applicable or inapplicable until the network releases it explicitly.

- No prohibit timer is introduced

Contents of applicability report (if time allows)

R2-2502636	On LCM for UE-sided Model for Beam Management Use Case	SHARP Corporation	discussion

Proposal 11: The UE may indicate preferred configuration (s) to the network (e.g., preferred CSI-ReportConfig) in step 4.

Not treated

R2-2502366	Left issues related to applicability report for AIML based BM	Lenovo	discussion	Rel-19

Proposal 6: In the applicability report, cell information is needed to unambiguously identify CSI report configurations for prediction.

Not treated

Data collection (if time allows)

Remaining details on data collection request

R2-2502124	Remaining issues on LCM procedure of UE-sided model for AI/ML based beam management	Apple	discussion	Rel-19	NR_AIML_air-Core

Proposal 1: The UE request signaling for data collection of AI/ML based beam management may include:

•	The indication on start/stop of data collection

•	Preferred configuration from a list of candidate configurations provided by NW, where each candidate configuration includes at least CSI resource set A/B and associated ID. Its details are up to RAN1.

Proposal 2: Introduce UAI message for UE request of data collection configuration. And it is up to UE implementation when to send the request.

Noted

R2-2502599	Discussion on LCM for UE-sided model for Beam Management use case	Huawei, HiSilicon	discussion	Rel-19	NR_AIML_air-Core

Proposal 15:  RAN2 to agree that the UE just indicates a simple request to NW if it needs data collection configurations, and UE capability reporting is left to RAN1.

Noted

Discussion

-	Qualcomm agrees with apple as the UE may have already done some training and it is interested on data for certain configurations.  Xiaomi also agrees thinks it is impossible for the network to guess.  Samsung, LG, Ericsson, Lenovo, ZTE also agrees.   Ericsson thinks we should leave the content of signaling FFS.    Interdigital agrees but just asks if the preferred config information is optional

-	Nokia has serious concerns with this as this appears to be to support option 1a.

-	Oppo thinks that the network can configure based on UE capability.

-	Tmobile has concerns with this proposal as how the network controls.  Samsung explains that we have already agreed that it is up to the network when and how to configure.

-	Nokia suggests that we provide the preference in the RRC configuration complete.  Samsung thinks it is not urgent to send a request at the RRC configuration complete.

- Agreements on data collection configuration

The UE can request measurement configuration for data collection of AI/ML based beam management.   The request can contain one or more of the following:

•	An indication on start/stop of data collection

•	Preferred configuration from a list of candidate configurations provided by NW.  Details of signaling are FFS.  It is up to network what it configures at the end.

Introduce UAI message for UE request of data collection measurement configuration. And it is up to UE implementation when to send the request.

Not treated

UE behaviour when it cannot fulfil data collection request

R2-2502729	Discussion on LCM for UE-sided model for BM	CMCC	discussion	Rel-19	NR_AIML_air-Core

Proposal 4: When the UE can’t perform data collection for model training based on received configuration, other UE behaviors in connected mode should not be affected.

Proposal 5: An indication is introduced in RRCReconfigurationComplete message when UE can’t perform data collection based on received configuration.

R2-2502124	Remaining issues on LCM procedure of UE-sided model for AI/ML based beam management	Apple	discussion	Rel-19	NR_AIML_air-Core

Proposal 3: As the UE may send data collection request via UAI based on its implementation, no need to introduce a separate indication signaling when UE can’t perform data collection based on received configuration.

Prohibit timer for data collection configuration request

R2-2502263	LCM for UE-sided Model for BM	Google	discussion	Rel-19	NR_AIML_air-Core

Proposal 9: For data collection configuration with UE request, a prohibit timer is introduced to prevent the UE from immediately re-initiating a data collection request.

Performance monitoring (if time allows)

R2-2502024	Discussion on LCM for UE-sided Model for Beam Management Use Case	Fujitsu	discussion	Rel-19	NR_AIML_air-Core

Proposal 5 RAN2 starts at least the following discussions while waiting for further RAN1 input.

	Mechanism to trigger the performance monitoring procedure.

	Potential signaling to complete the performance monitoring procedure.

R2-2502020	LCM for UE-sided model for BM	LG Electronics	discussion	Rel-19	NR_AIML_air-Core

R2-2502115	Discussion on LCM for UE-sided model for beam management	Samsung	discussion	Rel-19	NR_AIML_air-Core

R2-2502239	Further consideration on LCM for UE-sided model for BM use case	China Telecom	discussion	Rel-19	NR_AIML_air-Core

R2-2502280	Discussion on signalling procedure of supporting applicability report Option B	NEC	discussion	Rel-19	NR_AIML_air-Core

R2-2502290	On LCM for UE-sided Models for Beam Management	Qualcomm Incorporated 	discussion	Rel-19

R2-2502421	Discussion on UE-sided model LCM for BM	Transsion Holdings	discussion	Rel-19

R2-2502434	Discussion on LCM for UE-sided model for Beam Management	Spreadtrum, UNISOC	discussion	Rel-19

R2-2502483	Some aspects for model monitoring on UE side	Sony	discussion	Rel-19	NR_AIML_air-Core

R2-2502590	Discussion on LCM for UE-Side Model for Beam Management	Futurewei Technologies	discussion	Rel-19

R2-2502772	LCM for UE-sided Model for Beam Management Use Case	SHARP Corporation	discussion	R2-2502636

R2-2502637	LCM for UE-sided model for Beam Management use case	InterDigital	discussion	Rel-19	NR_AIML_air-Core

R2-2502816	Discussion on LCM for UE-sided models	ASUSTeK	discussion	Rel-19	NR_AIML_air-Core

R2-2502854	Remaining Issues on LCM for UE-sided model in Beam Management use case	Kyocera	discussion

#### 8.1.2.3	LCM for Positioning use case

Contributions should focus on LCM for UE-sided model, but can discuss NW-sided model. Aspects related to data collection should be covered in 8.1.3

LMF control on applicability reporting

R2-2502137	Discussion on LCM for POS use case	Samsung	discussion	Rel-19	NR_AIML_air-Core

Proposal. 2: UE reports the applicable functionality to the LMF by the LPP provide capabilities message without any additional LMF control.

Noted

R2-2502367	LCM for AIML based positioning with UE-sided model	Lenovo	discussion	Rel-19

Proposal 2: If any mechanism is needed to avoid UE reporting “applicability” right after “inapplicability” has been reported for the same AIML based positioning, a prohibit timer based approach can be considered.

Noted

Fallback/switching from AIML to non-AIML

R2-2502758	Discussion on UE autonomous switching between AI/ML and non-AI/ML methods	LG Electronics Inc.	discussion	Rel-19

Proposal 2: UE autonomous switching between AI/ML and non-AI/ML methods should be supported.

Proposal 3: Options/configurations for UE autonomous switching can be configured in RequestLocationInformation message e.g., LocationInformationType.

Proposal 4: Four options can be utilized in LocationInformationType for UE autonomous switching.

    - Option 1: Switching AI/ML-based calculation to legacy UE-based calculation using same measurement, e.g., switch AI/ML-based DL TDoA to legacy DL TDoA

    - Option 2: Switching AI/ML-based method to legacy method, e.g., switch AI/ML-based DL TDoA to legacy DL AoD:

    - Option 3: Switching AI/ML-based UE-based method to legacy UE-assisted method, e.g., switch AI/ML-based UE-based DL TDoA to legacy UE-assisted DL TDoA)

    - Option 4: Switching AI/ML-based UE-based method to AI/ML-based UE-assisted method, e.g., switch AI/ML positioning Case 1 to AI/ML positioning Case 2b

Noted

R2-2502617	LCM for positioning use case	Qualcomm Incorporated	discussion

Proposal 1: UE autonomous switching/fallback between AI/ML and non-AI/ML methods is not allowed.

Proposal 2: Switching/fallback to non-AI/ML positioning can be supported by including multiple positioning methods in a LPP Request Location Information message. No additional specification work is foreseen specifically for supporting "switching/fallback operation".

Noted

Discussion

-	Apple, Huawei, Vivo, Ericsson, Nokia agrees with Qualcomm and there is no RAN2 impacts

-	Samsung thinks it is not clear the LMF can indicate which method that can used.

-	ZTE is concerned that the UE may always use the legacy mechanism.   Qualcomm explains that the only reason it will allow multiple methods is to increase the probability of getting location and it shouldn’t care what the UE uses to get the location information.

-	LG thinks that the UE behavior is not clear when it gets multiple positioning methods

Other details (if time allows)

Functionality management

R2-2501808	Discussion on remaining issues of AI/ML enhanced positioning	vivo	discussion	NR_AIML_air-Core

Proposal 5: LMF is responsible for functionality management based on performance monitoring results calculated by target UE or LMF.

Noted

(De)activation of inference configuration

R2-2502638	LCM for Positioning use case	InterDigital	discussion	Rel-19	NR_AIML_air-Core

Proposal 2: An AIML positioning functionality is considered “activated” once UE receives an LPP RequestLocationInformation from the LMF requesting inferred location information.

Proposal 3:  LMF is expected to deactivate activated functionality when it receives non-applicability indication from UE (i.e., UE doesn’t autonomously deactivate).

Noted

R2-2501941	Discussion on the LCM for AI positioning case 1	Xiaomi	discussion	Rel-19	NR_AIML_air-Core

Proposal 2: For triggered reporting, AI/ML positioning case 1 is deactivated autonomously after providing inference results in ProvideLocationInformation or after UE reports error indication?. For periodical reporting, LMF/UE deactivates AI/ML positioning case 1 via Abort message.

-	Qualcomm doesn’t think this is needed as this is all part of the normal positioning functionality.  Xiaomi and vivo thinks that the abort part should be part of stage 2.

Noted

Agreements for positioning case 1

- LMF is responsible for functionality management

- UE reports the applicable functionality to the LMF by the LPP provide capabilities message without any additional LMF control.

- Switching/fallback to non-AI/ML positioning can be supported by including multiple positioning methods in a LPP Request Location Information message. No additional specification work is foreseen specifically for supporting "switching/fallback operation".

- An AIML positioning functionality is considered “activated” once UE receives an LPP RequestLocationInformation from the LMF requesting inferred location information.

- For triggered and periodical reporting, we rely on existing positioning framework mechanisms to deactivate AI/ML positioning (no spec impact is foreseen)

Not Treated

R2-2501787	LCM for Positioning use case	OPPO	discussion	Rel-19	NR_AIML_air-Core

R2-2501827	Discussion on LCM for Positioning Use Case	HONOR	discussion	Rel-19	NR_AIML_air-Core

R2-2501856	LCM for Positioning use case	NEC	discussion	Rel-19	NR_AIML_air-Core

R2-2501922	Issues to address for AIML Positioning stage-2	CATT	discussion	Rel-19	NR_AIML_air-Core

R2-2502025	Discussion on LCM for Positioning Use Case	Fujitsu	discussion	Rel-19	NR_AIML_air-Core

R2-2502084	Discussion on LCM for positioning use case	ZTE Corporation	discussion	Rel-19	NR_AIML_air-Core

R2-2502125	Remaining issues on LCM procedure of AI/ML based positioning	Apple	discussion	Rel-19	NR_AIML_air-Core

R2-2502240	Performance monitoring for positioning use case	China Telecom	discussion	Rel-19	NR_AIML_air-Core

R2-2502443	Association of measurements and ground truth labels for positioning use-cases	Fraunhofer IIS, Fraunhofer HHI	discussion	R2-2500174

R2-2502639	Reporting of applicability and inference configurations	Nokia	discussion	Rel-19	NR_AIML_air-Core

R2-2502662	LCM For Positioning	Ericsson	discussion	Rel-19

R2-2502697	Discussion on LCM for positioning	CMCC	discussion	Rel-19	NR_AIML_air-Core

R2-2502795	Discussion on LCM for Positioning use case	Huawei, HiSilicon	discussion	Rel-19	NR_AIML_air-Core

R2-2502802	Discussion on Functionality-based LCM for Positioning Use Case	CEWiT	discussion

### 8.1.3	NW side data collection

Contributions should focus on the mechanisms and principles identified for data collection for network side model training during rel-18.   Contributions should discusss type of data required to be collected for NW sided model and UE sided model (common to NW sided and different).  Question to RAN1 should also be identified.

Handling during mobility/state transition:

R2-2502126	Remaining issues on NW-sided data collection	Apple	discussion	Rel-19	NR_AIML_air-Core

Proposal 10: On the issue of how to handle logged data in HO/IDLE/INACTIVE/RLF, RAN2 down select between the following 2 solutions:

- Solution 1: Introduce area scope similar to logged MDT (applicable to HO/RLF/INACTIVE/IDLE)

  - Area scope (i.e. PCI list) is configured in data collection configuration similar to logged MDT. It is up to NW implementation to configure area scope with vendor/PLMN info being considered.

  - The UE releases the un-retrieved data when moving out of the configured area.

- Solution 2: Introduce indication in HO/Resume/Reestablishment (applicable to HO/RLF/INACTIVE)

  - Introduce 1-bit indication on whether to release or retain un-retrieved data in RRCReconfiguration (HO command) / RRCResume / RRCReestablishment message.

  - Source cell determines whether target cell share the same PLMN/vendor based on its implementation without spec change on inter-node message (e.g. based on OAM).

Noted

R2-2502114	Disuccsion on NW side data collection	Samsung	discussion	Rel-19	NR_AIML_air-Core

Proposal 8. Upon going to RRC_IDLE or RRC_INACTIVE, UE discards any logged data

Proposal 9. In case of RLF, the new gNB indicates UE whether UE should retain or discard logged data via RRCReestablishment message. If UE is indicated to retain and has any logged data, UE can send availability via RRCReestablishmentComplete message.

Noted

R2-2501933	Discussion on NW side data collection	Xiaomi	discussion

Proposal 8: NW can configure UE whether to keep the logged data upon RLF, as part of the data collection configuration.

Proposal 9: NW can configure UE whether to keep logged data upon leaving RRC_CONNECTED in RRCRelease.

Proposal 10: If NW didn’t configure UE to keep the data, UE releases the logged data upon RLF and leaving RRC_CONNECTED.

Noted

Discussion

- Solution 1: Introduce area scope similar to logged MDT (applicable to HO/RLF/INACTIVE/IDLE)

- Solution 2: Introduce indication in HO/Resume/Reestablishment (applicable to HO/RLF/INACTIVE)

Upon going to RRC_IDLE or RRC_INACTIVE, UE discards any logged data

Discussion

-	Nokia doesn’t see a reason why the UE should keep the data when moving to idle, and we can just discard in RLF.

-	Qualcomm thinks that area solution works for all cases.

-	LG thinks it would be wasteful to discard the data so any of the solutions from apple work.

-	Lenovo thinks that both area and explicit signaling can co-exist and it should be up to the source gNB whether the UE should keep the data.

-	Ericsson, ZTE, Mediatek and Vivo think that we should discard the data in idle, inactive, rlf.

-	Ericsson, ZTE, thinks that we should just have explicit network control for HO case only.  The area solution can create a very large overhead.   Apple indicates that this is how it works in MDT.   Ericsson indicates that for MDT this is to keep logging not reporting.

-	CMCC, CATT, thinks that releasing the data is wasting UE power/resources and we should have a unified solution.

-	Oppo doesn’t see a technical reason to not keep as the data may still be valid and as long as it is network controlled.

-	Mediatek thinks it is not a problem if the data is lost as network can get data from other UEs.

-	Huawei thinks that it can be simple for idle/inactive to keep the data and most important thing is that the NW decides.    Interdigital agrees and using explicit singaling at release is not at the right time.  Qualcomm agrees that a simple signaling is a not a solution that works.  Nokia doesn’t agree that there is no impact as when the UE goes back to connected then it provide to the network for which configuration that data is collected.

-	Xiaomi asks whether the source can send it directly to the UE rather than in HO command

Agreements on Idle/inactive and HO

Introduce 1-bit indication on whether to release or retain un-retrieved data in RRCReconfiguration during/before HO.  Source gNB decides whether the data should be kept.  The indication is provided in RRCReconfiguration (i.e. not in RRC Reconfiguration from target cell).   FFS signaling details.

Upon going to RRC_IDLE, RLF, or RRC_INACTIVE, UE discards any logged data

Availability indciation (Triggering):

R2-2502730	Discussion on NW side data collection	CMCC	discussion	Rel-19	NR_AIML_air-Core

Proposal 1: The data availability indication can be reported to the network after the buffer is full.

Noted

R2-2502414	Further Discussion on NW Side Data Collection	ZTE Corporation	discussion	Rel-19	NR_AIML_air-Core

Proposal 7: A threshold can be configured to the UE for triggering the data availability report before the AS buffer becoming full. FFS: percentage threshold, or remaining buffer size threshold.

Noted

R2-2501933	Discussion on NW side data collection	Xiaomi	discussion

Proposal 2: UE triggers data availability indication upon 1) network requests, 2) reporting low power bit indication.

Noted

Discussion

-	LG, Apple, Oppo, CATT, Mediatek and Vivo thinks that this UE implementation.  The buffer size is not known at the network.   ZTE explains that the network doesn’t need to know the buffer size, it can configure remaining buffer size (e.g. 4kB).

-	Nokia thinks that the indication should be before the buffer becomes full.   Ericsson agrees with buffer full, but thinks that the data availability should be triggered as soon as the data becomes available.

-	Samsung thinks that we should report two, when it is full or threshold.

-	Qualcomm thinks that for period there is no need for the UE to report anything as the network can estimate based on what it has received.

-	Interdigital agrees with threshold and if it is configurable the network can set it to full and both can be achieved.

-	Lenovo thinks the threshold is important.

-	Huawei, CMCC thinks that we can’t leave it implementation and it is important to report before the buffer is full, but prefers to have some control on the amount of data to fetch at a time, for QoE we support what the UE supports as minimum size.

Agreements on availability indication - Availability indication can be triggered due to: - Full buffer being reached (if configured) - Buffer threshold being reached (if configured). - Low power (if configured) - The UE send a UAI that indicates: - Data is available - Reason for trigger (full buffer, threshold) - Low power indication - The encoding of the data is available/UAI and the cause value is FFS NOTE: it is up to UE Implementation how buffer threshold reached and low power is determined

Availability indication (Content):

Not treated

R2-2501809	Discussion on NW side data collection	vivo	discussion	NR_AIML_air-Core

Proposal 5:  A single bit indication would be sufficient for UE to indicate to NW that UE has available logged data. No additional information, e.g., logged data size, is needed

R2-2502369	Data collection for NW-sided model training	Lenovo	discussion	Rel-19

Proposal 6: UE can optionally indicate data size information together with the data availability indication for gNB to fetch the logged data efficiently.

R2-2502465	Discussion on NW-sided data collection for training	Huawei, HiSilicon	discussion	Rel-19	NR_AIML_air-Core

Proposal 4: The UE should include the corresponding configuration identifier in the data available indication to avoid NW requests unavailable data or support NW to request a certain data collected by UE.

R2-2502264	Data Collection for NW-sided Model Training for BM	Google	discussion	Rel-19	NR_AIML_air-Core

Proposal 5: The UE includes a cause indication (e.g., event not triggered, event triggered infrequently, low memory, low power state, overheating, or mobility) for the CSI resources with no or insufficient logged data.

Availability indciation (Signaling):

Not treated

R2-2502004	Discussion on NW-side data collection 	NEC	discussion	Rel-19	NR_AIML_air-Core

Proposal 3: Data availability indication can be carried in UAI and RRC complete message.

R2-2501785	NW side data collection	OPPO	discussion	Rel-19	NR_AIML_air-Core

Proposal 9: RRCSetupComplete/RRCResumeComplete message can be used to carry availability indication.

Proposal 11: RRCReestablishmentComplete message can be used to carry availability indication.

R2-2502904	NW-side data collection for beam management and positioning	Ericsson	discussion

Proposal 13: The indications for data availability, low power, and buffer full are reported independently of each other.

Data collection configuration (single vs mulitple):

R2-2502109	Further Discussion on Network-Side Data Collection	MediaTek Inc.	discussion

Proposal 1:The UE maintains only one logged measurement configuration for data collection for network-side model training. When the network provides a new logged measurement configuration, it completely replaces any previously configured logged measurement configuration.

Noted

R2-2502465	Discussion on NW-sided data collection for training	Huawei, HiSilicon	discussion	Rel-19	NR_AIML_air-Core

Proposal 11: The NW can configure one data collection configuration per use case, i.e. multiple configurations for the same use case are not supported.

Noted

R2-2502026	Discussion on NW side Data Collection	Fujitsu	discussion	Rel-19	NR_AIML_air-Core

Proposal 1 For the measurement configuration of AI/ML data collection, multiple configurations can be supported to allow measurements for multiple Set A/B combinations at UE side.

Proposal 2 Data reporting for different measurement configurations should be separated and indicated so that NW can distinguish collected data for different AI/ML-enabled features/FGs/models.

Noted

Data collection configuration (Framework):

R2-2502414	Further Discussion on NW Side Data Collection	ZTE Corporation	discussion	Rel-19	NR_AIML_air-Core

Proposal 2:	For NW side data collection with data logging for beam management, the RS resources for data collection (e.g. CSI-ResourceConfigId) are configured in a MeasObjectNR, and the data logging related configuration for data collection are configured in the reportConfigNR associated with a MeasId that is linked to the MeasObjectNR.

Noted

R2-2502594	Data Collection for Training of network-side ML Models	Nokia	discussion	Rel-19	NR_AIML_air-Core

Proposal 1: Legacy CSI-ReportConfig is reused to provide configuration for measurement and logging of CSI-RS for network-side models, i.e., added or modified with csi-ReportConfigToAddModList, and released with csi-ReportConfigToRemoveList.

Proposal 2: The UE shall be able to identify from the CSI-ReportConfig which report configurations are for logging and data collection.

Noted

Discussion

-	Qualcomm thinks that CPU management the NW should know that the logging is triggered.   LG thinks this would increase signaling.

-	LG thinks that we should use the CSI framework for the measurement configuration similar to UE Sided.

-	Vivo doesn’t think we need to introduce the Nokia.

-	Xiaomi and Apple would like to ensure forward compatibly with mobility.  Apple would like to ensure we minimize the RAN1 work.

-	Ericsson would like to use CSI-measconfig rather the CSI config report.

-	Qualcomm explains that RAN1 agreed to use CSI confir report.  Apple thinks that we can take CSI report config and create a L3 message with same information.

-	Interdigital thinks that they both work

-	Huawei, Samsung doesn’t think it makes sense to put this into Mobility framework, so we should re-use CSI framework.

-	ZTE just thinks that we need to associate the config with MO and L3 framework can have L1 measurements.   CMCC supports ZTE proposal, similar to MDT that can take L1 framework.

Next meeting proponents should work together and bring complete proposals to show specification impact and consider future use cases.

Agreements on data collection configuration

- The measurement configuration of AI/ML data collection can configure measurements for multiple sets of resources and use cases (e.g. BM, Mobility, etc)

Content of collected data:

R2-2501923	Consideration on NW side data collection	CATT	discussion	Rel-19	NR_AIML_air-Core

Proposal 4: The report of collected training data includes:

- For training data collection time marker, the absoluteTimeStamp could be the absolute time when the training data configuration is received by the UE. And the time of each measurement can be represented as a relative time from the absoluteTimeStamp;

- For training data marker, the associated ID received in the training data configuration may be used;

- For training data collection node, collection node index such as TCE ID for MDT could be used;

- The main content of training data collection could be a list similar as logMeasInfoList, and the detailed content can be revised according to further RAN1 input.

Noted

R2-2502114	Disuccsion on NW side data collection	Samsung	discussion	Rel-19	NR_AIML_air-Core

Proposal 10. No timestamp is reported for logged data.

- For periodic logging, UE uses a common list, in order to log multiple L1-RSRPs per each beam.

- For event-based logging, UE uses different lists to differentiate each event instance, in order to log multiple L1-RSRPs per each beam.

Noted

Discussion

-	Xiaomi doesn’t think the second time is not needed.  TCE ID is only needed for OAM centric

-	Apple thinks we should focus on absoluteTimeStamp, which is useful.  CMCC, Oppo the timestamp for each measuring is very important for time domain and handover.  Interdigital, Ericsson, also agrees on the importance of known the time stamp.  Ericsson also thinks we may need CGI.  Huawei agrees but there is also overhead so no need to repeat overhead, so just one absolute timestamp is sufficient.

-	Qualcomm doesn’t think timestamp is useful.   TCE ID is not needed.

-	Nokia doesn’t think the timestamp is needed as it doesn’t matter whether the data was collected at 4 or 5pm also the UE is in connected mode for only a brief period so the NW knows when the data is collected.   Samsung agrees with Nokia but for temporal domain prediction relative time is needed, but there is overhead so no need to explicit need to signal.   Lenovo thinks that the timing information would be useful.

-	Interidigtal thinks for periodic login there is no need but for event based we can use Samsung’s proposal.   Huawei agrees the NW needs to know whether the measurements are consecuitive or not.

-	ZTE thinks that the network would like to have the timing information other it would degrade the performance so can sympathize with interdigital.

-	Mediatek thinks that we have further cases like full buffer and power where the UE stops logging so the network needs to know.   Vivo agrees with mediatek and also thinks its important for HO case to know.   Huawei thinks that the UE indicates when these events happens.   Samsung explains that there is no issue at handover.

-	CMCC thinks that the UE should include the periodicity for the logging as the periodicity may change or at HO the gNB isn’t aware of the periodicity of the configuration.   This can be configurable.   Xiaomi agrees.

-	Oppo thinks that the absolute timestamp reduces specification complexity.

-	include timestamp per group of measurements

For temporal domain, the network is made aware whether there is a gap between two consecutive samples.   FFS amount of gap and whether this is implicit or explicit

Noted

SRB to be used:

R2-2502004	Discussion on NW-side data collection 	NEC	discussion	Rel-19	NR_AIML_air-Core

Proposal 6: Consider using SRB2 or a new SRB to transmit UEInformationResponse for AI data collection.

Noted

R2-2502592	Discussion on Data Collection for NW-side Model Training	Futurewei Technologies	discussion	Rel-19

Proposal 4: SRB4 can be used for data collection for NW-side model training.

Noted

Agreements

- For temporal domain, the network is made aware whether there is a gap between two consecutive samples.   FFS amount of gap and whether this is implicit or explicit

- New SRB can be configured for NW-side data collection  (with lower priority)

Not treated

R2-2502021	NW side data collection	LG Electronics	discussion	Rel-19	NR_AIML_air-Core

R2-2502126	Remaining issues on NW-sided data collection	Apple	discussion	Rel-19	NR_AIML_air-Core

R2-2502251	NW side data collection	Interdigital Inc.	discussion	Rel-19	NR_AIML_air-Core

R2-2502291	On Network Side Data Collection 	Qualcomm Incorporated 	discussion	Rel-19

R2-2502368	Data collection issues related to RAN3 and SA5	Lenovo	discussion	Rel-19

R2-2502435	Discussion on NW side data collection	Spreadtrum, UNISOC	discussion	Rel-19

R2-2502817	Discussion on NW-sided data collection	ASUSTeK	discussion	Rel-19	NR_AIML_air-Core

R2-2502832	Discussion on NW side data collection for positioning	TCL	discussion

R2-2502908	Discussion on NW-Side AI/ML Data Logging Framework	Rakuten Mobile, Inc	discussion	Rel-19

R2-2502937	Discussion on NW side data collection aspects	Continental Automotive	discussion

### 8.1.4	UE side data collection

Type of data required to be collected for UE sided model can be discussed in contributions in 8.1.3

Discuss any of the aspects identified in RANP WF

o	Study RAN aspects related to data transfer over UP

o	Discuss level of NG-RAN involvement in the control and configuration of UE side data collection.

o	Discuss NG-RAN involvement in the data transfer of UE side data collection (if any) (including visibility discussion).

o	Discuss aspects/solutions from RAN perspective that enable the data transfer to CN domain or OAM domain.

o	Discuss on the scalability aspects of CP

Not treated

LS from SA5:

R2-2501934	Discussion on UE side data collection	Xiaomi	discussion

Proposal 1: Send reply LS to SA5 to indicate current specification doesn’t support UP tunnel between UE and gNB.

R2-2501786	UE side data collection	OPPO	discussion	Rel-19	NR_AIML_air-Core

Proposal 4: For Option 3 UP solution, the data transfer path is UE ->gNB
->OAM-> Server for data collection for UE-side model training/OTT server.

Proposal 5: Reply SA5 LS to confirm the understanding in P4.

UP data transfer:

R2-2502127	Further discussion on UE-side data collection	Apple	discussion	Rel-19	NR_AIML_air-Core

Proposal 10: On data transfer solution over UP, RAN2 wait SA2 conclusion on whether/how NG-RAN can be aware of AI/ML dataset transfer (e.g. via AI/ML specific QoS flow, or AI/ML specific PDU session, or AI/ML specific protocol stack, etc.) to configure DRB accordingly.

R2-2502292	On UE Side Data Collection 	Qualcomm Incorporated 	discussion	Rel-19

Proposal 6: For user plane-based data transfer, the NG-RAN may perform PDU admission control based on information/parameters configured by 5GC to NG-RAN to achieve controllability requirements for data transfer, as in legacy PDU admission control.

Proposal 7: The legacy RRC and NAS-based procedure can be reused by NG-RAN to achieve controllability requirements for data transfer in the control-plane-based data transfer.

R2-2502027	Discussion on Data Collection via UP Tunnel for UE-sided model	Fujitsu	discussion	Rel-19	NR_AIML_air-Core

Proposal 1: RAN2 applies lower priority to UE-sided model training data compared to UL traffic of other services over UP tunnel.

Proposal 2: Utilize dedicated UP tunnel to transfer UE-sided model training data, e.g., dedicated PDU session, dedicated QoS Flow, or dedicated DRB.

CP data transfer (scalability):

R2-2502292	On UE Side Data Collection 	Qualcomm Incorporated 	discussion	Rel-19

Proposal 11: With more AI/ML-enabled use cases anticipated to be introduced in the future, the control plane-based data collection will not remain futureproof or extendable.

R2-2502952	Remaining issues on UE-side data collection	Ericsson	discussion

Proposal 5: CP based solutions require a heavier update of the NG-RAN both for the configuration of the data to be collected and for the fetching (via CP) of the collected data. Without a capillary update of the NG-RAN nodes, the ability of the UE to perform an efficient UE-side data collection will be limited, as well as the possibility for the network to balance the radio overhead. NG-RAN nodes compliance with beyond Rel.19 AIML use cases need also to be considered.

R2-2501810	Discussion on UE side data collection	vivo	discussion	NR_AIML_air-Core

Observation 2: The main difference between CP-based solutions of UE side data collection and NW side data collection is only whether the NW will further transfer the training data to the server for data collection for UE-side model training/OTT server.

Proposal 7: The framework/agreement of NW side data collection is reused for CP-based UE side data collection.

Proposal 8: No scalability issue for CP-based UE data collection and data transfer.

Visibility of collected data:

R2-2502796	Discussion on UE-sided data collection for training	Huawei, HiSilicon, OPPO, ZTE, NTT DoCoMo, China Unicom, CMCC, China Telecom, Apple, vivo	discussion	Rel-19	NR_AIML_air-Core

Proposal 1: For option 2 and option 3, RAN2 to only adopt standardized data to implement full visibility, and exclude non-standardized data, i.e. partial/no visibility.

R2-2502110	Further Discussion on UE-side Data Collection	MediaTek Inc.	discussion

Proposal 8: Support non-standardized and partially standardized data content in Solution 2 and Solution 3. Whether the non-standardized data is allowed to be carried is configured by the MNOs.

R2-2502252	UE side data collection	Interdigital Inc.	discussion	Rel-19	FS_NR_AIML_air_Ph2

Proposal 1: The format of the data/measurement to be collected will be specified per use case basis. UE vendor-specific/proprietary data can be included in a transparent container along with the standardized data

Not treated

R2-2501857	Analysis of solutions for UE side model data collection	NEC	discussion	Rel-19	NR_AIML_air-Core

R2-2501924	Consideration on UE side data collection	CATT	discussion	Rel-19	NR_AIML_air-Core

R2-2501973	Discussion on UE-side data collection	LG Electronics Inc.	discussion	Rel-19	NR_AIML_air-Core

R2-2501978	Performance metrics / KPI’s for UE sided data collection	T-Mobile USA Inc.	discussion	Rel-19	NR_AIML_air-Core

R2-2502370	UE requests data collection for AI based positioning	Lenovo	discussion	Rel-19

R2-2502413	Further Considerations on UE Side Data Collection	ZTE Corporation	discussion	Rel-19	NR_AIML_air-Core

R2-2502593	Discussion on Data Collection for UE-side Model Training	Futurewei Technologies	discussion

R2-2502595	Data collection for UE side models	Nokia	discussion	Rel-19	FS_NR_AIML_air_Ph2

R2-2502640	Data collection for positioning case 1	Nokia	discussion	Rel-19	NR_AIML_air-Core

R2-2502731	Discussion on UE side data collection	CMCC,China Telecom	discussion	Rel-19	NR_AIML_air-Core

R2-2502752	On UE request of data collection configuration	T-Mobile USA Inc., Nokia	discussion	Rel-19	FS_NR_AIML_air_Ph2

R2-2502818	Discussion on LCM for UE-sided data collection	ASUSTeK	discussion	Rel-19	NR_AIML_air-Core

R2-2502906	Further Requirements for UE Side Data Collection	Rakuten Mobile, Inc	discussion	Rel-19

R2-2502961	UE side data collection	Samsung R&D Institute UK	discussion

### 8.1.5	Model transfer/delivery

Focus on RAN1 LS (R1-2410922) and RAN2 evaluation of feasibility of RAN1 identified solutions on two-sided model.

Including outcome of [POST129][029][AI Phy] Model transfer (Xiaomi/Ericsson)

For RAN2#129b contributions on requirements for 1-sided and 2-sided models are only expected from operators.  .  Non-operator companies are not expected to submit contributions (but are encouraged to collaborate with operators).  NOTE: the discussion on requirements may be downprioritized.

Email Discussion

R2-2501939	Report of [POST129][029][AI Phy] Model transfer (Xiaomi/Ericsson)	Xiaomi, Ericsson	discussion	Rel-19	NR_AIML_air-Core

Noted

Agreements 1 Dataset/model parameter transfer solution from NW to UE training entity shall follow below principles: - A1 - Size: From RAN2 point of view, aim to define a unified solution (e.g. OTA or non-OTA) to support various sizes of dataset/model parameter transfer (dataset and/or parameter sharing size could range from tens of KBs to hundreds of MBs, but in average around hundreds of MBs); - A2 - Continuity: Service continuity of dataset and/or parameter transfer/delivery during UE mobility needs to be supported; - A3 - Controllability: NW decides on if and when to transfer/delivery the dataset and/or model parameter from NW to UE or UE training entity (a server inside MNO or an OTT server); - A4 - Latency: Relaxed latency requirement and infrequent update; - A5 - Visibility: dataset and model parameter to be understandable by UE/UE-side training entity (a server inside MNO or an OTT server). - A6: Proprietary information of the network and UE should be respected and not disclosed. 2 The following approaches for dataset/model parameter transfer Direction A are agreed to be included in the TR: Alternative 1 (non-OTA approach): gNB -> NW dataset/model parameters collection entity -> UE training entity (a server inside MNO or an OTT server) data RAN2 analyzed area dataset/model parameter transfer CSI compression data collection at gNB NW-side dataset/model parameters collection entity (gNB/CN/OAM) for two-sided UE-part model training UE-side training entity for two-sided UE part model training. Alternative 2 (OTA approach): gNB -> NW dataset/model parameters collection entity (if needed) -> gNB -> UE -> UE training entity (a server inside MNO or an OTT server) RAN2 analyzed area UE-side training entity for two-sided UE part model training dataset/model parameter for training dataset/model parameter transfer CSI compression data collection at gNB NW-side dataset/model parameters collection entity (gNB/CN/OAM) for two-sided UE-part model training data. NOTE: The data transfer (up to RAN1 further details on what to transfer) between gNB and NW dataset/model parameters collection entity (OAM/CN) in Alternative 1/2, if needed, is up to RAN3/SA2/SA5.. 3 For non-OTA solutions ‘NW dataset/model parameters collection entity -> UE training entity (a server inside MNO or an OTT server)’, RAN2 identified the following candidates. From RAN2 point of view it is assumed they can be supported within Rel-19 existing architecture framework. RAN3, SA2, and SA5 can confirm this assumption (i.e. we can send an LS). It does not preclude RAN3/SA2/SA5 to identify other candidate solutions beyond options listed below. Option; Impacted WG; Specification impact/Implementation impact. - OAM -> UE-side training entity (a server inside MNO or an OTT server), where OAM is NW-side dataset/model parameter collection entity; SA5, SA3; Up to SA5 (any intermediate node, if any, between OAM and UE-side OTT server is up to SA5; CN involvement if needed is up to SA2/SA5 discussion). - CN -> UE-side training entity (a server inside MNO or an OTT server), where CN is NW-side dataset/model parameter collection entity; SA2, SA3; Up to SA2 (any intermediate node, if any, between CN and UE-side OTT server is up to SA2). - gNB -> OAM/CN -> UE-side training entity (a server inside MNO or an OTT server), where gNB is NW-side dataset/model parameter collection entity; RAN3, SA2, SA5, SA3; Up to RAN3, SA2, SA5 (any intermediate node, if any, between gNB/OAM, OAM/UE-side OTT server, CN/UE-side OTT server is up to RAN3/SA2/SA5). 4 From RAN2 point of view, when gNB is the NW dataset/model parameter collection entity, solution 1b (i.e. gNB -> UE via UP) is not considered as a candidate solution in 5GA Rel-19/20 5 In OTA approach, UE transfers the received dataset/model parameter to UE training entity (an OTT server) transparently to 3GPP network. 6 For OTA solutions ‘gNB -> NW dataset/model parameters collection entity (if needed) -> gNB -> UE -> UE training entity (a server inside MNO or an OTT server)’, RAN2 identified following candidate solutions but there’s no consensus on feasibility: - OTA solution 1a: gNB -> UE via CP - OTA solution 2: CN -> UE via gNB - OTA solution 3: OAM -> UE via gNB 7 The following table is captured in the TR Table 2. OTA candidate solutions gNB -> UE via CP; Challenges; - UE is only required to support 45kB RRC buffer size, according to TS 38.306. - Significant specification impact: - Other segmentation beyond RRC layer requires a new SRB protocol stack to perform segmentation, including functions such as handling segmentation, retransmission, etc - UE selection - Challenges to support E2E reliability, considering dataset/model parameter transfer is shared by different gNB/vendors during UE mobility and different RRC state transition - Uu overhead for data collection from UE and dataset/model parameter transfer to UE - Overloading CP with large datasets would disrupt core control message transmission (e.g. service degradation, reliability, etc). Potential suitable scenario; - Small dataset/model parameter size. However, the maximum RRC segment needs to be further studied - Split large dataset/model parameter into small pieces, and potentially send to multiple UEs, then gather by UE training entity. RAN2 has not study the feasibility of split dataset/model parameter to multiple UEs.. CN -> UE via gNB; Challenges; - Same challenges as OTA solution ‘gNB->UE via CP’, if OTA solution ‘CN -> UE via gNB’ via CP - No benefit over non-OTA solution, as dataset/model parameter needs to transmit to CN, then transmit back to gNB. Relaying dataset/model parameter via gNB to UE then back to UE training entity is not desirable. - Unclear how to guarantee E2E reliability across multiple hops - A risk of proprietary information exposure if gNB and CN are not from the same NW vendor. Potential suitable scenario; - Feasibility analysis of OTA solution ‘CN -> UE via gNB’ via CP is the same as OTA solution ‘gNB -> UE via CP’ - OTA solution ‘CN -> UE via gNB’ and its feasibility is required to be evaluated by RAN3 and SA2.. OAM -> UE via gNB; Challenges; - Same challenges as OTA solution ‘gNB -> UE via CP’, if OTA solution ‘OAM -> UE via gNB’ via CP - No benefit over non-OTA solution, as dataset/model parameter needs to transmit to OAM, then transmit back to gNB. - Unclear how to guarantee E2E reliability across multiple hops - A risk of proprietary information exposure that OAM may share to a second NW vendor. Potential suitable scenario; - Feasibility analysis of OTA solution ‘OAM -> UE via gNB’ via CP is the same as OTA solution ‘gNB -> UE via CP’ - OTA solution ‘OAM -> UE via gNB’ and its feasibility is required to be evaluated by RAN3 and SA5..

[AT129bis][011][AI PHY] LS reply to RAN1 (Xiaomi)

Intended outcome: agreable LS and table capturing OTA solutions and challenges

Deadline:  Thursday

R2-2501942	[Draft] Reply LS on signalling feasibility of dataset and parameter sharing	Xiaomi, Ericsson	LS out	Rel-19	NR_AIML_air-Core	To:RAN1, RAN3, SA2, SA5

Revised in R2-2503165

R2-2503165	Reply LS on signalling feasibility of dataset and parameter sharing	RAN2	LS out	Rel-19	NR_AIML_air-Core	To:RAN1, RAN3, SA2, SA3, SA5

-	Samsung is concerned with adding new WGs as RAN1 only asked RAN2.  Nokia thinks that this normal that we can ask other work groups.

Put SA3 to cc and remove from action

Remove comment bubbles

Add Rel-19 to non-OTA before existing architecture

The LS is approved in R2-2503169

R2-2503169	Reply LS on signalling feasibility of dataset and parameter sharing	RAN2	LS out	Rel-19	NR_AIML_air-Core	To:RAN1, RAN3, SA2, SA5	Cc:SA3

=> Approved

Not treated

Requirements (if time allows)

R2-2502732	Discussion on AIML model transfer delivery	CMCC,China Telecom,CATT,ZTE,Apple,Samsung	discussion	Rel-19	NR_AIML_air-Core

Proposal 1: There is no requirements on controllability or visibility for UE-sided model transfer/delivery case y, since the model trained in OTT server can be sent to the UE directly from OTT-server (transparent to 3GPP).

Proposal 2: It is proposed to de-prioritize case z1 for one-sided model transfer/delivery, since there is no strong requirement on visibility and controllability for UE-sided model transfer/delivery from the perspective of operator.

Proposal 3: It is proposed to follow case y for UE-sided model transfer/delivery in R19, i.e. the UE-sided model is delivered over the top, which is transparent to 3GPP.

R2-2502926	Discussion on model transfer/delivery	NTT DOCOMO, INC.	discussion	Rel-19

Proposal 1: For Case y, the scenario is out of scope, and RAN2 doesn’t have to discuss this case.

Proposal 2: For Case z1, there are following requirements.

    - The model is in proprietary format and MNO cannot comprehend it (e.g., model structure, model contents).

    - MNO can be aware of the model.

    - MNO can control the procedure of model transfer (i.e., start/stop to transfer the model).

Proposal 3: For the case that NW transfers model (or model parameters) to UE-side model training entity directly, there are following requirements.

    - NW and UE-side model training entity should have common understanding about the model (or model parameters).

Proposal 4: For the case that NW transfers model (or model parameters) to UE-side model training entity via UE with OTA, there are following requirements.

    - NW, UE and UE-side model training entity should have common understanding about the model (or model parameters).

    - The model (or model parameters) should be transferred in open format.

Proposal 5: For the case that NW transfers the dataset to UE-side model training entity directly or via UE with OTA, there are following requirements.

    - User consent is needed to transfer the dataset to outside MNO from privacy and legal perspective.

R2-2501811	Discussion on signaling feasibility of OTA solutions	vivo	discussion	NR_AIML_air-Core

R2-2501858	Signalling feasibility of dataset and parameter sharing	NEC	discussion	Rel-19	NR_AIML_air-Core

R2-2501925	Signalling feasibility of AIML model transfer	CATT	discussion	Rel-19	NR_AIML_air-Core

R2-2502111	Feasibility Analysis on the Alternatives for Two-sided Model	MediaTek Inc.	discussion

R2-2502128	Further discussion on parameters/model transfer in two-sided model	Apple	discussion	Rel-19	NR_AIML_air-Core

R2-2502293	Discussion on Dataset and Parameter Sharing from the Network to the UE for Two-Sided Model Training	Qualcomm Incorporated 	discussion	Rel-19

R2-2502412	On Model parameter and Data Set Sharing For Two-side Model	ZTE Corporation	discussion	Rel-19	NR_AIML_air-Core

R2-2502797	Discussion on signalling feasibility of dataset and parameter sharing for CSI compression	Huawei, HiSilicon	discussion	Rel-19	NR_AIML_air-Core

R2-2502953	On signalling feasibility of dataset and parameter sharing	Ericsson	discussion

R2-2502959	Outstanding issues on signalling feasibility of dataset and parameter sharing for CSI compression	Samsung R&D Institute UK	discussion

## 8.2	Ambient IoT

(Ambient_IoT_solutions, leading WG: RAN1; REL-19; WID: RP-243326)

Time budget: 2.5 TU

Tdoc Limitation: 3 tdocs

### 8.2.1	Organizational

LS, Rapporteur input, including workplan, etc.

R2-2502258	Skeleton of A-IoT MAC specification (TS 38.391)	Huawei, HiSilicon	draft TS	Rel-19	38.391	0.0.1	Ambient_IoT_Solutions

Delete (MAC) abbreviation from title

The skeleton is endorsed

R2-2502259	Initial Text Proposal for A-IoT MAC specification	Huawei, HiSilicon	pCR	Rel-19	38.391	0.0.1	Ambient_IoT_Solutions

The CR should only contain agreements and indicate what agreement the text is trying to capture with comments

The CR will be revised with further agreements from RAN#129bis and reviewed over email

[POST129bis][017][AIoT] 38.391 Running CR (Huawei)

Intended outcome:

1 Update and review running CR

2 Create list of remaining open issues

Deadline:  long

R2-2502704	Introduction of Ambient IoT	CMCC	draftCR	Rel-19	38.300	18.5.0	B	Ambient_IoT_Solutions

The CR will be used as baseline for further revisions taking into account RAN2#129bis and reviewed by email

[POST129bis][018][AIoT] 38.300 Running CR (CMCC)

Intended outcome:

1 Update and review running CR

Deadline:  long

R2-2502262	Illustrative figures for a deeper understanding of A-IoT MAC operations	Huawei, HiSilicon, China Southern Power Grid	discussion	Rel-19	Ambient_IoT_Solutions

Not treated

### 8.2.2	A-IoT Paging

Contributions should focus on paging message content and format, including subsequent paging for the same service, paging identifier details,temporary device ID (if any RAN2 impact), etc.

Including outcome of [POST129][035][AIoT] Paging (Qualcomm)

Related to email discussion

Behavior for Parallel service requests

R2-2502211	Email discussion report: [POST129][035][AIoT] Paging	Qualcomm Incorporated	discussion	Rel-19	Ambient_IoT_Solutions-Core

Proposal 1:	Discuss and agree on the device behavior if it gets a new service request while one procedure is still ongoing (i.e. not completed or failed yet): (a) ignore all new requests and R2D messages addressed to itself but not associated with the ongoing procedure, or (b) terminate the ongoing procedure and respond to the latest request. (a/b/ffs = 17/8/4)

-	Huawei thinks that for Rel-19 indoor we have only intra scenario and for inter we can rely on some coordination.  We should keep it simple and go with option b and not discuss how to identify neighbor message and what is ongoing.   Xiaomi agrees with Huawei we rely on reader implementation for multi reader and the issue is a rare issue.

-	Xiaomi and Qualcomm think that we need to define what is ongoing and the new request is related to ongoing procedure if we go for a.  Xiaomi thinks that it is difficult for device to determine is the trigger is an error or a real message.

-	Samsung thinks that option b is not forward compatible and it would be a bad decision.   There is no coordination between readers so it is better to define UE behavior.  Oppo agrees.   Huawei thinks that Rel-20 will have a different paging type so there is no forward compatibility issues.   Sasmung asks how the reader knows whether the device supports Rel-19 or 20.    Huawei thinks that the reader first sends Rel-19 paging and then R20 paging.

-	Mediatek had a revelation since email discussion and option a has a risk if the NACK is lost, so the device still has to process paging.  We would be adding a lot of complexity and we shouldn’t fork the decisions with new paging type.  We should have a reader ID to handle forward compatibility.   Samsung thinks that we have a timer to determine the procedure is ongoing.   Mediatek doesn’t assume that the device can run a timer.

-	CATT thinks that with option a we would need a release message.  Option b should have the assumption that there is no unexpected message from another reader.

-	CMCC thinks that we should go with option b and we should use different paging type for r20.

-	ZTE thinks that the multi-reader case will not happen in this release and the strongest argument is forward compatibility and we should discuss whether transaction ID is always there or not.

-	Nokia thinks we should properly define things and not leave thing up to implementation, and doing option a with a good definition of stop and end.   Ofinno agrees with Nokia.

-	Apple thinks that option b is preferrable and defining what is ongoing increase complexity on UE and the reader should handle all these scenario.

-	Interdigital thikns that we can have forward compatibility even with option b, the network is in control and can chose to complete the inventory and the other reader starts it when the other one completes.

-	Ericsson points out that RAN1 will have some requirements on how long the UE waits for the reader response.  Even without the timer the query type will be used to advance the slots and you can use those to understand whether the procedure is ongoing or not.   For forward compatibility we have to think of all messages and not only paging.

-	Qualcomm thinks that forward compatible devices should co-exist with rel-19.    Option b is the best option as it will have less things to define.

-	ZTE thinks that the proponents don’t want transaction ID for option b.

Noted

[AT129bis][010][AIoT] Paging (Qualcomm)

Intended outcome: discussion option a (how to get out of deadlock, how to consider end of procedure, transaction ID overhead) and option b (how to ensure forward compatibility, implications on transaction ID etc). discuss transaction ID assumptions

Deadline:  Thursday

R2-2503155	Offline discussion report: [AT129bis][010][AIoT] Paging	Qualcomm Incorporated	discussion	Rel-19	Ambient_IoT_Solutions

Observation 1: If device needs to be able to determine whether a R2D message is associated to an ongoing procedure or not, some kind of indication (e.g. transaction ID, reader ID, some other ID) would be needed in the R2D messages.

Observation 2: At least for the purpose of current discussions, RAN2 should consider opt1 from section 4.1 as the intended ‘forward compatibility’.

Opt 1: Rel20 Reader can transit a common paging message. Rel19 AIOT device can partially understand the (Rel19) part of the paging message and act accordingly while gracefully ignoring the parts it does not understand. Rel20 message formats are generally ‘extension’ of Rel-19 message formats with the same Message Type. (This does not exclude the possibility of new Messages only used by Rel20 devices to accommodate new Rel20 use cases.)

Observation 3: No consensus on which option to go (a or b). Multiple companies suggested nothing needs to be specified as multi-reader scenario is not supported in Rel-19.

Proposal: For forward compatibility, RAN2 aims to design Rel-19 AIoT messages extensible to accommodate devices and features of future release.

-	ZTE thinks that this doesn’t solve our problem we need to have a device behaviour.   Qualcomm thinks that according to the discussion we may not specify anything.  Xiaomi thinks that it can be up to device implementation.  Mediatek thinks that this is not just for multi-reader case, but we should spend some time to understand what this means for the system.

-	Apple think that the discussion showed that there are solutions for forward compatibility for option b.  Samsung doesn’t want to leave this to device implementation.

-	CMCC agrees that we need to ensure a known common behavior.

Noted

Agreements

FFS which solution if any for device behavior if it gets a new service request while one procedure is still ongoing or leave it to implementation.

RAN2 aims to design Rel-19 AIoT R2D messages extensible to accommodate devices and features of future release.

Reader ID

Proposal 6:	Discuss and decide whether Reader ID information should be provided to the device (yes/no/FFS = 11/17/1).

R2-2501962	Discussion on A-IOT paging procedure	Xiaomi	discussion	Rel-19	Ambient_IoT_Solutions

Proposal 1: RAN2 confirm that Implementation based coordination among readers is sufficient. The device does not need to distinguish whether a Msg is from same or different reader. The reader ID is not included in R2D Msg.

Transaction ID for multiple-reader scenario

Proposal 4:	Discuss and decide whether the device needs to distinguish and behave differently between when the same service request is received again from the same reader vs a different reader (yes/no/ffs = 14/12/2).

Proposal 5:	Discuss and decide whether RAN2 needs to consider the case where the device has already successfully responded to the same service request but now received the same request from a different reader e.g. for location use case (yes/no/unclear = 9/16/3).

R2-2502206	Open issues for A-IoT paging	ZTE Corporation, Sanechips	discussion

Proposal 3: RAN2 should discuss which alternative in below can be a baseline assumption for addressing these cases:

	Alt1: to make Paging messages from different readers but triggered by the same CN AIoT service contain the same transaction ID

	Alt2: to make Paging messages from different readers but triggered by the same CN AIoT service contain different transaction ID

R2-2502554	Paging Aspects for Ambient IOT	InterDigital	discussion	Rel-19	Ambient_IoT_Solutions

Proposal 9:	RAN2 discuss the following options for the design of the transaction ID: 1) Portion of transaction ID identifies the reader while the other portion is derived from the correlation ID; 2) Entire transaction ID is generated from the correlation ID

Proposal 10:	If RAN2 cannot decide on the above, RAN2 sends LS to SA2 to ask whether different correlation IDs can be used for the same service request.

Calculation of transaction ID

Proposal 7:	Wait for SA2/RAN3 progress before further discussing on how the reader calculates transaction ID.

Proposal 8:	Discuss whether to send an LS: To SA2 asking whether correlation ID is expected to be same or different when the same service is requested from different readers, and to RAN3 asking whether coordination between readers is expected.

Transaction ID Size

R2-2502186	Discussion on Ambient IoT Paging	Apple	discussion	Rel-19	Ambient_IoT_Solutions

Proposal 4	4-6 bit “Transaction ID” is used for Paging message.

Need for Transaction ID in CFRA

R2-2501828	Discussion on A-IoT paging	HONOR	discussion	Rel-19	Ambient_IoT_Solutions

Proposal 8: The transaction ID in the paging message can be optional for the CFRA.

R2D Trigger Message (Way 1 vs Way 2)

R2-2502705	Discussion on A-IoT paging	CMCC	discussion	Rel-19	Ambient_IoT_Solutions

Proposal 15: A new R2D message other than the paging message (Way1-1) is introduced for A-IoT device determining MSG1 resources unless RAN1 concludes to use L1 signaling.

Noted

R2-2502607	Ambient IoT Paging	Qualcomm Incorporated	discussion	Rel-19	Ambient_IoT_Solutions-Core

Proposal 3:	After the initial paging, for the R2D transmission which determines the Msg1 resource(s), adopt Way-2 from the RAN2#129 way forward (i.e. reuse the same paging message, using field(s) to indicate it is only to determine the Msg1 resource(s) and omitting the paging identifier (device ID/group ID) field).

Noted

Discussion

-	Qualcomm asks why we are complicating and adding a different paging type.   ZTE thinks that making fields optional has implications on extra bits to indicate presence, need to design something lightweight.

Implicit or Explicit Signaling for CFRA/CBRA

R2-2502765	Discussion on DL messages for Ambient IoT UEs	Ericsson	discussion	Rel-19	FS_Ambient_IoT_solutions

=> Revised in R2-2502989

R2-2502989	Discussion on DL messages for Ambient IoT UEs	Ericsson	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 9	There is no need to indicate explicitly whether a device should respond to the paging message with contention free or contention-based RA.

Noted

How to specify implicit

R2-2502873	Paging aspects of AIoT	Nokia	discussion	Rel-19

Proposal 10: To simplify signaling, RAN2 to agrees that

-	an AIoT paging of a single-device implies “Contention-free access” i.e. the gNB assigns the resources for the single device,

-	an AIoT paging of a device group or all devices implies “Contention-based random access”, since this may be an unknown set of devices, and

-	an AIoT paging of multiple device IDs (if supported) implies “Contention-free access”

Noted

R2-2502268	A-IoT paging	Huawei, HiSilicon	discussion	Rel-19

Proposal 5:	Whether the random access is contention-based or contention-free can be implicitly indicated by the number of access occasion(s), e.g., one access occasion for one/each device ID implies contention-free access, i.e., there is no need for a dedicated explicit indication of the access type.

Noted

Discussion

-	Interdigital thinks that we should have a forward compatible soclution when in the future we have multiple CFRA devices and Huawei’s proposal is good.

-	Mediatek, LG, CATT, thinks Nokia’s proposal is better for device complexity.  Qualcomm asks if the device can figure it is only for that UE.   Understand is that the device only figures out that it is for that device but doesn’t mean that it is only for that device.

-	Vivo thinks that Nokia’ solution would require the NAS to indicate whether this ID is just for this UE.

-	Xiaomi thinks that the UE will know that a single resource means CFRA.  Mediatek thinks there are scenarios where the reader gives one resources for single group ID.

-	ZTE thinks that given same resources to multiple devieces won’t work, so  Huawei’s proposal makes sense.  Apple agrees.

-	Xiaomi thinks we can make an assumption that for CFRA the reader always assigns with resource and for CBRA more than one.  Qualcomm thinks that we are making multiple assumptions for saving 1 bit.

Agreements on CFRA

Introduce an explicit 1 bit indication to indicate whether it is CFRA or CBRA per paging message.

Paging Identifier Length

R2-2502215	Further discussions on A-IoT paging	Futurewei	discussion	Rel-19	Ambient_IoT_Solutions

Proposal 4. An ID length field is always included together with the paging ID field in the A-IoT paging message, except the case where no ID is included in the A-IoT paging message.

Noted

R2-2502268	A-IoT paging	Huawei, HiSilicon	discussion	Rel-19

Proposal 4a:	In an A-IoT paging message, there is no need of a length field for the paging identifier field. This is based on the assumption that there is no MAC padding in the paging message, and the paging identifier field will be placed at the end of the paging message.

Proposal 4b:	If Proposal 4a is not agreed, RAN2 asks SA2/CT1/CT4 for the size of paging identifier (e.g., permanent ID, temporary ID, mask/filter/group ID).

Noted

Discussion

-	Xiaomi thinks that SA2 is discussing so we should wait and we need to length ID.   LG, CATT agrees we need length ID

-	ZTE thinks that it is important that the field length is very small as this has radio implication and hopefully SA2 can only have a limited set of paging ID sizes.

Agreements on paging ID length

- A field indicating Paging ID length information is always included together with the paging ID field in the A-IoT paging message, except the case where no ID is included in the A-IoT paging message.

- The number of bits required for paging ID length field should be as small as possible.  This would require the number of different Paging ID lengths to be small.

- Send an LS to SA2 to take this into account for their design.

[POST129bis][027][AIoT] LS on paging ID length to SA2 (cc CT4) (CATT)

Intended outcome: agree to LS to SA2 and (cc CT4) capturing agreement on paging ID length and ask SA2 to take this agreement into account.

Deadline:  short

R2-2503197	LS on paging ID length	RAN2	LS out	Rel-19	Ambient_IoT_Solutions	To:SA2, CT4;	Cc:RAN3

=> Approved

Not treated

Forward Compatibility for Multiple Paging IDs in Paging message

R2-2502186	Discussion on Ambient IoT Paging	Apple	discussion	Rel-19	Ambient_IoT_Solutions

Proposal 5 	RAN2 to clarify the meaning of “forward-compatibility”: whether there will be a single common Paging message for both Rel-19 and Rel-20 or two different paging message formats in the respective release.

R2-2501788	Discussion on ambient IoT paging	LG Electronics Inc.	discussion	Rel-19

Proposal 2: To ensure forward compatibility with scenarios where multiple paging identifiers can be contained in a single paging message, A-IoT paging message should include a reserved field to accommodate multiple paging identifiers.

R2-2502819	Discussion on Ambient IoT paging message design	ASUSTeK	discussion	Rel-19	Ambient_IoT_Solutions

Proposal 2:  To support multiple IDs in a paging message for forward compatibility, a paging message includes a field to indicate the number of IDs.

Association of Device ID and reader ID

R2-2501812	Discussion on AIoT Paging	vivo	discussion	FS_Ambient_IoT_solutions

Proposal 1.	The reader needs to know the association between the assigned AS ID and the upcoming command request towards a specific device, where the reader being informed of the paging identifier does not solve the issue.

Proposal 2.	RAN2 to send LS to inform SA2 and RAN3 about the requirement at the reader to associate between the assigned AS ID and the upcoming command request toward a specific device. Wait for their solution(s) on such issue.

R2-2501788	Discussion on ambient IoT paging	LG Electronics Inc.	discussion	Rel-19

R2-2501846	Discussion on Paging for Ambient IoT	CATT	discussion	Rel-19	Ambient_IoT_Solutions

R2-2502022	Discussion on A-IoT paging message format	Tejas Network Limited	discussion	Rel-19

R2-2502028	Discussions on AIoT paging	Fujitsu	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2502040	Discussion on paging procedure for Ambient IoT	OPPO	discussion	Rel-19	Ambient_IoT_Solutions

R2-2502220	Discussion on A-IoT Paging	ETRI	discussion

R2-2502286	Ambient-IoT Paging	NEC	discussion	Rel-19	Ambient_IoT_Solutions

R2-2502345	Discussion on paging procedure for Ambient IoT	Lenovo	discussion	Rel-19

R2-2502375	Discussion on A-IoT paging	Panasonic	discussion	Rel-19

R2-2502422	Discussion on Paging for A-IoT	Transsion Holdings	discussion	Rel-19

R2-2502429	Discussion on paging procedure of A-IoT	Spreadtrum, UNISOC	discussion	Rel-19

R2-2502484	Considerations on paging for Ambient IoT	Sony	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2502499	Multiple Paging involving reader(s) and service(s)	Sony	discussion	Rel-19	Ambient_IoT_Solutions

R2-2502745	Further consideration of A-IoT paging for Ambient IoT 	Kyocera 	discussion	Rel-19

R2-2502753	Discussion on A-IoT paging	ITL	discussion	Rel-19

R2-2502759	Discussion on Ambient IoT Paging	China Telecom	discussion

R2-2502775	Discussions on AIoT paging	Samsung	discussion	Rel-19	Ambient_IoT_Solutions-Core

R2-2502874	Discussion on Ambient IoT Paging	CEWiT	discussion

R2-2502938	AIoT Paging: Handling a New Service Request	Philips International B.V.	discussion	Ambient_IoT_Solutions

R2-2502954	Discussion on paging procedure for Ambient-IoT 	III	discussion

R2-2502958	Discussion on AIoT paging	NTT DOCOMO, INC.	discussion	Rel-19

### 8.2.3	A-IoT Random Access

Contributions should focus on details of contention-based and contention-free access, including re-access for failure handling, msg content/format, format (Msg1, Msg2, NACK based feedback for re-access, etc.)

MSG1 contents for CBRA

R2-2502346	Discussion on random access for Ambient IoT	Lenovo	discussion	Rel-19

Proposal 2: In case of CBRA, only 16 bits random ID is included in Msg1.

-	Mediatek thinks that this will depend on whether we need to have message type for future cases.   Xiaomi doesn’t think we need to have an additional ID as the reader would know as the resources are reserved by reader.

Noted

Random ID for MSG1 in CFRA

R2-2502207	Open issues for A-IoT random access	ZTE Corporation, Sanechips	discussion

Proposal 9: RN16 is included in MSG1 for CFRA

Noted

R2-2502043	Discussion on random access for A-IoT	OPPO	discussion	Rel-19	Ambient_IoT_Solutions

Proposal 10: RN16 is not needed included in the A-IOT Msg1 in the CFRA procedure

Noted

Discussion

-	Vodafone thinks that using the same procedure for both makes sense.  Mediatek also thinks we should have a unified procedure for simplicity purposes.    Huawei thinks that anyways they are different procedure.   Xiaomi thinks that if we want to do unified procedure then we shouldn’t have CFRA at all.

-	CATT, Vivo, Samsung, Futurewei, Qualcomm, Lenovo, Ericsson, LG,  don’t think this is needed because the resources are reserved.

-	Nokia is concerned that this may be premature given that we don’t know everything about AS ID.

-	ZTE asks how we address the UE if there is a subsequent command message assigning an AS ID.   Qualcomm thinks we can use the same ID used to address the UE use for sending CFRA.  Mediatek thinks that it is long ID so not very efficient.   Xiaomi, Oppo, thinks that we don’t need to assign an ID as we are only addressing one UE.

Options to handle:

-	including AS ID in msg0

-	no AS ID is used for CFRA

-	including RN16

-	assume that a single device can perform a procedure with a reader

Agreements on msg1

      - In case of CBRA, only 16 bits random ID is included in Msg1.  FFS can be revisited if message type will be needed for other D2R messages purposes

      - RN16 is not included in the first D2R message in the CFRA procedure.  AS ID is the only ID needed for addressing the device in R2D command message assuming for CFRA no multiple devices are performing the procedures with the given reader.   FFS if we can assume or need to support multiple device scenario.

MSG1 Resource Definition

R2-2502207	Open issues for A-IoT random access	ZTE Corporation, Sanechips	discussion

Proposal 2: In order to establish and maintain a common timing between the reader and the devices that are paged, the reader sends R2D trigger message(s) after the paging message

Noted

Resource selection for MSG1 in CBRA

R2-2502555	Random Access Procedure for Ambient IOT	InterDigital	discussion	Rel-19	Ambient_IoT_Solutions

Proposal 1:	RAN2 decide between the following for MSG1 resource selection by the device: 1) randomly select among the access occasions indicated in the paging message, then randomly select the resources within that access occasion that are allocated by the “queryRep-like” message 2) randomly select among the total number of MSG1 resources to be provided by each “queryRep-like” message.

The device randomly select among the total number of MSG1 resources provided by paging message.

randomly select among the total number of MSG1 resources to be provided by paging message.

-	Huawei, Xiaomi thinks we can exclude slot number/count down number.

Noted

Agreements on new R2D message

- A new R2D message other than the paging message is introduced for A-IoT device determining MSG1 resources unless RAN1 concludes to use L1 signaling.   The R2D message indicates the start of a set of MSG1 resources that were configured in paging message.

- Assumption: The R2D message does not include slot number/count down number.

Content and format of MSG2 for CBRA

R2-2502585	Views on Random Access Aspects of Ambient IoT	Qualcomm Incorporated	discussion	Ambient_IoT_Solutions-Core

Noted

R2-2502964	Discussion on A-IoT random access	Samsung Electronics Czech	discussion	Rel-19	Ambient_IoT_Solutions-Core

Proposal 3: Same Msg2 format is used for initial transmission and retransmission of Msg2.

Noted

R2-2502346	Discussion on random access for Ambient IoT	Lenovo	discussion	Rel-19

Proposal 4: AIoT Msg2 includes successfully received RN16 and may include Msg1 resource identifier to avoid RN16 collision.

-	Vivo thinks that in case of one resources there is no need for a ID.

-	LG agrees but we need to discuss what is identifier.   Samsung asks whether it is a slot index.    Lenovo explains that it corresponds to the order of the msg1 resources allocated in paging message.

-	Huawei thinks that the collision probability is very low and the reader can avoid problems so it is not necessary in msg2.   CATT agrees with Huawei and thinks that we should avoid including more things signalled.

-	Xiaomi supports the proposal as otherwise we will incur delay.   Ericsson agrees and this index should correspond to frequency resource.   Fujitsu agrees but should be both frequency and time domain.   Interdigital supports as well and for forward compatibility we don’t know how much resources/frequencies we have in future.   Nokia, Oppo, Offino supports.

-	Vivo thinks that this should be discussed by RAN1 first.

-	Qualcomm thinks we can do it across frequency but definitely not time.

-	ZTE and Qualcomm doesn’t think this is needed, when we designed the length we decided 16 to reduce collision probability.  Then if we have a problem we should increase the length.   Apple agrees.

Noted

Agreements on msg 2

- A-IoT Msg2 contains one or multiple echoed random ID(s) from A-IoT Msg1 of different A-IoT devices.

- Same Msg2 format is used for initial transmission and retransmission of Msg2.

Determination of MSG3 failure

R2-2501963	Discussion on access procedure for A-IOT	Xiaomi	discussion	Rel-19	Ambient_IoT_Solutions

Proposal 8: RAN2 confirms that Subsequent A-IoT paging is used to trigger the re-access for Msg3 failure. Msg2 transmission is used to trigger the retransmission of Msg 3. NACK is used to determine whether re-access will be triggered. Timer, window are not needed for the device to determine the failure.

Not treated

Determination of MSG3 success (Timer, paging reception, R2D reception, command reception)

R2-2502749	Discussion on random access aspects for Ambient IoT	LG Electronics Inc.	discussion	FS_Ambient_IoT_solutions

Proposal 4. Introduce a new timer for NACK feedback reception to determine whether the Msg3 is successfully transmitted or not.

Noted

R2-2502470	A-IoT random access procedure	Huawei, HiSilicon	discussion	Rel-19	Ambient_IoT_Solutions

Proposal 9:	There is no need for a timer for reception of “NACK feedback indication”. The device shall consider itself successful by default, if no “NACK feedback indication” is received before subsequent paging.

Noted

R2-2502466	Discussion on UL multiple access	Ericsson	discussion	Rel-19	Ambient_IoT_Solutions

Proposal 3	The device interprets its D2R transmission (e.g., Msg3) is successful if the device has not received NACK signalling until the D2R timer is expired or the beginning of next CBRA occasion whichever occurs first.

Noted

R2-2501980	A-IoT: ACK/NACK for Msg3 and re-access	MediaTek Inc.	discussion	Rel-19	Ambient_IoT_Solutions-Core

Proposal 6: If the device has transmitted Msg3 and receives any R2D data transmission without a NACK indication, it interprets it as a positive ACK for Msg3.

Noted

NACK feedback applicability to MSG3/Command

R2-2502470	A-IoT random access procedure	Huawei, HiSilicon	discussion	Rel-19	Ambient_IoT_Solutions

Proposal 7:	The reader can send “NACK feedback indication” either after Msg3 or Msg5 to device(s) that failed, whenever it intends to terminate the data transmission for device(s) in this “paging round”.

Noted

R2-2502749	Discussion on random access aspects for Ambient IoT	LG Electronics Inc.	discussion	FS_Ambient_IoT_solutions

Proposal 6. NACK based mechanism is applied only to the Msg3.

Noted

Discussion

-	Lenovo supports msg3 and 5.   Ericsson, Xiaomi, CATT, Apple, Samsung thinks that msg3 is sufficient and there is an issue where there reader sends the response twice.   Huawei thinks this can be handled and asked how would we capture this in the specification.   Interdigital thinks that we are adding complexity to the device for a uncertain use case.

-	ZTE, CMCC  agrees with Huawei.  During RACH there may be a collision and if you don’t have a mechasnism to retrigger RACH and this mechanism it seems to nice way to do this.

-	Samsung asks why can’t we use the paging message to re-trigger access.   The reader can just use another transaction ID.  Huawei thinks that both of this reader implementation are allowed and NACK comes for free.   Xiaomi doesn’t think it is free as it impacts other groups.

NACK Multiplexing

R2-2502717	Further consideration on A-IoT random access	CMCC	discussion	Rel-19	Ambient_IoT_Solutions

Proposal 5a: A-IoT Msg4 (NACK message) for multiple A-IoT devices can be multiplexed into one A-IoT MAC PDU, with one message header to indicate message type.

Not treated

NACK indication for CFRA

R2-2501813	Random Access Procedure for AIoT Device	vivo	discussion	FS_Ambient_IoT_solutions

Proposal 8.	Failure indication for CFRA Msg1 is not supported since the device may receive a new CFRA Msg0 triggering when the previous Msg1 is not received successfully by reader.

Noted

Agreements on NACK

- For CBRA, as a baseline, NACK based mechanism is applied only to the Msg3.   May come back for D2R data, if the NACK feedback indication is needed for the purpose to stop/terminate the “on-going procedure” and release the AS ID accordingly (depending on other later discussion).

- For msg3, we rely on whether the device receives NACK indication before subsequent R2D message to determine re-access.    No need for a timer.   FFS whether subsequent R2D message is trigger message or paging

- For CFRA, NACK feedback and re-access is not supported.  FFS how to achieve

- FFS on end of procedure

Not treated

R2-2501829	Discussion on A-IoT random access	HONOR	discussion	Rel-19	Ambient_IoT_Solutions

R2-2501847	Discussion on random access for A-IoT	CATT	discussion	Rel-19	Ambient_IoT_Solutions

R2-2501859	Random Access for Ambient IoT device	NEC	discussion	Rel-19	Ambient_IoT_Solutions

R2-2501987	Ambient-IoT Random Access	Ofinno, LLC	discussion

R2-2502023	Discussion on A-IoT message format for CBRA and CFRA	Tejas Network Limited	discussion	Rel-19

R2-2502029	Discussions on AIoT Random Access	Fujitsu	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2502151	Aspects on RA for AIoT	Nokia	discussion	Ambient_IoT_Solutions

R2-2502187	Discussion on Random Access for Ambient IoT	Apple	discussion	Rel-19	Ambient_IoT_Solutions

R2-2502216	Further discussions on A-IoT random access	Futurewei	discussion	Rel-19	Ambient_IoT_Solutions

R2-2502221	Considerations for re-access in Ambient IoT	ETRI	discussion

R2-2502243	Random access procedure for ambient IoT	China Telecom	discussion	Rel-19	Ambient_IoT_Solutions

R2-2502430	Discussion on A-IoT random access	Spreadtrum, UNISOC	discussion	Rel-19

R2-2502578	Discussion on re-access mechanism for D2R failures	Panasonic	discussion	Rel-19

R2-2502623	Discussion on re-access for A-IoT	Continental Automotive	discussion

R2-2502625	Random access types supported by AIoT devices	NTT DOCOMO, INC.	discussion	Rel-19

R2-2502691	Discussion on random access procedures	Fraunhofer HHI, Fraunhofer IIS	discussion

R2-2502746	Further consideration of A-IoT random access for Ambient IoT 	Kyocera 	discussion	Rel-19

R2-2502820	Discussion on Ambient IoT Msg2 design	ASUSTeK	discussion	Rel-19	Ambient_IoT_Solutions

R2-2502875	Discussion on random access for Ambient IoT	CEWiT	discussion

R2-2502905	Discussion on random access aspects of Ambient IoT	KT Corp.	discussion	Rel-19	Ambient_IoT_Solutions-Core

### 8.2.4	A-IoT Data Transmission and Other general aspects

Contributions should focus on MAC PDU/signaling general format (the content of paging, Msg1, Msg2 etc. should be discussed in above specific agendas), data (re)transmission for failure handling, segmentation for D2R, AS ID, message size information pending SA2 input, etc.Including outcome of [POST129][036][AIoT] AS ID (Xiaomi)

AS ID

R2-2501965	[POST129][036][AIoT] AS ID (Xiaomi)	Xiaomi	discussion	Rel-19	Ambient_IoT_Solutions

General AS ID (Easy Proposals)

R2D Message that assigns AS ID for CFRA

Ph2-Proposal 6a (0): For CBRA, exclude Option 3: “New Msg” for AS ID assignment

Ph2-Proposal 6b (3): For CBRA, exclude option 4: “Msg 4 (First Command message) for AS ID assignment”

Ph2-Proposal 6c (11): For CBRA, Option1: Msg 2 is used for AS ID assignment;

-	Futurewei, ZTE, Qualcomm and Apple support using command message for both CFRA and CBRA.   LG, Vivo thinks that we need to send it with msg2 for contention resolution.

-	Samsung is ok with option 1 but it means that when msg2 is retransmitted it has to include the AS ID again.

-	ZTE points out that msg2 is a multiplexed message so we shouldn’t add unnecessary.  Also AS ID is only needed for command so it is sufficient to put it in msg4.

-	Futurewei thinks that not all devices will need to be given AS ID so we would need to have flexible indication whether AS ID is assigned or not.

-	Interdigital is not concerned with msg size as it will only be provided when needed.   Also if there was two devices selecting same random id, then the devices will not know for which random ID this AS ID is for, so it should be in msg2.   Fujitsu agrees as there will be collisions.  Huawei, CATT agrees it is only workable option.   Nokia asks how do we handle the case of collisions.

-	Ericsson indicates another reason is that it should apply to inventory as well as we have NACK as well and we need to differentiate the AS ID for the NACK.

Noted

Validity of AS ID

- Validity options

Option 1: The device releases the AS ID upon receiving Paging with with new transaction id, i.e. same/different session/service

Option 1a: The device releases the AS ID when it triggers new msg1 transmission as a result of receiving Paging message (i.e. it has to generate a random ID)

Option 2: The device releases the AS ID upon timer expiry; The Timer could be configured by the reader, or pre-defined in the specification;

Option 3: The device releases the AS ID upon receiving new assigned AS ID from the Reader

Option 4: The device releases the AS ID after completion of the command procedure

Option 5: The device releases the AS ID upon power off

Option 6: The device releases the AS ID upon receiving explicit release indication from the Reader

Ph2-Proposal 9a (3): For validity of AS ID, exclude option 2 The device releases the AS ID upon timer expiry

Ph2-Proposal 9b (3): For validity of AS ID, exclude option 4 The device releases the AS ID after completion of the command procedure

Ph2-Proposal 9c-alt1: For validity of AS ID, RAN2 further down-selection among option 6 (11) and Combined option 1+3, taking into account of the pros/cons in the discussion, FFS on whether the AS ID is only valid in the same paging round, paging session, whether “the explicit indication is missing” is a rare case or not.

Noted

Agreements

1	AS ID is applied for Inventory + command case;

2	AS ID is not included in D2R message except Msg 1 (RN16 in Msg 1 has been agreed.

3	For both CFRA and CBRA, the AS ID size is same as RN 16, i.e. 16 bits.

4	Do not specify the reader behaviour on how exactly the ASID is generated.

5	The device releases the AS ID upon power off (no stage 3 specification impact);

6	The device only keeps one AS ID at a time.

7	For CFRA, command message is used for AS ID assignment

8	For CBRA, Msg 2 is used for AS ID assignment

9	The device releases the AS ID at least:

- upon receiving Paging with new transaction id for that device, i.e. different session/service

- when it triggers new msg1 transmission as a result of receiving Paging message (i.e. it has to generate a random ID for CBRA)

- FFS other cases for release ASID to avoid keeping it indefinitely.

Down-selection of AS ID Validity

Not treated

R2-2502244	A-IoT data transmission and other general aspects	China Telecom	discussion	Rel-19	Ambient_IoT_Solutions

Proposal 5.	Regarding the validity of AS ID, AS ID could become invalid upon receiving a paging message.

R2-2502556	Remaining Aspects on AS ID and Segmentation	InterDigital	discussion	Rel-19	Ambient_IoT_Solutions

Proposal 5:	The device maintains the AS ID upon reception of paging with the same transaction ID.

Proposal 6:	The device releases the AS ID upon reception of paging with a different transaction ID.

Proposal 8:	The device releases its AS ID and replaces it with a new AS ID in the message used by the reader to assign the new AS ID.  RAN2 assumes no restriction on when such message can be sent by the reader.

R2-2502776	Discussion on A-IoT data transmission	Samsung	discussion	Rel-19	Ambient_IoT_Solutions-Core

Proposal 2-1: RAN2 is kindly asked to agree that, as a baseline, the explicit release indication from the reader can be used to release the AS ID.

Segmentation

Whether retransmission of segments applies also to unsegmented data

R2-2501848	Discussion on data transmission for A-IoT	CATT	discussion	Rel-19	Ambient_IoT_Solutions

Proposal 2: RAN2 to confirm that packets without segmentation can also be retransmitted using the same mechanism as segmented retransmissions, based on the 1-bit ‘notTheLast’ indication from device to reader and the offset indication from reader to device.

Noted

R2-2502269	A-IoT data transmission	Huawei, HiSilicon	discussion	Rel-19

Proposal 8b:	Don’t support the entire D2R message re-transmission triggered by “received size/offset=0”, since the failure may be due to R2D command missing.

Noted

Retransmission of the first segment

R2-2502776	Discussion on A-IoT data transmission	Samsung	discussion	Rel-19	Ambient_IoT_Solutions-Core

Proposal 4-3: for the retransmission of the first segment/unsegmented D2R message, the reader can send the R2D message by including the upper layer command again instead of indicating the number of successfully received bits to be zero.

Noted

Inclusion of the command in the R2D message for segmentation

R2-2502030	Discussions on Data Transmission and Other General Aspects	Fujitsu	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 2: To achieve a simple and unified design of segmentation and unsegmented transmission, option 2 is preferred, i.e., reader includes the upper layer command each time together with the offset information in the R2D message triggering the A-IoT device to (re)send the segment.

Noted

R2-2502696	Discussion on data transmission for A-IoT	CMCC	discussion	Rel-19	Ambient_IoT_Solutions

Proposal 6-2: In case first segment and unsegmented packet retransmission, the command message should be re-contained in the R2D message.

Noted

Discussions on segmentation

-	Mediatek thinks that device complexity is bad so we should have one behavior and it makes sense to include the command again with the offset always included.   Xiaomi also agrees as it is easier from device perspective to not remember anything.   Vivo doesn’t think we need to unify.

-	Qualcomm thinks that the reader doesn’t know that there will be segmentation.

Discussion on offset for first transmission and whether command is always included

-	ZTE and Apple think that whenever there is a new upper layer PDU and give it to MAC the offset becomes zero so we don’t need to care whether we signal it again as it is implicitly set

-	Xiaomi The offset is not needed for the first transmission but for unified command it would be good.

-	Honor thinks that always including the command would require processing.

-	Mediatek thinks that if we don’t include the offset then we are designing a flexible header so we should always include offset and command.   Interdigital agrees and points out that we wanted to make it as simple for the device and we shouldn’t have to store the offset information so always having it makes it simpler.  Ericsson and CATT agrees.

-	ZTE is concerned that the command for retransmission will trigger a new upper layer PDU.  Huawei indicates that the device if it sees that offset it will not send the command to upper layer.  Xiaomi and Mediatek thinks that this is device implementations, it can either forward the command and offset.    ZTE thinks that we can’t assume that the upper layer PDU is the same.

-	Samsung doesn’t see why we have to include command again.

-	Oppo thinks from device complexity the device should pass it to upper layer and device would produce another PDU.

Whether to support segmentation of MSG3

R2-2501964	Protocol design  principle and considerations on Data transmission	Xiaomi	discussion	Rel-19	Ambient_IoT_Solutions

Proposal 1: RAN2 confirms there is no additional effort to support Msg3 segmentation/segmentation retransmission. Whether to segment a Msg3 is up to the reader implementation.

Noted

R2-2502201	Discussion on A-IoT data transmission and other general aspects	Lenovo	discussion	Rel-19	Ambient_IoT_Solutions

Proposal 5: RAN2 assumes that segmentation is not applied to Msg3 CBRA.

Noted

R2-2502042	Discussion on AIoT data transmission related functionalities	OPPO	discussion	Rel-19	Ambient_IoT_Solutions

Proposal 4: segmentation on Msg3 is allowed only for CFRA.

Noted

Discussion

-	Xiaomi thinks we shouldn’t do segmentation after all as there is complexity.  Vivo thinks that we would need to design two formats to support this case.

MSG Size for Segmentation

R2-2501814	AIoT Data Transmission	vivo	discussion	FS_Ambient_IoT_solutions

Proposal 1.	Support remaining D2R message size report from device to reader.

Noted

R2-2502586	Data Transmission and Other General Aspects of Ambient IoT	Qualcomm Incorporated	discussion	Ambient_IoT_Solutions-Core

Proposal 3:	RAN2 confirms 1-bit indication about whether more data will be sent is sufficient for D2R data segmentation.

Noted

Discussion on msg size for segmentation of read command

-	Oppo and Lenovo thinks that providing more information to the reader is beneficial so it can optimize the grant. Mediatek, Interdigital, CATT, Honor, Nokia agrees and it could be a few bits.

-	Ericsson the padding information would help the reader to know whether there is more data.

-	Apple, Samsung, ZTE thinks that this is an optimization as we may get some indication from the CN.   Samsung thinks that this is not a usual case and to reduce device complexity we should just use 1 bit.

-	Qualcomm indicates that the study item conclusion was 1 bit and also if we go for 2-3 bits then we have to define BSR table

After coffee break suggestion is size information only once 7 bits

-	Qualcomm thinks that device complexity is bad and there would be a need for the device to keep track when to send it and not. Samsung also thinks that this is crazy to define two separate messages.    CMCC also thinks that this will increase complexity for device to calculate buffer and buffer size table.  Mediatek doesn’t like having dynamic formats but can live with different fixed format.

Agreements on segmentation

- For the retransmission of the first segment/unsegmented D2R message, the reader sends the R2D message by including the upper layer command again.  FFS whether offset zero is always included.

- FFS whether the reader always includes the command for retransmission of segments.

- 1-bit indication is sufficient to indicate whether more D2R data will be sent

- For inventory response, RAN2 assumes that segmentation is not applied.  RAN2 assumes that the reader can avoid segmentation by reader being aware of inventory response size.  Notify SA2 about this assumption.

[POST129bis][026][AIoT]  LS on D2R message size to SA2 (InterDigital)

Intended outcome: agree to LS to SA2 (cc RAN3) capturing agreement on D2R message size for inventory response.

Deadline:  short

R2-2503199	LS on D2R message size for inventory response	RAN2	LS out	Rel-19	Ambient_IoT_Solutions	To:SA2	Cc:RAN3

=> Approved

MAC PDU Format

Byte Alignment

R2-2502244	A-IoT data transmission and other general aspects	China Telecom	discussion	Rel-19	Ambient_IoT_Solutions

Proposal 1.	No need to support byte-alignment for A-IoT MAC PDU.

Noted

R2-2501981	Ambient IoT MAC PDU formats	MediaTek Inc.	discussion	Rel-19	Ambient_IoT_Solutions-Core

Proposal 11: The D2R and R2D data PDU formats have their control parts filled to an integer number of octets, so that the data portion starts on an octet boundary.  The fill bits can be treated as reserved.

Noted

R2-2502269	A-IoT data transmission	Huawei, HiSilicon	discussion	Rel-19

Proposal 11:	For the very small R2D message sending frequently, byte-alignment (i.e. deliberate padding) is not required, and can be discussed case by case (e.g., QueryRep-like message).

Proposal 12:	The D2R message should be byte-aligned, assuming the allocated TBS value is in the unit of byte.

Proposal 13:	RAN2 assumes that the upper layer data SDU is byte-aligned, and an LS can be sent to CT1.

Noted

Discussions

-	Ericsson agrees that byte alignment should simplify the design.  Vivo thinks that at least for D2R we should have by alignment

-	Mediatek and Samsung thinks that all the combined headers (control part of MAC PDU) not being byte aligned it will increase device complexity.   ZTE thinks that we need to consider overhead if we want to make it byte aligned.   Samsung and Vivo doesn’t think device complexity shouldn’t be sacrificed.  Oppo indicates that if we design fixed control fields then it doesn’t matter much to have byte alignment.

Agreements on MAC PDU format

- The MAC PDU should be byte-aligned, assuming the allocated TBS value is in the unit of byte.  The actual TBS value depends on RAN1.   FFS for R2D trigger message

- RAN2 assumes that the upper layer data SDU is byte-aligned, and an LS can be sent to CT1.

- The D2R MAC PDU size will correspond to the TBS size indicated in the R2D message

- The MAC padding is supported at least for D2R from RAN2 perspective.   The device includes padding bits if there is no more data and there is still space available in the TBS.

- In case where MAC PDU includes both MAC SDU and padding, for D2R a field to indicate how many SDU bits are present is required.  FFS how this is provided (i.e. SDU length field or padding length field).  The size of length field is FFS.

[POST129bis][025][AIoT] LS on byte-aligned SDU to CT1 (Vivo)

Intended outcome: agree to LS to CT1 capturing agreement on byte-aligned SDU.

Deadline:  short

R2-2503186	[DRAFT] LS on byte-alignment of upper layer data SDU	vivo	LS out	Rel-19	Ambient_IoT_Solutions	To:CT1	Cc:RAN3, SA2

=> Revised in R2-2503198

R2-2503198	LS on byte-alignment of upper layer data SDU	RAN2	LS out	Rel-19	Ambient_IoT_Solutions	To:CT1	Cc:RAN3, SA2

=> Approved

D2R Padding and Length Field

R2-2502586	Data Transmission and Other General Aspects of Ambient IoT	Qualcomm Incorporated	discussion	Ambient_IoT_Solutions-Core

Proposal 7:	The MAC padding is supported at least from D2R from RAN2 perspective.

Proposal 9:	A length field is supported for D2R data MAC SDU to support varying lengths of D2R data. The size of length field is FFS.

Noted

Message Type for D2R

R2-2502303	Data Transmission Aspects for A-IoT	Ericsson	discussion	Rel-19	Ambient_IoT_Solutions-Core

Proposal 2	No need of message type field in D2R message.

Noted

R2-2502208	Open issues for data transmission and MAC PDU design	ZTE Corporation, Sanechips	discussion

Proposal 5: The general message structure for MAC includes the following fields

•	Message type field (FFS if this is needed for all messages – e.g. for RN16)

•	Header fields (these fields contain the information that it terminated in MAC layer) – i.e. not passed to upper layers

•	Payload (the information in the payload is passed to upper layers)

•	Padding

Noted

Discussions

-	Apple thinks that we should have message type for future proofness and there may be error cases.   Vivo, Xiaomi explain that the device will never send something without the reader knowing.   Apple, we can’t guarantee that the message from the reader is not received from other devices.    ZTE has some sympathy but we should consider overhear over the air.

-	Huawei thinks that we may have some reserved bits at the end after we have seen the whole MAC PDU design.   Fujitsu agrees with apple.

-	Qualcomm indicates that if we DOA we will have device originated, so we should be future proof.  Interdigital thinks that even for DOA we know it will be data and Xiaomi explains that even for first message the resources used by DOA may be different than the other set of resources.

FFS whether we introduce D2R message type.  Discuss after looking at the overall MAC header design and space before deciding whether we introduce message type or reserved bits

Not treated

R2-2501830	Discussion on Data Transmission for Ambient IoT	HONOR	discussion	Rel-19	Ambient_IoT_Solutions

R2-2501890	Ambient-IoT Data transmission	NEC	discussion	Rel-19

R2-2502152	AIoT data transmission aspects	Nokia Denmark	discussion	Ambient_IoT_Solutions

R2-2502175	A-IoT MAC design for data transmission	Apple	discussion	Ambient_IoT_Solutions

R2-2502217	Further discussions on A-IoT data transmission and other aspects	Futurewei	discussion	Rel-19	Ambient_IoT_Solutions

R2-2502343	Discussion on AS ID	Panasonic	discussion	Rel-19

R2-2502431	Discussion on A-IoT data transmission	Spreadtrum, UNISOC	discussion	Rel-19

R2-2502485	Considerations on segmentation	Sony	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2502671	Discussion on A-IoT data transmission	LG Electronics Inc.	discussion	Rel-19	Ambient_IoT_Solutions

R2-2502686	Discussion on AIoT data transfer	NTT DOCOMO, INC.	discussion	Rel-19

R2-2502747	Further consideration of A-IoT data transmission for Ambient IoT 	Kyocera 	discussion	Rel-19

R2-2502821	Discussion on the assistance information from device	ASUSTeK	discussion	Rel-19	Ambient_IoT_Solutions

R2-2502960	Discussion on A-IoT data segmentation and transmission	III	discussion

## 8.3	AI/ML for Mobility

(FS_NR_AIML_Mob; leading WG: RAN2; REL-19; SID: RP-242393)

Time budget: 2 TUs

Tdoc Limitation: 3 tdocs

### 8.3.1	Organizational

LS, Rapporteur input, including workplan, etc.

Including outcome of [POST129][021][AI Mob] TR update (Oppo)

R2-2501821	Summary of [POST129][021][AI Mob] TR update (OPPO)	OPPO.	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 1: In “sample and hold” scheme, the last actual measurement result in OW is held as predicted measurement result in whole PW for sub case 1 and case 2.

- Capture a generic description of sample and hold in the TR, but no need to capture anything detailed in TR

Proposal 2: RAN2 discuss and conclude which option is for sub case 3:

Option 2: the sample is the latest L1 measurement result in the OW. The L3 measurement result is derived based on this sample in PW.

Option 2: the sample is the L3 measurement results corresponding to the time instance of the latest L1 measurement result in OW.

Proposal 3: Align the HO model option number with TR 38.744 in the contributions.

Noted

R2-2501822	Text proposal of TR 38.744	OPPO	pCR	Rel-19	38.744	0.0.7	FS_NR_AIML_Mob

The TR is endorsed and will be further revised post meeting

[POST129bis][019][AI Mob] TR update (Oppo)

Intended outcome:  review TR containing agreements from RAN2#129bis

Deadline:  long

R2-2502107	Discussion on the simulation results illustration for AI mobility	MediaTek Inc.	discussion	Rel-19	FS_NR_AIML_Mob

[POST129bis][020][AI Mob] Sim. Results figures  (Mediatek)

Intended outcome: email discussion to gather specific comments related to result illustration

Deadline:  long

Noted

### 8.3.2	RRM measurement prediction

#### 8.3.2.1	Simulation results

No further simulation evaluations are expected for this AI

#### 8.3.2.2	Model Generalization

Contributions on generalization should focus on the configuration #1 and configuration #2 that were agreed in RAN2-129 for FR1. Generalization for FR2 can also be submitted.

R2-2502001	Discussion on Generalization Issues for AI/ML Mobility	Samsung	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 1: For Case B temporal domain prediction in FR1, RAN2 to capture the observation that it is generalizable over cell configurations with different deployment scenarios.

-	Xiaomi thinks it is a bit premature to conclude this as with other assumptions they get different observation depending on MRRT.   Apple, Ericsson, Interdigital have the same observation.

Proposal 2: For Case B temporal domain prediction in FR1, RAN2 to capture the observation that GC#2 slightly improves the accuracy of the AI/ML model compared to GC#1, while offering comparable accuracy as baseline.

Noted

R2-2502463	RRM measurement prediction model generalization evaluation for different cell configurations	Huawei, HiSilicon	discussion	Rel-19	FS_NR_AIML_Mob

Observation 3: For FR2, AIML model generalizes very well for different cell configurations and across different channel models. The prediction accuracy difference is non-existent or negligible, i.e. 0.05 dB for GC#1 and 0.01 dB for GC#2.

Observation 4: For FR2, AIML model generalizes equally well for both “directions”, i.e. model trained with Configuration#1 and used for inference for Configurtion#2 and the other way around.

Noted

R2-2501790	Simulation results of model generalization on cell configuration	vivo	discussion	Rel-19	FS_NR_AIML_Mob

Observation 2: For intra-frequency temporal domain prediction in FR1/FR2, the model trained in UMi scenario shows better prediction accuracy when tested in UMa scenario.

Observation 3: For intra-frequency temporal domain prediction in FR1/FR2, the model trained in UMa scenario shows worse prediction accuracy when tested in UMi scenario.

Noted

Discussions

-	Ericsson points out that frequency domain doesn’t perform as well but other companies haven’t provided results.

Agreements

- For Case A and B temporal domain prediction in FR1 and FR2, RAN2 to capture the observation that it is generalizable for the studied cases over cell configurations with different deployment scenarios

- For Case A and B temporal domain prediction in FR1 and FR2, RAN2 to capture the observation that GC#2 slightly improves the accuracy of the AI/ML model compared to GC#1, while offering comparable accuracy as baseline.

- For intra-frequency temporal domain prediction in FR1/FR2, the model trained in UMi scenario shows better prediction accuracy when tested in UMa scenario than the model trained in UMa scenario when tested in UMi scenario

- No more generalization work is expected

All contributions below are noted

R2-2501824	Discussion on model generalization of RRM measurement prediction	OPPO	discussion	Rel-19	FS_NR_AIML_Mob

R2-2501926	Simulation results of Model Generalization	CATT, Turkcell	discussion	Rel-19	FS_NR_AIML_Mob

R2-2501935	Discussion on generalization performance over cell configuration	Xiaomi	discussion

R2-2502112	Generalization performance evaluation	MediaTek Inc.	discussion

R2-2502176	On generalization across cell configurations	Apple	discussion	FS_NR_AIML_Mob

R2-2502253	Generalization of AIML models for RRM measurement prediction	Interdigital Inc.	discussion	Rel-19	FS_NR_AIML_Mob

R2-2502281	Simulation Results for Model Generalization	Qualcomm Incorporated	discussion	Rel-19	FS_NR_AIML_Mob

R2-2502627	Generalization of the AI/ML models for RRM prediction	Ericsson	discussion	Rel-19	FS_NR_AIML_Mob

R2-2502879	Evaluation results for RRM measurement prediction generalization	Nokia	discussion	Rel-19	FS_NR_AIML_Mob

R2-2502969	Discussion on model generalization  of RRM measurement prediction	ZTE Corporation	discussion	Rel-19	FS_NR_AIML_Mob

### 8.3.3	Measurement event predictions

Contributions on evaluations/simulation results.   System level performance results can also be submitted in this AI for companies providing such results.

Objective in Case A (FR2): Predict A3 events to enhance HO execution via earlier HO Prep upon A3 event prediction (as shown in Option 2 or Option 3), i.e., HO performance enhancement.

Objective in Case B (FR1): Predict A3 events in the presence of skipped measurements, i.e., overhead reduction while maintaining similar HO performance.

R2-2501823	Simulation results of measurement event prediction	OPPO	discussion	Rel-19

Observation 4: AI/ML with handover decision option 2 (handover model option 1 in TR) shows a comparable total number of HOF per UE per second when compared with the legacy HO mechanism.

-	Samsung and Nokia doesn’t support to capture this observation as companie show different results

Noted

R2-2501927	Simulation results of measurement event prediction and handover performance	CATT, Turkcell	discussion	Rel-19	FS_NR_AIML_Mob

Observation 6: Option3 is superior to option2 both in terms of handover failure rate and handover failure number.

Noted

Discussions

-	ZTE thinks that different assumption will result in different outcomes so we shouldn’t compare solutions.   Samsung agrees so it really depends on the situation.   Vivo thinks that with AI prediction we can improve HO KPI.

-	Ericsson explains that a lot of companies have shown results that option 3 is much better.   Mediatek thinks that we can capture both for now but for wI phase we will need to downselct.

R2-2502464	Further simulation results for measurement event prediction in FR2	 Huawei, HiSilicon	discussion	Rel-19	FS_NR_AIML_Mob

Observation 1: For FR2 to FR2 intra-frequency temporal domain case A (Case 4), F1 score is higher for shorter TTT values of the predicted event.

Observation 4: The performance gain is observed in terms of both HOF rate and total number of HOF per UE per second, e.g. in the RAN2 agreed scenario HOF rate drops from 70% to 25% while the total number of HOF per UE per second drops from 0.419 to 0.105.

Observation 5: AIML models can help to improve the mobility performance significantly, even when the achieved F1 score is low.

Noted

R2-2502106	Discussion and preliminary results for direct and indirect event prediction	MediaTek Inc.	discussion	Rel-19	FS_NR_AIML_Mob

Observation 9: Both direct event prediction and indirect prediction methodologies demonstrate a reduction in HOF number per UE per second when compared to the legacy approach at the system level performance.

Noted

Agreements on measurement event prediction

1.  Option 2 and Option3 outperform legacy solutions in terms of HO failure number per UE per second

2.	Capture that most companies show in their simulation results that option 3 outperforms option 2 when prediction accuracy is good enough.  There are some companies that show the opposite and indicate why.

3	For FR2 to FR2 intra-frequency temporal domain case A (Case 4), F1 score is higher for shorter TTT values of the predicted event.

4	From the small number of companies that provided results for direct prediction: Both direct event prediction and indirect prediction methodologies demonstrate a reduction in HOF number per UE per second when compared to the legacy approach at the system level performance.

5	No further simulation results are expected for future meetings

Noted

R2-2501791	Simulation results for measurement event prediction and SLS	vivo	discussion	Rel-19	FS_NR_AIML_Mob

R2-2501863	Simulation assumptions, metrics and specification impact for measurement event prediction	NEC	discussion	Rel-19	FS_NR_AIML_Mob

R2-2501905	Discussion on measurement event prediction using large language model	BJTU	discussion

R2-2501936	Discussion on event prediction simulation results	Xiaomi	discussion

R2-2502138	Simulation results for measurement event prediction and system level performance	Samsung	discussion	Rel-19	FS_NR_AIML_Mob

R2-2502230	Evaluations on measurement event prediction	NTT DOCOMO, INC.	discussion

R2-2502254	Measurement event predictions	Interdigital Inc.	discussion	Rel-19	FS_NR_AIML_Mob

=> Withdrawn

R2-2502282	Simulation Results for Measurement Event Prediction	Qualcomm Incorporated	discussion	Rel-19	FS_NR_AIML_Mob

R2-2502628	SLS Results for Event Predictions	Ericsson	discussion	Rel-19	FS_NR_AIML_Mob

R2-2502880	On the measurement event prediction	Nokia	discussion	Rel-19	FS_NR_AIML_Mob

R2-2502968	Discussion on measurement event prediction	ZTE Corporation	discussion	Rel-19	FS_NR_AIML_Mob

### 8.3.4	RLF/HO failure prediction

RLF failure prediction will not be further studied in Rel-19

### 8.3.5	LCM and spec impact for AI/ML mobility

Contributions should focus on the LCM framework/signaling for RRM measurement prediction and measurement event prediction, and the gap from the currently agreed LCM framework for the beam management use case under the AI/ML for NR air interface work item.

Contributions can address required signaling and protocol to enable reporting of inference outcome for RRM measurement prediction and measurement event prediction for a UE sided model, or the required signaling and protocol aspects to enable reporting of UE assistance for inference operation of a network sided model for RRM measurement prediction.

Scenarios/Sub-UseCases/Domains:

R2-2501789	Way forward on study of beam-level prediction	vivo, Ericsson, ZTE, Huawei, HiSilicon, CATT, NTT DOCOMO, CMCC, Samsung, Interdigital	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 1: Consider L3 beam-level prediction in the specification impact analysis in the SI phase.

Proposal 2: L3 beam-level prediction includes temporal, frequency and spatial domain predictions. Temporal and frequency domain prediction to be considered with higher priority for detailed specification impact analysis, if needed.

-	Interdigital thinks that it depends on the sub-use case we support and if we support sub-use case 1 this comes for free.

-	Ericsson and Huawei thinks we need this for the solution to work as the network needs to allocate RA resources.  ZTE also thinks that this is very important and should be prioritized over other things like direct prediction.

L3 Beam-level prediction specification impact will be studied

Noted

R2-2501825	Discussion on spec impact of AI mobility	OPPO	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 14: By taking all the observations and proposals into account, here is summary of study scope:

Discussion on measurement even prediction

-	Huawei thinks that even if we don’t study NW sided model measurement even prediction, the network can use measurements to do event prediction.    Samsung agrees that it is worth capturing this but we don’t need to study it.

-	Oppo thinks we shouldn’t capture this in the TR as it is NW implementation

-	Apple thinks that we never studied network sided model and never accounted for delays etc.

Discussion temporal domain case A

-	ZTE, Huawei, Ericsson, Nokia, and CMCC doesn’t agree to limit only to sub-case2.   Oppo is concerned with sub-case1 we will increase signalling overhead.   Interdigital agrees that for this to work the UE has to constantly send beam level measurements.  Apple agrees and we haven’t studied the implications of doing all the subcases.    CMCC doesn’t think that overhead is a big problem.   Samsung has the same concern as Oppo.

-	Mediatek thinks that this is also a matter of configuration as up to now we have assumed RRM measurement framework.

Discussion temporal domain case B

-	Xiaomi, Samsung, Ericsson think that we shouldn’t support NW side model as the purpose was for measurement reduction in UE.  Huawei, Docomo, Nokia and Mediatek doesn’t agree as this can also be for RS reduction.

Spatial domain

-	Mediatek, Ericsson, Vivo, Qualcomm, Nokia think we should support it as it is a low hanging fruit and companies showed gains results.   It can enable turn off some SSBs for network energy saving.   Samsung indicates that companies have different interpretation and we would need to align the understanding on this.

-	Xiaomi asks if it is intra-cell or inter-cell.   Mediatek eplains that it is for intra-cell

-	ZTE Thinks that for NW side we can consider both inter and intra but for UE only intra-cell

Things to continue study for specification and RAN2 impact

|  | UE sided model | Network sided model |
| --- | --- | --- |
| Measurement event prediction | Yes, | No (up to NW implementation) |
| Temporal domain case A | Yes, all sub cases | Yes |
| Temporal domain case B | Yes , all sub cases | Yes |
| Frequency domain | Yes , all sub cases | Yes |
| Spatial domain | Pending | Pending |
| L3 beam level measurement prediction | Yes, | Yes |

General LCM:

R2-2501792	Discussion on LCM and spec impact	vivo	discussion	Rel-19	FS_NR_AIML_Mob

Noted

R2-2502177	On LCM for AI/ML mobility	Apple	discussion	FS_NR_AIML_Mob

Proposal 2: only functionality-based LCM is considered for AI/ML mobility.

Proposal 3: only the following LCM functions are considered: functionality activation/deactivation and fallback, applicability reporting, data collection for network-sided models, and UE-sided model training (for now, may be revised later depending on the progress in the AI/ML air interface).

Proposal 4: monitoring discussion is put on hold pending RAN4 progress.

-	Oppo thinks that the KPIs studied in R2 and R4 are different and we should have a discussion in RAN2.   Apple asks how do we know that performance monitoring is needed, it depends on requirements set by RAN4.  Interdigital agrees with Oppo as it is not realistic that all models will be tested for meeting requirement.

Noted

Agreements 1. The general LCM framework for beam management can be the baseline (where applicable) for AI mobility, such as the following aspects: - Data collection for model training - UE capability - Applicability reporting - Inference configuration and reporting - Performance monitoring and management 2 Only functionality-based LCM is considered for AI/ML mobility (i.e. we don’t support model based LCM)

UE capability/functionality granularity:

R2-2502711	Discussion on LCM for RRM measurement prediction with UE sided model	CMCC	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 4: AI/ML-enabled features in AI/ML for Mobility could be defined based on the use cases, including:

- AI/ML-enabled RRM measurement prediction

- AI/ML-enabled Measurement events prediction

Proposal 5: RAN2 considers the granularity of functionality, for example,

- Option 1: Based on the prediction type, the AI/ML functionalities for RRM measurement prediction could be:

- Temporal domain prediction

- Frequency domain prediction

- Spatial domain prediction

- Option 2: Based on the finer granularity (e.g., combine prediction types and sub-use cases), in Temporal domain prediction, Frequency domain prediction and Spatial domain prediction the AI/ML functionalities could be further divided into:

- To predict beam level results, then generate cell level results based on the predicted beam results

- To directly predict cell level results based on cell level results

- To directly predict cell level results based on beam level results

Noted

R2-2501825	Discussion on  spec impact of AI mobility	OPPO	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 25: To support following functionalities for LCM discussion:

- Functionality 1: Intra-frequency RRM measurement reduction

- Functionality 2: RRM measurement prediction in frequency domain

- Functionality 3: Intra-frequency RRM measurement prediction in future window

- Functionality 4: Measurement event prediction*

- Relationship between this and RRM measurement prediction is FFS.

Noted

Discussion

-	Samsung thinks that this should be based on granularity of configuration or applicability reporting.  Xiaomi thinks it is a bit earlier and option 2 from CMCC makes more sense.

-	Mediatek thinks that this is more related to inference and oppo’s proposal should be used as a baseline.

-	Nokia thinks that we can substitute functionality with configuration and nw configuration granularity is sufficient.

-	Oppo asks how do we discuss inference if we don’t know functionality.   Huawei agrees with the starting point from Oppo

-	ZTE thinks that we can just use the table to discuss for each case the details.

-	Qualcomm would like to capture the reason/benefits of what we are introducing.  Interdigital points out that we have already justified the use cases.   Apple thinks companies should link their proposals to the study goals.

Start by discussing spec/ran2 impact of the agreed cases/subcases and then discuss functionality granularity

Inference/Reporting configuration

R2-2502822	Discussion on LCM for AIML Mobility	ASUSTeK	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 1: The inference configuration for AI/ML mobility can be based on RRM measurement configuration (e.g., MeasObject(s), ReportConfig(s), and/or MeasId(s)).

Proposal 4: The current measurement reporting could be enhanced to consider predicted measurements.

Noted

R2-2502255	LCM and spec impact for AI/ML mobility	Interdigital Inc.	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 11: The UE automatically activates the applicable RRM measurement prediction and measurement event configurations (i.e., starts the measurement/event predictions according to the configurations, and sends measurement reports according to the reporting configurations).

-	Xiaomi thinks that some cases the prediction is only useful in some cases and if it always activated then this has impact on UE power.  Interdigital thinks that this can be linked to Ssearch when the UE performs inference and only do it then.  Samsung agrees.   Vivo thinks that the intention is that we don’t need additional explicit signaling to activate/deactive.

-	Qualcomm thinks that this is early too make this agreements.

The UE applies inference configuration for the applicable RRM measurement prediction and measurement event prediction upon receiving configuration via RRC.  As baseline, no dynamic signaling (e.g. L1/2) is required.    FFS when the UE can perform inference

Proposal 12: The UE can be configured to perform one or more of the following reporting based on predicted RRM measurements/events:

-	Periodic measurement reports that include current measurements and predicted measurements.

-	Events triggered measurement reports based on current measurements, where the measurement report includes current and/or predicted measurements and/or indications for predicted events.

-	Event triggered measurement reports based on current/predicted measurements/events, where the measurement report includes current and predicted measurements.

-	Vivo, Ericsson think that the content of the report can be FFS as the predicted measurement can also be reported separated.  CATT agrees but it can also be up to the NW to configure.

-	ZTE asks if the network can configure whether the reporting should be based only on predicted or configured.   We shouldn’t have just predicted

-	CATT asks if we are limiting this only to A3 like simulations.  Oppo thinks it can be applicable to all events A1-6. Mediatek thinks that we need to develop something useful in the future so we should focus on useful events.

-	Mediatek and Apple asks about SINR/RSRQ.  Oppo, Huawei, Interdigital think that for specification there is no problem.   ZTE explains that there may be impact.   Samsung has concerns with SINR as we didn’t model interference from other cell and we can’t guarantee good prediction accuracy.   MEdiatek that RAN4 would be impacted if introduce more measurements.

- Agreements

- For RRM prediction, UE sided model, the UE can be configured with periodic or event triggered reporting of predicted and/or actual RRM measurements.   FFS details

- For event prediction, UE-sided model, the UE can be configured with event-triggered reporting based on prediction and/or actual measurements.  FFS details

- Baseline is that this applies to all A1-6 events, unless technical problems are identified.  Baseline quantity is RSRP.

R2-2501792	Discussion on LCM and spec impact	vivo	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 15.	For RRM measurement prediction of UE-sided model, the network may configure the following parameters in inference configuration:

- Temporal domain case B prediction: information related to MRRT;

- Temporal domain case A prediction: information related to PW length;

- Spatial domain prediction: information related to MRRS.

R2-2501928	Specification impact for AIML mobility	CATT, Turkcell	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 18: For indirect measurement event prediction, the model inference configuration e.g., input and output configuration of model inference is the same as that for RRM measurement prediction.

Proposal 20: For direct measurement event prediction, the input/output configuration of model can be configured from network to UE:

- Input configuration: measurement resources of serving/neighbouring cells, event configuration parameters (e.g., TTT), other configuration information is FFS;

- Output configuration: the probability of event occurrence within a time window.

Proposal 21: For direct measurement event prediction, enhanced RRM measurement reporting mechanism is needed for the inference result reporting, e.g. reporting the probability of event occurrence within a time window.

Applicability determination/reporting:

R2-2502139	Discussion on LCM and spec impact for AI/ML mobility	Samsung	discussion	Rel-19	FS_NR_AIML_Mob

Proposal. 3: For AI/ML for mobility, UE can report the applicable functionalities to NW via UAI or RRCReconfigurationComplete message.

Noted

R2-2502650	Discussion on LCM for AI for Mobility	Ericsson	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 16	Associated ID from AIML for PHY Beam Management is taken as baseline for Mobility use case.

Proposal 17	Similar to Beam Management use case, what the associated ID represents for mobility use case is left to network implementation.

Proposal 18	RAN2 discuss how to extend the associated ID to consider the state of the neighbouring cells in model training.

-	Huawei thinks that we need to discuss what associated ID would mean in the context of mobility and when it is really needed.   CATT thinks that associated ID is only used for BM cas an for mobility it doesn’t sound very useful.

-	Samsung thinks that for beam related sub-cases we would still need associated ID.

-	Lenovo and Apple think that it is not possible that generalization works in all cases so we will need some ID.  CMCC thinks that the generalization is quite good.

-	MEdiatek take thinks that we needed to understand per use cases discussion and then we can come back to discuss associated ID

-	Qualcomm thinks that this can be useful for different scenarios like network energy savings.

Noted

R2-2502970	Discussion on spec impact for AI mob	ZTE Corporation	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 4: For the granularity for the AI mobility specific associated id, RAN2 to discuss the following options:

- Option 1: per area level associated id;

- Option 2: per scenario level associated id.

Noted

Data collection:

R2-2502881	Considerations on LCM for AIML Mobility	Nokia	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 6: Consider immediate MDT and the measurement reporting framework as the starting point for data collection for NW-side deployments. Study additional data elements and methods needed for data collection at the NW-side.

Proposal 7: Postpone discussions on UE-side data collection until there is more progress in the Rel. 18-19 AIML Phy use-cases.

Noted

R2-2502711	Discussion on LCM for RRM measurement prediction with UE sided model	CMCC	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 12: For the data content for UE sided model training, at least L3 cell level measurement results and/or L1 beam level measurement results and/or L3 beam level measurement results should be collected from UE for RRM measurement prediction, e.g., beam/cell level RSRPs of one or more than one intra-frequency cell(s) or inter-frequency cell(s).

Noted

Performance monitoring (if time allows):

R2-2502139	Discussion on LCM and spec impact for AI/ML mobility	Samsung	discussion	Rel-19	FS_NR_AIML_Mob

Proposal. 7: For AI/ML for mobility, for NW-side model, the model/functionality management decision can be up to NW implementation and transparent to UE.

Proposal. 9: For AI/ML for mobility, for UE-side model, the functionality management decision can be made by NW as a baseline. FFS. Whether to support UE-side decision.

Proposal. 10: For performance monitoring of NW-side model, UE can provide the label data (i.e., actual measurement results) for gNB. The existing measurement/report configuration can be reused to configure UE to report the actual measurement.

Proposal. 11: For AI/ML for mobility, for UE-side model, UE can perform the performance monitoring (e.g., performance metric calculation) and report the monitoring results to NW as baseline.

Noted

R2-2502462	Discussion on specification impacts of AIML aided mobility	Huawei, HiSilicon	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 8: The two options for performance monitoring agreed in AIML air interface can be reused for AIML aided mobility, i.e. NW-side and UE-assisted performance monitoring:

- For NW-side monitoring, the reported measurement result can be L3 cell/beam level measurement result.

- For UE-assisted monitoring, RAN2 needs to discuss the performance metric(s) calculated at UE side, e.g., RSRP difference for RRM measurement prediction, F1 score for indirect measurement event prediction.

Noted

Discussion

-	Xioami thinks that we should be careful as calculation of f1 score may be complicated

-	Apple thinks that not every feature requires monitoring like measurement event case as the monitoring can just be based on whether you get the actual event or not.    Nokia agrees that the NW would know whether the even actually happened or not.

Agreements

- The inference configuration and reporting for AI/ML mobility can be based on RRM measurement framework

- For AI/ML for mobility, UE can report the applicable functionalities to NW via UAI or RRCReconfigurationComplete message.

- FFS whether association ID is required and should provide motivation on per use case bases

- As a baseline, use the AI ML PHY NW-side data collection procedures, configuration and reporting framework.

- Study UE side data collection configuration, taking AI ML PHY relevant procedure as baseline.   Postpone UE side data collection transfer discussion until further progress is made in AI ML PHY in R20.

- Model transfer will not be discussed in this study item

7	For UE sided model, the performance monitoring considered in AIML air interface can be studied as a baseline for AIML aided mobility, i.e. NW-side and UE-sided performance monitoring.

FFS metrics per use case

Not treated

R2-2501862	RRM measurement prediction specification impact discussion	NEC	discussion	Rel-19	FS_NR_AIML_Mob

R2-2501937	Discussion on LCM and spec impact for AI/ML mobility	Xiaomi	discussion

R2-2501990	LCM and Spec Impact for AI/ML Mobility	Lenovo	discussion	Rel-19

R2-2502003	Discussion on AIML mobility LCM procedure 	NEC	discussion	Rel-19	FS_NR_AIML_Mob

R2-2502113	Discussion on LCM and Specificaton Impact for AI Mobility	MediaTek Inc.	discussion

R2-2502179	On standards impacts other than LCM	Apple	discussion	FS_NR_AIML_Mob

R2-2502231	Discussions on LCM and specification impact for AI/ML mobility	NTT DOCOMO, INC.	discussion

R2-2502283	LCM and specifications impact for AI/ML for mobility	Qualcomm Incorporated	discussion	Rel-19	FS_NR_AIML_Mob

R2-2502437	Discussion on LCM and spec impact for AIML mobility	Spreadtrum, UNISOC	discussion	Rel-19

R2-2502460	Discussion on Specification-related Aspects for AI/ML-based Mobility	Sharp	discussion

R2-2502500	Data collection for event prediction	Sony	discussion	Rel-19	FS_NR_AIML_Mob

R2-2502712	Discussion on LCM for RRM measurement prediction with NW sided model	CMCC	discussion	Rel-19	FS_NR_AIML_Mob

R2-2502733	Discussion on LCM for measurement event prediction	CMCC	discussion	Rel-19	FS_NR_AIML_Mob

R2-2502925	Discussion on LCM and spec impact	ETRI	discussion

R2-2502949	Discussion on specification impact for event prediction	KDDI Corporation	discussion	Rel-19

## 8.4	Low-power wake-up signal and receiver for NR (LP-WUS/WUR)

(NR_LPWUS-Core; leading WG: RAN1; REL-19; WID RP-241824)

Time budget: 1 TU

Tdoc Limitation: 3 tdocs

### 8.4.1	Organizational

LS, Rapporteur input, including workplan, Running CRs, etc.

LS

R2-2501743	LS on LP-WUS UE RF (R4-2503003; contact: vivo)	RAN4	LS in	Rel-19	NR_LPWUS-Core	To:RAN1	Cc:RAN2

Noted

Running CRs

R2-2501954	Running 37.340 CR for LP-WUS	ZTE Corporation, Sanechips	draftCR	Rel-19	37.340	18.5.0	NR_LPWUS-Core

Noted, will update/endorse in post meeting email discussion

[Post129bis][206][LPWUS] Running CR for 37.340 (ZTE)

Intended outcome: Updated running CR based on new agreements for endorsement, open issue list (if needed)

Deadline:  Long

R2-2502913	Introduction of Low-Power Wake-Up Signal and Receiver for NR	Ericsson	draftCR	Rel-19	38.300	18.5.0	NR_LPWUS-Core

Noted, will update/endorse in post meeting email discussion

[Post129bis][207][LPWUS] Running CR for 38.300 (Ericsson)

Intended outcome: Updated running CR based on new agreements for endorsement

Deadline:  Long

R2-2502141	38.304 Running CR for LP-WUS (CATT)	CATT	discussion	Rel-19	NR_LPWUS-Core

Noted, will update/endorse in post meeting email discussion

[Post129bis][208][LPWUS] Running CR for 38.304 (CATT)

Intended outcome: Updated running CR based on new agreements for endorsement, open issue list (if needed)

Deadline:  Long

R2-2502153	RRC Running CR for LP-WUS WUR	vivo (Rapporteur)	draftCR	Rel-19	38.331	18.5.1	B	NR_LPWUS-Core

Noted, will update/endorse in post meeting email discussion

[Post129bis][209][LPWUS] Running CR for 38.331 (vivo)

Intended outcome: Updated running CR based on new agreements for endorsement, open issue list (if needed)

Deadline:  Long

R2-2502307	Running MAC CR for LP-WUS	Apple	draftCR	Rel-19	38.321	18.5.0	B	NR_LPWUS-Core

Noted, will update/endorse in post meeting email discussion

[Post129bis][210][LPWUS] Running CR for 38.321 (Apple)

Intended outcome: Updated running CR based on new agreements for endorsement, open issue list (if needed)

Deadline:  Long

Others

R2-2502098	Discussion on UE capability for LP-WUS	Huawei, HiSilicon	discussion	Rel-19

Noted

Subgrouping capability

Proposal 1: Support of LP-WUS in IDLE/INACTIVE (either OOK-based or OFDM-based) indicates support of UE ID-based subgrouping by default.

Proposal 2: The capability of LP-WUS in IDLE/INACTIVE (coupled with UE ID-based subgrouping capability) is included in UE-RadioPagingInfo-r17 in RRC UECapabilityInformation message. The granularity (such as per band) and other details for capability of LP-WUS in IDLE/INACTIVE depends on RAN1 features list.

RRM measurement capability

Proposal 3: RAN2 capability defines RRM measurement fully offloading and RRM measurement relaxation for LP-WUS.

Proposal 4: Define RRM measurement capability as “Conditionally mandatory features without UE radio access capability parameters”. It is mandatory to support RRM measurement fully offloading and RRM measurement relaxation in RRC_IDLE/RRC_INACTIVE for UEs which support LP-WUS.

DISCUSSION

P1

-	Xiaomi ok with this.

-	Ericsson agree, think we do not need a spereate UE capability only for UE-ID based subgrouping.

-	QC wonders what is the difference for OOK-based or OFDM-based. QC think the wording ‘support of LP-WUS’ is not that clear.

P3, P4

-	Xiaomi think they are discussed in R1, think we can just wait. Vivo agree.

-	vivo think we can discuss whether we need separate capabilities for offloading and relaxation.

-	QC want to separate the capabilities for LPWUS reception and offloading/relaxation.

-	Ericsson think there is no much power saving gain if we only support LPWUS monitoring but not relaxation.

[Post129bis][218][LPWUS] UE capabilities (Huawei)

Intended outcome: Summary with proposals on RAN2 UE capabilities for LP-WUS

Deadline:  Long

R2-2502142	Summary of [Post129][212][LPWUS] Running CR for TS 38.304 (CATT)	CATT	discussion	Rel-19	NR_LPWUS-Core

R2-2502154	Whether/How to reduce the number of thresholds for LP-WUS monitoring and RRM relaxation/offloading 	vivo	discussion	Rel-19	NR_LPWUS-Core

=> The above two contributions are moved to 8.4.3

R2-2501955	Summary of [Post129][213] LP-WUS in MR-DC	ZTE Corporation, Sanechips	discussion	Rel-19	NR_LPWUS-Core

=> Moved to 8.4.4

### 8.4.2	Procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE

Procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping, and entry/exit condition for LP-WUS monitoring

Sub-grouping related aspects

R2-2502324	Discussion on LP-WUS procedure and configuration	OPPO	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 5	Regarding the UE_ID based subgrouping formula for LP-WUS, revise description of Np:

Np is the number of subgroupNumForUEID for PEI, if configured and UE supports could monitor PEI in this cell; otherwise, Np is 1.

Proposal 6	Support LP-WUS and subgrouping for both DRX and eDRX UEs.

Proposal 7	For UE-ID based subgrouping for LP-WUS, UE_ID is defined as 5G-S-TMSI mod X, where X value is given as below.

<table>

Proposal 8	UE-ID based subgrouping for LP-WUS is not used when the UE has an active emergency PDU session.

R2-2502910	LP-WUS in idle and inactive	Ericsson	discussion

Noted

Proposal 1	LP-WUS is supported with eDRX without additional changes. I.e. the UE monitors LP-WUS during PTW instead of monitoring normal DRX during PTW as in regular eDRX.

Proposal 2	Ask RAN3 to provide 18 bits of the Hashed UE Identity Index Value IE for UE-ID based LP-WUS subgrouping in Rel-19.

Proposal 3	When the UE has an emergency PDU session LP-WUS is not used. Send an LS to CT1, RAN3 and SA2 to inform them.

R2-2502143	LP-WUS in RRC_IDLE/INACTIVE	CATT	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 1: LP-WUS is supported to be operated with eDRX simultaneously.

Proposal 2: For UE_ID based subgrouping for LP-WUS, UE_ID = 5G-S-TMSI mod X, X needs to be 2^18 for DRX case and 2^20 for eDRX case if supported.

Proposal 3: Send LS to RAN3 (CC SA2/CT1) to ask their concern on the agreed formula of UE_ID based subgrouping for LP-WUS.

Proposal 4: Confirm the principle that determines CN assigned subgrouping or UE_ID based subgrouping for PEI is reused for LP-WUS subgrouping.

DISCUSSIONS

On P5 from OPPO paper

-	Xiaomi think it requires more checking from UE and think the previous agreement is fine. Ericsson agree with Xiaomi. HW think this proposed change will create mis-match btw NW and UE. QC share the view from OPPO P5.

On emergency PDU session related issue

-	OPPO think there is some related discussion in another BO session, better to align.

-	QC think SA2 already remove any restriction so from R2 point of view we do no need to discuss. Vivo agree.

LP-WUS is supported with eDRX, FFS on exact impact if any

[AT129bis][201][LPWUS] Proposals for UE ID based sub-grouping (CATT)

Scope: Discuss remaining details for the formula of UE ID based sub-grouping, and also cover potential issue for emergency PDU session

Intended outcome: Summary with proposals in R2-2503031.

Deadline: Before Thursday CB

R2-2503031	[AT129bis][201][LPWUS] Proposals for UE ID based sub-grouping	CATT	discussion	Rel-19	NR_LPWUS-Core

Noted

DISCUSSION

P1, P2

-	NEC think either 31 or 32 can work, and think 32 matches better with UE ID design. NEC suggest to decide.

-	Ericsson think there can be a special group which cover all the UEs, so think 32 is better.

Use 5G-S-TMSI to determine the UE_ID in the formula of UE_ID based subgrouping for LP-WUS, i.e., UE_ID=5G-S-TMSI mod X.

X is based on 32 subgrouping number. Details can be discussed in the running CR.

P3

-	Tejas prefer Option 1. QC want to further check.

-	CATT suggest to put in into the open issue list.

P4

-	HW think P4 depends on P3, so we need to wait until we conclude on P3. ZTE share this view.

-	Ericsson think we need to ask R3 whether it is feasible to have 5 additional bits for UE ID in the formula.  OPPO share this view.

-	ZTE agree with P4 to send a LS. NEC agree. OPPO ok to send LS.

-	vivo suggest we only inform conclusion. Ericsson think it is not sufficient to inform the LS.

Send LS to RAN3 (CC SA2/SA3) to inform our agreements on UE ID based subgrouping.

Correct the typo as following for the previous agreed formula of UE_ID based subgrouping for LP-WUS:

        - Np is the number of subgroupsNumForUEID for PEI, if configured and UE supports PEI; otherwise, Np is 1.

Confirm the principle for determining CN assigned subgrouping or UE_ID based subgrouping for PEI is reused for LP-WUS subgrouping. Details will be discussed in the running CR.

[Post129bis][217][LPWUS] LS to RAN3 (CC SA2/SA3) to inform agreements on UE ID based subgrouping (NEC)

Intended outcome: Approve the LS

Deadline:  Short

R2-2503182	LS on LP-WUS subgrouping progress	RAN2	LS out	Rel-19	NR_LPWUS-Core	To:RAN3	Cc: SA2, SA3

=> Approved

LPWUS configuration

R2-2502308	Procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE	Apple	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 9: The LP-WUS subgrouping configuration is provided in SIB1.

R2-2502097	Discussion on procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE	Huawei, HiSilicon	discussion	Rel-19

Noted

Proposal 2: Dedicated configuration in RRC signaling is not needed for providing LP-WUS related configuration in RRC_IDLE/INACTIVE modes.

DISCUSSIONS

-	Apple explains that their P9 is already included in the current running CR. NEC think P9 can be updated to cover all LP-WUS related configuration except for measurement configurations. Ericsson not sure about putting all configuration in SIB1.

-	Xiaomi agree all the configuration is from SIB, and no dedicated configuration.

-	Samsung, IDT, CATT support HW P2.

-	Samsung ok to have the modified P9 as working assumption. Lenovo suggest to leave measurement configurations FFS.

All the LP-WUS related configurations except for measurement configurations are provided in SIB1. FFS the details on measurement configurations.

Dedicated configuration in RRC signaling is not needed for providing LP-WUS related configuration in RRC_IDLE/INACTIVE modes.

Entry/exit condition of LPWUS monitoring

R2-2502447	LP-WUS operation in IDLE/Inactive state	Qualcomm Incorporated	discussion	NR_LPWUS-Core

Noted

Proposal 7	Use existing Srxlev/Squal for MR measurement based entry/exit condition evaluation.

Proposal 8	Introduce Srxlev_lp/Squal_lp for LR measurement based entry/exit condition evaluation. The formula to derive Srxlev/Squal from measured MR value can be  used for deriving Srxlev_lp/Squal_lp from measured LR value.

R2-2502143	LP-WUS in RRC_IDLE/INACTIVE	CATT	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 5: Cell selection value, i.e., existing Srxlev/ Squal in TS 38.304, is used for serving cell quality by MR for conditions of LP-WUS monitoring, measurement relaxation and serving cell measurement offloading for LP-WUS UEs.

Proposal 6: Measured value is used for serving cell quality by LR for conditions of LP-WUS monitoring, measurement relaxation and serving cell measurement offloading for LP-WUS UEs.

DISCUSSION

P7 in QC paper, P5 in CATT paper

-	Ericsson think it is not crystal clear. QC clarify the intention is to reuse what we have in legacy. VDF from QC P7 is clear and good.

-	OPPO ask whether it is for serving cell meas offloading or for relaxation. CATT think it is for both.

-	Xiaomi, Samsung, OPPO understand these two proposals have the same intention. Xiaomi think CATT proposal is more clear.  Samsung Ok with these two proposals.

Use existing Srxlev/Squal for all MR measurement based entry/exit condition evaluation.

P8 in QC paper, P6 in CATT paper

-	Ericsson think we do not need offset values other than Qrxlevmin/ Qqualmin. QC, VDF agree this is useful. Ericsson think we should use ‘dB’ also for LR. NEC agree.

-	Vivo think there is not need to introduce additional. Samsung agree and support CATT proposal. Xiaomi, Apple agree.

-	CATT explains after some offline this seems not so critical issue.

Use measured value for all LR measurement based entry/exit condition evaluation.

Other aspects (e.g., enabling/disabling, activation/deactivation, etc.)

R2-2502659	Discussion on LP-WUS operation in RRC_IDLE/INACTIVE modes	InterDigital, Inc.	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 2: Support dedicated LP-WUS monitoring (de-)activation per UE via RRC release message.

R2-2502097	Discussion on procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE	Huawei, HiSilicon	discussion	Rel-19

Noted

Proposal 3: The CN indicates whether LP-WUS capable UE(s) is/are allowed to use the LP-WUS functionality by NAS signalling; the absence of indication means UE is allowed to use LP-WUS functionality

DISCUSSION

P3 from HW paper

-	Ericsson agree with P3 from HW paper, since network can notice some issue and should be able to disable LPWUS for those UEs. VDF also like HW P3.

-	Xiaomi think NW can assign UEs to different sub-groups so the issue is not that severe.

-	QC, Samsung think this is up to SA2. HW think it is just similar mechanism as PEI and think we can decide and inform SA2.

-	LG E think this is also related to potential handling of emergency PDU session, so think it is useful.

-	vivo think using NAS singaling is better.

P2 from IDT paper

-	VDF do not see the benefit of IDT P2 since NW may not know the future status/activities of UE. Xiaomi, QC share this view.

-	DCM support IDT from P2. OPPO ok to have this functionality. OPPO think if we ever have some mechanism then it is better to use RRC release. NEC share this view.

-	HW think if gNB do so then UE cannot use LPWUS function until after going back to connected.

-	Samsung think we already agree not to use dedicated signalling.

Agreements for LP-WUS in IDLE/INACTIVE

LP-WUS is supported with eDRX, FFS on exact impact if any Use 5G-S-TMSI to determine the UE_ID in the formula of UE_ID based subgrouping for LP-WUS, i.e., UE_ID = 5G-S-TMSI mod X. X is based on 32 subgrouping number. Details can be discussed in the running CR. Send LS to RAN3 (CC SA2/SA3) to inform our agreements on UE ID based subgrouping. Correct the typo as following for the previous agreed formula of UE_ID based subgrouping for LP-WUS: - Np is the number of subgroupsNumForUEID for PEI, if configured and UE supports PEI; otherwise, Np is 1. Confirm the principle for determining CN assigned subgrouping or UE_ID based subgrouping for PEI is reused for LP-WUS subgrouping. Details will be discussed in the running CR. All the LP-WUS related configurations except for measurement configurations are provided in SIB1. FFS the details on measurement configurations. Dedicated configuration in RRC signaling is not needed for providing LP-WUS related configuration in RRC_IDLE/INACTIVE modes. Use existing Srxlev/Squal for all MR measurement based entry/exit condition evaluation. Use measured value for all LR measurement based entry/exit condition evaluation.

R2-2501831	LP-WUS in RRC_IDLE/INACTIVE	HONOR	discussion	Rel-19	NR_LPWUS-Core

R2-2501893	Discussion on LP-WUS in RRC_IDLE INACTIVE 	NEC	discussion	Rel-19	NR_LPWUS-Core

R2-2501960	Procedure and configuration of LP-WUS for IDLE and INACTIVE mode	ZTE Corporation, Sanechips	discussion	Rel-19	NR_LPWUS-Core

R2-2501997	Procedure and Configuration of LP-WUS in RRC Idle Inactive Mode	Samsung	discussion	Rel-19

R2-2502005	Remaining issues on LP-WUS paging monitoring	Xiaomi Communications	discussion

R2-2502014	IDLE/INACTIVE mode procedures for supporting LP-WUS	Tejas Network Limited	discussion	Rel-19

R2-2502097	Discussion on procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE	Huawei, HiSilicon	discussion	Rel-19

R2-2502143	LP-WUS in RRC_IDLE/INACTIVE	CATT	discussion	Rel-19	NR_LPWUS-Core

R2-2502155	Discussion on LP-WUS WUR in RRC_IDLE INACTIVE	vivo	discussion	Rel-19	NR_LPWUS-Core

R2-2502212	LP-WUS in IDLE and INACTIVE	Nokia	discussion	Rel-19	NR_LPWUS-Core

R2-2502227	Remaining issues on LP-WUS in RRC IDLE or INACTIVE	LG Electronics Inc.	discussion	Rel-19	NR_LPWUS-Core

R2-2502308	Procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE	Apple	discussion	Rel-19	NR_LPWUS-Core

R2-2502324	Discussion on LP-WUS procedure and configuration	OPPO	discussion	Rel-19	NR_LPWUS-Core

R2-2502447	LP-WUS operation in IDLE/Inactive state	Qualcomm Incorporated	discussion	NR_LPWUS-Core

R2-2502486	RAN2 aspects on LP-WUS/WUR in RRC Idle/Inactive mode	Sony	discussion	Rel-19	NR_LPWUS-Core

R2-2502597	Procedure and Configuration of LP-WUS in RRC IDLE/INACTIVE	Lenovo	discussion	Rel-19	NR_LPWUS-Core

R2-2502659	Discussion on LP-WUS operation in RRC_IDLE/INACTIVE modes	InterDigital, Inc.	discussion	Rel-19	NR_LPWUS-Core

R2-2502743	Discussion on LP-WUS in RRC_IDLE and RRC_INACTIVE	Sharp	discussion	Rel-19

R2-2502901	Further Consideration on LP-WUS operation in IDLE/INACTIVE	CMCC	discussion	Rel-19	NR_LPWUS-Core

R2-2502910	LP-WUS in idle and inactive	Ericsson	discussion

R2-2502976	Discussion on the LP-WUS handling for Emergency call back	NTT DOCOMO INC..	discussion	Rel-19	NR_LPWUS-Core

R2-2502977	Discussion on the LP-WUS capability issue within non-homogeneous deployment	NTT DOCOMO INC..	discussion	Rel-19	NR_LPWUS-Core

### 8.4.3	RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE

RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions

RRM measurement relaxation

R2-2502142	Summary of [Post129][212][LPWUS] Running CR for TS 38.304 (CATT)	CATT	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 1: General description and criteria of RRM relaxation and offloading for LP-WUS are captured in TS 38.304. Other details of RRM relaxation and offloading for LP-WUS are captured in RAN4.

Proposal 2: Send an LS to inform RAN4.

R2-2501998	RRM measurement relaxation and offloading in RRC Idle Inactive Mode	Samsung	discussion	Rel-19

Noted

Proposal 3. For the entry/ exit conditions of serving cell measurement offloading and serving cell RRM measurement relaxation,

-	Separate thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs.

R2-2502156	Discussion on RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE	vivo	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 9: The metrics for RRM measurement offloading/relaxation criteria include (LP-)RSRP and optional (LP-)RSRQ.

DISCUSSION

-	On CATT P1, ZTE think it is straightforward.

-	Nokia, vivo, Apple support Samsung P3. On Samsung P3, ZTE thik ‘if..’ should be removed. HW think R4 is also discussing this and think we should wait for R4 progress, Nokia think R2 can discuss threshold and R4 discuss requirements.

-	ZTE support P9 from vivo paper.

R2-2501894	Discussion on LP-WUS RRM 	NEC	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal-1: introduce separate entry/exit condition, i.e., the LP-WUS UE performs relaxed MR serving cell measurement when entry condition is fulfilled and stops performing relaxed MR serving cell measurement when exit condition is fulfilled.

Entry condition: based on MR measurement results and optional LR measurement results;

Exit condition: based on LR measurement results.

Proposal-2: same criterion should be applied to both MR serving cell and neighbour cell measurement relaxation.

Proposal-3: once the criteria for MR serving cell measurement offloading is met, then both MR serving cell and neighbour cell measurements should be stopped.

R2-2502351	RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE	Lenovo	discussion	Rel-19

Noted

Proposal 4: RAN2 to consider ‘not fulfilling the entry condition’ as the exit condition for serving cell measurement relaxation.

Proposal 5: The entry condition for serving cell RRM measurement relaxation is configured independently with the neighboring cell RRM measurement relaxation.

R2-2502154	Whether/How to reduce the number of thresholds for LP-WUS monitoring and RRM relaxation/offloading 	vivo	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 1: RAN2 to discuss whether/how to reduce the number of thresholds for LP-WUS/WUR WI considering the following one or more directions:

•	Direction 1: Remove the LR based thresholds for all entry conditions in the above 4 RRM relaxation/offloading and LP-WUS monitoring procedures

•	Direction 2: Merge the entry/exit condition for Serving Cell RRM measurement relaxation and Neighboring Cell RRM measurement relaxation

•	Direction 3: Merge the entry/exit conditions for Serving Cell RRM measurement offloading or Serving Cell RRM measurement relaxation or Neighboring Cell RRM measurement relaxation and LP-WUS monitoring

•	Direction 4: Merge the entry/exit conditions for Serving Cell RRM measurement offloading and the current threshold for stopping neighboring cell measurement

DISCUSSION

On vivo P1

-	IDT think with LPWUS we have separate receivers so it is unsafe to combine with legacy thresholds. CATT ok with high level principle and think we should check in details how to reduce. Ericsson think we should discuss how to reduce, Apple, HW, Samsung agree. Apple fine with all directions listed. HW not ready for direction 4, since it is too restrictive. ZTE think it is too early to down-select, Samsung agree. QC do not agree with direction 1, for the others we can check later.

-	Ericsson has concern on serving cell relaxation, and think it should not be configured together with serving cell measurement offloading. Apple think it is up to NW implementation.

[AT129bis][202][LPWUS] Proposals for RRM measurement relaxation (vivo)

Scope: Discuss remaining aspects for RRM measurement relaxation, including definition of the thresholds/conditions, can also discuss how to reduce the number of thresholds

Intended outcome: Summary with proposals in R2-2503032.

Deadline: Before Thursday CB.

R2-2503032	Discussion report on [AT129bis][202][LPWUS]	vivo	discussion	Rel-19	NR_LPWUS-Core

Noted

DISCUSSION

-	HW think the first one need to update to reflect R4 terminology of different types of LP WURs. Vivo think it is fine to update. Ericsson think we should follow R1/R4 when it comes to type of WUR.

-	ZTE ask if we consider low mobility condition when we merge the conditions for serving and neighbour cell measurement relx. Vivo explains that by now we haven’t agreed low mobility based condition, and think it is FFS.

-	Nokia has strong concern on the last agreement point.

RAN2 assumes for the entry/ exit conditions of serving cell measurement offloading and serving cell RRM measurement relaxation: separate MR thresholds (according to RAN1 agreement)/LR thresholds can be configured for different types of LP WUR if a cell supports both types of LRs (can revisit based on RAN1 and RAN 4 progress, if any).

RAN2 assumes the entry/exit thresholds for RRM relaxation/offloading for OFDM-based WUR measuring LP-SS only are the same as that for OOK-based WUR measuring LP-SS. It can be revisited based on RAN1/RAN4 process, if any. Network is allowed to provide either OOK based threshold or OFDM based WUR mesasuring SSB threhold or both.

It is up to NW to configure either serving cell relaxation or serving cell offloading or both in one cell.

The metrics for RRM measurement offloading/relaxation criteria include (LP-)RSRP and optional (LP-)RSRQ.

How to define LP-RSRP and LP-RSRQ is up to RAN1.

The duplication between RAN2 and RAN4 specification on RRM relaxation and offloading should be avoided, details up to running CR rapporteur and companies’ review.

Merge the entry/exit condition for Serving Cell RRM measurement relaxation and Rel-19 Neighboring Cell RRM measurement relaxation (higher priority frequency is separate discussion).

Chair observation: only one company have strong concern on the following discussion point

- Can further discuss Direction 3:

      - Direction 3a. merge LPWUS monitoring and serving cell offloading

      - Direction 3b. merge LPWUS monitoring and Rel-19 RRM relaxation

Other agreements on RRM measurement relaxation and offloading

RAN2 assumes for the entry/ exit conditions of serving cell measurement offloading and serving cell RRM measurement relaxation: separate MR thresholds (according to RAN1 agreement)/LR thresholds can be configured for different types of LP WUR if a cell supports both types of LRs (can revisit based on RAN1 and RAN 4 progress, if any). RAN2 assumes the entry/exit thresholds for RRM relaxation/offloading for OFDM-based WUR measuring LP-SS only are the same as that for OOK-based WUR measuring LP-SS. It can be revisited based on RAN1/RAN4 process, if any. Network is allowed to provide either OOK based threshold or OFDM based WUR mesasuring SSB threhold or both. It is up to NW to configure either serving cell relaxation or serving cell offloading or both in one cell. The metrics for RRM measurement offloading/relaxation criteria include (LP-)RSRP and optional (LP-)RSRQ. How to define LP-RSRP and LP-RSRQ is up to RAN1. The duplication between RAN2 and RAN4 specification on RRM relaxation and offloading should be avoided, details up to running CR rapporteur and companies’ review. Merge the entry/exit condition for Serving Cell RRM measurement relaxation and Rel-19 Neighboring Cell RRM measurement relaxation (higher priority frequency is separate discussion).

R2-2501894	Discussion on LP-WUS RRM 	NEC	discussion	Rel-19	NR_LPWUS-Core

R2-2501967	Further discussion on the criteria for RRM measurement relaxation and offloading	Huawei, HiSilicon	discussion	Rel-19	NR_LPWUS-Core

R2-2501998	RRM measurement relaxation and offloading in RRC Idle Inactive Mode	Samsung	discussion	Rel-19

R2-2502006	Discussion on RRM measurement relaxation for RRC_IDLE_INACTIVE	Xiaomi Communications	discussion

R2-2502144	RRM Relaxation and Offloading in RRC_IDLE/INACTIVE	CATT	discussion	Rel-19	NR_LPWUS-Core

R2-2502156	Discussion on RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE	vivo	discussion	Rel-19	NR_LPWUS-Core

R2-2502213	RRM measurement relaxation in RRC_IDLE/INACTIVE	Nokia	discussion	Rel-19	NR_LPWUS-Core

R2-2502228	Remaining issues on measurement offloading and relaxation	LG Electronics Inc.	discussion	Rel-19	NR_LPWUS-Core

R2-2502309	RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE	Apple	discussion	Rel-19	NR_LPWUS-Core

R2-2502325	Discussion on RRM measurement in RRC IDLE and INACTIVE	OPPO	discussion	Rel-19	NR_LPWUS-Core

R2-2502351	RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE	Lenovo	discussion	Rel-19

R2-2502449	LP-WUS RRM measurement relaxation and offloading	Qualcomm Incorporated	discussion	NR_LPWUS-Core

=> Revised in R2-2502995

R2-2502995	LP-WUS RRM measurement relaxation and offloading	Qualcomm Incorporated	discussion	NR_LPWUS-Core

R2-2502660	Discussion on RRM measurement relaxation and offloading	InterDigital, Inc.	discussion	Rel-19	NR_LPWUS-Core

R2-2502722	Discussion on RRM measurement relaxation and offloading in RRC_IDLE INACTIVE	CMCC	discussion	Rel-19	NR_LPWUS-Core

R2-2502744	Discussion on RRM measurement relaxation and offloading	Sharp	discussion	Rel-19

R2-2502757	Discussion on neighboring cell measurement with LR	InterDigital, Ericsson, Nokia, Sony, Vodafone, KT	discussion	Rel-19	NR_LPWUS-Core

R2-2502760	Discussion on RRM measurement relaxation and offloading for RRC_IDLE and INACTIVE	China Telecom	discussion

R2-2502911	LP-WUS and RRM measurements	Ericsson	discussion

R2-2502931	RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE	ZTE Corporation, Sanechips	discussion	Rel-19	NR_LPWUS-Core	R2-2501090

### 8.4.4	Procedures for LP-WUS in RRC_CONNECTED

Procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring.

On short DRX cycle

R2-2502477	LP-WUS in CONNECTED mode	InterDigital	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 6: For Option 1-1, when Short DRX cycle is used, the UE does not monitor LP-WUS outside the Active Time.

Proposal 7: For Option 1-2, the UE monitors LP-WUS outside the Active Time regardless of if Short DRX cycle or Long DRX cycle is used.

R2-2502882	LP-WUS in RRC_CONNECTED	Nokia, Nokia Shanghai Bell	discussion	NR_LPWUS-Core

Noted

Proposal 1: For option 1-1, it is up to network configuring short DRX cycle with LP-WUS. Send an LS to RAN1 asking if LP-WUS is applicable to short DRX cycle as well as long DRX cycle for option 1-1.

Proposal 2: For option 1-2, it is up to network configuring short DRX cycle with LP-WUS. Send an LS to RAN1 asking if there are any technical concerns regarding configuring of short DRX cycle in option 1-2.

R2-2502448	LP-WUS operation in CONNECTED state	Qualcomm Incorporated	discussion	NR_LPWUS-Core

Noted

Proposal 1	For Option 1-1, short DRX can be configured, and UE will stop LP-WUS monitor when short DRX is started.

Proposal 2	For Option 1-2, short DRX is not configured.

DISCUSSION

Short DRX for Option 1-1

-	Xiaomi support IDT proposal 6. NEC, Tejas agree.

-	OPPO fine to follow DCP mechanism but think QC P1 is more clear. Lenovo agree.

Short DRX for Option 1-2

-	Xiaomi support IDT P7. Samsung, OPPO, CATT agree.

-	Nokia think R1 is discussing different options regarding the relationship btw LPWUS monitoring and DRX cycle, so we should ask R1.

-	CATT think we can have working assumption in R2 and inform R1.

For Option 1-1, the UE does not monitor LP-WUS when Short DRX cycle is used.

Working assumption: For option 1-2, it is up to network configuring short DRX cycle with LP-WUS. The UE monitors LP-WUS outside the Active Time regardless of if Short DRX cycle or Long DRX cycle is used.

Send an LS to RAN1 to inform the above conclusions, can revisit if needed based on RAN1 feedback.

Other aspects related to the procedure (e.g., collision handling, UAI, etc.)

R2-2502157	Discussion on LP-WUS WUR in RRC_Connected	vivo	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 3: For LP-WUS Option 1-2, during the timer triggered by LP-WUS is running, periodic CSI/L1-RSRP report and SP CSI report behaviour are the same as legacy drx-onDurationTimer duration without DCP configuration, i.e. always report.

Proposal 4: The preferred value of time offset between LP-WUS and PDCCH reported via UAI is expected to be longer than the value for the minimum time gap between LP-WUS and MR to start PDCCH monitoring corresponding to the SCS reported via UE capability.

Proposal 5: For LP-WUS Option 1-1 and Option 1-2, RAN2 should consider the UE behaviour when all LP-WUS occasion collides with C-DRX Active Time/ measurement gap/BWP switching interruption length/RAR for BFR.

Proposal 6: For LP-WUS Option 1-1 and Option 1-2, in case all the LP-WUS occasion of an LP-WUS collides with C-DRX Active Time/ measurement gap/BWP switching interruption length/RAR for BFR, UE starts the corresponding drx-onDurationTimer for LP-WUS Option 1-1 and the corresponding new timer for LP-WUS Option 1-2.

Proposal 7: For LP-WUS Option 1-1, in case UE cannot detect all LP-WUS occasion of a LP-WUS, RAN2 assumes it is up to the network configuration for UE to determine whether to start the corresponding drx-onDurationTimer, e.g. lpwus-Wakeup.

R2-2502912	LP-WUS in connected	Ericsson	discussion

Noted

Proposal 4	In Option 1-1, when the UE is not able to monitor the LP-WUS occasion the UE should start the drx-OnDurationTimer (as if LP-WUS was detected). FFS for Option 1-2.

Proposal 6	If configured, the UE can signal a preferred time offset via UAI signalling.

DISCUSSION

On collision cases

-	Xiaomi not sure if these collisions cases are valid for LPWUS because it is different receiver than MR. HW think we cannot assume UE is able to use MR and LR at the same time. OPPO think Xiaomi question is valid and think we in RAN2 cannot decide. CATT agree and think we should send LS to R1. WI rapp think the design depends on R1 but it can be discussed in R2. Vivo also OK to send LS to R1. IDT think we can first agree Ericsson P4 and then inform/ask R1.

-	HW support P4 from Ericsson. HW think LPWUS is a sequence then it can be detected by UE when it is transmitted by NW.

-	Apple wonders whether such collision handling impact MAC spec or not.

Working assumption for the case of potential collision (if any): In Option 1-1, when the UE is not able to monitor the LP-WUS occasion(s) the UE should start the drx-OnDurationTimer (as if LP-WUS was detected). FFS for Option 1-2.

Send LS to inform this working assumption, can also ask a) what are the cases when UE cannot monitor LP-WUS, b) whether UE can monitor LR and MR simultaneously.

On UAI

-	Xiaomi not sure whether UAI is preferred value or it is a rather static UE capability.

-	Vivo think R1 already agree UAI and UE capability.

-	Apple think it is still FFS in R1 what the UAI is providing (whether it is minimum value or maximum). Apple suggest to wait. Nokia think R2 can discuss UAI.

-	IDT, vivo ok to have Ericsson P6.

-	vivo suggest to ask R1 whether the UAI value is smaller or greater than UE capability. Nokia think it is sufficient to ask R1 what should be reported in UAI.

If configured, the UE can signal a preferred time offset via UAI signalling.

Ask RAN1 for further information regarding their conclusions.

[Post129bis][203][LPWUS] LS to R1 LP-WUS in RRC_CONNECTED (InterDigital)

Intended outcome: Approve the LS.

Deadline: Short

R2-2503187	Draft LS on LP-WUS in RRC_CONNECTED	RAN2	LS out	Rel-19	NR_LPWUS-Core	To:RAN1, RAN4

=> Approved (but then coversheet revised by MCC: "Draft" in the LS title)

R2-2503202	LS on LP-WUS in RRC_CONNECTED	RAN2	LS out	Rel-19	NR_LPWUS-Core	To:RAN1, RAN4

=> Approved

Dual DRX group

R2-2501769	Discussing on LP-WUS monitoring in Connected mode	Xiaomi	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 5	In CA with dual DRX groups, LP-WUS can be configured per DRX group or for all serving cells.

Proposal 6	LP-WUS configured in a DRX group is used to trigger PDCCH monitoring in all serving cells in the same DRX group.

Proposal 7	Whether UE can monitor LP-WUS for two DRX groups is up to UE capability.

R2-2501992	Remainng issues on LP-WUS in RRC_CONNECTED	LG Electronics Inc.	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 2. RAN2 asks RAN1 whether LP-WUS and the secondary DRX group can be configured simultaneously, different from DCP.

DISCUSSION

-	Ericsson think this was not agreed for DCP and think it is good to have LPWUS and dual DRX together, e.g., for the case of FR1 + FR2 (latency improvement for busty data). LG E, OPPO, ZTE see the benefit.

-	Lenovo have a different view than Ericsson and think there would be impact, e.g.,  to CSI report. Lenovo think we do not support this for at least option 1-1, and also OK ask R1 first.

-	CATT think R1 discussed this during the week and there is no consensus, so think we can wait for R1 progress.

-	Nokia do not think R1 can discuss further since they only have 1 more meeting for R19 WID. Nokia think if there is no consensus then we should conclude like that.

-	OPPO think we can conclude in R2 and support LPWUS + dual DRX.

-	ZTE think we can agree in R2 and we ask R1 to see if they have concern.

-	Apple ask what is the intentioned behaviour, i.e., whether LPWUS is waking up to two cell groups simultaneously. Vivo think the two cell groups are waking up by LPWUS at the same time.

-	WI rapp suggest to conclude in R2 and ask R1. Nokia think the LS is not beneficial for now.

-	HW not ready send LS.

FFS whether/how to support LP-WUS (including Option 1-1 and 1-2) and dual DRX group

MRDC

R2-2501955	Summary of [Post129][213] LP-WUS in MR-DC	ZTE Corporation, Sanechips	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 1: For NR-DC, the LP-WUS can be configured to be monitored at least on the PCell and PSCell. Wait for RAN1 progress on whether to allow LP-WUS configuration and monitoring on other Cells.

Proposal 2: For NR-DC, the LP-WUS in MCG and SCG can be configured independently.

Proposal 3: Apart from NR-DC, LP-WUS can also be supported in NE-DC, EN-DC, NGEN-DC. And proposal 1 and 2 also apply to NE-DC, EN-DC, NGEN-DC.

DISCUSSION

-	Xiaomi generally ok with the proposals, for think for P2 it is open whether MCG and SCG can be configured together. ZTE think such configuration is possible and do not see showstopper. HW agree with ZTE.

-	Apple think such configurations depend on UE capability. QC agree.

For NR-DC, the LP-WUS can be configured to be monitored at least on the PCell and PSCell. Wait for RAN1 progress on whether to allow LP-WUS configuration and monitoring on other Cells.

For NR-DC, the LP-WUS in MCG and SCG can be configured independently.

Apart from NR-DC, LP-WUS can also be supported in NE-DC, EN-DC, NGEN-DC. And proposal 1 and 2 also apply to NE-DC, EN-DC, NGEN-DC.

Agreements on LP-WUS in RRC_CONNECTED

For Option 1-1, the UE does not monitor LP-WUS when Short DRX cycle is used. Working assumption: For option 1-2, it is up to network configuring short DRX cycle with LP-WUS. The UE monitors LP-WUS outside the Active Time regardless of if Short DRX cycle or Long DRX cycle is used. Send an LS to RAN1 to inform the above conclusions, can revisit if needed based on RAN1 feedback. Working assumption for the case of potential collision (if any): In Option 1-1, when the UE is not able to monitor the LP-WUS occasion(s) the UE should start the drx-OnDurationTimer (as if LP-WUS was detected). FFS for Option 1-2. Send LS to inform this working assumption, can also ask a) what are the cases when UE cannot monitor LP-WUS, b) whether UE can monitor LR and MR simultaneously. If configured, the UE can signal a preferred time offset via UAI signalling. Ask RAN1 for further information regarding their conclusions. FFS whether/how to support LP-WUS (including Option 1-1 and 1-2) and dual DRX group For NR-DC, the LP-WUS can be configured to be monitored at least on the PCell and PSCell. Wait for RAN1 progress on whether to allow LP-WUS configuration and monitoring on other Cells. For NR-DC, the LP-WUS in MCG and SCG can be configured independently. Apart from NR-DC, LP-WUS can also be supported in NE-DC, EN-DC, NGEN-DC. And proposal 1 and 2 also apply to NE-DC, EN-DC, NGEN-DC.

R2-2501769	Discussing on LP-WUS monitoring in Connected mode	Xiaomi	discussion	Rel-19	NR_LPWUS-Core

R2-2501832	Procedures for LP-WUS in RRC_CONNECTED	HONOR	discussion	Rel-19	NR_LPWUS-Core

R2-2501895	Discussion on LP-WUS in RRC_CONNECTED 	NEC	discussion	Rel-19	NR_LPWUS-Core

R2-2501961	Procedures for LP-WUS in RRC_CONNECTED	ZTE Corporation, Sanechips	discussion	Rel-19	NR_LPWUS-Core

R2-2501992	Remainng issues on LP-WUS in RRC_CONNECTED	LG Electronics Inc.	discussion	Rel-19	NR_LPWUS-Core

R2-2501999	Procedures for LP-WUS in RRC Connected Mode	Samsung	discussion	Rel-19

R2-2502016	LP-WUS operation in Connected mode	Tejas Network Limited	discussion	Rel-19

R2-2502145	Analysis on LP-WUS for RRC_CONNECTED	CATT	discussion	Rel-19	NR_LPWUS-Core

R2-2502157	Discussion on LP-WUS WUR in RRC_Connected	vivo	discussion	Rel-19	NR_LPWUS-Core

R2-2502310	Procedures for LP-WUS in RRC_CONNECTED	Apple	discussion	Rel-19	NR_LPWUS-Core

R2-2502326	Discussion on LP-WUS in RRC_CONNECTED	OPPO	discussion	Rel-19	NR_LPWUS-Core

R2-2502448	LP-WUS operation in CONNECTED state	Qualcomm Incorporated	discussion	NR_LPWUS-Core

R2-2502471	Further discussion on LP-WUS for RRC_CONNECTED mode	Huawei, HiSilicon	discussion	Rel-19	NR_LPWUS-Core

R2-2502477	LP-WUS in CONNECTED mode	InterDigital	discussion	Rel-19	NR_LPWUS-Core

R2-2502598	LP-WUS in RRC Connected Mode	Lenovo	discussion	Rel-19	NR_LPWUS-Core

R2-2502723	Discussion on LP-WUS operation in CONNECTED mode	CMCC	discussion	Rel-19	NR_LPWUS-Core

R2-2502882	LP-WUS in RRC_CONNECTED	Nokia, Nokia Shanghai Bell	discussion	NR_LPWUS-Core

R2-2502912	LP-WUS in connected	Ericsson	discussion

## 8.5	Network Energy Saving Enh.

(Netw_Energy_NR_enh-Core; leading WG: RAN1; REL-19; WID: RP-242354)

Time budget: 1 TU

Tdoc Limitation: 3 tdocs

### 8.5.1	Organizational

Incoming LS, WI rapporteur inputs, CR rapporteur inputs (including post email discussion [POST129][101], [102], [103], [104], summary of identified open issues that need online discussion and rapporteur’s suggestions if needed).

R2-2501722	LS on time location of on-demand SSB for Scell (R1-2501633; contact: LGE)	RAN1	LS in	Rel-19	Netw_Energy_NR_enh	To:RAN4	Cc:RAN2

Noted.

R2-2501984	Introduction of Network Energy Savings Enhancements running CR	Huawei, HiSilicon	draftCR	Rel-19	38.300	18.5.0	B	Netw_Energy_NR_enh-Core

Endorsed.

R2-2502173	Running RRC CR for enhancements for network energy efficiency 	Ericsson	draftCR	Rel-19	38.331	18.5.1	Netw_Energy_NR_enh-Core

Endorsed.

R2-2502150	Summary of Comments to 38.331 CR for NES	Ericsson	discussion	Rel-19	Netw_Energy_NR_enh-Core

[AT129b][105][NES] (Ericsson)

Scope: Discuss the identified FFSs (offline discussion rapporteur can pick up which FFSs) in R2-2502150 (offline discussion rapporteur can pick up company’s proposal if needed) and make conclusions as much as possible. Note discussion should not be overlapped with the identified issues that are already included in the agenda.

Intended outcome: Discussion summary in R2-2503004.

Deadline: Comeback in CB session => Completed.

R2-2503004	[AT129bis ][105] [NES] FFSs to 38.331 CR for NES (Ericsson)	Ericsson	discussion	Rel-19	Netw_Energy_NR_enh-Core

Define features, not devices. That is, definition of OD-SIB1, OD-SSB, paging adaptation etc in 38.300 and in other specifications refer to 38.300:

- Avoid NES, Cell A, UL-WUS

- “UE supporting OD-SIB1” where/if needed, and to replace with the UE capability name

- UL-WUS is replaced with SIB1 request in e.g. field descriptions

- ul-WUS-Config renamed as od-sib1-Config

Current RRC only has this for idle inactive in 5.2.2.5, add what is needed for connected mode. SIBxx is not defined as essential SI and that EN is deleted.

Keep FFS “FFS how to capture the UE first should acquire a valid SIB1 (e.g. via SIB1 request) for camping on an OD-SIB1 NES cell.” And “FFS how does UE check is SIB1 is already provided.” for now but change “whether to capture”.

Keep the FFS for normal uplink for now.

“acquire the requested SIB1 message as defined in FFS, immediately;” changed to “acquire the requested SIB1 message as defined in FFS, immediately;”

Add in 331 a branch where UE receives notification from MAC that number to max RACH attempts were reached and UE barrs the cell according to 304.

R2-2502323	Running 38.304 CR for network energy saving	Apple (Rapporteur)	draftCR	Rel-19	38.304	18.4.0	B	Netw_Energy_NR_enh-Core

Endorsed.

R2-2502129	Summary report of [POST129][104][NES] 38.304 running CR and stage 3 open issues	Apple (Rapporteur)	discussion	Rel-19	Netw_Energy_NR_enh-Core

[AT129b][106][NES] (Apple)

Scope: Discuss the identified FFSs (offline discussion rapporteur can pick up which FFSs) in R2-2502129 (offline discussion rapporteur can pick up company’s proposal if needed) and make conclusions as much as possible. Note discussion should not be overlapped with the identified issues that are already included in the agenda.

Intended outcome: Discussion summary in R2-2503005.

Deadline: Comeback in CB session => Completed.

R2-2503005	Summary of [AT129b][106][NES] (Apple)	Apple (Rapporteur)	discussion	Rel-19	Netw_Energy_NR_enh-Core

Following agreement on “barring behavior clean-up”, capture the UE barring behavior of OD-SIB1 in both TS 38.331 and TS 38.304. Check offline whether we can rely on existing legacy UE barring specification in 38.304 (e.g. the case of RACH failure).

(modified) On the unbarring behavior of OD-SIB1 UE, capture it as normative text in TS 38.304. Way-forward Proposal (on offline check of Proposal 1): For UE barring the OD-SIB1 cell in case of RACH failure, capture in TS 38.304, e.g.

- If the cell is to be treated as if the cell status is "barred" due to the OD-SIB1 request number reaching preambleTransMax;

2> UE may exclude the barred cell as a candidate for cell selection/reselection for up to 300 seconds;

[Session chair]: Suggest “shall” to be changed to “may” to be aligned with MIB and SIB1 acquisition failure. [Vivo]: Understand we discussed some related topic in the past and it was decided as shall. [Session chair]: Let’s have “may” now. If problem, we can reconsider it later.

R2-2502587	Running 38.321 CR for network energy saving	InterDigital	draftCR	Rel-19	38.321	18.5.0	B	Netw_Energy_NR_enh-Core

Endorsed.

R2-2502584	Summary of Comments to 38.321 CR for NES	InterDigital	discussion	Rel-19

[AT129b][107][NES] (IDC)

Scope: Discuss the identified FFSs (offline discussion rapporteur can pick up which FFSs) in R2-2502584 (offline discussion rapporteur can pick up company’s proposal if needed) and make conclusions as much as possible. Note discussion should not be overlapped with the identified issues that are already included in the agenda.

Intended outcome: Discussion summary in R2-2503006.

Deadline: Comeback in CB session => Completed.

R2-2503006	[AT129bis][107][NES] Open issues for 38.321 CR (InterDigital)	InterDigital	discussion	Rel-19

Capture the following understanding in meeting notes: RA on an SpCell can include a cell on which the UE is requesting OD-SIB1. No specification changes needed.

Reuse legacy SSB selection procedure in MAC spec for RA initiated for SIB1 request, i.e. the UE selects any SSB if no SSB is above the configured RSRP threshold. This can be revisited if RAN1 agrees otherwise.

R2-2501956	Running 38.306 CR for NES UE capability	ZTE Corporation, Sanechips	draftCR	Rel-19	38.306	18.5.0	Netw_Energy_NR_enh-Core

Endorsed.

[POST129b][111][NES] (Ericsson)

Scope: Update RRC running CR based on RAN2#129bis progress and maintain essential open issue list in a separate contribution (RRC running CR can keep editor’s notes for readability).

Intended outcome: Updated RRC running CR and essential RRC open issue list.

Deadline: Long email discussion

[POST129b][112][NES] (IDC)

Scope: Update MAC running CR based on RAN2#129bis progress and maintain essential open issue list in a separate contribution (MAC running CR can keep editor’s notes for readability).

Intended outcome: Updated MAC running CR and essential MAC open issue list.

Deadline: Long email discussion

[POST129b][113][NES] (Apple)

Scope: Update 38.304 running CR based on RAN2#129bis progress and maintain essential open issue list in a separate contribution (38.304 running CR can keep editor’s notes for readability).

Intended outcome: Updated 38.304 running CR and essential 38.304 open issue list.

Deadline: Long email discussion

[POST129b][114][NES] (Huawei)

Scope: Update 38.300 running CR based on RAN2#129bis progress.

Intended outcome: Updated 38.300 running CR.

Deadline: Long email discussion

[POST129b][121][NES] (ZTE)

Scope: Discuss initial discussion points for UE capabilities.

Intended outcome: Discussion summary.

Deadline: Long email discussion

### 8.5.2	On-demand SSB SCell operation

Remaining open issues, including details of OD-SSB MAC CE (with the consideration of RAN1 progress), L3 RRM measurements, stage-3 identified open issues if needed, etc.

1. OD-SSB Activation/deactivation MAC CE Format

R2-2502130	Remaining issues on on-demand SSB for Scell	Apple	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 1 (modified): For explicit activation/deactivation, the OD-SSB MAC-CE includes fixed sized bitmap to indicate whether OD-SSB is activated in each SCell (i.e., similar to legacy SCell A/D MAC-CE).

[Huawei]: Our proposal is to include only SCells supporting OD-SSB. [OPPO, Nokia, ZTE, IDC]: Prefer simple option (Apple P1). [IDC]: Can we also agree with A/D bit, “1” means activation, “0” means deactivation? [Apple]: Yes, for explicit deactivation case.

For explicit activation/deactivation, the OD-SSB MAC-CE includes fixed sized bitmap to indicate whether OD-SSB is activated in each SCell (i.e., similar to legacy SCell A/D MAC-CE).

A/D bit: “1” means activation, “0” means deactivation for explicit deactivation case.

Proposal 2 (modified): For explicit activation/deactivation, the OD-SSB MAC-CE supports two formats: one format indicates up to 7 SCells and the other format indicates up to 31 SCells.

For explicit activation/deactivation, the OD-SSB MAC-CE supports two formats: one format indicates up to 7 SCells and the other format indicates up to 31 SCells.

Proposal 3 (modified): OD-SSB MAC-CE includes a configuration index for each SCell activating OD-SSB.

OD-SSB MAC-CE includes a configuration index for each SCell activating OD-SSB.

R2-2502840	Discussion on on-demand SSB SCell operation for NES	Huawei, HiSilicon	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 1: For OD-SSB MAC CE, RAN2 to consider a variable size MAC CE, in which a bitmap is used for SCells supporting OD-SSB in increasing order, and a configuration index is indicated for each SCell supporting OD-SSB.

Proposal 3: Use an eLCID for the OD-SSB transmission MAC CE.

2. Need of separate MAC CE for implicit deactivation

R2-2502130	Remaining issues on on-demand SSB for Scell	Apple	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 4a: For one Rel-19 UE, the Network configures whether to use either implicit deactivation or explicit deactivation (but not both) of OD-SSB transmission in all its SCells (i.e. per UE rather than per SCell config).

Proposal 4b: If Proposal 4a can be agreed, RAN2 confirm that it is not necessary to introduce two separate MAC-CEs for implicit deactivation and explicit deactivation respectively.

[Vivo, Nokia]: Do not think we need separate MAC CE for implicit deactivation. [Nokia]: Even without Apple’s proposal 4a, we still can have common MAC CE to handle both implicit deactivation and explicit deactivation. For example, the UE can start a timer whenever it receives activation indication for the corresponding SCell. [Session chair]: Without proposal 4a, there may be some issue. Let’s see the following Xiaomi’s contribution, observation#1. Understand w/ proposal 4a, if implicit deactivation is configured for the UE, network will set “0” for the SCell that already activated its OD-SSB and the UE will simply ignore it (since “0” deactivation is only applicable to explicit deactivation) and continue the ongoing counting.

R2-2501805	Discussion on on-demand SSB	Xiaomi	discussion

Proposal 4: For OD-SSB activation and deactivation, RAN2 to discuss and further down select among the following options:

·	Alternative1: separate MAC CEs for OD-SSB activation and deactivation.

·	Alternative2: single MAC CE for OD-SSB activation and deactivation with two bitmaps

·	Alternative3: single MAC CE for OD-SSB activation and deactivation with one bitmap (similar as legacy SCell activation/deactivation MAC CE)

- Alternative3.1: NW ensures either implicit deactivation or explicit deactivation is used for all OD-SSB SCells

- Alternative3.2: NW updates the associated number N for the SCells of which OD-SSB is “re-activated”

Offline discussion among interested companies: Whether common MAC CE format can handle both implicit deactivation and explicit deactivation. If not, do we need a separate MAC CE format for implicit deactivation or do we need any additional information field in the MAC CE (OPPO).

Will comeback in CB session.

CB session:

RAN2 aims to define single MAC CE format for both explicit and implicit OD-SSB deactivations. Details are FFS.

[Ericsson]: Wonder if we need to send LS to RAN1, e.g. ask the number of OD-SSB configurations, do we really need periodicity change, etc. [LG]: RAN2 can make decision (do not see the need to send an LS). [Apple]: Share the view with Ericsson. For example, RAN1 only defined periodicity change up to now. [IDC]: RAN1 is working on them and RAN1 understands configuration index/id is enough. [Nokia]: RAN2 can make decision on the number of OD-SSB configurations.

3. L3 RRM: MO configuration

R2-2502130	Remaining issues on on-demand SSB for Scell	Apple	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 8 (modified): When A-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB.

R2-2502148	Discussion on on-demand SSB for NES	Ericsson	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 3: RAN2 to agree to use the existing serving cell measurement object to support the serving cell measurements of OA-SSB and OD-SSB when these are within the same BWP.

Working assumption:

When A-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB.

Send LS to inform RAN2 working assumption to RAN4, and to check if ok to them.

Comeback in CB session (Apple, LS in R2-2503015)

FFS if we allow multiple OD-SSBs with the different frequencies for a given SCell.

[Ericsson]: Clarifies RAN1 decided both A-SSB and OD-SSB are located in the same BWP, that’s why we changed our position. [Nokia]: The requirement that one MO per carrier has been there from Rel-15. Afraid to change this principle based on Ericsson proposal. For Apple proposal, if multiple OD-SSBs are configured for a given SCell, how to handle it? [Apple]: Do not consider multiple OD-SSB for a given SCell. [OPPO]: Agree with Apple proposal. For multiple OD-SSB configurations, we can handle it by MO index in the corresponding configuration. [ZTE]: Multiple OD-SSB per SCell is possible, but not sure whether it is possible with the different frequencies. [Huawei]: Agree with Apple proposal. However, it will be good to remove “RedCap” since the detailed signalling can be different. [Ericsson]: Should RAN4 decide it? [Nokia]: Understand it is RAN2 job. Ok with the proposal, but wants to add “FFS whether/where multiple OD-SSBs are on the different frequencies for a given SCell.” [Apple, LG]: For Nokia’s FFS proposal, it is RAN1 scope. We do not need to create this explicit FFS in RAN2. If RAN1 introduces it, we can discuss it otherwise not needed. [Ericsson]: If we go Apple proposal, it may bring some issues when OD-SSB is activated, e.g. how to combine measurements, how to apply filtering from different MO, etc. Good to check with RAN4.

R2-2503015	LS on on-demand SSB	LSout	To: RAN4	Rel-19	Netw_Energy_NR_enh-Core

Add “A-SSB” to “always-on SSB” and “OD-SSB” to “on-demand SSB”.

Change “RAN4” to “RAN4/1” in Note.

Add RAN1 into Cc.

Will remove the sentence “That is, when A-SSB and OD-SSBs have different center frequencies, except the servingCellMO to indicate MO of A-SSB, RAN2 to introduce a new servingCellMOs in ServingCellConfig to indicate MOs of configured OD-SSBs.”

Change “frequency” to “frequencies” in the note.

With the changes above, LS is approved in R2-2503019.

[OPPO, Huawei]: Multiple OD-SSBs are not agreed. Would like to focus on single case now. [Ericsson]: Without adding FFS on multiple OD-SSBs, RAN4 may consider additional MO will handle all multiple OD-SSBs.

4. L3 RRM: Measurement on neighbouring cells

R2-2502384	Remaining issues of on-demand SSB SCell operation	vivo	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 4: No specification impact is needed for the RRM measurement on intra-frequency neighbour cells when the OD-SSB of SCell is activated.

Proposal 5: RAN2 assumes the L3 measurement of neighbour cell OD-SSB is based on legacy SMTC.

5. Need of MAC-CE triggering MR for OD-SSB

R2-2501886	Discussion on On-Demand SSB	OPPO	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 10: R2 not pursue MAC-CE triggered MR for OD-SSB.

R2-2502148	Discussion on on-demand SSB for NES	Ericsson	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 2: RAN2 to support MAC CE based trigger of serving cell OD-SSB measurement report.

Agreements on OD-SSB

- For explicit activation/deactivation, the OD-SSB MAC-CE includes fixed sized bitmap to indicate whether OD-SSB is activated in each SCell (i.e., similar to legacy SCell A/D MAC-CE).

- A/D bit: “1” means activation, “0” means deactivation for explicit deactivation case.

- For explicit activation/deactivation, the OD-SSB MAC-CE supports two formats: one format indicates up to 7 SCells and the other format indicates up to 31 SCells.

- OD-SSB MAC-CE includes a configuration index for each SCell activating OD-SSB.

- RAN2 aims to define single MAC CE format for both explicit and implicit OD-SSB deactivations. Details are FFS.

- Working assumption: When A-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB. FFS if we allow multiple OD-SSBs with the different frequencies for a given SCell.

R2-2501805	Discussion on on-demand SSB	Xiaomi	discussion

R2-2501833	Discussion on on-demand SSB	HONOR	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2501867	On-demand SSB SCell Operation	Samsung	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2501886	Discussion on On-Demand SSB	OPPO	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2501901	Consideration on on-demand SSB SCell operation	CATT	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2501957	Remaining issues of on demand SSB Scell operation	ZTE Corporation, Sanechips	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2502130	Remaining issues on on-demand SSB for Scell	Apple	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2502136	Discussion on OD-SSB	Rakuten Mobile, Inc	discussion	Rel-19

R2-2502148	Discussion on on-demand SSB for NES	Ericsson	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2502219	NES SCell handling for OD-SSB	ETRI	discussion

R2-2502225	Remaining issues on on-demand SSB SCell operation	Fujitsu	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2502320	Further discussion on On-demand SSB for SCell	NEC	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2502371	Issues on the procedure of on-demand SSB SCell operation	Lenovo	discussion	Rel-19

R2-2502384	Remaining issues of on-demand SSB SCell operation	vivo	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2502487	On-demand SSB Scell operation discussion	Sony	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2502540	Discussion on On-demand SSB SCell Operation	Qualcomm Incorporated	discussion

R2-2502582	On demand SSB transmission for SCell	InterDigital	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2502724	Discussion on On-demand SSB for Scell	CMCC	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2502742	On-demand SSB SCell operation	Panasonic	discussion	Rel-19

R2-2502780	Discussion on on-demand SSB SCell operation	NTT DOCOMO, INC.	discussion	Rel-19

R2-2502840	Discussion on on-demand SSB SCell operation for NES	Huawei, HiSilicon	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2502847	Discussion on on-demand SSB transmission	LG Electronics France	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2502864	Discussion on On-demand SSB	KDDI Corporation (TTC)	discussion

R2-2502907	Discussion on on-demand SSB SCell operation	Sharp	discussion

R2-2502915	On demand SSB handling	Nokia, Nokia Shanghai Bell	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2502002	Discussion on on-demand SSB SCell operation	Sharp	discussion

=> Withdrawn

### 8.5.3	On-demand SIB1

Remaining open issues, including the unsettled issue for RRC connected UEs, details of OD-SIB1 request procedure, stage-3 identified open issues if needed, etc.

1. Handling RRC connected UE

R2-2502131	Remaining issues on on-demand SIB1	Apple	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 1: For Rel-19 NES UE in RRC_CONNECTED, if searchSpaceSIB1 is configured and SIB1 is not broadcasting:

•	Rely on the NW implementation (e.g. dedicated RRC message to deliver SIB1 or not configure searchSpaceSIB1) as baseline.

•	If majority prefer enhancement, allow the UE to send request for SIB1 via dedicated RRC message DedicatedSIBRequest.

NW ensures that the RRC connected UE has the latest SIB1 (e.g. dedicated RRC message to deliver SIB1 or not configure searchSpaceSIB1), as baseline. UE understands that the stored SIB1 is the latest SIB1.

[ZTE]: Ok with relying on NW implementation. Understand OD-SIB1 request is only applicable to RLF case according to RAN guideline. Also think searchSpaceSIB1 is configured and sib1 is not broadcasting is not often. [Huawei]: With relying on NW implementation, how does NW expect the UE behaviour? [Session chair]: Understand relying on NW implementation means whenever broadcast status is changed, NW sends SIB1 in dedicated RRC. [Nokia]: Do not understand what NW difficulties that Apple assume if OD-SIB1 request procedure is applied (like idle/inactive UE). [Samsung]: With the proposal, it would bring burden to NW and it may not be desirable for NES point of view. For dedicatedSIBRequest, we can just add one code-point. [CATT]: Understand the first option works in NES cell assuming there are not many connected UEs. [Samsung]: Heard some technical concerns for the first option, however for the second option, no technical concern. [LG, ZTE]: With option2, some companies still have concerns that it is not aligned with RAN discussion and guideline. [Nokia]: Agree with ZTE, we can first agree with option1 then if proponent companies want other option that may not be aligned with RAN conclusion, they need to bring it to RAN.

2. Whether legacy UE can camp in NES Cell if it is broadcasting SIB1

R2-2501866	On-demand SIB1	Samsung	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 5: When the cell supporting on demand SIB1 is broadcasting SIB1 (e.g. upon SIB1 request), legacy UE can camp on the cell if the legacy UE is able to acquire the broadcasted SIB1.

When the cell supporting on demand SIB1 is broadcasting SIB1 (e.g. upon SIB1 request), legacy UE can camp on the cell if the legacy UE is able to acquire the broadcasted SIB1. No specification impact is foreseen.

[Ericsson]: Isn’t it dependent on Kssb configuration, which is up to NW implementation. [ZTE]: We can confirm proposal 5 but do not see any specification impact. [CATT]: Agree with Ericsson and there is no specification impact.

3. UL WUS in RRC release

R2-2502580	On-demand SIB1 request and reception	InterDigital	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 1: Confirm the working assumption that “UL-WUS configuration in RRC release is not supported”.

Confirm the working assumption that “UL-WUS configuration in RRC release is not supported”.

SIBX that was acquired during RRC connected state can be used for OD-SIB1 request in RLF.

[Nokia]: If SIBX that was acquired during RRC connected state can be used for OD-SIB1 REQ in RLF, we are ok with the proposal. [Samsung]: Do not see any difficult to maintain SIBX that was required for RRC connected state and use it in RLF. Agree with the proposal and Nokia’s comment.

4. Understanding of RAR format

R2-2502916	On demand SIB1 handling	Nokia, Nokia Shanghai Bell	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 3: RAN2 to align with legacy RAR for OSI for OD-SIB1 operation. Legacy RAR MAC PDU subheader with RAPID only to be used as NW acknowledgement for OD-SIB1 request.

Align with legacy RAR for OSI for OD-SIB1 operation. Legacy RAR MAC PDU subheader with RAPID only to be used as NW acknowledgement for OD-SIB1 request.

R2-2502385	Discussion on OD-SIB1 for RRC IDLE and INACTIVE UE	vivo	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 4: RAN2 to discuss the following options for a Rel-19 NES UE to correctly decode the RAR MAC PDU containing the MAC subPDU for the OD-SIB1 request upon cell reselection to a NES cell:

-	Option 1: The UL WUS configuration includes the IE totalNumberOfRA-Preambles;

-	Option 2: For the MAC subPDU with RAPID not used for the OD-SIB1 request, R19 NES UE considers the size of the MAC subPDU with RAPID < ra-PreambleStartIndex for the OD-SIB1 request is 8 bytes and the size of the MAC subPDU with other RAPID is 1 byte.

-	Option 3: The MAC subPDU for the OD-SIB1 request is placed at the second place after the MAC subPDU with BI only.

The UL WUS configuration includes the IE totalNumberOfRA-Preambles.

[Apple]: Option1 is the easiest option and all other options need a new UE behaviour.

5. Backoff in RAR

R2-2502226	Remaining issues on on-demand SIB1 procedure	Fujitsu	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 7: No need to change the preamble backoff handling in the RAR MAC PDU during SIB1 request procedure.

Backoff is not applied to OD-SIB1 request when backoff is included in RAR.

[Samsung, Ericsson, Nokia]: Do not see a need to use backoff considering SIB1 is essential SIB.

6. Paging interruption

R2-2502131	Remaining issues on on-demand SIB1	Apple	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 3: On paging interruption issue during OD-SIB1 acquisition, RAN2 discuss the following 2 solutions:

•	Alt-1: leave it to UE implementation. RAN2 send LS to RAN4 to check whether the paging interruption requirement during cell reselection specified in TS 38.133 needs change.

•	Alt-2: include PCCH-Config of NES cell in UL WUS configuration provided by Cell A, so that Rel-19 UEs can monitor paging from NES cell during OD-SIB1 acquisition.

R2-2502229	Remaining issues on OD-SIB1	LG Electronics Inc.	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 2: If the RAR window or the OD-SIB1 window is overlapped with the paging occasion, the UE monitors the serving cell during the overlapping period.

[Nokia]: Do not see a need of LS. Interested companies can submit the corresponding contribution directly in RAN4. [ZTE]: Agree with alternative 1 and we can reword that RAN2 understand it’s up to RAN4 for any update of paging interruption requirements. [Vivo, OPPO]: Understand RAN4 already started this discussion, so LS is not needed. [Lenovo]: Agree with alternative 1.

Leave this issue (how to handle paging interruption during OD-SIB1 acquisition) to RAN4.

7. Condition to bar the cell due to SIB1 acquisition failure

R2-2502104	Remaining issues on the support for OD-SIB1	Google	discussion	Rel-19	Netw_Energy_NR_enh-Core	R2-2500463

Proposal 4 (modified): If UE has not received the PDCCH scheduling SIB1 upon the expiry of the SIB1 monitoring window, UE may consider the cell as being barred.

If UE has not received the PDCCH scheduling SIB1 upon the expiry of the SIB1 monitoring window, UE may consider the cell as being barred.

[Nokia, Xiaomi]: Think we can leave it to UE implementation. [LG, Samsung]: Support the proposal. [Samsung]: If the UE fails to acquire SIB1 within SIB1 monitoring window, can the UE perform RACH procedure again to acquire OD-SIB1? [ZTE, OPPO]: Want to have a consistent UE behaviour. Since we already agreed to bar the cell when RACH for OD-SIB1 fails, it should ok with the proposal. [Nokia]: Think 300s restriction is too harsh in the case (including RACH failure for OD-SIB1 acquisition). Prefer up to 300s (not as fixed 300s). [Ericsson]: For barring, some case is specified as “shall” and some other case is specified as “may”. We may need to consider consistent behaviour on this wording as well. [Session chair]: For 300s restriction and wording difference (may and shall), suggest interested companies have offline, so we can check in CB session.

CB session: 
It was covered by Apple’s earlier offline discussion.

8. Area validity

R2-2501958	Remaining issues of on demand SIB1	ZTE Corporation, Sanechips	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 4: Define and provide valid area for WUS configuration.

Proposal 5: The valid area for WUS configuration can be a list of cell identities or a new system information area id referring to multiple cells.

WUS configuration can be associated with a list of cells if the whole WUS configuration is same.

Area id will be used as legacy.

[Huawei]: We need to discuss area for WUS and list of cells for WUS in separate. If we introduce new area for SIBx or each WUS, it will bring additional bits for the new area id. [Ericsson]: Agree with Huawei. Note area and area id concept is the legacy mechanism. [Nokia]: Wonder if same WUS configuration used for multiple cells is realistic. [Huawei]: Understand similar discussion happened in RAN1, and it was not forbidden.

9. Access related information in UL WUS

R2-2502541	Discussion on On-demand SIB1	Qualcomm Incorporated	discussion

Proposal 3: RAN2 discusses the indication of feature-specific cell barring for a NES cell with on-demand SIB1.

R2-2502916	On demand SIB1 handling	Nokia, Nokia Shanghai Bell	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 7: Cell access related information should be included in the the UL WUS configuration to avoid unnecessary SIB1 requests on a cell where UE does not have access.

[Ericsson, ZTE, NTTDCM]: Prefer not to have this optimization. WUS configuration can be more updated often than access related information in SIB1. [Huawei, Apple, Google, Vivo, Xiaomi, Samsung]: Support the proposal. [Huawei]: Assume no big additional signaling overhead to include access related information. [Lenovo]: Additional information is not small including PLMN id, TAC, and other access related information. [Nokia]: This information will be defined as optional, so if NW does not want it, it can be omitted (NW choice). [Vivo]: Wonder if we need all information. [Ericsson]: If we put this information, it will make a list of cells associated with WUS configuration more difficult. [CMCC]: RAN1 already decided not to include barring information into WUS. [Samsung]: Barring is RAN2 scope. [Huawei]: We can only consider feature-specific cell barring in WUS since it needs only a few bits. [Nokia]: It does not make sense. Should be common to all access related information.

Offline among interested companies. Comeback in CB session (Nokia).

[Comeback session]:

[Nokia]: Understand companies are not strongly against making this information optional. However there are more details we need to consider for determination. [Lenovo]: NPN, slice, blabla.. so many IEs are related to access related information. Understand it is not so simple. [Nokia]: Understand slice does not impact on access determination. [Lenovo, Ericsson]: Have different view than Nokia.

Revisit it next meeting.

10. Others

R2-2502131	Remaining issues on on-demand SIB1	Apple	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 8: Trigger condition of OD-SIB1 acquisition (i.e. reusing cell reselection criteria) is different from legacy. So, RAN2 capture it in both TS 38.331 and TS 38.304.

We do not need a separate new triggering condition of OD-SIB1 acquisition.

R2-2502226	Remaining issues on on-demand SIB1 procedure	Fujitsu	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 4:	When timer T311 is running, the cell selection criterion S is evaluated by the existing cell reselection parameters provided from Cell A.

When timer T311 is running, SIB1 acquisition triggering condition is same as legacy. No additional spec impact is foreseen.

Agreements on OD-SIB1

- NW ensures that the RRC connected UE has the latest SIB1 (e.g. dedicated RRC message to deliver SIB1 or not configure searchSpaceSIB1), as baseline. UE understands that the stored SIB1 is the latest SIB1.

- When the cell supporting on demand SIB1 is broadcasting SIB1 (e.g. upon SIB1 request), legacy UE can camp on the cell if the legacy UE is able to acquire the broadcasted SIB1. No specification impact is foreseen.

- Confirm the working assumption that “UL-WUS configuration in RRC release is not supported”.

- SIBX that was acquired during RRC connected state can be used for OD-SIB1 request in RLF.

- Align with legacy RAR for OSI for OD-SIB1 operation. Legacy RAR MAC PDU subheader with RAPID only to be used as NW acknowledgement for OD-SIB1 request.

- The UL WUS configuration includes the IE totalNumberOfRA-Preambles.

- Backoff is not applied to OD-SIB1 request when backoff is included in RAR.

- Leave this issue (how to handle paging interruption during OD-SIB1 acquisition) to RAN4.

- If UE has not received the PDCCH scheduling SIB1 upon the expiry of the SIB1 monitoring window, UE may consider the cell as being barred.

- WUS configuration can be associated with a list of cells if the whole WUS configuration is same.

- Area id will be used as legacy.

- We do not need a separate new triggering condition of OD-SIB1 acquisition.

- When timer T311 is running, SIB1 acquisition triggering condition is same as legacy. No additional spec impact is foreseen.

R2-2501795	Discussion on on-demand SIB1	Xiaomi	discussion	Rel-19

R2-2501834	Discussion on on-demand SIB1	HONOR	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2501864	Remaining issues for On-demand SIB1 request and UE behaviour	NEC	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2501866	On-demand SIB1	Samsung	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2501902	Consideration on on-demand SIB1	CATT	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2501958	Remaining issues of on demand SIB1	ZTE Corporation, Sanechips	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2501982	Discussion on remaining issues of on-demand SIB1 operation for NES	Huawei, HiSilicon	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2502104	Remaining issues on the support for OD-SIB1	Google	discussion	Rel-19	Netw_Energy_NR_enh-Core	R2-2500463

R2-2502131	Remaining issues on on-demand SIB1	Apple	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2502134	Discussion on on-demand SIB1 for NES	Rakuten Mobile, Inc	discussion	Rel-19

R2-2502149	Discussion on on-demand SIB1 for NES	Ericsson	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2502226	Remaining issues on on-demand SIB1 procedure	Fujitsu	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2502229	Remaining issues on OD-SIB1	LG Electronics Inc.	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2502261	Discussion on Ondemand-SIB1	KDDI Corporation (TTC)	discussion	Rel-19

R2-2502385	Discussion on OD-SIB1 for RRC IDLE and INACTIVE UE	vivo	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2502400	Discussion on on-demand SIB1 for handling RLF	ITRI	discussion	Netw_Energy_NR_enh-Core

R2-2502488	Remaining details for on-demand SIB1	Sony	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2502541	Discussion on On-demand SIB1	Qualcomm Incorporated	discussion

R2-2502577	SIB1 acquisition for Rel-19 NES UE in RRC_CONNECTED state	Fainity Innovation	discussion

R2-2502580	On-demand SIB1 request and reception	InterDigital	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2502596	Remaining OD SIB1 issues	Lenovo	discussion

R2-2502725	Discussion on On-demand SIB1	CMCC	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2502761	Consideration on on-demand SIB1	OPPO	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2502781	Discussion on on-demand SIB1	NTT DOCOMO, INC.	discussion	Rel-19

R2-2502876	Discussion on remaining issues of on-demand SIB1	SHARP Corporation	discussion	Rel-19

R2-2502900	Discussion on On-demand SIB1 for NES	Fraunhofer IIS, Fraunhofer HHI	discussion	Rel-19

R2-2502916	On demand SIB1 handling	Nokia, Nokia Shanghai Bell	discussion	Rel-19	Netw_Energy_NR_enh-Core

### 8.5.4	Adaptation of common signal/channel transmissions

Remaining open issues, including RAN2 spec impacts and discussion with consideration of RAN1 progress, stage-3 identified open issues if needed, etc.

1. Paging adaptation: NES UE’s PEI monitoring

R2-2501959	Remaining issues of common signal/channel transmissions	ZTE Corporation, Sanechips	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 2a: For the case when both pei-Config-r17 and pagingAdaptationPEI-Config-r19 are configured, R19 UE supporting paging adaption should monitor PEI according to pagingAdaptationPEI-Config-r19 while other UE should monitor PEI according to pei-Config-r17.

Proposal 2b: For the case when pei-Config-r17 is configured and pagingAdaptationPEI-Config-r19 is absent, both R19 UE supporting paging adaption and other UE should monitor PEI according to pei-Config-r17.

R2-2502917	Adaptation of common signals	Nokia, Nokia Shanghai Bell	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 15: A single PEI with DCI format 2_7 can address both legacy POs and additional release 19 POs using a mixture of multiplexing and separate signalling.

Proposal 16: A separate PEI configuration can be used for release 19 UEs to define how the release 19 POs are mapped to the same DCI format 2_7 used by legacy UEs for PEI.

R2-2502841	Discussion on adaptation of common signals/channels transmissions	Huawei, HiSilicon	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 1: (related to 38.331 FFS) A new firstPDCCH-MonitoringOccasionOfPO-r19 is added, value range is extended to “(0..143359)” to accommodate SCS=480kHz and N=T/32. RAN2 to down-select from the following options:

•	Option 1) R19 NES UEs use firstPMO-r19 for all POs in the PF

•	Option 2) R19 NES UEs use legacy PMO for POs that overlap with legacy POs, use firstPMO-r19 for additional POs not overlapping with legacy Pos

[LG]: NES UE should determine based on Rel-19 configuration. Huawei proposal sounds very complicated. [Apple, OPPO, Samsung, Vivo]: Support ZTE proposal. Nokia’s proposal will impact RAN1 and we do not prefer it now. RAN1 almost finalizes WI completion. [Huawei]: Do not think our proposal is controversial to RAN2 previous agreements. [Vivo]: Agree with ZTE’s first proposal. Ok with the second proposal (after some discussion).

For the case when both pei-Config-r17 and pagingAdaptationPEI-Config-r19 are configured, R19 UE supporting paging adaption should monitor PEI according to pagingAdaptationPEI-Config-r19 while other UE should monitor PEI according to pei-Config-r17.

For the case when pei-Config-r17 is configured and pagingAdaptationPEI-Config-r19 is absent, both R19 UE supporting paging adaption and other UE should monitor PEI according to pei-Config-r17.

2. Paging adaptation: RRC connected UE

R2-2501868	Adaptation of common signal channel transmissions	Samsung	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 2: Paging clustering/bundling/adaptation is not supported/applied in RRC_CONNECTED

[Nokia]: What is specification impact? We can leave it to UE implementation (e.g. RRC connected UE can monitor either R18 or R19 PO). [OPPO, Xiaomi]: Agree with Nokia. [Ericsson, Lenovo]: Agree with proposal. [Samsung]: Would like to have time to explain the proposal to the companies.

Paging clustering/bundling/adaptation is not supported/applied in RRC_CONNECTED.

3. Paging adaptation: UE capability

R2-2502386	Discussion on adaptation of common signal transmissions	vivo	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 2: No need to define a common capability for all Rel-19 NES features.

4. Additional RO configuration

R2-2501796	Discussion on common signal adaptation	Xiaomi	discussion	Rel-19

Proposal 3: The additional PRACH configuration is configured in SIB1.

R2-2502841	Discussion on adaptation of common signals/channels transmissions	Huawei, HiSilicon	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 4: New RACH-ConfigCommon is added in BWP-ConfigCommon for R19 NES UEs.

R2-2501868	Adaptation of common signal channel transmissions	Samsung	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 3: Introduce prach-ConfigurationIndex and msg1-FrequencyStart for additional rach resouces in RACH-ConfigGeneric IE.

5. RA-RNTI Calculation

R2-2502132	Remaining issues on common signal transmission adaptation	Apple	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 8: RAN2 discuss RA-RNTI ambiguity issue, and consider the following alternatives:

•	Alt 1: Introduce a fixed k_offset for the additional PRACH resources, i.e. RA-RNTI = 1 + s_id + 14 x t_id + 14 x 80 x f_id + 14 x 80 x8 x ul_carrier_id + 14 x 80x 8 x 2 x k_offset (k_offset>1)

•	Alt 2: f_id for the additional PRACH resources is reinterpreted as the index of the PRACH occasion in the frequency domain plus the number of FDMed legacy ROs.

6. RACH Adaptation for CFRA

R2-2501817	Discussion on adaptation of common signal channel transmission	OPPO	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 1: R2 confirms time-domain RACH adaptation is supported for CFRA initiated by normal PDCCH order, but not for other CFRA cases. Send LS to R1 if any concern.

[Nokia]: Any technical reason not to apply PDCCH order based CFRA RACH adaptation to other use case? [OPPO]: For example, BFR, RACH configuration is presented as mandatory configuration. And if needed, NW can put additional RACH RO into this configuration. [Samsung, Apple]: Agree with OPPO proposal. And note RAN1 decided that additional RO is only applicable to initial BWP. [Huawei]: Understand RAN1 introduced PDCCH order can be applicable to CBRA also. [Apple]: Understand there is no restriction that PDCCH order is applied to CBRA. [Ericsson]: Not sure if new mechanism for additional RO cannot be used for other cases. [Nokia]: Understand the additional RO activation/deactivation still can be applicable e.g. into BFR case. [OPPO]: Understand without additional RO activation/deactivation, if network wants, the network just includes additional RO into BFR configuration, then the UE uses it unless it is reconfigured or released. [Nokia]: Can we try to agree no additional mechanism is needed for BFR case in order to use additional RO. [Samsung]: For any kind of HO mechanisms, RACH generic is optional and if network wants to use additional RACH RO, network can configure the additional RACH RO to RACH generic configuration. Dynamic activation/deactivation is based on the serving cell’s short message. It seems clear no need of dynamic activation/deactivation to the non-serving cell(s). [Spreadtrum]: Wonder how it works if multiple initial BWPs are configured. [OPPO]: Propose to have post email discussion on this issue.

[POST129b][122][NES] (OPPO)

Scope: Discuss and make conclusions on proposal 1 in R2-2501817.

Intended outcome: Discussion summary.

Deadline: Long email discussion.

7. Handling of RRC Connected UE

R2-2501868	Adaptation of common signal channel transmissions	Samsung	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 5: RAN2 to discuss and agree one of the following for RRC_CONNECTED

Option 1: if adaptation for additional PRACH resources is supported in active UL BWP, network ensures that paging search space is configured in active DL BWP.

Option 2: Introduce other mechanism (such RRC Reconfiguration message or MAC CE or group DCI) for adaptation for additional PRACH resources in RRC_CONNECTED.

Proposal 6: RAN2 to discuss and agree one of the following for RRC_CONNECTED

Option 1: For DCI-based adaptation indication for additional PRACH resources in RRC_CONNECTED, UE monitors the indication in any paging occasion at least once every defaultPagingCycle.

Option 2: For DCI-based adaptation for additional PRACH resources in RRC_CONNECTED, UE monitors the indication in any paging occasion at least once every modification period.

8. Need of MAC CE based signalling

R2-2502782	Discussion on adaptation of common signal and channel	NTT DOCOMO, INC.	discussion	Rel-19

Proposal 1: Not support MAC CE based signalling to indicate SSB adaptation in addition to DCI agreed in RAN1.

R2-2501865	PRACH, paging and SSB adaptation for NES	NEC	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 5: RAN2 should adopt a unified MAC CE design for signaling SSB adaptation and OD-SSB to optimize efficiency and streamline standardization.

[Session chair]: How many companies object to introduce MAC-CE based signalling solution in addition to DCI? NTTDCM, CATT, Fujitsu, LG, Apple, Xiaomi, IDC, Huawei, Samsung, Ericsson, KDDI. [Session chair]: Considering we already have one mechanism based on DCI, suggest to agree with NTTDCM proposal.

Not support MAC CE based signalling to indicate SSB adaptation in addition to DCI agreed in RAN1.

9. Signalling for SMTC adaptation

R2-2501903	Adaptation of Common signal channel transmissions	CATT	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 6: For SSB adaptation, multiple additional SMTC configurations are configured to UE via RRC, together with the mapping between SSB burst periodicity and SMTC configuration.

Proposal 7: Upon reception of DCI format 2_9 for SSB burst periodicity switching, UE selects one SMTC configuration accordingly.

For SSB adaptation, multiple additional SMTC configurations are configured to UE via RRC, together with the mapping between SSB burst periodicity and SMTC configuration.

Upon reception of DCI format 2_9 for SSB burst periodicity switching, UE selects one SMTC configuration accordingly.

Agreements on common channel adaptation

- For the case when both pei-Config-r17 and pagingAdaptationPEI-Config-r19 are configured, R19 UE supporting paging adaption should monitor PEI according to pagingAdaptationPEI-Config-r19 while other UE should monitor PEI according to pei-Config-r17.

- For the case when pei-Config-r17 is configured and pagingAdaptationPEI-Config-r19 is absent, both R19 UE supporting paging adaption and other UE should monitor PEI according to pei-Config-r17.

- Paging clustering/bundling/adaptation is not supported/applied in RRC_CONNECTED.

- Not support MAC CE based signalling to indicate SSB adaptation in addition to DCI agreed in RAN1.

- For SSB adaptation, multiple additional SMTC configurations are configured to UE via RRC, together with the mapping between SSB burst periodicity and SMTC configuration.

- Upon reception of DCI format 2_9 for SSB burst periodicity switching, UE selects one SMTC configuration accordingly.

R2-2501796	Discussion on common signal adaptation	Xiaomi	discussion	Rel-19

R2-2501817	Discussion on adaptation of common signal channel transmission	OPPO	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2501865	PRACH, paging and SSB adaptation for NES	NEC	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2501868	Adaptation of common signal channel transmissions	Samsung	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2501903	Adaptation of Common signal channel transmissions	CATT	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2501959	Remaining issues of common signal/channel transmissions	ZTE Corporation, Sanechips	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2502031	Adaptation of common signal or channel	Fujitsu	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2502132	Remaining issues on common signal transmission adaptation	Apple	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2502372	Paging and PRACH adaptation	Lenovo	discussion	Rel-19

R2-2502386	Discussion on adaptation of common signal transmissions	vivo	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2502489	RAN2 impacts on SSB and RACH adaptations	Sony	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2502542	Discussion on Adaptation of Common Signal/Channel Transmissions	Qualcomm Incorporated	discussion

R2-2502581	Time domain adaptation of common signalling and channels	InterDigital	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2502726	Discussion on Common signalling adaptation	CMCC	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2502766	Adaptation of common signal/channel transmissions for NES	Ericsson	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2502782	Discussion on adaptation of common signal and channel	NTT DOCOMO, INC.	discussion	Rel-19

R2-2502841	Discussion on adaptation of common signals/channels transmissions	Huawei, HiSilicon	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2502917	Adaptation of common signals	Nokia, Nokia Shanghai Bell	discussion	Rel-19	Netw_Energy_NR_enh-Core

## 8.6	Mobility Enhancement Ph4

(NR_Mob_Ph4-Core; leading WG: RAN2; REL-19; WID: RP-242356)

Time budget: 2 TU

Tdoc Limitation: 3 tdocs

### 8.6.1	Organizational

Incoming LS, WI rapporteur inputs, CR rapporteur inputs (including post email discussion [POST129][106], [107], [108], [109], [110], [111], summary of identified open issues that need online discussion and rapporteur’s suggestions if needed).

R2-2501709	LS on activation/deactivation of semi-persistent CSI-RS resource for LTM candidate cells (R1- 2501500; contact: Fujitsu)	RAN1	LS in	Rel-19	NR_Mob_Ph4-Core	To:RAN2	Cc:RAN3

Noted.

[Qualcomm]: Not sure if semi-persistent CSI-RS is also applied to event-triggered MR. [Apple]: It is about semi-persistent CSI-RS activation/deactivation, RAN2 does not need to discuss how to use it. Understand there is no restriction to NW. [Samsung, Huawei]: Understand it can be used for both periodic and event-triggered MR. [ZTE]: Wonder if SP CSI-RS can be used for early DL/UL synchronization. [Apple]: Yes, up to NW.

R2-2501715	LS on L1filtering for LTM event triggered reporting (R1-2501577; contact: Fujitsu)	RAN1	LS in	Rel-19	NR_Mob_Ph4-Core	To:RAN2

Noted.

[Ericsson, Nokia]: Wonder if we need any MAC-based filtering. [Apple]: RAN1 clearly clarified that L1 filtering is up to UE implementation. [Vivo]: Agree with Apple. We can capture it in stage 2 spec, and we do not need any kind of L2 filtering. [LG]: Agree with Ericsson and Nokia. [ZTE]: We should not discuss this in RAN2 since it was already discussed and concluded in RAN1 and it is more complicated to RAN2. Also consider there will be minimum RAN4 performance requirements. [Apple]: Running CR already capture this RAN1 LS correctly.

No L1/L2 filtering issue will be discussed in RAN2.

R2-2501756	Reply LS on security handling for inter-CU LTM in non-DC cases (S3-251124; contact: CATT)	SA3	LS in	Rel-19	NR_Mob_Ph4-Core	To:RAN2, RAN3

Noted.

[OPPO]: Wonder if it is also applicable to DC case. Do need to ask it to SA3? [Apple]: No. Once we ask it to SA3, it will take more than 2-3 meeting cycles, which is not acceptable. For NCC, understand it is related to MCG only.

R2-2502198	Observations on some topics related to Rel-19 Mobility Enh WI	Apple	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 1: RAN2 to design MAC CE based NCC transfer and can evaluate the option of indexing the NCC (instead of sending the actual NCC value), but limit the discussion scope to only this, to ensure that the solution is simple.

Proposal 2: Inform RAN3 of this so that RAN3 discussions would be scoped better and practical.

Proposal 3: For Rel-19 WIs which are enhancements (for LTM, NES, MIMO) RAN2 to discuss the inter-working of Rel-19 LTM with not only Rel-19 but also with earlier versions of WIs, if the previous release WI inter-working is agreed to be not prohibited.

Proposal 4: RAN2 to agree on inter-operability of WIs with Rel-19 LTM using the below driving guidelines:

1.	If the inter-working works without any changes or only depends on NW ensuring consistent configuration to the UE and;

2.	If the inter-working does NOT need any standardized addition of interfaces or parameters to the existing interfaces arcoss NW nodes and between UE and the NW.

Proposal 5: When discussing inter-working of Rel-19 LTM with other WIs, RAN2 should evaluate inter-CU LTM inter-working seperately from CLTM inter-working.

Noted.

[Ericsson]: For inter-working, we should not start this discussion until the whole Rel-19 LTM becomes mature enough. [ZTE]: At least Rel-18 MIMO need to be discussed this meeting.

R2-2502197	Report of [POST129][106][Mob] Stage-2 CR 38.300 (Apple)	Apple	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 1: RAN2 to discuss and conclude that the new sequence diagram from draftCR R2-2502196 replaces the Figure 9.2.3.5.2-1 from existing TS38.300.

Proposal 2: Endorse the draft CR from R2-2502196.

[Apple]: Propose to have more time for offline discussion. [Nokia]: Prefer to add a figure for Rel-19 LTM in addition to the existing Rel-18 LTM.

Comeback in CB session.

R2-2502196	Introduction of NR mobility enhancements Phase 4 in TS 38.300	Apple	draftCR	Rel-19	38.300	18.5.0	B	NR_Mob_Ph4-Core

[Session chair]: Two updated figures 9.2.3.5.2.-1. [Apple]: It is editorial error.

Comeback in CB session.

R2-2503154	Introduction of NR mobility enhancements Phase 4 in TS 38.300	Apple	draftCR	Rel-19	38.300	18.5.0	B	NR_Mob_Ph4-Core

Endorsed.

R2-2502234	Introduction of NR mobility enhancements Phase 4 in TS 37.340	China Telecom	draftCR	Rel-19	37.340	18.5.0	B	NR_Mob_Ph4-Core

Endorsed.

R2-2502534	Running RRC CR for inter-CU and conditional LTM	Ericsson	draftCR	Rel-19	38.331	18.5.1	B	NR_Mob_Ph4-Core

Endorsed.

R2-2502535	List of FFSs for LTM and CLTM	Ericsson	discussion	Rel-19	NR_Mob_Ph4-Core

[AT129b][102][MOB] (Ericsson)

Scope: Discuss the identified RRC FFSs (offline discussion rapporteur can pick up which FFSs) in R2-2502535 (offline discussion rapporteur can pick up company’s proposal if needed) and make conclusions as much as possible. Note discussion should not be overlapped with the identified issues that are already included in the agenda.

Intended outcome: Discussion summary in R2-2503001.

Deadline: Comeback in CB session => Completed.

R2-2503001	Summary of offlines [AT129b][102][MOB] and [AT129b][110][MOB]	Ericsson	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 1: A list of sk-counters is linked to a Rel-19 ID configured by the SN.

Proposal 2 (modified): At RLF and reconfiguration with sync failure:

a.	If RLF:

i.	If the selected candidate cell has the same Rel-19 ID as source (no security key change), the UE performs fast RLF recovery (same as in Rel-18).

b.	if reconfiguration failure (inter-CU LTM)

i.	If the selected candidate cell has the same Rel-19 ID as target, the UE performs fast failure recovery. FFS if fast failure recovery with different Rel-19 IDs is allowed.

Proposal 3: How to indicate whether an LTM candidate configuration is a complete configuration is the same as Rel-18 (no different indication for MCG and SCG part of the candidate configuration).

Proposal 4: The indication on whether to allow or not the SN to configure an inter-SN candidate is included in the inter-node RRC message. We include this agreement in the LS to RAN3.

Proposal 5: Network can send an LTM Cell Switch Command MAC CE indicating a CLTM candidate configuration (no specification change).

A list of sk-counters is linked to a Rel-19 ID configured by the SN.

At RLF and reconfiguration with sync failure:

- If RLF: If the selected candidate cell has the same Rel-19 ID as source (no security key change), the UE performs fast RLF recovery (same as in Rel-18).

- if reconfiguration failure (inter-CU LTM): If the selected candidate cell has the same Rel-19 ID as target, the UE performs fast failure recovery. FFS if fast failure recovery with different Rel-19 IDs is allowed.

How to indicate whether an LTM candidate configuration is a complete configuration is the same as Rel-18 (no different indication for MCG and SCG part of the candidate configuration).

The indication on whether to allow or not the SN to configure an inter-SN candidate is included in the inter-node RRC message. We include this agreement in the LS to RAN3.

Network can send an LTM Cell Switch Command MAC CE indicating a CLTM candidate configuration (no specification change).

[Fujitsu]: Wonder if the UE revert old configuration back in proposal 2.b? [Ericsson]: We need to further define the exact UE behaviours. [Fujitsu]: If the UE reverts old configuration back, proposal does not make sense. [Session chair]: Seems it is not clear on that, with this uncertainty, can we still agree with proposals? [Ericsson]: Confirmed yes.

R2-2502095	Summary of [POST128][108][MOB] RRC running CR (Huawei)	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal1: Discuss whether ltm-CSI-RS-ReportConfigToAddModList should only be configured on SpCell.

Will be discussed as part of offline discussion [103].

Proposal2: Add additional candidate values {20ms, 40ms, 60ms} for report interval for event-triggered periodic report for LTM.

[MediaTek, Ericsson]: Prefer to restrict up to two additional values otherwise we need to extend the whole IE.

We will aim the additional values up to two values.

Which values will be further discussed as part of offline discussion [103].

R2-2502096	Draft running CR for event-triggered meas report for RRC	Huawei, HiSilicon	draftCR	Rel-19	38.331	18.5.1	B	NR_Mob_Ph4-Core

Endorsed.

[AT129b][103][MOB] (Huawei)

Scope: Discuss the identified RRC FFSs (offline discussion rapporteur can pick up which FFSs) in R2-2502096 (offline discussion rapporteur can pick up company’s proposal if needed) and make conclusions as much as possible. Note discussion should not be overlapped with the identified issues that are already included in the agenda.

Intended outcome: Discussion summary in R2-2503002.

Deadline: Comeback in CB session => Completed.

R2-2503002	Summary of [AT129b][103][MOB] RRC running CR (Huawei)	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal2: Select two values of 20ms and 60ms for the additional values

Select two values of 20ms and 60ms for the additional values.

R2-2502158	Running MAC CR for Mob Ph4	vivo (Rapporteur)	draftCR	Rel-19	38.321	18.5.0	B	NR_Mob_Ph4-Core

Noted.

[Ericsson]: Think it is still premature to be endorsed.

[AT129b][104][MOB] (Vivo)

Scope: Discuss the identified FFSs (offline discussion rapporteur can pick up which FFSs) in R2-2502158 (offline discussion rapporteur can pick up company’s proposal if needed) and make conclusions as much as possible. Note discussion should not be overlapped with the identified issues that are already included in the agenda.

Intended outcome: Discussion summary in R2-2503003.

Deadline: Comeback in CB session => Completed.

R2-2503003	Discussion report on [AT129bis][104][MOB] MAC open issues (vivo)	Vivo	discussion	Rel-19	NR_Mob_Ph4-Core

-	Introducing a new R19 LTM cell switch command MAC CE with indicating the NCC value for R19 inter-CU LTM cell switch. Details will be further discussed during CR phase.

-	For differential L1-RSRP reporting, the best quality beam among the beams included in L1 MR MAC CE is taken as the reference beam as the first one. The differential L1-RSRP value is derived based on the absolute L1-RSRP of the reference beam. FFS for truncated MAC CE.

-	(modified) When UL grant is enough to accommodate only limited number of beams in MR MAC CE, the triggered beam should be included in the truncated MR MAC CE.

-	When current beam is configured to be included in the L1 MR MAC CE, 7 bits of RSRP is included and no RSRI is needed.

-	The current beam information of SpCell (if configured to be included) is always placed at the end of the normal (not truncated) MAC CE.

-	Same as Rel-18: When the C-LTM execution is triggered, if the TA of the target cell acquired by early RACH is valid, UE applies the TA acquired by early RACH, otherwise if UE has successfully measured the TA of the target cell, UE applies the measured TA.

-	FFS For MR triggered by LTM2, whether only include the current beam information in the MR MAC CE or the MR can include measurements for LTM candidates.

[Ericsson]: For truncated MR MAC CE, the reported beam order will be changed? [Apple]: For truncated MR MAC CE, the event triggered beam should be included first, then if more beams are included, differential measured results should be based on the first one. [Vivo]: First here means with the first priority (it does not propose exact order in all cases).

Introducing a new R19 LTM cell switch command MAC CE with indicating the NCC value for R19 inter-CU LTM cell switch. Details will be further discussed during CR phase.

For differential L1-RSRP reporting, the best quality beam among the beams included in L1 MR MAC CE is taken as the reference beam as the first one. The differential L1-RSRP value is derived based on the absolute L1-RSRP of the reference beam. FFS for truncated MAC CE.

When UL grant is enough to accommodate only limited number of beams in MR MAC CE, the triggered beam should be included in the truncated MR MAC CE.

When current beam is configured to be included in the L1 MR MAC CE, 7 bits of RSRP is included and no RSRI is needed.

The current beam information of SpCell (if configured to be included) is always placed at the end of the normal (not truncated) MAC CE.

Same as Rel-18: When the C-LTM execution is triggered, if the TA of the target cell acquired by early RACH is valid, UE applies the TA acquired by early RACH, otherwise if UE has successfully measured the TA of the target cell, UE applies the measured TA.

FFS For MR triggered by LTM2, whether only include the current beam information in the MR MAC CE or the MR can include measurements for LTM candidates.

R2-2501900	Report of [POST129][111][MOB] (CATT)	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 1a: Rel-18 LTM capabilities are the baseline for Rel-19 inter-CU LTM.

[Huawei]: What does the proposal 1a mean? [Ericsson, CATT]: Intention was that for Rel-19 LTM operation, if we can reuse Rel-18 LTM capability, no additional new capability is needed for Rel-19.

Proposal 3: Define a per-band capability for conditional LTM(e.g. cltm-MCG-r19) . It indicates whether the UE supports conditional LTM on the band where the UE is in non-DC. This includes supports execution condition, subsequent LTM. UE supports this capability should support ltm-MCG-IntraFreq-r18 on the same band. FFS whether UE should set the value consistently for all FDD-FR1 band, all TDD-FR1 bands and so on respectively.

Proposal 5: Define a per-band capability for L3 execution condition, e.g. cltm-ExecutionConditionL3-r19 is defined to indicate whether the UE supports L3 execution condition for conditional LTM and whether the UE supports 2 trigger events for same execution condition. UE supports this capability should support capability for conditional LTM(e.g. cltm-MCG-r19) on the same band.

Proposal 6: A UE that supports conditional LTM(e.g. cltm-MCG-r19) should indicate the support for at least one of cltm-ExecutionConditionL3-r19 or cltm-ExecutionConditionL1-r19.

Proposal 7: When a UE indicates support for both conditional LTM(e.g. cltm-MCG-r19)  and ltm-RACH-LessCG-r18, it implies that the UE has the capacity to support RACH-less conditional LTM with a configured grant. Whether/how to update the field description of ltm-RACH-LessCG-r18 can be addressed in the running CR review.

Proposal 8a: Reuse the R18 capability (i.e., ue-TA-Measurement-r18) to indicate whether UE supports UE-based TA measurement for C-LTM. There is no need to define a separate capability for this purpose.

Proposal 9: Define a per-band capability, such as “cltm-TA-Indication-r19”. This capability serves to indicate whether the UE can support TA indication within the early TA MAC CE for conditional LTM.

Proposal 10: Reuse the R18 capability (i.e., ltm-MAC-CE-JointTCI-r18 and ltm-MAC-CE-SeparateTCI-r18) to indicate whether the UE supports MAC-CE activated joint/separate LTM TCI states for CLTM.

[AT129b][108][MOB] (CATT)

Scope: Discuss proposals in R2-2501900 and check if proposals can be endorsed/agreed. If not, identify what companies’ concerns are.

Intended outcome: Discussion summary in R2-2503011.

Deadline: Comeback in CB session => Completed.

R2-2503011	Report of [POST129][111][MOB] (CATT)	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

No need to define a separate capability for the reference configuration for inter-CU LTM.

Define per-UE capabilities for security key change of MCG LTM (e.g. ltm-KeyUpdate-MCG-r19) and security key change of SCG LTM (e.g. ltm-KeyUpdate-SCG-r19). UE supports these capabilities should also support ltm-MCG-IntraFreq-r18 or ltm-SCG-IntraFreq-r18 respectively. No new UE capability on inter-CU MCG LTM with SN unchanged and inter-CU MCG LTM SN with SCG addition.

Define a per-band capability for L1 execution condition, e.g. cltm-ExecutionConditionL1-r19 is defined to indicate whether the UE supports L1 execution condition for CLTM and subsequent CLTM.

Define a per-band capability for L3 execution condition, e.g. cltm-ExecutionConditionL3-r19 is defined to indicate whether the UE supports L3 execution condition for CLTM and subsequent CLTM and whether the UE supports 2 trigger events for same execution condition.

A UE that supports conditional LTM should indicate the support for at least one of cltm-ExecutionConditionL3-r19 or cltm-ExecutionConditionL1-r19.

When a UE indicates support for both conditional LTM and ltm-RACH-LessCG-r18, it implies that the UE supports RACH-less conditional LTM with a configured grant. Whether/how to update the field description of ltm-RACH-LessCG-r18 can be addressed in the running CR review.

Rely on the R18 capability (i.e., ue-TA-Measurement-r18) to indicate whether UE supports UE-based TA measurement for C-LTM. There is no need to define a separate capability for this purpose.

Define a new per UE capability for UE support of early TA MAC CE reception for CLTM and also the max number of maintaining TA values. The value range is (1~8).

Rely on the R18 capability (i.e., ltm-MAC-CE-JointTCI-r18 and ltm-MAC-CE-SeparateTCI-r18) to indicate whether the UE supports MAC-CE activated joint/separate LTM TCI states for CLTM.

R2-2502256	Draft 306 running CR for UE capability for Mob Ph4	CATT	draftCR	Rel-19	38.306	18.5.0	NR_Mob_Ph4-Core

R2-2502257	Draft 331 running CR for UE capability for Mob Ph4	CATT	draftCR	Rel-19	38.331	18.5.1	NR_Mob_Ph4-Core

Noted.

[POST129b][115][MOB] (Ericsson)

Scope: Update RRC running CR for inter-CU LTM and C-LTM based on RAN2#129bis progress and maintain essential open issue list in a separate contribution (RRC running CR can keep editor’s notes for readability).

Intended outcome: Updated RRC running CR and essential RRC open issue list for inter-CU LTM and C-LTM.

Deadline: Long email discussion

[POST129b][116][MOB] (Huawei)

Scope: Update RRC running CR for L1 event-driven MR based on RAN2#129bis progress and maintain essential open issue list in a separate contribution (RRC running CR can keep editor’s notes for readability).

Intended outcome: Updated RRC running CR and essential RRC open issue list for L1 event-driven MR.

Deadline: Long email discussion

[POST129b][117][MOB] (Vivo)

Scope: Update MAC running CR based on RAN2#129bis progress and maintain essential open issue list in a separate contribution (MAC running CR can keep editor’s notes for readability).

Intended outcome: Updated MAC running CR and essential MAC open issue list.

Deadline: Long email discussion

[POST129b][118][MOB] (CATT)

Scope: Update UE capability running CRs (including 38.331 and 38.306) based on RAN2#129bis progress.

Intended outcome: Updated UE capability running CRs (including 38.331 and 38.306).

Deadline: Long email discussion

[POST129b][119][MOB] (Apple)

Scope: Update 38.300 running CR based on RAN2#129bis progress.

Intended outcome: Updated 38.300 running CR.

Deadline: Long email discussion

[POST129b][120][MOB] (China Telecom)

Scope: Update 37.340 running CR based on RAN2#129bis progress.

Intended outcome: Updated 37.340 running CR.

Deadline: Long email discussion

[Apple]: For detailed stage 3 issues, please contact CR rapporteurs first.

For stage 3 essential issues, please contact CR rapporteurs first.

### 8.6.2	Inter-CU LTM

Essential remaining open issues, including security key change handling based on S3-251124, details of fast recovery, FFS on R19 LTM coexistence with MIMO 2TA/mTRP, stage-3 identified open issues if needed, etc.

1. Signaling for NCC delivery

R2-2502340	Discussion on inter-CU LTM	ZTE Corporation, Sanechips	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 1: For security key update in inter-CU LTM, RAN2 agree to include NCC value in the LTM cell switch command MAC CE.

R2-2502045	On remaining issues for Inter-CU LTM	Nokia	discussion

Proposal 1: RAN2 to support delivery of master key update related parameters via RRC message prior to LTM switch.

[Session chair]: Note that security related concern is SA3 scope and SA3 already responded it. Let’s check companies’ views. Rakuten, LG, ZTE, Lenovo, Apple, NTTDCM, Sharp, Samsung, Vivo, NEC, Xiaomi, MediaTek, OPPO, Ofinno, China Telecom, CATT, Ericsson, Fujitsu, CMCC, IDC, Denso, TCL (22) support proposal 1 (ZTE). Qualcomm, Nokia (2) support proposal 1 (Nokia). [Qualcomm]: If we go with proposal 1 (ZTE), it will be good to clarify to include actual NCC value (i.e. no need of further optimization, e.g. index for NCC, etc.) [NTTDCM]: Concern that putting plain NCC to CSC MAC CE.

For security key update in inter-CU LTM, RAN2 agree to include actual NCC value in the LTM cell switch command MAC CE.

2. NCC and Rel-19 ltm-NoSecurityChangeID

R2-2501897	Discussion on Inter-CU LTM	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 2: The NW ensures that NCC is included in the LTM cell switch command MAC CE if the R19 set ID is different between the target cell and source cell. Conversely, if the R19 set ID is same for both cells, the NCC will not be included.

R2-2502509	Discussion on remaining issues in Inter-CU LTM	Apple	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 2: RAN2 to confirm that NCC info can be provided to UE via LTM Command MAC CE when the source and target cell(s) have the same Rel-19 ltm-NoSecurityChangeID (intra-CU LTM).

R2-2502444	Discussion on security for inter-CU LTM	Ericsson	discussion	NR_Mob_Ph4-Core

Proposal 2: The existing LTM cell switch MAC CE is baseline but enhanced by including the NCC value and an indication on whether the UE should perform security key change or not (this indication override the Rel-19 security IDs).

[Rakuten]: Support Apple proposal. We also have intra-CU CU-UP change. [Apple]: The concerned scenario is 1) UE switched to inter-CU target cell, 2) new target cell receives a new NCC from AMF, 3) there was no time for the new target cell provides the NCC to the UE (e.g. UE is moving so fast), 4) So NCC needs to be provided by CSC MAC CE. [Huawei]: Do not think Apple’s concerned scenario is really realistic in the field. [Lenovo]: Wonder if inclusion of NCC really needs to be associated with R19 ID. If included, the UE will check and perform either horizontal key change or vertical key change. [Ericsson]: We may first agree that NCC is included in CSC MAC CE when R19 set id is different, and then we can use the legacy mechanism to change security for intra-cell. [Samsung, Nokia, Qualcomm]: Agree with Ericsson.

NCC is included in the LTM cell switch command MAC CE if the R19 set ID is different between the target cell and source cell. Conversely, if the R19 set ID is same for both cells, the NCC will not be included.

[Lenovo]: Wonder if we need to send an LS to inform RAN2 progress? [Apple]: Support sending LS to RAN3 to inform the related RAN2 progress.

[AT129b][109][MOB] (ZTE)

Scope: Prepare LS to RAN3 to inform RAN2 progress that is related to security aspect.

Intended outcome: LS to RAN3 in R2-2503012.

Deadline: Comeback in CB => Completed.

R2-2503012	LS on RAN2 agreements for security key handling in inter-CU LTM	LSout	To: RAN3	Rel-19	NR_Mob_Ph4-Core

Approved.

3. SN key regeneration in inter-CU MCG LTM

R2-2502159	Discussion on inter-CU LTM	vivo	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 18: RAN2 to discuss and choose one of the two options for the security key handling of inter-CU MCG LTM with DC:

-	Option 1: Default sk-Counter = 0 is used by UE for generating the SN key when security key update is performed in MCG.

-	Option 2: Network configures sk-Counter = 0 in all LTM candidate configurations, and UE uses the configured value for generating the SN key when security key update is performed in MCG.

NW configures the corresponding sk-Counter in all LTM candidate configurations, and UE uses the configured value for generating the SN key when security key update is performed in MCG.

[Nokia]: Prefer option2 and think we do not need to configure sk-Counter for all candidates, it should be sufficient to configure sk-Counter for each Rel-19 ID. [LG]: Understand “when a new AS root key, KNG-RAN, in the associated 5G AS security context is established” does not include horizontal or vertical key change due to HO/LTM cell switch. [Ericsson]: Do not think it should be set to fixed value, i.e. “0” in the security point of view. Prefer network-configuration based option. [Apple, CATT]: Setting sk-Counter to “0” comes from SA3 spec. [LG]: Note Kng-ran is different than Kng*. Kng-ran is used when the new security key should be created by AMF due to e.g. key mismatch. [CATT, OPPO]: Also prefer option 2. Network can configure the appropriate value whatever is. We don’t need to discuss whether sk-Counter 0 is correct or not. [Samsung]: Agree with Apple and CATT. [Ericsson]: Note security key change can happen for intra-CU case. [ZTE, Ericsson]: Current running CR is aligned with the agreement below. If we go with Nokia’s proposal, it needs further discussion to change the current running CR. Not sure if it is really needed.

4. Fast recovery

R2-2502285	Discussion on inter-CU LTM	Fujitsu	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 2 (modified): RAN2 to not support fast LTM recovery if the cell selected during RRC re-establishment procedure is one of the LTM candidate cell(s) that has different Rel-19 ID.

Proposal 3 (modified): RAN2 to not support fast LTM recovery after LTM failure that has different Rel-19 ID.

[Lenovo]: Support proposal 2 and proposal 3. Note in Rel-18, after CHO/HO failure, LTM recovery is not applied. [Ericsson]: Rel-19 ID does not distinguish whether intra-LTM or inter-LTM at least in the specification point of view (though it can be specified dependent on Rel-19 ID, the UE behaviour is defined when the key change is required or when LTM recovery is applied).

R2-2502509	Discussion on remaining issues in Inter-CU LTM	Apple	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 4: RAN2 to confirm that UE can perform fast LTM recovery to all candidate cells (from same or different CU as source cell), regardless intra-CU or inter-CU LTM failure, as long as NCC info is provided in LTM Command MAC CE.

Proposal 5: For fast LTM recovery, UE performs AS key derivation to the cell for recovery using NCC info provided in LTM Command MAC CE.

Proposal 6: When NCC is not provided (as in Rel-18) in LTM Command MAC CE, UE can only perform fast LTM recovery to intra-CU candidate cells with the same Rel-19 ltm-NoSecurityChangeID as source cell.

[Apple]: Think proposals are feasible, but it may be too optimistic. One compromise would be that we will allow LTM recovery to the candidates that belong to the same Rel-19 ID as the target cell. [Huawei]: Good to support the proposal and there would be another alternative, even though the candidate does not get the latest key, it can fetch from the source cell. [ZTE]: Understand if the selected candidate fetches the context from the source, it is not fast recovery. [Rakuten]: As Ericsson mentioned, Rel-19 id does not distinguish between intra-CU LTM and inter-CU LTM. [CATT]: Think common understanding is that, if Rel-19 ID is different, the UE needs to reestablish PDCP and change the security key (we do not need to know whether it is intra- or inter-). Think we do not need a complicated solution for recovery since it is anyway a kind of optimization. Ok with not supporting LTM recovery in inter-CU LTM failure. [Nokia]: Prefer supporting original Apple proposal.

R2-2502852	Further Considerations to Support Inter-CU LTM	Samsung	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 1-6 (modified): For inter-CU LTM cell switch failure (due to expiry of T304), RAN2 is kindly asked to discuss whether to support the fast failure recovery if the selected LTM candidate cell has same Rel-19 ID w.r.t. the target cell (Case 2).

Proposal 1-4: After inter-CU/intra-CU LTM cell switch failure (due to the expiry of T304), RAN2 is kindly asked to agree that the UE can perform the fast recovery if the selected LTM candidate cell is the intra-CU cell w.r.t. to the source cell (Case 1 & 4).

Proposal 1-5: After intra-CU LTM cell switch failure (due to the expiry of T304), RAN2 is kindly asked to agree that the UE performs RRC reestablishment procedure (not fast recovery) if the selected LTM candidate cell is the inter-CU candidate cell w.r.t. to the source cell (Case 3) .

Proposal 1-7: For RLF after the LTM cell switch, RAN2 is kindly asked to agree that the UE can perform the fast recovery if the selected LTM candidate cell is the intra-CU cell w.r.t. the target cell (Case 1 & 2).

[Fujitsu]: Understand one NCC is only used for single candidate. [Apple]: In NW, there is only one NCC (either used NCC or just new NCC). Understand NCC can be used for multiple candidates belonging to the same Rel-19 ID. [MediaTek]: Understand in inter-CU LTM, PDCP is reestablished and all variables will be released. Like what we discussed in Rel-18, then no LTM recovery can be applied. [Ericsson]: Ok with Rel-19 ID based LTM recovery. [Apple]: Note LTM recovery here is different compared to Rel-18 LTM. In Rel-18 LTM recovery, no PDCP reestablishment -> UE revert back to the source configuration -> the UE uses old key. In Rel-19 LTM recovery, PDCP reestablishment -> UE revert back to the source configuration -> apply the latest NCC to derive a new key for the newly selected candidate cell.

[Session chair]: Let’s summarize:

- Option1: No LTM recovery is applied for inter-CU LTM (once NCC is included in CSC MAC CE)

- Option2: LTM recovery can be applied if the selected candidate cell belongs to the same Rel-19 with the target cell.

- Option3: LTM recovery can be applied for all candidates.

[Session chair]: Ok to exclude option3? Seems only few companies support option3 now.

Option3 (LTM recovery can be applied for all candidates) is not considered.

Offline discussion to clearly understand how option2 (LTM recovery can be applied if the selected candidate cell belongs to the same Rel-19 with the target cell) works and check if there is a real technical blocking issue. If it works w/o real technical blocking issue, then we can go to option2. If there is a real technical blocking issue for option2, we’ll go to option1 (No LTM recovery is applied for inter-CU LTM).

[AT129b][110][MOB] (Ericsson)

Scope: Clearly clarify how option2 works and check if there is a real technical blocking issue. If it works w/o real technical blocking issue, then we can go to option2. If there is a real technical blocking issue for option2, we’ll go to option1.

Intended outcome: Discussion summary in R2-2503013.

Deadline: Comeback in CB session => Completed (it was included in [AT129b][102]).

5. UE-based TA measurement

R2-2502159	Discussion on inter-CU LTM	vivo	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 7 (modified): From RAN2 point of view, UE-based TA measurement can be also applicable for the synchronized inter-CU LTM scenario, and no additional RAN2 specification impact is expected. Final decision is up to RAN4.

From RAN2 point of view, UE-based TA measurement can be also applicable for the synchronized inter-CU LTM scenario, and no additional RAN2 specification impact is expected. Final decision is up to RAN4.

[Lenovo]: Wonder if we need to send an LS to RAN3 to inform this agreement. They may need to take it into account for NW interface. [ZTE]: Understand RAN3 impacts, but we may need to make further decision before sending an LS to RAN3 (e.g. who will allocate ID). [Ericsson]: Understand RAN3 already discussed it, so there is no need to send an LS. And think one option would be source will decide it.

6. Co-existence with Rel-18 MIMO 2 TA

R2-2502727	Discussion on Inter-CU LTM	CMCC	discussion	Rel-. 19	NR_Mob_Ph4-Core

Proposal 4: MIMO 2TA can be configured with inter-CU LTM using PDCCH-order based TA measurement.

Proposal 6: If UE-based TA measurement is supported for inter-CU LTM, it cannot be configured if candidate cell is configured with MIMO 2TA

MIMO 2TA can be configured with inter-CU LTM using PDCCH-order based TA measurement.

If UE-based TA measurement is supported for inter-CU LTM, it cannot be configured if candidate cell is configured with MIMO 2TA.

7. Co-existence with Rel-18 NES

R2-2502944	Inter-CU LTM	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 6: An LTM target cell will not apply cell DTX/DRX until LTM cell switch is completed. RAN2 will not consider any specification impacts needed for LTM RACH-less cell switch to an NES candidate cell.

An LTM target cell will not apply cell DTX/DRX until LTM cell switch is completed. RAN2 will not consider any specification impacts needed for LTM RACH-less cell switch to an NES candidate cell.

Agreements on inter-CU LTM

- For security key update in inter-CU LTM, RAN2 agree to include actual NCC value in the LTM cell switch command MAC CE.

- NCC is included in the LTM cell switch command MAC CE if the R19 set ID is different between the target cell and source cell. Conversely, if the R19 set ID is same for both cells, the NCC will not be included.

- NW configures the corresponding sk-Counter in all LTM candidate configurations, and UE uses the configured value for generating the SN key when security key update is performed in MCG.

- From RAN2 point of view, UE-based TA measurement can be also applicable for the synchronized inter-CU LTM scenario, and no additional RAN2 specification impact is expected. Final decision is up to RAN4.

- MIMO 2TA can be configured with inter-CU LTM using PDCCH-order based TA measurement.

- If UE-based TA measurement is supported for inter-CU LTM, it cannot be configured if candidate cell is configured with MIMO 2TA.

- An LTM target cell will not apply cell DTX/DRX until LTM cell switch is completed. RAN2 will not consider any specification impacts needed for LTM RACH-less cell switch to an NES candidate cell.

R2-2501835	Further discussion on inter-CU LTM	HONOR	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2501887	Discussion on open issues for inter-CU LTM	OPPO	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2501897	Discussion on Inter-CU LTM	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2501929	Security discussion and other remaining issues of inter-CU LTM	MediaTek Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2501977	Discussion on inter-CU LTM	Google	discussion	Rel-19

R2-2501995	Remaining issues of inter-CU LTM	LG Electronics Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502017	Remaining Issues for Inter-CU LTM	Qualcomm Incorporated	discussion

R2-2502045	On remaining issues for Inter-CU LTM	Nokia	discussion

R2-2502066	Discussion on inter-CU LTM	Xiaomi	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502120	Discussion on Inter-CU LTM	Rakuten Mobile, Inc	discussion	Rel-19

R2-2502121	Discussion on Inter-CU LTM	Rakuten Mobile, Inc	discussion	Rel-19

R2-2502122	Discussion on Inter-CU LTM	Rakuten Mobile, Inc	discussion	Rel-19

R2-2502135	Discussion on Inter-CU LTM security and fast recovery	InterDigital, Europe, Ltd.	discussion	Rel-19

R2-2502159	Discussion on inter-CU LTM	vivo	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502235	Discussion on inter-CU LTM	China Telecom	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502285	Discussion on inter-CU LTM	Fujitsu	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502299	Discussion on inter-CU LTM	NEC	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502340	Discussion on inter-CU LTM	ZTE Corporation, Sanechips	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502359	Remaining issues for Inter-CU LTM	Lenovo	discussion	Rel-19

R2-2502381	Discussion on issues for supporting inter-CU LTM	Sharp	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502423	Further discussion remaining issues of inter-CU LTM cell switch	Transsion Holdings	discussion	Rel-19

R2-2502444	Discussion on security for inter-CU LTM	Ericsson	discussion	NR_Mob_Ph4-Core

R2-2502481	Discussion on inter-CU LTM	NTT DOCOMO, INC.	discussion	Rel-19

R2-2502490	LTM Resource consumption for the target cells	Sony	discussion	Rel-19	NR_Mob_Ph4

R2-2502509	Discussion on remaining issues in Inter-CU LTM	Apple	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502601	Discussion on open issues for inter-CU LTM	Ofinno, LLC	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502683	Discussion on security handling for inter-CU LTM	KT Corp.	discussion

R2-2502727	Discussion on Inter-CU LTM	CMCC	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502852	Further Considerations to Support Inter-CU LTM	Samsung	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502944	Inter-CU LTM	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502965	Discussion on inter-CU LTM	DENSO CORPORATION	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502774	Further Considerations to Support Inter-CU LTM	Samsung	discussion	Rel-19	NR_Mob_Ph4-Core

=> Withdrawn

### 8.6.3	L1 event triggered measurement reporting

Essential remaining open issues, including MAC CE introduction based on R1-2501500, details of MR MAC CE, stage-3 identified open issues if needed, etc.

1. Activation/deactivation of SP CSI-RS for LTM

R2-2502433	Discussion on L1 event triggered measurement reporting	Spreadtrum, UNISOC	discussion	Rel-19

Proposal 1: Introduce new MAC CE to activate/deactivate the semi-persistent CSI-RS resource.

Introduce new MAC CE to activate/deactivate the semi-persistent CSI-RS resource.

Proposal 2 (modified): The new MAC CE includes:

- A/D indication

- Target configuration id or {SP CSI-RS resource set id and/or candidate id}

- TCI state id

The new MAC CE includes: A/D indication, Target configuration id or {SP CSI-RS resource set id and/or candidate id}, TCI state id.

R2-2502799	Remaining Issues of L1 Event Triggered Measurement Report	Samsung	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 2: RAN2 decide How to handle the activated semi-persistent CSI-RS resources after LTM cell switch (i.e. support the Subsequent LTM) between following options.

•	Option 1: UE stop to measure the activated semi-persistent CSI-RS resources after cell switch

•	Option 2: UE continues to measure the activated semi-persistent CSI-RS resources after cell switch

•	Option 3: UE stop to measure the activated semi-persistent CSI-RS resources of the candidate cells (i.e. other than target cell) after cell switch. UE continues to measure the activated semi-persistent CSI-RS resources of the target cell after cell switch

[LG, Nokia, OPPO]: Think option1 is sufficient. [CATT, MediaTek, Apple, IDC, Honor]: Think option2 is more reasonable. [ZTE]: We need to consider RAN3 impact. For option2, source should indicate to the target regarding the activated SP CSI-RS information. [Samsung]: Even with option1, the target may need to indicate the deactivation to all corresponding candidates. Probably similar impact on RAN3. In the functional point of view, option2 will be helpful and make decision this meeting will also help RAN3. [Honor]: With option1, it will also bring additional signalling overhead since it needs to send activation again for the same SP CSI-RS resources.

Will be revisited next meeting.

2. Restriction of evaluated RSs via candidate cell indication

R2-2502950	L1 event triggered measurement report 	LG Electronics Inc.	discussion	NR_Mob_Ph4-Core

Proposal 13: Network can optionally configure a candidate cell via RRC within report configuration of event LTM. If the candidate cell is configured, UE only evaluates RSs associated with the candidate cell among RSs associated with the event.

R2-2501898	L1 event triggered measurement reporting	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 4: RAN2 confirms the previous agreement that "For association between measurement resource configuration and measurement reporting configuration, R18 LTM way is reused if possible". No additional enhancements are needed.

[ZTE, Nokia, Ericsson]: Support the LG proposal. Question is how the source cell can configure measurement RS without coordination with candidate cells. [CATT, OPPO, Samsung, Apple]: Support CATT proposal. Coordination is RAN3 scope. [Samsung]: Consider it is a kind of optimization and seems somewhat overlapped with possible Rel-20 scope. [ZTE]: There is clear difference compared to possible Rel-20 scopes. [Apple]: It may bring the additional works to RAN3 if we go to LG proposal. [ZTE]: Have different understanding. [Session chair]: Understand in Rel-18, measurement RS will be generated in the source, and it will be forwarded to all candidates. Then candidate will configure reporting configuration accordingly. What makes Rel-19 now different? [ZTE]: In Rel-19, difference is to have event triggering related parameters in reporting configuration, which makes candidate configure appropriately more difficult. [Session chair]: Maybe good to have offline among interested companies to understand the issue.

Comeback in CB session with some offline among interested companies (ZTE)

CB Session:

[ZTE] Propose the following WF, based on the offline:

FFS The beam specific offset Obn and Obs is configured as per-beam or per-report.

Proposed WF: In L1 reporting configuration, the UE can be configured with a list of candidate IDs. For each listed candidate ID, the network can configure an offset, this offset applies to all the RSs belong the candidate ID that associated with the reporting configuration (the offset value also includes 0dB); If a candidate ID is not provided in the reporting configuration, it means the UE is not required to measure/evaluate the RSs that belonging to the candidate ID for this event, even if these are configured within the L1 resource configuration (to which the L1 reporting configuration points to).

In L1 reporting configuration, the UE can be configured with a list of candidate IDs. For each listed candidate ID, the network can configure an offset, this offset applies to all the RSs belong the candidate ID that associated with the reporting configuration (the offset value also includes 0dB); If a candidate ID is not provided in the reporting configuration, it means the UE is not required to measure/evaluate the RSs that belonging to the candidate ID for this event, even if these are configured within the L1 resource configuration (to which the L1 reporting configuration points to).

3. MR MAC CE: Indication of beams(s) that meets the event condition

R2-2502445	Discussion on event-based L1 measurements	Ericsson	discussion	NR_Mob_Ph4-Core

Proposal 2	RAN2 to consider the following options to indicate the report-on-leave and event-triggered beams in the MR MAC CE:

a)	A bitmap to indicate whether the reported beam is an event-triggered or a non-triggered beam and whether the reported beam fulfills the entering condition(s) or the leaving condition(s) for the given LTM event.

b)	A single-bit per beam to indicate whether the beam is a triggering beam and an additional single-bit indication per reported beam to distinguish the event-triggered and non-triggered beams for the reported beam.

R2-2502552	Further Details on L1 Measurement Reporting Enhancements for Rel-19 LTM	Nokia	discussion	Rel-19	NR_Mob_Ph4-Core	R2-2500332

Proposal 8: RAN2 agrees that MR MAC CE has a single field indicating that the report is for report on leave.

Proposal 9: If the MR MAC CE is for report on leave it contains only RSRIs that are reported as they have met the event leaving condition.

R2-2501898	L1 event triggered measurement reporting	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 2a: In the MR MAC CE, adopt a uniform format for all the reported beams (i.e., using the same format for beams leaving the entering condition, beams met the entering condition, beams not satisfying the event).

[LG]: Prefer Ericsson option b). When leaving condition is met, the NW needs to know other beam information also, which means we need an indication per beam level. [ZTE, Xiaomi, Apple]: We should allow to report both beam(s) that entered the condition and beam(s) that left the condition. [ZTE]: Understand we need four code points, indicating beam(s) that newly entered the condition, beam(s) that left the condition, beam(s) that maintained as the beam(s) that already entered the condition and reported, and beam(s) that additional beam(s) that doesn’t met any of entering and leaving conditions. [Apple]: Understand Ericsson 2b) is supported by majority companies. [Nokia]: Although not popular, with our proposal, it would be beneficial in the signalling overhead point of view. [Xiaomi]: With differential RSRP, we can already have two spare bits per beam. Do not think signalling overhead is something critical here. [Nokia]: Do not understand why we really need explicit four code points. Now, we can agree with two indications per beam, which indicates beam(s) that entered the condition and beam(s) that left the condition. [Huawei]: Nokia proposal seems ok since it is aligned with how legacy L3 MR works.

In the MR MAC CE, adopt a uniform format for all the reported beams (i.e., using the same format for beams leaving the entering condition, beams met the entering condition, beams not satisfying the event).

Introduce two bits per beam.

Comeback in CB session with further offline w.r.t. how to define two bits. (LG)

CB session:

[LG]: Propose the following WF, based on the offline:

1. RAN2 considers the following beams types subject to reporting (not other types):

Type1 beam: a beam that has satisfied the entry condition of the event for TTT and thus triggers the MR MAC CE.

- Type1 beam is now in beamsTriggeredList since the beam has been just included in beamsTriggeredList.

Type2 beam: a beam that has satisfied the leaving condition of the event for TTT and thus triggers the MR MAC CE.

- Type2 beam is NOT in beamsTriggeredList since the beam has been just excluded from beamsTriggeredList.

Type3 beam: a beam that is neither type1 nor type2 but has remained in beamsTriggeredList (the beam was reported as type1 beam before)

- Type3 beam is still in beamsTriggeredList.

Type4 beam: a beam that is neither type1 nor type2 and not included in beamsTriggeredList (i.e., just extra beam info)

- Type4 beam is NOT in beamsTriggeredList.

2. All companies agreed to choose either one of option1 or option3.

option1: To introduce four codepoints to represent one of {type1, type2, type3, type4}

option3: To introduce three codepoints to represent one of {type1_or_type3, type2, type4}, i.e., type1 beam and type3 beam are not distinguished in reporting

3. Majority of the companies are fine to go for option1, since it is straightforward.

4. A few companies think option3 is sufficient, since distinction between type1 and type3 from network side is not essential.

[Nokia]: Prefer not to make decision now. [Apple]: At least want to make a working assumption with option1 and if problem is really justified with option1, we can revert it back.

RAN2 will consider only the following options:

Option1: To introduce four codepoints to represent one of {type1, type2, type3, type4}

Option3: To introduce three codepoints to represent one of {type1_or_type3, type2, type4}, i.e., type1 beam and type3 beam are not distinguished in reporting

Type1 beam: a beam that has satisfied the entry condition of the event for TTT and thus triggers the MR MAC CE.

Type2 beam: a beam that has satisfied the leaving condition of the event for TTT and thus triggers the MR MAC CE.

Type3 beam: a beam that is neither type1 nor type2 but has remained in beamsTriggeredList (the beam was reported as type1 beam before).

Type4 beam: a beam that is neither type1 nor type2 and not included in beamsTriggeredList (i.e., just extra beam info)

Working assumption: Option1: To introduce four codepoints to represent one of {type1, type2, type3, type4}

4. Truncated MR MAC CE

R2-2502799	Remaining Issues of L1 Event Triggered Measurement Report	Samsung	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 6: UE considers the LTM MR procedure is completed and cancel the LTM MR procedure after UE send the truncated event triggered L1 measurement report MAC CE for the reported ID.

R2-2502094	Discussion on event-triggered meas report	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 13: If only part of the MR for the corresponding ReportConfigId is transmitted in the truncated MAC CE, the triggering status of the corresponding event should not be cancelled. The triggering state should only be cancelled once the complete MR for the corresponding ReportConfigId has been transmitted.

R2-2502950	L1 event triggered measurement report 	LG Electronics Inc.	discussion	NR_Mob_Ph4-Core

Proposal 12: Network configures UE via RRC with whether UE should still consider triggered L1 MAC CE as pending or cancel it after transmitting truncated L1 MAC CE.

[Lenovo]: Exact format of MR MAC CE is not decided yet. It will be difficult to make progress on this topic now. [CATT]: It should be ok to discuss general principle. Agree with Huawei’s proposal. [Vivo]: Prefer Samsung proposal. [Apple]: Understand Samsung proposal is aligned with BFR, support Samsung proposal. Truncated MR MAC CE will be generated after reception of grant (if not enough), so there would be no remaining part of MR MAC CE because MR MAC CE was not generated. [Nokia]: Also prefer Samsung proposal. Remaining measurement reporting information may be changed when the following additional grant is received. [ZTE]: Do not understand how Samsung proposal works. If multiple beams met the condition, which beam will be included? [Apple]: Probably Huawei’s proposal and Samsung’s proposal wouldn’t be much different. For single beam that met a condition, anyway there would be no additional MR once truncated MR was sent. For multiple beams that met a condition simultaneously, once the truncated MR MAC CE was sent, if following additional grant is received, the UE still can trigger another MR procedure for the remained event-triggered beam(s). [Apple]: Huawei confirms it. [Nokia]: Think still not clear whether initially triggered MR procedure is cancelled or not. [Huawei]: If cancelled, does it mean the UE needs to perform new event triggering condition. [Session chair]: Understand MR procedure is triggered by event (not by remained event-triggered beam(s) that were not reported by the truncated MR MAC CE). So, MR needs to be either cancelled or suspended. Cancelled and trigger another new MR procedure by the remained event-triggered beam(s) that were not included in the truncated MR MAC CE, sounds new.

Intention is that the UE should be able to report the event-triggered beam(s) that were not included in the truncated MR MAC CE by the following grant. Detailed wording can be further discussed as part of the running CR.

R2-2502552	Further Details on L1 Measurement Reporting Enhancements for Rel-19 LTM	Nokia	discussion	Rel-19	NR_Mob_Ph4-Core	R2-2500332

Proposal 10: The UE uses truncated MR MAC CE when it is not able to report all the beams that triggered the report, up to the configured number of beams.

Not agreed.

[LG]: Prefer simple approach. If the UE cannot report whole MR MAC CE, then the UE uses truncated MR MAC CE. [Lenovo]: Prefer Nokia’s proposal. The additional results up to N is optional for NW side. [CATT]: LG’s comment is quite aligned with RAN2 previous agreement.

5. Co-existence with mTRP

R2-2502360	Event based L1 measurement report	Lenovo	discussion	Rel-19

Proposal 2: mTRP at serving cell is not supported for event based L1 measurement report in Rel-19.

R2-2501898	L1 event triggered measurement reporting	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 6a: Support the coexistence of LTM event triggered reporting and mTRP operation at serving cells.

Proposal 6b: When LTM event-triggered reporting and mTRP operation coexist in the serving cells, the network configures which of the indicated TCI states of the serving cell is to be used for LTM event-triggered measurements evaluation.

[Samsung, Apple, Xiaomi]: Agree with the proposal. [Samsung]: Would like to remove “at serving cell” in the proposal. [Apple]: Note it was discussed for MIMO and no consensus was made in RAN1. [ZTE, Vivo, Huawei, Nokia, OPPO, NTTDCM. Ericsson, Rakuten]: Prefer supporting mTRP for event-triggered MR in Rel-19. [Apple]: As WI rapporteur, propose to complete all essential functions first, then we can further consider if mTRP can be easily introduced if time allows. [Nokia]: Understand there are two main questions to support mTRP. Q1) How to define the serving beam? Q2) What should be reported in MR MAC CE? Consider they are not so complicated. [Ericsson]: Although we want to support mTRP, we don’t need any discussion for optimization. It may be achieved w/o any specification impact (or w/ little change). [Apple]: As compromise, would like to suggest to see if it can be supported easily once MAC running CR becomes mature (assuming in August meeting). [Session chair]: Would like to suggest proponent companies propose one simple option based on offline discussion among the companies.

Will be revisited in August. If one simple solution is not prepared / agreed until / in August meeting, we will not apply mTRP in Rel-19 event-triggered MR.

Agreements on L1 event-triggered MR

- Introduce new MAC CE to activate/deactivate the semi-persistent CSI-RS resource.

- The new MAC CE includes: A/D indication, Target configuration id or {SP CSI-RS resource set id and/or candidate id}, TCI state id.

- In L1 reporting configuration, the UE can be configured with a list of candidate IDs. For each listed candidate ID, the network can configure an offset, this offset applies to all the RSs belong the candidate ID that associated with the reporting configuration (the offset value also includes 0dB); If a candidate ID is not provided in the reporting configuration, it means the UE is not required to measure/evaluate the RSs that belonging to the candidate ID for this event, even if these are configured within the L1 resource configuration (to which the L1 reporting configuration points to).

- In the MR MAC CE, adopt a uniform format for all the reported beams (i.e., using the same format for beams leaving the entering condition, beams met the entering condition, beams not satisfying the event). Introduce two bits per beam.

- RAN2 will consider only the following options:

- Option1: To introduce four codepoints to represent one of {type1, type2, type3, type4}

- Option3: To introduce three codepoints to represent one of {type1_or_type3, type2, type4}, i.e., type1 beam and type3 beam are not distinguished in reporting.

Type1 beam: a beam that has satisfied the entry condition of the event for TTT and thus triggers the MR MAC CE.

Type2 beam: a beam that has satisfied the leaving condition of the event for TTT and thus triggers the MR MAC CE.

Type3 beam: a beam that is neither type1 nor type2 but has remained in beamsTriggeredList (the beam was reported as type1 beam before).

Type4 beam: a beam that is neither type1 nor type2 and not included in beamsTriggeredList (i.e., just extra beam info)

- Working assumption: Option1: To introduce four codepoints to represent one of {type1, type2, type3, type4}

- Intention is that the UE should be able to report the event-triggered beam(s) that were not included in the truncated MR MAC CE by the following grant. Detailed wording can be further discussed as part of the running CR.

- For co-existence with mTRP, will be revisited in August. If one simple solution is not prepared / agreed until / in August meeting, we will not apply mTRP in Rel-19 event-triggered MR.

R2-2501836	Discussion on measurement event evaluation and report	HONOR	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2501879	Remaining issues of L1 event triggered measurement reporting	Xiaomi	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2501888	Open issues for event triggered  L1 measurement reporting	OPPO	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2501898	L1 event triggered measurement reporting	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2501930	Remaining issues on event triggered L1 MR	MediaTek Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502019	L1 event-triggered measurement reporting for LTM	Qualcomm Incorporated	discussion

R2-2502032	Discussions on measurement related enhancements for LTM	Fujitsu	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502094	Discussion on event-triggered meas report	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502133	Discussion on L1 event triggered measurement reporting	Rakuten Mobile, Inc	discussion	Rel-19

R2-2502160	Discussion on measurement event evaluation and reporting	vivo	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502236	Discussion on L1 event triggered measurement reporting	China Telecom	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502311	Discussion on L1 event triggered  measurement	Apple	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502321	Details of MAC CE for event-triggered L1 measurement report	NEC	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502341	Discussion on L1 event triggered measurement reporting	ZTE Corporation, Sanechips	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502360	Event based L1 measurement report	Lenovo	discussion	Rel-19

R2-2502382	Discussion on issues for supporting L1 event triggered measurement reporting	Sharp	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502424	Discussion on L1 event triggered measurement reporting	Transsion Holdings	discussion	Rel-19

R2-2502433	Discussion on L1 event triggered measurement reporting	Spreadtrum, UNISOC	discussion	Rel-19

R2-2502445	Discussion on event-based L1 measurements	Ericsson	discussion	NR_Mob_Ph4-Core

R2-2502552	Further Details on L1 Measurement Reporting Enhancements for Rel-19 LTM	Nokia	discussion	Rel-19	NR_Mob_Ph4-Core	R2-2500332

R2-2502602	Discussion on L1 event triggered measurement report	Ofinno, LLC	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502606	LTM event-triggered measurement reporting 	Fraunhofer HHI, Fraunhofer IIS	discussion

R2-2502721	Discussion on L1 event triggered measurement reporting	CMCC	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502799	Remaining Issues of L1 Event Triggered Measurement Report	Samsung	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502823	Discussion on MR MAC CE for LTM	ASUSTeK	discussion	Rel-19	NR_Mob_Ph4-Core	R2-2500417

R2-2502831	Discussion on L1 event triggered measurement reporting for LTM	KDDI Corporation	discussion	Rel-19

R2-2502950	L1 event triggered measurement report 	LG Electronics Inc.	discussion	NR_Mob_Ph4-Core

### 8.6.4	Conditional intra-CU LTM

Essential remaining open issues, including details of early TAT handling, stage-3 identified open issues if needed, etc.

1. TAT handling for candidate cells (except the target cell that becomes the new serving cell)

R2-2502018	Conditional intra-CU LTM	Qualcomm Incorporated	discussion

Proposal 5: (modified) Upon C-LTM execution, UE performs the following for received TA values and TA timers for other candidate cells than the target candidate cell:

•	Maintain the TA value.

•	Keep the early TA timer running.

Upon C-LTM execution, UE performs the following for received TA values and TA timers for other candidate cells than the target candidate cell: Maintain the TA value and Keep the early TA timer running.

2. TAT handling for the target cell that becomes the new serving cell

R2-2501899	Discussion on Conditional Intra-CU LTM	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 2a: CLTM TAT for target cell is stopped upon CLTM cell switch execution to the target cell. Then UE starts the timeAlignmentTimer associated with PTAG as in Rel-18.

R2-2502945	Intra-CU conditional LTM	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 5: Upon C-LTM cell switch execution, the UE stops the C-LTM timer and starts the PTAG using the remaining time from the C-LTM timer (Option 2.3).

R2-2502800	Remaining Issues of Conditional Intra-CU LTM	Samsung	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 2: C-LTM TAT for target cell is not stopped upon C-LTM cell switch execution to the target cell if it is running, RAN2 clarify the running TAT to act as the timeAlignmentTimer associated with PTAG.

R2-2501931	Further discussion on Conditional LTM	MediaTek Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 1: When UE perform CLTM cell switch to the target cell, the timeAlignmentTimer associated with PTAG is started. The remaining time of the CLTM TAT timer for target cell is used for the pTAG TAT if RACH-less CLTM is performed. Otherwise (RACH-based), the TAT of pTAG start from a pre-configured TAT value as legacy.

Proposal 2: When the UE performs CLTM RACH-less cell switch to the target cell, the CLTM TAT timer of that cell is maintained until it expires. When the timer expires, the UE does not perform any special behavior.

[Xiaomi]: Support CATT proposal. [LG]: Support Huawei proposal. CATT proposal is not aligned with legacy principle. To CATT proposal, TAT is started with old TA value that was received in early UL sync procedure. [ASUSTek, OPPO, NEC]: Support Huawei proposal. [CATT]: In Rel-18 LTM, when the UE receives CSC MAC CE, the UE starts TAT according to target configuration. [Samsung]: Agree with LG. [Session chair]: Agree with LG’s comment. We can decide between Huawei and Samsung proposals. [Ericsson, Apple]: Support Samsung proposal. [MediaTek]: Ask if PTAG timer does not run at all then? [Samsung]: Clarifies that if the UE receives TA from the new source, PTAG timer should run. [Ericsson]: It would be up to UE implementation. [ZTE]: Understand partially Samsung and Huawei’s proposals are correct, we can combine each part.

C-LTM TAT for target cell is not stopped upon C-LTM cell switch execution to the target cell if it is running, and starts the PTAG using the remaining time from the C-LTM timer.

3. Selection of target cell if multiple cells met C-LTM execution condition, based on L3 measurement

R2-2502067	Discussion on conditional LTM	Xiaomi	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 8: For L3 based CLTM, the CLTM execution is triggered when at least one candidate cell fulfills the CLTM condition. When multiple candidate cells satisfy the L3 CLTM condition, it is up to UE implementation to select a candidate cell and perform CLTM.

R2-2502342	Discussion on conditional intra-CU LTM	ZTE Corporation, Sanechips	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 2: In case more than one CLTM candidate cells fulfill the condition, the CLTM candidate cell that UE has valid TA is prioritized.

Proposal 3: In case more than one CLTM candidate cells fulfill the condition and UE has valid TA (or UE does not have valid TA for any of them), the CLTM candidate cell which associated with L3 execution condition is prioritized.

Proposal 4: In case more than one CLTM candidate cells fulfill the condition with the same type (L1 event or L3 event), and UE has valid TA (or UE does not have valid TA for any of them), it is up to UE to decide which LTM candidate cell will be selected for CLTM execution.

4. Selection of the target beam in L3-based RACH-less C-LTM

R2-2502438	Discussion on conditional LTM	Spreadtrum, UNISOC	discussion	Rel-19

Proposal 2: For L3 based C-LTM, beam selection for RACH-less LTM with CG is based on the configured RSRP threshold, e.g. similar to cg-RRC-RSRP-ThresholdSSB.

Proposal 3: If multiple candidate beams satisfy the RSRP threshold, it is up to UE implementation to select a beam.

R2-2501931	Further discussion on Conditional LTM	MediaTek Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 7:  For L3 based C-LTM, it is up to UE implementation to select the target beam.

5.1 Co-existence with MIMO 2 TA

R2-2502342	Discussion on conditional intra-CU LTM	ZTE Corporation, Sanechips	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 5: RAN2 confirm CLTM with MIMO 2TA is supported, introducing TAG ID field in LTM candidate TAC MAC CE.

Proposal 6: For CLTM with MIMO 2TA, two TA timer length (i.e. ltm-TimeAlignmentTimer) can be configured via RRC.

[Samsung]: Is TAG id one bit index or full id? [ZTE]: 1bit. [Apple]: In addition to P5 and P6, we need to define when RACH-less C-LTM is performed and when RACH-based CLTM is performed based on the selected beam. [CATT]: Do not see real complication to support 2TA. There will be mapping between TCI-state and TAG id. If the beam meets C-LTM condition and it does not belong to TAG id where the UE has the corresponding TA, the UE performs RACH-based C-LTM. [Apple]: Not to prefer to make decision now. Want to see co-existence later.

RAN2 confirm CLTM with Rel-18 MIMO 2TA is supported, introducing TAG ID field in LTM candidate TAC MAC CE.

For CLTM with Rel-18 MIMO 2TA, two TA timer length (i.e. ltm-TimeAlignmentTimer) can be configured via RRC.

R2-2502312	Discussion on conditional Intra-CU LTM	Apple	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 10: Do not support the coexistence of CLTM and MIMO 2TA.

5.2 Co-existence with Other mobility schemes

R2-2501889	Discussion on conditional LTM	OPPO	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 8: CLTM and L3 HO coexistence is supported.

Proposal 9: CLTM and DAPS coexistence is not supported.

Proposal 10: Network is allowed to trigger LTM cell switch execution for a candidate cell which is associated with CLTM execution condition.

R2-2502945	Intra-CU conditional LTM	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 10: The UE can be simultaneously configured with Rel-18 LTM candidate configurations and with C-LTM candidate configurations.

Proposal 11: Upon reception of an LTM cell switch command MAC CE, the UE performs LTM cell switch to the indicated candidate, regardless whether it is configured for Rel-18 LTM or for conditional LTM.

R2-2502878	On conditional LTM	Nokia	discussion	Rel-19	NR_Mob_Ph4

Proposal 10: Follow similar principle as Rel. 18 LTM, CLTM and CHO can be configured independently for different candidate cells.

Proposal 11: RAN2 to discuss if co-existence of CHO and CLTM for the same candidate target cell is supported.

R2-2502067	Discussion on conditional LTM	Xiaomi	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 1c: The coexistence of LTM and CHO is not supported in Rel-19 (which revises the associated agreement in RAN2 #127). And the coexistence of CLTM and CHO is not supported in Rel-19.

5.3 Co-existence with (e)RedCap and CovEnh

R2-2501996	Remaining issues of CLTM	LG Electronics Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 5. Support coexistence between Rel-19 CLTM and (e)RedCap.

Proposal 6. Support coexistence between Rel-19 CLTM and CovEnh.

Agreements on C-LTM

- Upon C-LTM execution, UE performs the following for received TA values and TA timers for other candidate cells than the target candidate cell: Maintain the TA value and Keep the early TA timer running.

- C-LTM TAT for target cell is not stopped upon C-LTM cell switch execution to the target cell if it is running, and starts the PTAG using the remaining time from the C-LTM timer.

- RAN2 confirm CLTM with Rel-18 MIMO 2TA is supported, introducing TAG ID field in LTM candidate TAC MAC CE.

- For CLTM with Rel-18 MIMO 2TA, two TA timer length (i.e. ltm-TimeAlignmentTimer) can be configured via RRC.

R2-2501826	Discussion on Conditional Intra CU LTM	Lekha Wireless Solutions	discussion	Rel-19

R2-2501837	Discussion on conditional LTM	HONOR	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2501889	Discussion on conditional LTM	OPPO	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2501899	Discussion on Conditional Intra-CU LTM	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2501931	Further discussion on Conditional LTM	MediaTek Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2501996	Remaining issues of CLTM	LG Electronics Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502018	Conditional intra-CU LTM	Qualcomm Incorporated	discussion

R2-2502033	Discussion on conditional Intra-CU LTM	Fujitsu	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502067	Discussion on conditional LTM	Xiaomi	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502118	Conditional LTM.	Interdigital, Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502119	Remaining issues of Conditional LTM	Rakuten Mobile, Inc	discussion	Rel-19

R2-2502161	Discussion on conditional LTM	vivo	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502237	Discussion on conditional intra-CU LTM	China Telecom	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502287	Remaining open issues for CLTM	NEC	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502289	Discussion on Conditional intra-CU LTM remaining issues	Baicells Technologies Co. Ltd	discussion

R2-2502312	Discussion on conditional Intra-CU LTM	Apple	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502342	Discussion on conditional intra-CU LTM	ZTE Corporation, Sanechips	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502383	Discussion on issues for supporting conditional LTM	Sharp	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502396	CLTM Scenarios and remaining points	Lenovo	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502399	Discussion on coexistence of conditional LTM and LTM	ITRI	discussion	NR_Mob_Ph4-Core

R2-2502425	Discussion on Early TA handling	Transsion Holdings	discussion	Rel-19

R2-2502438	Discussion on conditional LTM	Spreadtrum, UNISOC	discussion	Rel-19

R2-2502446	Discussion on Conditional LTM	Ericsson	discussion	NR_Mob_Ph4-Core

R2-2502603	Discussion on open issues for conditoinal LTM	Ofinno, LLC	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502728	Discussion on Conditional LTM	CMCC	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502748	Remaining issues on conditional LTM	ETRI	discussion	Rel-19

R2-2502756	Discussion on Conditional intra-CU LTM	ITL	discussion	Rel-19

R2-2502800	Remaining Issues of Conditional Intra-CU LTM	Samsung	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502824	Discussion on early TAT handling for CLTM	ASUSTeK	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502846	Considerations on CLTM TAT handling 	Kyocera 	discussion	Rel-19

R2-2502878	On conditional LTM	Nokia	discussion	Rel-19	NR_Mob_Ph4

R2-2502945	Intra-CU conditional LTM	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core

## 8.7	XR Enhancements Ph3

(NR_XR_Ph3-Core; leading WG: RAN2; REL-19; WID: RP-243318)

Time budget: 2 TU

Tdoc Limitation: 4 tdocs

### 8.7.1	Organizational

LS, Rapporteur input, workplan, running CRs etc.Including initial input/CRs from R19 XR UE capabilities rapporteur.

WI rapporteur input

R2-2502275	Rapporteur Inputs	Nokia, Qualcomm (Rapporteurs)	discussion	Rel-19	NR_XR_Ph3-Core

Noted

Incoming LS(es)

R2-2501754	LS reply on multi-modality awareness (S2-2502465; contact: CMCC)	SA2	LS in	Rel-19	XRM_Ph2, NR_XR_Ph3-Core	To:RAN2, RAN3, SA	Cc:SA4

Noted

R2-2501757	Reply to LS on Application-Layer FEC Awareness at RAN (S4-250252; contact: Qualcomm)	SA4	LS in	Rel-19	NR_XR_Ph3-Core, FS_5G_RTP_Ph2, FS_XRM_Ph2	To:RAN2, SA2	Cc:RAN3

Noted

Running CRs

R2-2501761	Introduction to R19 XR enhancements	Qualcomm Incorporated	draftCR	Rel-19	38.321	18.5.0	NR_XR_Ph3-Core

R2-2501950	PDCP running CR for R19 XR	LG Electronics Inc. (Rapporteur)	draftCR	Rel-19	38.323	18.5.0	NR_XR_Ph3-Core

R2-2502089	Draft runnnig RRC CR for R19 XR	Huawei, HiSilicon	draftCR	Rel-19	38.331	18.5.1	B	NR_XR_Ph3-Core

R2-2502162	RLC Running CR for XR	vivo	draftCR	Rel-19	38.322	18.2.0	B	NR_XR_Ph3-Core

R2-2502276	Draft Stage 2 CR for XR	Nokia (Rapporteur)	draftCR	Rel-19	38.300	18.5.0	B	NR_XR_Ph3-Core

All the CRs above are endorsed as baseline for further changes

Discussion on running CRs

R2-2501951	Discussion of [POST129][511][XR] PDCP running CR	LG Electronics Inc. (Rapporteur)	report	Rel-19	NR_XR_Ph3-Core

Noted

Proposal: Discuss the following issues in the RAN2#129bis meeting.

1.	Whether the RAN2 agreement “non-delay critical data ahead of delay critical data” applies to non-delay-reporting data ahead of delay-reporting data.

2.	Whether to use COUNT to describe the non-delay-critical data ahead of delay-critical data.

3.	Whether to use “PDU Set remaining time” to describe the shortest remaining time till discardTimer expiry among the PDCP SDUs belonging to the PDU Set.

4.	Whether to keep the text “The dsr-ReportingThresholds configured for the PDCP entity are ordered in ascending order.” in the PDCP specification.

5.	Whether to specify delay-reporting indication in the PDCP specification or leave it up to UE implementation.

6.	Whether to specify update of delay-reporting indication in the PDCP specification or leave it up to UE implementation.

- LGE clarifies issue 1 and 2 can be discussed based on Tdocs. We can discuss point 3-6 now.

On bullet 3:

- Xiaomi thinks the definition is not needed.

On bullet 4:

- OPPO thinks this should be captured in RRC. Sharp, Samsung agrees, as otherwise some impact on UE cannot be avoided.

- Huawei thinks does not think we need such restriction in RRC as long as it is used properly in PDCP.

- Vivo supports capturing this in PDCP, no strong view on RRC, but see no strong motivation.

- Nokia is also OK to keep this in PDCP, but we also need some more clarifications on how data is calculated.

- Huawei wonders if UE will reject configuration if they are not ordered properly.

- Nokia thinks that if we capture in RRC, then in PDCP we can just capture that UE calculates from lowest threshold.

On bullet 5:

- CMCC agrees to have this specified.

Ob bullet 6:

- Apple thinks this should be ensured by the UE implementation, no need to specify.

- LGE thinks that we should at least specify that the UE makes sure that the data that is indicated is up to date.

- ZTE thinks that it would be good to specify this at least for DL, e.g. for CU-DU split. Lenovo indicates this is for DSR so only UL.

- Huawei wonders why we have an initial indication but no update? Lenovo thinks it is obvious it needs to be up to date.

- Apple thinks that is already specified in MAC that the data refers to PUSCH start time. LGE thinks that with multiple thresholds some description is needed in PDCP.

Agreements related to PDCP CR - In PDCP specifications, use “PDU Set remaining time” to describe the shortest remaining time till discardTimer expiry among the PDCP SDUs belonging to the PDU Set. - Keep the text “The dsr-ReportingThresholds configured for the PDCP entity are ordered in ascending order.” in the RRC specification. - Specify delay-reporting indication in the PDCP specification. - Do not specify explicitly an update of delay-reporting indication in the PDCP. FFS (when discussing the CR) whether some explanation/note can be added to clarify that the UE should ensure the indicated data volume is up to date.

R2-2502091	Summary for [POST129][510][XR] RRC running CR (Huawei)	Huawei, HiSilicon	discussion	Rel-19	NR_XR_Ph3-Core

Noted

Potentially easy to agree:

Proposal3: Introduce RRC configuration to enable/disable the fallback to default priority in the 2nd stage of LCP

Proposal4: Introduce RRC configuration to enable/disable the inclusion of non-delay critical data ahead of delay critical data in the buffer size calculation for DSR

Proposal5: The maximum number of entries in the dsr-ReportingThresList is 4.

Proposal6: For the available data rate query MAC CE, confirm on the following:

	Introduce a prohibit timer for the UL transmission of the data rate query MAC CE

	Enable/disable the rate query MAC CE by the presence of the prohibit timer in the RRC configuration

Proposal7: The granularity of the bit rate query prohibit timer (QoS flow level or DRB level) should be the same as that of bit rate recommendation. FFS based on further progress on bit rate recommendation.

Need further discussion:

Proposal1: Discuss whether the entries of drs-ReportingThresholdList need to be configured in ascending order.

Proposal2: Determine the name of new R19 DSR from two options have been proposed currently: (a) multiple entry DSR; (b) enhanced DSR.

- QCM asks whether there will be different UE behaviour depending on the configuration? Nokia thinks we have a rule that no capability should be defined without network configuration.

- Vivo thinks we should revisit P4 after we make the decision for the priority fallback. Xiaomi thinks this is different as for this one we can already agree.

Agreements related to RRC CR - Introduce RRC configuration to enable/disable the inclusion of non-delay critical data ahead of delay critical data in the buffer size calculation for DSR - The maximum number of entries in the dsr-ReportingThresList is 4. - For the available data rate query MAC CE, confirm on the following: - Introduce a prohibit timer for the UL transmission of the data rate query MAC CE - Enable/disable the rate query MAC CE by the presence of the prohibit timer in the RRC configuration - FFS the details of prohibit timer, e.g. granularity

UE capabilities

R2-2501880	UE capabilities for XR	Xiaomi	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 1: RAN2 to discuss whether to define separate UE capabilities for different XR features (as in Rel-18 XR), or to define a generic Rel-19 XR capability which can include multiple features.

Proposal 2: RAN2 to confirm that AS UE capability for multi-modality is not needed in Rel-19.

Proposal 3: An optional UE capability (e.g. lcp-PriorityAdjustment-r19) is introduced to indicate the support of dynamic logical channel priority based on delay status of buffered data. FFS whether a UE supporting this feature shall also indicate support of delayStatusReport-r18.

Proposal 4: RAN2 to discuss the capability handling regarding fallback to default priority in the 2nd round of LCP, e.g. whether it is a capability with or without signalling.

Proposal 5: An optional UE capability (e.g. enhancedDelayStatusReport-r19) is introduced to indicate the support of enhanced delay status report of the buffered data associated with multiple thresholds. A UE supporting this feature shall also indicate support of delayStatusReport-r18.

Proposal 6: An optional UE capability (e.g. delayStatusReportNonCriticalData-r19) is introduced to indicate the support of including non-delay critical data ahead of delay critical data in the buffer size calculation for DSR. A UE supporting this feature shall also indicate support of enhanced delay status report.

Proposal 7: An optional UE capability (e.g. autonomousRLC-Retx-r19) is introduced to indicate the support of autonomous RLC retransmission based on delay status. The capability does not have pre-requisites.

Proposal 8: An optional UE capability (e.g. enhancedPolling-r19) is introduced to indicate the support of enhanced polling based on delay status. The capability does not have pre-requisites.

Proposal 9: Separate UE capabilities are introduced for UE Tx and Rx operations for unnecessary retransmission avoidance.

Proposal 10: An optional UE capability (e.g. supportOfStopRetxObsoleteSDU-r19) is introduced to indicate the support of stopping retransmission of obsolete SDUs. The capability does not have pre-requisites.

Proposal 11: An optional UE capability (e.g. rlc-Rx-DiscardTimer-r19) is introduced to indicate the support of RLC receiver timer to abandon obsolete SDUs. The capability does not have pre-requisites.

Proposal 12: An optional UE capability (e.g. ul-RateControl-r19) is introduced to indicate the support of UL rate control MAC CE from the gNB to the UE. The capability does not have pre-requisites.

Proposal 13: If working assumption is confirmed, an optional UE capability (e.g. ul-RateQuery-r19) is introduced to indicate the support of bit rate query message from the UE to the gNB. A UE supporting this feature shall also indicate support of UL rate control MAC CE.

Proposal 14: Rel-19 XR UE capabilities defined by RAN2 are per UE, not FDD-TDD DIFF, not FR1-FR2 DIFF.

DISCUSSION on P1:

- OPPO, QCM, Ericsson thinks we should have separate capabilities per feature, not combined one.

- Nokia asks if we can now have a R19 XR UE which does not support any XR feature. Nokia thinks there should be a minimum set of UE capabilities that XR UE supports.

- Ericsson indicates we have the same situation in Rel-18 and we can follow this.

- Ofinno thinks we only need to define relations between capabilities at a later stage (after we define capabilities).

- Nokia would like to move away from how we define capabilities so far, if not now then we will have to re-discuss in 6G. CMCC share the concern.

- ZTE understands the concern from Nokia, but thinks that whether to have a minimum set should be discussed at plenary.

- LGE indicates that we specify rather independent features and it is hard to define a minimum set.

- Huawei wonders what “XR UE” means, is it another UE category?

- Sharp also believes we should have separate capabilities.

- Ericsson agrees with LGE and Huawei. Ericsson does not see any specific requirements for XR UEs.

- vivo thinks it is too late to discuss this, we should have done this from Rel-18.

- QCM thinks we only define UE type if we have mandatory behaviours for the UE which is not the case for XR.

- Nokia thinks that we risk that the feature will never be deployed if we do not specify anything. The problem will not go away if we do not address it in 3GPP, then other bodies will have to do it.

DISCUSSION on P3:

- QCM does not think R19 LCP needs to be dependent on R18 DSR. Lenovo, Samsung, OPPO, Ericsson agrees.

On P5:

- LGE, QCM thinks we do not need to couple R18 and R19 DSR. Vivo thinks reasonable UE will implement R18 if implements R19, but does not want to couple in specs.

- Huawei does not think it is possible to support R19 without R18

On P12:

- QCM thinks we should have separate capabilities for the UL and DL MAC CEs.

On P14:

- QCM thinks that DSR capability in DC configurations can be band combination dependent.

Agreements on UE capabilities - RAN2 assumes that we define separate UE capabilities for different XR features (as in Rel-18 XR). - RAN2 to confirm that AS UE capability for multi-modality is not needed in Rel-19. - An optional UE capability with signalling (e.g. lcp-PriorityAdjustment-r19) is introduced to indicate the support of dynamic logical channel priority based on delay status of buffered data. No dependency on support of delayStatusReport-r18. - An optional UE capability with signalling (e.g. enhancedDelayStatusReport-r19) is introduced to indicate the support of enhanced delay status report of the buffered data associated with multiple thresholds. FFS A UE supporting this feature shall also indicate support of delayStatusReport-r18. - An optional UE capability with signalling (e.g. autonomousRLC-Retx-r19) is introduced to indicate the support of autonomous RLC retransmission based on delay status. The capability does not have pre-requisites. - An optional UE capability with signalling (e.g. enhancedPolling-r19) is introduced to indicate the support of enhanced polling based on delay status. The capability does not have pre-requisites. - An optional UE capability with signalling (e.g. ul-RateControl-r19) is introduced to indicate the support of UL rate control MAC CE from the gNB to the UE. The capability does not have pre-requisites. FFS whether there is a separate UE capability for UL rate query. - For the above Rel-19 XR UE capabilities defined by RAN2 are per UE, not FDD-TDD DIFF, not FR1-FR2 DIFF.

### 8.7.2	Multi-modality support

No contributions are expected for this AI for RAN2#129bis

### 8.7.3	RRM measurement gaps/restrictions related enhancements

No contributions are expected for this AI for RAN2#129bis

R2-2501803	Discussion on Measurement Gap enhancements	OPPO	discussion	Rel-19	NR_XR_Ph3-Core

### 8.7.4	Scheduling enhancements

#### 8.7.4.1	LCP enhancements

Further details of handling of the additional priority for LCH, e.g. configuration details, impact on Bj, impact on SR priority determination during intra-UE prioritization.

Bj enhancement

R2-2501838	Discussion on LCP enhancements	HONOR	discussion	Rel-19	NR_XR_Ph3-Core

Noted

Proposal 1: Do not support an LCH with an upgraded priority to be transmitted even if Bj is negative, while the remaining time is less the configured threshold.

R2-2502934	LCP enhancements	Ericsson	discussion	Rel-19	NR_XR_Ph3-Core

Noted

Proposal 1	RAN2 to agree on the FFS that all LCH priority adjusted data can be transmitted from a LCH, despite Bj being negative, before moving to the next LCH in the LCP procedure.

Proposal 2	The behaviour in Proposal 1 should be controlled by network configuration, as the wanted behaviour depends on the traffic served and LCP parameters configured.

DISCUSSION:

- QCM thinks Bj enhancement is not needed and priority adjustment is sufficient.

- LGE supports Ericsson proposal. It may happen that due to big burst of data Bj may become negative and if this data is urgent, it should be sent. Fairness over long-term period will be met anyway.

- Huawei agrees with LGE and Ericsson. In current LCP we can already include data if Bj is negative in 2nd phase.

- OPPO does not think this is fundamental, but is OK with considering Bj negative case.

- Xiaomi agrees with QCM, they think this is equivalent to PBR enhancement. We already apply additional priority in 2nd round, so there should be no issue.

- Apple thinks we should either do nothing or have proper solution. Negative Bj will not solve the issue. E.g. we if an LCH has small positive Bj, then they will be in worse situation.

- Nokia agrees there is no need to do anything. Network can conifgure PBR properly. Sharp agrees.

- Sharp thinks there is an issue with fairness.

- vivo agrees with LGE on fairness, so there is no technical concern with Bj enhancement.

- IDT agrees with LGE and Ericsson.

- Fujitsu also agrees with Ericsson proposal. This is simpler than PBR adjustment and the specificaitons impact is very small.

- Samsung thinks this will happen very often, so the optimization is not needed.

- Lenovo also thinks this is an optimization, can follow majority.

Working assumption (to be revisited next meeting): No Bj enhancement is introduced.

It will only be considered provided that gains are proven and that we have a common understanding on the change that is needed in specifications

Intra-UE prioritization – SR priority

R2-2502163	Remaining issues on LCP enhancements for XR	vivo	discussion	Rel-19	NR_XR_Ph3-Core

Noted

Proposal 2	Additional LCH priority does not impact SR priority determination.

R2-2502202	LCP enhancements for XR	ZTE Corporation, Sanechips	discussion

Proposal 3: For “LCH priority-adjusted data”, the additional LCP priority is also used for SR priority determination.

Noted

DISCUSSION:

- Samsung agrees with ZTE, Samsung clarifies that we just using additional priority, but it is not identifying a specific channel.

- OPPO supports vivo’s proposal as additional priority is determined during LCP, so it is not known for SR. Think there are many additional topics to discuss, e.g. whether we use the same threshold.

- LGE indicates we already agreed that we do not consider additional priority for triggering SR, so we do not have to use it in this case as well.

- CATT supports ZTE proposal.

- Xiaomi agrees that there are additional spec impacts and prefers not to consider add priority for SR.

- IDT also supports vivo proposal, it is OK to just use it for LCP and no need to enhance URLLC.

- Lenovo also supports vivo view, the gain is unclear.

- QCM thinks triggering and priority determination for intra-UE prioritization is different thing. Thinks that it is useful to adjust the priority of SR so that UE has bigger chance to get a grant from the NW.

- Huawei supports ZTE and thinks the spec impact is small.

- Ericsson, Nokia agrees with Lenovo, it is hard to understand the gain.

- Sharp agrees with ZTE. We agreed to use it for grant, so it is straightforward to reuse it for SR priority determination.

- CMCC is afraid that we might need to study it for BSR as a next step.

- Apple thinks that for SR vs grant conflict, the UE can include urgent data in this grant. For SR vs SR conflict, this does not seem to be a big problem.

- Huawei indicates that such grant may not be used because we have LCH restrictions and in this case urgent data cannot use it. In those cases SR should be prioritized. ZTE and Samsung agree.

- Sharp thinks that intra-UE prioritization happens per MAC entity, what Huawei mentioned is not a rare.

- LGE thinks there is additional aspect of how to determine the time point of SR priority determination.

We will check the impact of using additional LCP priority for SR priority determination during intra-UE prioritization in specifications and make a final decision on whether to have it based on that, i.e. it should be simple enough.

Intra-UE prioritization – others

R2-2502579	remaining details on Intra-UE prioritization / LCP enhancements	Lenovo	discussion	Rel-19	NR_XR_Ph3-Core

Noted

Proposal 1: RAN2 to agree that it should be explicitly specified which priority, e.g. “default” or additional priority, to use for a LCH in order to determine the priority of the corresponding UL grant.

Proposal 2: RAN2 to agree that UE should use for the retransmission case the additional priority of a LCH for determining the priority of a corresponding UL grant only if the additional priority was used during the complete LCP, i.e. first and second round of LCP.

Proposal 3:  RAN2 to agree that UE should use for the initial transmission case the priority that will be used for a LCH during first round of LCP for determining the priority of the corresponding UL grant.

Proposal 4: RAN2 to agree that UE shall apply same rules for the LCH priority being used for determining the priority of a HARQ process as for determining the priority of an UL grant.

DISCUSSION:

- Lenovo clarifies P4 is for shared spectrum, but it can be easily supported.

- Fujitsu supports P4, we should not exclude shared spectrum usage.

- OPPO is not sure whether we need to support shared spectrum for XR, but they are OK, if companies want it.

It should be explicitly specified which priority, e.g. “default” or additional priority, to use for a LCH in order to determine the priority of the corresponding UL grant, i.e. highest priority used/to be used during LCP.

For shared spectrum case, UE shall apply same rules for the LCH priority being used for determining the priority of a HARQ process as for determining the priority of an UL grant.

Understanding in RAN2 is that we will not address any other NR-U specific issues to support XR.

2nd round priority fallback capability

R2-2502265	LCP Enhancements for XR	Nokia, Nokia Shanghai Bell	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 5: RAN2 to choose from one of the following options:

-	Option #1: No capability

-	Option #1a: the additional LCH priority is always applied to both the 1st and 2nd round of the LCP procedure,

-	Option #1b: all UEs support fallback to default priority in the 2nd round of the LCP procedure

-	Option #2: capability without signaling

-	It is left for UE implementation whether to fallback to default priority in the 2nd round of LCP procedure

-	Option #3: capability with signaling

-	The NW is enabled to configure whether the UE shall fallback to default priority in the 2nd round of the LCP procedure

- QCM would like to have capability with signalling, but no configuration is needed. QCM think we can have a configuration which is always enabled.

- Ofinno thinks that it can be enabled by default but the network should eb able to disable it.

- Vivo still wonders why we need NW configuration if anyway it is always enabled. Xiaomi agrees.

- LGE thinks we should have NW configurability and no need to restrict NW behavior.

- CMCC thinks it should be up to NW to configure or not. Lenovo agrees. Lenovo thinks we need the NW to be able to enable/disable the feature. ZTE agrees the NW should contril this, the UE should not reject the configuration if the NW does not indicate to use this.

After offline discussion on the following: “FFS capability with signaling and the NW is allowed to configure whether the UE shall fallback to default priority in the 2nd round of the LCP procedure (offline- Nokia)”:

- Nokia reports that it seems all companies are OK with supporting option 1b from their paper, i.e. all UEs support fallback to default priority in the 2nd round of the LCP procedure.

All UEs falls back to default priority in the 2nd round of the LCP procedure if there is no LCH-priority adjusted data left, i.e. no capability is needed (this reverts previous RAN2 agreement on this).

Agreements on LCP enhancements - Working assumption (to be revisited next meeting): No Bj enhancement is introduced. - It will only be considered provided that gains are proven and that we have a common understanding on the change that is needed in specifications - We will check the impact of using additional LCP priority for SR priority determination during intra-UE prioritization in specifications and make a final decision on whether to have it based on that, i.e. it should be simple enough. - It should be explicitly specified which priority, e.g. “default” or additional priority, to use for a LCH in order to determine the priority of the corresponding UL grant, i.e. highest priority used/to be used during LCP. - For shared spectrum case, UE shall apply same rules for the LCH priority being used for determining the priority of a HARQ process as for determining the priority of an UL grant. - Understanding in RAN2 is that we will not address any other NR-U specific issues to support XR. - All UEs falls back to default priority in the 2nd round of the LCP procedure if there is no LCH-priority adjusted data left, i.e. no capability is needed (this reverts previous RAN2 agreement on this).

R2-2501762	Discussion on LCP enhancements	Qualcomm Incorporated	discussion	Rel-19	NR_XR_Ph3-Core

R2-2501815	Discussion on LCH priority adjustment for XR	OPPO	discussion	Rel-19	NR_XR_Ph3-Core

R2-2501870	Consideration on LCP Enhancement	CATT	discussion	Rel-19	NR_XR_Ph3-Core

R2-2501946	Discussion on LCP Enhancements	Sharp	discussion	Rel-19	NR_XR_Ph3-Core

R2-2502008	Discussion on LCP enhancements of XR traffic	Xiaomi Communications	discussion

R2-2502034	Discussions on enhancements for LCH priority-adjusted data	Fujitsu	discussion	Rel-19	NR_XR_Ph3-Core

R2-2502180	Discussions on Delay-based LCP Enhancements	Apple	discussion	Rel-19	NR_XR_Ph3-Core

R2-2502295	Considerations on LCP enhancements for XR	NEC	discussion	Rel-19	NR_XR_Ph3-Core

R2-2502300	Discussion on additional priority based LCP enhancements in XR	Huawei, HiSilicon	discussion	Rel-19	NR_XR_Ph3-Core

R2-2502398	Discussion on enhanced LCP for XR	ITRI	discussion	NR_XR_Ph3-Core

R2-2502562	Considerations on LCP Enhancements	China Telecom	discussion

R2-2502583	LCP enhancement for XR	InterDigital	discussion	Rel-19	NR_XR_Ph3-Core

R2-2502718	Further consideration on LCP enhancement for XR	CMCC	discussion	Rel-19	NR_XR_Ph3-Core

R2-2502848	Remaining issues on LCP enhancement for XR	LG Electronics Inc.	discussion	Rel-19	NR_XR_Ph3-Core

R2-2502948	Finalising LCP design for XR Ph3	Samsung R&D Institute UK	discussion

#### 8.7.4.2	DSR enhancements

Further details of enhanced DSR configuration/procedure, data volume calculation etc..

Empty DSR issue

R2-2502007	Discussion on DSR enhancements of XR traffic	Xiaomi Communications	discussion

Noted

Proposal 3 To avoid the case that there is no delay-reporting data for DSR, at least one configured reporting threshold should be no lower than the DSR triggering threshold.

R2-2502932	DSR enhancements	Ericsson	discussion	Rel-19	NR_XR_Ph3-Core

Noted

Proposal 1	A trigger threshold can be configured with larger value than the range of the reporting thresholds.

Proposal 2	When a DSR is triggered but no data resides inside the reporting threshold range the multi threshold DSR can indicate the data volume above the reporting threshold range.

DISCUSSION:

- Nokia thinks sensible network implementation will ensure that there is data to report when the DSR is triggered.

- Samsung agrees and thinks this can be achieved with Xiaomi’s proposal. OPPO also agrees.

- ZTE also agrees with Xiaomi proposal, but not sure whether we need to capture it in field description.

To avoid the case that there is no delay-reporting data for DSR, at least one configured reporting threshold should be no lower than the DSR triggering threshold. This means we do not have to address “empty DSR” issue, but it is FFS whether we need to capture this restriction in specifications

C-PDU and retransmitted PDU

R2-2502181	On Data Volume Calculations for Rel-19 DSR	Apple, CATT, LG Electronics, OPPO, Samsung, Xiaomi	discussion	Rel-19	NR_XR_Ph3-Core

Noted

Proposal 1: For Rel-19 DSR data volume calculation, all PDCP/RLC control PDUs and retransmission data should always be associated to the smallest configured reporting threshold, which is the i-th dsr-ReportingThreshold with i=1 (i.e. Option 2).

Proposal 2: For cases where the calculated buffer size for the smallest configured reporting threshold only contains PDCP/RLC Control PDUs and/or retransmission data, RAN2 selects one of the following alternatives to set the remaining time field for this reporting threshold in the DSR MAC CE:

o	Alt. 1: Set the remaining time field in DSR MAC CE as 0;

o	Alt. 2: Set the remaining time field in DSR MAC CE as the i-th dsr-ReportingThreshold with i=1 (i.e. the smallest reporting threshold);

o	Alt. 3: Leave it to UE implementation.

R2-2501947	Discussion on DSR Enhancements	Sharp	discussion	Rel-19	NR_XR_Ph3-Core

Noted

Proposal 1	Consider Control PDU and PDUs/SDUs to be retransmitted as the data volume of the smallest reported dsr-ReportingThreshold for the PDCP entity.

Proposal 2	If there is no delay-reporting PDCP SDUs in a PDCP entity, Control PDU is not considered as PDCP data volume for DSR.

Proposal 3	If there is no delay-reporting PDCP SDUs in a PDCP entity, PDUs/SDUs to be retransmitted are not considered as PDCP data volume for DSR.

Proposal 4	RLC data volume calculation and PDCP data volume calculation on Control PDU and PDUs/SDUs to be retransmitted are consistent with each other.

DISCUSSION on first configured vs first reported threshold:

- QCM has no strong preference, but prefers a simpler option, with less conditions to check. We can use some special value to report and NW decides how to treat this data.

- LGE prefers Apple way. LGE think that with the proposal from Sharp the calculation of delay critical data will differ from R18 calculation and they prefer to have aligned method. Also, it will cause cross-layer interactions which should be avoided.

- Lenovo thinks we should not treat retransmitted PDUs differently from initially transmitted PDUs. It makes more sense to report them according to the remaining time. Apple clarifies that this is to align with R18 DSR where retransmitted data is always treated as delay critical data.

- IDT thinks that we do not specify anything special for retransmitted data and we should consider remaining time for this data.

- LGE clarifies that with the current PDCP specifications, retransmitted data is always delay critical data.

- CMCC agrees with Lenovo, even for retransmission we should consider remaining time.

- Xiaomi thinks that remaining time indicated in DSR only considers remaining time of new data.

- Huawei agrees with Lenovo view. Previously there was one threshold, so this is new case.

- OPPO thinks that considering the retransmission will complicate the UE behaviour a lot as the UE needs to recheck the remaining time.

- Nokia agrees with Lenovo, the point is to indicate the network what is the time left.

- Ericsson thinks that this is relate also with non-delay critical ahead of delay critical. Now we would also put the retransmitted data in the wrong threshold.

- Apple is mainly concerned about UE complexity, DSR is already very complex with multiple thresholds.

- QCM thinks that always configured works in all cases, but smallest reported does not. Nokia clarifies the DSR is triggered only if there is data to report.

During DSR data volume calculation the remaining time of retransmitted is not considered, i.e. it is always put either in the smallest configured or smallest reported threshold. FFS which one.

Non-delay critical data related clarifications

R2-2502849	Discussion on DSR enhancement for XR	LG Electronics Inc.	discussion	Rel-19	NR_XR_Ph3-Core

Noted

Proposal 1. Clarify RAN2#128 agreement as “the UE may also support including non-delay-reporting data ahead of delay-reporting data for buffer size calculation of Rel-19 DSR, based on the capability indication”

Proposal 2. Use COUNT value to determine non-delay-reporting data ahead of delay-reporting data for delay-reporting data volume calculation.

On P1:

- Xiaomi wonders whether we need to define non-delay reporting data? Vivo thinks we have a definition of delay-reporting data so all other data is non-delay reporting data.

On P2:

- Xiaomi thinks we can just say “data ahead of”. LGE thinks it is hard to specify this and interpret when implementing, so using COUNT is clearer.

- QCM thinks this proposal assumes a specific UE implementation, e.g. depends on when the UE assigns SN.

- Samsung thinks COUNT value is usable for this purpose. Since the data is delay critical, it should have SNs assigned already.

- Fujitsu does not think COUNT value is suitable. COUNT may not reflect an order in the buffer. Also some channels with higher priority should be treated as “ahead of delay reported data).

- IDT thinks maybe we over-specify this, while details can be left to UE implementation.

- Nokia thinks using COUNT value seems most straightforward.

- Mediatek has strong concerns on an impact on chipset implementation with this. Thinks it should be left to UE.

- LGE thinks it is hard to specify in PDCP specifications.

- Samsung thinks that with different implementations there is a fairness issue which is a big concern.

- Nokia also worries that the UE may not report it if it is left to UE implementation.

Clarify RAN2#128 agreement as “the UE may also support including non-delay-reporting data ahead of delay-reporting data for buffer size calculation of Rel-19 DSR, based on the capability indication” (the exact terminology to be discussed as part of CR review)

We will try to find a way to describe how the UE determines non-delay-reporting data ahead of delay-reporting data for delay-reporting data volume calculation. The aim is to have consistent UE behaviour to avoid fairness issues, but also consider different UE implementations.

Agreements on DSR enhancements - To avoid the case that there is no delay-reporting data for DSR, at least one configured reporting threshold should be no lower than the DSR triggering threshold. This means we do not have to address “empty DSR” issue, but it is FFS whether we need to capture this restriction in specifications - During DSR data volume calculation the remaining time of retransmitted is not considered, i.e. it is always put either in the smallest configured or smallest reported threshold. FFS which one. - Clarify RAN2#128 agreement as “the UE may also support including non-delay-reporting data ahead of delay-reporting data for buffer size calculation of Rel-19 DSR, based on the capability indication” (the exact terminology to be discussed as part of CR review) - We will try to find a way to describe how the UE determines non-delay-reporting data ahead of delay-reporting data for delay-reporting data volume calculation. The aim is to have consistent UE behaviour to avoid fairness issues, but also consider different UE implementations.

Coexistence with Rel-18 DSR

R2-2502751	Remaining Issues on DSR enhancements in Rel-19 XR	Samsung	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 2. RAN2 to confirm that enhanced DSR MAC CE is used if at least one LCG is configured with dsr-ReportingThresList.

Proposal 3. RAN2 to agree on the name of the new DSR MAC CE as Enhanced DSR MAC CE.

Proposal 4. RAN2 to confirm that, enhanced DSR MAC CE inclusively serves the LCG(s) not configured with reporting threshold, but with pending DSR, and its field descriptions should reflect it.

DSR/BSR cancellation issues

R2-2502301	Discussion on DSR enhancements in XR	Huawei, HiSilicon	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 5:	RAN2 to specify that all triggered BSRs can be cancelled if amount of data to be reported in the BSR is already indicated by the DSR included in the same MAC PDU, as given in Appendix.

Proposal 6:	In Dual Connectivity case, a MAC entity can cancel the pending DSR if the volume of delay-critical data associated with the DSR is zero even though no MAC PDU including PDCP SDUs associated with the DSR was transmitted.

R2-2501763	Discussion on DSR enhancements	Qualcomm Incorporated	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 1.	For Rel-19, define an optional UE capability, which is per band combination, for the support of canceling pending DSRs in both MAC entities when an associated DSR MAC CE is sent in one of the MAC entities.

Proposal 2.	For UEs that support the capability defined in Proposal 1, network configures UE whether it shall cancel pending DSRs in both MAC entities when an associated DSR MAC CE is sent in one of the MAC entities.

R2-2501816	Discussion on DSR enhancements for XR	OPPO	discussion	Rel-19	NR_XR_Ph3-Core

R2-2501839	Discussion on DSR enhancements	HONOR	discussion	Rel-19	NR_XR_Ph3-Core

R2-2501871	Consideration on DSR Enhancement	CATT	discussion	Rel-19	NR_XR_Ph3-Core

R2-2502035	Discussions on DSR enhancements	Fujitsu	discussion	Rel-19	NR_XR_Ph3-Core

R2-2502060	Remaining issues on DSR enhancements	TCL	discussion

R2-2502164	Remaining issues on DSR enhancements for XR	vivo	discussion	Rel-19	NR_XR_Ph3-Core

R2-2502204	DSR enhancements for XR	ZTE Corporation, Sanechips	discussion

R2-2502266	DSR Enhancements for XR	Nokia, Nokia Shanghai Bell	discussion	Rel-19	NR_XR_Ph3-Core

R2-2502296	Considerations on DSR enhancements for XR	NEC	discussion	Rel-19	NR_XR_Ph3-Core

R2-2502364	Enhanced delay status reporting for XR	Lenovo	discussion	Rel-19

R2-2502478	DSR enhancements for UL scheduling	InterDigital	discussion	Rel-19	NR_XR_Ph3-Core

R2-2502563	Remaining Issues on DSR Enhancements	China Telecom	discussion

R2-2502719	Further consideration on DSR enhancement for XR	CMCC	discussion	Rel-19	NR_XR_Ph3-Core

R2-2502877	Discussion on XR DSR enhancements	III	discussion	NR_XR_Ph3-Core

### 8.7.5	RLC enhancements

Further details of autonmous retransmission and enhanced polling mechanisms (e.g. timer to be used) and unnecessary retransmission avoidance.

Unnecessary retransmissions

R2-2502218	On RLC Status report after discarding outdated AMD PDU	Futurewei	discussion	Rel-19	NR_XR_Ph3-Core

Noted

Proposal. The UE shall not autonomously send a Status Report when t-RxDiscard expires.

R2-2501840	Discussion on RLC enhancements	HONOR	discussion	Rel-19	NR_XR_Ph3-Core

Noted

Proposal 1: When the t-RxDiscard expires and all SDUs maintained by t-Reassembly are outdated or delivered to upper layer, the expiration of t-RxDiscard can trigger an SR.

Proposal 2: If it is agreed that the t-RxDiscard expiration can trigger an SR in some cases, RX_Highest_Status should be updated before triggering SR.

Proposal 3: In the case that all SDUs maintained by t-Reassembly are outdated or delivered to upper layer, t-Reassembly should be stopped or restarted.

DISUCSSION:

- Nokia agrees with FTW, there is no need for SR to be triggered.

- Sharp also agrees with FTW and the side effect can be that we trigger unnecessary retransmissions.

- Samsung prefers to trigger the SR and the prohibit timer will prevent unnecessary reTx.

- OPPO also agrees with FTW and Nokia.

- vivo would also like to trigger SR, but there is no need to combine t reassembly and t discard operation.

- Huawei thinks SR should be triggered to advance the window at Tx. T Rx discard should be shorter than t reassembly.

- LGE has similar understanding as Huawei, it is important to send SR to solve the window stalling problem. We should send the SR without additional conditions.

- Fujitsu agrees with Huawei and LGE.

- Ericsson agrees with LGE, but has not strong preference. It could also be solved by proper timer configurations.

- Lenovo also thinks we need SR after timer expiry.

- Apple indicates this is for packets with short PDB. There were previously concerns with Tx window stalling and SR is needed for that.

- QCM thinks SR is needed at least for some UEs.

- MTK thinks SR is not needed.

- IDT wonders what the benefit of not sending the SR is.

- FTW still has concerns with the agreement.

When the t-RxDiscard expires, the expiration of t-RxDiscard triggers an SR. FFS whether this is just usual SR or some changes are needed, or if UE implementation can decide (to be discussed during CR review)

PDCP or RLC timer for polling/retransmission

R2-2502036	Discussions on RLC enhancements	Fujitsu	discussion	Rel-19	NR_XR_Ph3-Core

Noted

Proposal 1: for autonomous retransmission and polling, the remaining time is determined based on discardTimer at PDCP.

R2-2502436	Discussion on timely RLC retransmission(s)	Spreadtrum, UNISOC	discussion	Rel-19

Noted

Proposal 1: Introduce a new timer in RLC layer for autonomous RLC retransmission.

Proposal 2: Discuss the following two options for the value of the new timer:

Option 1: value of new timer = PDCP remaining time

Option 2: value of new timer = PDCP remaining time – threshold

Proposal 3: The new timer for one RLC PDU should start when RLC Tx side receives the RLC SDU from PDCP layer.

Proposal 4: The new timer should terminate after RLC Tx side received positive acknowledgement of the corresponding RLC PDU.

DISCUSSION:

- Nokia thinks there has to be a possibility to use HARQ reTx for a PDU, so the timer needs to be at RLC layer and started when PDU goes to MAC layer.

- QCM, Huawei agrees with Nokia.

- Huawei thinks the retransmission should be triggered only after initial transmission.

- CMCC also supports new timer at RLC layer and the length should be shorter than PDCP discardTimer.

- Lenovo would like to reuse PDCP timer as we do not know when PDCP submits SDU to lower layers.

- Samsung would also like to reuse PDCP timer, retransmission should be decided based on remaining time.

- Apple thinks there is much additional impact due to new timer, e.g. length, stopping etc. Xiaomi agrees.

- OPPO thinks we need to ensure that PDU is sent within PDB so we should reuse PDCP timer.

- LGE thinks it is a valid point that it’s implementation when to submit SDU to lower layers from PDCP. Discard timer expresses correct remaining time.

- Ericsson slightly prefers PDCP timer, the problem with RLC timer is that it can be non-synced with PDCP timer, so we may get unnecessary reTx.

- IDT thinks we need to somehow use discard timer anyway as it indicates how much remaining time there is.

- ZTE would slightly prefer RLC timer.

- Nokia thinks we need to avoid unnecessary retransmissions.

- Lenovo agrees we need some additional conditions to avoid too early reTx.

For autonomous retransmission and polling, the remaining time is determined based on discardTimer at PDCP. FFS whether/what additional conditions are needed to prevent too early and/or unnecessary retransmission

Further conditions for autonomous retransmission

R2-2502501	Remainning details on RLC AM enhancement	NEC	discussion	Rel-19	NR_XR_Ph3-Core

Noted

Proposal 5: Auto-Rx is triggered only for SDU pending for feedback, i.e., it should be neither pending for retransmission already nor positively acknowledged by a STATUS PDU. RAN2 consider the TP as shown in section 3.2 to merge auto-retransmission into retransmission.

R2-2502479	Discussion on RLC enhancements	InterDigital	discussion	Rel-19	NR_XR_Ph3-Core

Noted

Proposal 2: 	Autonomous retransmission is triggered for an RLC SDU (segment) provided that the original RLC SDU has been fully transmitted by lower layers.

Proposal 3: 	Autonomous retransmission is not triggered if the RLC SDU (segment) is already considered for retransmission (e.g., based on receiving negative acknowledgement).

DISCUSSION:

- QCM supports those conditions, but wonders how RLC knows when HARQ process is finished. Perhaps some timer is needed?

- IDT thinks that perhaps we can base on whether it was submitted to lower layers, not fully transmitted.

- Huawei supports first bullet, but second one is related to what we have already agreed. Huawei thinks it should be linked with PDU set.

- LGE would like to use “submitted to lower layers” as a baseline condition.

- LGE thinks the intention is to consider only those SDUs which are not already considered for retransmission.

- Ericsson is not sure for the second one.

Autonomous retransmission is triggered for an RLC SDU (segment) provided that the original RLC SDU has been submitted to lower layers.

After offline discussion on “FFS Autonomous retransmission is not triggered if the RLC SDU (segment) is already considered for retransmission”:

- IDT clarifies that companies seem to agree that we should only have one retransmission (either autonomous or normal). How this impacts specifications can be further checked.

Autonomous retransmission is not triggered if the RLC SDU (segment) is already pending for retransmission. FFS specifications impact.

Agreements on RLC enahncements - When the t-RxDiscard expires, the expiration of t-RxDiscard triggers an SR. FFS whether this is just usual SR or some changes are needed, or if UE implementation can decide (to be discussed during CR review) - For autonomous retransmission and polling, the remaining time is determined based on discardTimer at PDCP. FFS whether/what additional conditions are needed to prevent too early and/or unnecessary retransmission - Autonomous retransmission is triggered for an RLC SDU (segment) provided that the original RLC SDU has been submitted to lower layers. - Autonomous retransmission is not triggered if the RLC SDU (segment) is already pending for retransmission. FFS specifications impact.

Retransmission count and RLF detection impact

R2-2502972	Remaining issues for XR RLC AM enhancement	MediaTek Inc.	discussion	Rel-19	38.322	NR_XR_enh-Core

Proposal 2: RETX_COUNT should be increased by 1 due to autonomous retransmission.

R2-2501802	Discussion on RLC re-transmission related enhancements	OPPO	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 3	RETX_COUNT is not incremented for autonomous retransmission.

Proposal 4	RAN2 to discuss whether/how to solve RLC-based RLF miss-detection issue (i.e., maxRetxThreshold cannot be reached) caused by in-flight PDU discard enhancement.

R2-2502401	Discussion on RLC AM Enhancements 	CANON Research Centre France	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 1: Stopping the retransmission of a RLC PDU due to a remaining time condition shall not trigger an RLF.

Misc FFS points from the running CR

R2-2501872	Consideration on XR-specific RLC Enhancement	CATT	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 2: For the autonomous retransmission and legacy retransmission, they will be transmitted in the order SN of RLC SDU or segment, no further spec impact will be introduced.

Proposal 3:  Only a single enhanced polling will be triggered per RLC SDU.

R2-2502223	Remaining issues on RLC enhancements for XR	LG Electronics Inc.	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 4. Even if polling is included based on the remaining time, no special handling to avoid multiple polling is introduced.

Proposal 5. If the remaining time of an RLC SDU falls below a specified threshold, the RLC entity should include a poll even when the RLC SDU is retransmitted. (Related TP is given below)

Status reporting enhancements

R2-2501764	Discussion on RLC enhancements	Qualcomm Incorporated	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 1.	When sending a poll, the transmitter includes an indication whether it is a time-critical poll (i.e. triggered due to remaining time below a threshold).

Proposal 2.	Upon receiving a time-critical poll, the receiver triggers a time-critical status report without the restriction by t-StatusProhibit (or apply a shorter t-StatusProhibit).

Proposal 3.	When the remaining value of t-RxDiscard is below a configured threshold, the receiver sends a time-critical status report without the restriction by t-StatusProhibit (or apply a shorter t-StatusProhibit).

R2-2502491	Timely retransmissions for RLC AM	Sony, Canon	discussion	Rel-19	NR_XR_Ph3

Proposal 1: After Rx side receives polling information, the Rx side should bypass/ignore the t-Reassembly timer for any SDU or segments of SDU if the t-Reassembly timer is running when generating status report.

Proposal 2: For any SDU or segments of SDU with bypassed t-Reassembly timer or still stuck in the lower layer retransmissions should be reported as UNKNOWN status in the status report (SR) by indicating/ redefining the corresponding reserved bit as 1 (R = 1).

R2-2502277	RLC enhancements	Nokia, Nokia Shanghai Bell	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 2: to minimize autonomous retransmissions, allow the network to send frequent ACKs, by introducing a possibility to configure the UE not to consider SDUs for retransmission based on received NACKs (to eradicate the problem of premature NACKs).

UE capabilities

R2-2502673	Discussion on UE Capabilities for RLC AM Enhancements	Ericsson, Qualcomm Incorporated, ZTE Corporation, MediaTek Inc.	discussion	Rel-19

Proposal 1	Individual (optional) per-UE capabilities with signalling are defined for the autonomous retransmission and the polling enhancement feature.

Proposal 2	Define an (optional) per-UE capability with signalling for the Rx-side aspect, where an outdated SDU is abandoned based on a new RLC timer and the abandoned SDUs are positively acknowledged in an RLC status report.

Proposal 3	Define an (optional) per-UE capability with signalling for the Tx-side aspect, conditional on the UE capability for the Rx-side, where the Tx side stops transmissions for an outdated SDU based on an indication from the PDCP.

R2-2501881	RLC AM retransmission enhancements	Xiaomi	discussion	Rel-19	NR_XR_Ph3-Core

R2-2501948	Discussion on RLC Enhancements in XR	Sharp	discussion	Rel-19	NR_XR_Ph3-Core

R2-2502165	Discussion on RLC enhancement for XR	vivo	discussion	Rel-19	NR_XR_Ph3-Core

R2-2502182	Discussions on Fast RLC Retransmission	Apple, Ericsson	discussion	Rel-19	NR_XR_Ph3-Core

R2-2502183	Views on Avoidance of Unnecessary RLC Retransmissions	Apple	discussion	Rel-19	NR_XR_Ph3-Core

R2-2502203	RLC enhancements for XR	ZTE Corporation, Sanechips	discussion

R2-2502222	Discussion on RLC AM enhancements	Huawei, HiSilicon	discussion	Rel-19	NR_XR_Ph3-Core

R2-2502302	Further discussion on RLC retransmission for XR	Quectel	discussion

R2-2502365	AM RLC enhancement	Lenovo	discussion	Rel-19

R2-2502564	Consideration on RLC AM Enhancements	China Telecom	discussion

R2-2502710	Discussion on the left issue of RLC enhancements	CMCC	discussion	Rel-19	NR_XR_Ph3-Core

R2-2502853	Discussion on RLC enhancements	DENSO CORPORATION	discussion	Rel-19	NR_XR_Ph3-Core

R2-2502860	RLC Enhancements for XR	Samsung	discussion	Rel-19

R2-2502872	Remaining issues on timely RLC retransmission	TCL	discussion	Rel-19

### 8.7.6	XR rate control

Including details of per QoS flow indication, e.g. QoS flow ID in MAC CE or in RRC, new bit rate table design, whether multipliers are needed etc.

Rate indication details

R2-2501949	Discussion on Rate Control in XR	Sharp	discussion	Rel-19	NR_XR_Ph3-Core

Noted

Proposal 2	In Rel-19 UL Rate Control MAC CE, the size of bit rate field is 7 bits or 8 bits.

Proposal 3	Bit rate field supports the value range from 100kbps to 40Mbps.

Proposal 4	Bit rate in Rel-19 UL Rate Control MAC CE is a physical-layer bit rate.

Proposal 5	The value table of bit rate field uses an exponential distribution.

On P4:

- Nokia is OK with physical if we make it clear. LGE is also OK.

- Meta is not clear why we had PHY layer in legacy but prefers app layer.

- Ericsson thinks gNB measures physical layer, so it has to be physical. QCM agrees and indicates the gNB has no idea how to translate PHY bitrate to application layer bit rate. Vivo agrees.

- MTK thinks it should be app layer.

- Sharp think the situation is the same as in Rel-14, there is some interaction between app layer and lower layers, but MAC indicates PHY layer and we can follow this.

- Huawei think it should be app since it is indicated to and used at app layer.

- Ofinno also indicates that MAC actually clarifies how this is calculated.

R2-2502883	XR rate control	Nokia, Nokia Shanghai Bell	discussion	NR_XR_Ph3-Core

Noted

Proposal 1: Multiple tables with distinct ranges and granularities are specified for XR rate control.

R2-2502088	Discussion on XR rate control	Huawei, HiSilicon	discussion	Rel-19	NR_XR_Ph3-Core

Noted

Proposal 3: Define a rate control table with a smaller range, e.g. 100kbps ~ 1000kbps, with 1% exponential distribution.

Proposal 4: Use dynamic multipliers to extend the range of the recommended rate indication to cover the whole range of rate from 100kpbs to 40Mbps.

DISCUSSION on single vs. multiple tables vs a table with multiplier:

- Ericsson thinks another solution would be to have one table with linear distribution. Exponential does not work well for gradual increase.

- CATT thinks we should have an exponential distribution.

- LGE indicates that with 10 bits we can get 1% granularity Multiple tables and multiplier require additional bits as well. Single table with more bits is preferred instead.

- QCM would like a single table as we can achieve the same granularity, but we need more discussions on the details.

- Lenovo has similar views with Ericsson and also prefers a single table. Sony agrees.

- Xiaomi thinks the range is already clear and one table is sufficient. HONOR agrees.

- Nokia thinks with one table we can reduce the number of codepoints with multiple tables but is OK with single table. Nokia thinks no multiplier is needed.

- Futurewei thinks signalling needs to be adapted to what codec can use. Do not think 1% granularity is needed.

- Ericsson think we can first decide number of bits and then decide distribution. ZTE thinks we can first choose type of distribution and then decide the exact codepoints.

- Vivo thinks we should decide both together, propose exponential with 8-9 bits.

- Huawei thinks 1% requires 10 bits while 8 bits allows for 5% which should be sufficient. Exponential distribution is preferred.

- LGE agrees 8 bits should be sufficient and also prefers exponential distribution.

- Lenovo thinks SA4 said 10% is sufficient.

- ZTE thinks we should clarify no multiplier will be introduced. Huawei thinks this can impact the overhead, so would not like to exclude multiplier for now.

Bit rate in Rel-19 UL Rate Control MAC CE is a physical-layer bit rate.

We specify a single table.

As a starting point, we aim to have a table with 8 bits and exponential distribution and no multiplier. This can be subject to further agreements on MAC CE contents/design. Overhead will be considered.

Rate query

R2-2502037	Discussions on XR rate control	Fujitsu	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 3: The working assumption is changed to agreements on rate query MAC CE.

Proposal 4: After the generation or transmission of the rate query MAC CE, the triggered rate query is cancelled.

Proposal 5: The upper layer can cancel the pending rate query.

The working assumption is changed to agreements on rate query MAC CE.

R2-2502166	Discussion on XR rate control	vivo	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 6	NW configures a prohibit timer for each QoS flow to avoid frequent XR rate control query.

Proposal 7	One codepoint of bit rate (i.e., the maximum value of the recommeneded bit rate) is defined for the case of congestion relief; upon reaching this bit rate, the UE is allowed to send an XR rate control query for a proper bit rate in case that an XR rate control MAC CE has been received previously.

MAC CE granularity

R2-2501765	Discussion on XR rate control	Qualcomm Incorporated	discussion	Rel-19	NR_XR_Ph3-Core

Noted

Proposal 1.	Per-QoS-flow rate indication by RRC configuration is the baseline. Support for per-QoS-flow rate indication within MAC CE is an optional UE capability.

Proposal 2.	If rate indication for a DRB is based on RRC configuration, network configures UE with a ratio for allocating the indicated rate among QoS flows in the DRB.

Proposal 3.	Send an LS to SA2/SA4/RAN3 to inform them about Proposal 2 and request them to discuss and specify any remaining design details, if applicable.

R2-2502205	Data rate control for XR applications	ZTE Corporation, Sanechips	discussion

Noted

Proposal 1: If the understanding is that both solutions have RAN3 impact, then RAN2 should select the MAC based solution which seems to be the most flexible one (i.e. gNB to indicate the recommended bit rate on a per QoS level basis in the MAC CE)

Proposal 2: RAN2 should send an LS to RAN3 to inform them about the agreed solution and ask RAN3 to define any corresponding F1 signalling (if needed)

DISCUSSION:

- Meta support ZTE’s approach. Since there is similar impact on F1 from both solutions, it is straightforward to use MAC CE, it is up to gNB how to estimate rate.

- Nokia thinks RAN3 has no consensus on exact impact, Nokia thinks we need to wait for further information.

- Samsung supports ZTE’s approach. We can focus on what we want from R2 point of view and R3 can discuss impact.

- OPPO also supports MAC CE approach, both solutions have F1 impact. OPPO thinks we need to be able to adapt multiple flows in a single DRB.

- Xiaomi prefers combined approach. Xiaomi is concerned about the overhead of flow indication.

- CMCC prefers semi-static indication for QoS flow in RRC and then DRB is indicated in MAC CE.

- Lenovo prefers ZTE’s approach. There will be overhead with RRC solution as we may need to send reconfigurations via RRC.

- Ericsson indicates that we agreed that gNB can estimate QoS flow rate, but we did not discuss whether this is DU or CU which needs to be discussed by R3. If R3 decides it needs to be done in CU, then there is more F1 signalling. Would like to go with DRB indication as a baseline.

- Vivo thinks based on current feedback from R3 we should select an approach simpler from R2 side.

- Ofinno also thinks we should select per flow MAC CE, but we can make it conditional on R3 impact and can fallback to RRC solution based. Apple agrees, we can make a working assumption and if R3 concluseds otherwise, we can revisit.

- LGE thinks PDCP is an anchor point, so that is why both solutions require CU decision, so per flow is more straightforward, but we can also wait due to NW concerns.

- CATT supports ZTE’s solution and thinks it will help RAN3 and make more work for them.

- ZTE thinks RAN2 should make the decision and R3 should work on the solution accordingly, they did not indicate sth is not feasible. ZTE is afraid R3 cannot make the decision. We should make a decision on which is better from R2 point of view, but if this is not feasible then we can revert.

- QCM thinks that R3 assumed that there is F1 impact for RRC solution due to rate ratio indication, but this is not the only solution, but there are other solutions.

- Nokia thinks both solutions can control flow, MAC CE is more flexible, but it is not clear why we need such flexibility. Nokia does not see need for too much flexibility.

- LGE thinks the gNB has no possibility to indicate specific flow with RRC based solution.

- Ericsson thinks that RRC solution allows to use information from CN about the flows. Ericsson thinks we need at least LCID indication and then flow ID is on top.

- Ericsson thinks we need to discuss how we indicate QFI in MAC CE, there are different ways. E.g. DRB ID could be indicated and then QFI would be optional.

- Ericsson thinks we do not have to make working assumption and just wait for R3.

- Vivo thinks decision should be made by R2 as this is R2 feature and R3 can feedback us.

- Sharp thinks QoS flow ID indication is enough as one QoS flow maps to one DRB.

- Nokia thinks per flow indication has feasibility issues for UE chipset. Apple and Samsung do not see such issues.

From RAN2 point of view per flow indication in MAC CE is preferred, but there are concerns on F1 impact which needs to be verified by RAN3, so RAN2 will go with per flow approach unless R3 has issues with this.

FFS how QFI is indicated, e.g. with DRB ID + QFI ID.

Agreements on XR rate control - Bit rate in Rel-19 UL Rate Control MAC CE is a physical-layer bit rate. - We specify a single table. - As a starting point, we aim to have a table with 8 bits and exponential distribution and no multiplier. This can be subject to further agreements on MAC CE contents/design. Overhead will be considered. - The working assumption is changed to agreements on rate query MAC CE. - From RAN2 point of view per flow indication in MAC CE is preferred, but there are concerns on F1 impact which needs to be verified by RAN3, so RAN2 will go with per flow approach unless R3 has issues with this. - FFS how QFI is indicated, e.g. with DRB ID + QFI ID.

Single or multiple flow/DRB in MAC CE

R2-2502957	Discussion on Rate Control for XR	China Telecom	discussion

Proposal 2: One rate control MAC CE can comprise multiple flows, and the network can configure the maximum number of QoS flows per MAC CE via RRC.

Proposal 3: New MAC CE should be specified in Rel-19 to support Per-QoS flow granularity recommended bit rate.

Dual-connectivity issue

R2-2502298	Uplink rate control for XR	NEC	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 6: RAN2 to discuss how to apply the indicated recommended bit rate for a QoS flow when more than one indication is received from different cells or cell groups (e.g., during CA/DC).

R2-2501841	XR rate control	HONOR	discussion	Rel-19	NR_XR_Ph3-Core

R2-2501873	Discussion on XR Rate Control	CATT	discussion	Rel-19	NR_XR_Ph3-Core

R2-2501882	XR rate control	Xiaomi	discussion	Rel-19	NR_XR_Ph3-Core

R2-2502224	Remaining issues on rate control signaling for XR	LG Electronics Inc.	discussion	Rel-19	NR_XR_Ph3-Core

R2-2502461	XR Rate Control	Lenovo	discussion	Rel-19	NR_XR_Ph3-Core

R2-2502480	Discussion on UL congestion signaling	InterDigital	discussion	Rel-19	NR_XR_Ph3-Core

R2-2502492	Recommended bit rate based XR rate control	Sony	discussion	Rel-19	NR_XR_Ph3

R2-2502674	Views on XR Rate Control	Ericsson	discussion	Rel-19

R2-2502720	Further consideration on XR rate control	CMCC	discussion	Rel-19	NR_XR_Ph3-Core

R2-2502750	Discussion on UL rate control for Rel-19 XR	Samsung	discussion	Rel-19	NR_XR_Ph3-Core

R2-2502762	Discussion on XR rate control	OPPO	discussion	Rel-19	NR_XR_Ph3-Core

R2-2502889	Discussion on RAN Awareness and UL Rate Control for XR	Meta	discussion

R2-2502890	Discussion on XR rate control	MediaTek Inc.	discussion	Rel-19	38.321	NR_XR_enh-Core

## 8.8	NTN for NR Ph3

(NR_NTN_Ph3-Core; leading WG: RAN2; REL-19; WID: RP-243300)

LTE_TN_NR_NTN_mob, leading WG: RAN2, Rel-19 WID: RP-240924)

Time budget: 2 TU

Tdoc Limitation: 3 tdocs

### 8.8.1	Organizational

LS, Rapporteur input, including workplan, etc.

For the LTE_TN_NR_NTN_mob WI, including endorsed draft CRs from the WI spec rapporteurs.

Rapporteur inputs do not count towards the tdoc limitation.

Running CRs

R2-2502192	Stage 2 Running CR for NR NTN phase 3	THALES	draftCR	Rel-19	38.300	18.5.0	B	NR_NTN_Ph3-Core

To be reviewed in post meeting email discussion

R2-2502676	Running RRC CR for NR NTN phase 3	Ericsson	CR	Rel-19	38.331	18.5.1	5315	-	B	NR_NTN_Ph3-Core

To be reviewed in post meeting email discussion

R2-2502511	Discussion on NR NTN UE capabilities	Apple	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 1: Introduce an optional UE capability without signaling for ETWS geo-fencing.

- Vivo  thinks we can link to this the existing capability for ETWS, saying that the UE optionally supports also geo-fencing. Xiaomi agrees

- CATT thinks this is something new and then wonders if we can reuse the existing capability

- QC thinks this is also related to the discussion to support geo-fencing for TN

Come back in the next meeting

Proposal 2: Introduce an optional UE capability without signaling for intended service area provision for MBS broadcast service via NTN.

We add a sentence saying that the UE can optionally support intended service area provision for MBS broadcast service via NTN.

Proposal 3: No new UE capability is foreseen for regenerative payload.

Agreed

R2-2502512	Draft CR for Rel-19 NR NTN UE capabilities	Apple	draftCR	Rel-19	38.306	18.5.0	NR_NTN_Ph3-Core

To be reviewed in post meeting email discussion

Withdrawn

R2-2501770	Introduction of LTE TN to NR NTN Mobility UE Capability	vivo	CR	Rel-19	36.306	18.4.0	1900	3	B	LTE_TN_NR_NTN_mob-Core	R2-2501417

Agreements:

1.	We add a sentence saying that the UE can optionally support intended service area provision for MBS broadcast service via NTN.

2.	No new UE capability is foreseen for regenerative payload.

[Post129bis][304][R19 NR NTN] Stage2 CR (Thales)

Scope: discuss the running Stage 2 CR

Intended outcome: Endorsed CR

Deadline: long

[Post129bis][305][R19 NR NTN] RRC CR (Ericsson)

Scope: discuss the running RRC CR

Intended outcome: Endorsed CR and list of open issues

Deadline: long

[Post129bis][306][R19 NR NTN] MAC CR (Interdigital)

Scope: discuss the running MAC CR

Intended outcome: Endorsed CR

Deadline: long

[Post129bis][307][R19 NR NTN] 38.304 CR (ZTE)

Scope: discuss the running 38.304 CR

Intended outcome: Endorsed CR

Deadline: long

[Post129bis][308][R19 NR NTN] capability CR (Apple)

Scope: discuss the running capability CR

Intended outcome: Endorsed CR

Deadline: long

[Post129bis][314][LTE to NR NTN] RRC CR (CATT)

Scope: discuss the running RRC CR

Intended outcome: Endorsed CR

Deadline: long

### 8.8.2	Downlink coverage enhancements

Contributions should focus on RAN2 aspects of DL coverage enhancements due to extended SIB periodicity (up to 160ms), including e.g. possible SMTC impacts (while no contributions are expected on cell level / beam level DTX/DRX mechanism).

R2-2501798	Discussion on DL coverage enhancement	Xiaomi	discussion	Rel-19	NR_NTN_Ph3-Core

- Cell barring

Proposal 1: From SSB extension point of view, there is no need to bar legacy UE. From link level enhancement ( i.e. PDCCH CSS repetition, SIB1 PDSCH repetition, Msg4 PDSCH repetition) point of view, RAN2 can wait for RAN1 conclusion on whether the enhancements are backward compatible. Or RAN2 can send a LS to RAN1 on this.

-	Samsung thinks that link level enhancements will not require new access barring

-	Fujitsu thinks we should leave the possibility for the NW to bar legacy UEs in case of extended SSB periodicity

-	Samsung thinks that UEs not able to read SIB1 already assume the cell as barred. Furthermore there are no issues for UEs in connected mode.

From SSB extension point of view, RAN2 assumes there is no need to introduce new barring bits

We wait for further progress in RAN1 on link level enhancements before further discussing the possible impacts on access barring

- Cell reselection enhancements

Proposal 2: Postpone the discussion on the mechanisms to prevent legacy UE from reselecting to a DL CE cell until RAN2/RAN1 concludes on whether link level enhancements are backward compatible or not.

- SMTC aspects

  - Whether to configure different SMTC periodicity for neighbour cells with different SSB periodicity.

Proposal 3: Ask RAN4 whether to support separate per SMTC periodicity configuration for SMTCs in smtc4list-r17, considering both idle/inactive and connected mode.

-	Google is not sure this is essential

-	HW generally supports this at least for connected mode and thinks that RAN2 can decide on this. LG agrees. Ericsson also agrees

-	CATT thinks this would have RAN4 impact on RRM requirements. QC agrees

-	Samsung thinks there is no need for this for idle mode.

-	Fujitsu thinks the enhancements should apply for both connected and idle mode

-	Google thinks this can be solved by UE implementation

-	Samsung thinks the NW could provide neighbour cells SSB periodicity. Huawei thinks the current behaviour is per frequency, not per cell and then does not like this solution.

RAN2 considers to introduce signalling support for different SMTC periodicity and offset in the same frequency layer, for both idle and connected mode. Ask RAN4 for confirmation

Continue in offline 303 to discuss the wording of a possible LS to RAN4. Also discuss other possible SMTC enhancements for connected/idle mode

  - Whether to increase the size of SMTC list beyond 4

Proposal 4: The list size of SMTC is not extended.

  - Enhancement to SMTC mechanism

Proposal 5: No enhancement on SMTC mechanism is needed.

  - LS to RAN4

Proposal 6: send LS to RAN4 regarding the agreement on SMTC for confirmation.

- Link level coverage enhancements

Proposal 7: If msg4 PDSCH repetition is agreed to support, Msg3 is used to carry the indication of msg4 PDSCH repetition capability/request.

Proposal 8: RAN2 to agree on adding the following codepoints:

- CCCH of size 48 bits for PUCCH repetition of Msg4 HARQ-ACK & Msg4 PDSCH repetition, except for an (e)RedCap UE;

- CCCH of size 64 bits for PUCCH repetition of Msg4 HARQ-ACK & Msg4 PDSCH repetition, except for an (e)RedCap UE;

- CCCH of size 48 bits for PUCCH repetition of Msg4 HARQ-ACK & Msg4 PDSCH repetition of a RedCap UE;

- CCCH of size 64 bits for PUCCH repetition of Msg4 HARQ-ACK & Msg4 PDSCH repetition of a RedCap UE;

- CCCH of size 48 bits for PUCCH repetition of Msg4 HARQ-ACK & Msg4 PDSCH repetition of an eRedCap UE;

- CCCH of size 64 bits for PUCCH repetition of Msg4 HARQ-ACK & Msg4 PDSCH repetition of an eRedCap UE;

Proposal 9: RAN2 to agree on not adding the following codepoints:

- CCCH of size 48 bits for Msg4 PDSCH repetition of a RedCap UE;

- CCCH of size 64 bits for Msg4 PDSCH repetition of a RedCap UE;

- CCCH of size 48 bits for Msg4 PDSCH repetition for an eRedCap UE;

- CCCH of size 64 bits for Msg4 PDSCH repetition for an eRedCap UE;

- CCCH of size 48 bits for Msg4 PDSCH repetition, except for an (e)RedCap UE;

- CCCH of size 64 bits for Msg4 PDSCH repetition, except for an (e)RedCap UE;

and send LS to RAN1 to confirm the RAN2 understanding that Msg4 PDSCH channel is always better than PUCCH with Msg4 HARQ-ACK channel.

[AT129bis][303][R19 NR NTN] SMTC enhancements (Xiaomi)

Scope: discuss the wording of a LS to RAN4 regarding RAN2 intention to signalling support for different SMTC periodicity and offset in the same frequency layer, for both idle and connected mode. Also discuss other possible SMTC enhancements for connected/idle mode.

Intended outcome: summary of the offline discussion and Draft LS

Deadline for companies' feedback:  Wednesday 2025-04-09 20:00

Deadline for offline discussion summary and Draft LS:  Thursday 2025-04-10 08:00

R2-2503055	Report of [AT129bis][303][R19 NR NTN] SMTC enhancements	Xiaomi	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 1: RAN2 considers to take the following option for support for different SMTC periodicity and offset in the same frequency layer for both idle, inactive and connected mode, and asks RAN4 whether it is feasible to support this in Rel-19 time frame and provide feedback on the maximum number of periodicities, offsets, and together:

Option 1: support configuring different SMTC periodicities for SMTCs in one frequency layer.

RAN2 considers to support configuring two different SMTC periodicities (with different offsets) for SMTCs in one frequency layer for idle, inactive and connected mode.

We ask RAN4 whether it is feasible to support this in Rel-19 timeframe (also include previous agreement that at any time the UE will not use more SMTCs in parallel than in previous releases).

Proposal 2: Network can provide assistant information on the association between SMTC and location for UE to perform SMTC selection for idle/inactive mode. FFS on the details of location, e.g. serving cell SSB index, reference location, cell direction indication, etc.

We support configuring more than 4 SMTCs per frequency (e.g. 6) for idle/inactive UEs. It will be up to UE implementation to select which of the SMTCs to consider (send this RAN2 decision to RAN4 for checking)

Network can provide assistance information (for Rel-19 UEs, not necessarily supporting DL CE) on the association between SMTC and location to help UE to perform SMTC selection for idle/inactive mode. FFS on the details of location information, e.g. serving cell SSB index, reference location, etc. In any case it is up to UE implementation on how to utilize the assistance information for SMTC selection in idle/inactive mode.

R2-2503056	Draft LS on SMTC enhancements	Xiaomi	LS out	Rel-19	NR_NTN_Ph3-Core	To:RAN4

Revised in R2-2503067

R2-2503067	Draft LS on SMTC enhancements	Xiaomi	LS out	Rel-19	NR_NTN_Ph3-Core	To:RAN4

Add “, given that at any time the UE will not use more SMTCs/Gaps in parallel than in previous releases” in the introduction

Remove “but at any time the UE will not use more SMTCs/Gaps in parallel than in previous releases.” From the first agreement

Change e.g. 6 to i.e. 6

Remove Draft

Revised in R2-2503068

R2-2503068	LS on SMTC enhancements	Xiaomi	LS out	Rel-19	NR_NTN_Ph3-Core	To:RAN4

Approved

R2-2502524	Consideration on downlink coverage enhancements	ZTE Corporation, Sanechips	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 1: RAN2 discusses SMTC enhancements due to beam hopping for UE in all RRC states, and try to address this issue with a common solution if applicable.

Observation 3: Location and beam based SMTC selection achieve similar purpose of limiting UE used SMTC number, while beam based SMTC mechanism requires less signalling overhead and is more power saving friendly.

Observation 4: Opt3 (UE assistance information report) only works for connected mode, while different solutions are still needed for UE in idle/inactive mode for SMTC updates.

Observation 5: Original Opt4 assumes a common SMTC (e.g., same periodicity and offset) for all neighbors while UE measures each neighbor in different time instance which is only applicable when beam hopping mechanism is regular and periodically rotated among neighbor cells.

Observation 6: The same purpose of opt4 can be achieved with a more generic solution, i.e., time-based SMTC mechanism (opt4a), where a list of SMTC with different validity time to be used by UE in different time instance can be independently configured.

Observation 7: If UE based SMTC selection in connected mode is used, additional considerations on measurement gap/NW scheduling restrictions are needed.

Observation 8: For time-based SMTC selection, there is no issue for NW to provide the measurement gap configuration or make scheduling accordingly.

Observation 9: For beam based SMTC selection, there is no issue for NW scheduling restriction but the measurement gap configuration needs to configured in a beam specific method to match with the beam-specific SMTC configuration.

Observation 10: For location based SMTC selection, UE needs to report back the used SMTC in connected mode, which consumes more signalling overhead and delays in updating measurement gap for matching with the used SMTC at UE’ side.

Observation 11: Beam or time-based SMTC selection mechanism allows a unified solution for UE in all RRC states while the impact on measurement gap configuration in connected mode is more manageable.

Proposal 2: RAN2 performs down-selection of SMTC enhancements for all RRC states based on below options:

	Opt 2: Beam-based SMTC selection, where UE selects SMTCss based on used beams associated to SMTCs

	Opt4b: Time-based SMTC selection, where UE selects SMTCs based on measured time and the validity time of SMTCs.

Proposal 3: If beam-based SMTC selection is considered for connected mode, RAN2 considers enhancements on measurement gap configuration ( e.g., introducing beam-sepcific measurement gap configuration).

R2-2502678	DL coverage enhancements	Ericsson	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 1	RAN2 to discuss and clarify whether different SSB periodicities and SSB offsets are expected in the same frequency layer.

Proposal 2	Introduce a way to define 4 SMTCs with different periodicity and offset for a frequency layer.

Proposal 3	Discuss whether the new definition should be backwards compatible with pre-Rel-19 NTN UEs.

Proposal 4	Inter-frequency SMTC enhancements are not needed to support 160 ms default SSB periodicity.

Proposal 5	RAN2 to explore common solutions for RRC_IDLE/INACTIVE and RRC_CONNECTED so that the UE can be configured with the most appropriate measurement gaps and SMTCs for its current location.

Proposal 6	Cell DTX enhancements based on Rel-18 NES are not within the scope of Rel-19 downlink coverage enhancements.

R2-2502863	Discussion on Downlink Coverage Enhancements	CSCN	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 1: SSB index associated SMTC configuration at the cell level can be introduced for SMTC enhancement.

Proposal 2: SSB-specific SMTC configuration can be introduced to beneficial both UE and NW for measurement.

Proposal 3: If SSB-specific SMTC applied, UE should reacquire the SMTC configuration when SSB index changes.

Observation 1: Due to the SSB periodicity extension and beam hopping, gap configuration may be insufficient for DL-CE cells.

Proposal 4: RAN2 is recommended to discuss the method for accurate gap configuration for DL-CE cells.

Proposal 5: The gNB shall implement different configurations for UEs in different areas to avoid invalid configurations.

Proposal 6: A new NTN bar bit with the value {barred, notBarred} should be introduced to achieve selectively bar UEs supporting DL-CE.

Proposal 7: RAN2 may discuss on supporting beam level DTX and the UE behaviors in different period.

R2-2502513	DL coverage enhancement in NTN	Apple	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 1: Network enables Msg4 PDSCH repetition via RSRP threshold configuration in SIB19.

-	Ericsson thinks this is also being discussed in RAN1

Proposal 2: As baseline, introduces extra LCID code points when the LX field is set to 1. UE indicates its capability and/or request on Msg4 PDSCH repetition by transmitting Msg3 with those LCID code points:

•	Case 1: If UE has the capability of Msg4 repetition and the measured UE’s DL reference signal strength is lower than RSRP threshold, UE indicates its capability and request for Msg4 PDSCH repetition.

•	Case 2: If UE has the capability of Msg4 repetition and the measured UE’s DL reference signal strength is higher than RSRP threshold, UE does not indicate its capability and/or request for Msg4 PDSCH repetition.

•	Case 3: If UE does not have the capability of Msg4 repeition, UE does not indicate its capability and/or request of Msg4 repetition.

Proposal 3: RAN2 aims to support joint indication to request for both Rel-18 UL coverage enhancement of PUCCH repetition for Msg4 HARQ-ACK and Rel-19 DL coverage enhancement of Msg4 PDSCH repetition.

-	Ericsson thinks this is also being discussed in RAN1

Come back in the next meeting after further RAN1 progress

Agreements;

1.	From SSB extension point of view, RAN2 assumes there is no need to introduce new barring bits

2.	We wait for further progress in RAN1 on link level enhancements before further discussing the possible impacts on access barring

3.	RAN2 considers to support configuring two different SMTC periodicities (with different offsets) for SMTCs in one frequency layer for idle, inactive and connected mode. We ask RAN4 whether it is feasible to support this in Rel-19 timeframe (also include previous agreement that at any time the UE will not use more SMTCs in parallel than in previous releases).

4.	We support configuring more than 4 SMTCs per frequency (e.g. 6) for idle/inactive UEs. It will be up to UE implementation to select which of the SMTCs to consider (send this RAN2 decision to RAN4 for checking)

5.	Network can provide assistance information (for Rel-19 UEs, not necessarily supporting DL CE) on the association between SMTC and location to help UE to perform SMTC selection for idle/inactive mode. FFS on the details of location information, e.g. serving cell SSB index, reference location, etc. In any case it is up to UE implementation on how to utilize the assistance information for SMTC selection in idle/inactive mode.

R2-2501774	Discussion on DL Coverage in NTN	vivo	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2501804	Discussion on downlink coverage enhancement	LG Electronics Inc.	discussion	Rel-19

R2-2501842	Discussion on downlink coverage enhancement	HONOR	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2501974	NTN downlink coverage enhancements	Nokia, Nokia Shanghai Bell	discussion	NR_NTN_Ph3-Core

R2-2502038	Discussions on downlink coverage enhancement	Fujitsu	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2502048	Discussion on downlink coverage enhancements in NR NTN	ETRI	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2502057	Further discussion on downlink coverage enhancements	CATT	discussion	Rel-19

R2-2502072	Discussion on NR NTN downlink coverage enhancements	DENSO CORPORATION	discussion	NR_NTN_Ph3-Core

R2-2502195	Discussion on cell barring for NR NTN downlink coverage enhancements	THALES	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2502246	Further discussion of NR NTN coverage enhancement	China Telecom	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2502315	Downlink coverage enhancement for NTN	InterDigital, Europe, Ltd.	discussion	Rel-19

R2-2502328	Discussion on DL coverage enhancement for NTN	OPPO	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2502352	Further considerations on NR NTN DL-CE	Lenovo	discussion	Rel-19

R2-2502377	Discussion on Downlink Coverage Enhancements	Sharp	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2502397	Discussion on supporting location/time-based SMTC selection	ITRI	discussion	NR_NTN_Ph3-Core

R2-2502493	SMTC impacts due to NTN downlink coverage enhancements	Sony	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2502502	Downlink coverage enhancement	NEC	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2502629	Downlink coverage enhancements and different SMTCs	TOYOTA Info Technology Center	discussion	Rel-19

R2-2502652	Default extended SSB periodicity	Qualcomm Incorporated	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2502667	Discussion on Downlink Coverage Enhancement	Samsung	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2502739	Analysis on DL coverage enhancements due to extended SSB periodicity	CMCC	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2502839	Discussion on DL coverage enhancements	Huawei, HiSilicon, Turkcell	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2502870	Discussion on DL coverage enhancements	TCL	discussion	Rel-19

R2-2502947	Further discussion on NTN DL coverage enhancements	NERCDTV	discussion	Rel-19

### 8.8.3	Uplink Capacity/Throughput Enhancement

Contributions can be submitted on the possible RAN2 aspects of the agreements reached in RAN1.

R2-2502329	Discussion on Uplink Capacity Enhancement	OPPO	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 1	OCC related parameters, at least including OCC index and OCC length are indicated by NW.

Proposal 2	For CG type 1, the OCC related configuration for PUSCH should be configured to UE via ConfiguredGrantConfig.

Proposal 3	For CG type 2 and dynamic grant, how to configure the OCC related configuration for PUSCH can wait for R1 progress.

Proposal 4	Define separate UE capabilities for OCC length 2 and OCC length 4 and the UE supporting the OCC length 4 shall support OCC length 2.

-	Ericsson thinks that for all the proposals in this AI we can wait for a RAN1 LS.

-	Xiaomi agrees. Samsung as well.

R2-2502525	Consideration on uplink capacity enhancements	ZTE Corporation, Sanechips	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 1: RAN2 designs higher layer signalling to provide OCC configuration for both DG and type 1/2 CG. Detailed OCC parameters (e.g., OCC sequence index) needed can wait for more input from RAN1.

Proposal 2: If OCC configurations are provided in RRC signalling:

- For DG, PUSCH-Config is used

- For type 1 CG, configuredGrantConfig is used

- For type 2 CG, configuredGrantConfig is used, except for DCI relevant configurations (if any), which is included in PUSCH-Config.

Proposal 3: Confirm RAN1 agreements that separate UE capabilities are introduced for OCC length 2 and OCC length 4, where UE capability for OCC length 2 is a prerequisite for UE capability for OCC length 4.

R2-2502856	Discussion on UL Capacity and Throughput Enhancement	Nokia, Nokia Shanghai Bell	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 1: Introduce separate UE capabilities for OCC with length 2 and OCC with length 4. The UE capability for OCC length 2 is a prerequisite for UE capability for OCC length 4. The exact UE capability description can be finalized based on the RAN1 LS on feature list.

Proposal 2: The OCC sequence length and OCC sequence index should be indicated to UE via RRC and/or DCI. The exact solution can wait for further progress from RAN1.

Proposal 3: There is no need to introduce dedicated access control mechanism for OCC capable UEs.

-	HW agrees with p3. Xiaomi, Fujitsu and Oppo also agree

There is no need to introduce dedicated access control mechanism for OCC capable UEs

R2-2502699	Discussion on uplink capacity/throughput enhancement for NR NTN	CMCC	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 1: Kindly suggest RAN2 to discuss the access control mechanism for R19 UEs supporting OCC between the two options:

Option 1: introduce new bar bit in SIB1

Option 2: reuse the existing cellBarredNTN.

Proposal 2: Introduce 2 new UE capabilities to support OCC length 2 and OCC length 4 respectively, for OCC length 4, simultaneously supporting OCC length 2 is mandatory.

Proposal 3: It is proposed to introduce a new IE (e.g. occ-v19) including OCC parameters (e.g. occ-length and occ-index ) in PUSCH-Config to provide OCC related configuration for PUSCH as following:

R2-2502284	Discussion on Uplink Capacity/Throughput Enhancement for NTN	InterDigital, Europe, Ltd.	discussion	Rel-19

R2-2502612	Discussion on Uplink Capacity Enhancements	Huawei, HiSilicon, Turkcell	discussion	Rel-19	NR_NTN_Ph3-Core

Agreements;

1.	There is no need to introduce dedicated access control mechanism for OCC capable UEs

### 8.8.4	Support of Broadcast service

Contributions should address the signaling of the intended service area of a broadcast service.

R2-2502668	Discussion on Broadcast Service Area	Samsung	discussion	Rel-19	NR_NTN_Ph3-Core

- UE skipping MCCH acquisition

Proposal 1: Intended service areas of all MBS broadcast services of the current serving cell are included in the new SIBxx.

RAN2 understands the Intended service areas of all MBS broadcast services of the current serving cell that need to be geo-fenced will be included in the new SIBxx (no spec impacts)

Proposal 2: If UE is not in any intended service areas indicated in SIBxx, UE does not need to acquire MCCH.

If UE knows it’s not in any intended service areas of any MBS services the UE is interested into, the UE may not need to acquire MCCH

Observation 1: In case the intended service area of an MBS broadcast service covers the whole current serving cell, UE does not need to know the exact geographic area covered by the current cell, but only needs to know there are certain MBS broadcast service intended for the whole cell by reading the new SIBxx, and no matter where the UE locates in the cell, the UE acquires MCCH.

Proposal 3: In case the intended service area of an MBS broadcast service covers the whole current serving cell, discuss how to signal in SIBxx.

•	Option 1: signal the geographic area of the whole serving cell.

•	Option 2: indicate the corresponding intended service area by a one-bit flag.

If no intended service area is explicitly indicated (e.g. in SIBxx) for a MBS service the UE is interested into, existing behavior applies.

- Service continuity

Proposal 4: The new SIBxx includes ISAs of MBS broadcast services that are provided not only on the intra frequency but also on inter frequencies.

Proposal 5: For cell reselection, if UE can only receive an MBS broadcast service(s) by camping on a frequency and UE is in the ISA of that MBS broadcast service, UE may prioritize that frequency.

Proposal 6:  For cell reselection, if UE cannot receive an MBS broadcast service on a frequency or if UE not in the ISA of an MBS broadcast service which is provided on a frequency, UE may deprioritize that frequency.

Observation 2: In quasi-earth-fixed or earth-moving cells, the MBS broadcast service to an intended area is for a quite short duration due to satellite movement, which will lead to frequent updates of MBS broadcast service and the associate ISA and, thus, UE frequent acquisitions of SIB or MCCH.

Proposal 7: For a quasi-earth-fixed or earth-moving cell, NW can provide ISA for MBS broadcast services that are to be provided by the upcoming satellite(s) and indicate the time interval during which the service is going to broadcast.

- ETWS service area

Observation 3: ETWS primary notification SIB6 without segmentation can still include up to 367 octets for warning area coordinates, which is sufficient to signal a warning area to some accuracy extend. If needed, a more accurate warning area can be further signaled in ETWS secondary notification SIB7 with segmentation.

Proposal 8: The field warningAreaCoordinatesSegment is included in SIB6/SIB7 for ETWS primary/secondary notification to indicate Warning Area Coordinates IE.

-	NTT Docomo supports this

The field warningAreaCoordinates is included in SIB6 while the field warningAreaCoordinatesSegment is included in SIB7 for ETWS primary/secondary notification to indicate Warning Area Coordinates IE.

Proposal 9: The size of the included field warningAreaCoordinatesSegment should be ensured to fit into SIB6 message which has up to 2976 bits.

-	NTT Docomo thinks that CT1 is already discussing this

R2-2502946	Discussion on MBS broadcast over NTN	Huawei, HiSilicon, China Southern Power Grid, Turkcell	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 1: RAN2 discuss the following enhancements to allow UE to skip unnecessary MCCH acquisition:

Alt.1: The UE decides whether it is within the ISAs belonging to the serving cell, according to the new SIB or the SIB20, upon entering a new cell. If not, the UE does not perform MCCH acquisition. Besides, if there is new SIB update or the SIB20 update showing that the new add-in areas cover the UE’s location, the UE needs to perform MCCH acquisition.

Alt.2: The UE reads the MCCH when entering a new Cell. The UE decides to perform MCCH re-acquisition only when the MCCH change notification has 1 bit used to indicate that the mapping between the MBS session and ISA has changed.

Alt.3: The UE reads the MCCH when entering a new Cell. The UE outside the ISAs of interested MBS session monitor the MCCH change notification every new time interval introduced in SIBx, instead of every MCCH modification period.

Proposal 2:  Both current serving cell and neighbour cells MBS session specific intended service area ID(s) are indicated in the MCCH.

Proposal 3: To determine the ISA of an ISA ID indicated in MCCH of the serving cell for an MBS service in a neighbour cell, the UE can use the new SIB of the serving or of the neighbour cell.

Proposal 4: SIB21 can indicate the subset of ISAs used for each frequency and the UE needs not prioritize (RRC_IDLE) or indicate (RRC_CONNECTED) the frequency of an MBS service of interest if the UE is not in any of the ISAs used in the frequency that provides the service.

R2-2502039	Discussions on supporting broadcast service	Fujitsu	discussion	Rel-19	NR_NTN_Ph3-Core

Observation 1: Behaviours with respect to the initiation of MII procedure upon entering or leaving the area of broadcast service has already been defined.

Proposal 1: RAN2 confirms that, with respect to the MBS service, the term ‘broadcast service area’ refers to the ‘intended service area’.

Proposal 2: UE follows legacy specification to initiate MII procedure when it enters or leaves MBS intended service area.

Proposal 3: UE may skip monitoring MCCH when it is not in any of the ISAs in the new SIB.

Proposal 4: UE monitors changes of MCCH information from PDCCH when it is not in any ISA of interested MBS services but in at least one of the ISAs in the new SIB.

R2-2502700	Considerations on broadcast service for NR NTN	CMCC	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 1: It could be up to UE implementation to monitor or skip MCCH (re)acquisition based on the ISA included in the new SIB and its own location information.

Proposal 2: It is proposed to provide intended service area within each neighbor cell for each MBS broadcast service session in MBSBroadcastConfiguration.

Proposal 3: It is proposed to provide intended service area for each MBS broadcast service session in SIB21.

-	Nokia thinks it’s not so simple and might have impacts to other groups. Ericsson shares the same concerns.

-	CATT also has some concerns, also regarding the signalling overhead.

-	Lenovo supports this and thinks we can use the intended service area ID in SIB21. Apple and vivo agree

-	ZTE also supports this.

Continue in offline 304 (Apple)

Proposal 4: For the MBS Interest Indication initiation, we could just follow the legacy mechanism, further enhancement is not needed.

Proposal 5: For SIB6, introduce a new IE(e.g. warningAreaCoordinates) to provide a full warning area coordinates information.

Proposal 6: For SIB7, reuse warningAreaCoordinatesSegment in SIB8 directly.

[AT129bis][304][R19 NR NTN] Service continuity (Apple)

Scope: discuss the proposal to provide intended service area IDs for each MBS broadcast service session in SIB21, or other alternative approaches to support service continuity

Intended outcome: summary of the offline discussion

Deadline for companies' feedback:  Wednesday 2025-04-09 20:00

Deadline for offline discussion summary:  Thursday 2025-04-10 08:00

R2-2503057	Report of [AT129bis][304][R19 NR NTN] Service continuity	Apple	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 1:

WA: UE may prioritize the frequency for an interested service when UE can receive the service on the frequency and the UE is in intended service area associated with that frequency/service. Otherwise, UE may no longer prioritize the frequency.

FFS whether and how the association between intended service area and frequencies is acquired.

-	Nokia and Ericsson could be ok if we remove frequency

-	ZTE suggests to change to “the service provided in the frequency”

UE may prioritize the frequency(ies) for an interested service when UE can only receive the service on the frequency(ies) and the UE is in intended service area associated with the service provided in the frequency(ies). Otherwise, UE may de- prioritize the frequency.

- FFS whether and how the association between intended service area and frequencies is acquired.

R2-2501775	Remaining Issues on MBS Broadcast in NTN	vivo	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2501843	Discussion on the support of broadcast service	HONOR	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2502041	Discussion on providing MBS service area in NTN network	OPPO	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2502044	Discussion on the support of broadcast service in NTN	ETRI	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2502058	Further discussion on support of broadcast service in NR NTN	CATT	discussion	Rel-19

R2-2502064	Further Discussion on Support of MBS Broadcast Service	TCL	discussion

R2-2502247	The signaling design of service area for PWS and MBS	China Telecom	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2502353	MBS broadcast service continuity in NR NTN	Lenovo	discussion	Rel-19

R2-2502354	Further considerations on ETWS support in NR NTN	Lenovo	discussion	Rel-19

R2-2502376	Remaining issues on intended service area	Sharp	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2502514	Discussion on broadcast service over NTN	Apple	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2502526	Consideration on broadcast service ehancements	ZTE Corporation, Sanechips	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2502537	Discussion on the support of broadcast service	Xiaomi	discussion

R2-2502551	Remaining Aspects of MBS in Rel-19 NR NTN	Nokia, Nokia Shanghai Bell	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2502651	MBS broadcast service area information	Qualcomm Incorporated	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2502677	Support for broadcast services in NR NTN	Ericsson	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2502741	Discussion on support for broadcast service in NTN	LG Electronics Inc.	discussion

Agreements:

1.	RAN2 understands the Intended service areas of all MBS broadcast services of the current serving cell that need to be geo-fenced will be included in the new SIBxx (no spec impacts)

2.	If UE knows it’s not in any intended service areas of any MBS services the UE is interested into, the UE may not need to acquire MCCH

3.	If no intended service area is explicitly indicated (e.g. in SIBxx) for a MBS service the UE is interested into, existing behavior applies.

4.	The field warningAreaCoordinates is included in SIB6 while the field warningAreaCoordinatesSegment is included in SIB7 for ETWS primary/secondary notification to indicate Warning Area Coordinates IE.

### 8.8.5	Support of regenerative payload

Contributions, if any, should focus on the needed updates for Stage 2 description and on whether any other existing essential features (not considered so far) would be affected - and potentially need any modifications - in a regenerative payload architecture.

R2-2502885	Regenerative payload	Ericsson	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 1	Clarify that the 38.300 figure in 16.14.2.1 is for transparent payload.

Proposal 2	Consider the text proposal in section 4.

Stage 2 rapporteur is invited to submit a 38.330 CR to the next meeting including the RAN3 endorsed changes for regenerative payload so that we can check which additional changes might be needed.

Proposal 3	Specific configurations of common TA and Kmac in regenerative architecture are not captured in Stage 2.

Specific configurations of common TA and Kmac in regenerative architecture are not captured in the specs.

R2-2502494	Satellite switch with re-sync in regenerative payload	Sony	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2502630	Regenerative payload for NTN for NR Ph3	TOYOTA Info Technology Center	discussion	Rel-19

Agreements:

1.	Specific configurations of common TA and Kmac in regenerative architecture are not captured in the specs.

### 8.8.6	LTE to NR NTN mobility

Contributions, if any, should focus on any possible missing aspects for the support of idle mode mobility between LTE and NR NTN.

R2-2501776	Remaining Issues on LTE to NR NTN Mobility	vivo	discussion	Rel-19	LTE_TN_NR_NTN_mob-Core

Proposal 1: In case the satellite ID is not present in the redirection information for NR NTN and SIB33 is broadcast, the UE considers the cells on the frequency to be terrestrial cells.

Proposal 2: In case the satellite ID is present in the redirection information for NR NTN, but there is no corresponding satellite information for the satellite ID in SIB33, it is up to UE implementation whether to perform NTN neighbour cell measurements on the NTN cell.

Proposal 3: UE supporting the ntn-RedirectionNR-r19 shall also indicate the support of supportedBandListNR-SA-r15.

-	Xiaomi thinks there is no need to capture this in the specs.

-	ZTE supports this and this is related to the proposal in CATT’s paper

RAN2 understands that UE supporting the ntn-RedirectionNR-r19 shall also indicate the support of supportedBandListNR-SA-r15 (no need to capture this in the spec)

R2-2502056	Discussion on a remaining issue for LTE_TN_NR_NTN_mob WI	CATT	discussion	Rel-19

Proposal: RAN2 discusses whether/how to support the dedicated priority configuration for E-UTRA TN to NR NTN cell reselection, and down-selects from below options:

- Option #1: Introduce a new capability signaling specific for E-UTRA TN to NR NTN cell reselection. E-UTRAN that broadcasts NR satellite assistance info in SIB33 may configure dedicated priority for an NR NTN frequency, only if it receives this capability reported from the UE (based on NW implementation).

- Option #2: Extend the capability signaling for E-UTRA TN to NR NTN redirection to also indicate the support of IRAT E-UTRA TN to NR NTN cell reselection. E-UTRAN that broadcasts NR satellite assistance info in SIB33 may configure dedicated priority for an NR NTN frequency, only if it receives this capability common to E-UTRA TN to NR NTN redirection and cell reselection (based on NW implementation).

- Option #3: No UE capability signaling is needed to indicate support of E-UTRA TN to NR NTN cell reselection. E-UTRAN that broadcasts NR satellite assistance info in SIB33 may configure dedicated priority for an NR NTN frequency, only if it receives supported NR NTN band(s) indicated in UE capability signaling. FFS whether any clarification is needed in the Spec.

-	QC supports option 2. Apple also.

Extend the capability signaling for E-UTRA TN to NR NTN redirection to also indicate the support of IRAT E-UTRA TN to NR NTN cell reselection. E-UTRAN that broadcasts NR satellite assistance info in SIB33 may configure dedicated priority for an NR NTN frequency, only if it receives this capability common to E-UTRA TN to NR NTN redirection and cell reselection (based on NW implementation).

R2-2502690	Remaining issues on E-UTRAN to NR NTN mobility	Samsung	discussion	Rel-19	LTE_TN_NR_NTN_mob-Core

Proposal 1: The redirection to NR NTN information element is enclosed in an NTN specific CarrierInfoNR, which is introduced in the choice structure as nr-19.

Agreed

Proposal 2: RAN2 confirms that the following aspects apply to redirection to NR NTN (without specification impact):

-	AS security must be activated to redirect to NR

-	MPS redirection to NR NTN

Agreements:

1.	RAN2 understands that UE supporting the ntn-RedirectionNR-r19 shall also indicate the support of supportedBandListNR-SA-r15 (no need to capture this in the spec)

2.	Extend the capability signaling for E-UTRA TN to NR NTN redirection to also indicate the support of IRAT E-UTRA TN to NR NTN cell reselection. E-UTRAN that broadcasts NR satellite assistance info in SIB33 may configure dedicated priority for an NR NTN frequency, only if it receives this capability common to E-UTRA TN to NR NTN redirection and cell reselection (based on NW implementation).

3.	The redirection to NR NTN information element is enclosed in an NTN specific CarrierInfoNR, which is introduced in the choice structure as nr-19.

## 8.9	IoT NTN Ph3

(IoT_NTN_Ph3-Core; leading WG: RAN2; REL-19; WID: RP-243278)

Time budget: 1 TU

Tdoc Limitation: 3 tdocs

### 8.9.1	Organizational

LS, Rapporteur input, including workplan, etc.

Rapporteur inputs do not count towards the tdoc limitation.

Incoming LSs

R2-2501720	Reply LS on PWS support in RRC_CONNECTED for NB-IoT NTN (R1-2501613; contact: InterDigital)	RAN1	LS in	Rel-19	IoT_NTN_Ph3-Core	To:RAN2, RAN

Noted

R2-2501753	Reply LS on Satellite IDs for store-and-forward operation (S2-2502450; contact: CICT Mobile, CATT)	SA2	LS in	Rel-19	5GSAT_Ph3-ARC	To:RAN2	Cc:RAN3

Noted

Running CRs

R2-2502887	Draft Introduction of IoT NTN phase 3	Ericsson	draftCR	Rel-19	36.300	18.4.0	B	IoT_NTN_Ph3-Core

-	Xiaomi and Nokia has some concerns with naming the feature as CB-EDT as also normal EDT is based on RACH. ZTE agrees

-	Ericsson and QC thinks that referring to CB-msg3 is also not good and prefer CB-EDT or something similar

-	CATT thinks that if we carefully defined what CB-EDT is there should be no problem. HW agrees

We tentatively refer to the new procedure as CB-msg3-EDT in the specs (can come back)

To be reviewed in post meeting email discussion

R2-2501969	RRC Running CR for IoT NTN	Huawei, HiSilicon	draftCR	Rel-19	36.331	18.5.0	B	IoT_NTN_Ph3-Core

To be reviewed in post meeting email discussion

R2-2501979	Running CR for TS36.304 for IoT-NTN	Nokia 	draftCR	Rel-19	36.304	18.3.0	B	IoT_NTN_Ph3-Core

To be reviewed in post meeting email discussion

R2-2502768	MAC Running CR for Rel-19 IoT NTN	MediaTek Inc.	draftCR	Rel-19	36.321	18.4.0	B	IoT_NTN_Ph3-Core	R2-2501158

To be reviewed in post meeting email discussion

Agreements;

1.	We tentatively refer to the new procedure as CB-msg3-EDT in the specs (can come back)

[Post129bis][309][R19 IoT NTN] Stage2 CR (Ericsson)

Scope: discuss the running Stage 2 CR

Intended outcome: Endorsed CR

Deadline: long

[Post129bis][310][R19 IoT NTN] RRC CR (Huawei)

Scope: discuss the running RRC CR

Intended outcome: Endorsed CR and list of open issues

Deadline: long

[Post129bis][311[R19 IoT NTN] MAC CR (Mediatek)

Scope: discuss the running MAC CR

Intended outcome: Endorsed CR and list of open issues

Deadline: long

[Post129bis][312][R19 IoT NTN] 36.304 CR (Nokia)

Scope: discuss the running 36.304 CR

Intended outcome: Endorsed CR

Deadline: long

[Post129bis][313][R19 IoT NTN] capability CR (Qualcomm)

Scope: discuss the running capability CR

Intended outcome: Endorsed CR

Deadline: long

### 8.9.2	Support of Store & Forward

Contributions should focus on possible impacts to the radio interface.

R2-2502053	Discussion on RAN2 impacts for the support of S&F operation	CATT	discussion	Rel-19

- On MME-configured satellite list: Cell (re)selection enhancement

Observation 1: SA2 agreed that the MME-configured satellite list is used by the UE assist with saving power and determining which satellites to receive MT data/signalling.

Observation 2: As specified in the latest TS 23.401, there is some room left for the UE to decide which S&F satellite to access and it is specified that if the UE access a satellite that is not belonging to the S&F Monitoring List there is increased probability that it will not be able to complete the NAS procedure.

Observation 3: Following the current cell (re)selection procedure, the UE shall (re)select to/camp on a cell that meets related RSRP/RSRQ conditions and the NW configured priority. This leaves no room for UE implementation to further consider the MME-configured satellite list as specified in SA2 Spec, resulting in increasing UE access failure as highlighted by SA2 (in case of an access attempted to a satellite not in the MME-configured list).

Proposal 1: During cell reselection procedure, UE may prioritize frequency(ies)/cell(s) corresponding to the satellite(s) indicated in the MME-configured satellite list.

-	Google supports this. CMCC supports this but thinks this would apply to UEs supporting S&F

-	IDC thinks we don’t have cell specific priorities but only frequency priorities. Also what does it means that the UE prioritizes. IDC also thinks that NB-IoT does not support priority based cell reselection.

-	Nokia does not support this and thinks the observations do not refer to cell reselection

-	QC thinks p1 and p1a are in contradiction (p1 relates to MME configured satellite list)

-	Novamint supports the principle of p1 and p1a

-	Samsung wonders if there will be many satellites the UE can prioritize

-	Xiaomi thinks p1 makes sense if there are only S&F mode cells around.

Come back in the next meeting (the proponents could show the possible spec impact if we decide to go in this direction)

Proposal 1a: A frequency list and/or cell list associated with each satellite ID (indicated by the MME) are provided to the UE in system information.

- On MME-configured satellite list: Relaxation on IDLE mode tasks

Proposal 2: RAN2 discusses whether the UE may stop the IDLE mode tasks (e.g. cell (re)selection, paging monitoring, etc.), if the UE determines that it is out of coverage of all target satellite(s) indicated by MME. How the UE determines out of coverage of all target satellites is up to UE implementation.

Observation 4: To support the power saving operation for S&F operation in Proposal 2, satellite assistance information similar to that used for discontinuous coverage (i.e., footprint info/start serving time and the long-term valid ephemeris format (TLE set format)) needs to be provided by system information.

Proposal 2a: If Proposal 2 is agreed, introduce a separate list in system information to provide the satellite assistance information for S&F operation (i.e., used by UE to predict the coverage time of the satellites indicated by MME).

Proposal 2b: Take signalling format of SatelliteInfo-r17 in SIB32 as the baseline for assistance information of the satellite(s) indicated by MME for S&F operation.

- On transition time

Proposal 3: The UE AS indicates the information on transition time from current "S&F operation mode" to "normal mode" (if received) to the upper layers. Whether/how it is used by the upper layers is up to CT1. No other AS impact is expected.

-	Apple supports this but thinks we should also indicate the other direction. QC also agrees on the need for indication in the opposite direction.

-	vivo is ok with this. Nokia also

For UEs supporting S&F, the UE AS indicates the information on transition time (if any) from current "S&F operation mode" to "normal mode" (if received) to the upper layers. Whether/how it is used by the upper layers is up to CT1.

Proposal 4: Send LS to CT1, informing CT1 of RAN2 agreements related to the "S&F operation mode" to "normal mode" transition time. Take the draft LS in Annex as baseline.

-	HW is fine but thinks we should add additional info on why RAN2 agreed to introduce this

Agreed

Proposal 5: Do not introduce transition time to indicate when the current satellite operation mode will transit from normal operation to S&F operation.

-	Google thinks we should have such an indication redefining the time information we already agreed. Oppo agrees.

-	Lenovo also agrees we need such an indication

-	Samsung thinks that when we move to S&F there will be a system information update

-	Samsung thinks we should not reuse T-service for this. Nokia agrees. ZTE also

-	Honor thinks we need a new indication, different from T-service.

-	Samsung wonders what we use this for.

We introduce an indication in system information for the normal mode to S&F mode transition, at least for NAS use. FFS on the details (e.g. whether we can link this to other existing information). The information on transition time for the normal mode to S&F mode transition is sent from AS to NAS, and we inform CT1 about this in the LS

- On new release cause

Proposal 6: Do not introduce a new release cause when the NW wants to release a R19 UE to RRC_IDLE because the feeder link becomes unavailable.

-	vivo thinks we could ask CT1

-	Xiaomi supports p6

-	Samsung also supports p6

We don’t introduce any new release cause related to S&F operation

Proposal 7: Do not introduce a new release cause when the NW wants to release a R19 UE to RRC_IDLE because the service link becomes unavailable when the cell is operating in S&F mode.

R2-2502116	Store and Forward Cell Access Restrictions	Interdigital, Inc.	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 3: Introduce a flag to indicate whether t-Service represents the remaining time of normal mode before switching to S+F mode, or whether it represents the end of service (per legacy).

R2-2501818	Remaining issues on S&F operation in IoT NTN	ZTE Corporation, Sanechips	discussion	IoT_NTN_Ph3-Core

Proposal 1: Besides the agreed time information on transition from S&F operation mode to Normal mode, a duration of Normal mode could be provided to R19 UE, e.g., via SystemInformationBlockType31(-NB) message.

Proposal 2a: For quasi-earth fixed cell, it can be up to network’s implementation to set the t-ServiceStartNeigh in SystemInformationBlockType33(-NB) or t-ServiceStart in SystemInformationBlockType32(-NB) either the time when the neighbor satellite starts to operate in Normal mode or the time when the neighbor satellite starts serving the area.

Proposal 2b: The earth moving cell case can be further discussed based on the progress for quasi-earth fixed cell case.

Proposal 2c: It’s suggested to introduce a new time information when a neighbor satellite transits from S&F operation mode to Normal mode for each neighbor satellite in SystemInformationBlockType33(-NB).

Proposal 3a: For quasi-earth fixed cell, it can be up to network’s implementation to set the t-Service in SystemInformationBlockType3(-NB) the time when the serving satellite starts to operate in S&F mode or the time when the serving satellite stops serving the area.

Proposal 3b: The earth moving cell case can be further discussed based on the progress for quasi-earth fixed cell case.

Agreements:

1.	For UEs supporting S&F, the UE AS indicates the information on transition time (if any) from current "S&F operation mode" to "normal mode" (if received) to the upper layers. Whether/how it is used by the upper layers is up to CT1.

2.	We introduce an indication in system information for the normal mode to S&F mode transition, at least for NAS use. FFS on the details (e.g. whether we can link this to other existing information). The information on transition time for the normal mode to S&F mode transition is sent from AS to NAS

3.	We don’t introduce any new release cause related to S&F operation

[Post129bis][301][R19 IoT NTN] LS to CT1 on S&F mode indications to NAS (CATT)

Scope: Draft an LS to CT1 with meeting agreements on indication to NAS regarding transitions between normal mode and S&F mode operation

Intended outcome: Approved LS

Deadline: 1-week

R2-2503070	LS on S&F mode indications to NAS	RAN2	LSout	Rel-19	IoT_NTN_Ph3-Core	To:CT1

Discussed in [Post129bis][301]

=> Approved

R2-2501766	Discussion on Store and Forward operation	Xiaomi	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2501777	Further Discussion on S&F Operation	vivo	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2501844	Discussion on the Store and Forward satellite operation	HONOR	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2502046	RAN2 impacts for SF Operation	Nokia , Nokia Shanghai Bells	discussion

R2-2502068	Discussion on Store & Forward satellite operation	OPPO	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2502073	Discussion on Store & Forward operation	DENSO CORPORATION	discussion	IoT_NTN_Ph3-Core

R2-2502101	On the support for the S&F operation	Google	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2502245	Further consideration of IoT NTN Store & Forward	China Telecom	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2502355	Mode transition time for S&F operation	Lenovo	discussion	Rel-19

R2-2502426	Discussion on support of Store&Forward	Transsion Holdings	discussion	Rel-19

R2-2502503	Radio Interface Aspect of S&F	NEC	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2502515	Support of S&F operation in IoT NTN	Apple	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2502613	Further consideration on Store and Forward	Huawei, HiSilicon, Turkcell	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2502620	Discussion on Paging and Mode Switching	Toyota ITC	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2502622	CIoT UP solution for Store & Forward satellite operation	SHARP Corporation	discussion	Rel-19

R2-2502655	Switching of S&F mode	Qualcomm Incorporated	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2502679	Support for store and forward in IoT NTN	Ericsson	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2502685	IoT-NTN S&F Mode Change Indication	PANASONIC	discussion

R2-2502688	Further discussion on Store and Forward	Samsung	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2502698	Discussion on IoT NTN Store and Forward	CMCC	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2502769	RAN2 impact on S&F mode	MediaTek Inc.	discussion	IoT_NTN_Ph3-Core	R2-2501159

R2-2502805	Discussion on Store and Forward satellite operation	ETRI	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2502845	Discussion on Store and Forward satellite operation	ETRI, Korea University	discussion	Rel-19	IoT_NTN_Ph3-Core	R2-2502805

R2-2502825	Discussion on time information for S&F	ASUSTeK	discussion	Rel-19	IoT_NTN_Ph3-Core	R2-2500418

R2-2502871	Discussion on Store & Forward satellite operation	TCL	discussion	Rel-19

### 8.9.3	Uplink Capacity Enhancement

Contributions should focus on the possible enhancements to reduce the necessary uplink and downlink signaling to complete an EDT transaction (Msg3 transmission without msg1/RAR; efficient delivery of msg4 / RRCEarlyDataComplete).

Including outcome of email discussion [Post129][307].

R2-2502773	Report of [Post129][307][R19 IoT NTN] CB-msg3/CB-msg4	MediaTek Inc.	discussion

- CB-Msg3 configuration for both eMTC and NB-IoT

Proposal 1: RAN2 confirms that the start of DSA transmission window is aligned with the start of time domain resources for (N)PUSCH occasions.

-	ZTE supports this and p2 as well

The start of CB-msg3 EDT transmission window is aligned with the start of time domain (N)PUSCH resource.

Proposal 2: RAN2 to discuss whether DSA window length could be defined in unit of PUSCH resources periodicity.

Continue in the Post meeting email discussion/ next meeting

Proposal 3: For DSA transmission window, RAN2 to discuss whether the length and periodicity should always be the same. If yes, only one of the window length and window periodicity is needed (not both).

The CB-msg3 EDT transmission window length and periodicity may be different. FFS on possible signalling optimization in case the length and periodicity are the same.

Proposal 4: Send the LS to RAN1 on CB-Msg3 configurations for confirmation.

- CB-Msg3 configuration for eMTC

Proposal 5: For eMTC, introduce a new IE (e.g. CB-Msg3-ConfigSIB-r19) for shared resources configuration of CB-Msg3 in SIB2.

Agreed

Proposal 6: For eMTC, introduce MPDCCH configuration in shared resources configuration. The fields in IE PUR-MPDCCH-Config-r16 could be reused as baseline. Confirm with RAN1 on the detail parameters (e.g. whether additional narrow band is needed, whether TDD related parameters are needed).

For eMTC, introduce MPDCCH configuration in shared resources configuration. The fields in IE PUR-MPDCCH-Config-r16 could be reused as baseline. Confirm with RAN1 on the detail parameters (e.g. whether additional narrow band is needed).

We will not support TDD related parameters.

Proposal 7: For eMTC, introduce PUSCH configuration in shared resources configuration. The fields in IE PUR-PUSCH-Config-r16 could be reused as baseline. Confirm with RAN1 on the detail parameters. (e.g. whether we should preclude CE mode B parameters, whether pusch-CyclicShift-r16, pusch-NB-MaxTBS-r16, and locationCE-ModeB-r16 are needed, whether power parameters p0-UE-PUSCH-r19 and alpha-r19 should be modified, whether prb-AllocationInfo should be defined as a “set” format with intention to provide a set of or shared frequency-domain resources)

For eMTC, introduce PUSCH configuration in shared resources configuration. The fields in IE PUR-PUSCH-Config-r16 could be reused as baseline. Confirm with RAN1 on the detail parameters. (e.g. whether pusch-CyclicShift-r16, pusch-NB-MaxTBS-r16 are needed, whether prb-AllocationInfo should be defined as a “set” format with intention to provide a set of shared frequency-domain resources).

RAN2 assumes power ramping should be supported for CB-msg3-EDT (both eMTC and NB-IoT) should be supported and will ask RAN1 for confirmation and in case which parameters should apply

Proposal 8: For eMTC, introduce PDSCH configuration in shared resources configuration. It includes frequency hopping (as in pur-PDSCH-FreqHopping) and maximum TBS (as in pur-PDSCH-maxTBS). Confirm with RAN1 on the detail parameters.

For eMTC, check with RAN1 if anything is needed for PDSCH configuration in shared resources configuration

Proposal 9: For eMTC, introduce PUCCH configuration in shared resources configuration. The fields in IE PUR-PUCCH-Config-r16 could be reused as baseline. Confirm with RAN1 on the detail parameters.

Agreed

Proposal 10: Consider the TP in Annex A as a baseline for eMTC shared resources configuration. This could be further discussed in RRC running CR.

Agreed

- CB-Msg3 configuration for NB-IoT

Proposal 11: For NB-IoT, introduce a new IE (e.g. CB-Msg3-ConfigSIB-NB-r19) for shared resources configuration of CB-Msg3 in SIB2-NB. FFS whether this IE is needed in SIB22-NB for non-anchor carrier.

-	Xiaomi thinks we should not have a restriction for non-anchor carrier

For NB-IoT, introduce a new IE (e.g. CB-Msg3-ConfigSIB-NB-r19) for shared resources configuration of CB-Msg3 in SIB2-NB and SIB22-NB for non-anchor carrier.

Proposal 12: For NB-IoT, introduce below physical later parameters in shared resources configuration as below:

•	Number of resource units for NPUSCH (as in npusch-NumRUsIndex-r16)

•	Number of repetitions for NPUSCH (as in npusch-NumRepetitionsIndex-r16)

•	Set of subcarriers (similar to npusch-SubCarrierSetIndex but change it to a “set”), FFS parameter for 15kHz SCS is needed. FFS whether subcarriers are provided as a contiguous set.

•	MCS configuration for NPUSCH (as in npusch-MCS-r16). FFS parameter for 15kHz SCS is needed.

•	Power parameters (as in p0-UE-NPUSCH-r16 and alpha-r16). FFS whether update is needed.

•	PDCCH parameters (as in NPDCCH-ConfigDedicated-NB-r13)

•	The non-anchor carrier index for monitoring Msg4. If this field is absent, anchor carrier is assumed to be used.

•	NOTE: confirm with RAN1 is needed

For NB-IoT, introduce below physical layer parameters in shared resources configuration as below:

- •	Number of resource units for NPUSCH (as in npusch-NumRUsIndex-r16)

- •	Number of repetitions for NPUSCH (as in npusch-NumRepetitionsIndex-r16)

- •	Set of subcarriers (similar to npusch-SubCarrierSetIndex but change it to a “set”), FFS whether subcarriers are provided as a contiguous set.

- •	MCS configuration for NPUSCH (as in npusch-MCS-r16).

- •	PDCCH parameters (as in NPDCCH-ConfigDedicated-NB-r13)

- •	The non-anchor carrier index for monitoring Msg4. If this field is absent, anchor carrier is assumed to be used.

- •	NOTE: confirm with RAN1 is needed

Proposal 13: For NB-IoT, RAN2 assume that the periodicity of CB-Msg3 resource may be larger than H-SFN duration.

For NB-IoT, FFS whether periodicity of CB-Msg3 resource may be larger than H-SFN duration

Proposal 14: Consider the TP in Annex B as a baseline for NB-IoT shared resources configuration. This could be further discussed in RRC running CR.

Agreed

- Msg4 monitoring window and RNTI for DSA

Proposal 15: RAN2 to discuss the following options:

•	Option 1: Single Msg4 monitoring window. It starts at the end of Msg3 transmission window plus UE-eNB RTT. FFS processing time is needed or not.

•	Option 2: Multiple Msg4 monitoring windows. Each starts at the end of transmission of each replica plus UE-eNB RTT. FFS processing time is needed or not.

Proposal 16: RAN2 to discuss the following options:

•	Option 1: Single RNTI for MSG4 monitoring

o	Option 1a – The RNTI is derived on the resource of DSA transmission window.

o	Option 1b – The RNTI is preconfigured/predefined.

•	Option 2: Multiple RNTI for MSG4 monitoring. Each RNTI is derived on the resource of each replica.

-	Session chair thinks it’s a bundle: if we go for option 1 for p15 we should go for option 1 for p16 (and viceversa)

-	Xiaomi thinks the false alarm issue is the most important one.

-	Samsung thinks that with option 2 the overall monitoring window is longer and this is the biggest issue

-	QC and Samsung think that option 1 is the one that allows multiplexing multiple UEs in msg4. Ericsson agrees.

-	ZTE thinks that msg4 multiplexing should be an optimization but not the driver for this discussion.

Continue in offline 305

R2-2503058	Report of [AT129bis][305][R19 IoT NTN] CB-msg3-EDT – phase1	Mediatek	discussion	Rel-19	IoT_NTN_Ph3-Core

Observation 1: If typical DSA window length is larger than RTT and the UE consider to start monitor window after 1st replica + RTT, the UL/DL conflict issue should be taken into account.

Observation 2: Single v.s. multiple RNTI seems tradeoff between MSG4 multiplexing and false alarm. The traffic load should be consider for this design.

Proposal 1: Single Msg4 monitoring transmission window and Single RNTI. The UE starts a Msg4 monitoring window in the subframe containing the last (N)PUSCH repetition of the first replica plus UE-eNB RTT (FFS processing time). If the monitoring window is overlapping with replica, the UE prioritize the replica transmission.

For CB-msg3-EDT we adopt a Single Msg4 monitoring window and Single RNTI (the RNTI is derived on the transmit resource for the transmission window).

The lengths of the Msg3 transmission and Msg4 monitoring windows are configured by the network (in case of k=1 it will be possible to configure the parameters in a way to have the same behaviour as for normal Random Access procedure)

-	session chair suggests to limit the discussion on FFS whether

A) the Msg4 monitoring always starts at the end of Msg3 transmission window plus UE-eNB RTT (FFS NW/UE processing time is needed or not) or

B) We allow the network to configure one of the two options:

1)	The Msg4 monitoring starts at the end of Msg3 transmission window plus UE-eNB RTT (FFS NW/UE processing time is needed or not)

2) 	The Msg4 monitoring window starts in the subframe containing the last (N)PUSCH repetition of the first replica plus UE-eNB RTT (FFS NW/UE processing time) and is restarted after each replica plus UE-eNB RTT (FFS NW/UE processing time).  If the Msg4 monitoring window is overlapping with replica, FFS if the UE prioritize the replica transmission or monitoring

-	Nokia, ZTE and CMCC think that option 2 should be considered

-	Apple thinks we should not specify two options.

-	Ericsson thinks we should go for the simpler option (A). ESA agrees

-	HW also supports option B

-	NEC thinks A is sufficient. Oppo agrees. Viasat also agrees

The Msg4 monitoring starts at the end of CB-Msg3-EDT transmission window plus UE-eNB RTT (FFS NW/UE processing time is needed or not).

FFS it will also be possible for the NW to configure that the Msg4 monitoring window starts in the subframe containing the last (N)PUSCH repetition of the first replica plus UE-eNB RTT (FFS NW/UE processing time). To possibly resolve the FFS it needs to be clarified what happens if the Msg4 monitoring window is overlapping with replica, i.e. whether the UE prioritize the replica transmission or monitoring

- SA vs DSA

R2-2502099	Further consideration on UL capacity enhancement for IoT NTN	Nokia, Nokia Shanghai Bell	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 7: DSA is an optional sub-feature for CB-Msg3.

Proposal 8: CB-Msg3 resources can be shared between SA-capable UEs and DSA-capable UEs that support CB-Msg3.

Proposal 9: RAN2 to capture the UE behavior for CB-Msg3 SA and CB-Msg3 DSA UEs separately.

R2-2502886	UL capacity enhancements for IoT NTN	Ericsson	discussion	Rel-19	IoT_NTN_Ph3-Core

Observation 2	When DSA is supported, SA is a special setting of the DSA parameters.

R2-2502457	On procedures for contention-based Msg3	Samsung	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 8: DSA is a mandatory feature of contention-based Msg3/Msg4.

Both SA and DSA are mandatorily supported by UEs supporting CB-msg3-EDT

We will specify one single procedure to support both DSA and SA, i.e. SA is a special setting (k=1) of the overall procedure

For CB-msg3-EDT, the Transmission window can be configured by the network with a starting point (e.g. H-SFN offset), a window length, and a window periodicity (window length and periodicity could be the same).

- support of CB-Msg3 EDT for NB-IoT with 15kHz SCS and eMTC CE mode B / support of OCC

R2-2501767	Discussion on uplink capacity enhancements for IoT NTN	Xiaomi	discussion	Rel-19	IoT_NTN_Ph3-Core

- Applicability of CB-msg3 EDT

Proposal 3: Send LS to RAN1 asking whether CB-Msg3 EDT for NB-IoT with 15kHz SCS and eMTC CE mode B is supported or not in this release.

- CB-msg3 EDT with OCC

Proposal 4: RAN2 to not consider CB-msg3 EDT with OCC in this release.

-	MTK and Nokia support this, as this would require separate resources and because of the power imbalance issue. Ericsson agrees

-	QC thinks that if we separate resources there should be no problem. QC agrees with QC and we should support OCC.

R2-2502701	Further discussion on uplink capacity enhancement for IoT-NTN	CMCC	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 1: Provide different RSRP ranges for CB-msg3 with and without OCC respectively. And only when corresponding RSRP condition is fulfilled, CB-msg3 combined with OCC could be used.

Proposal 2: Support dedicated signaling to configure the TA validation parameters (e.g. TAT timer at least) for 15kHz SCS NB-IoT and eMTC CE mode B.

R2-2502516	Uplink capacity enhancement in IoT NTN	Apple	discussion	Rel-19	IoT_NTN_Ph3-Core

Msg4 delivery:

Proposal 1: RAN2 do not pursue TA validation solution for NB-IoT with 15kHz SCS and eMTC CE mode B if RAN1 has no further input.

-	HW thinks this would not work without TA validation.

We don’t specify support for eMTC CE mode B case (it will not be possible to signal resources to be used for this case)

We specify support for NB-IoT with 15kHz with no specific enhancements, leaving to NW implementation whether to implement this or not, accepting potential performance degradation.

R2-2502656	CB-Msg3 and Msg4 enhancements	Qualcomm Incorporated	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 1	Separate resources are configured for CB-Msg3 transmission using OCC.

If we will decide to support OCC for CB-msg3-EDT, separate resources will be used for non-OCC and OCC based transmission.

Agreements;

1.	Both SA and DSA are mandatorily supported by UEs supporting CB-msg3-EDT

2.	We will specify one single procedure to support both DSA and SA, i.e. SA is a special setting (k=1) of the overall procedure

3.	For CB-msg3-EDT, the transmission window can be configured by the network with a starting point (e.g. H-SFN offset), a window length, and a window periodicity (window length and periodicity could be the same).

4.	We don’t introduce support for eMTC CE mode B case (it will not be possible to signal resources to be used for this case)

5.	We specify support for NB-IoT with 15kHz with no specific enhancements, leaving to NW implementation whether to implement this or not, accepting potential performance degradation.

6.	If we will decide to support OCC for CB-msg3-EDT, separate resources will be used for non-OCC and OCC based transmission.

7.	The start of CB-msg3 EDT transmission window is aligned with the start of time domain (N)PUSCH resource.

8.	The CB-msg3 EDT transmission window length and periodicity may be different. FFS on possible signalling optimization in case the length and periodicity are the same.

9.	RAN2 assumes power ramping should be supported for CB-msg3-EDT (for both eMTC and NB-IoT) should be supported and will ask RAN1 for confirmation and in case which parameters should apply

(CB-Msg3-EDT configuration for eMTC)

10.	For eMTC, introduce a new IE (e.g. CB-Msg3-ConfigSIB-r19) for shared resources configuration of CB-Msg3 in SIB2.

11.	For eMTC, introduce MPDCCH configuration in shared resources configuration. The fields in IE PUR-MPDCCH-Config-r16 could be reused as baseline. Confirm with RAN1 on the detail parameters (e.g. whether additional narrow band is needed).

12.	We will not support TDD related parameters.

13.	For eMTC, introduce PUSCH configuration in shared resources configuration. The fields in IE PUR-PUSCH-Config-r16 could be reused as baseline. Confirm with RAN1 on the detail parameters. (e.g. whether pusch-CyclicShift-r16, pusch-NB-MaxTBS-r16 are needed, whether prb-AllocationInfo should be defined as a “set” format with intention to provide a set of shared frequency-domain resources).

14.	For eMTC, check with RAN1 if anything is needed for PDSCH configuration in shared resources configuration

15.	For eMTC, introduce PUCCH configuration in shared resources configuration. The fields in IE PUR-PUCCH-Config-r16 could be reused as baseline. Confirm with RAN1 on the detail parameters.

(CB-Msg3 configuration for NB-IoT)

16.	For NB-IoT, introduce a new IE (e.g. CB-Msg3-ConfigSIB-NB-r19) for shared resources configuration of CB-Msg3 in SIB2-NB and SIB22-NB for non-anchor carrier.

17.	For NB-IoT, introduce below physical layer parameters in shared resources configuration as below:

-	Number of resource units for NPUSCH (as in npusch-NumRUsIndex-r16)

-	Number of repetitions for NPUSCH (as in npusch-NumRepetitionsIndex-r16)

-	Set of subcarriers (similar to npusch-SubCarrierSetIndex but change it to a “set”), FFS whether subcarriers are provided as a contiguous set.

-	MCS configuration for NPUSCH (as in npusch-MCS-r16).

-	PDCCH parameters (as in NPDCCH-ConfigDedicated-NB-r13)

-	The non-anchor carrier index for monitoring Msg4. If this field is absent, anchor carrier is assumed to be used.

NOTE: confirm with RAN1 is needed

18.	For NB-IoT, FFS whether periodicity of CB-Msg3 resource may be larger than H-SFN duration

Agreements (part 2):

1.	For CB-msg3-EDT we adopt a Single Msg4 monitoring window and Single RNTI (the RNTI is derived on the transmit resource for the transmission window).

2.	The lengths of the Msg3 transmission and Msg4 monitoring windows are configured by the network (in case of k=1 it will be possible to configure the parameters in a way to have the same behaviour as for normal Random Access procedure)

3.	The Msg4 monitoring starts at the end of CB-Msg3-EDT transmission window plus UE-eNB RTT (FFS NW/UE processing time is needed or not).

4.	FFS it will also be possible for the NW to configure that the Msg4 monitoring window starts in the subframe containing the last (N)PUSCH repetition of the first replica plus UE-eNB RTT (FFS NW/UE processing time). To possibly resolve the FFS it needs to be clarified what happens if the Msg4 monitoring window is overlapping with replica, i.e. whether the UE prioritize the replica transmission or monitoring

R2-2502771	Discussion on CB-Msg3 procedure	MediaTek Inc.	discussion	IoT_NTN_Ph3-Core	R2-2501164

- TA validation

Proposal 1: RAN discusses the following options regarding the TA validation.

Option 1: NB-IoT with 15kHz SCS and eMTC CE mode B are precluded from the scope of CB-Msg3.

Option 2: All type of NB-IoT and eMTC UE are supported in CB-Msg3

Option 3: RAN2 send LS to RAN1 asking for progress.

- Details of DSA mechanism

Proposal 2: DSA transmission is an optional feature without UE capability signaling for UE supporting CB-Msg3.

Proposal 3: If UE only supports SA, the DSA configuration can be reused.

Proposal 4: The maximum TBS is specific to CE level.

Proposal 5: The DSA transmission window length is specific to CE levels.

- Msg4 monitor window

Proposal 6: The Msg4 response window configuration is specific to CE level.

Proposal 7: Multiple Msg4 monitoring windows is supported. Each window starts at the end of transmission of each replica plus UE-eNB RTT (FFS whether additional processing time is needed).

- CB-Msg3 RNTI

Proposal 8: Send LS to RAN1 to confirm that CB-Msg3 RNTI is used to scramble the contention-based Msg3.

Proposal 9: Multiple Msg4 monitoring windows. Each RNTI is derived on the resource of each replica.

Proposal 10: RAN2 confirm the working assumption that one CB-Msg4 can target multiple UEs simultaneously.

Proposal 11a: The Msg4s responding to the Msg3s from the same time domains, but the different frequency domains can be multiplexed in the same MAC PDU.

Proposal 11b: The time domain information of CB-Msg3 UL resource (e.g., subframe index, SFN) is taken into account for CB-Msg3 RNTI calculation. The frequency domain information (e.g., subcarrier index) is not.

Proposal 12a: For NB-IoT, CB-Msg3 transmission on non-anchor carrier shall be supported.

Proposal 12b: For NB-IoT, the carrier index of CB-Msg3 UL resource is taken into account for CB-MSg3 RNTI calculation.

- Details of Msg4

Proposal 13: A Msg4 for CB-Msg3 with only MAC CE(s) is allowed when RRC message is not mandatory.

Proposal 14: Do not support the use of L1 ACK as the Msg4 for the CB-Msg3.

Proposal 15: RAN2 to discuss whether and how the HARQ ACK/NACK is applied to CB-Msg4.

Proposal 16: For NB-IoT, if the Msg4 multiplex is used, the same subcarrier index used to transmit the Msg3 can be used to transmit the HARQ ACK/NACK of Msg4.

- Fallback to RRC Connected mode

Proposal 17: CB-Msg3 fallback to RRC connected mode is supported.

Proposal 18: A TAC MAC CE can be included in the Msg4 when fallback to RRC Connected mode.

Proposal 19: UE starts the time alignment timer when receiving Msg4 which indicates fallback to RRC Connected mode.

Proposal 20: New C-RNTI should be included in the Msg4 for the CB-Msg3 when fallback to RRC Connected mode.

- Fallback to 4-Step RA

Proposal 21: UE can initiate the legacy 4-step RA when the  CB-Msg3 procedure fails.

Proposal 22: New MAC CE in Msg4 is introduced to indicate the corresponding UEs to stop current CB-Msg3 and initiate legacy 4-step RA.

Proposal 23: A new backoff MAC CE in Msg4 can be introduced when UE fallback to 4-step RA as indicated by the network.

Proposal 24: RAN2 to discuss whether the CE level for 4-step RA starts from the beginning or from the CE level used in CB-Msg3.

- Re-attempt

Proposal 25: Re-attempt in the same CE level due to contention resolution failure is supported.

Proposal 26: Parameter for maximum re-attempt number per CE level is introduced. UE can re-attempt in the same CE level until the max re-attempt number has been reached.

Proposal 27: Backoff MAC CE in Msg4 is supported for re-attempt.

Proposal 28: Power ramping parameters are optionally configured by the network.

R2-2502516	Uplink capacity enhancement in IoT NTN	Apple	discussion	Rel-19	IoT_NTN_Ph3-Core

Msg4 delivery:

Proposal 6: For NB-IoT, time domain factor (SFN at least) and carrier_id of CB-Msg3 EDT resources are considered in the RNTI derivation.

Proposal 7: The frequency domain factor (f_id) of CB-Msg3 EDT resource and OCC index can be  explicitly indicated in MAC subheader for Msg4.

Proposal 8: For eMTC, RAN2 to discuss whether frequency domain info is considered in RNTI calculation (RAN1 confirmation is needed).

Proposal 9: Msg4 should be able to comprise of all or a subset of the following parts:

1) Contention resolution indication on successfully received Msg3 from a certain UE (confirmed last meeting);

2) RRC message part in response to the RRC message contained in received Msg3;

3) MAC subheader with Backoff indicator to other UE(s) whose Msg3 transmission do not succeed.

Proposal 10: Confirm the working assumption that one CB-MSG4 can target multiple UEs simultaneously, assuming one MSG4 RNTI can associate with multiple PUSCH occasions.

Proposal 11: UE stops monitoring Msg4 RNTI after receiving Contention resolution Identity, no matter RRC message is received or not. It is network responsibility to assign a UE specific C-RNTI to UE if network would like to continue DL transmission to the UE.

Proposal 12: RAN2 to discuss whether to support multiplexing of multiple Msg4-RRC messages for more than one UE(s).

[AT129bis][305][R19 IoT NTN] CB-msg3-EDT (Mediatek)

Scope:

Phase 1: discuss single v.s. multiple RNTI/window design

Phase 2: discuss remaining CB-msm33-EDT issues (mainly based on R2-2502771) and LS to RAN1 on CB-msg3-EDT shared resource configuration

Intended outcome: summaries of the offline discussion and Draft LS to RAN1

Deadline for companies' feedback (for phase2):  Thursday 2025-04-10 20:00

Deadline for offline discussion summary (for phase2) and Draft LS:  Friday 2025-04-11 09:00

R2-2503059	Report of [AT129bis][305][R19 IoT NTN] CB-msg3-EDT – phase2	Mediatek	discussion	Rel-19	IoT_NTN_Ph3-Core

Easy - 1

Proposal 1: The DSA configuration (e.g., number of replicas, number of time resources and number of frequency resources) is CE level specified.

The CB-msg3-EDT configuration (e.g., number of replicas, number of time resources and number of frequency resources) is CE level specific.

Proposal 2: Msg4 response window configuration is CE level specified.

The Msg4 monitoring window configuration (e.g. length) is CE level specific

Proposal 4: RAN2 confirms the working assumption that one CB-Msg4 can target multiple UEs simultaneously. FFS how the multiplexing is organized.

RAN2 confirms the working assumption that one CB-Msg4 can target multiple UEs simultaneously. FFS how the multiplexing is organized.

Easy - 3

Proposal 8: RAN2 confirms that existing UP-EDT and CP-EDT RRC message procedures shall be applicable for CB-Msg3-EDT.

•	For UP solution, after RRCConnectionResumeRequest, network may reply with RRCConnectionResume, RRCConnectionSetup, RRCConnectionRelease and RRCConnectionReject

•	For CP solution, after RRCEarlyDataRequest, the network can respond with RRCEarlyDataComplete, RRCConnectionSetup or RRCConnectionReject.

Agreed

RAN2 also intends to support CB-msg3-EDT for MT cases

Proposal 10: The C-RNTI MAC CE shall be included in CB-Msg4 if the UE is expected to receive additional RRC messages or data from the network after CB-Msg4.

The C-RNTI is included in CB-Msg4 if the UE is expected to receive additional RRC messages or data from the network after CB-Msg4 (FFS how to include the C-RNTI)

Potential easy - 1

Proposal 3: Introduce one RNTI (e.g. CB-Msg3-EDT RNTI) for CB-Msg4 monitoring and CB-Msg3 scrambling. FFS whether we send LS to RAN1 for confirmation on RAN1 SPEC impact.

Introduce a new RNTI (i.e. CB-RNTI) for CB-Msg4 monitoring and CB-Msg3 scrambling. We include this agreement in the LS to RAN1

Proposal 5: A CB-Msg4 without RRC message is allowed as the complete response to the CB-Msg3 in CP solution. FFS the CB-Msg4 structure.

-	Samsung thinks we need an RRC message

Come back in the next meeting

Proposal 9: The timing alignment information (FFS reusing TAC MAC-CE) can be included in the CB-Msg4.

Agreed

Potential easy - 2

Proposal 11: Parameter for maximum re-attempt number per CE level is introduced and UE can re-attempt in the same CE level due to contention resolution failure until the max re-attempt number has been reached.

Agreed

Proposal 12: Backoff information could be included in CB-Msg4.

Agreed

Proposal 13: UE can initiate the legacy 4-step RA when the CB-Msg3 procedure fails. FFS on the condition for the UE to initiate the fallback to legacy 4-step RA.

-	Ericsson thinks this should be formulated in a way that failure of the procedure is indicated to upper layers. MTK agrees. ZTE wonders which UL

Continue in the next meeting

May need some discussion (R1 related?)

Proposal 6: L1 ACK as the Msg4 for the CB-Msg3-EDT is not supported.

Agreed

Proposal 7: HARQ feedback is adopted to acknowledge Msg4. FFS for the detail (e.g., how

the HARQ feedback is used for each response in Msg4 when there is multiplexing in Msg4.).

Agreed

R2-2503060	Draft LS on CB-msg3-EDT shared resource configuration	Mediatek	LS out	Rel-19	IoT_NTN_Ph3-Core	To:RAN1

Revised in R2-2503175

R2-2503175	LS on CB-msg3-EDT shared resource configuration	RAN2	LS out	Rel-19	IoT_NTN_Ph3-Core	To:RAN1

Discussed in [Post129bis][302]

[Post129bis][302][R19 IoT NTN] LS to RAN1 on CB-msg3-EDT (Mediatek)

Scope: Draft an LS to RAN1 with meeting agreements CB-msg3-EDT

Intended outcome: Approved LS

Deadline: 1-week

=> Approved in R2-2503175

Agreements (part3):

1.	The CB-msg3-EDT configuration (e.g., number of replicas, number of time resources and number of frequency resources) is CE level specific.

2.	The Msg4 monitoring window configuration (e.g. length) is CE level specific

3.	RAN2 confirms the working assumption that one CB-Msg4 can target multiple UEs simultaneously. FFS how the multiplexing is organized.

4.	RAN2 confirms that existing UP-EDT and CP-EDT RRC message procedures shall be applicable for CB-Msg3-EDT

-	For UP solution, after RRCConnectionResumeRequest, network may reply with RRCConnectionResume, RRCConnectionSetup, RRCConnectionRelease and RRCConnectionReject

-	For CP solution, after RRCEarlyDataRequest, the network can respond with RRCEarlyDataComplete, RRCConnectionSetup or RRCConnectionReject.

5.	RAN2 also intends to support CB-msg3-EDT for MT cases

6.	The C-RNTI is included in CB-Msg4 if the UE is expected to receive additional RRC messages or data from the network after CB-Msg4 (FFS how to include the C-RNTI)

7.	Introduce a new RNTI (i.e. CB-RNTI) for CB-Msg4 monitoring and CB-Msg3 scrambling. We include this agreement in the LS to RAN1

8.	The timing alignment information (FFS reusing TAC MAC-CE) can be included in the CB-Msg4.

9.	Parameter for maximum re-attempt number per CE level is introduced and UE can re-attempt in the same CE level due to contention resolution failure until the max re-attempt number has been reached.

10.	Backoff information could be included in CB-Msg4.

11.	L1 ACK as the Msg4 for the CB-Msg3-EDT is not supported.

12.	HARQ feedback is adopted to acknowledge Msg4. FFS for the detail (e.g., how the HARQ feedback is used for each response in Msg4 when there is multiplexing in Msg4.).

R2-2501767	Discussion on uplink capacity enhancements for IoT NTN	Xiaomi	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2501778	Discussion on CB-EDT Mechanism	vivo	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2501794	Further considerations on Locating of CB-Msg3 Replicas for DSA	NTPU	discussion

R2-2501819	Further consideration on UL capacity enhancements in IoT NTN	ZTE Corporation, Sanechips	discussion	IoT_NTN_Ph3-Core

R2-2501845	Discussion on UL capacity enhancement	HONOR	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2501966	Further consideration on UL capacity enhancement	Huawei, HiSilicon, Turkcell	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2502054	Further consideration on UL capacity enhancements	CATT	discussion	Rel-19

R2-2502069	Discussion on CB-msg3 EDT and msg4 enhancement	OPPO	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2502099	Further consideration on UL capacity enhancement for IoT NTN	Nokia, Nokia Shanghai Bell	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2502117	CB-EDT	Interdigital, Inc.	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2502356	EDT for uplink capacity enhancement in NTN	Lenovo	discussion	Rel-19

R2-2502402	Discussion on UL Capacity Enhancement for IoT-NTN	NEC	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2502427	Discussion on Uplink Capacity enhancement	Transsion Holdings	discussion	Rel-19

R2-2502428	Remaining issues on CB-Msg3 transmission	Spreadtrum, UNISOC	discussion	Rel-19

R2-2502457	On procedures for contention-based Msg3	Samsung	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2502656	CB-Msg3 and Msg4 enhancements	Qualcomm Incorporated	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2502701	Further discussion on uplink capacity enhancement for IoT-NTN	CMCC	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2502886	UL capacity enhancements for IoT NTN	Ericsson	discussion	Rel-19	IoT_NTN_Ph3-Core

### 8.9.4	Support of PWS

Contributions should focus on the introduction of support for broadcast of PWS messages for NB-IoT, re-using the LTE mechanisms.

R2-2502680	Enhancements to support PWS in NB-IoT NTN	Ericsson	discussion	Rel-19	IoT_NTN_Ph3-Core

- Enhancements to eDRX

Proposal 1	Investigate ways to improve the reception of PWS messages for NB- NTN UEs in challenging scenarios (e.g., eDRX).

- SIB1-NB acquisition

Proposal 2	A UE interested in receiving PWS may indicate its interest to the network to adjust the assigned power saving configuration.

Proposal 3	RAN2 studies ways for allowing an NB-IoT UE to skip SIB1-NB acquisition to speed up the delivery of the ETWS primary notification.

- PWS notification reception in RRC_CONNECTED

Proposal 4	Support PWS delivery in RRC_CONNECTED for NB-IoT UEs without impact in other WGs.

Proposal 5	RAN2 to discuss how to provide the PWS notification in RRC_CONNECTED and down select among the following possibilities:

a.	Introduce a scheduling gap to obtain the PWS notification.

b.	Introduce time-critical information in dedicated signaling.

c.	Acquire system information before executing the actions to be released.

d.	Skip the 10 seconds delay upon receiving an RRCRelease message.

-	Google supports b

-	QC wonders if any option should be considered

-	Samsung thinks the RAN plenary agreement is not to support PWS in connected mode. Xiaomi agrees. Nokia agrees. Oppo agrees (no need for any change)

-	HW thinks we can just release to idle possibly with a release cause.

-	CMCC supports b

-	Google thinks that sending the UE back to idle is not the good solution.

-	Sasmung thinks it’s possible to skip the 10 seconds delay.

In case of PWS notification the network may release a RRC_CONNECTED UE to idle, in case the UE reports the capability to receive PWS in idle (no other spec impact other than the introduction of a UE capability for supporting PWS reception in idle)

- Reception of PWS segments when switching serving cell

Proposal 6	Provided it is served by the same gNB, the UE should be allowed to assemble warningMessageSegments received from different cells/satellites.

Proposal 7	The network can indicate whether the UE should discard or continue using the previously buffered warningMessageSegments upon cell/satellite switch.

R2-2501983	Remaining issues on PWS support for NB-IoT	Huawei, HiSilicon, Novamint, Sateliot, Thales, Inmarsat, Viasat, Turkcell	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal: To deliver PWS messages to UEs currently in RRC_CONNECTED, the network may release the UEs to RRC_IDLE and instruct the UEs to read the PWS SIB(s) immediately after transitioning to RRC_IDLE.

R2-2502770	Discussion on supporting PWS for NB-IoT	MediaTek Inc.	discussion	IoT_NTN_Ph3-Core	R2-2501161

Proposal 1: SIB1-NB acquisition is needed after receiving the PWS notification.

Proposal 2: The network will release the UE in connected mode to acquire PWS.

Proposal 3: New release cause PWS notification is used.

R2-2502458	Acceptable cell camping for NB-IoT	Samsung, Iridium, Vivo	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 1: A UE shall be able to camp on an NB-IoT NTN cell for the purpose of receiving emergency broadcast, without the cell being a part of UEs PLMNs.

Proposal 2: RAN2 to discuss introducing acceptable cell category for NB-IoT NTN.

Agreements:

1.	In case of PWS notification the network may release a RRC_CONNECTED UE to idle, in case the UE reports the capability to receive PWS in idle (no other spec impact other than the introduction of a UE capability for supporting PWS reception in idle)

R2-2501768	PWS support for NB-IoT over NTN	Xiaomi	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2501779	Further Discussion on PWS Support for NB-IoT	vivo	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2501820	Remaining issues on PWS support in IoT NTN	ZTE Corporation, Sanechips	discussion	IoT_NTN_Ph3-Core

R2-2502055	Remaining issue on support of PWS for NB-IoT NTN UE	CATT	discussion	Rel-19

R2-2502070	Discussion on PWS for NB-IoT	OPPO	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2502102	Remaining issues on support of PWS for NB-IoT NTN	Nokia, Nokia Shanghai Bell	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2502103	On the support for PWS in NB-IoT	Google	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2502357	Further considerations on PWS broadcast support in IoT NTN	Lenovo	discussion	Rel-19

R2-2502657	Discussion on PWS in NB-IoT NTN	Qualcomm Incorporated	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2502702	Support of PWS messages for NB-IoT	CMCC	discussion	Rel-19	IoT_NTN_Ph3-Core

## 8.10	SON/MDT Ph4

(NR_ENDC_SON_MDT_Ph4-Core; leading WG: RAN3; REL-19; WID: RP-234038)

Time budget: 0.5 TU

Tdoc Limitation: 4 tdocs

### 8.10.1	Organizational

LS, Rapporteur input, including workplan, etc.

Capabilities

R2-2501912	Views on R19 SONMDT UE capabilities	CATT	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1: an optional UE capability with signalling is defined for RLF report for MCG LTM.

Proposal 2: an optional UE capability without signalling is defined for SHR for MCG LTM.

Proposal 3: an optional UE capability with signalling is defined for RLF report for CHO with candidate SCG.

Proposal 4: an optional UE capability without signalling is defined for SHR/SCGFailureInformation for CHO with candidate SCG.

Proposal 5: an optional UE capability without signalling is defined for RLF report for time/location based CHO.

Proposal 6: an optional UE capability without signalling is defined for RA-report for SDT.

Proposal 7: an optional UE capability without signalling is defined for MHI Enhancement for SCG Deactivation/Activation.

Proposal 8: an optional UE capability without signalling is defined for MRO for EN-DC SCG failure in LTE specification.

Noted, we will discuss capabilities in a later meeting.

Running CRs

R2-2501913	Introduction of SONMDT UE Capabilities	CATT	draftCR	Rel-19	38.331	18.5.1	B	NR_ENDC_SON_MDT_Ph4-Core

R2-2501914	Introduction of SONMDT UE Capabilities	CATT	draftCR	Rel-19	38.306	18.5.0	B	NR_ENDC_SON_MDT_Ph4-Core

R2-2501915	Introduction of SONMDT UE Capabilities	CATT	draftCR	Rel-19	36.306	18.4.0	B	NR_ENDC_SON_MDT_Ph4-Core

R2-2502649	Running CR for SONMDT features	Ericsson	draftCR	Rel-19	38.331	18.5.1	B	NR_ENDC_SON_MDT_Ph4-Core

R2-2502787	Running 36.331 CR for R19 SONMDT	Huawei, HiSilicon	draftCR	Rel-19	36.331	18.5.0	B	NR_ENDC_SON_MDT_Ph4-Core

The above are endorsed

[Post129bis][603][SONMDT] Running NR RRC CR for SONMDT (Ericsson)

Scope:

    - Update the running CRs based on the progress in the meeting

Intended outcome:

    - Updated running CR to be submitted to next meeting

Deadline:

    - Long

[Post129bis][604][SONMDT] Running LTE RRC CR for SONMDT (Huawei)

Scope:

    - Update the running CRs based on the progress in the meeting

Intended outcome:

    - Updated running CR to be submitted to next meeting

Deadline:

    - Long

R2-2502788	Open issue list for running 36.331 CR for R19 SONMDT	Huawei, HiSilicon	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Noted

Proposal 1: For ASN.1 definitions in TS 36.331, it is proposed to introduce the perRAInfoListNR IE, and it includes PerRAInfoList-r16, PerRAInfoList-v1660 and PerRAInfoList-v1800 (which are specified in TS 38.331).

Proposal 2: It is proposed to keep the condition for UE setting timeSCGFailure and previousPSCellId, i.e. 3>	if the last RRCReconfiguration message including the reconfigurationWithSync for the SCG was received to enter the PSCell in which the SCG failure was declared.

On P2:

-	Samsung wonders if it covers PSCell change and or PSCell addition? Huawei is OK to leave this as FFS.

-	Nokia wonders if we can instead have a container which carries all the fields (PerRAInfoList-r16, PerRAInfoList-v1660 and PerRAInfoList-v1800)? Samsung think we can use a container (OCTET STRING) and then in the field description we clarify the content.

Agreements For ASN.1 definitions in TS 36.331, it is proposed to introduce the perRAInfoListNR IE, and it includes PerRAInfoList-r16, PerRAInfoList-v1660 and PerRAInfoList-v1800 (which are specified in TS 38.331).

R2-2502974	Discussion on Rel-19 SONMDT open issues	Xiaomi	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Noted

[AT129bis][601][SONMDT] Address open issues for RRC (Ericsson)

Scope:

    - Discuss offline how to proceed with the proposals in R2-2502974 and R2-2502927

Intended outcome:

    - Report in R2-2503101 with agreeable proposals which later can be used to progress running CRs.

Deadline:

    - Wednesday 17:00. The intention is to agree over email.

R2-2503101	[AT129bis][601][SONMDT] Address open issues for RRC	Ericsson

Noted

Agreements - For CHO with candidate SCGs, RAN2 to reuse the current stage-3 description, i.e. include candidate PCell and candidate PSCell measurements results in neighbor measurements within RLF reports (or SCGFailureInformation). - For CHO with candidate SCGs, RAN2 to clarify that the time duration between two fulfilled events (i.e., timeBetweenFulfillmen) shall only be included when both CHO and CPAC conditions are satisfied for failure cases. - For CHO with candidate SCGs, RAN2 explicitly define a new lastHO-Type for CHO with candidate SCGs. - For CHO with candidate SCGs, logging of elapsed time between fulfilling the last triggering event and handover execution in SHR is not required when only one condition (CHO or CPAC) is fulfilled. - For CHO with candidate SCGs, RAN2 to agree to include “firstFulfilledConfig”, “timeBetweenFulfillment”, and “timeBetweenLastFulfillmentAndEvent” in the choWithCandidateSCGInfoList IE within the SCGFailureInformation message. Keep the current ASN.1 structure and no change are needed. - Network is informed in the next RRC complete message about the SHR availability in case the UE performs RACH-less LTM cell switch. No need to change the current formulation of determining the SHR in the current running CR. - Follow CHO like mechanism (i.e., a single rlf-Report for the RLF/HOF, including LTM recovery cell ID) for a consecutive LTM cell switch failure, i.e., an RLF/HOF at source or target cell followed by an HOF during LTM recovery. Relevant FFS can be removed from the running CR. No change is needed in the relevant text of running CR. - FFS whether it is needed to avoid duplication of information in case of two reports being generated CHO with candidate SCGs, any redundancy (e.g., measurements) are recorded in the reports for PCell (i.e., in SHR, SPR).

### 8.10.2	MRO enhancements for Rel-18 mobility features

LTM has 1st priority. CHO with candidate SCGs has 2nd priority

Subsequent CPAC is paused until if/when we get a RAN3 LS on the subject

R2-2502927	Remaining issues for MRO	ZTE Corporation, Sanechips	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

#### 8.10.2.1	LTM

R2-2502707	MRO enhancements for LTM	CMCC	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1: Include the unnecessary LTM configuration, e.g. LTM candidate cell(s) unused, in SON reports, for both failure and near failure cases.

Proposal 2: Include the following UE-based TA related information in RLF report:

－	Difference between the TA value received from the LTM cell switch command and TA value measured by the UE

－	Elapsed time between UE obtains the target cell TA and UE receives the cell switch command, or an indication to indicate whether the UE obtains TA before UE receives LTM cell switch command

－	Time between the PDCCH ordered TA acquisition and the reception of the LTM Cell Switch Command MAC CE that triggered the execution of the failed LTM cell switch

On P1:

-	vivo think the NW can have this info based on config and UE history. Samsung agrees with vivo, and this is not about failure and is out of scope. Nokia supports this proposal since it may be useful for the NW. Ericsson wonders how the NW based solution should work, does the NW have to remember? Vivo think its based on config and UE history. Xiao think this is not needed.

On P2:

-	vivo think this is not needed since if the UE got the LTM command the UE got a TA value. Samsung agrees with vivo about the first bullet and the second and third bullet the NW can know. Xiaomi think we discussed third bullet this already and agreed not to do it. NEC agrees with first bullet. Huawei supports the third bullet. ZTE want all bullets of P2.

R2-2502789	Discussion on MRO for LTM	Huawei, HiSilicon	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Focus on P2

Proposal 1: For RACH-less access based on network TA, include the following information in RLF report:

－	Time period between the reception of the PDCCH order and the reception of HO command MAC CE related to the failed LTM cell switch

Proposal 2: It is proposed RAN2 to agree to enhance the RLF report with the beam information contained in the LTM cell switch command.

On P2:

-	CATT agrees with P2. Ericsson prefers to have the beam info for the case when the UE recovered. vivo think we already indicated to RAN3 that they should solve this. LG think the previous discussion was for SHR, LG supports the proposal. Apple think we agreed the opposite earlier. Qualcomm think we discussed this already and sent to RAN3 that they should solve it. Samsung think that the target has this info. Ericsson want to log the beam info when doing recovery. Samsung think also for recovery its known.

R2-2502826	Discussion on random access report for LTM	ASUSTeK	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core	R2-2500419

Focus on P2-P5

Proposal 1:	RA reports for LTM shall include the following cases:

-	RACH for Early UL synchronization on a candidate cell.

-	NW-triggered RACH-based LTM towards a target cell.

-	UE-triggered RACH-based LTM towards a selected Cell after failed LTM execution.

Proposal 2:	Discuss whether to reuse current values of raPurpose or introduce new values of raPurpose for LTM-related RA reports.

Proposal 3:	For LTM-related RA reports, the UE sets the cellId based on ltm-CandidatePCI and ssb-Frequency in corresponding LTM-Candidate.

Proposal 4:	Discuss whether to set values of spCellID for LTM-related RA reports.

Proposal 5:      A single RA report is used to indicate random access information for all early UL synchronization on the same candidate cell.

On P2:

-	Ofinno want a new codepoint. Vivo want reuse reconfiWithSync.

On P5:

-	vivo think no reporting of early UL sync info is needed. ZTE agrees.

Agreements - We will not add support for including the unnecessary LTM configuration, e.g. LTM candidate cell(s) unused, in SON reports, for both failure and near failure cases. - Do not specify support for reporting difference between the TA value received from the LTM cell switch command and TA value measured by the UE - Do not specify support for reporting of elapsed time between UE obtains the target cell TA and UE receives the cell switch command, or an indication to indicate whether the UE obtains TA before UE receives LTM cell switch command - For LTM-related RA reports, the UE sets the cellId to the CGI if available, otherwise sets it based on ltm-CandidatePCI and ssb-Frequency in corresponding LTM-Candidate.

R2-2501916	MRO Enhancements for LTM	CATT	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1: MRO enhancement for SCG LTM is not considered in Rel-19 SON/MDT.

Enhance RLF report for LTM MRO

Proposal 2: Include TCI state ID from LTM cell switch command in RLF report for RACH-less LTM failure.

Proposal 3: The legacy SHR trigger conditions based on T310 threshold and T304 threshold can be applied to LTM.

Proposal 4: Include L3 measurement results of serving cell, target cell and other LTM candidate cell(s) in SHR for LTM MRO.

Proposal 5: Enhance raPurpose in RACH report to support successful RACH-based LTM execution. The detail can be discussed further, e.g., whether to reuse legacy raPurpose or introduce a new raPurpose.

Proposal 6: A new raPurpose in RACH report is introduced to indicate the network-based early TA acquisition.

Proposal 7: RAN2 to discuss how to log the RACH information of network-based early TA acquisition to avoid the impact on the legacy RACH information logging (e.g., RACH information for handover):

-	Option 1: one entry in RACH report to log the RACH information of all network-based early TA acquisition before one LTM execution;

-	Option 2: one entry in RACH report to log the RACH information of all network-based early TA acquisition before LTM configuration release;

-	Option 3: log the RACH information of network-based early TA acquisition per candidate cell in one or multiple entries.

Other issues

Proposal 8: To address the issue of frequent overriding RLF report/SHR for subsequent LTM, recording multiple reports can be considered as a candidate solution.

Proposal 9: Indication information is needed in RLF report/SHR to indicate whether it’s L1 measurement based LTM or L3 measurement based LTM.

R2-2502288	MRO enhancement for LTM	NEC	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

R2-2502347	Discussion on MRO enhancements for LTM	Lenovo	discussion	Rel-19

Proposal 1: The UE can report L1 measurement results during LTM procedure, and a new RA purpose to indicate that the random access procedure was triggered by a PDCCH order for early TA acquisition, in the RA report.

R2-2502408	MRO for LTM and near failure handling for CHO with candidate SCGs	Nokia	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1: Log additional information regarding TAs that were unnecessarily acquisitioned.

Proposal 2: RAN2 to discuss adding the following information into RLF report: TA difference between TA provided by network in Cell Switch Command for which the RACH-less LTM failed, and TA received in RAR for recovery/re-establishment.

Proposal 3: RAN2 to discuss signalling that the UE indicates that the use of the UE measurement-based TA is preferred.

Proposal 4: RAN2 to enhance SHR with new information and triggering criteria that allow the network to identify unnecessary LTM configurations.

Observation 2: The ACK for first UL transmission taking longer than expected during an LTM HO represents an undesired behaviour.

Proposal 5: RAN2 to define a threshold related to the number of RLC retransmissions of the first UL transmission in RLC AM for RACH-less LTM as a new SHR trigger.

Proposal 6: RAN2 to agree to also log near failure information for the secondary link in case of CHO with candidate SCG.

Proposal 7: RAN2 to agree to extend SPR with the same information agreed for SHR for CHO with candidate SCGs.

Proposal 8: RAN2 to discuss and decide on a correlation mechanism for SHR and SPR generated for the same CHO with candidate SCG execution, for example by introducing a coexistence flag in each report.

Proposal 9: RAN2 to agree on adding Target Pscell ID in SHR and Target Pcell ID in SPR to identify the cell pairs linked to an execution of CHO with candidate SCG.

Proposal 10: RAN2 to agree that there is a need to distinguish if the T310 started before or after fulfilment of the first execution conditions for CHO with Candidate SCG(s).

R2-2502450	SON and MDT for LTM	Qualcomm Incorporated	discussion	NR_ENDC_SON_MDT_Ph4-Core

R2-2502531	Discussion on MRO enhancements 	Samsung	discussion

Proposal 1: Define separate UE capability parameters for RLF report and SHR for LTM cell switch.

Proposal 2: No special handling is needed for reporting successHO-InfoAvailable in RACH less LTM cell switch.

Proposal 3: L1 measurements for LTM are logged only for the candidates for which LTM-SSB-Config-r18 is configured (i.e. only if configured for LTM based on L1 measurements).

Proposal 4: lastHO-Type is not extended with “CHO with Candidate SCG”.

R2-2502604	Discussion on MRO for LTM	Ofinno, LLC	discussion	Rel-18	NR_ENDC_SON_MDT_Ph4-Core

Measurement information in MRO

Proposal 1: Indicate in RLF report/ SHR report whether the LTM to the target cell is based on L3 measurement or L1 measurement.

Proposal 2: Indicate in RLF report/ SHR report whether the neighbor cell (i.e., LTM candidate cells) measurements is based on L3 measurement or L1 measurement.

RLF Report

Proposal 3: Indicate in RLF-report whether the UE received TA from cell switch command for the candidate cell.

Proposal 4: Introduce UE capability for indicating whether UE supports RLF report for LTM.

SHR Report

Proposal 5: Indicate whether RACH-less access is based on CG or DG based on one of:

-	Explicit indication whether RACH-less access is based on CG or DG

-	Explicit indication whether CG for RAHC-less access is valid or invalid

Proposal 6: Indicate index of SSB where the CG is determined to be valid/ invalid.

Proposal 7: Indicate whether a neighbour cell is an LTM candidate cell or not. Whether the indication is using explicit or implicit indication can be decided during stage-3 implementation.

TA information

Proposal 8: A UE logs information of whether early UL synchronization is performed for a neighbor cell (LTM candidate cell).

2.3 RA Report

Proposal 9: A UE logs in RA report, that the access (RACH-based/ RACH-less) is for LTM cell switch.

Proposal 10: In RA report, indicate whether TA of candidate cell was acquired for target cell for LTM cell switch, based on one of:

-	Indicate whether RACH-based or RACH-less access

-	Indicate TA availability

Proposal 10: A UE logs RA information for early uplink synchronization in raReport.

Proposal 11: Introduce new raPurpose or early uplink synchronization.

Proposal 12: A UE logs multiple early UL synchronization attempts to a candidate cell using a single entry in raReport. UE logs the number if early UL synchronization attempts(i.e., RACH attempts) made to the candidate cell. UE logs RA information corresponding to the last attempt.

Proposal 13: A UE logs the time elapsed between receiving PDCCH order for triggering RACH (for early UL synchronization) on a candidate cell, and receiving LTM cell switch command MAC CE for cell switch to the same candidate cell.

Proposal 14: RAN2 to discuss whether the first PDCCH order or the last PDCCH order for early UL synchronization on the candidate cell is considered for logging the time elapsed.

R2-2502646	Further considerations on Mobility Robustness Optimization	Ericsson	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1	Enhance the RLF report with beam information of.

a.	the beam toward which the UE failed to perform LTM cell switch,

b.	The beam indicated in the last successful LTM cell switch command, when failure occurs shortly after the LTM cell switch,

c.	The beam the UE succeeded to recover/re-establish or reconnect in.

Proposal 2	In case of RLF shortly after a successful LTM cell switch, the UE logs whether the UE has switched the beam by the target cell prior to the RLF. This enables the network to correctly classify Too Early HO failure from failure due to wrong beam configuration at the target cell.

Proposal 3	The UE logs and reports the time between the PDCCH ordered TA acquisition and the reception of the LTM Cell Switch Command MAC CE that triggered the execution of the failed LTM cell switch. For the UE-based TA estimation, there is no need to log this information.

Proposal 4	The UE logs the difference between a TA received in a RAR (or a MsgB) in the target cell and the TA received in the LTM Cell Switch Command MAC CE in the RLF report.

Proposal 5	UE logs available L1 measurement results for the serving cell, the target cell and other LTM candidate cells when the UE is LTM configured.

R2-2502806	MRO for LTM	LG Electronics Inc.	discussion	Rel-19	R2-2501212

Proposal 1:	In the RLF report for MCG LTM, RAN2 should revisit and decide whether the UE should explicitly log the access type.

Proposal 2:	In the RLF report for MCG LTM, the UE logs not only the access type but also the commanded access type which was indicated LTM CSC if the access type is different from the commanded access type.

Proposal 3:	In the SHR for MCG LTM, ltmCandidate is explicitly defined as the same enum type as choCandidate.

Proposal 4:	In the SHR for MCG LTM, RAN2 discuss that the UE log candidate cells not only for those candidate cells unused but also for those candidate cells selected yet not requiring actual cell switching.

R2-2502858	RA information enhancement for LTM	SHARP Corporation	discussion

Proposal 1: UE does not log RA information for early UL sync in RA report.

Proposal 2: multiple successive RA procedures for the same LTM candidate cell can be included in a single RA report.

Proposal 3: a new raPurpose in RA report can be used to differentiate the RA is for LTM early UL sync.

Proposal 4: PCell identity is included in RA report for LTM early UL sync on a LTM candidate cell.

Proposal 5: no enhancement is needed for RA report in case of RACH-based LTM cell switch.

#### 8.10.2.2	CHO with candidate SCGs

R2-2502861	SON enhancement for CHO with candidate SCG	SHARP Corporation	discussion

Proposal 1: new triggering conditions, e.g. time gap between the first met condition (CHO or CPAC) and the second met condition (CPAC or CHO) is above a threshold, can be considered for SHR/SPR procedure.

Proposal 2: Consider new triggering conditions for SHR/SPR procedure: 1) CPAC execution condition is fulfilled late than CHO execution condition can be considered; 2) CPAC execution condition is fulfilled early than CHO execution condition; but the time gap between the CHO condition fulfillment and CPAC condition fulfillment is above a threshold.

Proposal 3: UE includes the target PSCell ID in SHR for successful CHO with candidate SCGs.

Proposal 4: UE includes the target PCell ID in SPR for successful CHO with candidate SCG.

Proposal 5: in case of MCG RLF, introduce information in RLF-report to indicate whether condition evaluation for CHO with candidate SCGs is stopped due to an SCG failure.

On P1:

-	LG and Samsung think current conditions are enough. CATT wants new triggers. Nokia and Ericsson also think no new trigger condition is needed.

On P2:

-	 Samsung think same logic applies as for P1, i.e. not needed. LG agrees.

On P3 and P4

-	Ericsson thinik this is obvious and already in the running CR.

Agreements - No new triggering conditions such as time gap between the first met condition (CHO or CPAC) and the second met condition (CPAC or CHO) is above a threshold, can be considered for SHR/SPR procedure. - No new triggering conditions for SHR/SPR procedure for CHO with candidate SCG. - UE includes the target PSCell ID in SHR for successful CHO with candidate SCGs. - UE includes the target PCell ID in SPR for successful CHO with candidate SCG. - In general, and where applicable, agreements valid for SHR, RLF reports and SCG failure info applies also to SPR.

R2-2501917	New SHR Trigger for CHO with Candidate SCGs	CATT, Vivo, Lenovo, Huawei, HiSilicon, CMCC	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1: In order for the network not to miss abnormal time gap information, introduce a specific SHR trigger condition, i.e., the time gap between fulfilled CHO execution condition and fulfilled CPAC execution condition exceeds a configured threshold.

R2-2502348	Discussion on MRO enhancements for CHO with candidate SCGs	Lenovo	discussion	Rel-19

Proposal 1: Introduce new trigger condition(s) for SHR or SPR for a CHO with candidate SCG(s), e.g. a threshold related to time duration between fulfilment of CHO execution condition configured for the target PCell and fulfilment of CPAC execution condition configured for the associated target PSCell.

Proposal 2: The type of the first fulfilled execution condition (e.g. CHO execution condition or CPAC execution condition), and time from the last triggered event for the PCell (or PSCell) to the time to the last triggered event for the PSCell (or PCell), can be stored and reported by the UE in SHR.

Proposal 3: The type of the first fulfilled execution condition (e.g. CHO execution condition or CPAC execution condition), and time from the last triggered event for the PCell (or PSCell) to the time to the last triggered event for the PSCell (or PCell), can be stored and reported by the UE in SPR.

R2-2502451	SON and MDT for CHO with candidate SCGs	Qualcomm Incorporated	discussion	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1	Reuse the timer timeConnFailure in RLF Report to indicate time elapsed between the CHO with candidate SCG execution and RLF.

Proposal 2	Reuse the timer timeSCGFailure in SCGFailureInformation to indicate time elapsed between the CHO with candidate SCG execution and SCG failure.

Proposal 3	UE should log the CHO recovery cell ID in RLF Report when UE attempts CHO recovery after a CHO with candidate SCG execution failure.

Proposal 4	In case the UE attempts CHO recovery after a CHO with candidate SCG execution failure and observes that there are 2 or more suitable cells, UE may prioritize selecting a cell for CHO recovery based on whether its associated PSCell’s execution condition was met before or not.

R2-2502708	MRO enhancements for CHO with candidate SCGs	CMCC	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1: RAN2 discusses the fallback scenario of CHO with candidate SCGs, i.e. CHO with candidate SCGs and CHO-only are both configured.

Proposal 2: RAN2 discusses the following information to be included for the scenario of CHO-only and CHO with candidate SCGs are both configured:

－	An indication to indicate neither of the two conditions of CHO with candidate SCGs is fulfilled before received the CHO-only or CHO with SCG configuration.

－	The fulfilled CHO with candidate SCGs execution condition when CHO-only configuration is received.

－	The time difference between the first fulfilled execution condition and UE receives CHO-only configuration.

－	The time difference between UE receives CHO with candidate SCGs configuration and CHO-only configuration is fulfilled.

－	Indicate whether the CHO-only or CHO with SCG is configured in RLF report.

R2-2502790	Discussion on MRO for CHO with candidate SCGs	Huawei, HiSilicon	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1: It is proposed RAN2 to agreed that when CHO is fulfilled but CPC conditions are not fulfilled, in RLF report, UE reports identifier of candidate PCell(s) which met the configured CHO execution conditions when the RLF is encountered.

Proposal 2: It is proposed RAN2 to agreed that when CPC is fulfilled but CHO conditions are not fulfilled, in RLF report, UE reports identifier of candidate PSCell(s) which met the configured CPAC execution conditions when the RLF is encountered.

R2-2502807	MRO for CHO and Associated CPAC	LG Electronics Inc.	discussion	Rel-19	R2-2501213

Proposal 1: For CHO with associated CPA/CPC, the UE reports information for the near failure case to the network via SHR not SPR.

Proposal 2. For CHO with associated CPA/CPC, the UE should include the satisfaction time of the first execution condition in the SHR as part of the near failure information.

#### 8.10.2.3	Other

### 8.10.3	SON/MDT for Slicing

R2-2501734	LS on SON for Network Slicing (R3-250914; contact: ZTE)	RAN3	LS in	Rel-19	NR_ENDC_SON_MDT_Ph4-Core	To:RAN2

Moved from 8.10.1

R2-2502409	Enhancements for network slicing (LS R3-250914)	Nokia	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1: RAN2 should study how the UE should provide information that helps the network to identify the scenario when the UE did not manage to select a cell in the frequencies corresponding to the highest ranked NSAG(s).

Proposal 2: Enhance the Mobility History Information to enable the network to identify the scenario when the UE did not manage to select a cell in the frequencies corresponding to the highest ranked NSAG(s).

Proposal 3: Enhance the Mobility History Information with all the NSAG IDs that did not lead to successful cell reselection and the NSAG ID (or the lack of any NSAG ID) that was used to select a new cell.

Proposal 4: Enhance the Mobility History Information with information about the scanned frequencies (e.g., a list of scanned frequencies) used during cell reselection.

Proposal 5: The Mobility History Information can also include the used slice specific reselection parameters along with the NSAG IDs used during cell reselection.

On P1 and P2:

-	Samsung do not see the need for this, and instead agree with the Ericsson proposal below. CMCC see benefit of having slice related info reported from the UE. Huawei  supports the Nokia proposal. Qualcomm think that we can confirm the scenario in the RAN3 LS. Xiaomi agree with Samsung. Ericsson think there is no good justification for enhancing MHI and nothing is broken, the legacy MDT feature is enough. Samsung think that the info is not useful since there can be many reasons why the UE does not camp on the expected cell from a slice-prio p.o.v. Ericsson

FFS if/what RAN2 enhancements we will do to address the scenario when the UE did not manage to select a cell in the frequencies corresponding to the highest ranked NSAG(s).

R2-2502647	Discussion on slice base cell reselection information in Logged MDT	Ericsson	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1	The scenario described in the LS may occur only if there is a coverage hole in the frequencies corresponding to the highest ranked NSAG.

Proposal 2	Send LS reply to RAN3 mentioning the scenario identified in RAN3 can be supported using legacy logged MDT procedure, no further enhancements are needed.

R2-2502452	SON and MDT for slicing	Qualcomm Incorporated	discussion	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1	UE can indicate via a flag in logged MDT report that it collected logged MDT using the slice-based cell reselection parameters (and not general cell reselection parameters).

R2-2502518	Discussion on MDT for Slicing	Samsung	discussion

Proposal 1:RAN2 to inform RAN3 that it is possible to address the scenario of UE supporting slice-based cell reselection not finding any suitable cell in the frequencies corresponding to the highest ranked NSAG, without any enhancement of logged MDT.

R2-2502738	Discussion on SONMDT for network slicing	CMCC	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1: The NSAG(s) supported by the highest ranked cell or best cell, the NSAG(s) supported by the UE and the priority information can be reported to the network, which can be helpful for NW to optimize the sliceAllowedCellListNR and/or sliceExcludedCellListNR, or slice deployment, and can assist the UE to re-select a suitable cell for slice-based cell re-selection.

R2-2502791	Discussion on SONMDT for Slicing	Huawei, HiSilicon	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1: It is proposed RAN2 to agree on introducing the following report type in logged MDT configuration:

-	highest ranked cell or best cell does not support any or all of the intended NSAGs

Proposal 2: It is proposed RAN2 to agree on including the NSAG information received from NAS in logged MDT report.

Proposal 3: It is proposed RAN2 to discuss whether to address the case where the UE fall backs to legacy cell reselection procedure from slice-based cell reselection.

Proposal 4: It is proposed RAN2 to discuss possible solutions, including:

-	addition of a new report type related to P3 in logged MDT configuration

-	addition of a fallback indicator in logged MDT report

R2-2502928	SON/MDT for Slicing	ZTE Corporation, Sanechips	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1	RAN2 to discuss which option to take, logged MDT or RA report to optimize the slice based cell reselection.

Proposal 2	LS reply to RAN3 on RAN2 decision.

### 8.10.4	SON/MDT for NTN

R2-2502453	SON and MDT for NTN	Qualcomm Incorporated	discussion	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1	RAN2 assumes  that the signaling structure agreed for encoding geographical area in case of MBS NTN can be used as a reference for encoding geographical area based logged MDT for NTN.

Proposal 2	RAN2 should discuss whether to support reference location/radius or polygon-based or both.

Proposal 3	Use dedicated signalling to provide area scope configuration to the UE.

Proposal 4	Introduce a UE capability to support geographic area scope checking.

Proposal 5	gNB can only configure either mapped cell list or geographic area scope to the UE.

Proposal 6	Only if the UE can obtain location information in IDLE/Inactive state, the UE performs geographical area checking for logged MDT.

Proposal 7	RAN2 discusses which option is used in case that the area scope is not configured to the UE or UE cannot obtain location information in IDLE/Inactive state if geographic area scope is configured.

Option 1: The UE collects logged MDT data without checking area scope.

Option 2: UE does not perform logged MDT data collection.

On P1:

-	CATT think that we should “reuse” the area IEs and not “use as reference”. Nokia think that we should just reply to RAN3 and we should say it is feasible to reuse and we can discuss signalling structure later.

On P2:

-	Qualcomm want to support location/radius and FFS polygon-based. Xiaomi prefers only location/radius. CATT think we should reuse, i.e. both formats should be supported.

On P4:

-	CATT think this should be conditionally mandatory. Qualcomm think this should be a standalone capability since this is for IDLE/INACTIVE.

On P5:

-	Huawei think that RAN3 LS/procedures suggest that only area should be supported over Uu.

On P7:

-	Qualcomm think option 1 is simpler for the UE. Ericsson want to wait with this. Huawei also want to wait.

-	Nokia think UE always knows its location to some degree. Ericsson think its not mandatory for UE to always know.

Agreements: - RAN2 assumes that the signaling structure agreed for encoding geographical area in case of MBS NTN can be used as a reference for encoding geographical area based logged MDT for NTN. - Support reference location/radius and polygon-based area indication. - Use dedicated signalling to provide area scope configuration to the UE - Introduce a UE capability to support geographic area scope checking. FFS if with or without signalling. - Only geographic area scope is used to indicate applicable logging area to the UE. - Only if the UE can obtain location information in IDLE/Inactive state, the UE performs geographical area checking for logged MDT. FFS if, when the UE is in IDLE/INACTIVE, the UE logs or not if the UE cannot obtain its location.

-	Qualcomm thinks we need to send an LS to RAN3.

[Post129bis][605][SONMDT] LS to RAN3 on MDT for NTN (Qualcomm)

Scope:

    - Produce approvable LS

Intended outcome:

    - Approvable LS in R2-2503117

Deadline:

    - Short

R2-2503117	Relay LS on MDT for NTN	Qualcomm	LS out	Rel-19	NR_ENDC_SON_MDT_Ph4-Core	To:RAN3

=> Revised in R2-2503196

R2-2503196	Relay LS on MDT for NTN	RAN2	LS out	Rel-19	NR_ENDC_SON_MDT_Ph4-Core	To:RAN3

=> Approved (but then coversheet revised by MCC: “Draft” in the LS name inside the zip file)

R2-2503203	Relay LS on MDT for NTN	RAN2	LS out	Rel-19	NR_ENDC_SON_MDT_Ph4-Core	To:RAN3

=> Approved

R2-2501918	Consideration on SONMDT enhancements for intra-NTN mobility	CATT	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1: RAN2 to confirm that the geographical area defined for MBS NTN can be reused for Area Scope of logged MDT over Uu for NTN.

Proposal 2: Send a Reply LS to RAN3 with RAN2 agreements on the MDT area scope of NTN.

Proposal 3: The gNB uses the single format in Uu for configuring area scope of logged MDT, no matter it’s configured in the geographical area or mapped cell ID format in NGAP.

Proposal 4: RAN2 to consider how to set the TA information in the CGI report for NTN:

-	Option 1: It’s up to UE to select one TAC indicated in trackingAreaList and add it in CGI info IE.

-	Option 2: To include the whole trackingAreaList in CGI info IE for NTN.

R2-2502242	SON of intra-NTN mobility	China Telecom	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1: Regarding MRO for CHO with a time-based trigger condition, include relative time between the time RLF occurs and t1-Threshold in the RLF report.

Proposal 2: Regarding MRO for CHO with a time-based trigger condition, include relative time between CHO execution failure and t1-Threshold in the RLF report.

Proposal 3: Regarding MRO for CHO with a location-based trigger condition, include measured distance from UE to moving reference location of serving cell, and measured distance from UE to moving reference location of candidate cell when RLF occurred in the RLF report.

R2-2502349	Discussion on MRO for intra-NTN mobility	Lenovo	discussion	Rel-19

Proposal 1: RLF report can be enhanced to include measurement results of source cell, target cell and candidate cell(s) at the absolute time value (t1-Threshold + duration).

Proposal 2: RLF report can be enhanced to include measurement results of source cell, target cell and candidate cell(s) when location based CHO execution condition is fulfilled before RLF happens.

Proposal 3: RLF report can be enhanced to include the distance between UE and a reference location/moving reference location of source cell when measurement based CHO execution condition is fulfilled before RLF happens, or the distance between UE and a reference location/moving reference location of candidate cell(s) when measurement based CHO execution condition is fulfilled before RLF happens.

R2-2502439	SON/MDT for NTN	Samsung	discussion

Proposal 1: RAN2 to send LS to RAN3 informing that geographical area based Area Scope over Uu is not feasible for logged MDT.

Proposal 2: UE reports following additional information in RLF report for NTN related failures

a.	Timing Advance reported in the Timing Advance Report MAC CE, if it was triggered during the random access procedure during a handover.

b.	A flag indicating whether T430 had expired when RLF or HOF occurred.

c.	Information related to satellite position.

Proposal 3: UE may log tracking area list in CGI-Info-Logging in various SON/MDT reports.

R2-2502641	MRO for NTN RACH-less Access	Nokia	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1: RAN2 to consider enhancing SHR for RACH-less access for NTN.

Proposal 2: RAN2 to consider enhancing RLF Report for RACH-less access for NTN.

Proposal 3: Enhance the SHR Report message definition with NTN RACH-less specific reporting.

Proposal 4: Define a new SHR trigger for the UE to log the enhancements measurements specific to NTN RACH-less access.

Proposal 5: Enhance the RLF Report message definition with NTN RACH-less specific measurements and a new RLF Cause specific to NTN RACH-less access failure.

Proposal 6: RAN2 to send a reply LS for the incoming LS from RAN3, R2-2500026/R3-247908, and indicate it is technically feasible to use a similar Uu signalling structure as adopted for encoding geographical area in the case of MBS NTN, for geographical area based logged MDT for NTN.

R2-2502648	Discussion on the reply LS to RAN3 on SON-MDT enhancements for NTN	Ericsson	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1	RAN2 to confirm that the geographical area for MBS NTN can apply to MDT area scope, i.e. a geographical area represented by a (set of) referenceLocation and radius or by a (set of) polygon(s).

Proposal 2	RAN2 to discuss if and how to optimize location checking for logged MDT for the sake of UE energy saving. Detailed solutions (how frequent the UE should check the location) can be FFS.

R2-2502792	Discussion on SONMDT for NTN	Huawei, HiSilicon	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1: It is proposed RAN2 to agree on reusing the geographical area defined for MBS NTN for Area Scope of logged MDT over Uu for NTN.

Proposal 2: It is proposed RAN2 to discuss whether to introduce some means to minimize negative impacts at UE side, e.g. the UE performs geographical area checking only if it has available position information.

R2-2502808	NTN Logging for Unchanged PCI Mobility	LG Electronics Inc.	discussion	Rel-19	R2-2501214

Proposal 1: For unchanged PCI satellite mobility, the UE uses the RLF report, including a new cause to indicate a failure of the unchanged PCI satellite mobility if RLF is declared due to this mobility.

Proposal 2: For unchanged PCI satellite mobility, the UE reports the near failure information that out-of-sync indication(s) has been detected during the mobility if the UE detects the out-of-sync indications during the unchanged PCI satellite mobility but successfully complete this mobility.

Proposal 3: To determine a proper near failure of the unchanged PCI satellite mobility, a new logging condition is considered based on reception of a significant number of out-of-sync indications during the mobility.

R2-2502929	SON/MDT for NTN	ZTE Corporation, Sanechips	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1	RAN2 to discuss for the RACH-less HO, whether to report to network a supposedly RACH-less which falls back to RACH based, and in which report (e.g., Random Access report or rlf-Report.)

Proposal 2	Introduce absolute time, absolute location, and speed information in NTN RLF reports when UE triggers CHO and upon RLF.

Proposal 3	Adopt geographic area as the area scope information for NTN MDT.

Proposal 4	Reuse the geographic area defined in SIB25, for the absolute geographic area as the area scope information for NTN MDT. Can coordinate with Rel-19 NTN Working Item during stage 3 phase.

Proposal 5	LS back to RAN3 on above agreements.

R2-2502975	Discussion on SONMDT for NTN mobility	Xiaomi	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1: The geographical area (a reference location plus a radius) can be reused for area scope of logged MDT over Uu for NTN.

### 8.10.5	Leftovers from Rel-18

RACH optimization for SDT focus on RSRP and data volume in SON reports, and existing failure causes.

MHI Enhancement for SCG Deactivation/Activation.

R2-2502482	SON/MDT for Leftovers from Rel-18	Samsung	discussion

Proposal 1: Value range of SDT-UL-DataVolume is 0 to 96000. UE reports 96000 when the data volume is larger than 96000.

Proposal 2: UL data volume is reported only for MO SDT and only radio bearers (DRB and SRB2) configured for SDT are considered.

Proposal 3: perRA-InforList reported in SCGFailureInformationNR includes PerRAInfoList-r16, PerRAInfoList-v1660 and PerRAInfoList-v1800 as in SCGFailureInformation (same as in running LTE CR).

Proposal 4: UE reports the absolute time it has spend in PSCell with SCG activated in MHI.

On P1:

-	ZTE wants a ratio. Samsung clarifies its 96000 bits.

On P2:

-	Ericsson think that there is no condition in the current running CR, so no need to add SRB/DRB since the UE will only do SDT if configured to do SDT and on the configured bearers.

Agreements: - Value range of SDT-UL-DataVolume is 0 to 96000 bits. UE reports 96000 when the data volume is larger than 96000. This is encoded as ENUMERATED. - UL data volume is reported only for MO SDT and only radio bearers configured for SDT are considered. This has no specific specification impact. - perRA-InforList reported in SCGFailureInformationNR includes PerRAInfoList-r16, PerRAInfoList-v1660 and PerRAInfoList-v1800 as in SCGFailureInformation (same as in running LTE CR). - UE reports the absolute time it has spent in PSCell with SCG activated in MHI.

R2-2501919	MHI Enhancement for SCG Activation and Deactivation	CATT	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1: UE reports the absolute time the UE has spent in PSCell with SCG activated in MHI.

Proposal 2: RAN2 to discuss the following two options about logging the SCG activation/deactivation time for a PSCell in MHI:

-	Option 1: logging each SCG activation/deactivation time duration for a PSCell in chronological order;

-	Option 2: logging the accumulated time for SCG activation/deactivation for a PSCell;

R2-2502350	Discussion on RACH optimization for SDT	Lenovo	discussion	Rel-19

Proposal 1: The UE reports RSRP and data volume when triggered RA-SDT fails in the RA report for SDT optimization.

R2-2502410	MHI/UHI Enhancement for SCG Deactivation/Activation	Nokia	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1: It is proposed to include the information of SCG activation/deactivation, e.g., the ratio of time in SCG activated state together with some more assistance information related to the condition(s) for SCG activation/deactivation, in MHI.

R2-2502709	MHI/UHI Enhancement for SCG Deactivation/Activation	CMCC	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1: Include the accumulated time of UE spent in SCG activation state in MHI.

R2-2502859	RA report enhancements for SDT	SHARP Corporation	discussion

Proposal 1: UE does not log the failure cause for failed SDT in RA report for the case “upon the lower layers indicate that cg-SDT-TimeAlignmentTimer or the configuredGrantTimer expired before receiving network response for the UL CG-SDT transmission with CCCH message while SDT procedure is ongoing”.

Proposal 2: UE logs the DL RSRP and UL data volume in both RA-SDT initiation failure case and normal ongoing RA-SDT failure case.

Proposal 3: the elapsed time since the execution of RA-SDT is also included in RA report for successful RA-SDT case.

R2-2502862	MHI enhancement for SCG activation/deactivation	SHARP Corporation	discussion

Proposal 1: SCG activation related time is reported per PSCell in a PCell.

Proposal 2: percentage of time is used SCG activation time in MHI.

Proposal 3: percentage of time is specified as enumeration type with several typical values.

Proposal 4: UE only sets the SCG activation time for a PSCell entry only when the SCG has ever been in deactivation during the stay in the PSCell.

R2-2502930	Remaining issues for Rel-18 leftovers	ZTE Corporation, Sanechips	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1	RAN2 to discuss which option to define sdt-UL-DataVolume-r19:

(1) an absolute data volume, or

(2) a relative value (e.g., a ratio compared to the network configured threshold, from 1 to 10, in step of 0.1).

Proposal 2	RAN2 to discuss which option to take for timeSinceSdtExecution:

(1) if timeSinceSdtExecution reaches the maximum (i.e., 48 hours), network should not see it as a valid value, or

(2) UE should not report this value if it is equal or beyond the maximum value.

Proposal 3	UE includes traffic characteristics (e.g., average data rate during activation period) in the MHI together with the SCG activation/deactivation information.

R2-2502951	On remaining issues related to Rel.18 leftovers	Ericsson	discussion

Proposal 1	For each PSCell entry of the MHI, log the absolute, accumulated time with SCG state set to activated during the stay in the PSCell.

Proposal 2	Agree on the 38.331 TP implementing the inclusion of the absolute, accumulated time with SCG state set to activated during the stay in the PSCell in MHI provided in Appendix A.

Proposal 3	RA report enhancements for SDT is considered concluded in RAN2, no further enhancements are discussed in Rel-19.

## 8.11	Evolution of NR duplex operation: Sub-band full duplex (SBFD)

(NR_duplex_evo-Core; leading WG: RAN1; REL-19; WID: RP‑241614)

Time budget: 0.5 TU

Tdoc Limitation: 2 tdocs

### 8.11.1	Organizational

Incoming LS, Rapporteur input, including workplan, running CRs, etc..

Output of email discussion [Post129][217].

LS

R2-2501713	Reply LS on CSI-RS measurement with SBFD operation (R1-2501560; contact: MediaTek)	RAN1	LS in	Rel-19	NR_duplex_evo-Core	To:RAN4	Cc:RAN2

Noted

R2-2501731	SBFD information exchange among gNBs for CLI mitigation (R3-250888; contact: Huawei)	RAN3	LS in	Rel-19	NR_duplex_evo-Core	To:RAN2	Cc:RAN1

DISCUSSION

-	HW explains that R3 request us to introduce the configuration/parameter but currently we still need some input from R1.

-	OPPO think we can wait and think no need to reply. LG E agree.

Noted

R2-2501738	LS on L1 CLI measurement (R4-2502632; contact: Huawei)	RAN4	LS in	Rel-19	NR_duplex_evo-Core	To:RAN1	Cc:RAN2

Noted

Running CRs

R2-2501851	TS 38300 Running CR for SBFD	CATT	draftCR	Rel-19	38.300	18.5.0	NR_duplex_evo-Core

Noted, will update/endorse in post meeting email discussion

[Post129bis][211][SBFD] Running CR for 38.300 (CATT)

Intended outcome: Updated running CR based on new agreements for endorsement

Deadline:  Long

R2-2502549	RRC running CR for Evolution of NR duplex operation (SBFD)	Huawei, HiSilicon	draftCR	Rel-19	38.331	18.5.1	B	NR_duplex_evo-Core

=> Revised in R2-2502978

R2-2502978	RRC running CR for Evolution of NR duplex operation (SBFD)	Huawei, HiSilicon	draftCR	Rel-19	38.331	18.5.1	B	NR_duplex_evo-Core

Noted, will update/endorse in post meeting email discussion

[Post129bis][212][SBFD] Running CR for 38.331 (Huawei)

Intended outcome: Updated running CR based on new agreements for endorsement, open issue list (if needed)

Deadline:  Long

R2-2502279	TS38.304 impacts on supporting Rel-19 SBFD	NEC	discussion	Rel-19	NR_duplex_evo-Core

DISCUSSION

-	NEC explains there is no 304 impact so no need for running CR.

Noted

No running CR is needed for TS38.304, unless RAN1/RAN4 indicate that it is needed.

R2-2502591	MAC running CR for Evolution of NR duplex operation: SBFD	Samsung	draftCR	Rel-19	38.321	18.5.0	B	NR_duplex_evo-Core

DISCUSSION

-	Apple has concern on CSI-RS based SBFD, think R1 and R4 didn’t agree. Apple think if we do so we should first ask R1 and R4. Samsung point out this is taken into account with the EN ‘Editor’s Note: Will reflect agreement(s), if any, on CSI-RS based CFRA.’

Noted, will update/endorse in post meeting email discussion

[Post129bis][213][SBFD] Running CR for 38.321 (Samsung)

Intended outcome: Updated running CR based on new agreements for endorsement, open issue list (if needed)

Deadline:  Long

R2-2502567	Introduction of SBFD UE capabilities (Running CR)	Ericsson	draftCR	Rel-19	38.306	18.5.0	B	NR_duplex_evo-Core

R2-2502568	Introduction of SBFD UE capabilities (Running CR)	Ericsson	draftCR	Rel-19	38.331	18.5.1	B	NR_duplex_evo-Core

DISCUSSION

-	Xiaomi think the CR only contain R1/R4 capabilities, so we do it in mega CR. Samsung share this view.

-	OPPO think for some cases, e.g., CA, we may need some discussions in R2.

Noted, with the understanding that running CR is needed only when we introduce new capabilities form R2 point of view.

Output of email discussion [Post129][217]

R2-2502210	Summary of [Post129][217][SBFD] List of open issues of RRC impact	Huawei, HiSilicon (Rapporteur)	discussion	Rel-19	NR_duplex_evo-Core

Noted

[Proposal 1] On RO type signaling for CFRA, discuss/conclude on:

1. For CFRA triggered by BFR, the RO type is indicated in BeamFailureRecoveryConfig.

2. For CFRA triggered by ReconfigurationwithSync, the RO type is indicated in RACH-ConfigDedicated.

3. Within the corresponding RACH configuration for the additional ROs, one bit is used to indicate the RO type for CFRA.

4. For CFRA triggered by PDCCH order including UL early sync, RAN2 can wait/follow RAN1 agreement.

[Proposal 2] Discuss option issue on the UE behaviour when the signalling on the RO type and RSRP threshold are absent. Consider solution options 1) either NT indication on RO type or on RSRP threshold is madatorily present. 2) When both NT indication on RO type and RSRP threshold are absent, the SBFD-aware UE should prioritize the additional RO over the legacy RO.

[Proposal 3]  Choose one option below on whether confirm WA "Random access procedure in SBFD symbols is supported for all the existing RACH trigger events. " and discuss related RRC impact:

Option 1: Confirm the WA "Random access procedure in SBFD symbols is supported for all the existing RACH trigger events. ".

Option 2: Random access procedure in SBFD symbols is supported for all the existing RACH trigger events except for Position SI request and SI request.

[Proposal 4] RAN2 waits for conclusion from RAN3 and RAN1 on NZP CSI-RS resources configuration and SBFD frequency configuration information.

DISCUSSION

P1

-	LG E ok with P1 in general. LG E not sure whether LTM is supported with SBFD, but if they are used together then point 3 in the proposal need to be further checked. HW think we do not need to discuss the combination of the features in this stage. ZTE think we can disucss.

-	QC ok with point 1, 2 and 4 in P1.

-	Ericsson ok with point 1 and 2, for point 4 not sure whether LTM and SBFD can be configured together.

-	ZTE point 4 is already agreement in R1, so no need to agree again in R2. OPPO think R1 agreement is only for PDCCH order.

-	CATT think for point 4 we should also discuss the RA type indicating by RRC parameter. HW think if R1 has such configuration in the parameter list we can add.

-	Apple understand UE need to know which option (1 or 2) is used in the cell for SBFD configuration, so UE need to combine SIB configuration with other configurations as included in point 1 and 2.

-	Nokia think point 1 and 2 already clear.

On RO type signaling for CFRA

- 1. For CFRA triggered by BFR, the RO type is indicated in BeamFailureRecoveryConfig.

- 2. For CFRA triggered by ReconfigurationwithSync, the RO type is indicated in RACH-ConfigDedicated.

P2

-	LG E ok with either option 1 or 2, but option 2 is better.

-	vivo agree with option 1 and think option 2 is not needed. IDT, CMCC, CATT, Ericsson prefer option 1. QC do not agree.

-	QC, ZTE prefer option 2.

-	Nokia, Samsung, Sony think there is one more option that UE select first available RO. HW explain there is limited support in email discussion so not listed here.

-	Ericsson think if we do not make it mandatory then it should be up to UE implementation.

When both NW indication on RO type and RSRP threshold are absent, it is up to UE implementation to select the RO type.

P3

-	HW think there is benefit for cell edge UE even for SI request case. CATT, vivo think there is benefit for positioning SI to get A-GNSS assist info.

-	LG E think detailed approach is needed for SI request, e.g., to handle CLI, and think time is limited to finalize this in the WI. Apple do not see the benefit in SI request case, and there is great spec impact. Xiaomi, Samsung share this view.

Random access procedure in SBFD symbols is supported for all the existing RACH trigger events except for SI request. FFS for LTM.

P4

-	LG E think we can wait, no need for an agreement.

Agreements on SBFD RACH aspects

On RO type signaling for CFRA - 1. For CFRA triggered by BFR, the RO type is indicated in BeamFailureRecoveryConfig. - 2. For CFRA triggered by ReconfigurationwithSync, the RO type is indicated in RACH-ConfigDedicated. When both NW indication on RO type and RSRP threshold are absent, it is up to UE implementation to select the RO type. Random access procedure in SBFD symbols is supported for all the existing RACH trigger events except for SI request. FFS for LTM.

### 8.11.2	Random access in SBFD

RAN2 impacts to support SBFD operation to support random access in SBFD symbols by UEs in RRC _CONNECTED mode and RRC_IDLE/INACTIVE mode.

On RSRP threshold for RO type selection (e.g., configuration, related procedure, etc.)

R2-2502316	Random Access Operation of SBFD	Nokia Corporation	discussion	Rel-19	NR_duplex_evo-Core

Noted

Proposal 1: For Option 2, in case the legacy ROs and the additional ROs have the same value for one of the RACH configuration mandatory parameters, only the legacy RACH configuration will be configured with this parameter.  For Option 2, in case the SBFD aware UEs did not receive a dedicated mandatory parameter in the additional RACH configuration, the SBFD-aware UEs determine this dedicated parameter from the legacy RACH configuration.

Proposal 2: Since the RACH configuration Option 1 is focused on reducing the latency of RA for SBFD UEs in good coverage area, we propose SBFD UEs selects the additional RO for initial access when the measured RSRP is above the RSRP threshold.

Proposal 3:  For RACH configuration Option 2, in case the additional prachConfigurationIndex indicates long PRACH preamble formats, the UE selects the additional ROs for initial access when the measured RSRP is below the RSRP threshold.

R2-2501860	Random Access for SBFD Operation	NEC	discussion	Rel-19	NR_duplex_evo-Core

Noted

Proposal 1:	The RO type (legacy RO or additional RO) indication is provided by SIB1 for the case of CBRA. FFS whether UE specific dedicated RRC signalling is needed.

Proposal 2:	Selection of the additional RO type below and above the configured SSB RSRP are both supported.

Proposal 3: The network configures when the additional RO to apply, the SSB measurement result is either below or above the configured SSB RSRP threshold.

R2-2502000	Random access in SBFD	Samsung	discussion	Rel-19

Noted

Proposal 2. NW may indicate whether the SBFD RO is selected when SSB RSRP are 'below' or 'above' the configured threshold.

DISCUSSION

P2 from Samsung

-	Apple think it is reasonable. Ericsson understand P2 means we use a choice structure to indicate, and agree with the proposal.

-	ZTE think the main case is for cell center UEs, no need for duplicated functionality for cell edge. IDT, Lenovo share this view. IDT agree with Nokia proposal.

-	Nokia explains that in their proposal it goes one step further to provide in which resource configuration we use ‘above’ or ‘below’ the threshold as the condition. QC understand the difference btw Nokia proposal and Samsung proposal is that the latter requires explicit signalling. QC prefer Nokia proposal. Samsung think UE may not support all the options.  Ericsson think Nokia proposal require more complexity in UE.

-	HW do not think it is good to use implicit signalling. LG E share this view and do not want to tie the threshold configuration with other configurations. Apple agree.

NW indicate via explicit signaling whether the SBFD RO is selected when SSB RSRP are 'below' or 'above' the configured threshold.

RO type selection before or after RA type selection

R2-2501849	Random Access in SBFD symbols	CATT	discussion	Rel-19	NR_duplex_evo-Core

Noted

Proposal 1: For SBFD-aware UE, the selection of RO type is suggested to be performed before the selection of the set of Random Access resources.

Proposal 2: For SBFD-aware UE, whether msg1/3 repetition is applied is determined based on the selected RO type.

R2-2501878	Impacts on the random access by the evolution of duplex operation	Huawei, HiSilicon	discussion	Rel-19	NR_duplex_evo-Core

Noted

Proposal 7: Support to select RO type after the RA type selection.

DISCUSSION

-	LG E has concern on HW P7. Ericsson, IDT, Samsung share this concern. Apple think there is no big issue from UE processing point of view.

-	Charter, Apple, ZTE, OPPO support HW proposal, since R1 did not agree 2step RA.

-	QC slightly prefer CATT proposal considering the complexity of the procedure.

-	MAC rapp think CATT proposal is simpler from spec impact point of view. Sharp agree.

-	OPPO think we should consider both spec impact and UE implementation.

-	CMCC think this agreement relate to fallback mechanism.

-	HW suggest to have post meeting email discussion on this topic. Samsung do not think this is needed, as pros and cons have been discussed sufficiently.

-	Samsung not sure how to implement the HW proposal in MAC. CATT agree.

Working assumption: For SBFD-aware UE, the selection of RO type is suggested to be performed before the selection of the set of Random Access resources.

On RO type fallback

R2-2501945	Discussion on Random Access in SBFD	Sharp	discussion	Rel-19	NR_duplex_evo-Core

Noted

Proposal 1	RO-Type change procedure on RO type selection from legacy RO to additional RO in SBFD symbols is supported when the number of PRACH transmission attempts exceed a threshold.

R2-2502967	Random Access in SBFD	Lenovo	discussion	Rel-19	NR_duplex_evo-Core

Noted

Proposal 6: Switching from the PRACH resources in non-SBFD symbols to the PRACH resources in SBFD symbols is not supported.

DISCUSSION

-	Nokia, Ericsson, IDT, Samsung support P1 from Sharp.

-	LG E, CATT support Lenovo proposal.

-	Charter wonders why additional RO can succeed after fail on legacy RO.

-	Lenovo ask what is the procedure if UE still fails after fallback to SBFD RO.

-	CATT ask what if UE fallback from 2step to 4step, can UE again fallback to additional RO after 4step. ZTE think this is valid.

RO-Type change procedure on RO type selection from legacy RO to additional RO in SBFD symbols is supported when the number of PRACH transmission attempts exceed a threshold (we assume it is the same threshold with the fallback from additional RO to legacy RO). If fallback from legacy RO to additional RO occurs, no further fallback to legacy RO is supported.

RA-RNTI collision

R2-2502495	Random access for SBFD Operation	Sony	discussion	Rel-19	NR_duplex_evo-Core

Noted

Proposal 6: Simply introduce a configurable offset for the symbol index (s_id) in the RNTI equation, and the offset is signalled in SIB1 as a part of PRACH resource configuration for SBFD-aware UEs.

R2-2502565	SBFD RA aspects	Ericsson	discussion	Rel-19	NR_duplex_evo-Core

Noted

Proposal 10	No need to update the RA-RNTI formula for SBFD RA configuration.

DISCUSSION

-	Sony think we need to handle the collision. Xiaomi, Nokia agree.

-	Ericsson think in R1 there is understanding that it is up to NW implementation. Ericsson think if we enhance there would be more issues. OPPO agree.

R2-2502510	Remaining issues for RACH in SBFD	Apple	discussion	Rel-19	NR_duplex_evo-Core

Noted

Proposal 7: RAN2 should consult with RAN1 and RAN4 whether to support CSI-RS based CFRA on SBFD symbols.

DISCUSSIOS

-	OPPO think R1 already agreed CSI-RS meas on SBFD symbol

-	LG E think this issue is valid, and this requires more discussions in R1/R4 regarding measurement and reporting.

-	Ericsson think we do not need to discuss this, and it is up to UE implementation which is quite simple.

-	ZTE think this concern is valid but think we can FFS and do not think a LS is needed. Xiaomi agree.

-	Apple think current running CR already covers CSI-RS based RACH so this is not right. Apple encourages companies to do checking internally and can discuss in the future.

-	HW has sympathy on Apple comments and think MAC spec editor can remove this related part if that is controversial.

Additional agreements on SBFD RACH aspects

NW indicate via explicit signaling whether the SBFD RO is selected when SSB RSRP are 'below' or 'above' the configured threshold. Working assumption: For SBFD-aware UE, the selection of RO type is suggested to be performed before the selection of the set of Random Access resources. RO-Type change procedure on RO type selection from legacy RO to additional RO in SBFD symbols is supported when the number of PRACH transmission attempts exceed a threshold (we assume it is the same threshold with the fallback from additional RO to legacy RO). If fallback from legacy RO to additional RO occurs, no further fallback to legacy RO is supported.

R2-2501797	Discussion on RACH in SBFD	Xiaomi	discussion	Rel-19

R2-2501849	Random Access in SBFD symbols	CATT	discussion	Rel-19	NR_duplex_evo-Core

R2-2501860	Random Access for SBFD Operation	NEC	discussion	Rel-19	NR_duplex_evo-Core

R2-2501878	Impacts on the random access by the evolution of duplex operation	Huawei, HiSilicon	discussion	Rel-19	NR_duplex_evo-Core

R2-2501945	Discussion on Random Access in SBFD	Sharp	discussion	Rel-19	NR_duplex_evo-Core

R2-2502000	Random access in SBFD	Samsung	discussion	Rel-19

R2-2502082	Discussion on random access procedure in SBFD	ZTE Corporation	discussion	Rel-19	NR_duplex_evo-Core

R2-2502316	Random Access Operation of SBFD	Nokia Corporation	discussion	Rel-19	NR_duplex_evo-Core

R2-2502387	Discussion on random access procedure in SBFD	vivo	discussion	Rel-19	NR_duplex_evo-Core

R2-2502394	Remaining issues of SBFD RACH procedure	OPPO	discussion	Rel-19	NR_duplex_evo-Core

R2-2502495	Random access for SBFD Operation	Sony	discussion	Rel-19	NR_duplex_evo-Core

R2-2502510	Remaining issues for RACH in SBFD	Apple	discussion	Rel-19	NR_duplex_evo-Core

R2-2502565	SBFD RA aspects	Ericsson	discussion	Rel-19	NR_duplex_evo-Core

R2-2502566	CSI-RS measurements and SBFD operation in CA and DC	Ericsson	discussion	Rel-19	NR_duplex_evo-Core

R2-2502588	Views on random access for SBFD	Qualcomm Incorporated	discussion	NR_duplex_evo-Core

R2-2502642	Discussion on Random Access operation in SBFD	InterDigital, Inc.	discussion	Rel-19	NR_duplex_evo-Core

R2-2502706	Discussion on random access in SBFD	CMCC	discussion	Rel-19	NR_duplex_evo-Core

R2-2502850	Discussion on Random Access procedure for SBFD	LG Electronics Inc.	discussion	Rel-19	NR_duplex_evo-Core

R2-2502967	Random Access in SBFD	Lenovo	discussion	Rel-19	NR_duplex_evo-Core

### 8.11.3	Other aspects

Other RAN2 impacts with SBFD if not covered by the previous agenda items.

SBFD in DC

R2-2502083	Discussion on multiple carrier and measurements in SBFD	ZTE Corporation	discussion	Rel-19	NR_duplex_evo-Core

Noted

Proposal 1: Support SBFD cell(s) to be configured in DC, including NR-DC, (NG)EN-DC and NE-DC.

Proposal 2: RAN2 supports the following:

	In EN-DC and NGEN-DC, only the SN can configure SBFD;

	In NE-DC, only the MN can configure SBFD;

	In NR-DC, both the MN and the SN can configure SBFD.

R2-2502318	Other impacts by the evolution of duplex operation	Huawei, HiSilicon	discussion	Rel-19	NR_duplex_evo-Core

Noted

Proposal 1: Send an LS to RAN1 on whether SBFD can work with DC case and if yes, whether any special handling is needed.

DISCUSSION

-	LG E, CATT share the view from HW, and think we can check with R1.

-	IDT do not support DC since there is R3 signaling design, think DC is not so critical.

-	Ericsson think either DC or CA critical and think R4 conclusion is not clear yet. Ericsson think there are more issue from UE point of view. Ericsson want to ask both CA and DC if we send LS, QC share this view. CATT think we only ask DC is sufficient, because R1 already discussed CA.

-	CATT think we need to finalize to complete MAC spec.

-	QC also agree with HW proposal.

-	Samsung do not think DC is within WID scope and think RP involvement is needed. Vivo share this view.

-	HW also think R1 is currently working on CA, so we do not need to ask CA. ZTE agree.

-	Ericsson think it is clear NW can configured SBFD in MCG.

Send an LS to RAN1 (CC RAN3 and RAN4) on whether SBFD and DC can be configured simultaneously, and whether there is any issue for such configuration.

[Post129bis][219][SBFD] LS to RAN1 (CC RAN3/RAN4) on simultaneous configuration of SBFD and DC (Huawei)

Intended outcome: Approve the LS

Deadline:  Short

R2-2503191	LS on simultaneous configuration of SBFD and DC	RAN2	LS out	Rel-19	NR_duplex_evo-Core	To:RAN1	Cc:RAN3, RAN4

=> Approved

MAC CE design SP CLI Measurement Resource Set Activation/Deactivation

R2-2502801	Other Aspects of SBFD	Samsung	discussion	Rel-19	NR_duplex_evo-Core

Noted

Proposal 2: RAN2 introduce the new MAC CE for semi-persistent CLI resource activation/deactivation which include at least following information.

-        A/D field: indicates the activate or deactivate of the indicated CLI resources

-        Source cell/BWP ID which the CLI resources configuration is provided

-        CLI measurement resource type (either SRS-RSRP or CSI-RSSI)

-        CLI measurement resource set ID

-        CLI measurement resource(s) ID

Proposal 9: The configured SP CLI measurement resource sets are initially deactivated upon (re-)configuration by upper layers and after reconfiguration with sync.

Proposal 10: RAN2 consider the above MAC CE format for SP CLI Measurement Resource Set Activation/Deactivation as a baseline.

R2-2501883	Other aspects of SBFD	Xiaomi	discussion	Rel-19	NR_duplex_evo-Core

Noted

Proposal 3: SP CLI measurement resource set activation/deactivation MAC CE includes following fields: A/D, Serving Cell ID, BWP ID, CLI measurement resource set ID (for CLI-RSSI or SRS-RSRP measurement), TCI State IDs.

R2-2502589	Other aspects of SBFD	Qualcomm Incorporated	discussion	NR_duplex_evo-Core

Noted

Proposal 3: In the new SP CLI measurement resource set activation/deactivation MAC CE, the following fields are included, i.e., A/D field, serving cell/BWP ID field, CLI-RSSI or SRS-RSRP resource set ID field and the corresponding TCI state ID fields.

R2-2501850	Discussion on other aspects of SBFD	CATT	discussion	Rel-19    NR_duplex_evo-Core

Noted

Proposal 2: RAN2 to discuss which option to support for the new SP CLI MAC CE:

-              Option 1: Only one type of resource (L1 CLI-RSSI or L1 CLI-SRS-RSRP) can be included in one MAC CE,

-              Option 2:  Both types of SP L1 CLI measurement resources (L1 CLI-RSSI and L1 CLI-SRS-RSRP) can be included in one MAC CE.

DISCUSSION

P3 from Xiaomi paper

-	MAC editor suggest that Xiaomi proposal is quite simple.

-	Samsung want to confirm it as agreement. QC share this view.

P2 from CATT paper

ZTE ask for CATT P2 how do we introduce TCI state in the MAC CE, and only want to indicate one type of resource. LG E think Option 1 is sufficient but there is R1 dependency.

-	LG E suggest we try to make a WA based on option 1.

-	Samsung not sure how Option 2 can work from signalling design point of view.

-	OPPO think if RAN1 agree with simultaneous configuration of two types of resources we can design the MAC CE to serve that purpose.

Working assumption: The configured SP CLI measurement resource sets are initially deactivated upon (re-) configuration by upper layers and after reconfiguration with sync.

SP CLI measurement resource set activation/deactivation MAC CE includes following fields: A/D, Serving Cell ID, BWP ID, CLI measurement resource set ID (for CLI-RSSI or SRS-RSRP measurement), TCI State IDs.

Agreements on other SBFD aspects

Send an LS to RAN1 (CC RAN3 and RAN4) on whether SBFD and DC can be configured simultaneously, and whether there is any issue for such configuration. Working assumption: The configured SP CLI measurement resource sets are initially deactivated upon (re-) configuration by upper layers and after reconfiguration with sync. SP CLI measurement resource set activation/deactivation MAC CE includes following fields: A/D, Serving Cell ID, BWP ID, CLI measurement resource set ID (for CLI-RSSI or SRS-RSRP measurement), TCI State IDs.

R2-2501850	Discussion on other aspects of SBFD	CATT	discussion	Rel-19	NR_duplex_evo-Core

R2-2501883	Other aspects of SBFD	Xiaomi	discussion	Rel-19	NR_duplex_evo-Core

R2-2502083	Discussion on multiple carrier and measurements in SBFD	ZTE Corporation	discussion	Rel-19	NR_duplex_evo-Core

R2-2502318	Other impacts by the evolution of duplex operation	Huawei, HiSilicon	discussion	Rel-19	NR_duplex_evo-Core

R2-2502388	SBFD other aspects	vivo	discussion	Rel-19	NR_duplex_evo-Core

R2-2502395	Discussion on the SBFD related measurement and BFR	OPPO	discussion	Rel-19	NR_duplex_evo-Core

R2-2502644	Discussion on resource configuration in SBFD	InterDigital, Inc.	discussion	Rel-19	NR_duplex_evo-Core

R2-2502801	Other Aspects of SBFD	Samsung	discussion	Rel-19	NR_duplex_evo-Core

R2-2502851	Other aspects on SBFD	LG Electronics Inc.	discussion	Rel-19	NR_duplex_evo-Core

R2-2502918	Other aspects of SBFD	Nokia	discussion	Rel-19	NR_duplex_evo-Core

## 8.12	NR MIMO Phase 5

(NR_MIMO_Ph5-Core; leading WG: RAN1; REL-19; WID: RP-242394)

Time budget: 0.5 TU

Tdoc Limitation: 2 tdocs

### 8.12.1	Organizational

LSs and rapporteur input, including workplan, running CRs, etc.

Output of email discussion [Post129][208].

LS

R2-2501705	LS to RAN2 on MAC impacts for Rel-19 NR MIMO Ph5 (R1-2500846; contact: Samsung)	RAN1	LS in	Rel-19	NR_MIMO_Ph5	To:RAN2

Noted

Work plan

R2-2502715	Work Plan for Rel-19 on NR MIMO Phase 5	CMCC,Samsung,MediaTek	Work Plan	Rel-19	NR_MIMO_Ph5-Core

Noted

Running CRs

Chair: Plan is to directly note these draft CRs, and update/endorse with post meeting email discussions. Also, we will discuss and prepare open issue lists (e.g., per spec) after this meeting.

R2-2502545	Running CR for MIMO Phase 5	Ericsson	CR	Rel-19	38.331	18.5.1	5306	-	B	NR_MIMO_Ph5-Core

Noted, will be updated after the meeting

[Post129bis][214][ MIMO_Ph5] Running CR for 38.331 (Ericsson)

Intended outcome: Updated running CR based on new agreements for endorsement, open issue list (if needed)

Deadline:  Long

R2-2502664	Introduction of Rel-19 MIMO	Samsung	draftCR	Rel-19	38.321	18.5.0	B	NR_MIMO_Ph5-Core

Noted, will be updated after the meeting

[Post129bis][215][ MIMO_Ph5] Running CR for 38.321 (Samsung)

Intended outcome: Updated running CR based on new agreements for endorsement, open issue list (if needed)

Deadline:  Long

R2-2502716	Draft Running 38300 CR for Rel-19 MIMO Phase 5	CMCC,Samsung,MediaTek	draftCR	Rel-19	38.300	18.5.0	B	NR_MIMO_Ph5-Core

=> Revised in R2-2502988

R2-2502988	Draft Running 38300 CR for Rel-19 MIMO Phase 5	CMCC	draftCR	Rel-19	38.300	18.5.0	B	NR_MIMO_Ph5-Core

Noted, will be updated after the meeting

[Post129bis][216][ MIMO_Ph5] Running CR for 38.300 (CMCC)

Intended outcome: Updated running CR based on new agreements for endorsement, open issue list (if needed)

Deadline:  Long

Output of email discussion [Post129][208]

R2-2502546	Report of [Post129][208][ MIMO_Ph5]	Ericsson	discussion

=> Moved to 8.12.3

### 8.12.2	Asymmetric DL sTRP/UL mTRP

RRC/MAC aspects related to asymmetric DL sTRP/UL mTRP

Remaining aspects on MAC CE design

Number of TCI states

R2-2503033	Discussion on Asymmetric DL sTRP/UL mTRP	CMCC	discussion	Rel-19	NR_MIMO_Ph5-Core

Noted

Proposal 1：We think there is no need to add a maximum number restriction of the TCI states indicated by the PL offset MAC CE.

R2-2502834	Discussion on remaining issues on Asymmetric DL sTRP/UL mTRP	Huawei, HiSilicon	discussion	Rel-19	NR_MIMO_Ph5-Core

Noted

Proposal 1: Select one from the following two options for the new MAC CE format.

- Option1: Use 1-bit reserved field in the new MAC CE to indicate whether the next TCI State/PL offset exists, as shown in Figure 3.

- Option2: Use a new field in the new MAC CE to indicate the exact number of PL offsets, as shown in Figure 4.Proposal 2: The new MAC CE for PL offset update includes the impacted TCI State ID(s).

DISCUSSONS

-	Apple support P1 from CMCC. CATT agree.

-	Nokia do not support P1 from HW paper, since we can rely on length field. HW think the length filed is not typically used for that purpose. LG E disagree with HW comments.

-	LG E points out the maximum number depends on the number of configured TCI states.

No need to add a maximum number restriction of the TCI states indicated by the PL offset MAC CE.

Applicable channels

R2-2502146	Discussion on Asymmetric DL sTRP UL mTRP	CATT	discussion	Rel-19	NR_MIMO_Ph5-Core

Noted

Proposal 1: The PL offset update MAC CE is at least applicable to PUCCH, PUSCH, SRS, and PDCCH-order PRACH.

DISCUSSONS

-	Apple think we should make it clear it is for PDCCH-ordered CFRA.

-	Ericsson think this is just to align the understanding and there is no impact to R2 spec. Samsung agree.

RAN2 understand the PL offset update MAC CE is at least applicable to PUCCH, PUSCH, SRS, and PDCCH-order CFRA.

Whether a note is need in the MAC spec

R2-2502392	Discussion on the remaining issues of the pathloss offset update	OPPO	discussion	Rel-19	NR_MIMO_Ph5-Core

Noted

Proposal 1: To clarify the NOTE 1 in 5.4.6 of TS 38.321 that the pathloss used for triggering PHR includes both the measurement result of the pathloss reference and the pathloss offset.

R2-2501993	Consideration on Asymmetric DL sTRP/UL mTRP	LG Electronics Inc.	discussion	Rel-19	NR_MIMO_Ph5-Core

Noted

Proposal 1. RAN2 consider to revert the following agreement in the last meeting.

	RAN2 understands that if a joint/UL TCI state is configured with a PL offset, PHR trigger is based on the PL change of the PL-RS associated to the joint/UL TCI, where the PL change takes into account the PL offset. FFS whether/how to capture this.

DISCUSSIONS

-	LG E ok now to have clarifications in note. Samsung think we need further checking regarding the exact wording of the note.

-	Xiaomi think we should clarify the PL offset is only for triggering PHR type 1. Ericsson share this view.

-	QC think there is the case where PL offset is not configured at all.

We will capture in a note to reflect the previous understanding ‘RAN2 understands that if a joint/UL TCI state is configured with a PL offset, PHR trigger is based on the PL change of the PL-RS associated to the joint/UL TCI, where the PL change takes into account the PL offset.’. FFS on exact wording.

WA of PL update

R2-2502665	Discussion on Asymmetric DL sTRP/UL mTRP	Samsung	discussion	Rel-19	NR_MIMO_Ph5

Noted

Proposal 2: RAN2 assumes that UE applies the latest PL offset value received in RRC or MAC CE

R2-2502866	Consideration on the Remaining Issues of PL Offset MAC CE	ZTE Corporation	discussion	Rel-19	NR_MIMO_Ph5-Core

Noted

Proposal 1: RAN2 to discuss how to capture the working assumption that the “UE applies the latest PL offset value received in RRC or MAC CE.”

Proposal 1a: RAN2 to indicate the working assumption that the “UE applies the latest PL offset value received in RRC or MAC CE.” to RAN1 and to confirm with RAN1 whether this would be captured in RAN1 spec.

DISCUSSIONS

-	Samsung explain that with P2 in their contribution there is no need for further spec change. OPPO agree.

-	OPPO think the current MAC CR (deliver the updated value to lower layer) is not sufficient, and think UE also need to update the stored value in RRC. Apple do not agree.

-	HW think there is issue in HO case when target gNB do not know whether UE already updates the value based on MAC CE. Nokia think the case off RRC resume should also be considered.

-	Ericsson ok to confirm the WA, and think other details can perhaps be handled by need code or field description. CMCC agree.

From RAN2 point of view, UE applies the latest PL offset value received in RRC or MAC CE.

On 2TA

R2-2502665	Discussion on Asymmetric DL sTRP/UL mTRP	Samsung	discussion	Rel-19	NR_MIMO_Ph5

Noted

Proposal 4: For 2TA in asymmetric DL sTRP/UL mTRP scenario, Rel-18 2TA operation is applied with the following RRC changes:

•	remove the restriction that RRC field tag2 is configured only if coresetPoolIndex is configured with more than one value;

•	a single n-TimingAdvanceoffset is configured, i.e., n-TimingAdvanceOffset2 is not configured for 2TA in asymmetric DL sTRP/UL mTRP scenario.

DISCUSSIONS

-	Samsung think with these agreements, the impact is in RRC.

-	Samsung explains that after offline the concern was removed, and suggested to add that this is with pathloss offset configured.

For 2TA in asymmetric DL sTRP/UL mTRP scenario with pathloss offset configured Rel-18 2TA operation is applied with the following RRC changes:

- •	remove the restriction that RRC field tag2 is configured only if coresetPoolIndex is configured with more than one value;

- •	a single n-TimingAdvanceoffset is configured, i.e., n-TimingAdvanceOffset2 is not configured for 2TA in asymmetric DL sTRP/UL mTRP scenario.

On RACH

R2-2502834	Discussion on remaining issues on Asymmetric DL sTRP/UL mTRP	Huawei, HiSilicon	discussion	Rel-19	NR_MIMO_Ph5-Core

Noted

Proposal 6: For PRACH transmission, PL offset is applied only to PDCCH-order CFRA, and no further MAC impact is needed.

R2-2501943	Discussion on Asymmetric DL sTRP/UL mTRP	Xiaomi	discussion	Rel-19	NR_MIMO_Ph5-Core

Noted

Proposal 2: When PDCCH-order indicates the pathloss offset, RSRP used for the selection of RACH procedure combines both RSRP measured for pathloss reference and pathloss offset.

R2-2502317	RAN2 Aspects of Asymmetric DL sTRP/UL mTRP	Nokia Corporation	discussion	Rel-19	NR_MIMO_Ph5-Core

Noted

Proposal 3: Discuss  the impact of PL offsets on UL transmissions of RACH procedures, such as PRACH, Msg3 (for the 4-step RACH procedure) and MsgA (for the 2-step RACH procedure).

Proposal 4: If RAN2 determines that PL offset impacts the RACH procedures, then RAN2 should examine whether the PL offset affects the RSRP comparison in RA resource selection.

DISCUSSIONS

-	Samsung think HW P6 already confirmed by RAN1, and think with that we need to further check the MAC spec impact.

-	On Xiaomi P2, Samsung agree we need to discuss the RSRP in the RACH procedure. Ericsson agree.

-	Apple think there is no issue for PDCCH order RACH. HW, QC, LG E share Apple’s view.

-	QC point out the case for UE initiated RACH is quite difficult. ZTE think we can first decide whether this is only applicable to PDCCH-order CFRA, Nokia agree.

-	LG E wonders how do we know the association btw RACH resource and the TCI state (thus PL offset).

For PRACH transmission, PL offset is applicable only to PDCCH-order CFRA.

Agreements on Asymmetric DL sTRP/UL mTRP

No need to add a maximum number restriction of the TCI states indicated by the PL offset MAC CE. RAN2 understand the PL offset update MAC CE is at least applicable to PUCCH, PUSCH, SRS, and PDCCH-order CFRA. We will capture in a note to reflect the previous understanding ‘RAN2 understands that if a joint/UL TCI state is configured with a PL offset, PHR trigger is based on the PL change of the PL-RS associated to the joint/UL TCI, where the PL change takes into account the PL offset.’. FFS on exact wording. From RAN2 point of view, UE applies the latest PL offset value received in RRC or MAC CE. For 2TA in asymmetric DL sTRP/UL mTRP scenario with pathloss offset configured Rel-18 2TA operation is applied with the following RRC changes: - • remove the restriction that RRC field tag2 is configured only if coresetPoolIndex is configured with more than one value; - • a single n-TimingAdvanceoffset is configured, i.e., n-TimingAdvanceOffset2 is not configured for 2TA in asymmetric DL sTRP/UL mTRP scenario. For PRACH transmission, PL offset is applicable only to PDCCH-order CFRA.

R2-2501943	Discussion on Asymmetric DL sTRP/UL mTRP	Xiaomi	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2501993	Consideration on Asymmetric DL sTRP/UL mTRP	LG Electronics Inc.	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2502063	Further discussion on asymmetric DL sTRP and UL mTRP	SHARP Corporation	discussion	NR_MIMO_Ph5-Core

R2-2502146	Discussion on Asymmetric DL sTRP UL mTRP	CATT	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2502167	Discussion on MAC CE impact for asymmetric DL sTRP/UL mTRP scenarios	vivo	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2502304	Asymmetric DL/UL mTRP user plane impacts	Ericsson	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2502317	RAN2 Aspects of Asymmetric DL sTRP/UL mTRP	Nokia Corporation	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2502373	Discussion on PL offset	Lenovo	discussion	Rel-19

R2-2502392	Discussion on the remaining issues of the pathloss offset update	OPPO	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2502496	Enhancement for Asymmetric DL sTRP/UL mTRP	Sony	discussion	Rel-19	NR_MIMO_Ph5

R2-2502536	Discussion on asymmetric DL sTRP/UL mTRP	China Telecommunications Corp.	discussion

R2-2502543	Discussion on UL only mTRP 	Qualcomm Incorporated	discussion

R2-2502665	Discussion on Asymmetric DL sTRP/UL mTRP	Samsung	discussion	Rel-19	NR_MIMO_Ph5

R2-2502713	Discussion on Asymmetric DL sTRP/UL mTRP	CMCC	discussion	Rel-19	NR_MIMO_Ph5-Core

=> Revised in R2-2503033

R2-2502827	Discussion on PL offset value for Asymmetric DL sTRP/UL mTRP	ASUSTeK	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2502834	Discussion on remaining issues on Asymmetric DL sTRP/UL mTRP	Huawei, HiSilicon	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2502866	Consideration on the Remaining Issues of PL Offset MAC CE	ZTE Corporation	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2503033	Discussion on Asymmetric DL sTRP/UL mTRP	CMCC	discussion	Rel-19	NR_MIMO_Ph5-Core

### 8.12.3	Others

Other issues if not covered by the previous agenda items.

UE initiated beam report

DRX

R2-2501986	Enhancements for UE-initiated/event-driven beam management	Ofinno, LLC	discussion

Noted

Proposal 1: In Mode A of UE-initiated CSI reporting, the active time of a DRX operation includes the time when a new UCI for UE-initiated beam reporting is sent on first PUCCH.

Proposal 2: To discuss whether UE transmits UE-initiated CSI report outside of DRX active time.

R2-2502314	Discussion on UE-initiated Beam Reporting	Apple	discussion	Rel-19	NR_MIMO_Ph5-Core

Noted

Proposal 3: There may be some impact on CDRX operation if the event triggered BR is triggered outside DRX active time.

DISCUSSION

-	HW agree with P1 from Ofinno. HW wonders whether UE can transmit the new UCI if it is NOT in active time. Also HW ask whether the active time ends after UE sends the PUSCH.

-	Apple think UE can transmit the new UCI outside DRX active time, but it should go back to active time to receive the UL grant for mode A. QC, Nokia share this view.

-	LG E think new UCI is similar to SR in legacy so similar behaviour should apply.

-	Samsung wonders whether we need to further check mode B.

In Mode A of UE-initiated CSI reporting, the active time of a DRX operation includes the time after a new UCI for UE-initiated beam reporting is sent on first PUCCH.

UL Skipping

R2-2502393	Clarification on the uplink grant used for the UE initiated beam report	OPPO	discussion	Rel-19	NR_MIMO_Ph5-Core

Noted

Proposal: RAN2 is kindly requested to confirm the understandings that the pre-configured CG PUSCH for the UE initiated beam report in Mode-B does not carry MAC PDU.

R2-2502666	Discussion on UE-initiated Beam Reporting and CSI enhancement	Samsung	discussion	Rel-19	NR_MIMO_Ph5

Noted

Proposal 5: The current UL skipping procedure already exclude UEI BR since it is transmitted in UCI, no additional MAC impact.

Proposal 6: The type-1 CG allocated for the mode-B UEI BR is transparent to MAC and excluded from MAC UL-SCH transmission procedure.

FFS if any other MAC impact for UL skipping

R2-2503036	Way forward on the uplink grant used for UEIBR	OPPO, Huawei, Ofinno, Samsung, ZTE, Qualcomm, LG, Nokia, ASUSTek	discussion	Rel-19	NR_MIMO_Ph5-Core

Noted

DISCUSSION

-	Ericsson ask whether we send to R1. Nokia and OPPO think this is common understanding in R2 and no need to send LS.

Confirm the following RAN2 understandings:

      - The CG type-1 PUSCH carrying the beam report of Mode-B does not carry MAC PDU (i.e. UL-SCH).

      - The DG PUSCH carrying the beam report of Mode-A carries MAC PDU (i.e. UL-SCH) as legacy.

BWP

R2-2502833	Enhancements for UE-initiated/event-driven beam management	Huawei, HiSilicon	discussion	Rel-19	NR_MIMO_Ph5-Core

Noted

Proposal 7: The UE continues to perform CSI measurements for the UEIBM procedure when the active BWP is the dormant BWP.

Proposal 8: If the BWP in an SCell is a dormant BWP, the UE should not report mode-A beam measurement results. The UE cannot perform mode-B beam reporting on this BWP.

DISCUSSION

-	LG E agree and think this is same with legacy behaviour. Samsung agree.

-	Apple wonders if any spec is needed with P7 and P8. Samsung think we need to add something.

The UE continues to perform CSI measurements for the UEIBM procedure when the active BWP is the dormant BWP.

If the BWP in an SCell is a dormant BWP, the UE should not report mode-A beam measurement results. The UE cannot perform mode-B beam reporting on this BWP.

R2-2502828	Discussion on MAC impact regarding UEI reporting	ASUSTeK	discussion	Rel-19	NR_MIMO_Ph5-Core

Noted

Proposal:	For UE-initiated/event-driven beam reporting Mode-B, the MAC entity shall start or restart bwp-InactivityTimer associated with the Cell where the CG-PUSCH resource of a CSI report configuration is configured if the MAC entity performs the PUCCH transmission of the CSI report configuration.

DISCUSSION

-	LG E think UEI BR is different case and think UE does not re-start, since it is not for data transmission. Apple share this view.

-	OPPO think this is enhancement and think if UE switch BWP then the old results are less useful.

-	ASUSTeK think today the timer is restarted for CG grant, so it is not that different, and think without this proposal what is the UE behaviour.

Coordination btw R1&R2 work

R2-2501994	Discussion on UEI beam reporting impact	LG Electronics Inc.	discussion	Rel-19	NR_MIMO_Ph5-Core

Noted

Proposal 1. RAN2 does not discuss to capture the event evaluation procedure in MAC specification.

R2-2502544	Discussion on UE Initiated Beam Report 	Qualcomm Incorporated	discussion

Noted

Proposal 1. RAN2 confirms the following working assumption:

“RAN2 assume the event evaluation for UE-initiated beam reporting is captured in 38.321, where the evaluation is based on indications from measurements described in a RAN1 specification. The final decision is up to RAN1, we can revisit this if R1 has a different decision.”

Proposal 2. If event evaluation is handled by MAC layer, it contains the following steps:

PHY send indication per new beam at each evaluation occasion

MAC decides whether the event is triggered

If event is triggered, MAC indicates PHY to send the UCI and a CSI report on PUSCH.

DISCUSSION

-	LG E point out that R1 already agreed to include in their spec.

-	WI rapp suggest to capture a clear conclusion to reflect our understanding.

RAN2 understand the event evaluation and report triggering for UE-initiated beam report is captured by RAN1 spec.

RRC impact

R2-2502546	Report of [Post129][208][ MIMO_Ph5]	Ericsson	discussion

Noted

enabledCurrentBeamReport-r19 is added as an optional need-R field.

Reuse resourcesForChannelMeasurement in CSI-ReportConfig. Clarify in the field description that for UEI BM, the new beam to be measured is either CSI-RS (nzp-CSI-RS-ResourceSetList) or SSB (csi-SSB-ResourceSetList).

ng-n1-n2-r19, cri-typeI-SinglePanel-ri-restriction-r19/cri-typeII-ri-restriction-r19 and cri-typeI-SinglePanel-CBSR-r19/cri-typeII-CBSR-r19 in the same way as corresponding legacy fields i.e.:

        - ng-n1-n2-r19 is defined in the same way as ng-n1-n2 in R15 typeI-multiPanel

        - cri-typeI-SinglePanel-ri-restriction-r19/cri-typeII-ri-restriction-r19 are defined in the same way as legacy RI restrictions

        - cri-typeI-SinglePanel-CBSR-r19/cri-typeII-CBSR-r19 are defined in the same way as n1-n2-codebookSubsetRestriction-r18.

mrSelectedResources is defined as a SEQUENCE structure containing two fields with integer values from one to eight.

delayOffsetCompensation can be located under CSI-AperiodicTriggerState and outside of CSI-AssociatedReportConfigInfo and that the parameter triggeringScheme is not needed.

Define numberofSubbandsPO as a list (with size up to the number of subbands) where each element is an integer value within the maximum size of a BWP.

DISCUSSION

-	Samsung think P1 can be postponed and we should wait for R1 further input.

-	Ericsson point out that option 3 as it is does not work, so suggest to have option. 2.

The following is used the signaling of typeI-CBSR and typeII-CBSR and typeI-softScalingRank:

- (N1, N2) can be signaled as a separate parameter, and CBSR can be signaled as a CHOICE of (X1, X2) and a CHOICE of N1N2;

R2-2502547	Impacts from other NR MIMO Phase 5 objectives	Ericsson	discussion

Noted

Proposal 2	CSI-typeI/II group based CBSR is defined as option 2 i.e. (N1, N2) can be signaled as a separate parameter, and CBSR can be signaled as a CHOICE of (X1, X2) and a CHOICE of N1N2.

Proposal 3	The same structure defined for CBSR CSI-typeI/II is also used for typeI-softScalingRank.

Agreements on other aspects

MAC impact In Mode A of UE-initiated CSI reporting, the active time of a DRX operation includes the time after a new UCI for UE-initiated beam reporting is sent on first PUCCH. Confirm the following RAN2 understandings: - The CG type-1 PUSCH carrying the beam report of Mode-B does not carry MAC PDU (i.e. UL-SCH). - The DG PUSCH carrying the beam report of Mode-A carries MAC PDU (i.e. UL-SCH) as legacy. FFS if any other MAC impact for UL skipping The UE continues to perform CSI measurements for the UEIBM procedure when the active BWP is the dormant BWP. If the BWP in an SCell is a dormant BWP, the UE should not report mode-A beam measurement results. The UE cannot perform mode-B beam reporting on this BWP. RAN2 understand the event evaluation and report triggering for UE-initiated beam report is captured by RAN1 spec. RRC impact enabledCurrentBeamReport-r19 is added as an optional need-R field. Reuse resourcesForChannelMeasurement in CSI-ReportConfig. Clarify in the field description that for UEI BM, the new beam to be measured is either CSI-RS (nzp-CSI-RS-ResourceSetList) or SSB (csi-SSB-ResourceSetList). ng-n1-n2-r19, cri-typeI-SinglePanel-ri-restriction-r19/cri-typeII-ri-restriction-r19 and cri-typeI-SinglePanel-CBSR-r19/cri-typeII-CBSR-r19 in the same way as corresponding legacy fields i.e.: - ng-n1-n2-r19 is defined in the same way as ng-n1-n2 in R15 typeI-multiPanel - cri-typeI-SinglePanel-ri-restriction-r19/cri-typeII-ri-restriction-r19 are defined in the same way as legacy RI restrictions - cri-typeI-SinglePanel-CBSR-r19/cri-typeII-CBSR-r19 are defined in the same way as n1-n2-codebookSubsetRestriction-r18. mrSelectedResources is defined as a SEQUENCE structure containing two fields with integer values from one to eight. delayOffsetCompensation can be located under CSI-AperiodicTriggerState and outside of CSI-AssociatedReportConfigInfo and that the parameter triggeringScheme is not needed. Define numberofSubbandsPO as a list (with size up to the number of subbands) where each element is an integer value within the maximum size of a BWP. The following is used the signaling of typeI-CBSR and typeII-CBSR and typeI-softScalingRank: - (N1, N2) can be signaled as a separate parameter, and CBSR can be signaled as a CHOICE of (X1, X2) and a CHOICE of N1N2;

R2-2501944	Discussion on open issues of RRC spec impact	Xiaomi	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2501986	Enhancements for UE-initiated/event-driven beam management	Ofinno, LLC	discussion

R2-2501994	Discussion on UEI beam reporting impact	LG Electronics Inc.	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2502065	Further discussion on UE-initiated/event-driven beam management	SHARP Corporation	discussion	NR_MIMO_Ph5-Core

R2-2502147	Discussion on UE-initiated Beam Reporting and CSI Enhancement	CATT	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2502168	Discussion on UE-initiated/event-driven beam management	vivo	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2502267	Discussion on UE initiated beam reporting	China Telecommunications Corp.	discussion	NR_MIMO_Ph5

R2-2502314	Discussion on UE-initiated Beam Reporting	Apple	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2502319	RAN2 Aspects of the NR MIMO	Nokia Corporation	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2502374	Discussion on UEIBR	Lenovo	discussion	Rel-19

R2-2502393	Clarification on the uplink grant used for the UE initiated beam report	OPPO	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2502544	Discussion on UE Initiated Beam Report 	Qualcomm Incorporated	discussion

R2-2502547	Impacts from other NR MIMO Phase 5 objectives	Ericsson	discussion

R2-2502666	Discussion on UE-initiated Beam Reporting and CSI enhancement	Samsung	discussion	Rel-19	NR_MIMO_Ph5

R2-2502714	Discussion on UE-initiated/event-driven beam management	CMCC	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2502828	Discussion on MAC impact regarding UEI reporting	ASUSTeK	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2502833	Enhancements for UE-initiated/event-driven beam management	Huawei, HiSilicon	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2502867	Consideration on the UEIBM and Other Issues 	ZTE Corporation	discussion	Rel-19	NR_MIMO_Ph5-Core

## 8.13	NR sidelink multi-hop relay

(NR_SL_relay_multihop; leading WG: RAN2; REL-19; WID: RP-242349[to be updated after RAN#107])

Time budget: 1 TU

Tdoc Limitation: 3 tdocs

### 8.13.1	Organizational

LSs and rapporteur input, including workplan, etc.

Rapporteur input

R2-2502278	Terminology for specifications of L2 multi-hop relay	LG Electronics France	discussion	Rel-19	NR_SL_relay_multihop, NR_SL_relay_multihop-Core

Noted

Proposal 1: For drafting CRs, we slightly prefer Alt 1 in which we can focus on only the case when intermediate relay UEs does not follow a particular behaviour/configuration of the last relay UE.

Proposal 2: Both ‘parent relay UE’ and ‘child relay UE’ can be additionally used in RAN2 specifications, if needed.

Running CRs

R2-2501874	Introduction of Service Continuity for MH Sidelink Relay in 38331	CATT	draftCR	Rel-19	38.331	18.5.0	B	NR_SL_relay_multihop-Core

Revised in R2-2503071

[AT129bis][401][Relay] Rel-19 relay service continuity CR to 38.331 (CATT)

Scope: Collect any comments on the CR in R2-2501874.

Intended outcome: Endorsable CR in R2-2503071

Deadline: Wednesday 2025-04-09 1900 CST

R2-2503071	Introduction of Service Continuity for MH Sidelink Relay in 38331	CATT	draftCR	Rel-19	38.331	18.5.0	B	NR_SL_relay_multihop-Core

Endorsed

[Post129bis][401][Relay] Rel-19 relay service continuity CR to 38.331 (CATT)

Scope: Update the service continuity running CR from the baseline of R2-2503071 to take into account decisions of RAN2#129bis.

Intended outcome: Updated CR and open issue list for RAN2#130

Deadline: Long

R2-2501938	Introduction of multi-hop U2N relay in TS 38.304	MediaTek Inc.	draftCR	Rel-19	38.304	18.4.0	B	NR_SL_relay_multihop, NR_SL_relay_multihop-Core

Revised in R2-2503072

[AT129bis][402][Relay] Rel-19 relay CR to 38.304 (MediaTek)

Scope: Collect any comments on the CR in R2-2501938.

Intended outcome: Endorsable CR in R2-2503072

Deadline: Wednesday 2025-04-09 1900 CST

R2-2503072	Introduction of multi-hop U2N relay in TS 38.304	MediaTek Inc.	draftCR	Rel-19	38.304	18.4.0	B	NR_SL_relay_multihop, NR_SL_relay_multihop-Core

Endorsed

[Post129bis][402][Relay] Rel-19 relay CR to 38.304 (MediaTek)

Scope: Update the 38.304 running CR from the baseline of R2-2503072 to take into account decisions of RAN2#129bis.

Intended outcome: Updated CR for RAN2#130

Deadline: Long

R2-2502469	Introduction of multi-hop U2N relay in TS 38.323	Ericsson	discussion	Rel-19	NR_SL_relay_multihop

Revised in R2-2503073

[AT129bis][403][Relay] Rel-19 relay CR to 38.323 (Ericsson)

Scope: Collect any comments on the CR in R2-2502469.

Intended outcome: Endorsable CR in R2-2503073

Deadline: Wednesday 2025-04-09 1900 CST

R2-2503073	Introduction of multi-hop U2N relay in TS 38.323	Ericsson	discussion	Rel-19	NR_SL_relay_multihop

Endorsed

Discussion:

Ericsson understand some wording changes are needed but can be done in post-meeting discussion.

[Post129bis][403][Relay] Rel-19 relay CR to 38.323 (Ericsson)

Scope: Update the 38.323 running CR from the baseline of R2-2503073 to take into account decisions of RAN2#129bis.

Intended outcome: Updated CR for RAN2#130

Deadline: Long

R2-2502557	Draft Running CR for 38.321	InterDigital	discussion	Rel-19	NR_SL_relay_multihop

Revised in R2-2503074

[AT129bis][404][Relay] Rel-19 relay CR to 38.321 (InterDigital)

Scope: Collect any comments on the CR in R2-2502557.

Intended outcome: Endorsable CR in R2-2503074

Deadline: Wednesday 2025-04-09 1900 CST

R2-2503074	Draft Running CR for 38.321	InterDigital	discussion	Rel-19	NR_SL_relay_multihop

Endorsed

[Post129bis][404][Relay] Rel-19 relay CR to 38.321 (InterDigital)

Scope: Update the 38.323 running CR from the baseline of R2-2503074 to take into account decisions of RAN2#129bis.

Intended outcome: Updated CR for RAN2#130

Deadline: Long

R2-2502608	Introduction of NR sidelink multi-hop relay in TS 38.331	Huawei, HiSilicon	draftCR	Rel-19	38.331	18.5.0	F	NR_SL_relay_multihop-Core

Revised in R2-2503075

[AT129bis][406][Relay] Rel-19 relay main CR to 38.331 (Huawei)

Scope: Collect any comments on the CR in R2-2502608.

Intended outcome: Endorsable CR in R2-2503075

Deadline: Wednesday 2025-04-09 1900 CST

R2-2503075	Introduction of NR sidelink multi-hop relay in TS 38.331	Huawei, HiSilicon	draftCR	Rel-19	38.331	18.5.0	F	NR_SL_relay_multihop-Core

Endorsed

[Post129bis][405][Relay] Rel-19 relay main CR to 38.331 (Huawei)

Scope: Update the 38.331 running CR from the baseline of R2-2503075 to take into account decisions of RAN2#129bis.

Intended outcome: Updated CR and open issue list for RAN2#130

Deadline: Long

R2-2502695	Multi-hop U2N Relay in TS 38.300	LG Electronics Inc.	draftCR	Rel-19	38.300	18.5.0	B	NR_SL_relay_multihop

Revised in R2-2503076

[AT129bis][407][Relay] Rel-19 relay CR to 38.300 (LG)

Scope: Collect any comments on the CR in R2-2502695.  Address the terminology question from R2-2502278 (to affect all specifications) and whether to describe “intermediate relay UE acting as a remote UE” (to affect at least 38.331).

Intended outcome: Endorsable CR in R2-2503076 and report on terminology in R2-2503086

Deadline: Wednesday 2025-04-09 1900 CST

R2-2503076	Multi-hop U2N Relay in TS 38.300	LG Electronics Inc.	draftCR	Rel-19	38.300	18.5.0	B	NR_SL_relay_multihop

Revised in R2-2503088

[Post129bis][406][Relay] Rel-19 relay CR to 38.300 (LG)

Scope: Update the 38.300 running CR from the baseline of R2-2503088 to take into account decisions of RAN2#129bis.

Intended outcome: Updated CR for RAN2#130

Deadline: Long

R2-2503086	[AT129bis][407][Relay] Rel-19 relay CR to 38.300 (LG)	LG Electronics Inc.	discussion	Rel-19	NR_SL_relay_multihop, NR_SL_relay_multihop-Core

Proposal 1: We can use “U2N Relay UE” instead of “U2N last Relay UE” for spec implementation.

Proposal 2: We can discuss whether “U2N intermediate Relay UE” can be used instead of “U2N first Relay UE”.

	If we use “U2N intermediate Relay UE” instead of “U2N first Relay UE”, we can use parent “U2N Relay UE” or serving “U2N Relay UE” to refer to the "U2N first Relay UE".

Proposal 3: We can discuss whether “U2N Relay UE” can be used commonly instead of “U2N last Relay UE”, “U2N intermediate Relay UE” or “U2N first Relay UE”.

	If we have to describe some specific functionality of the "U2N last Relay UE”, "U2N intermediate Relay UE" or "U2N first Relay UE", we can use the last/intermediate/first “U2N Relay UE”.

Proposal 4: It is not needed to differentiate between when an intermediate U2N Relay UE has its data to transmit or receive, and when an intermediate U2N Relay UE acts as only Remote UE without its own data.

Proposal 5: We can discuss whether to differentiate between the U2N Remote UE in single-hop and U2N Remote UE in multi-hop.

Discussion:

LG indicate that the running CRs often address the existing relay UE behaviour and add a mew type of relay/remote UE.  They suggest reusing the existing terminology, i.e., “U2N relay/remote UE” includes the multihop case, and we can distinguish explicitly when needed.  They understand that the first/intermediate/last relay UEs often share legacy behaviour and we should not need to identify them explicitly every time.

Nokia want to understand if this would change the definition of relay UE in the definitions section; they understand that the proposal is not to differentiate in the spec text except where necessary, and they think this would be a good idea.

InterDigital think LG’s understanding makes sense and we should try to use “U2N relay UE” as a common term, but they think there will be cases where we need to capture a specific type.

AT&T want to make sure we do not lose the definitions we already have.

Huawei wonder how this would look in service continuity cases.  LG think we can differentiate when it is needed, e.g., a requirement specific to the last relay UE.

LG think “parent” and “child” can also be used.

Samsung think we should keep the existing roles and decide where we can use common terminology when we have a clear understanding of the functionality of each node.  Ericsson have the same concern and think we should always identify first/intermediate/last relay UE cases for now.

Apple think we should have common inclusive term for the different types of relay UEs, and it would simplify the specs if we expand the definition of “relay UE”.

LG think the definition needs to be updated.

MediaTek think the single-hop definition can be treated as incorporating the first and last relay UEs.

Huawei think we should be careful because the legacy relay UE has a Uu interface, and the intermediate relay UE has PC5 on both sides.

Agreements:

The term “U2N relay UE” can include first/intermediate/last relay UEs in multihop, if not otherwise qualified.  We can distinguish explicitly when a requirement applies only to single-hop or only to certain multihop roles.

The term “U2N remote UE” can include multihop remote UEs, if not otherwise qualified.  We can distinguish explicitly when a requirement applies only to single-hop or only to multihop.

Expand the definitions of “U2N relay UE” and “U2N remote UE” in the CR definition sections to include multihop.  This drafting can be done initially in the 38.300 running CR and migrated later into the other CRs.

The existing multihop definitions, e.g., first/intermediate/last relay UE, are kept.  FFS if they need debugging (business as usual).  The intention is that the first relay UE is an intermediate relay UE, as originally agreed.

The terms “parent” and “child” UE can be used in CR drafting.  FFS if they need to be defined, based on how we end up using them.

[Post129bis][411][Relay] Multihop terminology in 38.300 (LG)

Scope: Implement the terminology agreements of RAN2#129bis in 38.300 to allow running CR rapporteurs to align with the usage.

Intended outcome: Endorsed CR (based on R2-2503076 with deltas for the terminology) in R2-2503088

Deadline: Short

R2-2503088	Multi-hop U2N Relay in TS 38.300	LG Electronics (Rapporteur)	draftCR	Rel-19	38.300	18.5.0	B	NR_SL_relay_multihop

=> Endorsed in R2-2503088

R2-2502980	Introduction of NR sidelink multi-hop relay in TS 38.351	OPPO	draftCR	Rel-19	38.351	18.3.0	B	NR_SL_relay_multihop

Revised in R2-2503077

[AT129bis][408][Relay] Rel-19 relay CR to 38.351 (OPPO)

Scope: Collect any comments on the CR in R2-2502980.

Intended outcome: Endorsable CR in R2-2503077

Deadline: Wednesday 2025-04-09 1900 CST

R2-2503077	Introduction of NR sidelink multi-hop relay in TS 38.351	OPPO	draftCR	Rel-19	38.351	18.3.0	B	NR_SL_relay_multihop

Endorsed

[Post129bis][407][Relay] Rel-19 relay CR to 38.351 (OPPO)

Scope: Update the 38.351 running CR from the baseline of R2-2503077 to take into account decisions of RAN2#129bis.

Intended outcome: Updated CR for RAN2#130

Deadline: Long

### 8.13.2	Relay discovery and (re)selection

Enhancements to relay dscovery and (re)selection to support one additional hop relay (remote UE  first relay UE  last relay UE  gNB). Extensibility to a second additional hop in this WI is considered as a design criterion.

Discovery protocol stacks

P4

R2-2502777	Considerations on relay discovery and (re)selection	Samsung	discussion	Rel-19	NR_SL_relay_multihop-Core

Noted

Proposal 4: RAN2 is kindly asked to agree the protocol stack of discovery should be drawn between  the remote UE and the first relay UE, between two intermediate relay UEs, and between intermediate relay UE and the last relay UE.

Discussion:

CATT think since discovery is per-hop, the current figure with remote and relay is fine.  Samsung understand that the figure should address the relay-to-relay case, but we do not need to redraw.

ZTE agree with the general principle, but would like to note that L3 is not in the WI scope and the figure should not give the impression that it relates to L3 multihop too.  Huawei think we will capture L3 in the stage 2 spec in any case.  Chair understands this could be done separately if SA2 send us an LS.

Agreement:

The discovery protocol stack figure in 38.300 (currently between remote and relay) should include also the relay-to-relay case for L2 multihop.

FFS on discovery model B

P3

R2-2501875	Discussion on Multi-hop Discovery and (Re)selection	CATT	discussion	Rel-19	NR_SL_relay_multihop-Core

Noted

Proposal 3: The following discovery model B forwarding condition for Rel-19 multi-hop U2N relay can be baseline for further discussion:

- For discovery model B, the intermediate Relay UE forwards the solicitation message only if the PC5 RSRP between the Remote UE (or intermediate Relay UE) and the intermediate Relay UE is above a threshold.

- For discovery model B, the last Relay UE needs to check the PC5 AS condition before sending discovery response message to the (first)intermediate Relay UE.

- For discovery model B, no AS criterion is needed for the intermediate Relay UE(s) to forward the response message to the Remote UE.

- For discovery model B, upon discovery response messages reception, the Remote UE considers an intermediate Relay UE(s) as a candidate first relay UE(s) along the path to the last Relay UE if the SD-RSRP towards the first intermediate Relay UE is above a configured threshold.

Discussion:

OPPO think on the last bullet, the remote UE would need to check the PC5 link quality towards the intermediate relay UE, and at the time of discovery response message reception should already have been checked under the conditions of the first bullet; so they think the first bullet is not needed.  On the second and third bullets, they note that we agreed for model A and for model B without PC5 connection, the link quality is not needed, and they would prefer a unified solution where the threshold is checked by the intermediate relay.  In summary they agree only with the fourth bullet.

Qualcomm agree with all four bullets.  LG agree and think the first bullet checks the intermediate relay UE while the fourth bullet checks for the remote UE.  OPPO understand that the two bullets affect the same PC5 link even though they come from different UEs, and there is no filtering at the relay side; it would allow relay selection by another relay, and they think the decision should be left to the remote UE.

Qualcomm think we are only talking about the discovery solicitation/response forwarding, and the actual choice of relay for selection is still up to the upper layer in the remote UE.

OPPO understand that each link will be evaluated by the child relay UE when it selects its parent node, so there should be no situation where discovery is forwarded for a path with bad link quality.  They see the first bullet as reusing the UE-to-UE design instead of UE-to-network.

vivo are not fully persuaded, because the parent selection happens in model A, and in model B the final selection of the whole path is at the remote.  OPPO understand that from AS layer perspective we did not differentiate models A/B for (re)selection; they see that each UE will need to reach the network, and before acting as an intermediate relay UE the UE should always be acting as a remote UE, so they see no case where the intermediate relay UE has not already selected a parent based on link quality.

CATT indicate that we are looking at the situation where the PC5-S connection has not been established, for model B.  OPPO understand that the candidate intermediate relay UE will do discovery and selection as a remote UE first.

Qualcomm have a different understanding and think that before the relay path is determined, the intermediate relay UE does not have to act as a remote UE while it is only a candidate relay UE.

Kyocera think for model B, the relay UEs need not be PC5 connected before being selected and the path to the gNB should be already available to the candidate relay UE.

OPPO see different understandings on the behaviour of the intermediate relay UE; they consider that it is a normal UE with a USIM, which means it needs to camp on the network directly or indirectly.

Qualcomm understand the intention of discovery is to discover candidate relay UEs, and if a UE cannot camp on the network directly or indirectly, it should not forward discovery and is not a candidate.  They see the quality of the PC5 link as representing whether this camping is possible for the candidate relay UE.

CATT think the access to the network is not needed for discovery, only for RRC connection.  LG have the same understanding and think paging and SIB reception by the relay can come after discovery.

Huawei think the intermediate relay UE needs to get SIB12 to transmit discovery message, which means that the PC5 connection is established through the parent and to the network.

Kyocera thought the intermediate relay UE only needed to receive the discovery message from the last relay UE, and the last relay UE will check SIB12 itself.  Qualcomm think the intermediate relay UE could get SIB12 directly if in coverage, and if out of coverage it can forward based on preconfiguration.  OPPO understand that if it has a connected parent relay UE, it needs SIB12 from the parent’s serving cell.

LG wonder why the solicitation message would be forwarded if the candidate already has an upstream PC5 link.

Kyocera note that in Rel-17 operation, there is only one relay UE and it is in coverage; here there is no such guarantee.

Agreements:

If the candidate intermediate relay UE is out of coverage without a PC5 connection to a parent relay UE, it can forward discovery messages based on preconfiguration.  If it is in (direct) coverage, or out of coverage but PC5 connected to a parent relay UE (and thus indirectly to the network), it needs SIB12 or dedicated configuration to forward discovery messages.

For discovery model B when there is no established PC5 link between the remote UE and the intermediate relay UE, the intermediate Relay UE forwards the solicitation message only if the PC5 RSRP between the Remote UE (or intermediate Relay UE) and the intermediate Relay UE is above a threshold.

For discovery model B when there is no established PC5 link between the last relay UE and the intermediate relay UE, the last Relay UE needs to check the PC5 AS condition before sending discovery response message to the intermediate Relay UE.  FFS if this case can occur or if the intermediate relay UE always establishes itself as a remote UE first.

For discovery model B, the quality of the PC5 links is assumed already to have been checked when the solicitation messages were forwarded, and no AS criterion is needed for the intermediate Relay UE(s) to forward the response message towards the Remote UE.

For discovery model B, upon discovery response messages reception, the Remote UE considers an intermediate Relay UE(s) as a candidate first relay UE(s) along the path to the last Relay UE if the SD-RSRP towards the first intermediate Relay UE is above a configured threshold.

Forwarding by non-connected intermediate relay

P1

R2-2502829	One remaining issue on multi-hop U2N Relay Discovery message forwarding for model B	ASUSTeK	discussion	Rel-19	NR_SL_relay_multihop

Noted

Proposal 1:  A UE which is authorized to act as an Intermediate U2N Relay UE but cannot connect to the network directly or indirectly shall not forward Relay Discovery Solicitation messages received from other remote UEs.

Discussion:

LG wonder how the intermediate relay UE can know if it can reach the network; they assume it should always forward the solicitation message towards a parent relay UE.

Apple think the negative formulation of the proposal is not so useful.

NEC agree with LG and are not sure about the scenario.

Huawei see no spec impact and think it can be left to the UE to understand whether it can connect.

OPPO think the UE should act as a remote UE before acting as an intermediate relay UE.

FFS points on (re)selection

P3-P6

R2-2502609	Relay discovery and (re)selection for multi-hop Relay	Huawei, HiSilicon	discussion	Rel-19	NR_SL_relay_multihop-Core

Proposal 3: Link quality degradation of the upstream links will lead to path switching (for the CONNECTED state) or relay (re)selection or cell selection (for the IDLE/INACTIVE state), rather than triggering an immediate notification. The relay UE will send a notification once path switching or relay (re)selection or cell selection occurs.

Proposal 4: The notification message received from the U2N Last Relay UE indicating a failure on the Uu interface should be forwarded downstream by the Intermediate/First Relay UE to the Remote UE. To achieve this, we can reuse the PC5 RRC Notification message.

Proposal 5: Failure Scenarios in which the remote UE is not notified and recovery is handled by the intermediate relay UE not considered to be supported in R19.

Proposal 6. if there are multiple relay UEs in the multi-hop chain, there is a possibility of encountering a multi-hop failure on the PC5 links. In this case, it would be more beneficial for the child relay UE to forward the notification message received from the parent relay UE until the Notification Message is delivered to the Remote UE.

[AT129bis][414][Relay] FFS issues on (re)selection (Huawei)

Scope: Discuss the remaining FFS issues on (re)selection for Rel-19 relay, starting from the proposals in R2-2502609 and covering other issues as time permits.

Schedule: Tuesday 2025-04-08 0930-1030 in Brk2

Intended outcome: Report to Thursday CB session in R2-2503085

Deadline: Wednesday 2025-04-09 1900 CST

R2-2503085	FFS issues on (re)selection	Huawei, HiSilicon	discussion	Rel-19	NR_SL_relay_multihop-Core

Noted

Link quality degradation

Proposal 1: Notification for Link quality degradation is not supported. Link quality degradation of the upstream links will eventually lead to path switching (for the CONNECTED state) or relay (re)selection or cell selection (for the IDLE/INACTIVE state). The relay UE will send a notification once path switching or relay (re)selection or cell selection occurs. FFS on sending the notification if the path switching happens in the same cell.

Notification from the upstream Relay UE

Proposal 2: The notification message received from the U2N Last Relay UE indicating a failure on the Uu interface should be forwarded downstream by the Intermediate/First Relay UE to the Remote UE. To achieve this, we can reuse the PC5 RRC Notification message as a baseline.

Proposal 3. RAN 2 to further discuss and agree if the Notification Message is forwarded/ sent down stream, all the way to the Remote UE,  in case PC5 Link failures on the multi hop chain are encountered. FFS to check if some higher layer mechanism for forwarding already exists in SA2 specs

Group Mobility Support

Proposal 4: RAN 2 to discuss if group mobility/handling is to be supported in R19 where recovery is handled by the intermediate relay UE without remote UE being notified of any failures.

Content in the Notification Message

Proposal 5. Upon receiving the notification message form the parent relay UE, the intermediate relay UE/first relay UE may update the indication in the notification message to the remote UE to indicate the action it has taken. FFS if we need any new cause value

Applicability to L3

Proposal 6: RAN2 will focus solely on the L2 Multi-hop Relay design. Therefore, we do not need to discuss the applicability of L3 Multi-hop relays.

Discussion:

Huawei indicate that P1/P2/P5/P6 may have consensus and P3/P4 need discussion.

LG think on P1, “path switching” should say “handover” to differentiate from the FFS part.

CATT wonder what “quality degradation” means in P1; is RLF a kind of quality degradation?  Huawei understand that it means the degradation before RLF/HO.

Qualcomm think P1-P4 overlap with previous online discussion.  Huawei think this is true for P3/P4, but P1/P2 are different.

Kyocera understand that we should not combine the reselection and path-switch cases in P1.

ZTE think on P5, the motivation for the intermediate relay UE to update the notification is not clear, and the relay can just forward the notification unchanged towards the remote UE.  They understand that the remote UE will do the same thing with or without the update.

vivo understand that P5 would provide different cause values for downstream UEs, and they think this is too complex to introduce here.

Qualcomm think we could postpone this discussion until we are clear what values would be used, then determine if there is a case for the update.

Samsung wonder if the remote UE needs to distinguish whether it could select the same first relay UE to perform recovery; they see that the intention is to indicate where on the path the problem occurred.

OPPO are OK with postponing the P5 issue until next meeting, and they think the issue is whether the relay processes the notification before forwarding it.

Lenovo wonder on P2/P5 if it excludes the relay UE using release instead of notification; they understand that this should be allowed.  Huawei agree that both possibilities are there and the intention is not to restrict.

Huawei think there is a problem if we send the notification transparently, because it could cause double notifications, e.g., upstream RLF followed by relay reselection.

LG agree that the relay can send the notification or release the link, and on the same principle, they think the relay could update the notification or not, according to its implementation.  They understand that the remote UE will take the same action anyway (relay reselection or re-establishment).

Kyocera share Samsung’s concern; in multihop, they think we should look at the notification causes and potentially redesign how we handle failures.

Qualcomm understand that the intermediate relay UE is also acting as a remote UE and should just perform relay reselection.  They think this is a repeated discussion from yesterday.  Huawei clarify that this proposal is intended for the PC5 notification.

Lenovo think yesterday's discussion was about idle/inactive, and here we are agnostic to the state.  They wonder if “forwarding” means transparent or regenerating a new notification message.  Huawei indicate that it was intended to mean regenerating an identical notification message.

Samsung understand that if the failure happened at the last relay UE, it may be better to forward the notification rather than make every intermediate relay UE perform reselection and delay the recovery of the remote UE, maybe with an update to the notification message if an additional failure occurred in between.

Chair wonders what would prevent double notification.

OPPO understand that the intention is to have forwarding, either transparent (no reselection by the intermediate relay UE) or regenerative (reselection by the intermediate relay UE and its own notification), and we will pick an option rather than support both.  This should prevent double notification.

LG think the double trigger can be handled in UE implementation; the relay UE should not send the second notification.

Qualcomm think on P6, some of our work like discovery conditions is also applicable for L3.

ZTE note that SIB12 has separate indications for L2 and L3 discovery, and they wonder if this means that we will have only a common multihop discovery indication rather than distinguishing which layer.

Agreements:

Notification for Link quality degradation, as distinct from existing notification causes, is not supported (no spec impact expected).

When the intermediate relay UE receives a notification message from the last relay UE indicating a failure on Uu, the intermediate relay UE may transmit a notification message downstream (towards the remote UE).  FFS if the notification message is a forwarded copy of the original notification (same cause) or a regenerated message from the intermediate relay (cause might reflect its own reaction such as relay reselection or re-establishment).  This does not change the agreement that it is up to relay implementation whether to release the downstream link, and if the link is released there is no downstream notification message.

RAN2 will focus solely on the L2 Multi-hop Relay design within the scope of this work item.  This does not exclude that some work (e.g., discovery conditions, transmission resources) could be reused in L3 without specific impact under our WI.

Notification exceptional cases and details

P1-P3

R2-2502624	Relay (re)selection under multihop relay 	Kyocera	discussion

Noted

Proposal 1	While performing Model A discovery procedure, if the IDLE/INACTIVE intermediate relay UE is triggered to perform relay reselection, it does not need to notify its PC5 connected remote UE that is still in IDLE/INACTIVE upon the initiation of relay reselection.

Proposal 2	After successful relay reselection, the IDLE/INACTIVE intermediate relay UE does not need to notify the PC5 connected remote UE that is also still in IDLE/INACTIVE.

Proposal 3	RAN2 should consider whether it’s necessary for the intermediate relay UE to notify the PC5 connected remote UE in case the intermediate relay UE’s relay reselection is unsuccessful.

Discussion:

LG think on proposal 1, if the selected relay UE is on a different cell, a notification should be generated.

NEC think P3 is confusing because there is no clear definition of “unsuccessful” reselection.

Huawei think we need to understand all cases and avoid group mobility.  They think on P3, the unsuccessful reselection will not happen as indicated by NEC.

Qualcomm understand that model A discovery relies on the parent relay UE to prepare the upstream path, and when the upstream path is changed, the discovery messages should be updated and the change can be treated as a notification.  They think that for the unsuccessful case, if the intermediate relay UE cannot prepare the path, it should update discovery.

InterDigital agree with Qualcomm and think we should be careful about overusing the notification when we already have discovery signalling and upper layer information.

Kyocera think it is not a group mobility case, since we settled on approach 1 and this is about relay UEs in idle/inactive.

OPPO have the same understanding as Huawei and see this case as a group path switch.  They think the scenario should not be supported.

InterDigital consider that “group mobility” originally referred to handover in RRC_CONNECTED, and idle/inactive is a bit different; we excluded it in the single-hop case in Rel-18, but it may be more useful to allow multiple remote and intermediate UEs to move along with a parent relay if it does reselection.  LG agree with InterDigital and think this is not a group handover case.

Samsung think there are two issues, whether we need the notification message and whether we need some information (potentially outside the AS layer) to the remote UE to trigger relay reselection.  They consider that the notification message is not necessary and the remote UE can act based on the discovery messages.  They do not see this as a group mobility case.

ZTE understand that mobility happens also in idle/inactive, e.g., in relation to TAU.  Huawei think there is some group handling, whether or not we call it “mobility”, and they think the downstream UE could always reconnect to the same parent.

Lenovo wonder about the unsuccessful case from the remote point of view; it may mean that the parent UE has no path to the network.  They wonder what information can indicate this situation to the remote UE.  CATT think this is out of RAN2 scope.

Kyocera think if the relay reselection is not successful and the UE loses a path, it will not send discovery signalling, so they see that some indication to the remote UE might be needed.  They note that in the path switch case, we do not have relay reselection because it is based on gNB direction.

OPPO think we have agreement that some information should be given to the child UE, and the question is whether notification or upper layer signalling is used.

CATT think a similar procedure has been captured in SA2 specs already.  OPPO understand this is for L3 and for model A case.  Huawei agree it is specific to L3.

InterDigital think the information already in discovery may address it.

Agreements:

When the parent UE in idle/inactive performs reselection, RAN2 assume that an indication of the reselection is needed for the child UE.

FFS detailed information in the indication and child UE handling.

FFS if there are exceptional cases where the indication can be suppressed, e.g., reselection under the same serving cell without changing the hop count.

FFS if the notification message is used or we rely on upper layer signalling (e.g., discovery).

P7-P8

R2-2502693	Discussion on the discovery and relay (re)selection for multi-hop U2N relay	LG Electronics Inc.	discussion	Rel-19

Proposal 7: If an intermediate Relay UE detects PC5 RLF in the UL direction, the intermediate Relay UE generates a notification message with the cause value as PC5 RLF and sends it to the child intermediate Relay UE or Remote UE.

Proposal 8: When intermediate Relay UE receives a notification message, it is an implementation of the intermediate Relay UE whether to forward the notification message to the child Relay UE (/Remote UE) or to release the link.

Other contributions

R2-2501799	Discovery and relay (re)selection for multi-hop U2N relay	OPPO	discussion	Rel-19	NR_SL_relay_multihop-Core

R2-2501854	Discussion on Relay discovery and (re)selection	ZTE Corporation, Sanechips	discussion	Rel-19	NR_SL_relay_multihop

R2-2502010	Discussion on multi-hop U2N relay discovery and relay selection	NEC	discussion	Rel-19	NR_SL_relay_multihop-Core

R2-2502188	Relay discovery and selection for Multi-hop UE-to-NW Relay	Apple	discussion	Rel-19	NR_SL_relay_multihop

R2-2502200	Discovery and relay reselection for multihop relay	Nokia	discussion	NR_SL_relay_multihop

R2-2502241	Multi-hop relay discovery and reselection	China Telecom	discussion	Rel-19	NR_SL_relay_multihop-Core

R2-2502361	Relay (re)selection in Multi-hop relay	Lenovo	discussion	Rel-19

R2-2502378	discussion on Relay discovery and (re)selection for multi-hop relay	Sharp	discussion	Rel-19	NR_SL_relay_multihop-Core

R2-2502418	Discussion on relay UE (re)selection for intermediate relay UE	vivo	discussion	Rel-19

R2-2502454	Discovery and Relay (re)selection for multi-hop U2N relay	Qualcomm Incorporated	discussion	NR_SL_relay_multihop-Core

R2-2502468	Discussion on relay discovery and relay (re)selection	Ericsson	discussion	Rel-19	NR_SL_relay_multihop

R2-2502497	Multi-hop relay selection/re-selection	Sony	discussion	Rel-19	NR_SL_relay_multihop-Core

R2-2502558	Discovery and Relay (Re)Selection for Multi-hop U2N Relays	InterDigital	discussion	Rel-19	NR_SL_relay_multihop

### 8.13.3	Control Plane Procedures and SRAP impact

Contributions should focus on control plane procedures and can include SRAP impact and QoS handling to support additional hops.

Including outcome of email discussion [Post129][402][Relay] Control plane approach 2 impact (Apple/Ericsson)

Email discussion report

R2-2502191	Report of [POST129][402][Relay] Control plane approach 2 impact (Apple/Ericsson)	Apple, Ericsson	discussion	Rel-19	NR_SL_relay_multihop

Noted

Proposal 1 [To discuss]: To support Remote UE’s RRC message forwarding, at least the following scope of spec impact is identified:

-	New SRAP header format to include remote UE L2 ID

-	Remote/Relay UE’s UL (forwarding) transmission with new SRAP header to parent node; Relay UE stores the linkage of “L2 ID - ingress PC5 link”

-	gNB and relay UE’s DL (forwarding) transmission with new SRAP header conveying a L2 ID-local ID linkage; intermediate relay UE update the linkage of “local ID, L2 ID, egress PC5 link” for DL forwarding operation.

-	SL-RLC0 is reused as PC5 Relay RLC channel for intermediate relay forwarding SRB0 message

FFS whether the following is needed and FFS further spec impact:

-	SL-RLC1 is to be used for forwarding SRB1

-	SUI procedure to report remote UE’s L2 ID to solicit local ID assignment

-	Dependency on the RRC state of remote UE and SRAP header format

Proposal 2 [To discuss]: To support QoS Split in Approach 2, at least the following scope of spec impact is identified:

-	Remote UE report hop number in SUI to the gNB of the last relay UE

-	gNB of the last relay UE provide per-RB QoS split results to via a hop-by-hop signaling. FFS whether SRAP-based (i.e., SRAP control PDU) or RRC-based solution (Uu and PC5 RRC message) is to be used.

FFS whether the following is supported and FFS further spec impact:

- whether intermediate relay would be allowed to connected to a gNB different from the gNB of the last relay UE and if yes, which and how gNB(s) to conduct QoS split

Proposal 3 [To discuss]: To support PC5 Relay RLC channel configuration in Approach 2, at least the following scope of spec impact is identified:

-	Tx UE deriving configuration based on SIB and Pre-configuration as similar to L2 U2U relay. FFS which cell provides the SIB.

FFS whether the following is supported and FFS further spec impact:

- whether/how intermediate relay reconcile the derived configuration and gNB-provided configuration when intermediate relay UE enters CONNECTED.

Proposal 4 [To discuss]: Check with SA WGs whether IDLE or INACTIVE intermediate relay UE is authorized. If yes, the AS layer spec impact of authorization-related changes is expected to be very limited. FFS upper layer spec impact.

Discussion:

ZTE want to clarify that the objective is just to understand the scope of impact, and we may not need to agree any proposals.  They also note that there is an additional contribution on the fast/parallel proposal.

Approach 1/approach 2 downselection

P1/P6

R2-2502692	Discussion on the control plane procedure for multi-hop U2N relay	LG Electronics Inc.	discussion	Rel-19

Noted

Proposal 1: To support multi-hop relay operation, only one approach (e.g., approach 1 or approach 2) should be selected for description on the spec. Otherwise, we have to consider the integration of both approaches.

Proposal 6: Considering the time budget to complete the Rel-19 multi-hop relay, i.e., we have only two more meetings for completing the multi-hop relay operation, we think it’s better to focus on the remaining issues of approach 1.

Discussion:

ZTE are OK with the proposals provided the latency enhancements for approach 1 do not become a blocking issue and source of extra FFS points.

Lenovo think the fast forwarding has high priority, and if we have it approach 2 is not needed.

Apple do not agree that approaches 1 and 2 are infeasible to integrate in principle, but they think we do not have enough time.

Huawei think there are problems with approach 2 and consider it as supplementary to approach 1.

CATT want to be clear that downselecting from approach 2 is not linked to whether we ultimately agree to the fast forwarding.

Agreement:

Control plane design considers only approach 1 (along with the fast forwarding enhancement for SRB0 if agreed), and approach 2 is not further pursued in Rel-19.

Proposals specific to approach 2

P1-P8 (if necessary)

R2-2502467	Discussion on control plane procedures	Ericsson	discussion	Rel-19	NR_SL_relay_multihop

Proposal 1	Allow an intermediate relay UE to stay in RRC_INACTIVE when the remote UE triggers RRC connection establishment.

Proposal 2	RAN2 to send an LS to SA2 and SA3 to query whether there is any authorization concern to support an intermediate relay UE to stay in RRC_IDLE when the remote UE triggers RRC connection establishment.

Proposal 3	For Approach 2, if the (pre)configuration allows, each intermediate relay UE decides by itself to operate as an intermediate relay UE without moving to RRC_CONNECTED

Proposal 4	For Approach 2, the network (gNB) determines QoS split of the links between an intermediate relay in RRC_INACTIVE and its neighbor UE (remote or another relay UE) in the path.

Proposal 5	For Approach 2, the last relay UE reports the number of hops for the associated relay path(s) to the gNB for QoS split purpose via the SUI message.

Proposal 6	For Approach 2, adopt PC5 RRC or new control PDUs in the SRAP layer to forward path related information between UEs on the path, e.g., the local ID of the remote UE, number of hops on the relay path and QoS split.

Proposal 7	In Approach 2, each intermediate relay UE in RRC_INACTIVE uses specific PC5 Relay RLC channels to forward SRB0 messages received from the remote UE.

Proposal 8	Same as in legacy for U2U relay, each intermediate relay UE in RRC_INACTIVE derives its PC5 RLC channel configuration for relaying SRB1, SRB2 and DRB(s) based on SIB or pre-configurations in Approach 2.

Fast/parallel SRB0 forwarding

P1

R2-2502189	Fast & Parallel RRC Establishment/Configuration for Multi-hop U2N relaying	Apple, FirstNet, Ericsson, AT&T, Kyocera	discussion	Rel-19	NR_SL_relay_multihop

Noted

Proposal 1	To reduce latency in CP procedures for multi-hop U2N relay in the Baseline Approach, support forwarding of Uu RRC messages via SL-RLC0 and/or SL-RLC1 by intermediate relay UE(s) not yet in RRC_CONNECTED.

[AT129bis][409][Relay] Fast/parallel RRC forwarding for approach 1 (Apple)

Scope: Discuss the forwarding procedure proposed in R2-2502189:

- Understand the expected spec impact and details of the proposal

- See if there is consensus to include this proposal in approach 1

Intended outcome: Report to Tuesday evening relay session in R2-2503078

Schedule: Tuesday 2025-04-08 0830-0930 CST in Brk2

Deadline: Report available by Tuesday 2025-04-08 1600 CST

R2-2503078	Fast/Parallel RRC Setup for MH relay	Apple	discussion	Rel-19	NR_SL_relay_multihop

Noted

Proposal: Consider parallel forwarding of  RRC messages with the design constraints and scope limits as follows:

-   only for Uu SRB0 messages

-   keep 1st hop (remote-1st relay) operation on SL-RLC0 unchanged (same as legacy)

Avoid RAN3 impact if possible

At least for UL. FFS for DL

FFS whether/how gNB understand path information

FFS optional/mandatory of this feature for approach 1

Checking TP (331/351,etc.) in RAN2#130 to decide whether to support this scheme based on 1)spec impact; 2) whether main technical issue unsolved.

Other solutions for latency reduction are still based on company contributions.

Discussion:

Qualcomm think for checking the TPs in next meeting, we should also evaluate the level of benefit; they think it is unclear that addressing all of the FFSs will pay us back for the effort.

Samsung think companies are fine with this direction for latency reduction, and the goal of a TP would be to show the level of spec impact to support it.  They think first we need to agree if we pursue this direction, then decide on options.

NEC agree with the way forward, and on the FFS issue for DL signalling, they think DL support is not needed because the last relay UE is in RRC_CONNECTED state, and the other relay UEs might have time to connect.

LG think the FFS for DL is needed, because the gNB does not know the status of the non-connected relay UEs.  Samsung think the benefit of DL may come for free, and we might as well support it.  Apple have the same understanding as Samsung, and they think it is important because the SRAP configuration for the remote UE is in the setup message.

Huawei have a similar concern to LG; they think the gNB needs to understand the whole path.

OPPO understand LG’s concern and think the benefit of DL is not free; they are fine to keep it as FFS.

LG indicate that if the gNB already knows the path, DL enhancement is not needed and we can focus on UL; for the UL we do not need to worry about RAN3 impact.  They understand that data transmission cannot start until all the relay UEs are in RRC_CONNECTED anyway.

Apple think based on the baseline procedure, the gNB is not aware of the path in advance; they doubt that the overall benefit is there without the DL part.

Apple indicate that there are two alternatives on the table for local ID management, one with a new SRAP header including the L2ID and the other allowing the advance assignment of the local ID, with impact mainly to SRAP and RRC, respectively.  Samsung think we should not downselect between them yet.

ZTE consider that although we allow companies to submit the TPs, they should not block work on the baseline procedure itself.  Apple think that we can decide, e.g., how SRAP works in the connected case, and the idle/inactive part is complementary.

NEC wonder if the UE has the ability to do the fast operation, how it knows if it should use it for SRB0.  Apple think it can be based on the capability of the chain of UEs and it mainly depends on the first relay UE.

Ericsson agree that both approaches are feasible and they could even be supported at the same time, and we should look at the TPs.

OPPO think companies may be thinking about additional approaches and it could be better to encourage joint contributions towards the next meeting rather than have official email discussions.  Qualcomm also think we should be open for other solutions.

LG think if we cannot converge next meeting we should just continue with the baseline approach, and they wonder why it is preferable to have unofficial email discussions rather than official.

Agreements:

Continue to consider forwarding of SRB0 messages by relay UEs not in RRC_CONNECTED with respect to control plane approach 1.

TPs showing the two approaches for fast forwarding of SRB0 (SRAP header and local ID assignment by RRC signalling) are invited for next meeting (co-sourcing strongly encouraged).

Other approaches are not precluded (contribution-driven) but should be shown at a mature stage considering the time left.

Strive to avoid additional RAN3 impact specific to fast forwarding.

FFS if applicable to DL.

FFS what level of gNB awareness of the path information would be needed.

FFS if fast forwarding is optional/mandatory for UEs to support.

Local ID allocation

P5-P6

R2-2502610	Control plane procedures for multi-hop relay	Huawei, HiSilicon	discussion	Rel-19	NR_SL_relay_multihop-Core

Noted

Proposal 5: Reusing the single-hop relay mechanism to support the Local ID allocation for multi-hop relay:

•	Parent relay UE of the remote UE reports the L2 ID to the gNB to request the local ID allocation, the uniqueness of the local ID can be guaranteed by the gNB by implementation.

•	The remote UE local ID is 8 bits.

Proposal 6: RAN2 need to discussion the uniqueness of the remote UE local ID which can impact the data forwarding behaviour of the relay UEs.

Discussion:

ZTE are fine with P5, but they do not see why P6 is necessary after taking P5.  Huawei clarify that a local ID could be unique within the scope of a parent relay UE but not unique across different relay UEs.  ZTE think this may depend on RAN3 discussion; they think we should have uniqueness at least within the scope of the last relay UE, maybe within the cell.

Huawei think if the gNB enforces uniqueness within the cell, there is no problem, but if we want to have more remote UEs than such a regime will support, we might only have uniqueness within a smaller scope.

OPPO wonder if there is specification impact if the network ensures that there is no collision.

Samsung think “parent relay UE of the remote UE” should be “first relay UE”; they understand the first relay UE sends SUI to the gNB and the gNB allocates the local ID.  They do not see the benefit of an agreement on P6 if the gNB implementation already enforces uniqueness.

Agreement:

Reuse the single-hop relay mechanism to support the Local ID allocation for multi-hop relay:

•	First relay UE reports the L2 ID of the remote UE to the gNB to request the local ID allocation, the uniqueness of the local ID within the cell is assumed to be guaranteed by the gNB by implementation.

•	The remote UE local ID is 8 bits.

SRAP configuration

P4-P7

R2-2501801	Control plane procedures of multi-hop U2N relay	OPPO	discussion	Rel-19	NR_SL_relay_multihop-Core

Noted

Proposal 4	As in single-hop U2N Relay mechanism, R2 confirm, for the DL and UL SRB0 of remote UE in multi-hop U2N Relay, SRAP header is present over each hop except the PC5 hop between the remote UE and first relay UE.

Proposal 5	As in single-hop U2N Relay mechanism, R2 confirm, for the DL and UL SRB0 of remote UE in multi-hop U2N Relay:

-	At the link between remote UE and the first relay UE, reuse the specified PC5 RLC channel (i.e., SL-RLC0);

-	At the link between intermediates relay UEs or the link between intermediate relay and the last relay, or the link between the last relay and the network, the RLC channel is configured by the network via dedicated RRC message.

Proposal 6	Remote UE ID and BEARER ID are configured by the network via dedicated RRC signalling as in R17.

Proposal 7	For the SRAP configuration at the relay UE (both last relay and intermediate relay UE), rely on network to provide each child UE’s SRAP configuration based on the L2 ID of the direct connected child UE, which further includes entries for indirect child UE with associated local ID.

Discussion:

Apple understand that P4-P6 are legacy operation, and P6 may be redundant with previous agreements.  They think for P7, there are alternative solutions without the network providing an explicit configuration for every child UE.  They think P6 may be intended to apply to the whole SRAP configuration.  OPPO think it is helpful to have an explicit agreement.

ZTE think on P7, there is another option in reflective bearer mapping, where the gNB does not provide the DL next-hop UE ID to save signalling and the relay UE determines the next hop based on stored information.  They intend this as an option, and if the network does provide the ID the UEs have to follow it.

OPPO point out that P7 does not mention the next-hop UE ID, but they understand that no matter what, the relay UE needs to know all its children’s local IDs, and that information should be provided by the network; the proposal is more about the signalling format.

Qualcomm have the same understanding as OPPO and support P7.  They understand the network will configure a list of local IDs on the downstream path, and the relay UE will refer to this list to determine the egress link.

Samsung wonder about the intention of the last part of P7.  OPPO indicate it is to give the relay the whole list as Qualcomm described.

Apple do not agree with the last part of P7; they think it implies that whenever a new child UE is added, the SRAP configuration will need to be updated, and this could be avoided by defining the egress link on a path basis and using the reflective behaviour described by ZTE/

Samsung think the intention is basically to do packet routing and each relay UE becomes a router based on the local ID.  They agree that if there is only one path to the remote UE, the information may not be needed, but the RRC signalling can support the feature for the general case.

OPPO understand in legacy, the relay UE needs the SRAP configuration for the remote UE, and they want to understand if Apple intend something different here.  They see an unknown local ID in the header as an error case.

Lenovo think we do not need to specify the network behaviour in P7 if we can rely on the reflective bearer mapping, where the relay UE remembers from whom it saw UL traffic and uses that to determine the next hop.  Qualcomm think this works since the uplink is always seen first.  Ericsson agree that explicit mapping of the next hop is not needed.

OPPO indicate that the SRAP configuration includes other things besides the local ID; they want to understand if companies think the whole configuration of the child UEs would be omitted, or if it is still needed, e.g., for BEARER ID.

Samsung think the intention of ZTE’s proposal is whether we need to include the next-hop ID, but the remote UE SRAP configuration is still needed for bearer mapping.

OPPO think this aspect is important for CR drafting and we could take the explicit next-hop list as a baseline.

Agreements:

As in single-hop U2N Relay mechanism, R2 confirm, for the DL and UL SRB0 of remote UE in multi-hop U2N Relay, SRAP header is present over each hop except the PC5 hop between the remote UE and first relay UE.

As in single-hop U2N Relay mechanism, R2 confirm, for the DL and UL SRB0 of remote UE in multi-hop U2N Relay:

-	At the link between remote UE and the first relay UE, reuse the specified PC5 RLC channel (i.e., SL-RLC0);

-	At the link between intermediate relay UEs or the link between intermediate relay and the last relay, or the link between the last relay and the network, the RLC channel is configured by the network via dedicated RRC message.

SRAP configuration including remote UE ID and BEARER ID is configured by the network via dedicated RRC signalling as in R17.

For the SRAP configuration at the relay UE (both last relay and intermediate relay UE), rely on network to provide SRAP configuration for at least the directly connected child UEs, based on the L2 IDs of the direct connected child UEs.

For CR drafting, assume as a baseline that the child UE’s SRAP configuration can include entries for indirect child UE with associated local ID for next-hop determination.  We will review next meeting if this explicit mapping functionality is needed.

FFS whether to introduce reflective bearer mapping as an optional feature.

P1-P3

R2-2502419	Discussion on SRAP impact for baseline procedure	vivo	discussion	Rel-19

Noted

Proposal 1: From a Remote UE perspective, the dedicated Uu radio bearer mapping configuration at least includes the following information:

•	Local identity of the Remote UE

•	The Remote UE’s Uu SRB/DRB identity to Egress PC5 RLC channel mapping

Proposal 2: From an intermediate Relay UE perspective (including the first Relay UE), the dedicated Uu radio bearer mapping configuration at least includes the following information:

•	For relaying at least one Remote UE’s Uu RRC signaling and data

o	Local identity of each Remote UE

o	Each Remote UE’s Uu SRB/DRB identity to Egress PC5 RLC channel mapping

•	For its own Uu RRC singnalling

o	Local identity of this Relay UE

o	This Relay UE’s SRB identity to Egress PC5 RLC channel mapping

•	For relaying its downstream intermediate Relay UE’s Uu RRC signaling, if any

o	Local identity of each downstream intermediate Relay UE

o	Each downstream intermediate Relay UE’s SRB identity to Egress PC5 RLC channel mapping

Proposal 3: From the last Relay UE perspective, the dedicated Uu radio bearer mapping configuration at least includes the following information:

•	For relaying at least one Remote UE’s Uu RRC signaling and data

o	Local identity of each Remote UE

o	Each Remote UE’s Uu SRB/DRB identity to Egress PC5/Uu RLC channel mapping

•	For relaying its downstream intermediate Relay UE’s Uu RRC signaling

o	Local identity of each intermediate Relay UE

o	Each intermediate Relay UE’s SRB identity to Egress PC5/Uu RLC channel mapping.

Discussion:

Apple are fine with P1 but think P2/P3 are tangled with the previous discussion and may not add new information.

OPPO indicate the intention of the previous P7 was to cover this information, but they are not sure if everyone has the same understanding.

Samsung suggest we just agree that the legacy SRAP configuration is applicable for the first/intermediate/last relay UEs.

Agreements:

From a Remote UE perspective, the dedicated Uu radio bearer mapping configuration at least includes the following information:

•	Local identity of the Remote UE

•	The Remote UE’s Uu SRB/DRB identity to Egress PC5 RLC channel mapping

From a relay UE perspective (including first/intermediate/last relay UEs), the legacy contents of the SRAP configuration are applied for at least the directly connected child UEs.  FFS (depending on outcome of the reflective mapping) if they are also applied for indirectly connected child UEs.

Paging

P3-P8

R2-2502455	Open issue for control plane	Qualcomm Incorporated	discussion	NR_SL_relay_multihop-Core

Noted

Proposal 3	Only the last Relay UE monitors paging for the remote UE.

Proposal 4	The intermediate does not monitor paging for the child UE over Uu interface.

Proposal 5	The intermediate Relay UE always forwards paging request to the parent UE if receiving from the child UE.

Proposal 6	The intermediate Relay UE does not release paging information for the child UEs in the parent relay UE when the intermediate relay UE is in RRC_CONNECTED.

Proposal 7	When an intermediate relay UE is in RRC_IDLE/RRC_INACTIVE acting as Remote UE functionality, it can obtain paging for itself directly on Uu by implementation when the intermediate relay UE is in coverage, like today’s Remote UE.

Proposal 8	When an intermediate relay UE moves in or out of coverage, it does not initiate or cancel paging monitoring by parent relay UE.

Discussion:

ZTE understand on P3 and P4 that we also support the case where the last relay UE monitors paging for the remote UE, but we do not need to exclude the intermediate UE also monitoring, so they do not support P4 and want to remove “only” from P3.  InterDigital have the same understanding and think we should let the intermediate relay UE monitor if it is in coverage and can monitor.  Chair thinks we would need a guard against double-paging.

OPPO think the intention is generally to avoid specification impact, e.g., by allowing P8.  They support P3/P4.

Lenovo agree with ZTE and InterDigital; the restriction may result in paging delays.

Qualcomm intend to avoid duplicate paging.  They assume that an intermediate relay UE is normally at cell edge and may move in or out of coverage frequently, resulting in updates to the parent relay UE, hence P8.

ZTE do not want to introduce more optimizations for the intermediate relay UE monitoring paging, but they want to allow it in cases where it happens to be possible; generally we specify what the UE should do, not what it should not do.

InterDigital understand that in legacy we have cases where the remote UE disables paging/SI monitoring by the relay, and this is similar; they see it as somewhat legacy behaviour and think the intermediate relay UE can turn paging monitoring on/off acting as a remote UE.  Qualcomm think we do not have the ability to notify the parent today when coverage changes.  InterDigital think it depends on what we specify; if the intermediate relay UE monitors paging, it does not need the parent to monitor for it, and it is not directly tied to coverage.

Huawei agree with InterDigital and ZTE; they think there is no problem allowing this.

Ericsson think it is not fully legacy behaviour, but they are fine with the impact.

LG support Qualcomm’s proposals as they are; they think the last relay UE should monitor paging, and they are concerned about the complexity of managing who monitors if we allow monitoring by downstream UEs.  InterDigital see the complexity in the opposite case, where we need to specify a restriction on an intermediate relay UE in coverage that can monitor paging for itself but would now be excluded from monitoring for its children.

OPPO understand that both sides are trying to avoid specification impact, and they wonder if we can agree that the child UE relies on the parent for paging monitoring and we will not take additional effort to either support or restrict the UE behaviour for direct monitoring.

Qualcomm think if we do not restrict, we will have spec impact related to whether to forward the paging monitoring request.

ZTE think we do not need to have explicit control of turning monitoring on and off; the paging request is always forwarded.

Agreements:

Support the child UE relying on the parent relay UE for paging monitoring.

Strive to minimize spec impact to support intermediate relay UEs in coverage monitoring paging for a child UE on Uu interface, while avoiding duplicated paging delivery to the remote UE due to double-monitoring by upstream UEs.

SI

P1-P2

R2-2502194	Control plane and SRAP for multi-hop relay	Nokia	discussion	NR_SL_relay_multihop

Proposal 1: An intermediate relay UE can acquire SI from both the last relay UEs serving cell, if applicable, as well as through PC5-RRC from the parent relay UE.

Proposal 2: The intermediate relay UE can respond directly instead of forwarding the request to the parent node if it has the requested SI.

P15/P18/P19

R2-2501853	Discussion on control plane procedures for multi-hop SL Relay	ZTE Corporation, Sanechips	discussion	Rel-19	NR_SL_relay_multihop

Proposal 15.	No need to include remote UE ID in SI request.

Proposal 18.	Proposal：Below b), c), and e) apply in all RRC states:

The intermediate relay UE can send SI (e.g., in UuMessageTransferSidelink) to a child node:

b)	Upon acquisition (from the network) of SI requested by a child node (intermediate relay or remote UE)

c)	Upon receiving updated SIBs from the network which have been requested by a child node (intermediate relay or remote UE)

e)	Upon receiving updated SIB1 from the network (as in Rel17)

Proposal 19.	When there is a change in the ability of the intermediate UE to receive SIB broadcast on Uu (e.g., moving in/out of coverage) to initiate/cancel SI forwarding by the parent relay, it’s left to UE implementation to initiate/cancel the SI forwarding by the parent relay.

[Post129bis][412][Relay] FFS issues on system information (ZTE)

Scope: Discuss the identified FFS issues on system information (proposals flagged for treatment and not treated in RAN2#129bis) and converge where possible.

Intended outcome: Report to next meeting

Deadline: Long

Other contributions

R2-2501861	Control Plane aspects for Multi-hop Relay	NEC	discussion	Rel-19	NR_SL_relay_multihop

R2-2501876	Discussion on the Control Plane Procedures	CATT	discussion	Rel-19	NR_SL_relay_multihop-Core

R2-2502190	Text Proposal for TS 38.351 for fast SRB0/1 forwarding	Apple	discussion	Rel-19	NR_SL_relay_multihop

R2-2502232	Discussion on control plane aspects for NR sidelink multi-hop relay	China Telecom	discussion	Rel-19	NR_SL_relay_multihop-Core

R2-2502362	Control plane in Multi-hop relay	Lenovo	discussion	Rel-19

R2-2502379	discussion on C-plane procedure for multi-hop relay	Sharp	discussion	Rel-19	NR_SL_relay_multihop-Core

R2-2502432	Discussion on control plane for NR sidelink multi-hop relay	Spreadtrum, UNISOC	discussion	Rel-19

R2-2502559	Control Plane Handling for Multi-hop U2N Relays	InterDigital	discussion	Rel-19	NR_SL_relay_multihop

R2-2502684	Discussion on control plane procedure for SL relay	KT Corp.	discussion

R2-2502778	Consideration on CP issues for multi-hop SL relay	Samsung	discussion	Rel-19	NR_SL_relay_multihop-Core

R2-2502830	Issues on SRAP operations for supporting multi-hop L2 U2N Relay	ASUSTeK	discussion	Rel-19	NR_SL_relay_multihop

R2-2502939	SRAP design for R19 multi-hop SL relaying	Samsung R&D Institute UK	discussion

### 8.13.4	Service continuity

First priority scenarios: (A) intra-gNB multi-hop indirect to direct path switch, (B) intra-gNB multi-hpo indirect to single-hop indirect path switch.  Second priority scenarios: (C) intra-gNB direct to multi-hop indirect path switch, (D) intra-gNB single-hop indirect to multi-hop indirect path switch.

Scenarios A/B

FFS points

P1-P4

R2-2501877	Intra-gNB Service Continuity for Multi-hop U2N Relay	CATT	discussion	Rel-19	NR_SL_relay_multihop-Core

Noted

Agreements:

In scenarios A/B, the Intermediate Relay UE could be configured to perform the measurement on adjacent upstream relay link, Uu link and candidate relay link.

In scenarios A/B, the measurement report from Intermediate relay UE will also include at least serving intermediate/last relay UE's source L2 ID, serving cell ID and sidelink measurement quantity result.

Upon Intermediate Relay UE (including first relay UE) receiving reconfigurationWithSync, it either indicates to upper layers (to trigger PC5 unicast link release) or sends NotificationMessageSidelink message to the downstream intermediate Relay/Remote UE.  The choice is implementation-based.

Measurement event X2 could be applied to scenario B with multi-hop indirect to single-hop indirect path switching.

Reconfiguration signalling

P1-P2

R2-2502363	Service continuity for Multi-hop system	Lenovo	discussion	Rel-19

Noted

Proposal 1: reconfigurationWithSync without sl-PathSwitchConfig can be reused to trigger Case A (Intra-gNB multi-hop indirect to direct path switching).

Proposal 2: reconfigurationWithSync with sl-PathSwitchConfig can be reused to trigger Case B (Intra-gNB multi-hop indirect to single-hop indirect path switching).

Discussion:

ZTE think the stage 3 CRs are already reflecting these proposals and we can check them in CR review.  CATT agree.

Samsung think the proposals are straightforward and we could agree to them.

Agreements:

reconfigurationWithSync without sl-PathSwitchConfig can be reused to trigger Case A (Intra-gNB multi-hop indirect to direct path switching).

reconfigurationWithSync with sl-PathSwitchConfig can be reused to trigger Case B (Intra-gNB multi-hop indirect to single-hop indirect path switching).

Scenarios C/D

Target RRC states

P5-P6

R2-2501855	Discussion on Service continuity	ZTE Corporation, Sanechips	discussion	Rel-19	NR_SL_relay_multihop

Noted

Proposal 5.	For the selected target multi-hop indirect path, the Last relay UE could be in any RRC state. If the Last relay UE is in RRC_IDLE/INACTIVE state, it will be triggered to go into RRC_CONNECTED if RLC1 message(i.e. RRCReconfigurationComplete message) is received.

Proposal 6.	For each intermediate relay along the selected multi-hop indirect path, it could be in OOC or RRC_IDLE/INACTIVE state. If the intermediate relay is not in RRC_CONNECTED, it will be triggered to go into RRC_CONNECTED towards the serving cell of last relay UE, if RLC1 message(i.e. RRCReconfigurationComplete message) is received.

Discussion:

OPPO think this is not aligned with the WID.  LG think these cases were discussed in WID drafting and we restricted away from them.

Qualcomm think the WID could be interpreted in two ways: All the candidate relay UEs should be connected before path switching, or after path switching.  They are not sure how we would ensure the first interpretation.

CATT agree with OPPO and LG.  They think the first understanding from Qualcomm is correct, and gNB implementation can ensure it.

Qualcomm understand for model A discovery, the remote UE does not know all the information on the relay path and cannot report everything to the gNB.

NEC agree with OPPO and LG; for Qualcomm’s interpretation, they think it is the first one.

Apple have a similar concern to Qualcomm about how the state can be known in advance and how the gNB can be sure of finding a valid path.

Agreements:

RAN2 will not address cases of service continuity towards a relay path with intermediate relays in idle/inactive under the current WID.

Events for C and D

P2/P4

R2-2502233	Discussion on service continuity for multi-hop relay	China Telecom	discussion	Rel-19	NR_SL_relay_multihop-Core

Proposal 2: Event Y1/ Y2 could be used for intra-gNB direct to multi-hop indirect path switching. The PC5 quality between the remote UE and candidate U2N relay/candidate first relay is considered for evaluating Event Y1/ Y2.

Proposal 4: Event Z1/ Y2 could be used for intra-gNB multi-hop indirect to direct path switching. The remote UE shall consider the first relay as serving L2 U2N relay for evaluating Event Z1.

Contents of report for C and D

P16-P17

R2-2502380	discussion on service continuity for multi-hop relay	Sharp	discussion	Rel-19	NR_SL_relay_multihop-Core

Proposal 16. Candidate UE’s cell info, Candidate relay UE’s source ID, Measurement result and information indicating whether the measurement result is SL-RSRP or SD-RSRP are reported to the gNB as with legacy.

Proposal 17. Potential remote UE reports to the gNB the PC5 link qualities of each hops of the path first relay UE belongs or accumulated QoS for the PC5 links (i.e. Achievable PDB).

Source relay becomes target

P6-P7

R2-2502420	Discussion on service continuity for scenario C and D	vivo	discussion	Rel-19

Proposal 6: For scenario D, RAN2 supports the case that the target first/intermediate/last relay UE are all new relay UEs which are not the source single-hop U2N relay UE.

Proposal 7: For scenario D, RAN2 does not specify anything to support the case that that either the target first/intermediate/last relay UE is the source single-hop U2N relay UE.

Issues outside scenarios A/B/C/D

Path switch by intermediate relay (following proposals treated jointly)

P4

R2-2502675	Service Continuity for Multi-Hop Relays	Ericsson	discussion	Rel-19

Proposal 4	Do not pursue downstream notification of the path switch/handover to trigger relay reselection at the remote UE, reuse Rel-17/18 mechanisms.

P7-P8

R2-2502626	Service Continuity for U2N multihop relay 	Kyocera	discussion

Proposal 7	It should be an option for intermediate relay UE to perform direct or indirect path switch to the same cell without sending notification to the remote UE prior to path switch.

Proposal 8	The intermediate relay UE will only need to notify the PC5 connected remote UE in case the path switch fails or the path switch resulted in a cell change.

R2-2501800	Service continuity of multi-hop U2N relay	OPPO	discussion	Rel-19	NR_SL_relay_multihop-Core

R2-2501892	Considerations on Service Continuity of Multi-hop Relay	NEC	discussion	Rel-19

R2-2502199	Service continuity aspects of multi-hop relay	Nokia	discussion	NR_SL_relay_multihop

R2-2502456	Service continuity discussion	Qualcomm Incorporated	discussion	NR_SL_relay_multihop-Core

R2-2502611	Discussion on service continuity for Multi-hop Relay	Huawei, HiSilicon	discussion	Rel-19	NR_SL_relay_multihop-Core

R2-2502694	Discussion on service continuity for multi-hop U2N relay	LG Electronics Inc.	discussion	Rel-19

Withdrawn/Not available

R2-2501891	Considerations on Service Continuity of Multi-hop Relay	NEC Corporation (ARIB)	discussion	Rel-19

Withdrawn

## 8.14	Additional topological enhancements

(NR_WAB_5GFemto; leading WG: RAN3; REL-19; WID RP-243009)

Time budget: 0 TU

Tdoc Limitation: 0 tdocs

Work on this WI will only be triggered by LS from RAN3 so work on this WI is not expected to start RAN2#127bis or RAN2#128.

No contributions expected for this meeting

R2-2501725	Reply LS on Support of UE move between CAG cell of 5G Femto and CSG cell (R3-244830; contact: Ericsson)	RAN3	LS in	Rel-19	FS_NR_WAB_5GFemto	To:SA2	Cc:RAN2

Noted

R2-2501732	Reply LS on FS_VMR_Ph2 solution impacts to RAN (Additional ULI) (R3-250901; contact: ZTE)	RAN3	LS in	Rel-19	FS_VMR_Ph2, NR_WAB_5GFemto-Core	To:SA2	Cc:RAN2

Noted

R2-2501733	Reply LS on Support of Location Service Involving WAB-Nodes (R3-250902; contact: Huawei)	RAN3	LS in	Rel-19	VMR_Ph2, NR_WAB_5GFemto-Core	To:SA2	Cc:RAN2

Noted

R2-2501749	Reply LS on Clarification regarding definition of 5G NR femto ownership (S2-2411241; contact: LGE)	SA2	LS in	Rel-19	FS_5G_Femto_Sec, 5G_Femto	To:SA3	Cc:RAN2, RAN3

Noted

R2-2501750	Reply LS on FS_VMR_Ph2 solution impacts to RAN (Access Control and Additional ULI) (S2-2501324; contact: Ericsson)	SA2	LS in	Rel-19	FS_VMR_Ph2, VMR_Ph2	To:RAN3	Cc:RAN2

Noted

R2-2501751	Reply LS on FS_VMR_Ph2 solution impacts to RAN (MWAB mobility) (S2-2501336; contact: Nokia)	SA2	LS in	Rel-19	VMR_Ph2	To:RAN3	Cc:RAN2

Noted

R2-2501755	LS on NR Femto node shared by PLMN and PNI-NPN (S2-2502787; contact: LGE)	SA2	LS in	Rel-19	5G_Femto, NR_WAB_5GFemto-Core, eNPN	To:RAN3	Cc:RAN2

Noted

## 8.15	NavIC L1 SPS A-GNSS support

(LCS_NAVIC_L1_SPS_NR_LTE-Core; leading WG: RAN2; REL-19; WID RP-243247[to be updated after RAN#107])

Time budget: 0 TU

Tdoc Limitation: 0 tdocs

No input is expected on this agenda item for RAN2#129bis

## 8.16	BDS B2b in A-GNSS

LCS_BDS_B2b_LTE_NR; leading WG: RAN2; REL-19; WID RP-242459[to be updated after RAN#107])

Time budget: 0 TU

Tdoc Limitation: 0 tdocs

No input is expected on this agenda item for RAN2#129bis

## 8.17	IoT-NTN TDD mode

(IoT_NTN_TDD; leading WG: RAN1; REL-19; WID RP-243293)

Time budget: 0.5 TU

Tdoc Limitation: 1 tdoc

Workplan

R2-2501988	Work plan for WID: introduction of IoT-NTN TDD mode	Iridium Satellite LLC	Work Plan	Rel-19	IoT_NTN_TDD

Noted

R2-2501780	Discussion on RAN2 Imapcts of IoT-NTN TDD mode	vivo	discussion	Rel-19	IoT_NTN_TDD-Core

R2-2501970	Discussion on RAN2 impacts of IoT-NTN TDD	Huawei, HiSilicon	discussion	Rel-19	IoT_NTN_TDD

R2-2502059	Discussion on support of NB-IoT NTN TDD	CATT	discussion	Rel-19

R2-2502071	Discussion on IoT NTN TDD mode	OPPO	discussion	Rel-19	IoT_NTN_TDD

R2-2502100	Discussion on support of TDD mode for IoT-NTN	Nokia, Nokia Shanghai Bell	discussion	Rel-19	IoT_NTN_TDD

R2-2502193	Discussion on support of IoT-NTN TDD mode	THALES	discussion	Rel-19	IoT_NTN_TDD-Core

R2-2502358	Discussion on TDD support in IoT NTN	Lenovo	discussion	Rel-19

R2-2502459	On RAN2 aspect of IoT NTN TDD	Samsung	discussion	Rel-19	IoT_NTN_TDD

R2-2502517	Discussion on supporting IoT NTN TDD mode	Apple	discussion	Rel-19	IoT_NTN_TDD

R2-2502527	Consideration on IoT-NTN TDD mode	ZTE Corporation, Sanechips	discussion	Rel-19	IoT_NTN_TDD

R2-2502538	Discussion on the IoT NTN TDD mode	Xiaomi	discussion

R2-2502560	On SI scheduling, H-SFN change and postponing impacts in IoT-NTN TDD mode	Nordic Semiconductor ASA	discussion	Rel-19

R2-2502621	Indication of IoT-NTN TDD Mode Support	Toyota ITC	discussion	Rel-19	IoT_NTN_TDD

R2-2502658	Discussion on new NB-IoT NTN TDD mode	Qualcomm Incorporated	discussion	Rel-19	IoT_NTN_TDD

R2-2502703	Support of IoT-NTN TDD mode	CMCC	discussion	Rel-19	IoT_NTN_TDD

- Paging

R2-2502527	Consideration on IoT-NTN TDD mode	ZTE Corporation, Sanechips	discussion	Rel-19	IoT_NTN_TDD

Proposal 1: RAN2 discusses and decides which of the following options is used for Paging occasion determination:

Option 1: New paging DRX cycle with multiple 9 radio frames and radio frame offset for PF determination are used to determine the PF and PO accurately.

Option 2: Legacy NB-IoT PO determination mechanism is used. When the determined paging subframe#n is not a valid downlink subframe, the PO is postponed to the nearest valid downlink subframe#n.

-	Iridium prefers option 2 and thinks it would be easier for chipset implementations. Nordic and QC agree. QC thinks the same applies to a number of other open issues

-	vivo thinks we should align to other procedures discussed in RAN1 where there is the option to drop rather than postpone some occasions. Nokia thinks we can decide in RAN2. HW agrees with Nokia.

Regarding paging occasion determination, legacy NB-IoT PO determination mechanism is used. When the determined paging subframe is not a valid downlink subframe, the Paging monitoring is postponed to the nearest valid downlink subframe.

-	Cell barring

R2-2501989	Discussion on IoT-NTN TDD mode	Iridium Satellite LLC	discussion	Rel-19	IoT_NTN_TDD

Proposal 13: In IoT-NTN TDD mode, existing cell barring mechanism using the IE cellBarred-r13 and cellBarred-NTN-r17 in SIB1 is sufficient to control access to the IoT-NTN TDD cell.

Agreed

R2-2501970	Discussion on RAN2 impacts of IoT-NTN TDD	Huawei, HiSilicon	discussion	Rel-19	IoT_NTN_TDD

Proposal 1: The IE cellBarred in the SIB1-NB message can be reused to control whether a UE supporting NB-IoT NTN TDD mode is allowed to access an NB-IoT TDD NTN cell.

- Timers value range extension

R2-2502527	Consideration on IoT-NTN TDD mode	ZTE Corporation, Sanechips	discussion	Rel-19	IoT_NTN_TDD

Proposal 4: RAN 2 confirms that the existing value range of timers with unit of pp are enough for IoT NTN TDD.

-	QC wonders if we need to take into account that invalid subframes need to be considered. Xiaomi thinks the issue of invalid subframes is not specific of IoT NTN but NB-IoT in general and this is not considered a problem in NB-IoT.

Existing value ranges of timers in unit of PDCCH periods are reused for IoT NTN TDD (FFS on the possible clarification to take into account the impact of invalid subframes)

-	PUR transmission

R2-2502059	Discussion on support of NB-IoT NTN TDD	CATT	discussion	Rel-19

Proposal 5: For SPS/PUR, RAN2 discusses whether to capture in the MAC Spec that UE skips an SPS/PUR occasion if it is a non-UL slot.

R2-2502527	Consideration on IoT-NTN TDD mode	ZTE Corporation, Sanechips	discussion	Rel-19	IoT_NTN_TDD

Proposal 2: When PUR resource start subframe does not align with the UL subframes in the H-SFN, UE postpones the PUR resource start subframe to the latest UL subframe for PUR transmission.

-	HW thinks we should align the behaviour for PUR and SPS. Vivo agrees.

-	Xiaomi also would like to have a unified solution to postpone the transmission

When PUR resource start subframe does not align with the UL subframes in the H-SFN, UE postpones the PUR resource start subframe to the next valid UL subframe

- UL SPS

R2-2501970	Discussion on RAN2 impacts of IoT-NTN TDD	Huawei, HiSilicon	discussion	Rel-19	IoT_NTN_TDD

Proposal 2a: To reduce the overlap of UL SPS with non-U NB-IoT subframes, new SPS periodicities should be introduced to align with the TDD structure.

-	CMCC supports this

-	Xiaomi thinks this cannot avoid collisions and then we might not need this

Proposal 2b: If UL SPS transmission can’t be finished within a TDD pattern, the remaining repetitions can be postponed to the NB-IoT UL subframes of the following TDD patterns.

R2-2502527	Consideration on IoT-NTN TDD mode	ZTE Corporation, Sanechips	discussion	Rel-19	IoT_NTN_TDD

Proposal 8: UL SPS transmission is dropped when the UL SPS overlaps with non-U NB-IoT subframes

When the UL SPS overlaps with non-U NB-IoT subframes UE postpones the UL SPS resource to the next valid UL subframe

- SI reception

R2-2501780	Discussion on RAN2 Imapcts of IoT-NTN TDD mode	vivo	discussion	Rel-19	IoT_NTN_TDD-Core

Proposal 4: Introduce new periodicities based on the time unit of the TDD frame and/or the hyper TDD frame to decide the SI window and SI transmission resource.

R2-2501970	Discussion on RAN2 impacts of IoT-NTN TDD	Huawei, HiSilicon	discussion	Rel-19	IoT_NTN_TDD

Proposal 7: For the SI reception within an SI-window, UE should prioritize the previous SI reception in case there is collision between the previous SI repetitions and the subsequent SI reception.

R2-2501989	Discussion on IoT-NTN TDD mode	Iridium Satellite LLC	discussion	Rel-19	IoT_NTN_TDD

Proposal 5: In IoT-NTN TDD mode, if start of the SI window falls within the downlink inactive time, it shall be postponed to the next downlink active time.

Proposal 6: In IoT-NTN TDD mode, subframes from a single repetition of the SI transmission shall be allowed to be postponed to the next downlink active time opportunity.

Proposal 7: In IoT-NTN TDD mode, SI transmissions shall be truncated if, due to the proposed postponement, they overlap with the transmission of the next SI repetition within the SI window.

R2-2502059	Discussion on support of NB-IoT NTN TDD	CATT	discussion	Rel-19

Proposal 2: RAN2 discusses whether to capture in RRC Spec (system information procedure) that if a subframe is within the SI window as determined by the current Spec but is a non-DL subframe, UE skips the reception of the corresponding SI thereon.

R2-2502560	On SI scheduling, H-SFN change and postponing impacts in IoT-NTN TDD mode	Nordic Semiconductor ASA	discussion	Rel-19

Proposal-3: The scheduling procedure of other SI messages remains unchanged.

Proposal-4: Postponing of other SI subframes to the next 90ms TDD frame is supported.

Proposal-5: A network configuration shall ensure SI repetitions within a SI-window never overlap.

Proposal-6: Other SI transmission that overflows a configured SI-window shall be truncated.

- k-mac

R2-2502193	Discussion on support of IoT-NTN TDD mode	THALES	discussion	Rel-19	IoT_NTN_TDD-Core

Proposal 7	Support k-Mac-r19 with a value range of 1 – 1023 ms

For IoT NTN TDD mode, support k-Mac with a value range up to 1023 ms (add corresponding a restriction in the field description)

Can further discuss extending this proposal to IoT NTN and NR NTN in the next meetings

R2-2502100	Discussion on support of TDD mode for IoT-NTN	Nokia, Nokia Shanghai Bell	discussion	Rel-19	IoT_NTN_TDD

Proposal 4: RAN2 to confirm that extending the Kmac beyond 512 ms will block the cell for legacy release 17 and 18 NTN UEs.

Proposal 5: Further details about the scenarios requiring an extended Kmac should be provided to justify the Kmac value extension.

Agreements:

1.	Regarding paging occasion determination, legacy NB-IoT PO determination mechanism is used. When the determined paging subframe is not a valid downlink subframe, the Paging monitoring is postponed to the nearest valid downlink subframe.

2.	In IoT-NTN TDD mode, existing cell barring mechanism using the IE cellBarred-r13 and cellBarred-NTN-r17 in SIB1 is sufficient to control access to the IoT-NTN TDD cell.

3.	Existing value ranges of timers in unit of PDCCH periods are reused for IoT NTN TDD (FFS on the possible clarification to take into account the impact of invalid subframes

4.	When PUR resource start subframe does not align with the UL subframes in the H-SFN, UE postpones the PUR resource start subframe to the next valid UL subframe

5.	When the UL SPS overlaps with non-U NB-IoT subframes UE postpones the UL SPS resource to the next valid UL subframe

6.	For IoT NTN TDD mode, support k-Mac with a value range up to 1023 ms (add corresponding a restriction in the field description)

## 8.18	TEI19

Time budget: 1 TU

Tdoc Limitation: 1 tdoc for new proposals and 1 tdoc for old proposals.

1 additional tdoc for primary co-sourcing company on top of the limit is allowed for co-sourced contribution with 4 or more companies.

Companies are encouraged to submit co-sourced contributions, which will have priority for discussion in RAN2#129bis.

BWP restriction

R2-2501869	Handling downlink BWP restriction for MT-SDT	Samsung	discussion	Rel-19	TEI19

R2-2501991	BWP restriction in MT-SDT	LG Electronics Inc.	discussion	Rel-19	TEI19

R2-2502209	SDT on separate BWP	ZTE Corporation, Sanechips	discussion

R2-2502403	BWP restriction for SDT	NEC	discussion	Rel-19	TEI19

R2-2502498	Removing initial DL BWP restriction for MT-SDT	Sony	discussion	Rel-19	TEI19

This topic will not be pursued as Rel-19 TEI unless companies bring co-source converged Tdoc to next meeting

[ANR_HSDN]

R2-2502783	Introduction of ANR reporting of HSDN cells [ANR_HSDN]	Huawei, HiSilicon, CMCC, China Unicom, China Telecom, CATT, NTT DoCoMo, Samsung	CR	Rel-19	38.331	18.5.1	5318	-	B	TEI19

Revised in R2-2502996

R2-2502996 	Introduction of ANR reporting of HSDN cells [ANR_HSDN]	Huawei, HiSilicon, CMCC, China Unicom, China Telecom, CATT, NTT DoCoMo, Samsung	CR	Rel-19	38.331	18.5.1	5318	1	B	TEI19

The CR is in principle agreed

R2-2502784	Introduction of ANR reporting of HSDN cells [ANR_HSDN]	Huawei, HiSilicon, CMCC, China Unicom, China Telecom, CATT, NTT DoCoMo, Samsung	CR	Rel-19	38.306	18.5.0	1264	-	B	TEI19

The CR is in principle agreed

R2-2502785	Introduction of ANR reporting of HSDN cells [ANR_HSDN]	Huawei, HiSilicon, CMCC, China Unicom, China Telecom, CATT, NTT DoCoMo, Samsung	CR	Rel-19	36.331	18.5.0	5110	-	B	TEI19

Revised in R2-2502997

R2-2502997	Introduction of ANR reporting of HSDN cells [ANR_HSDN]	Huawei, HiSilicon, CMCC, China Unicom, China Telecom, CATT, NTT DoCoMo, Samsung	CR	Rel-19	36.331	18.5.0	5110	1	B	TEI19

The CR is in principle agreed

R2-2502786	Introduction of ANR reporting of HSDN cells [ANR_HSDN]	Huawei, HiSilicon, CMCC, China Unicom, China Telecom, CATT, NTT DoCoMo, Samsung	CR	Rel-19	36.306	18.4.0	1911	-	B	TEI19

The CR is in principle agreed

UE aggregation

Missing TEI identifiers

[treated Wednesday]

R2-2502734	Discussion on UE aggregation enhancement	CMCC, ZTE, MediaTek, vivo, Huawei, CATT, Meta, Nokia, Nokia Shanghai Bell, xiaomi, Spreadtrum, UNISOC	discussion	Rel-19	TEI19

=> Revised in R2-2503151

R2-2503151	Discussion on UE aggregation enhancement [N3C-M-Relay Ues]	CMCC, ZTE, MediaTek, vivo, Huawei, CATT, Meta, Nokia, Nokia Shanghai Bell, xiaomi, Spreadtrum, UNISOC	discussion	Rel-19	TEI19

Noted

Agreements

1: RAN2 is suggested to support multiple indirect paths for multi-path relay via N3C.

2：It is proposed to only consider the scenario where multiple indirect paths under the same gNB, and CU-DU split structure is not considered in TEI19.

3: It is proposed to reuse the maximum number (i.e. 8) for multiple indirect paths via N3C.

4: Introduce the new IE n3c-ExtIndirectPathAddChange to provide or release the configuration of N3C relay UEs.

5: The legacy mechanism for data split for multiple indirect paths via N3C can be reused, and no spec impact is needed, i.e. it is up to UE implementation to select a path for the data split when the total amount of PDCP data volume is equal to or larger than ul-DataSplitThreshold.

6: For PDCP duplication, it can be up to UE implementation to decide which MP Relay UE is used when more than one MP Relay UEs are configured in N3C interface, without impacts on PDCP spec and MAC spec.

7: the existing n3c-Failure type in FailureTypeIndirectPath-r18 IE can be reused by the remote UE to indicate the indirect path failure.

8: A new remote UE capability to support multiple indirect paths via N3C should be introduced.

R2-2502735	Corrections to TS 38.300 on multi-path relay enhancement	CMCC, ZTE, MediaTek, vivo, Huawei, CATT, Meta, Nokia, Nokia Shanghai Bell, xiaomi, Spreadtrum, UNISOC	draftCR	Rel-19	38.300	18.5.0	TEI19

=> Revised in R2-2503152

R2-2503152	Corrections to TS 38.300 on multi-path relay enhancement [N3C-M-Relay Ues]	CMCC, ZTE, MediaTek, vivo, Huawei, CATT, Meta, Nokia, Nokia Shanghai Bell, xiaomi, Spreadtrum, UNISOC	draftCR	Rel-19	38.300	18.5.0	TEI19

=> Revised in R2-2503188

R2-2502736	Corrections to TS 38.331 on multi-path relay enhancement	CMCC, ZTE, MediaTek, vivo, Huawei, CATT, Meta, Nokia, Nokia Shanghai Bell, xiaomi, Spreadtrum, UNISOC	draftCR	Rel-19	38.331	18.5.1	TEI19

=> Revised in R2-2503153

R2-2503153	Corrections to TS 38.331 on multi-path relay enhancement [N3C-M-Relay Ues]	CMCC, ZTE, MediaTek, vivo, Huawei, CATT, Meta, Nokia, Nokia Shanghai Bell, xiaomi, Spreadtrum, UNISOC	draftCR	Rel-19	38.331	18.5.1	TEI19

=> Revised in R2-2503189

R2-2502737	Corrections to TS 38.306 on multi-path relay enhancement	CMCC, ZTE, MediaTek, vivo, Huawei, CATT, Meta, Nokia, Nokia Shanghai Bell, xiaomi, Spreadtrum, UNISOC	draftCR	Rel-19	38.306	18.5.0	TEI19

=> Revised in R2-2503156

R2-2503156	Corrections to TS 38.306 on multi-path relay enhancement [N3C-M-Relay UEs]	CMCC, ZTE, MediaTek, vivo, Huawei, CATT, Meta, Nokia, Nokia Shanghai Bell, xiaomi, Spreadtrum, UNISOC	draftCR	Rel-19	38.306	18.5.0	TEI19

=> Revised in R2-2503190

[POST129bis][021][TEI19] Multi-path relay enhancements (CMCC)

Intended outcome:  in principle agree to CR

Deadline:  Short

R2-2503188	Corrections to TS 38.300 on multi-path relay enhancement [N3C-M-Relay Ues]	CMCC, ZTE, MediaTek, vivo, Huawei, CATT, Meta, Nokia, Nokia Shanghai Bell, xiaomi, Spreadtrum, UNISOC	draftCR	Rel-19	38.300	18.5.0	TEI19

=> Agreed in principle

R2-2503189	Corrections to TS 38.331 on multi-path relay enhancement [N3C-M-Relay Ues]	CMCC, ZTE, MediaTek, vivo, Huawei, CATT, Meta, Nokia, Nokia Shanghai Bell, xiaomi, Spreadtrum, UNISOC	draftCR	Rel-19	38.331	18.5.1	TEI19

=> Agreed in principle

R2-2503190	Corrections to TS 38.306 on multi-path relay enhancement [N3C-M-Relay UEs]	CMCC, ZTE, MediaTek, vivo, Huawei, CATT, Meta, Nokia, Nokia Shanghai Bell, xiaomi, Spreadtrum, UNISOC	draftCR	Rel-19	38.306	18.5.0	TEI19

=> Agreed in principle

MPS

R2-2502914	Discussion on RNA update for MPS [MPS_RNAU_ResumeCause]	Nokia	discussion

Proposal RAN2 to further study the issue of resumeCause used by high priority users during RNA update procedure taking into consideration provided observations.

Noted

R2-2502575	MPS at gNB overload	Ericsson	discussion	Rel-19	TEI19

Noted

To be discussed next meeting

UE Radio Paging Capability

Paging

R2-2502214	Addition of band specific number of Rx branches supported by a UE	Nokia	CR	Rel-19	38.331	18.5.1	5293	-	B	TEI19, NR_redcap-Core, NR_redcap_enh-Core, NR_XR_enh-Core

Missing TEI19 identifier

-	Qualcomm thinks that technically this makes sense but we have a discussion on paging container on other topics so we should wait to see how to handle this.

-	Xiaomi doesn’t see a motivation to have this as it is ok to have capability per cell instead of per band.

-	Tmobile would like some more time to check

We will address the issue of band specific capability.  No UE behaviour impact.   The solution is FFS and depending on other paging discussion.

The CR is postponed

R2-2502576	UE Radio Paging Capability	Ericsson	discussion	Rel-19	TEI19

Proposal 1	Use a “container” using OCTET STRING with content generated by UE when new UE Radio Paging Capabilities are introduced.

-	Nokia sympathizes with the idea but not sure if this is the only solution is to send it twice, we can find a solution where the UE sends it only once.

-	Vivo supports the proposal but asks whether if we go this way it would mean that we don’t address legacy.  Huawei thinks that we can do nothing for legacy and we leave it to network implementation.  Vivo is ok with this way but should update SA2.

-	Qualcomm agrees with proposal but only for Rel-19 and future capabilities to be added in paging.  Huawei always agrees with this proposal.

-	Xiaomi is concerned with this duplication

-	Tmobile is questioning the usefulness of this feature.

Noted

Agreements

- Use a “container” using OCTET STRING with content generated by UE when new UE Radio Paging Capabilities are introduced.   This is applicable only to new Rel-19 and future capabilities added to paging capabilities.  This will only work for a gNB that supports this Rel-19 “container”

- We will not change legacy UE capabilities and rely on network implementation

- Send LS to SA2

[AT129bis][009][UE cap] Reply LS to SA2 (Huawei)

Intended outcome: LS to SA2 to capture agreements and agree by email

Deadline:  Thursday

R2-2503172	Reply LS on paging capability loss issue	RAN2	LS out	Rel-19	NR_LPWUS-Core, TEI19	To:SA2, RAN3, CT1

The LS is approved

Simultaneous RAN and CN paging

R2-2502616	Discussion on simultaneous RAN and CN paging	Nokia	discussion

Proposal 1: RAN2 to decide whether the 38.331 Clause 5.3.2.2 ‘one PagingRecord for each UE’ requirement for the simultaneous 5G-S-TMSI CN-initiated and fullI-RNTI RAN-initiated paging scenario is needed.

Proposal 2: If the existing limitation is not needed relax ‘one PagingRecord for each UE’ requirement so that simultaneous 5G-S-TMSI CN-initiated and fullI-RNTI RAN-initiated paging is allowed. Draft CR for 38.331 Rel-18 is provided for discussion and agreement [1].

Proposal 3: If the existing limitation is needed send LS to RAN3 asking for specification of signaling enabling implementation of ‘one PagingRecord for each UE’ requirement for the simultaneous 5G-S-TMSI CN-initiated and fullI-RNTI RAN-initiated paging scenario.

-	Samsung, Vivo and CATT thinks this is a very rare case and network implementation can avoid scenario 1 and 2.   Nokia indicates that this is seen when we start using DRX and it is related to DRX cycle and it has a negative impact to system performance.   Samsung this that this problem is only valid if we have timing requirement.

-	CATT indicates that there were similar discussions in RAN3 and the conclusion was that there is no issues as CN can continue to finish paging.

-	Xiaomi thinks that even if there are two paging we can leave it to UE implementation

-	ZTE agrees no need for this proposals.

-	Qualcomm wants to avoid introducing such changes.

No support for now

Noted

R2-2502619	Simultaneous RAN and CN paging	Nokia	CR	Rel-19	38.331	18.5.1	5311	-	F	TEI19

RAT restrictions

R2-2502672	Discussion on RAN impact of UE usage of RAT restrictions	Philips International B.V., NEC	discussion	Rel-19	TEI19, ECRATU

Proposal 1: RAN2 is kindly requested to further discuss the CONNECTED mode mobility AS layer impact of the new CT1 work item ECRATU and decide how to proceed.

-	Samsung thinks this should be discussed in CT1 first and we have only been requested to only do Idle mode

This case was not requested by CT1 and we will not work on this

Noted

NTN related

LTE TN to NB-IoT NTN mobility

R2-2501781	Introduction of LTE TN to NB-IoT NTN Mobility UE Capability	vivo, Samsung, Google, THALES, MediaTek Inc.	draftCR	Rel-19	36.306	18.4.0	TEI19

R2-2502803	36331CR for the inclusion of NB-IoT satellite information in E-UTRAN	Google, Samsung, vivo, THALES	draftCR	Rel-19	36.331	18.5.0	B	TEI19

R2-2502804	36300CR for the inclusion of NB-IoT satellite information in E-UTRAN	Google, Samsung, vivo, THALES	draftCR	Rel-19	36.300	18.4.0	B	TEI19

R2-2501971	ETWS geo-fencing for eMTC NTN and for TN	Huawei, HiSilicon, China Southern Power Grid, Turkcell	discussion	Rel-19	TEI19

R2-2502689	Redirection to IoT NTN and NR NTN	Samsung	discussion	Rel-19	TEI19

All contributions in this AI will be treated in the next meeting

Positioning

SRS frequency hopping

R2-2501714	LS on non-RedCap UE UL SRS frequency hopping for positioning (R1-2501573; contact: ZTE)	RAN1	LS in	Rel-19	TEI19	To:RAN2

Noted

Discussion:

Huawei wonder why this affects us rather than RAN3.  ZTE indicate that the hopping is configured by gNB.  vivo assume the intention is to introduce a new capability, which would affect RAN2.

ZTE indicate that the RAN1 agreement leaves it to RAN2 to decide if new RRC parameters are needed, and the discussion paper addresses this issue.

Huawei think we do not need to discuss UE capability, because eventually the feature list will reflect it.  ZTE agree with this.

R2-2502085	Discussion on the higher layer spec impact of positioning SRS FH for non-RedCap UE	ZTE Corporation	discussion	Rel-19	TEI19

Noted

Proposal 1: There is no need to introduce new RRC parameters for extending pSRS FH to non-RedCap UEs.

Proposal 2: The current 38.331 Rel-18 restriction on maximum bandwidth per hop as 20MHz should be restricted to RedCap UE only, not to non-RedCap UE. RAN2 to adopt the corresponding RRC CR R2-2502087.

Proposal 3: RAN2 to adopt the 38.305 CR R2-2502086 which reflects the pSRS FH feature for both RedCap UE and non-RedCap UE in stage-2.

Proposal 4: Send LS to RAN1 and RAN3 to indicate the identified RRC/stage-2 and NRPPa spec impact of pSRS FH for non-RedCap UEs. Take the draft LS out R2-2502260 as baseline.

Discussion:

Samsung are generally fine with the proposals, but for P3 they are not sure if it is essential; they checked the DL-PRS hopping part and found no similarly detailed explanation.  ZTE indicate that SRS hopping is missing in 38.305.  Samsung are concerned that the newly proposed text is more detailed than for DL-PRS hopping.

vivo have a concern about P4; they think the current draft is too detailed about the enhancements in RAN3 scope.

Huawei think it is OK to involve RAN3, but maybe we just inform them that RAN1 have introduced the feature and we want them to check their spec.  They think for the stage 2 CR, RAN3 is responsible for the relevant text and should handle the CR.

Qualcomm also think the stage 2 part is under RAN3 management, and it may not be needed anyway since the table is quite generic.

ZTE indicate that stage 2 should introduce the feature completely, and it should reflect the applicability to both RedCap and non-RedCap UEs.

Xiaomi agree with Qualcomm and think we may not need to mention the hopping parameters in the stage 2 table; it would open the door to a lot of changes to the table, parameter by parameter.  They also think it is not RAN2 responsibility to notify RAN3.

Nokia understand FH for RedCap was introduced because of the bandwidth limitation of the RedCap UEs, and they wonder what the motivation is to extend it to non-RedCap.  ZTE indicate that the intention is related to configuring a separate BWP for the SRS hopping.

ZTE think RAN3 cannot initiate discussion without being on the LS or having TUs for it, so we need to involve them.  They note that RAN1 requested a reply LS if there is spec impact.

Xiaomi think RAN1 are asking about RAN2 impact.

CATT see spec impact in RRC, as proposed by ZTE, and they think at least we can reply to RAN1 and Cc: RAN3.

Xiaomi agree that we should reply to RAN1 if there is RRC impact, and we can Cc: RAN3, which would enable them to discuss.

R2-2502086	Introduction of the positioning SRS FH for non-RedCap UE in 38305	ZTE Corporation, Ericsson	CR	Rel-19	38.305	18.5.0	0185	-	B	TEI19

Not pursued (section is under RAN3 responsibility)

R2-2502087	Introduction on the SRS frequency hopping for non-RedCap UE in 38331 [PosSrsFH-nonRedCap]	ZTE Corporation, Ericsson	CR	Rel-19	38.331	18.5.1	5290	-	B	TEI19

Postponed

Discussion:

Xiaomi would like to postpone the CR since RAN1 have not discussed the UE capability for non-RedCap UEs.  They also note that it would be good to submit the RAN2 and RAN3 CRs to the same plenary.

CATT think from plenary perspective we can submit together, but we could endorse the CR now.

Xiaomi think there will be a UE capability change, and we do not need to endorse a CR until we have the whole picture.  ZTE understand it will be treated as part of the whole capability discussion, not as part of this CR.  Xiaomi think we should endorse here and merge with the capability discussion.  ZTE understand that there is no RAN2 capability expected that would be endorsed in the positioning session.

DOCOMO indicate that RAN1 are not sure if there is a new parameter in RRC or not.  They agree with Xiaomi that we do not need to endorse the CR this time.

Huawei understand that we have consensus that no new parameter is needed, but they are not sure if there could be additional spec impact.

Agreements:

No new RRC parameter is needed for support of SRS frequency hopping for non-RedCap UE.  This does not exclude a related capability, to be discussed in RAN1.

The current Rel-18 restriction on maximum bandwidth per hop as 20MHz should be restricted to RedCap UE only, not to non-RedCap UE.  Inform RAN1 that we expect this spec impact.

R2-2502260	Draft reply LS on the non-RedCap UE UL SRS frequency hopping for positioning	ZTE Corporation	LS out	Rel-19	TEI19	To:RAN1, RAN3

Indication that RAN2 have agreed a CR to be replaced by quoting our agreements above

Reference to 38.305 CR to be removed

General sentence to let RAN3 consider the spec impact

[Post129bis][408][POS] LS to RAN1/RAN3 on SRS frequency hopping for non-RedCap UE (ZTE)

Scope: Revise the LS in R2-2502260 in line with meeting agreements.

Intended outcome: Approved LS

Deadline: Short

=> Reserved in R2-2503185

R2-2503185	Reply LS on the non-RedCap UE UL SRS frequency hopping for positioning	RAN2	LS out	Rel-19	TEI19	To:RAN1, RAN3

=> Approved

On-demand posSIB

R2-2502074	Introduction of control parameters for on-demand posSIB request [PosOdSIB-Req]	Huawei, HiSilicon, Ericsson	CR	Rel-19	38.331	18.5.1	5288	-	B	TEI19

Revised in R2-2503157

R2-2503157	Introduction of control parameters for on-demand posSIB request [PosOdSIB-Req]	Huawei, HiSilicon, Ericsson	CR	Rel-19	38.331	18.5.1	5288	1	B	TEI19

Discussion:

ZTE indicate that there may be some reference to the interval left on the coversheet.

Xiaomi understand that RAN1 requested a start and stop indicator.  Huawei understand that it is a pure RAN2 feature.

=> Revised in R2-2503192

R2-2502075	Introduction of control parameters for on-demand posSIB request [PosOdSIB-Req]	Huawei, HiSilicon, Ericsson	CR	Rel-19	38.306	18.5.0	1248	-	B	TEI19

Revised in R2-2503158

R2-2503158	Introduction of control parameters for on-demand posSIB request [PosOdSIB-Req]	Huawei, HiSilicon, Ericsson	CR	Rel-19	38.306	18.5.0	1248	1	B	TEI19

=> Revised in R2-2503193

R2-2502076	Introduction of control parameters for on-demand posSIB request [PosOdSIB-Req]	Huawei, HiSilicon, Ericsson	CR	Rel-19	38.300	18.5.0	0978	-	B	TEI19

Revised in R2-2503159

R2-2503159	Introduction of control parameters for on-demand posSIB request [PosOdSIB-Req]	Huawei, HiSilicon, Ericsson	CR	Rel-19	38.300	18.5.0	0978	1	B	TEI19

=> Revised in R2-2503195

R2-2502077	Introduction of control parameters for on-demand posSIB request [PosOdSIB-Req]	Huawei, HiSilicon, Ericsson	CR	Rel-19	38.305	18.5.0	0184	-	B	TEI19

Revised in R2-2503160

R2-2503160	Introduction of control parameters for on-demand posSIB request [PosOdSIB-Req]	Huawei, HiSilicon, Ericsson	CR	Rel-19	38.305	18.5.0	0184	1	B	TEI19

=> Revised in R2-2503194

[Post129bis][409][POS] CRs on control parameters for on-demand posSIB request (Huawei)

Scope: Check the CRs in R2-2503157 / R2-2503158 / R2-2503159 / R2-2503160.

Intended outcome: Agreed in principle CRs

Deadline: Short

R2-2503192	Introduction of control parameters for on-demand posSIB request [PosOdSIB-Req]	Huawei, HiSilicon, Ericsson	CR	Rel-19	38.331	18.5.1	5288	2	B	TEI19

=> Agreed in principle.

R2-2503193	Introduction of control parameters for on-demand posSIB request [PosOdSIB-Req]	Huawei, HiSilicon, Ericsson	CR	Rel-19	38.306	18.5.0	1248	2	B	TEI19

=> Agreed in principle.

R2-2503195	Introduction of control parameters for on-demand posSIB request [PosOdSIB-Req]	Huawei, HiSilicon, Ericsson	CR	Rel-19	38.300	18.5.0	0978	2	B	TEI19

=> Agreed in principle.

R2-2503194	Introduction of control parameters for on-demand posSIB request [PosOdSIB-Req]	Huawei, HiSilicon, Ericsson	CR	Rel-19	38.305	18.5.0	0184	2	B	TEI19

=> Agreed in principle.

## 8.19	NR Others

Tdoc limit: 2

Specific items may be allocated to a breakout session for treatment.

Impacts from Other RAN WGs and TSGs that has no separate TU budget in RAN2. LS ins for Rel-19 specific WIs/SIs that has no RAN WI.

Additional tdocs on top of limit can be allowed for co-sourced contribution with 3 or more companies

Number of UEs in RRC_INACTIVE with data transmission

R2-2501758	Reply to RAN2 LS on Number of UEs in RRC_INACTIVE state with data transmission (S5-250827; contact: China Telecom)	SA5	LS in	Rel-19	PM_KPI_5G_Ph4	To:RAN2	Cc:RAN3

DISCUSSION

-	Nokia think the request is not very clear, e.g., in SA spec there is some related measurements which is DU level, and R2 proposals are per cell.

-	CT think we can discuss based on company proposals.

-	CATT think SA5 request is clear and we can discuss in R2.

Noted

R2-2501910	Discussion on the number of SDT UEs (LS S5-250827)	CATT	discussion	Rel-19	PM_KPI_5G_Ph4

Noted

Proposal 1: According to SA5 reply LS, the measurement metrics “Mean/max number of RRC_INACTIVE UEs with ongoing SDT per cell” are introduced in TS 38.314.

Proposal 2: RAN2 to discuss whether the following measurement metrics are also introduced:

1.	Mean/max number of RRC_INACTIVE UEs with ongoing SDT in the DL per DRB per cell

2.	Mean/max number of RRC_INACTIVE UEs with ongoing SDT in the UL per DRB per cell

3.	Mean/max number of RRC_INACTIVE UEs with ongoing SDT per DRB per cell

4.	Mean/max number of RRC_INACTIVE UEs with ongoing SDT in the DL per SRB per cell

5.	Mean/max number of RRC_INACTIVE UEs with ongoing SDT in the UL per SRB per cell

6.	Mean/max number of RRC_INACTIVE UEs with ongoing SDT per SRB per cell

Proposal 3: As “active UEs” and “inactive UEs” are defined in different levels, R16-R18 spec change in TS 38.314 is needed to clarify in clause 4.2.1.4 inactive UEs refer to UEs in RRC_INACTIVE.

DISCUSSION

P1

-	Nokia think there is no CR proposal to change according to P1. Nokia ask whether we can use RRC values instead.

-	CATT clarifies that there is a co-coursed CR.

P2

-	Nokia think those are not requested by SA5. Nokia think the existing measurments can already reflect connected and inactive combined, so we should not change what we have now.

P3

-	Nokia think inactive refers to buffer status not RRC state so we should not touch that.

R2-2502248	Introduction of number of UEs in RRC_INACTIVE state with data transmission [KPI_SDT_SA5]	China Telecom, Huawei, HiSilicon, ZTE Corporation, Sanechips, CATT	CR	Rel-19	38.314	18.0.0	0034	-	B	TEI19

=> Revised in R2-2503035

DISCUSSION

-	Nokia think in cover page we should remove ME impact.

-	Nokia think the CR should be correction, not for R19. Nokia think it is not good to refer to the RRC_INACTIVE in 38.331. Xiaomi not sure about this, and ask what it the definition in RRC.

-	Nokia point out the definition of active is clear in the talbe i.e., ‘Mean number of Active UEs in the DL per DRB per cell.’

-	HW think we can try to remove the reference to 331 but the intention is clear and we all know how SDT in inactive state works.

-	Ericsson understand this is requested by S5 and want to make sure this is from Rel-19.

-	HW understand with the proposal we reuse existing measurement as much as possible, and the wording ‘separately’ reflect that.

[AT129bis][204][NR_Others] CR for Introduction of number of UEs in RRC_INACTIVE state with data transmission (China Telecom)

Intended outcome: agreeable CR in R2-2503035

Deadline: before Friday CB

R2-2503035	Introduction of number of UEs in RRC_INACTIVE state with data transmission	China Telecom, Huawei, HiSilicon, ZTE Corporation, Sanechips, CATT, Ericsson 	CR	Rel-19	38.314	18.0.0	0034	1	B	PM_KPI_5G_Ph4

DISCUSSION

-	CT explains in offline the content is stable, but there is one company think the cat should not be B but it should be F.

-	Nokia still have concern on the CR, thinking this is not prvoding what SA5 wants, and also think this should be backward compatible. Nokia think it is not that clear when the SDT procedure start from CU UP point of view. Nokia think change to the section title is not normative.

-	HW think we already discuss and replied to SA5 that gNB knows the # of SDT UEs in RRC INACTIVE. HW think we can agree in principle and send LS back to SA5.

-	ZTE agree with HW. ZTE think it should be cat B. CATT agree.

-	CATT think some Nokia concern is only internal gNB functions so no need to reflect in the spec.

-	Ericsson think this has been discussed couple of meetings, and wonders if adding a new chapter will solve the concern.

-	Nokia think we can have two CRs, one is to change the title, and the other is to use new capture to capture what SA5 wants additionally.

-	CT and HW think the concerns are already resolved by the previous discussions and interactions with SA5 and think the current updated CR is good.

The CR is in principle agreed. Can check further whether the cat of the CR needs to be changed.

Send LS to SA5 to check if this CR solves their requirements. We request SA5 to reply it ASAP so that we can finalize the issue before end of May meeting.

[Post129bis][220][NR_Others] LS to SA5 on number of UEs in RRC_INACTIVE state with data transmission  (China Telecom)

Intended outcome: Approve the LS

Deadline:  Short

R2-2503181	Reply LS on Number of UEs in RRC_INACTIVE state with data transmission	RAN2	LS out	Rel-19	PM_KPI_5G_Ph4	To:SA5	Cc:RAN3

=> Approved

Support of 7Mhz channel bandwidth

R2-2501744	LS on Signalling for 7 MHz Channel Bandwidth (R4-2503017; contact: T-Mobile)	RAN4	LS in	Rel-19	NR_FR1_7MHz_BW-Core	To:RAN2	Cc:RAN3

Noted

R2-2501911	Discussion on signalling for 7 MHz Channel Bandwidth (LS R4-2503017)	CATT	discussion	Rel-19	NR_FR1_7MHz_BW-Core

Noted

R2-2502635	Discussion on 7MHz bandwidth capabilities	Nokia	discussion	Rel-19	NR_FR1_7MHz_BW-Core

Noted

R2-2502809	RAN2 impacts for 7 MHz Channel Bandwidth	Huawei, HiSilicon	discussion	Rel-19	NR_FR1_7MHz_BW-Core

Noted

R2-2502869	Consideration on Supporting 7M Channel Bandwidth	ZTE Corporation	discussion	Rel-19	NR_FR1_7MHz_BW-Core

Noted

DISCUSSION

Whether we wait for more R4 input

-	QC think we can wait since we in R2 are not sure whether 7M can be min or max. Xiaomi agree regarding capability of min and max bandwidth.

-	Ericsson think we can do the CR now and for now we can make the signaling as generic as possible. T-Mobile share this view.

-	HW think from the LS the other case than  a band with a minimum channel bandwidth of 3MHz or 5MHz.’ we need to wait for R4. Vivo share this view. OPPO also fine to wait.

-	QC think the WID is still ongoing in R4 so no hurry for R2. Xiaomi agree.

-	QC not sure whether this also impact R1?

-	Ericsson think if we wait then we need to have plan, e.g., whether we check with R4 on the issues?

-	T-mobile think we can send LS to R4 to ask about min BW. Xiaomi fine with this.

UE capability signalling

-	Xiaomi think we need two capabilities: the supported BW is clear and can start from R15.

-	HW think R4 indicate they do not think it critical to have 7M as min and max BW.

Which release to start from

-	QC think R19 is too late, but no strong view which earlier release is best, can think further.

-	vivo think we can do similar as 35/45M so start from Rel-19.

R2-2502569	Introduction of 7MHz channel bandwidth	Ericsson, T-Mobile	CR	Rel-17	38.306	17.12.0	1257	-	B	NR_FR1_7MHz_BW-Core, TEI17

R2-2502570	Introduction of 7MHz channel bandwidth	Ericsson, T-Mobile	CR	Rel-18	38.306	18.5.0	1258	-	A	NR_FR1_7MHz_BW-Core, TEI18

R2-2502571	Introduction of 7MHz channel bandwidth	Ericsson, T-Mobile	CR	Rel-17	38.331	17.12.0	5307	-	B	NR_FR1_7MHz_BW-Core, TEI17

R2-2502572	Introduction of 7MHz channel bandwidth	Ericsson, T-Mobile	CR	Rel-18	38.331	18.5.1	5308	-	A	NR_FR1_7MHz_BW-Core, TEI18

The CRs are postponed.

[Post129bis][205][NR_Others] Reply LS to RAN4 on 7MHz channel bandwidth (T-mobile)

Scope: Discuss questions to RAN4 (e.g., the support of 7Mhz as the minimum or maximum channel bandwidth)

Intended outcome: Approve the LS.

Deadline:  Short

R2-2503183	Reply LS on Signalling for 7 MHz Channel Bandwidth	RAN2	LS out	Rel-19	NR_FR1_7MHz_BW-Core	To:RAN4	Cc:RAN3

=> Approved (but then coversheet revised by MCC: In next RAN2 meetings clause, Bengaluru should be RAN2#131

R2-2503201	Reply LS on Signalling for 7 MHz Channel Bandwidth	RAN2	LS out	Rel-19	NR_FR1_7MHz_BW-Core	To:RAN4	Cc:RAN3

=> Approved

On CSSF optimization

R2-2501739	LS on CSSF optimization for NR RRM Phase 5 (R4-2502662; contact: Apple)	RAN4	LS in	Rel-19	NR_RRM_Ph5-Core	To:RAN2

Noted

R2-2501972	Discussion on CSSF optimization for NR RRM Phase 5	vivo	discussion	Rel-19	NR_RRM_Ph5-Core

Noted

R2-2502313	RAN2 impact on CSSF optimization	Apple, Ericsson, CATT	discussion	Rel-19	NR_RRM_Ph5-Core

Noted

DISCUSSION

-	Apple think vivo P1 is not needed.

-	ZTE suggest to send LS to R4 asking the benefit of this whole feature. ZTE think currently if NW configure multiple MOs in the same band then UE will not measure any, so not sure why NW configure so. QC share this view.

-	CATT think R2 should follow the LS and discuss signaling. CATT think with this feature there is more flexible for NW to enable configuration in the band. HW share this view.

-	HW think if the feature itself should be discussed in RAN4.

-	OPPO think we should discuss both options, a) go with R4 requst, and b) try to use existing MO configuration to achieve the same goal. OPPO think we can either wait or ask R4.

-	Ericsson think R4 is still discussing so we just wait.

-	QC think we can explain to R4 what is already possible with R2 spec, and ask R4 whether we need more.

-	Apple suggest we wait until after the next meeting, so based on new agreement from R4 we can discuss the LS to them. Apple suggest we do not challenge the motivation as the WID obj is defined by RAN.

Chair: we can discuss in the next meeting based on further progress in RAN4

Others

R2-2501742	LS on switching periods for low-low band switching (R4-2502877; contact: AT&T)	RAN4	LS in	Rel-19	NR_LBCA_Sw-Core	To:RAN1	Cc:RAN2

Noted

R2-2501760	LS on Vendor Specific Trace Record (S5-251089; contact: Ericsson)	SA5	LS in	Rel-19	TraceQoE_OAM	To:RAN3	Cc:RAN2

Noted

The following documents will be treated in other breakout sessions.

R2-2501753	Reply LS on Satellite IDs for store-and-forward operation (S2-2502450; contact: CICT Mobile, CATT)	SA2	LS in	Rel-19	5GSAT_Ph3-ARC	To:RAN2	Cc:RAN3

R2-2501718	Reply LS on Ku band numerology (R1-2501609; contact: Eutelsat)	RAN1	LS in	Rel-19	NR_NTN_Ku_bands-Core	To:RAN4	Cc:RAN, RAN2

Noted

R2-2501759	Reply LS on AI/ML UE sided data collection (S5-250828; contact: Intel, NEC, Huawei)	SA5	LS in	Rel-19	AIML_MGT_Ph2	To:RAN, RAN2	Cc:SA, SA2, SA3

# 9	Breakout session reports

No documents shall be submitted to this AI or its sub-AIs. It is only for at-meeting-generated contents.

## 9.1	Session on V2X/SL, R19 NES and MOB

R2-2502981	Report from session on V2X/SL, R19 NES and MOB	Vice Chairman (Samsung)	report

Approved

## 9.2	Session on R18 MIMO, R19 MIMO, R19 LP-WUS, and SBFD

R2-2502982	Rel-18 MIMO, Rel-19 MIMO, LPWUS, and SBFD	Vice Chairman (CATT)	report

Approved

## 9.3	Session on NR NTN and IoT NTN

R2-2502983	Report from session on NR NTN and IoT NTN	Session chair (ZTE)	report

Approved

## 9.4	Session on positioning and sidelink relay

R2-2502984	Report from session on positioning and sidelink relay	Session chair (MediaTek)	report

Approved

## 9.5	Session on R18 MBS, R18 QoE and R19 XR

R2-2502985	Report from session on R18 MBS, R18 QoE and R19 XR	Session chair (Huawei)	report

Approved

## 9.6	Session on maintenance and SON/MDT

R2-2502986	Report from session on maintenance and SON/MDT	Session chair (Ericsson)	report

Approved

# Closing of the meeting

The meeting was closed by the chair at 12:38 CST on Friday, 11th of April.

# Annex A:	List of participants

RAN2#129bis participants list is attached to this report.

Total number of participants: 385

# Annex B:	List of Tdocs

The list of tdocs from RAN2#129bis is attached to this report.

Total of 1430 tdoc numbers were allocated of which 1416 tdocs were made available.

# Annex C:	Incoming liaison statements

| TDoc | Title | Source | Status | Rel | Related WIs | To | Cc | Original LS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R2-2501704 | LS on stage 1 requirements for the support for PWS over satellite NGRAN in Rel-17 (C1-250715; contact: Qualcomm) | CT1 | noted | Rel-17 | 5GSAT | SA1 | SA2, CT4, RAN2, RAN3 | C1-250715 |
| R2-2501705 | LS to RAN2 on MAC impacts for Rel-19 NR MIMO Ph5 (R1-2500846; contact: Samsung) | RAN1 | noted | Rel-19 | NR_MIMO_Ph5 | RAN2 |  | R1-2500846 |
| R2-2501706 | LS on Rel-18 RAN1 UE features list for NR after RAN1#120 (R1-2501390; contact: NTT DOCOMO, AT&T) | RAN1 | noted | Rel-18 | NR_MIMO_evo_DL_UL, NR_pos_enh2, Netw_Energy_NR, NR_netcon_repeater, NR_NTN_enh, NR_Mob_enh2, NR_SL_enh2, NR_redcap_enh, NR_MC_enh, NR_XR_enh, NR_FR1_lessthan_5MHz_BW, NR_DSS_enh, NR_BWP_wor, NR_cov_enh2, TEI18 | RAN2 | RAN4 | R1-2501390 |
| R2-2501707 | LS on Rel-19 RAN1 UE features list for NR after RAN1#120 (R1-2501399; contact: NTT DOCOMO, AT&T) | RAN1 | noted | Rel-19 | NR_MIMO_Ph5, NR_duplex_evo, Netw_Energy_NR_enh, NR_LPWUS, NR_XR_Ph3 | RAN2 | RAN4 | R1-2501399 |
| R2-2501708 | Reply LS on co-existence of SL-CA and SL PRS transmission/reception in Dedicated SL-PRS resource pool (R1-2501479; contact: vivo) | RAN1 | noted | Rel-18 | NR_pos_enh2-Core | RAN2 |  | R1-2501479 |
| R2-2501709 | LS on activation/deactivation of semi-persistent CSI-RS resource for LTM candidate cells (R1- 2501500; contact: Fujitsu) | RAN1 | noted | Rel-19 | NR_Mob_Ph4-Core | RAN2 | RAN3 | R1- 2501500 |
| R2-2501710 | Reply LS on LMF-based AI/ML Positioning for Case 2b (R1-2501523; contact: vivo) | RAN1 | noted | Rel-19 | AIML_CN | SA2 | RAN2, RAN3 | R1-2501523 |
| R2-2501711 | Reply LS on LMF-based AI/ML Positioning for Case 3b (R1- 2501525; contact: Ericsson) | RAN1 | noted | Rel-19 | AIML_CN | SA2 | RAN2, RAN3 | R1- 2501525 |
| R2-2501712 | LS on RRC parameter for PRACH transmission in 2TA (R1-2501540; contact: CATT) | RAN1 | noted | Rel-18 | NR_MIMO_evo_DL_UL-Core | RAN2 |  | R1-2501540 |
| R2-2501713 | Reply LS on CSI-RS measurement with SBFD operation (R1-2501560; contact: MediaTek) | RAN1 | noted | Rel-19 | NR_duplex_evo-Core | RAN4 | RAN2 | R1-2501560 |
| R2-2501714 | LS on non-RedCap UE UL SRS frequency hopping for positioning (R1-2501573; contact: ZTE) | RAN1 | noted | Rel-19 | TEI19 | RAN2 |  | R1-2501573 |
| R2-2501715 | LS on L1filtering for LTM event triggered reporting (R1-2501577; contact: Fujitsu) | RAN1 | noted | Rel-19 | NR_Mob_Ph4-Core | RAN2 |  | R1-2501577 |
| R2-2501716 | LS on Slot aggregation configuration with multi-PUSCH (R1-2501593; contact: Nokia) | RAN1 | noted | Rel-17 | NR_ext_to_71GHz-Core | RAN2 |  | R1-2501593 |
| R2-2501717 | Reply LS on soft satellite switch with re-sync (R1-2501606; contact: Huawei) | RAN1 | noted | Rel-18 | NR_NTN_enh-Core | RAN2 |  | R1-2501606 |
| R2-2501718 | Reply LS on Ku band numerology (R1-2501609; contact: Eutelsat) | RAN1 | noted | Rel-19 | NR_NTN_Ku_bands-Core | RAN4 | RAN, RAN2 | R1-2501609 |
| R2-2501719 | Reply LS on differentiation of sDCI mTRP, mDCI mTRP and sTRP (R1- 2501611; contact: CATT) | RAN1 | noted | Rel-18 | NR_MIMO_evo_DL_UL-Core | RAN2 |  | R1- 2501611 |
| R2-2501720 | Reply LS on PWS support in RRC_CONNECTED for NB-IoT NTN (R1-2501613; contact: InterDigital) | RAN1 | noted | Rel-19 | IoT_NTN_Ph3-Core | RAN2, RAN |  | R1-2501613 |
| R2-2501721 | LS on AI/ML Positioning Case 3b (R1- 2501628; contact: Ericsson) | RAN1 | noted | Rel-19 | NR_AIML_air | RAN3, RAN2 |  | R1- 2501628 |
| R2-2501722 | LS on time location of on-demand SSB for Scell (R1-2501633; contact: LGE) | RAN1 | noted | Rel-19 | Netw_Energy_NR_enh | RAN4 | RAN2 | R1-2501633 |
| R2-2501723 | Reply LS on UL 8Tx (R1-2501636; contact: Samsung) | RAN1 | noted | Rel-18 | NR_MIMO_evo_DL_UL-Core | RAN2 |  | R1-2501636 |
| R2-2501724 | LS on Rel-19 higher layers parameters list Post RAN1#120 (R1-2501644; contact: Ericsson) | RAN1 | noted | Rel-19 | NR_MIMO_Ph5, NR_duplex_evo, Netw_Energy_NR_enh, NR_LPWUS, NR_XR_Ph3 | RAN2, RAN3 | RAN4 | R1-2501644 |
| R2-2501725 | Reply LS on Support of UE move between CAG cell of 5G Femto and CSG cell (R3-244830; contact: Ericsson) | RAN3 | noted | Rel-19 | FS_NR_WAB_5GFemto | SA2 | RAN2 | R3-244830 |
| R2-2501726 | Reply LS on UE Location Information for NB-IoT NTN (R3-250761; contact Qualcomm) | RAN3 | noted | Rel-18 | IoT_NTN_enh-Core | SA2 | RAN2, CT1 | R3-250761 |
| R2-2501727 | Reply LS on inter-gNB RACH-less HO in NTN (R3-250762; contact: Nokia) | RAN3 | noted | Rel-18 | NR_NTN_enh-Core | RAN2 | RAN1 | R3-250762 |
| R2-2501728 | Reply LS on LMF-based AI/ML Positioning for case 3b (R3-250796; contact: ZTE) | RAN3 | noted | Rel-19 | AIML_CN, NR_AIML_air, NR_AIML_air-Core | SA2 | RAN1, RAN2 | R3-250796 |
| R2-2501729 | Reply LS on MBS Communication Service Type (R3-250858; contact: ZTE) | RAN3 | noted | Rel-18 | NR_QoE_enh-Core | SA4 | RAN2, SA5 | R3-250858 |
| R2-2501730 | Reply LS on L3 measurement based LTM support over F1 (R3-250879; contact: Vodafone) | RAN3 | noted | Rel-18 | NR_Mob_enh2-Core | RAN2 | RAN4, RAN | R3-250879 |
| R2-2501731 | SBFD information exchange among gNBs for CLI mitigation (R3-250888; contact: Huawei) | RAN3 | noted | Rel-19 | NR_duplex_evo-Core | RAN2 | RAN1 | R3-250888 |
| R2-2501732 | Reply LS on FS_VMR_Ph2 solution impacts to RAN (Additional ULI) (R3-250901; contact: ZTE) | RAN3 | noted | Rel-19 | FS_VMR_Ph2, NR_WAB_5GFemto-Core | SA2 | RAN2 | R3-250901 |
| R2-2501733 | Reply LS on Support of Location Service Involving WAB-Nodes (R3-250902; contact: Huawei) | RAN3 | noted | Rel-19 | VMR_Ph2, NR_WAB_5GFemto-Core | SA2 | RAN2 | R3-250902 |
| R2-2501734 | LS on SON for Network Slicing (R3-250914; contact: ZTE) | RAN3 | noted | Rel-19 | NR_ENDC_SON_MDT_Ph4-Core | RAN2 |  | R3-250914 |
| R2-2501735 | LS on AdditionalSpectrumEmission in NR SL Pre-configuration (R4-2418075; contact: LGE) | RAN4 | noted | Rel-18 | NR_SL_enh2-Core | RAN2 | RAN1 | R4-2418075 |
| R2-2501736 | Reply LS on the fast RRC processing for LTM (R4-2420218; contact: Ericsson) | RAN4 | noted | Rel-18 | NR_Mob_enh2-Core | RAN2 |  | R4-2420218 |
| R2-2501737 | LS on UE capability for L1-RSRP measurement in LTM (R4-2502619; contact: vivo) | RAN4 | noted | Rel-18 | NR_Mob_enh2-Core | RAN2 |  | R4-2502619 |
| R2-2501738 | LS on L1 CLI measurement (R4-2502632; contact: Huawei) | RAN4 | noted | Rel-19 | NR_duplex_evo-Core | RAN1 | RAN2 | R4-2502632 |
| R2-2501739 | LS on CSSF optimization for NR RRM Phase 5 (R4-2502662; contact: Apple) | RAN4 | noted | Rel-19 | NR_RRM_Ph5-Core | RAN2 |  | R4-2502662 |
| R2-2501740 | LS on neighboring cell measurement for Rel-18 SSB-less (R4-2502694; contact: ZTE) | RAN4 | noted | Rel-18 | Netw_Energy_NR | RAN2 |  | R4-2502694 |
| R2-2501741 | reply LS on soft satellite switch with re-sync (R4-2502701; contact: Huawei) | RAN4 | noted | Rel-18 | NR_NTN_enh-Core | RAN2, RAN1 |  | R4-2502701 |
| R2-2501742 | LS on switching periods for low-low band switching (R4-2502877; contact: AT&T) | RAN4 | noted | Rel-19 | NR_LBCA_Sw-Core | RAN1 | RAN2 | R4-2502877 |
| R2-2501743 | LS on LP-WUS UE RF (R4-2503003; contact: vivo) | RAN4 | noted | Rel-19 | NR_LPWUS-Core | RAN1 | RAN2 | R4-2503003 |
| R2-2501744 | LS on Signalling for 7 MHz Channel Bandwidth (R4-2503017; contact: T-Mobile) | RAN4 | noted | Rel-19 | NR_FR1_7MHz_BW-Core | RAN2 | RAN3 | R4-2503017 |
| R2-2501745 | LS on clarification for intraBandNR-CA-non-collocated-r18 and interBandMRDC-WithOverlapDL-Bands-r16 (R4-2503033; contact: Apple, Huawei) | RAN4 | noted | Rel-18 | NonCol_intraB_ENDC_NR_CA | RAN2 |  | R4-2503033 |
| R2-2501746 | LS on NR NB-IoT in-band operation (R4-2503035; contact: Ericsson) | RAN4 | noted | Rel-19 | TEI18 | RAN2 | RAN1 | R4-2503035 |
| R2-2501747 | Reply LS to RAN2 on UE capabilities for FR2-NTN (R4-2503039; contact: vivo) | RAN4 | noted | Rel-18 | NR_NTN_enh-Core | RAN2 | RAN1 | R4-2503039 |
| R2-2501748 | Reply LS on L3 measurement based LTM support over F1 (RP-250790; contact: Vodafone) | RAN | noted | Rel-18 | NR_Mob_enh2-Core | RAN3 | RAN2, RAN4 | RP-250790 |
| R2-2501749 | Reply LS on Clarification regarding definition of 5G NR femto ownership (S2-2411241; contact: LGE) | SA2 | noted | Rel-19 | FS_5G_Femto_Sec, 5G_Femto | SA3 | RAN2, RAN3 | S2-2411241 |
| R2-2501750 | Reply LS on FS_VMR_Ph2 solution impacts to RAN (Access Control and Additional ULI) (S2-2501324; contact: Ericsson) | SA2 | noted | Rel-19 | FS_VMR_Ph2, VMR_Ph2 | RAN3 | RAN2 | S2-2501324 |
| R2-2501751 | Reply LS on FS_VMR_Ph2 solution impacts to RAN (MWAB mobility) (S2-2501336; contact: Nokia) | SA2 | noted | Rel-19 | VMR_Ph2 | RAN3 | RAN2 | S2-2501336 |
| R2-2501752 | Reply LS on emergency call back and paging (S2-2502427; contact: ZTE) | SA2 | noted | Rel-17 | NR_newRAT-Core, NR_redcap-Core | RAN2 | CT1, RAN3 | S2-2502427 |
| R2-2501753 | Reply LS on Satellite IDs for store-and-forward operation (S2-2502450; contact: CICT Mobile, CATT) | SA2 | noted | Rel-19 | 5GSAT_Ph3-ARC | RAN2 | RAN3 | S2-2502450 |
| R2-2501754 | LS reply on multi-modality awareness (S2-2502465; contact: CMCC) | SA2 | noted | Rel-19 | XRM_Ph2, NR_XR_Ph3-Core | RAN2, RAN3, SA | SA4 | S2-2502465 |
| R2-2501755 | LS on NR Femto node shared by PLMN and PNI-NPN (S2-2502787; contact: LGE) | SA2 | noted | Rel-19 | 5G_Femto, NR_WAB_5GFemto-Core, eNPN | RAN3 | RAN2 | S2-2502787 |
| R2-2501756 | Reply LS on security handling for inter-CU LTM in non-DC cases (S3-251124; contact: CATT) | SA3 | noted | Rel-19 | NR_Mob_Ph4-Core | RAN2, RAN3 |  | S3-251124 |
| R2-2501757 | Reply to LS on Application-Layer FEC Awareness at RAN (S4-250252; contact: Qualcomm) | SA4 | noted | Rel-19 | NR_XR_Ph3-Core, FS_5G_RTP_Ph2, FS_XRM_Ph2 | RAN2, SA2 | RAN3 | S4-250252 |
| R2-2501758 | Reply to RAN2 LS on Number of UEs in RRC_INACTIVE state with data transmission (S5-250827; contact: China Telecom) | SA5 | noted | Rel-19 | PM_KPI_5G_Ph4 | RAN2 | RAN3 | S5-250827 |
| R2-2501759 | Reply LS on AI/ML UE sided data collection (S5-250828; contact: Intel, NEC, Huawei) | SA5 | noted | Rel-19 | AIML_MGT_Ph2 | RAN, RAN2 | SA, SA2, SA3 | S5-250828 |
| R2-2501760 | LS on Vendor Specific Trace Record (S5-251089; contact: Ericsson) | SA5 | noted | Rel-19 | TraceQoE_OAM | RAN3 | RAN2 | S5-251089 |
| R2-2503161 | LS on DCI-based PRACH adaptation (R1-2503086; contact: Ericsson) | RAN1 | available | Rel-19 | Netw_Energy_NR_enh | RAN2 |  | R1-2503086 |

58 incoming LS, of which 57 LS were noted. The remaining non-treated LSin will be treated in RAN2#130.

# Annex D:	Outgoing liaison statements

| TDoc | Title | Rel | Related WIs | To | Cc |
| --- | --- | --- | --- | --- | --- |
| R2-2503012 | LS on RAN2 agreements for security key handling in inter-CU LTM | Rel-19 | NR_Mob_Ph4-Core | RAN3 |  |
| R2-2503019 | LS on on-demand SSB | Rel-19 | Netw_Energy_NR_enh-Core | RAN4 | RAN1 |
| R2-2503068 | LS on SMTC enhancements | Rel-19 | NR_NTN_Ph3-Core | RAN4 |  |
| R2-2503069 | Reply LS on soft satellite switch with re-sync | Rel-18 | NR_NTN_enh-Core | RAN1, RAN4 |  |
| R2-2503070 | LS on S&F mode indications to NAS | Rel-19 | IoT_NTN_Ph3-Core | CT1 |  |
| R2-2503087 | LS on the startSFN parameter for positioning SRS with TX frequency hopping | Rel-18 | NR_pos_enh2-Core | RAN1 |  |
| R2-2503163 | LS on Collision between MUSIM Gap and Measurement Gap | Rel-18 | NR_DualTxRx_MUSIM-Core | RAN4 |  |
| R2-2503168 | Reply LS on AI/ML UE sided data collection | Rel-19 | AIML_MGT_Ph2 | SA5 | SA, SA2, SA3, RAN |
| R2-2503169 | Reply LS on signalling feasibility of dataset and parameter sharing | Rel-19 | NR_AIML_air-Core | RAN1, RAN3, SA2, SA5 | SA3 |
| R2-2503170 | LS on OAM-centric solution for NW-side data collection | Rel-19 | NR_AIML_air-Core | SA5, RAN3 |  |
| R2-2503172 | Reply LS on paging capability loss issue | Rel-19 | NR_LPWUS-Core, TEI19 | SA2, RAN3, CT1 |  |
| R2-2503175 | LS on CB-msg3-EDT | Rel-19 | IoT_NTN_Ph3-Core | RAN1 |  |
| R2-2503180 | LS on Rel-19 higher layer parameter and UE Feature | Rel-19 |  | RAN1, RAN4 |  |
| R2-2503181 | Reply LS on Number of UEs in RRC_INACTIVE state with data transmission | Rel-19 | PM_KPI_5G_Ph4 | SA5 | RAN3 |
| R2-2503182 | LS on LP-WUS subgrouping progress | Rel-19 | NR_LPWUS-Core | RAN3 | SA2, SA3 |
| R2-2503185 | Reply LS on non-RedCap UE UL SRS frequency hopping for positioning | Rel-19 | TEI19 | RAN1, RAN3 |  |
| R2-2503191 | LS on simultaneous configuration of SBFD and DC | Rel-19 | NR_duplex_evo-Core | RAN1 | RAN3, RAN4 |
| R2-2503197 | LS on paging ID length | Rel-19 | Ambient_IoT_Solutions | SA2, CT4 | RAN3 |
| R2-2503198 | LS on byte-alignment of upper layer data SDU | Rel-19 | Ambient_IoT_Solutions | CT1 | RAN3, SA2 |
| R2-2503199 | LS on D2R message size for inventory response | Rel-19 | Ambient_IoT_Solutions | SA2, RAN3 |  |
| R2-2503200 | Reply LS on NR NB-IoT in-band operation | Rel-18 | TEI18 | RAN4 | RAN1 |
| R2-2503201 | Reply LS on Signalling for 7 MHz Channel Bandwidth | Rel-19 | NR_FR1_7MHz_BW-Core | RAN4 | RAN3 |
| R2-2503202 | LS on LP-WUS in RRC_CONNECTED | Rel-19 | NR_LPWUS-Core | RAN1 |  |
| R2-2503203 | Relay LS on MDT for NTN | Rel-19 | NR_ENDC_SON_MDT_Ph4-Core | RAN3 |  |

24 outgoing LS.

# Annex E:	List of agreed in principle CRs

| TDoc | Title | Source | Rel | Spec | Related WIs | CR | Rev | Cat |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R2-2501782 | Correction on NTN in FR2-NTN bands | vivo, ZTE Corporation, Ericsson, Sanechips | Rel-18 | 38.306 | NR_NTN_enh-Core | 1200 | 1 | F |
| R2-2501793 | Correction on condition-based IUC for shared SL-PRS resource pool | vivo, Ericsson | Rel-18 | 38.331 | NR_pos_enh2-Core |  |  | F |
| R2-2501896 | Missing rate of change direction of azimuth/elevation for relative velocity | Xiaomi, Qualcomm, CATT, Huawei, MediaTek Inc. | Rel-18 | 38.355 | NR_pos_enh2-Core | 0015 |  | F |
| R2-2502062 | MAC correction on Release-18 Sidelink evolution | LG Electronics Inc. | Rel-18 | 38.321 | NR_SL_enh2-Core | 2057 |  | F |
| R2-2502090 | Rapporteur CR for RACH-less HO and LTM [RACH-lessHO] | Huawei, HiSilicon, CATT, Nokia, Qualcomm, Samsung | Rel-18 | 38.321 | TEI18, NR_Mob_enh2-Core, NR_NTN_enh-Core, NR_mobile_IAB-Core | 2059 |  | F |
| R2-2502185 | Correction on MCSt Restriction for Resource Selection | Apple | Rel-18 | 38.321 | NR_SL_enh2-Core | 2060 |  | F |
| R2-2502271 | Correction to nrofCandidates-CI for DCI format 2_4 | Huawei, HiSilicon | Rel-16 | 38.331 | NR_L1enh_URLLC-Core | 5295 |  | F |
| R2-2502272 | Correction to nrofCandidates-CI for DCI format 2_4 | Huawei, HiSilicon | Rel-17 | 38.331 | NR_L1enh_URLLC-Core | 5296 |  | A |
| R2-2502273 | Correction to nrofCandidates-CI for DCI format 2_4 | Huawei, HiSilicon | Rel-18 | 38.331 | NR_L1enh_URLLC-Core | 5297 |  | A |
| R2-2502335 | Correction on SCS and CP configuration in RedCap-specific initial BWP | ZTE Corporation, Sanechips, Vivo, Ericsson, MediaTek Inc, Qualcomm Incorporated, OPPO, Samsung | Rel-17 | 38.331 | NR_redcap-Core | 5212 | 1 | F |
| R2-2502336 | Correction on SCS and CP configuration in RedCap-specific initial BWP | ZTE Corporation, Sanechips, Vivo, Ericsson, MediaTek Inc, Qualcomm Incorporated, OPPO, Samsung | Rel-18 | 38.331 | NR_redcap-Core | 5213 | 1 | A |
| R2-2502344 | Correction on number of NZP CSI-RS resources | Ericsson | Rel-15 | 38.331 | NR_newRAT-Core | 5298 |  | F |
| R2-2502389 | Correction on number of NZP CSI-RS resources | Ericsson | Rel-16 | 38.331 | NR_newRAT-Core | 5299 |  | A |
| R2-2502390 | Correction on number of NZP CSI-RS resources | Ericsson | Rel-17 | 38.331 | NR_newRAT-Core | 5300 |  | A |
| R2-2502391 | Correction on number of NZP CSI-RS resources | Ericsson | Rel-18 | 38.331 | NR_newRAT-Core | 5301 |  | A |
| R2-2502404 | Correction to Immediate MDT measurements | Nokia | Rel-16 | 37.320 | NR_SON_MDT-Core | 0137 |  | F |
| R2-2502405 | Correction to Immediate MDT measurements | Nokia | Rel-17 | 37.320 | NR_SON_MDT-Core | 0138 |  | A |
| R2-2502406 | Correction to Immediate MDT measurements | Nokia | Rel-18 | 37.320 | NR_SON_MDT-Core | 0139 |  | A |
| R2-2502407 | Updating references to TS 28.558 from TS 28.552 | Nokia | Rel-18 | 37.320 | NR_ENDC_SON_MDT_enh2-Core | 0140 |  | F |
| R2-2502441 | Correction on cell type specific cell reselection prioritisation [NR_HSDN] | Huawei, HiSilicon, Nokia, Qualcomm, CMCC, Xiaomi, CATT, Apple | Rel-17 | 38.300 | TEI17, NR_mobile_IAB-Core | 0981 |  | F |
| R2-2502442 | Correction on cell type specific cell reselection prioritisation [NR_HSDN] | Huawei, HiSilicon, Nokia, Qualcomm, CMCC, Xiaomi, CATT, Apple | Rel-18 | 38.300 | TEI17, NR_mobile_IAB-Core | 0982 |  | A |
| R2-2502473 | Corrections on DCP | Huawei, HiSilicon, Nokia (Rapporteur) | Rel-16 | 38.300 | NR_UE_pow_sav-Core | 0983 |  | F |
| R2-2502474 | Corrections on DCP | Huawei, HiSilicon, Nokia (Rapporteur) | Rel-17 | 38.300 | NR_UE_pow_sav-Core | 0984 |  | A |
| R2-2502475 | Corrections on DCP | Huawei, HiSilicon, Nokia (Rapporteur) | Rel-18 | 38.300 | NR_UE_pow_sav-Core | 0985 |  | A |
| R2-2502476 | Correction on UE capability for SIB17bis | Huawei, HiSilicon | Rel-18 | 38.306 | NR_UE_pow_sav_enh-Core, TEI18 | 1250 |  | F |
| R2-2502505 | Type clarification for interBandMRDC-WithOverlapDL-Bands-r16 | Apple, Huawei, HiSilicon, xiaomi | Rel-16 | 38.306 | NR_newRAT-Core, TEI16 | 1251 |  | F |
| R2-2502506 | Type clarification for interBandMRDC-WithOverlapDL-Bands-r16 | Apple, Huawei, HiSilicon, xiaomi | Rel-17 | 38.306 | NR_newRAT-Core, TEI16 | 1252 |  | A |
| R2-2502507 | Type clarification for interBandMRDC-WithOverlapDL-Bands-r16 | Apple, Huawei, HiSilicon, xiaomi | Rel-18 | 38.306 | NR_newRAT-Core, TEI16 | 1253 |  | A |
| R2-2502508 | Type clarification for intraBandNR-CA-non-collocated-r18 | Apple, Huawei, HiSilicon, xiaomi | Rel-18 | 38.306 | NonCol_intraB_ENDC_NR_CA, TEI18 | 1254 |  | F |
| R2-2502519 | Corrections to location-based measurement | ZTE Corporation, Ericsson, CATT, Sanechips | Rel-17 | 38.331 | NR_NTN_solutions-Core | 5303 |  | F |
| R2-2502520 | Corrections to location-based measurement initiation | ZTE Corporation, Ericsson, CATT, Sanechips | Rel-17 | 38.306 | NR_NTN_solutions-Core | 1255 |  | F |
| R2-2502521 | Corrections to location-based measurement initiation | ZTE Corporation, Ericsson, CATT, Sanechips | Rel-18 | 38.306 | NR_NTN_solutions-Core | 1256 |  | A |
| R2-2502530 | Correction on timeSinceSHR in SHR report | ZTE Corporation, Sanechips | Rel-18 | 38.331 | NR_ENDC_SON_MDT_enh2-Core | 5304 |  | F |
| R2-2502614 | Correction on IAB related MAC CEs | ZTE Corporation, Samsung, Sanechips | Rel-17 | 38.321 | NR_IAB_enh-Core | 2066 |  | F |
| R2-2502631 | Correction to pdsch-256QAM-FR1 for IAB-MT | Nokia | Rel-16 | 38.306 | NR_IAB-Core | 1259 |  | F |
| R2-2502632 | Correction to pdsch-256QAM-FR1 for IAB-MT | Nokia | Rel-17 | 38.306 | NR_IAB-Core | 1260 |  | A |
| R2-2502633 | Correction to pdsch-256QAM-FR1 for IAB-MT | Nokia | Rel-18 | 38.306 | NR_IAB-Core | 1261 |  | A |
| R2-2502669 | Correction to Rel-18 NR NTN CHO with only location/time-based trigger | Samsung | Rel-18 | 38.331 | NR_NTN_enh-Core | 5314 |  | F |
| R2-2502670 | Correction to Rel-18 NR NTN CHO with only location/time-based trigger | Samsung | Rel-18 | 38.306 | NR_NTN_enh-Core | 1263 |  | F |
| R2-2502784 | Introduction of ANR reporting of HSDN cells [ANR_HSDN] | Huawei, HiSilicon, CMCC, China Unicom, China Telecom, CATT, NTT DoCoMo, Samsung | Rel-19 | 38.306 | TEI19 | 1264 |  | B |
| R2-2502786 | Introduction of ANR reporting of HSDN cells [ANR_HSDN] | Huawei, HiSilicon, CMCC, China Unicom, China Telecom, CATT, NTT DoCoMo, Samsung | Rel-19 | 36.306 | TEI19 | 1911 |  | B |
| R2-2502798 | Correction on SON for NPN in TS 38.300 | Huawei, HiSilicon, CATT, Samsung, ZTE, CMCC | Rel-18 | 38.300 | NR_ENDC_SON_MDT_enh2-Core | 0986 |  | F |
| R2-2502837 | Correction to ul-GapFR2-r17 | Huawei, HiSilicon, Apple, Nokia, Nokia Shanghai Bell, Ericsson, MediaTek Inc. | Rel-17 | 38.306 | NR_RF_FR2_req_enh2 | 1265 |  | F |
| R2-2502838 | Correction to ul-GapFR2-r17 | Huawei, HiSilicon, Apple, Nokia, Nokia Shanghai Bell, Ericsson, MediaTek Inc. | Rel-18 | 38.306 | NR_RF_FR2_req_enh2 | 1266 |  | A |
| R2-2502844 | Correction on logging intendedSIBs | Samsung | Rel-18 | 38.331 | NR_ENDC_SON_MDT_enh2-Core | 5322 |  | F |
| R2-2502920 | Slot aggregation configuration with multi-PUSCH | Nokia | Rel-17 | 38.331 | NR_ext_to_71GHz-Core | 5328 |  | F |
| R2-2502921 | Slot aggregation configuration with multi-PUSCH | Nokia | Rel-18 | 38.331 | NR_ext_to_71GHz-Core | 5329 |  | A |
| R2-2502996 | Introduction of ANR reporting of HSDN cells [ANR_HSDN] | Huawei, HiSilicon, CMCC, China Unicom, China Telecom, CATT, NTT DoCoMo, Samsung | Rel-19 | 38.331 | TEI19 | 5318 | 1 | B |
| R2-2502997 | Introduction of ANR reporting of HSDN cells [ANR_HSDN] | Huawei, HiSilicon, CMCC, China Unicom, China Telecom, CATT, NTT DoCoMo, Samsung | Rel-19 | 36.331 | TEI19 | 5110 | 1 | B |
| R2-2502998 | Correction for cell barring for NES CellDTX-DRX UEs | Huawei, HiSilicon | Rel-18 | 38.331 | Netw_Energy_NR-Core | 5284 | 1 | F |
| R2-2502999 | Correction to UE capability for retx-less CG | Huawei, HiSilicon, Apple, Futurewei, Qualcomm | Rel-18 | 38.306 | NR_XR_enh-Core | 1272 |  | F |
| R2-2503000 | Correction to RRC spec for retx-less CG | Huawei, HiSilicon, Apple, Futurewei, Qualcomm | Rel-18 | 38.331 | NR_XR_enh-Core | 5333 |  | F |
| R2-2503008 | Handling of the default SRB configuration upon LTM execution | CATT | Rel-18 | 38.331 | NR_Mob_enh2-Core | 5285 | 1 | F |
| R2-2503009 | Handling of PUCCH resources for L1 LTM reports at TAT expiry | Huawei, HiSilicon | Rel-18 | 38.331 | NR_Mob_enh2-Core | 5332 | 2 | F |
| R2-2503010 | Correction on sidelink RRC specification | ZTE Corporation, Sanechips, Ericsson | Rel-18 | 38.331 | NR_SL_enh2-Core | 5330 | 1 | F |
| R2-2503016 | Correction on UE capability for L1-RSRP measurement in LTM | vivo, Ericsson, Xiaomi | Rel-18 | 38.306 | NR_Mob_enh2-Core | 1249 | 1 | F |
| R2-2503018 | Capability coordination for LTM | Ericsson | Rel-18 | 38.331 | NR_Mob_enh2-Core | 5305 | 1 | F |
| R2-2503020 | Corrections on eEMR/IMR and LTM L2 Reset/UE-based TA measurement | Samsung | Rel-18 | 38.331 | NR_Mob_enh2-Core | 5334 | 1 | F |
| R2-2503021 | Correction on the maximum number of SSB rsources for L1 maeasurement without gaps in LTM | vivo, Ericsson, Xiaomi | Rel-18 | 38.331 | NR_Mob_enh2-Core | 5302 | 2 | F |
| R2-2503034 | Correction on supporting 8Tx in MAC specification | ASUSTeK, Samsung, ZTE | Rel-18 | 38.321 | NR_MIMO_evo_DL_UL-Core | 1990 | 3 | F |
| R2-2503035 | Introduction of number of UEs in RRC_INACTIVE state with data transmission [KPI_SDT_SA5] | China Telecom, Huawei, HiSilicon, ZTE Corporation, Sanechips, CATT | Rel-19 | 38.314 | TEI19 | 0034 | 1 | B |
| R2-2503061 | Correction on SIB33 reception in RRC_CONNECTED | Huawei, HiSilicon | Rel-18 | 36.331 | IoT_NTN_enh-Core | 5105 | 1 | F |
| R2-2503062 | Corrections to location-based measurement | ZTE Corporation, Ericsson, CATT, Sanechips | Rel-18 | 36.300 | IoT_NTN_enh-Core | 1417 | 1 | F |
| R2-2503066 | Clarification on ssb-TimeOffset | Huawei | Rel-18 | 38.331 | NR_NTN_enh-Core | 5335 | 1 | F |
| R2-2503079 | Correction on NW restriction for dedicated SL-PRS resource pool | vivo, Ericsson | Rel-18 | 38.331 | NR_pos_enh2-Core | 5104 | 2 | F |
| R2-2503080 | Corrections on the descriptions of nr-PeriodicOrOneShotTimeWindow and nr-StartSFN-TimeWindow | CATT, Qualcomm Incorporated | Rel-18 | 37.355 | NR_pos_enh2-Core | 0548 | 1 | F |
| R2-2503081 | Correction to PC5 RLC Channel Release | Apple, ASUSTek | Rel-18 | 38.331 | NR_SL_relay_enh-Core | 5292 | 1 | F |
| R2-2503084 | Correction on sidelink relay RRC specification | ZTE Corporation, Sanechips | Rel-18 | 38.331 | NR_SL_relay_enh-Core | 5331 | 1 | F |
| R2-2503102 | Correction on timeUntilReconnection in RLF report | ZTE Corporation, Sanechips | Rel-17 | 36.331 | NR_ENDC_SON_MDT_enh-Core | 5108 | 1 | F |
| R2-2503103 | Correction on timeUntilReconnection in RLF report | ZTE Corporation, Sanechips | Rel-18 | 36.331 | NR_ENDC_SON_MDT_enh-Core | 5109 | 1 | A |
| R2-2503104 | Correction on SRS capability reporting | Huawei, HiSilicon, Ericsson, OPPO | Rel-15 | 38.331 | NR_newRAT-Core | 5324 | 1 | F |
| R2-2503105 | Correction on SRS capability reporting | Huawei, HiSilicon, Ericsson, OPPO | Rel-16 | 38.331 | NR_newRAT-Core | 5325 | 1 | A |
| R2-2503106 | Correction on SRS capability reporting | Huawei, HiSilicon, Ericsson, OPPO | Rel-17 | 38.331 | NR_newRAT-Core | 5326 | 1 | A |
| R2-2503107 | Correction on SRS capability reporting | Huawei, HiSilicon, Ericsson, OPPO | Rel-18 | 38.331 | NR_newRAT-Core | 5327 | 1 | A |
| R2-2503108 | Correction on SRS capability reporting | Huawei, HiSilicon, Ericsson, OPPO | Rel-15 | 38.306 | NR_newRAT-Core | 1268 | 1 | F |
| R2-2503109 | Correction on SRS capability reporting | Huawei, HiSilicon, Ericsson, OPPO | Rel-16 | 38.306 | NR_newRAT-Core | 1269 | 1 | A |
| R2-2503110 | Correction on SRS capability reporting | Huawei, HiSilicon, Ericsson, OPPO | Rel-17 | 38.306 | NR_newRAT-Core | 1270 | 1 | A |
| R2-2503111 | Correction on SRS capability reporting | Huawei, HiSilicon, Ericsson, OPPO | Rel-18 | 38.306 | NR_newRAT-Core | 1271 | 1 | A |
| R2-2503112 | CR on the usage of msg3-transmitPrecoder | CATT, Huawei, HiSilicon | Rel-17 | 38.331 | NR_SmallData_INACTIVE-Core, NR_cov_enh-Core, NR_redcap-Core, NR_slice-Core | 5286 | 1 | F |
| R2-2503113 | CR on the usage of msg3-transmitPrecoder | CATT, Huawei, HiSilicon | Rel-18 | 38.331 | NR_SmallData_INACTIVE-Core, NR_cov_enh-Core, NR_redcap-Core, NR_slice-Core | 5287 | 1 | A |
| R2-2503116 | Correction on timeUntilReconnection in RLF report | ZTE Corporation, Xiaomi, Sanechips | Rel-16 | 36.331 | NR_SON_MDT-Core | 5113 |  | F |
| R2-2503164 | Adding enabledDefaultBeamFormultiCellScheduling | Huawei, HiSilicon, NTT DOCOMO INC., Samsung, Ericsson | Rel-18 | 38.331 | NR_MC_enh-Core | 5294 | 1 | F |
| R2-2503167 | Correction on RACH-less HO Use Case | vivo, Nokia, Nokia Shanghai Bell, Ericsson, Samsung | Rel-18 | 38.300 | NR_NTN_enh-Core, NR_mobile_IAB-Core, TEI18 | 0904 | 3 | F |
| R2-2503174 | Clarification on capability for inter-frequency configuration for less than 5MHz | Huawei, HiSilicon, Xiaomi, Samsung, Qualcomm Incorporated | Rel-18 | 38.306 | NR_FR1_lessthan_5MHz_BW | 1196 | 3 | F |
| R2-2503184 | Miscellaneous non-controversial corrections Set XXV | Ericsson | Rel-18 | 38.331 | NR_newRAT-Core, TEI18 | 5310 | 1 | F |
| R2-2503188 | Corrections to TS 38.300 on multi-path relay enhancement [N3C_M_Relay] | CMCC, ZTE, MediaTek, vivo, Huawei, CATT, Meta, Nokia, Nokia Shanghai Bell, Xiaomi, Spreadtrum, UNISOC | Rel-19 | 38.300 | TEI19 |  |  | B |
| R2-2503189 | Corrections to TS 38.331 on SL Relay enhancement [N3C_M_Relay] | CMCC, ZTE, MediaTek, vivo, Huawei, CATT, Meta, Nokia, Nokia Shanghai Bell, Xiaomi, Spreadtrum, UNISOC | Rel-19 | 38.331 | TEI19 |  |  | B |
| R2-2503190 | Corrections to TS 38.306 on multi-path relay enhancement [N3C_M_Relay] | CMCC, ZTE, MediaTek, vivo, Huawei, CATT, Meta, Nokia, Nokia Shanghai Bell, Xiaomi, Spreadtrum, UNISOC | Rel-19 | 38.306 | TEI19 |  |  | B |
| R2-2503192 | Introduction of control parameters for on-demand posSIB request [PosOdSIB-Req] | Huawei, HiSilicon, Ericsson | Rel-19 | 38.331 | TEI19 | 5288 | 2 | B |
| R2-2503193 | Introduction of control parameters for on-demand posSIB request [PosOdSIB-Req] | Huawei, HiSilicon, Ericsson | Rel-19 | 38.306 | TEI19 | 1248 | 2 | B |
| R2-2503194 | Introduction of control parameters for on-demand posSIB request [PosOdSIB-Req] | Huawei, HiSilicon, Ericsson | Rel-19 | 38.305 | TEI19 | 0184 | 2 | B |
| R2-2503195 | Introduction of control parameters for on-demand posSIB request [PosOdSIB-Req] | Huawei, HiSilicon, Ericsson | Rel-19 | 38.300 | TEI19 | 0978 | 2 | B |

89 Agreed in principle CRs.

# Annex F:	List of email discussions during the meeting

Discussions during the RAN2#129bis meeting:

List of AT meeting email discussions

[AT129bis][002][ASN.1 review]

[AT129bis][003][UE caps] LS to RAN1RAN4 (Xiaomi)

[AT129bis][004][RACH-less] HO use case CR (vivo)

[AT129bis][005][MUSIM] (ZTE)

[AT129bis][006][XR] Re-tx less CG (Huawei)

[AT129bis][007][less5MHz] CR (Huawei)

[AT129bis][008][NR MC] CR (Huawei)

[AT129bis][009][UE cap] Reply LS to SA2 (Huawei)

[AT129bis][010][AIoT] Paging (Qualcomm)

[AT129bis][011][AI PHY] LS reply to RAN1 (Xiaomi)

[AT129bis][012][AI PHY] Reply LS to SA5 (Huawei)

[AT129bis][101][V2XSL] SL resource (re)selection in MCSt (LG

[AT129bis][102][MOB] (Ericsson)

[AT129bis][103][MOB] (Huawei)

[AT129bis][104][MOB] MAC open issues (vivo)

[AT129bis][105][NES] FFSs to 38.331 CR for NES (Ericsson)

[AT129bis][106][NES] (Apple) Open issues of runinng 38.304 CR

[AT129bis][107][NES] MAC open issues (IDC)

[AT129bis][108][MOB] (CATT)

[AT129bis][109][MOB] LS to RAN3 (ZTE)

[AT129bis][110][MOB] (Ericsson)

[AT129bis][201][LPWUS] Proposals for UE ID based sub-grouping (CATT)

[AT129bis][202][LPWUS] Proposals for RRM measurement relaxation (vivo)

[AT129bis][204][NR_Others] CR for number of UEs in RRC_INACTIVE (China Telecom)

[AT129bis][301][R18 NR NTN] Soft satellite switch with re-sync (Huawei)

[AT129bis][302][TEI18] In-band operation for NB-IoT (Ericsson)

[AT129bis][303][R19 NR NTN] SMTC enhancements (Xiaomi)

[AT129bis][304][R19 NR NTN] Service continuity (Apple)

[AT129bis][305][R19 IoT NTN] CB-msg3-EDT (Mediatek)

[AT129bis][30x][R19 IoT NTN] Single vs. Multiple (MediaTek)

[AT129bis][401][Relay] Rel-19 relay service continuity CR to 38.331 (CATT)

[AT129bis][402][Relay] Rel-19 relay CR to 38.304 (MediaTek)

[AT129bis][403][Relay] Rel-19 relay CR to 38.323 (Ericsson)

[AT129bis][404][Relay] Rel-19 relay CR to 38.321 (InterDigital)

[AT129bis][406][Relay] Rel-19 relay main CR to 38.331 (Huawei)

[AT129bis][407][Relay] Rel-19 relay CR to 38.300 (LG)

[AT129bis][408][Relay] Rel-19 relay CR to 38.351 (OPPO)

[AT129bis][409][Relay] Fast parallel RRC forwarding for approach 1 (Apple)

[AT129bis][410][POS] LS to RAN1 on startSFN parameter (ZTE)

[AT129bis][411][Relay] Release of PC5 RLC channels (Apple)

[AT129bis][412][Relay] Relay capability prerequisites (Samsung)

[AT129bis][413][Relay] Local ID pair list (ASUSTeK)

[AT129bis][414][Relay] FFS issues on (re)selection (Huawei)

[AT129bis][601][SONMDT] Address open issues for RRC (Ericsson)

[AT129bis][602][Maint]Correction on timeUntilReconnection in RLF report(ZTE)

[AT129bis][603][Maint] Correction on SRS capability reporting (Huawei)

[AT129bis][604][Maint] Usage of msg3-transmitPrecoder (CATT)

[AT129bis][605][Maint] SCS and CP configuration in RedCap-specific initial BWP (ZTE)

[AT129bis][xxx][AI PHY] Draft LS on OAM-centric solution (Huawei)

[AT129bis][xxx][NES] LS on on-demand SSB (Apple)

[AT129bis][xxx][R18Mob] Capability coordination (Ericsson)

[AT129bis][xxx][R18Mob] Capability coorection based on RAN4 LS (vivo)

52 at-meeting offline discussions.

# Annex G:	Post-meeting email discussions after RAN2#129bis

Guidelines for email discussions:

General guidelines for SHORT email discussions, to be concluded approved endorsed at current meeting.

- Aim to have the final version of the agreed documents provided by the rapporteur at or shortly after the deadline.

- Please provide comments on the first version of the document in good time before the deadline. This allows the rapporteur to make an update addressing all companies' comments and there still be time for a quick round of comments on the update.

- If you have provided comments in the discussion then please indicate to the rapporteur if you are ok with the update provided (preferably via reflector). This avoids the rapporteur having to wait before they can conclude that their update is acceptable to you.

- Rapporteurs, if not already available, please request your tdoc number from the Secretary when you initiate your email discussion and then provide the final version as soon as you are confident that it is agreeable. You do not need to wait for a reminder from chair, session chair or Secretary before sending the final version.

- To avoid any confusion, Secretary, chair, or session chair will send an email to confirm the final status of the document.

For emails discussion to the next meeting (long):

- Rapporteurs, feel free to set an intermediate deadline for companies to provide initial comments, so that the conclusions and proposals can be prepared and distributed before the final deadline.

- Participants, please respect any intermediate deadline indicated by the rapporteur, and preferably provide your feedback as soon as possible.

Deadlines

Apr. 18th	1000 UTC	Deadline short email discussions

May  2nd	1000 UTC	Deadline long email discussions

May  9th	1000 UTC	Submission Deadline RAN2#130

Weekends are inactive periods.

It is recommended to not send emails or update files on the server during inactive periods while It is not strictly prohibited. Rapporteurs may kick-off discussions during inactive period. However, no intermediate deadlines and no interactive discussion, no decision making may occur during the inactive period. It shall be possible for a delegate to stay away from reflector and 3GPP server during the inactive period, and still fully participate. Rapporteur announcements during the inactive period, if any, or other updates, can be taken into account after the inactive period.

# Short email discussions, Deadline Apr. 18th, 10:00 UTC

Please request R2-129bis TDoc numbers for the following email discussions from MCC if not already allocated. Approval / endorsement will be declared at or shortly after the deadline.

[POST129bis][021][TEI19] Multi-path relay enhancements (CMCC)

Intended outcome:  in principle agree to CRs

Deadline:  Short

=> Agreed in principle in:

R2-2503188 (38.300)

R2-2503189 (38.331)

R2-2503190 (38.306)

[POST129bis][025][AIoT] LS on byte-aligned SDU to CT1  (Vivo)

Intended outcome: agree to LS to CT1 capturing agreement on byte-aligned SDU.

Deadline:  short

=> Approved in R2-2503198

[POST129bis][026][AIoT] LS on D2R message size to SA2 (InterDigital)

Intended outcome: agree to LS to SA2 (cc RAN3) capturing agreement on D2R message size for inventory response.

Deadline:  short

=> Approved in R2-2503199

[POST129bis][027][AIoT] LS on paging ID length to SA2 (cc CT4) (CATT)

Intended outcome: agree to LS to SA2 and (cc CT4) capturing agreement on paging ID length and ask SA2 to take this agreement into account.

Deadline:  short

=> Approved in R2-2503197

[Post129bis][203][LPWUS] LS to R1 LP-WUS in RRC_CONNECTED (InterDigital)

Intended outcome: Approve the LS.

Deadline: Short

=> Approved in R2-2503202

[Post129bis][205][NR_Others] Reply LS to RAN4 on 7MHz channel bandwidth (T-Mobile)

Scope: Discuss questions to RAN4 (e.g., the support of 7Mhz as the minimum or maximum channel bandwidth)

Intended outcome: Approve the LS.

Deadline:  Short

=> Approved in R2-2503201

[Post129bis][217][LPWUS] LS to RAN3 (CC SA2/SA3) to inform agreements on UE ID based subgrouping (NEC)

Intended outcome: Approve the LS

Deadline:  Short

=> Approved in R2-2503182

[Post129bis][219][SBFD] LS to RAN1 (CC RAN3/RAN4) on simultaneous configuration of SBFD and DC (Huawei)

Intended outcome: Approve the LS

Deadline:  Short

=> Approved in R2-2503191

[Post129bis][220][NR_Others] LS to SA5 on number of UEs in RRC_INACTIVE state with data transmission  (China Telecom)

Intended outcome: Approve the LS

Deadline:  Short

=> Approved in R2-2503181

[Post129bis][301][R19 IoT NTN] LS to CT1 on S&F mode indications to NAS (CATT)

Scope: Draft an LS to CT1 with meeting agreements on indication to NAS regarding transitions between normal mode and S&F mode operation

Intended outcome: Approved LS

Deadline: 1-week

=> Approved in R2-2503070

[Post129bis][302][R19 IoT NTN] LS to RAN1 on CB-msg3-EDT (Mediatek)

Scope: Draft an LS to RAN1 with meeting agreements CB-msg3-EDT

Intended outcome: Approved LS

Deadline: 1-week

=> Approved in R2-2503175

[Post129bis][303][TEI18] LS to RAN4 on NR NB-IoT in-band operation (Ericsson)

Scope: Draft a reply LS to RAN4 on NR NB-IoT in-band operation

Intended outcome: Approved LS

Deadline: 1-week

=> Approved in R2-2503200

[Post129bis][408][POS] LS to RAN1/RAN3 on SRS frequency hopping for non-RedCap UE (ZTE)

Scope: Revise the LS in R2-2502260 in line with meeting agreements.

Intended outcome: Approved LS

Deadline: Short

=> Approved in R2-2503185

[Post129bis][409][POS] CRs on control parameters for on-demand posSIB request (Huawei)

Scope: Check the CRs in R2-2503157 / R2-2503158 / R2-2503159 / R2-2503160.

Intended outcome: Agreed in principle CRs

Deadline: Short

=> Agreed in principle in:

R2-2503192 (38.331)

R2-2503193 (38.306)

R2-2503194 (38.305)

R2-2503195 (38.300)

[Post129bis][411][Relay] Multihop terminology in 38.300 (LG)

Scope: Implement the terminology agreements of RAN2#129bis in 38.300 to allow running CR rapporteurs to align with the usage.

Intended outcome: Endorsed CR (based on R2-2503076 with deltas for the terminology) in R2-2503088

Deadline: Short

=> Endorsed in R2-2503088

[Post129bis][601][Maint] Miscellaneous non-controversial corrections (Ericsson)

Scope:

- Produce agreeable CRs

Intended outcome:

    - In principle agreed CRs

Deadline:

    - Short

=> Agreed in principle in R2-2503184

[Post129bis][605][SONMDT] LS to RAN3 on MDT for NTN (Qualcomm)

Scope:

- Produce approvable LS

Intended outcome:

    - Approvable LS in R2-2503117

Deadline:

    - Short

=> Approved in R2-2503203

17 short email discussions.

# Long email discussions, for R2-130, May.  2nd 10:00 UTC (unless otherwise stated)

Please request R2-130 TDoc numbers for the following email discussions by 3GU according to normal tdoc submission procedure.

[POST129bis][002][ASN.1 review] Process improvements (Nokia)

Intended outcome: proposals to next meeting

Deadline:  Long

[POST129bis][013][AI PHY] 38.300 Running CR (Vivo)

Intended outcome: Update 300 CR based on agreements from RAN2#129bis

Deadline:  Long

[POST129bis][014][AI PHY] 38.305 Running CR (CATT)

Intended outcome: Update CR based on agreements from RAN2#129bis

Deadline:  Long

[POST129bis][015][AI PHY] 37.355 Running CR (Qualcomm)

Intended outcome:

- Update CR based on agreements from RAN2#129bis

- List of remaining open issues

Deadline:  Long

[POST129bis][016][AI PHY] 38.331 Running CR (Ericsson)

Intended outcome:

- Update CR based on agreements from RAN2#129bis

- List of remaining open issues

Deadline:  Long

[POST129bis][017][AIoT] 38.391 Running CR (Huawei)

Intended outcome:

1 Update and review running CR

2 Create list of remaining open issues

Deadline:  long

[POST129bis][018][AIoT] 38.300 Running CR (CMCC)

Intended outcome:

1 Update and review running CR

Deadline:  long

[POST129bis][019][AI Mob] TR update (Oppo)

Intended outcome:  review TR containing agreements from RAN2#129bis

Deadline:  long

[POST129bis][020][AI Mob] Sim. Results figures  (Mediatek)

Intended outcome: email discussion to gather specific comments related to result illustration

Deadline:  long

[POST129b][111][NES] (Ericsson)

Scope:

- Update RRC running CR based on RAN2#129bis progress

- Essential open issue list in a separate contribution (RRC running CR can keep editor’s notes for readability).

Intended outcome: Updated RRC running CR and essential RRC open issue list.

Deadline: Long email discussion.

[POST129b][112][NES] (IDC)

Scope:

- Update MAC running CR based on RAN2#129bis progress

- Essential open issue list in a separate contribution (MAC running CR can keep editor’s notes for readability).

Intended outcome: Updated MAC running CR and essential MAC open issue list.

Deadline: Long email discussion.

[POST129b][113][NES] (Apple)

Scope:

- Update 38.304 running CR based on RAN2#129bis progress

- Essential open issue list in a separate contribution (38.304 running CR can keep editor’s notes for readability).

Intended outcome: Updated 38.304 running CR and essential 38.304 open issue list.

Deadline: Long email discussion.

[POST129b][114][NES] (Huawei)

Scope: Update 38.300 running CR based on RAN2#129bis progress.

Intended outcome: Updated 38.300 running CR.

Deadline: Long email discussion.

[POST129b][115][MOB] (Ericsson)

Scope:

1   Update RRC running CR for inter-CU LTM and C-LTM based on RAN2#129bis progress

2   Essential open issue list in a separate contribution (RRC running CR can keep editor’s notes for readability).

Intended outcome: Updated RRC running CR and essential RRC open issue list for inter-CU LTM and C-LTM.

Deadline: Long email discussion.

[POST129b][116][MOB] (Huawei)

Scope:

- Update RRC running CR for L1 event-driven MR based on RAN2#129bis progress

- Essential open issue list in a separate contribution (RRC running CR can keep editor’s notes for readability).

Intended outcome: Updated RRC running CR and essential RRC open issue list for L1 event-driven MR.

Deadline: Long email discussion.

[POST129b][117][MOB] (Vivo)

Scope:

- Update MAC running CR based on RAN2#129bis progress

- Essential open issue list in a separate contribution (MAC running CR can keep editor’s notes for readability).

Intended outcome: Updated MAC running CR and essential MAC open issue list.

Deadline: Long email discussion.

[POST129b][118][MOB] (CATT)

Scope: Update UE capability running CRs (including 38.331 and 38.306) based on RAN2#129bis progress.

Intended outcome: Updated UE capability running CRs (including 38.331 and 38.306).

Deadline: Long email discussion.

[POST129b][119][MOB] (Apple)

Scope: Update 38.300 running CR based on RAN2#129bis progress.

Intended outcome: Updated 38.300 running CR.

Deadline: Long email discussion.

[POST129b][120][MOB] (China Telecom)

Scope: Update 37.340 running CR based on RAN2#129bis progress.

Intended outcome: Updated 37.340 running CR.

Deadline: Long email discussion.

[POST129b][121][NES] (ZTE)

Scope: Discuss initial discussion points for UE capabilities.

Intended outcome: Discussion summary.

Deadline: Long email discussion.

[POST129b][122][NES] (OPPO)

Scope: Discuss and make conclusions on proposal 1 in R2-2501817.

Intended outcome: Discussion summary.

Deadline: Long email discussion.

[Post129bis][206][LPWUS] Running CR for 37.340 (ZTE)

Intended outcome: Updated running CR based on new agreements for endorsement

Deadline:  Long

[Post129bis][207][LPWUS] Running CR for 38.300 (Ericsson)

Intended outcome: Updated running CR based on new agreements for endorsement

Deadline:  Long

[Post129bis][208][LPWUS] Running CR for 38.304 (CATT)

Intended outcome: Updated running CR based on new agreements for endorsement, open issue list (if needed)

Deadline:  Long

[Post129bis][209][LPWUS] Running CR for 38.331 (vivo)

Intended outcome:

- Updated running CR based on new agreements for endorsement

- open issue list

Deadline:  Long

[Post129bis][210][LPWUS] Running CR for 38.321 (Apple)

Intended outcome:

- Updated running CR based on new agreements for endorsement

- open issue list

Deadline:  Long

[Post129bis][218][LPWUS] UE capabilities (Huawei)

Intended outcome: Summary with proposals on RAN2 UE capabilities for LP-WUS

Deadline:  Long

[Post129bis][211][SBFD] Running CR for 38.300 (CATT)

Intended outcome: Updated running CR based on new agreements for endorsement

Deadline:  Long

[Post129bis][212][SBFD] Running CR for 38.331 (Huawei)

Intended outcome:

- Updated running CR based on new agreements for endorsement

- open issue list

Deadline:  Long

[Post129bis][213][SBFD] Running CR for 38.321 (Samsung)

Intended outcome:

- Updated running CR based on new agreements for endorsement

- open issue list

Deadline:  Long

[Post129bis][214][ MIMO_Ph5] Running CR for 38.331 (Ericsson)

Intended outcome:

- Updated running CR based on new agreements for endorsement

- open issue list

Deadline:  Long

[Post129bis][215][ MIMO_Ph5] Running CR for 38.321 (Samsung)

Intended outcome:

- Updated running CR based on new agreements for endorsement

- Open issue list

Deadline:  Long

[Post129bis][216][ MIMO_Ph5] Running CR for 38.300 (CMCC)

Intended outcome: Updated running CR based on new agreements for endorsement

Deadline:  Long

[Post129bis][304][R19 NR NTN] Stage2 CR (Thales)

Scope: discuss the running Stage 2 CR

Intended outcome: Endorsed CR

Deadline: long

[Post129bis][305][R19 NR NTN] RRC CR (Ericsson)

Scope: discuss the running RRC CR and create list of open issues

Intended outcome: Endorsed CR and list of open issues

Deadline: long

[Post129bis][306][R19 NR NTN] MAC CR (Interdigital)

Scope: discuss the running MAC CR

Intended outcome: Endorsed CR

Deadline: long

[Post129bis][307][R19 NR NTN] 38.304 CR (ZTE)

Scope: discuss the running 38.304 CR

Intended outcome: Endorsed CR

Deadline: long

[Post129bis][308][R19 NR NTN] capability CR (Apple)

Scope: discuss the running capability CR

Intended outcome: Endorsed CR

Deadline: long

[Post129bis][309][R19 IoT NTN] Stage2 CR (Ericsson)

Scope: discuss the running Stage 2 CR

Intended outcome: Endorsed CR

Deadline: long

[Post129bis][310][R19 IoT NTN] RRC CR (Huawei)

Scope: discuss the running RRC CR and create list of open issues

Intended outcome: Endorsed CR and list of open issues

Deadline: long

[Post129bis][311[R19 IoT NTN] MAC CR (Mediatek)

Scope: discuss the running MAC CR and create list of open issues

Intended outcome: Endorsed CR and list of open issues

Deadline: long

[Post129bis][312][R19 IoT NTN] 36.304 CR (Nokia)

Scope: discuss the running 36.304 CR

Intended outcome: Endorsed CR

Deadline: long

[Post129bis][313][R19 IoT NTN] capability CR (Qualcomm)

Scope: discuss the running capability CR

Intended outcome: Endorsed CR

Deadline: long

[Post129bis][314][LTE to NR NTN] RRC CR (CATT)

Scope: discuss the running RRC CR and create open issue list

Intended outcome: Endorsed CR and open issue list

Deadline: long

[Post129bis][401][Relay] Rel-19 relay service continuity CR to 38.331 (CATT)

Scope: Update the service continuity running CR from the baseline of R2-2503071 to take into account decisions of RAN2#129bis and create list of open issues

Intended outcome: Updated CR and open issue list for RAN2#130

Deadline: Long

[Post129bis][402][Relay] Rel-19 relay CR to 38.304 (MediaTek)

Scope: Update the 38.304 running CR from the baseline of R2-2503072 to take into account decisions of RAN2#129bis

Intended outcome: Updated CR for RAN2#130

Deadline: Long

[Post129bis][403][Relay] Rel-19 relay CR to 38.323 (Ericsson)

Scope: Update the 38.323 running CR from the baseline of R2-2503073 to take into account decisions of RAN2#129bis.

Intended outcome: Updated CR for RAN2#130

Deadline: Long

[Post129bis][404][Relay] Rel-19 relay CR to 38.321 (InterDigital)

Scope: Update the 38.323 running CR from the baseline of R2-2503074 to take into account decisions of RAN2#129bis.

Intended outcome: Updated CR for RAN2#130

Deadline: Long

[Post129bis][405][Relay] Rel-19 relay main CR to 38.331 (Huawei)

Scope: Update the 38.331 running CR from the baseline of R2-2503075 to take into account decisions of RAN2#129bis.

Intended outcome: Updated CR and open issue list for RAN2#130

Deadline: Long

[Post129bis][406][Relay] Rel-19 relay CR to 38.300 (LG)

Scope: Update the 38.300 running CR from the baseline of R2-2503088 to take into account decisions of RAN2#129bis.

Intended outcome: Updated CR for RAN2#130

Deadline: Long

[Post129bis][407][Relay] Rel-19 relay CR to 38.351 (OPPO)

Scope: Update the 38.351 running CR from the baseline of R2-2503077 to take into account decisions of RAN2#129bis and create open issue lists

Intended outcome: Updated CR for RAN2#130 and open issue lists

Deadline: Long

[Post129bis][410][Relay] Local ID pair list (ASUSTeK)

Scope: Further check the CR in R2-2503083 and determine:

- if the CR is necessary or the list can be left as a “ToAdd” only;

- if the CR is functionally NBC;

- if something is needed, whether the procedural change is correct.

Intended outcome: CR to next meeting

Deadline: Long

[Post129bis][412][Relay] FFS issues on system information (ZTE)

Scope: Discuss the identified FFS issues on system information (proposals flagged for treatment and not treated in RAN2#129bis) and converge where possible.

Intended outcome: Report to next meeting

Deadline: Long

[POST129bis][501][XR] Stage-2 running CR (Nokia)

Scope: Update and review the CR

Intended outcome: Running CR for endorsement in the next meeting

Deadline:  Long

[POST129bis][502][XR] MAC running CR and open issues (Qualcomm)

Scope:

- Update and review the CR

- List open issues related to the CR

Intended outcome:

- Running CR for endorsement in the next meeting

- List of open issues for discussion at the next meeting

Deadline:  Long

[POST129bis][503][XR] RRC running CR and open issues (Huawei)

Scope:

- Update and review the CR

- List open issues related to the CR

Intended outcome:

- Running CR for endorsement in the next meeting

- List of open issues for discussion at the next meeting

Deadline:  Long

[POST129bis][504][XR] PDPC running CR and open issues (LGE)

Scope:

- Update and review the CR

- List open issues related to the CR

Intended outcome:

- Running CR for endorsement in the next meeting

- List of open issues for discussion at the next meeting

Deadline:  Long

[POST129bis][505][XR] RLC running CR and open issues (vivo)

Scope:

- Update and review the CR

- List open issues related to the CR

Intended outcome:

- Running CR for endorsement in the next meeting

- List of open issues for discussion at the next meeting

Deadline:  Long

[POST129bis][506][XR] UE capabilities CRs (Xiaomi)

Scope: Update and review the CRs

Intended outcome: Running CRs for endorsement in the next meeting

Deadline:  Long

[POST129bis][507][XR] Incoming LS(es) from SA4 (Qualcomm)

Scope: Gather companies views on how to reply to SA4 LS(es) in S4-250736 and S4-250737

Intended outcome: Report including draft reply LS(es) to be submitted to RAN2#130

Deadline:  9 May 10:00 UTC

[Post129bis][603][SONMDT] Running NR RRC CR for SONMDT (Ericsson)

Scope:

- Update the running CRs based on the progress in the meeting

- Create open issue lists

Intended outcome:

    - Updated running CR to be submitted to next meeting

Deadline:

    - Long

[Post129bis][604][SONMDT] Running LTE RRC CR for SONMDT (Huawei)

Scope:

- Update the running CRs based on the progress in the meeting

Intended outcome:

    - Updated running CR to be submitted to next meeting

Deadline:

    - Long

61 long email discussions.

# Annex H:	History

| Document history |  |  |
| --- | --- | --- |
| Date | TSG RAN WG2 Tdoc | Subject |
| 17.04.2025 |  | First draft version |
| 23.04.2025 |  | Second draft version |
| 06.05.2025 |  | Third draft version |
| 09.05.2025 | R2-2503302 | Version submitted for approval |
| Author: Dr. Juha Korhonen ETSI Mobile Competence Centre (MCC) Tel. +33 (0)4 92 94 42 00 email: Juha.Korhonen@etsi.org |  |  |