3GPP TSG-RAN WG2 Meeting #130 	 R2-2503353

St. Julian’s, Malta, May 19th – 23rd, 2025

Agenda item:	8.9.3

Source:	vivo

Title:	Further Discussion on CB-msg3-EDT Mechanism

Document for:	Discussion and Decision

1 Introduction

In the last RAN2 meetings, great progress on the CB-msg3-EDT mechanism was made. For ease of reference, the achieved agreements regarding the general machanism of CB-msg3-EDT are listed below [1]:

RAN2#129bis Agreements: - Both SA and DSA are mandatorily supported by UEs supporting CB-msg3-EDT. - We will specify one single procedure to support both DSA and SA, i.e. SA is a special setting (k=1) of the overall procedure. - For CB-msg3-EDT, the transmission window can be configured by the network with a starting point (e.g. H-SFN offset), a window length, and a window periodicity (window length and periodicity could be the same). - We don’t introduce support for eMTC CE mode B case (it will not be possible to signal resources to be used for this case). - We specify support for NB-IoT with 15kHz with no specific enhancements, leaving to NW implementation whether to implement this or not, accepting potential performance degradation. - If we will decide to support OCC for CB-msg3-EDT, separate resources will be used for non-OCC and OCC based transmission. - The start of CB-msg3 EDT transmission window is aligned with the start of time domain (N)PUSCH resource. - The CB-msg3 EDT transmission window length and periodicity may be different. FFS on possible signalling optimization in case the length and periodicity are the same. - RAN2 assumes power ramping should be supported for CB-msg3-EDT (for both eMTC and NB-IoT) should be supported and will ask RAN1 for confirmation and in case which parameters should apply. - For CB-msg3-EDT we adopt a Single Msg4 monitoring window and Single RNTI (the RNTI is derived on the transmit resource for the transmission window). - The lengths of the Msg3 transmission and Msg4 monitoring windows are configured by the network (in case of k=1 it will be possible to configure the parameters in a way to have the same behaviour as for normal Random Access procedure). - The Msg4 monitoring starts at the end of CB-Msg3-EDT transmission window plus UE-eNB RTT (FFS NW/UE processing time is needed or not). - FFS it will also be possible for the NW to configure that the Msg4 monitoring window starts in the subframe containing the last (N)PUSCH repetition of the first replica plus UE-eNB RTT (FFS NW/UE processing time). To possibly resolve the FFS it needs to be clarified what happens if the Msg4 monitoring window is overlapping with replica, i.e. whether the UE prioritize the replica transmission or monitoring. - The CB-msg3-EDT configuration (e.g., number of replicas, number of time resources and number of frequency resources) is CE level specific. - The Msg4 monitoring window configuration (e.g. length) is CE level specific. - RAN2 confirms the working assumption that one CB-Msg4 can target multiple UEs simultaneously. FFS how the multiplexing is organized. - RAN2 confirms that existing UP-EDT and CP-EDT RRC message procedures shall be applicable for CB-Msg3-EDT - -For UP solution, after RRCConnectionResumeRequest, network may reply with RRCConnectionResume, RRCConnectionSetup, RRCConnectionRelease and RRCConnectionReject; - -For CP solution, after RRCEarlyDataRequest, the network can respond with RRCEarlyDataComplete, RRCConnectionSetup or RRCConnectionReject. - RAN2 also intends to support CB-msg3-EDT for MT cases. - The C-RNTI is included in CB-Msg4 if the UE is expected to receive additional RRC messages or data from the network after CB-Msg4 (FFS how to include the C-RNTI). - Introduce a new RNTI (i.e. CB-RNTI) for CB-Msg4 monitoring and CB-Msg3 scrambling. We include this agreement in the LS to RAN1. - The timing alignment information (FFS reusing TAC MAC-CE) can be included in the CB-Msg4. - Parameter for maximum re-attempt number per CE level is introduced and UE can re-attempt in the same CE level due to contention resolution failure until the max re-attempt number has been reached. - Backoff information could be included in CB-Msg4. - L1 ACK as the Msg4 for the CB-Msg3-EDT is not supported. - HARQ feedback is adopted to acknowledge Msg4. FFS for the detail (e.g., how the HARQ feedback is used for each response in Msg4 when there is multiplexing in Msg4.).

In this contribution, we will further discuss the CB-msg3-EDT mechanism used for IoT-NTN. In the following, we will provide our understanding from CB-msg3-EDT configuration & selection, CB-msg3-EDT transmission, CB-RNTI, and response reception & MAC PDU format points of view.

2 Discussion

  - 2.1 CB-msg3-EDT configuration & selection

In the previous discussion, the configuration details for CB-msg3-EDT transmission window remained unsolved. From our perspective, the CB-msg3-EDT transmission window serves to aggregate a defined number of the scattered and periodic CB-msg3-EDT occasions for the DSA transmission. To this end, both its length and periodicity can be indicated in multiples of the CB-msg3-EDT occasion periodicity, similar to how the configuredGrantTimer is specified in NR. This allows the UE to have the same understanding as the network to calculate and determine each CB-msg3-EDT transmission window and the corresponding valid occasions, as illustrated in Figure 1.

Further, if the network intends to set the same value for the window length and periodicity (i.e., making all occasions within the periodicity available for DSA), it can opt to configure only the window periodicity for the sake of singalling overhead reduction. Moreover, considering that DSA replica number is configured per CE level, we propose that the CB-msg3-EDT transmission window should also be separately configured per CE level. This approach enables alignment with the DSA replica configuration and flexible resource control. If the DSA replica number is set to 1, we understand that configuring CB-msg3-EDT transmission window may still be beneficial when the network aims to multiplex multiple responses associated with different occasions into a single MAC PDU (i.e., ensuring UEs utilizing different occasions derive the same calculated CB-RNTI and share the same starting point for CB-msg4 monitoring). Meanwhile, to ensure consistency across implementations for other DSA replica numbers, the window should still be configured. For example, the network only configures the periodicity and sets its value to 1.

Figure 1: CB-msg3-EDT transmission window

Proposal 1: CB-msg3-EDT transmission window length and periodicity are configured in multiples of the CB-msg3-EDT occasion periodicity (e.g., value ranging from 1..32).

Proposal 2: If the network intends to set the same value for the window length and periodicity, it configures only the window periodicity. In this case, UE considers all occasions within the periodicity as valid for DSA.

Proposal 3: Regardless of whether the number of DSA replicas is 1 or another value, the network always configures the CB-msg3-EDT transmission window.

Proposal 4: The starting point of the very first CB-msg3-EDT transmission window is located at the beginning of the first symbol of the first available CB-msg3-EDT occasion occurring after the configured starting point offset.

In the previous meeting, RAN2 asked RAN1 whether a set of PRB allocation information (i.e., allocated resource block and allocated resource subcarriers) should be configured for CB-msg3-EDT. If RAN1 confirms the necessity and feasibility, RAN2 should then consider how to determine the appropriate PRB allocation to be used. For simplicity, we propose that it is up to UE implementation to select any one of the indicated PRB allocations from the PUSCH resource set for CB-msg3.

Proposal 5: If RAN1 confirms support for a set of PRB allocation configurations for CB-msg3 specific to a CE level, it is up to UE implementation to select any one of the indicated PRB allocations.

  - 2.2 CB-msg3-EDT transmission

After CB-msg-3-EDT occasion selection, the UE should do UL-SCH data transfer and the actual transport block transmission via the physical layer. The main preparation steps for the PUSCH transmission are given as follows:

- The MAC entity receives the UL grants;

- The MAC entity delivers the UL grants and the associated HARQ information to the HARQ entity;

- The HARQ entity instructs the multiplexing and assembly entity to generate a MAC PDU;

- The HARQ entity obtains the MAC PDU to transmit from the multiplexing and assembly entity;

- The HARQ entity triggers the corresponding HARQ process to perform a new transmission/retransmission;

- The corresponding HARQ process	instructs the PHY layer to generate a transmission;

- The PHY layer transmits the PUSCH for the obtained MAC PDU with PHY procedure.

Observation 1: HARQ operation is applicable for both Msg3 and Msg4.

To minimize the potential RAN1 impacts with the introduction of DSA, we think each replica should be modeled as a new transmission. On one hand, since the network side cannot perform soft combining for DSA replicas due to the lack of information (based on the WA that the pointer is not supported in Rel-19) on which specific occasions a UE selects. This eliminates the need to model replicas as retransmissions, as soft combining is unfeasible. On the other hand, treating replicas as retransmissions would necessitate coordination with RAN1 to define a power control scheme for retransmission or RV order sequence, introducing unnecessary complexity.

By treating the replica transmission as a new transmission, the MAC needs to generate a MAC PDU for each replica transmission. As the content in each replica should be the same, a possible and simple modeling is to obtain the MAC PDU from the HARQ buffer for the 2nd replica. At the physical layer, it can treat each replica transmission as a new transmission as in the legacy case (e.g., independently doing the power control, resource mapping, etc), as depicted in Figure 2 below. Additionally, it is proposed to reuse HARQ process ID 0 and RV0, similar to Msg3 transmission.

Figure 2: CB-msg3-EDT transmission with replica

Further, regarding the applicability of HARQ feedback, we propose adhering to legacy rules, where HARQ feedback applies to CB-msg3-EDT. Specifically, HARQ retransmission triggered by DCI (similar to TC-RNTI DCI used to trigger Msg3 HARQ retransmission in legacy) is assumed to be supported for the initial transmission of CB-msg3 (check with RAN1), and the UE is required to feedback HARQ-ACK after successful contention resolution. The primary consideration is that CB-msg4 includes RRC signaling and high-priority data. Enabling HARQ retransmission ensures robust reliability.

Observation 2: HARQ feedback is always applied to both Msg4 and PUR.

Proposal 6: HARQ operation is used for CB-msg3 and CB-msg4

Proposal 7: HARQ process id 0 and RV 0 are used for CB-msg3-EDT.

Proposal 8: MAC considers each replica transmission as a new transmission.

Proposal 9: HARQ entity obtains the MAC PDU from the HARQ process buffer for the secondary replica.

Proposal 10: RAN2 assumes that HARQ retransmission triggered by DCI is supported for CB-msg3(check with RAN1 on the feasibility in Rel-19 timeline).

Proposal 11: HARQ feedback is applied to CB-msg4.

  - 2.3 CB-RNTI

Regarding the CB-RNTI calculation, to accommodate the new MAC PDU format for CB-msg4 and prevent cross-procedure interference (in case there is be overlap between the PDCCH resource for RAR and CB-msg4), we propose defining the CB-RNTI value range to be entirely independent of the RA-RNTI used in legacy RACH procedures. This isolation ensures that the UEs performing legacy RACH/EDT procedures will not erroneously decode or process the new CB-msg4 and the UEs initiating CB-msg3-EDT will not misinterpret legacy RACH signaling.

Proposal 12: RAN2 to define the CB-RNTI value range to be independent of the RA-RNTI.

In addition, the CB-RNTI calculation should account for multiplexing impacts. For instance, the network may aim to multiplex responses for different occasions occurring across distinct time-frequency resources. Given our understanding that the CB-msg4 monitoring window should start at the end of the corresponding CB-msg3-EDT transmission window, it logically follows that all potential responses associated with occasions spanning different time subframes can be multiplexed to enable a single response for all successfully decoded replicas. In the frequency domain, we propose avoiding multiplexing responses across resources spanning different frequencies (e.g., PRB allocations or NB-IoT carriers). This prevents the TBS of CB-msg4 from becoming large and facilitates independent frequency-specific decoding and response generation at the network side.

Then, we recommend that the UE determines the CB-RNTI based on the last valid CB-msg3 occasion within the corresponding CB-msg3-EDT transmission window, similar PRACH repetition case. Meanwhile, RNTI calculation is tied to the total length of the CB-msg4 monitoring window, which helps to avoid RNTI ambiguities caused by the same calculated RNTI values within a monitoring window period. For example, the CB-RNTI calculation for eMTC can be CB-RNTI=2400+t_id+10*f_id+ 60*(SFN_id mod (Wmax/10)) and CB-RNTI=12288+t_id+10*f_id+ 48*(SFN_id mod (Wmax/10)) for NB-IoT.

Proposal 13: CB-RNTI calculation considers the following parameters,

- the subframe timing index of the last valid CB-msg3 occasion within the corresponding CB-msg3-EDT transmission window;

- the frequency resource index associated with that last valid CB-msg3 occasion (e.g. PRB index or carrier ID);

- the maximum length of CB-msg4 monitoring window.

  - 2.3 Response reception & MAC PDU format

Following the CB-msg3 transmission, the UE needs to start a CB-msg4 monitoring window for response reception. For eMTC case, in the previous online discussion, views are diverging. Some companies advocate starting CB-msg4monitoring after the very first selected replica transmission. This approach allows the UE to monitor for responses as early as possible, potentially enabling it to discard remaining replica transmissions to save power and avoid unnecessary CB-msg3 collisions. Others suggest that RAN2 adopts a unified design for both eMTC and NB-IoT cases to simplify UE implementation (i.e., the UE starts the CB-msg4 monitoring window at the end of the corresponding CB-msg3-EDT transmission window plus UE-eNB RTT). Given that EDT is fundamentally designed to enable low-capability UEs to efficiently handle infrequent, delay-tolerant services, additional optimizations for early termination of replica transmissions are deemed unnecessary. For simple UE implementation, we would like to support a unified solution. Consequently, the DSA pointer solution is not needed in Rel-19.

Proposal 14: For both eMTC and NB-IoT, the UE starts the CB-msg4 monitoring window at the end of the corresponding CB-msg3-EDT transmission window plus UE-eNB RTT. No additional network or UE processing time is considered in Rel-19.

Proposal 15: RAN2 confirms that a DSA pointer solution is not needed in Rel-19.

Last but not least, considering the network may respond to a UE with an RRC message (e.g., transiting the UE into CONNECTED), then RAN2 should consider a new MAC PDU design to allow a MAC subPDU indicating 12-bit TA command, 16-bit C-RNTI, and 48-bit UE Contention Resolution Identity, HARQ-ACK timing offset, and TPC, potentially together with the RRC message. To this end, RAN2 needs to ensure that the new MAC PDU consists of one or more MAC subPDUs and optionally padding. Each MAC subPDU consists of one of the following:

-	a MAC subheader with Backoff Indicator only;

-	a MAC subheader and SetupRAR including C-RNTI, CRID, TAC, HARQ-ACK feedback parameters;

-	a MAC subheader and MAC SDU for CCCH, DCCH or DTCH;

-	padding.

Proposal 16: The new MAC PDU for Cb-msg4 consists of one or more MAC subPDUs and optionally padding. Each MAC subPDU consists of one of the following:

-	a MAC subheader with Backoff Indicator only;

-	a MAC subheader and SetupRAR including C-RNTI, CRID, TAC, HARQ-ACK feedback parameters;

-	a MAC subheader and MAC SDU for CCCH, DCCH or DTCH;

-	padding.

Specifically, the newly designed MAC PDU consists of a MAC header and zero or more SetupRAR, MAC SDU for CCCH, DCCH or DTCH and optionally padding as illustrated in Figure 3.

Figure 3: MAC PDU foramt for CB-msg4

The MAC headers and the SetupRAR are defined as following,

Figure 4: E/T/R/R/BI MAC subheader

Figure 5: E/T/D/R/R/R/R/R MAC subheader

Figure 6: R/R/R/LCID/F/L MAC subheader

Proposal 17: E/T/R/R/BI MAC subheader can be reused for Cb-msg4 MAC PDU format.

Proposal 18: E/T/D/R/R/R/R/R MAC subheader is introduced for SetupRAR. The D field is used to indicate whether the MAC PDU for DTCH is present.

Proposal 19: R/R/R/LCID/F/L MAC subheader is introduced for MAC SDU carrying CCCH, DCCH or DTCH.

Proposal 20: SetupRAR is introduced to include 48-bit CRID, 12-bit TAC, 16-bit C-RNTI, 2-bit TPC for HARQ-ACK, and  2-bit TPC for HARQ-ACK timing offset.

Proposal 21: For a given UE, SetupRAR and the corresponding DL RRC message are always sent in the same MAC PDU.

Figure 7: SetupRAR

3 Conclusion

In this contribution, we have provided our understanding of CB-EDT procedure. And we made the following proposals:

CB-msg3-EDT configuration & selection:

Proposal 1: CB-msg3-EDT transmission window length and periodicity are configured in multiples of the CB-msg3-EDT occasion periodicity (e.g., value ranging from 1..32).

Proposal 2: If the network intends to set the same value for the window length and periodicity, it configures only the window periodicity. In this case, UE considers all occasions within the periodicity as valid for DSA.

Proposal 3: Regardless of whether the number of DSA replicas is 1 or another value, the network always configures the CB-msg3-EDT transmission window.

Proposal 4: The starting point of the very first CB-msg3-EDT transmission window is located at the beginning of the first symbol of the first available CB-msg3-EDT occasion occurring after the configured starting point offset.

Proposal 5: If RAN1 confirms support for a set of PRB allocation configurations for CB-msg3 specific to a CE level, it is up to UE implementation to select any one of the indicated PRB allocations.

CB-msg3 Tx:

Observation 1: HARQ operation is applicable for both Msg3 and Msg4.

Observation 2: HARQ feedback is always applied to both Msg4 and PUR.

Proposal 6: HARQ operation is used for CB-msg3 and CB-msg4

Proposal 7: HARQ process id 0 and RV 0 are used for CB-msg3-EDT.

Proposal 8: MAC considers each replica transmission as a new transmission.

Proposal 9: HARQ entity obtains the MAC PDU from the HARQ process buffer for the secondary replica.

Proposal 10: RAN2 assumes that HARQ retransmission triggered by DCI is supported for CB-msg3(check with RAN1 on the feasibility in Rel-19 timeline).

Proposal 11: HARQ feedback is applied to CB-msg4.

CB-RNTI:

Proposal 12: RAN2 to definite the CB-RNTI value range to be entirely independent of the RA-RNTI.

Proposal 13: CB-RNTI calculation considers the following parameters,

- the subframe timing index of the last valid CB-msg3 occasion within the corresponding CB-msg3-EDT transmission window;

- the frequency resource index associated with that last valid CB-msg3 occasion (e.g. PRB index or carrier ID);

- the maximum length of CB-msg4 monitoring window.

Response reception & MAC PDU format:

Proposal 14: For both eMTC and NB-IoT, the UE starts the CB-msg4 monitoring window at the end of the corresponding CB-msg3-EDT transmission window plus UE-eNB RTT. No additional network or UE processing time is considered in Rel-19.

Proposal 15: RAN2 confirms that a DSA pointer solution is not needed in Rel-19.

Proposal 16: The new MAC PDU for Cb-msg4 consists of one or more MAC subPDUs and optionally padding. Each MAC subPDU consists one of the following:

-	a MAC subheader with Backoff Indicator only;

-	a MAC subheader and ReleaseRAR including CRID, TAC, HARQ-ACK feedback parameters;

-	a MAC subheader and SetupRAR including C-RNTI, CRID, TAC, HARQ-ACK feedback parameters;

-	a MAC subheader and MAC SDU for CCCH, DCCH or DTCH;

-	a MAC subheader and padding.

Proposal 17: E/T/R/R/BI MAC subheader can be reused for Cb-msg4 MAC PDU format.

Proposal 18: E/T/D/R/R/R/R/R MAC subheader is introduced for SetupRAR. The D field is used to indicate whether the MAC PDU for DTCH is present.

Proposal 19: R/R/R/LCID/F/L MAC subheader is introduced for MAC SDU carrying CCCH, DCCH or DTCH.

Proposal 20: SetupRAR is introduced to include 48-bit CRID, 12-bit TAC, 16-bit C-RNTI, 2-bit TPC for HARQ-ACK, and  2-bit TPC for HARQ-ACK timing offset.

Proposal 21: For a given UE, SetupRAR and the corresponding DL RRC message are always sent in the same MAC PDU.

4 Reference

[1] R2-2503302, RAN2#129bis Meeting Report, MCC.

[2] R2-2502773, Report of [Post129][307][R19 IoT NTN] CB-msg3/CB-msg4, MediaTek Inc.