3GPP TSG-RAN WG2 Meeting #130	R2-2503381

St.Julians, Malta, May. 19th – 23rd, 2025

Agenda item:		8.6.2

Source:		Lenovo

Title:		Some issues for Inter-CU LTM

Document for:		Discussion and Decision

- Introduction

We have already achieved some agreements in last meeting [1]. This contribution aims to discuss the further potential issues for Inter-CU LTM.

- Discussion

Fast Recovery

We support three types of fast recovery including Intra-gNB based recovery, Inter-gNB based recovery and CHO based recovery. Also, there are failures cases including RLF, Intra-gNB MCG LTM failure and Inter-gNB MCG LTM failure. CLTM failure and CLTM based recovery are discussed in another paper [2]. The following lists all the potential pair between failure and recovery. Regarding case 1 and case 2, there are no left issue. But two sub-cases in case 3 are not addressed yet (see TBD below).

Case 1: after RLF

- Intra-gNB based recovery i.e., a candidate cell from the same gNB (Rel.19 ID) as the source: Support

- Inter-gNB based recovery i.e., a candidate cell from a different gNB (Rel.19 ID) than the source: No support

- CHO based recovery: Support

Case 2: after Intra-gNB MCG LTM cell switch failure (target and the source cell have the same Rel. 19 ID)

- Intra-gNB based recovery: Support

- Inter-gNB based recovery: No support

- CHO based recovery: Support

Case 3: after Inter-gNB MCG LTM cell switch failure (target and the source cell have different Rel. 19 ID)

- Intra-gNB (candidate with Rel.19 ID same as source) based recovery: TBD

- Inter-gNB (candidate with Rel.19 ID same as target) based recovery: Supporting based on Rel-19 ID

- Inter-gNB (candidate with Rel.19 ID different from both source and target) based recovery: TBD

- CHO based recovery: TBD

In legacy Rel-18 LTM, Intra-gNB LTM recovery is not supported for L3 HO failure and CHO failure. The reason is that HO/CHO execution may incur PDCP and RLC re-establishment, which prohibits UE from reverting back to configuration and L2 state used in the source cell before HO/CHO attempt [3]. Similarly, Intra-gNB LTM recovery is not supported for Inter-CU LTM failure. Namely, after inter-CU LTM failure, if the selected candidate cell has the same Rel-19 ID as source, the UE cannot perform fast failure recovery.

Observation 1: Intra-gNB LTM based recovery is not supported for L3 HO/CHO failure in legacy Rel-18. Namely, after L3 HO/CHO failure, if the selected cell is configured with Rel-18 LTM, UE transmits re-establishment request directly.

Proposal 1: After inter-CU LTM failure, RAN2 to discuss whether fast failure recovery can be performed if the selected candidate cell has the same Rel-19 ID as source.

We already agreed to the coexistence between CHO and Rel-19 LTM. Therefore, we need to discuss whether CHO based recovery can be supported for Inter-CU LTM failure. Currently, CHO based recovery can be supported for RLF, Rel-18 LTM failure, HO/DAPS/CHO failure. Therefore, it is straightforward that CHO based recovery is supported for Inter-CU LTM failure.

Proposal 2: After Inter-CU LTM failure occurs, if the selected candidate cell is configured with CHO candidate configuration, UE performs CHO.

UE based TA measurement

It was agreed that UE-based TA measurement can be supported for inter-CU LTM scenario. The issue we need to address is which node allocate UE-based TA measurement ID for the candidate cell. As usual, UE based TA can not be used for Inter-CU cell switch. So, the issue could be whether UE based TA can be used for the cell switch between candidate cell#1 and candidate cell#2 which belong to the same candidate gNB. Specifically, if the candidate cell#1 from the candidate gNB#1 has been prepared, the UE-based TA measurement ID for the candidate cell has been allocated e.g. 1. The UE-based TA measurement ID for the source cell is e.g., 9. If source gNB decides to prepare the candidate cell#2 belonging to the candidate gNB#1, how to set the ID. There are two options listed below.

- Option 1: source gNB is responsible for UE-based TA measurement ID allocation.

  - In this option, the candidate gNB should provide the information whether UE based TA can be used for the cell switch between the candidate cell#1 and the candidate cell#2. If UE based TA can be used, source gNB can set the same value i.e., 1 to the candidate cell#2. Otherwise, the different ID value e.g., 2 will be allocated.

- Option 2: the candidate gNB is responsible for UE-based TA measurement ID allocation.

  - The candidate gNB is aware of whether UE based TA can be applicable for the cell switch between the candidate cell#1 and the candidate cell#2. But the candidate gNB may not know the ID value of the serving cell and all other prepared candidate cell(s). Therefore, source gNB should provide all assigned ID for the serving cell and other prepared cell(s). Then, the candidate gNB assigns the same value as the candidate cell#1 if the UE based TA can be applicable for the cell switch between the candidate cell#1 and the candidate cell#2.

Proposal 3: Source gNB is responsible for UE measured TA ID assignment for each candidate cell.

Proposal 4: The candidate gNB should provide the information whether UE based TA can be used for the cell switch between the candidate cells belonging the candidate gNB.

Coexistence of Inter-CU MCG LTM and Subsequent CPAC

In Rel-18 LTM, simultaneous configuration for both LTM and CPAC is allowed. After Inter-MN PCell change, the configuration for subsequent CPAC should be released by network via explicit indication. We can follow the same behavior that the configuration for subsequent CPAC should be released by network via explicit indication after Inter-CU MCG LTM.

- RAN2#121 In SCG selective activation, the CPC/CPA configurations of the UE should be released after Pcell change, at least for inter MN (by explicit indication from network, FFS other case).

Proposal 5: Subsequent CPAC can be configured while Inter-CU MCG LTM or SCG LTM is configured.

Proposal 6: The configuration for subsequent CPAC should be released by network via explicit indication after Inter-CU MCG LTM as legacy.

Coexistence of Inter-CU SCG LTM and MCG failure recovery

In Rel-18 Intra-CU SCG LTM, Intra-CU SCG LTM switch is allowed while MCG failure recovery procedure is ongoing. More specifically, UE will trigger fast MCG link recovery upon RLF on MCG. After UE transmits MCGFailureInformation message to MCG via SCG, UE starts T316 and waits for the response e.g handover command. Intra-CU SCG LTM could be triggered while T316 is running. That means the response will be pending in SCG until UE completes SCG change triggered LTM.

Regarding Inter-CU SCG LTM, if Inter-CU SCG LTM cell switch is allowed while MCG failure recovery procedure is ongoing. The response from MCG to UE should be transmitted to target SN. Therefore, once source SN activates Inter-CU LTM switching, source SN indicates it to MN. Then, MN transmits the response e.g reconfiguration message for PCell change to target SN. Specifically, the SN-DU inform the SN-CU about the initiation of the cell switch command to the UE via DU-CU Cell Switch Notification. Then, SN-CU informs it to MN-CU.

Proposal 7: Inter-CU SCG LTM switch is allowed to be triggered while MCG failure recovery procedure is ongoing as legacy Intra-CU SCG LTM.

Proposal 8: Once source SN activates Inter-CU LTM switching while MCG failure recovery procedure is ongoing, source SN indicates it to MN. Then, MN transmits the response of MCGFailureInforamtion to target SN.

Security in Inter-CU mobility

RAN2 has RAN2 had previously received a reply from SA3 in R2-2501756_S3-251124

Considering the RAN2’s request on avoiding RRC configuration between cell switches for subsequent inter-CU LTM in solution selection, SA3 reached a compromise that the NCC can be transmitted to the UE in plaintext via MAC CE message, even though the unprotected MAC leads to increased attack scenarios and probability of handover failure. In addition, SA3 will not consider the protection of MAC CE messages in this release, but may consider studying MAC CE protection in the future. The details on key distribution between the source gNB and the target gNB are left to RAN3 to decide and inform SA3. RAN3 should consider the scenario where the serving gNB utilizes the unused NH (if any is received from a path switch procedure at execution phase) to derive the KNG-RAN* for LTM handover.

And following agreements were reached in RAN2#129bis:

Agreements - Introducing a new R19 LTM cell switch command MAC CE with indicating the NCC value for R19 inter-CU LTM cell switch. Details will be further discussed during CR phase. - For security key update in inter-CU LTM, RAN2 agree to include actual NCC value in the LTM cell switch command MAC CE. - NCC is included in the LTM cell switch command MAC CE if the R19 set ID is different between the target cell and source cell. Conversely, if the R19 set ID is same for both cells, the NCC will not be included.

In addition to SA3 response, it is important for UE to distinguish if a key derivation is applied (horizontal or vertical), or not. If a new NH, NCC pair would become available at any time (i.e., due to a new NAS AKA run), then the network should transmit keySetChangeIndicator to the UE.

Proposal 9: Regarding inter-CU security change, MAC signalling should enable UE to distinguish if key derivation is to applied (same as previous NCC value means horizontal derivation and a different NCC means vertical derivation), or not (in which case the NCC is not included in the MAC CE).

A NAS AKA may have been run and therefore an [NH, NCC] pair can be available – but this is not very frequent case and therefore use of RRC signalling (instead of MAC) could be acceptable to transfer the keySetChangeIndicator to the UE. This can be done using an Intra-Cell handover procedure or, using a L3 handover procedure to a candidate cell. No new specification impact is seen.

Proposal 10: If a NAS AKA is run (and NH, NCC pair becomes available), then a fresh RRC signalling including keySetChangeIndicator is sent to the UE. No new Specification impact.

Finally, we also asked SA3 question on security Algorithm change. Since this should be relatively rare, implementing the key change using RRC should be acceptable.

Proposal 11: When the security Algorithm change is required, it is performed using layer 3 handover. No new specification impact.

Conclusion

In this contribution, the following proposals are given based on the discussion:

Fast Recovery

Observation 1: Intra-gNB LTM based recovery is not supported for L3 HO/CHO failure in legacy Rel-18. Namely, after L3 HO/CHO failure, if the selected cell is configured with Rel-18 LTM, UE transmits re-establishment request directly.

Proposal 1: After inter-CU LTM failure, RAN2 to discuss whether fast failure recovery can be performed if the selected candidate cell has the same Rel-19 ID as source.

Proposal 2: After Inter-CU LTM failure occurs, if the selected candidate cell is configured with CHO candidate configuration, UE performs CHO.

UE based TA measurement

Proposal 3: Source gNB is responsible for UE measured TA ID assignment for each candidate cell.

Proposal 4: The candidate gNB should provide the information whether UE based TA can be used for the cell switch between the candidate cells belonging the candidate gNB.

Coexistence of Inter-CU MCG LTM and Subsequent CPAC

Proposal 5: Subsequent CPAC can be configured while Inter-CU MCG LTM or SCG LTM is configured.

Proposal 6: The configuration for subsequent CPAC should be released by network via explicit indication after Inter-CU MCG LTM as legacy.

Coexistence of Inter-CU SCG LTM and MCG failure recovery

Proposal 7: Inter-CU SCG LTM switch is allowed to be triggered while MCG failure recovery procedure is ongoing as legacy Intra-CU SCG LTM.

Proposal 8: Once source SN activates Inter-CU LTM switching while MCG failure recovery procedure is ongoing, source SN indicates it to MN. Then, MN transmits the response of MCGFailureInforamtion to target SN.

Security in Inter-CU mobility

Proposal 9: Regarding inter-CU security change, MAC signalling should enable UE to distinguish if key derivation is to applied (same as previous NCC value means horizontal derivation and a different NCC means vertical derivation), or not (in which case the NCC is not included in the MAC CE).

Proposal 10: If a NAS AKA is run (and NH, NCC pair becomes available), then a fresh RRC signalling including keySetChangeIndicator is sent to the UE. No new Specification impact.

Proposal 11: When the security Algorithm change is required, it is performed using layer 3 handover. No new specification impact.

References

[1] Chair notes for RAN2#129bis

[2] R2-2503382 Conditional LTM remaining points, Lenovo

[3] R2-2402235 Fast Recovery with LTM Candidates, MTK