3GPP TSG-RAN WG2 Meeting #130              			                           R2-2503382

St.Julians, Malta, May. 19th – 23rd, 2025

Agenda Item:	8.6.4

Source: 	Lenovo

Title: 	CLTM Scenarios and remaining points

Document for:	Discussion and decision

Introduction

In the past RAN2 meeting, we have made a good progress for Intra-CU conditional MCG LTM. This document aims to address the remaining points for the Intra-CU MCG C-LTM work.

Discussion

- Early TA handling

It was agreed that LTM Cell Switch Command MAC CE can be used to trigger cell switch also for candidate cells configured for CLTM. It is possible that UE receives early TA MAC CE for CLTM candidate cells e.g., CLTM candidate cell#1. When UE receives LTM cell switch command for CLTM candidate cell#1 without early TA value, RAN2 needs to discuss whether RACH based LTM or RACH-less based LTM is performed by UE. If RACH based LTM is performed for C-LTM execution triggered by LTM cell switch command, UE needs to release the early TA for the target cell if configured. It will impact current specification.

Observation 1: After UE receives early TA MAC CE for CLTM candidate cell, UE performs RACH based cell switch upon the reception of LTM cell switch command without early TA value. In this case, UE needs to release the stored early TA which still impact the specification.

Proposal 1: After UE receives early TA MAC CE for CLTM candidate cell, UE performs RACHless cell switch upon the reception of LTM cell switch command without early TA value.

After LTM cell switch command MAC CE is received to trigger LTM cell switch towards CLTM candidate cell#1, how to handle the early TA for other candidate cells e.g., CLTM candidate cell#2. It was agreed that UE maintains the TA value and keep the early TA timer running for other candidate cells than the target candidate cell upon C-LTM execution due to the fulfilment of LTM condition. To avoid more impact, we propose to align with the CLTM execution triggered by the fulfilment of LTM condition. Namely, Upon C-LTM execution triggered by LTM cell switch command MAC CE, UE maintains the TA value and keep the early TA timer running for other candidate cells than the target candidate cell.

Proposal 2: If an LTM cell switch command is received for a C-LTM candidate, UE initiates mobility execution to the target cell. It maintains the TA value and keeps the early TA timer running for candidate cell(s) other than the target) cell.

- Coexistence cases

It seems unnecessary to restrict the simultaneous configuration of CLTM and intra-CU MCG LTM for the different candidate cell. The question could be whether the simultaneous configuration of CLTM and intra-CU MCG LTM for the same candidate cell can be allowed. We have agreed that LTM cell switch command can be used to trigger CLTM execution. Therefore, it seems unnecessary to configure both CLTM candidate configuration and LTM candidate configuration for the same cell. But we think it can be up to network implementation.

Observation 2: It seems unnecessary to configure both CLTM candidate configuration and LTM candidate configuration for the same cell since LTM cell switch command can be used to trigger CLTM execution. But it can be up to network implementation.

Proposal 3: Simultaneous configuration of Intra-CU MCG CLTM and Intra-CU MCG LTM is supported.

In Rel-18, coexistence of CHO and LTM is allowed. It means that both LTM candidate cell and CHO candidate cell can be simultaneously configured. Therefore, we can support the coexistence of CHO and CLTM.

Proposal 4: Simultaneous configuration of CHO and CLTM is supported.

- Candidate cell/beam selection

Regarding CLTM candidate cells associated with L1 or L3 execution condition, UE will perform CLTM execution if at least one of these CLTM candidate cell meets the execution condition. It is possible that some of these candidate cells have valid early TA but not the others. If the execution conditions from more than one CLTM candidate cells are met, UE can select one of them for cell switch. It is better to select one CLTM candidate cell which has valid early TA. There could be different ways to select a target cell e.g., based on best measurement quality, or select a candidate as target for which the remaining timer alignment timer has the highest value etc. But for simplicity if multiple candidate cells meeting the execution condition have valid early TA, it can be selected as target cell.

Proposal 5: If multiple CLTM candidate cells meet the L1 (or L3) execution condition, the target cell for CLTM execution can be selected among the candidate cells for which the UE has a valid TA. If the valid TA is not available for any candidate cell, the UE can select any candidate cell.

For L1 based CLTM candidate cell without MIMO 2TA, the C-LTM execution is triggered when at least one beam fulfils the CLTM event condition. When multiple candidate beams satisfy the CLTM condition, it is up to UE implementation to select a beam and perform C-LTM. Now, we consider the case that CLTM candidate cell is configured with MIMO 2TA. UE receives LTM TAC MAC CE for TAG#1 of CLTM candidate cell. If multiple beams of LTM candidate cell meet the condition, the beam which has valid early TA should be selected in priority.

Proposal 6: In the case of CLTM candidate cell with MIMO 2TA, UE prioritizes the beam which has valid early TA and also satisfy the execution condition.

- Fast Recovery on top of CLTM

We have agreed that R18 LTM failure recovery mechanism can be applied for CLTM failure. But we have not discussed whether CLTM based recovery can be supported and what failure case can be supported by CLTM based recovery. CLTM based recovery means after failure, UE can perform CLTM cell switch if the selected cell is configured with CLTM candidate configuration. CLTM based recovery can be supported for RLF, Intra-gNB MCG LTM failure or CLTM failure as legacy Intra-gNB LTM based recovery. Similarly, CLTM based recovery cannot be supported for Inter-gNB handover e.g., L3 HO, CHO, Inter-gNB LTM, DAPS etc.

Proposal 7: After RLF, Intra-gNB MCG LTM failure or CLTM failure, UE can perform cell switch to a selected cell if the selected cell is configured with CLTM configuration.

- Fallback from RACH-less

This section aims to discuss whether to support fallback from RACH-less CLTM cell switch to RACH-based CLTM cell switch if TAT (for the target cell) expires while T304 is still running. In Rel. 18, UE just wait for T304 expiry. We can follow same legacy UE behaviour for CLTM execution as well mainly in the hope that a right network configuration can configure the timers sensibly, minimizing such corner scenarios.

Proposal 8: Not to support Fallback from RACH-less CLTM CS to RACH-based CLTM CS when/ if TAT (for the target cell) expires while T304 is still running.

Conclusion

This document attempted to develop a basic understanding of “conditional LTM” and proposed some first scenarios for RAN2 consideration. In summary, we have following observations and proposals:

Early TA delivery

Observation 1: After UE receives early TA MAC CE for CLTM candidate cell, UE performs RACH based cell switch upon the reception of LTM cell switch command without early TA value. In this case, UE needs to release the stored early TA which still impact the specification.

Proposal 1: After UE receives early TA MAC CE for CLTM candidate cell, UE performs RACHless cell switch upon the reception of LTM cell switch command without early TA value.

Proposal 2: If an LTM cell switch command is received for a C-LTM candidate, UE initiates mobility execution to the target cell. It maintains the TA value and keeps the early TA timer running for candidate cell(s) other than the target) cell.

Coexistence cases

Observation 2: It seems unnecessary to configure both CLTM candidate configuration and LTM candidate configuration for the same cell since LTM cell switch command can be used to trigger CLTM execution. But it can be up to network implementation.

Proposal 3: Simultaneous configuration of Intra-CU MCG CLTM and Intra-CU MCG LTM is supported.

Proposal 4: Simultaneous configuration of CHO and CLTM is supported.

Candidate cell/beam selection

Proposal 5: If multiple CLTM candidate cells meet the L1 (or L3) execution condition, the target cell for CLTM execution can be selected among the candidate cells for which the UE has a valid TA. If the valid TA is not available for any candidate cell, the UE can select any candidate cell.

Proposal 6: In the case of CLTM candidate cell with MIMO 2TA, UE prioritizes the beam which has valid early TA and also satisfy the execution condition.

Fast Recovery for CLTM

Proposal 7: After RLF, Intra-gNB MCG LTM failure or CLTM failure, UE can perform cell switch to a selected cell if the selected cell is configured with CLTM configuration.

Fallback from RACH-less

Proposal 8: Not to support Fallback from RACH-less CLTM CS to RACH-based CLTM CS when/ if TAT (for the target cell) expires while T304 is still running.

References

- Chair notes for RAN2#129bis