3GPP TSG-RAN WG2 Meeting #130	R2-2503389

St Julian’s, Malta, 19-23 May 2025

Agenda Item:	8.17

Source:	NEC

Title:	SI transmission for IoT-NTN TDD mode

Document for:	Discussion, Decision

# Introduction

In last RAN1 meeting (RAN1#120bis), the following agreement was reached for SIB1 transmission for the operation of IoT-NTN TDD mode.

Agreement

SIB1-NB is dropped in non-D NB-IoT subframes

NOTE 1: “non-D NB-IoT subframes” are subframes not contained within the set of D=8 usable consecutive downlink subframes in the TDD structure.

NOTE 2: RAN1 observes that some combinations of {TBS, number of repetitions, additional SIB1 repetitions}, and depending on aspects such as minimum elevation angle or antenna gain, may not meet the link budget. It is up to the network to properly configure such values to ensure the link budget is met. No specification impact.

In RAN1#120 meeting, the impacts on the system information other than SIB1-NB were left to RAN2 according to the following conclusion reached:

Conclusion

It is RAN1’s understanding that the issue of scheduling and transmitting SI messages in the presence of non-D subframes will be solved mainly (or completely) by RAN2 specifications. RAN1 may discuss potential impact on RAN1 specifications, if any.

This paper intends to discuss how the other SI message is transmitted for IoT-NTN TDD mode.

# Discussion

RAN1 defined the periodic transmission pattern for IoT-NTN TDD mode as 8 downlink NB-IoT subframes followed by a fixed guard period, and then followed with 8 uplink NB-IoT subframes, where the periodic transmission spans over 9 radio frames (equal to 90 ms).

RAN1 has concluded the transmission of SIB1-NB, in which the SIB1-NB transmission will be dropped in non-D NB-IoT subframes. However the transmission of other SI message is left to RAN2 discussion.

The other SIs for the NB-IoT NTN system include SIB2-NB and SIB31-NB (NTN configuration information). In addition, SIB5-NB and SIB22-NB may be also needed for mobility purpose. These SIBs-NB are transmitted in a SI-Window following the scheduling information (e.g. si-RadioFrameOffset, si-WindowLength, si-Periodicity, si-RepetitionPattern and si-TB) within SIB1-NB. The SI-Window starts at subframe #0 in the radio frame such that: (H-SFN*1024 + SFN) mod T = FLOOR(x/10) + Offset [TS36.331]. The NB-IoT NTN UE accumulates the reception of SI messages on DL-SCH over the usable NPDSCH downlink subframes for the whole SI-window.

Due to the configuration of NPSS/NSSS and NPBCH, a 90ms TDD frame has maximum 6 PSDCH “D NB-IoT subframes” available for the transmission of other SI. Within the SI-window, the corresponding other SI message may be transmitted over 2 or 8 consecutive NB-IoT downlink subframes according to its TBS. It should be noted that the TBS higher than 120 bits always uses 8 consecutive NB-IoT downlink subframes. This implies at least two 90ms IoT-NTN TDD frames are needed to transfer one SIB (in other SI message) such as SIB31-NB. This means, the SIB(in other SI message) can not be hosted by the whole set of D NB-IoT subframes within one TDD radio frame (which is 6 subframes for PSDCH as maximum) may need to transmitted by the next TDD radio frame. Hence logically, the other SI-message transmission can be postponed to the next valid D NB-IoT subframe within the SI-Window.

However, postponing the transmission of the other SI-message transmission from the first TDD radio frame to the next TDD radio frame may fall on the repetitions for the same other SI message transmission, due to the configuration of si-RepetitionPattern within that SI-window. In our understanding, a proper configuration based on network implementation may be employed to avoid such case. Or otherwise, the postponed transmission from the first radio frame to the second radio frame should be discarded by the network to avoid its overlapping with the other repetitions.

Proposal-1: The other SI-message transmission can be postponed to the next valid D NB-IoT subframe within the SI-Window.

Proposal-2: A proper configuration based on network implementation ensure no overlapping between the postponed transmission and the other repetitions in the next TDD radio frame within the SI-Window.  Or otherwise, the postponed transmission from the first radio frame to the second radio frame should be discarded by the network to avoid its overlapping with the other repetitions.

# Conclusion and Proposal

We have the following proposals:

Proposal-1: The other SI-message transmission can be postponed to the next valid D NB-IoT subframe within the SI-Window.

Proposal-2: A proper configuration based on network implementation ensure no overlapping between the postponed transmission and the other repetitions in the next TDD radio frame within the SI-Window.  Or otherwise, the postponed transmission from the first radio frame to the second radio frame should be discarded by the network to avoid its overlapping with the other repetitions.