3GPP TSG-RAN WG2 Meeting #130	R2-2503407

Malta, MT, 19th May – 23th May, 2025

Agenda Item		:	8.5.2

Source		:	LG Electronics Inc.

Title		:	Remaining issues on OD-SSB SCell operation

Document for		:	Discussion and Decision

1. Introduction

In this contribution, we provide our views on remaining issues for OD-SSB.

2. Discussion

2-1. Details on MAC CE format

In the last meeting, RAN2 agreed that MAC CE format of activating/deactivating OD-SSB as Figure 1. For serving cell with ServCellIndex i, transmission of OD-SSB is activated when Ci=1 and is deactivated when Ci=0. For ServCellIndex i when Ci=1, the corresponding additional octet for OD-SSB configuration index field is appended to OD-SSB activation/deactivation MAC CE. The additional octets are appended in order of ServCellIndex. If Ci=0, the corresponding octet for OD-SSB configuration index field is not present.

Figure 1. Example of a new MAC CE format indicating activation/deactivation of OD-SSB transmission. 
(Left: short-form; Right: long-form)

During the last RAN2 meeting, some company have an understanding that UE ignores OD-SSB deactivation indication (i.e., Ci=0) when the serving cell has activated OD-SSB along with the number N of OD-SSB bursts (od-ssb-nrofTx) [1]. In our understanding, the network can indicate the deactivation of OD-SSB via MAC CE even if the network activates the OD-SSB along with od-ssb-nrofTx. In RAN1#120bis, RAN1 agreed that OD-SSB can be deactivated via MAC CE when od-ssb-nrofTx is configured as follows.

RAN1#120bis Agreement Agreement For a cell supporting on-demand SSB SCell operation, for configuring the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb-nrofBurst), - Alt 2: The value range of od-ssb-nrofBurst is {N2 integer values} - N2= [8] - If od-ssb-nrofBurst for an on-demand SSB is NOT configured, the on-demand SSB is deactivated via MAC CE. - If od-ssb-nrofBurst for an on-demand SSB is configured, the on-demand SSB is deactivated based on the configured value for od-ssb-nrofBurst [or via MAC CE].

Therefore, the network should set activation/deactivation bit as 1 during OD-SSB bursts transmission. We want to note that the network shall append the corresponding additional octet to align with the appending rule.

Observation 1	When od-ssb-nrofTx is configured, UE considers that OD-SSB transmission is deactivated when network indicates A/D field as 0 regardless of transmitted number of OD-SSB bursts.

Proposal 1	When od-ssb-nrofTx is configured, network should set A/D field as 1 during OD-SSB transmission.

For OD-SSB activated along with od-ssb-nrofTx, there may be a mismatch of understanding between UE and network about the application of OD-SSB configuration. As the OD-SSB is transmitted to accelerate activation of SCell, the network may want to prolong the OD-SSB transmission when the SCell activation is delayed than expectation. To prolong the OD-SSB transmission, the network may indicate the previous OD-SSB configuration. In this case, the expected UE behaviour is that the UE refresh the number of OD-SSB burst to be transmitted as od-ssb-nrofTx. However, the UE understanding may not be the same with such network’s intention. With MAC CE, it is possible that the UE maintains previously applied configuration and does not refresh the number of OD-SSB bursts to be transmitted.

For example, let us consider the scenario that network indicates the MAC CE consecutively.

- First MAC CE indicating OD-SSB activation on SCell#X with OD-SSB configuration#1

    - OD-SSB configuration#1 contains od-ssb-nrofTx

- Second MAC CE indicating OD-SSB activation on SCell#Y

When the second MAC CE is indicated during SCell#X OD-SSB transmission, the network indicates OD-SSB configuration #1 in the second MAC CE. In that case, the network intends to indicate two different UE behavior. First (Intention-1), the network intends the UE to keep the OD-SSB configuration applied by the first MAC CE. Second (Intention-2), the network intends the UE to re-apply the number of OD-SSB bursts to be transmitted according to the OD-SSB configuration#1. However, the UE cannot distinguish between two network intentions only with the field for OD-SSB configuration.

When the UE behavior is not matched with the network intention, a power waste problem occurs. For Intention-1, the UE performs measurement despite the OD-SSB transmission is terminated. For Intention-2, the network transmits OD-SSB bursts despite the UE does not perform measurement. Therefore, we propose discussing how to indicate the network intention on such case.

Observation 2	UE cannot distinguish the network’s intention whether maintaining or re-applying OD-SSB configuration indicated via MAC CE when the OD-SSB configuration contains od-ssb-nrofTx.

To indicate intention of maintaining the previous configuration, the network may use reserved value of the additional octet. Accordingly, the network indicates the previous OD-SSB configuration index only when it intends to re-apply the OD-SSB configuration. Alternatively, the network reserve a bit to indicate whether OD-SSB configuration shall be applied or not.

Proposal 2	Introduce reserved value into additional octet to indicate that UE maintains previous OD-SSB configuration. FFS how to define reserved value.

2-2. OD-SSB measurement

In the past RAN1 meetings, following agreement is made.

RAN1#120 Agreement Agreement Regarding the relation in terms of frequency location (i.e., center frequency) between the always-on SSB and on-demand SSB, - Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB. - On-demand SSB is not on sync raster - AO-SSB and OD-SSB are located in the same BWP - FFS: Additional conditions - Subject to separate UE capability - Note: UE is not required to measure both AO-SSB and OD-SSB RAN1#120bis Agreement Agreement For the case when the center frequency locations of always-on SSB and on-demand SSB are different, at least the following is supported for QCL between AO-SSB and OD-SSB - SS/PBCH blocks with the same SSB indexes for always-on SSB and on-demand SSB are quasi co-located with respect to Doppler spread, Doppler shift, average gain, average delay, delay spread, and when applicable, spatial RX parameters. - When a signal/channel is configured to be QCLed with a SSB index, the signal/channel is QCLed with the same SSB index of always-on SSB and on-demand SSB (if transmitted) with the same QCL parameters according to existing specifications

If the activated OD-SSB has QCL relation with AO-SSB, UE does not need to perform measurement on both AO-SSB and OD-SSB according to the above agreements. To prevent redundant UE power consumption, the UE should perform measurement on only one SSB among them. As the OD-SSB is activated to support more precise and fast measurement, the UE performs measurements on the OD-SSB in this case.

Observation 3	If activated OD-SSB is QCLed with AO-SSB, UE should not perform measurements on AO-SSB.

Proposal 3	UE performs measurements only on OD-SSB when the activated OD-SSB is QCLed with AO-SSB.

If the activated OD-SSB has no QCL relation with AO-SSB, the UE should perform measurements on AO-SSB as well as OD-SSB even though the UE is not required to measure both AO-SSB and OD-SSB. If proposal 3 is agreeable, the UE behaviour for performing measurement on AO-SSB is different from the QCL relation between AO-SSB and OD-SSB. Therefore, the network should indicate whether the UE performs measurements on AO-SSB or not.

UE is not required to include the SSB based measurement results of the SCell configured with servingCellMO in the measurement report, if the SSB based measurement results of the SCell are not available. Therefore, even if the SSB transmission is deactivated, it will not impact the L3 measurement reporting.

Proposal 4	Even if the servingCellMO is configured for the OD-SSB, UE does not derive the SSB based measurement results of the OD-SSB when the OD-SSB transmission is deactivated.

According to RAN1’s agreement, RAN2 should support signaling of the on-demand SSB and the always-on SSB-based L3 measurements. To analyze the impact on the existing measurement configuration, we should first consider whether the existing measurement configuration can be re-used for OD-SSB/always-on SSB based measurements.

The AO-SSB is transmitted with a long periodicity. Although RAN1 agreed that the AO-SSB is SSB supported in Rel-18 specifications, considering that the primary measurement timing configuration, i.e. smtc1, is used also for measurements on neighbor cells, it does not seem suitable to configure the AO-SSB using the existing smtc1.

The measurement object configuration, i.e. MeasObjectNR, contains also smtc2, smtc3list, and smtc4list, to support the secondary measurement timing which is different from the primary measurement timing for the listed cells. For AO-SSB based SCell measurements, similar signaling can be used, e.g. smtc5. The new measurement timing configuration for AO-SSB is applicable only to an SCell which is associated with the measurement object, so the cell list is not needed.

Proposal 5	Introduce SSB-MTC5 in MeasObjectNR associated with the OD-SSB SCell to configure the measurement timing of AO-SSB.

RAN1 also agreed that multiple periodicities should be support for OD-SSB. It is already possible in the measurement object configuration to configure multiple measurement timing for neighbour cells using smtc3list or smtc4list. The usage of them is limited to IAB-MT and NTN cells, respectively. Therefore, it seems reasonable to introduce smtc6list to configure multiple measurement timing configuration for OD-SSB.

Proposal 6	Introduce SSB-MTC6List in MeasObjectNR associated with the OD-SSB SCell to configure the measurement timing of OD-SSB.

3. Conclusion

In this contribution, following statements are proposed:

2-1. Details on MAC CE format

Observation 1	When od-ssb-nrofTx is configured, UE considers that OD-SSB transmission is deactivated when network indicates A/D field as 0 regardless of transmitted number of OD-SSB bursts.

Proposal 1	When od-ssb-nrofTx is configured, network should set A/D field as 1 during OD-SSB transmission.

Observation 2	UE cannot distinguish the network’s intention whether maintaining or re-applying OD-SSB configuration indicated via MAC CE when the OD-SSB configuration contains od-ssb-nrofTx.

Proposal 2	Introduce reserved value into additional octet to indicate that UE maintains previous OD-SSB configuration. FFS how to define reserved value.

2-2. OD-SSB measurement

Observation 3	If activated OD-SSB is QCLed with AO-SSB, UE should not perform measurements on AO-SSB.

Proposal 3	UE performs measurements only on OD-SSB when the activated OD-SSB is QCLed with AO-SSB.

Proposal 4	Even if the servingCellMO is configured for the OD-SSB, UE does not derive the SSB based measurement results of the OD-SSB when the OD-SSB transmission is deactivated.

Proposal 5	Introduce SSB-MTC5 in MeasObjectNR associated with the OD-SSB SCell to configure the measurement timing of AO-SSB.

Proposal 6	Introduce SSB-MTC6List in MeasObjectNR associated with the OD-SSB SCell to configure the measurement timing of OD-SSB.

4. Reference

[1] R2-2503302, RAN2#129bis Meeting Report.

5. Appendix

Agreements on the latest RAN1/RAN2 meetings

RAN2#129bis Agreement: 1. For explicit activation/deactivation, the OD-SSB MAC-CE includes fixed sized bitmap to indicate whether OD-SSB is activated in each SCell (i.e., similar to legacy SCell A/D MAC-CE). 2. A/D bit: “1” means activation, “0” means deactivation for explicit deactivation case. 3. For explicit activation/deactivation, the OD-SSB MAC-CE supports two formats: one format indicates up to 7 SCells and the other format indicates up to 31 SCells. 4. OD-SSB MAC-CE includes a configuration index for each SCell activating OD-SSB. 5. RAN2 aims to define single MAC CE format for both explicit and implicit OD-SSB deactivations. Details are FFS. 6. Working assumption: When A-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB. FFS if we allow multiple OD-SSBs with the different frequencies for a given SCell.

RAN1#120bis Agreement Agreement For a cell supporting on-demand SSB SCell operation, the following combinations of scenarios and cases are supported for indicating OD-SSB using a MAC-CE. - Scenario #3B and Case #1 - In the above combinations of scenarios and cases, the MAC-CE is used only for updating the transmission parameter of a transmitted OD-SSB for the cell since the OD-SSB has been transmitted according to NW indication. - Scenario #3B and Case #2 - In the above combinations of scenarios and cases, the MAC-CE is used only for updating the transmission parameter of a transmitted OD-SSB for the cell since the OD-SSB has been transmitted according to NW indication. Agreement For a cell supporting on-demand SSB SCell operation, for Case #1 (i.e., No always-on SSB on the cell) UE does not expect the OD-SSB transmission indicated by RRC/MAC-CE to be deactivated while the SCell is activated.