3GPP TSG-RAN WG2 Meeting #130	                                                                                   R2-2503409

St.Julians, Malta, May 19th – 23rd, 2025

Source:	CATT

Title:	Discussion on inter-CU LTM

Agenda Item:    8.6.2

Document for:  Discussion and Decision

Introduction

In this contribution, we further discuss the remaining issues on inter-CU LTM [1]. The contribution is organized as the following. In section 2, we discuss the following aspects,

- On the inclusion of appliedLTM-CandidateID

- Further details on fast recovery

- On the interaction for the configuration of inter-CU SCG LTM candidate

- Clarification on the SK-counter list

Our proposals are summarized in Section 3.

Discussion

On the inclusion of appliedLTM-CandidateID

There is one open issue suggested by the rapporteur as follows,

## 2.2 RRC-2 Editor’s Note: FFS if appliedLTM-CandidateId needs to be included in both MCG and SCG RRCReconfigurationComplete message. Rapporteur comment: Whether this is an issue need to be clarified, but probably good to discuss this based on companies’ contributions (if companies think that current text is not okay).

Regarding this FFS, there are two issues to clarify,

Issue 1: In which cases UE needs to send the appliedLTM-CandidateID to NW upon LTM execution?

For intra-CU MCG/SCG LTM, it had been agreed in R18 to include the appliedLTM-CandidateID in the RRCReconfigurationComplete message to avoid the potential run out of transaction ID, i.e., the same candidate cell is configured as both CHO candidate and LTM candidate or the one candidate PCell/PSCell is associated with multiple candidate configurations. The similar cases are also applied to inter-CU MCG/SCG LTM. Therefore, to differentiate the candidate configuration by the NW, the appliedLTM-CandidateID is needed for R18 intra-CU MCG/SCG LTM and R19 inter-CU MCG/SCG LTM.

Proposal 1a: (RRC-2) UE needs to include appliedLTM-CandidateID in RRCReconfigurationComplete upon all cases of LTM execution, i.e.,

- Upon R18 intra-CU MCG/SCG LTM

- Upon inter-CU MCG LTM

- Upon inter-CU SCG LTM

- Upon intra-CU CLTM

Issue 2: To which node the UE should include the appliedLTM-CandidateID in the RRCReconfigurationComplete message?

We understand the general principle could be that appliedLTM-CandidateID is sent to the node which generated the candidate configuration to UE, e.g., appliedLTM-CandidateID is included in the RRCReconfigurationComplete to MN upon inter-CU MCG LTM/inter-CU SCG LTM due to the ltm-CandidateId and the LTM configuration is included in the message generated by the MN. More details for each case are as follows,

Table I: On the inclusion of appliedLTM-CandidateID in RRCReconfigurationComplete

| Scenario | Configuration | Remarks |
| --- | --- | --- |
| R18 intra-CU MCG LTM/R19 CLTM | Via SRB1 | appliedLTM-CandidateID needs to be included in the RRCReconfigurationComplete corresponding to the applied RRCReconfiguration which is generated by MN |
| R18 intra-CU SCG LTM | Via SRB3 | appliedLTM-CandidateID needs to be included in the RRCReconfigurationComplete corresponding to the applied RRCReconfiguration which is generated by SN |
|  | Via SRB1 within nr-SCG within mrdc-SecondaryCellGroup |  |
| R19 inter-CU MCG LTM | Via SRB1 | appliedLTM-CandidateID needs to be included in the RRCReconfigurationComplete corresponding to the MCG RRCReconfiguration but not included in the RCReconfigurationComplete corresponding to the SCG RRCReconfiguration |
| R19 inter-CU SCG LTM | Via SRB1 | appliedLTM-CandidateID needs to be included in the RRCReconfigurationComplete corresponding to the MCG RRCReconfiguration but not included in the RCReconfigurationComplete corresponding to the SCG RRCReconfiguration |

Therefore, it is proposed that,

Proposal 1b: (RRC-2) The appliedLTM-CandidateID is sent to the node which generated the candidate configuration to UE, i.e.,

- included in RRCReconfigurationComplete to MN upon inter-CU MCG LTM

- included in RRCReconfigurationComplete to MN upon inter-CU SCG LTM.

- included in RRCReconfigurationComplete to MN upon intra-CU MCG LTM

- included in RRCReconfigurationComplete to SN upon intra-CU SCG LTM

Proposal 1c: TP in Annex 1 is adopted if p1a and p1b are agreed.

Further details on fast recovery

Whether to support fast recovery with different Rel-19 IDs

In the last meeting, it remains open on whether to support fast recovery to the candidate cells with different R19 set ID,

## 2.8 RRC-8 FFS if fast failure recovery with different Rel-19 IDs is allowed. Rapporteur comment: This is an open issue from last RAN2 meeting and, even if we don’t support the case mentioned in the FFS, things should work fine anyway. However, our suggestion is to discuss this based on companies’ contribution.

As per the RAN3 progress,

For the initial inter-CU LTM preparation, the source gNB computes the KgNB*(s) (per candidate cell) for the candidate gNB, and forwards the {KgNB*, NCC} pair(s) to the candidate gNB via a Handover Request message as legacy. The new source gNB needs to provide the new KgNB*(s) to the corresponding candidate gNB via a LTM configuration update procedure. FFS on the procedure design either per cell or per gNB

For both initial LTM and subsequent LTM, the candidate cell can get the {KgNB*, NCC} pairs via the Handover Request or LTM configuration update procedure. UE could derive the new KgNB based on the NCC received via LTM cell switch command, and the selected cell’s frequency and PCI. So we think there is no block on the support of LTM fast recovery to the candidate cell with a different R19 ID to the target cell indicated in the LTM command.

Proposal 2: (RRC-8) Support the LTM fast recovery to a candidate cell that different R19 ID from the target cell after inter-CU LTM failure.

Another issue is whether the candidate cell with the same R19 ID as the source cell should be excluded for for LTM fast recovery after the inter-CU MCG LTM failure, considering it also belongs to the candidate cell with different R19 ID of the target cell. In legacy, if UE receives the NCC, UE should update the security with Horizontal derivation or Vertical derivation no matter the CU is changed or not. The same principle could be used for LTM. If the selected cell is one candidate cell with the same R19 ID as the source cell after the inter-CU MCG LTM failure, the UE could perform LTM fast recovery to this candidate cell with the security key update with the received NCC included in the LTM CSC. And from the network perspective, the CU of the source cell knows the UE performed the inter-CU MCG LTM with the NCC included in the LTM CSC, so it could perform key update with the NCC if the UE perform access to one candidate cell of the source CU.  Hence it is feasible to perform fast recovery to the candidate cell of the source CU upon inter-CU MCG LTM failure.

Proposal 3a: (RRC-8) If the selected cell has same R19 ID as the source cell after the inter-CU MCG LTM failure, UE perform LTM fast recovery to the selected cell with the key update using the received NCC included in the LTM cell switch command and the selected cell’s PCI and frequency.

Finally, it needs to further clarify how to acquire the security key if the selected cell has the same R19 ID with the target cell after the inter-CU MCG LTM failure, i.e., whether the UE should derive the new key based on the NCC received in the LTM command or just use the key that had been derived for the inter-CU MCG LTM before LTM failure.

At RLF and reconfiguration with sync failure: - If RLF: If the selected candidate cell has the same Rel-19 ID as source (no security key change), the UE performs fast RLF recovery (same as in Rel-18). - if reconfiguration failure (inter-CU LTM): If the selected candidate cell has the same Rel-19 ID as target, the UE performs fast failure recovery. FFS if fast failure recovery with different Rel-19 IDs is allowed.

Considering a unified solution for all candidate cells no matter to which CU it belongs for inter-CU MCG LTM failure, we propose to derive new key based on the selected cell even the selected cell has the same R19 ID as the target cell after the inter-CU MCG LTM failure.

Proposal 3b: If the selected cell has same R19 ID as the target cell after the inter-CU MCG LTM failure, UE performs LTM fast recovery to the selected cell with the security key update using the NCC in the LTM cell switch command and the selected cell’s PCI and frequency.

Revert back to the source configuration

On the revert back to the source configuration upon T304 expires, the current RRC running CR captures the following description,

2> else: 3> if attemptLTM-Switch is configured and the T304 of the MCG expiry is caused by an LTM cell switch execution triggered by an indication from lower layers as described in 5.3.5.18.6: 4> if the LTM cell switch execution procedure does not require a security key refresh: 5> revert back to the UE configuration used in the source Pcell except for the PDCP state variables for SRB(s) associated to the MCG; 3> else if the T304 of the MCG expiry is caused by an LTM cell switch following cell selection performed while timer T311 was running, as specified in 5.3.7.3: 4> revert back to the UE configuration used in the source PCell (i.e., the source Pcell prior to the starting of timer T311) except for the PDCP state variables for SRB(s) associated to the MCG; Editor’s Note: FFS on which point in time, for fast recovery, the UE reverts back to the previous PCell configuration. 3> else: 4> revert back to the UE configuration used in the source Pcell;

Then the rapporteur put an open issue for this FFS,

## 2.6 RRC-6 Editor’s Note: FFS on which point in time, for fast recovery, the UE reverts back to the previous PCell configuration. Rapporteur comment: It is still not crystal clear how the fast recovery procedure would work, and especially when the UE should revert back to the previous PCell configuration. We suggest to discuss this based on companies’ contribution.

There are two issues on the above description.

Issue 1 – redundancy UE behaviour on reverting back to the source configuration

The above highlight part mentioned a scenario where the UE fails to access to the selected cell after determining a suitable cell while T311 is running, i.e., fast recovery failure. In this situation, the UE behaviour is covered by the else condition where UE revert back to the source PCell configuration to perform RRC re-establishment by sending the RRCReestablishmentRequest, because UE can only attempt a single LTM fast recovery operation. So the highlight description should be deleted.

Observation 1: UE behaviour on reverting back to the source PCell configuration upon fast recovery failure has been covered by the legacy procedure text.

Proposal 4a: (RRC-6) Rely on legacy procedure text for the UE behaviour on reverting back to the source PCell configuration upon fast recovery failure, i.e., the related change in the running CR should be removed.

Issue 2 – miss the UE behaviour on reverting back to the source configuration when security key refresh is required for LTM cell switch

If the security key refresh is performed upon the LTM execution, the NCC value used for the target cell security key update will be indicated in the enhance LTM cell switch command MAC CE. Since at least the fast recovery to the candidate cell with the same R19 set ID with the target cell indicated in LTM command is supported, UE should keep the NCC to derive new security for the selected cell. Furthermore as our above proposals, if the selected cell is one LTM candidate cell no matter which CU it belongs to, UE could perform the fast recovery with security update using the received NCC, hence the NCC should be kept.

Observation 2: UE needs to revert back to the source PCell configuration upon T304 expires caused by LTM cell switch execution procedure which requires a security key refresh. The related UE behaviour is missing in the RRC running CR.

In summary, it is proposed that,

Proposal 4b: (RRC-6) Upon T304 expires caused by LTM cell switch execution procedure which requires a security key refresh, UE needs to revert back to the source PCell configuration but keeps NCC received in the LTM command.

Proposal 4c: (RRC-6) TP in Annex 2 is adopted if P4b is agreed.

On the interaction for the configuration of inter-CU SCG LTM candidate

In last meeting we has agreed to include one indication to indicate the source SN whether inter-CU SCG LTM is allowed. Another issue is how many candidate PSCell could be prepared for inter-CU SCG LTM which is also discussed in RAN3. From RAN2 perspective, now the inter-node RRC message has introduced the following field,

maxNumberLTM-CandidatesSCG Indicates the maximum number of LTM candidate configurations that the SN is allowed to configure. If the field is absent the SN is not allowed to configure LTM candidate configurations. This field is only used in NR-DC.

The above indication is not limited to the intra-CU SCG LTM. So we can reuse this field to indicate the candidate PSCell for SCG LTM no matter it is inter-CU SCG LTM or intra-CU LTM, i.e., no need to introduce separate indication on the maximum LTM candidate configuration number for inter-CU SCG LTM.

Proposal 5a: The legacy field maxNumberLTM-CandidatesSCG in the inter-node RRC message can be used to indicate the maximum number of the candidate configuration for SCG LTM no matter intra-CU or inter-CU. Additional indication for inter-CU SCG LTM is not needed.

Proposal 5b: Send LS to RAN3 if P5a is agreed.

Clarification on the SK-counter list

In the current RRC running CR, it captures the following editor note upon LTM execution,

2> if the LTM cell switch is triggered on the MCG: 3> update the master security key by performing the AS security key update procedure as specified in 5.3.5.7; 3> if the LTM-Candidate IE indicated by lower layers does not include an mrdc-SecondaryCellGroupConfig set to release: 4> consider the first sk-Counter value in the ltm-SK-Counters within the VarLTM-ServingCellNoSecurityChangeID according to the field ltm-NoSecurityChangeID as the selected sk-Counter value, and perform security key update procedure as specified in 5.3.5.7; 4> remove the selected sk-Counter value from the ltm-SK-Counters within the VarLTM-ServingCellNoSecurityChangeID; Editor’s note: FFS whether the list of sk-counters can be used only for the case of inter-MN LTM.

Then the corresponding open issue is captured as follows,

## 2.7 RRC-7 Editor’s note: FFS whether the list of sk-counters can be used only for the case of inter-MN LTM. Rapporteur comment: This is something that we never discussed. Current Running CR re-use the list of sk-counters also for the case of inter-MN LTM (to support subsequent LTM) so our suggestion is to keep current text.

In Previous RAN2 meetings, we have agreed the SK counter list is used for inter-CU SCG LTM, according to the following agreement,

For inter-CU SCG LTM, introducing the SK counter list in LTM-ConfigSCG-r19, each list associated with one ltm-NoSecurityChangeID.

And for SN key regeneration in inter-CU MCG LTM, we have agreed that UE uses the configured sk-Counter value in  LTM candidate configurations,according to the following agreements,

For the SN key update in inter-CU MN LTM with DC, the UE applies legacy R15 RRC reconfiguration with sync procedure. NW configures the corresponding sk-Counter in all LTM candidate configurations, and UE uses the configured value for generating the SN key when security key update is performed in MCG.

So it should be straightforward to confirm that the SK counter list is not applied to inter-CU MCG LTM. Therefore, it is proposed that,

Proposal 6: Confirm that the SK counter list is only applied to inter-CU SCG LTM.

Conclusion

Based on the previous analysis in Section 2, our observations and proposals are summarized as follows:

Observation 1: UE behaviour on reverting back to the source PCell configuration upon fast recovery failure has been covered by the legacy procedure text.

Observation 2: UE needs to revert back to the source PCell configuration upon T304 expires caused by LTM cell switch execution procedure which requires a security key refresh. The related UE behaviour is missing in the RRC running CR.

On the inclusion of appliedLTM-CandidateID

Proposal 1a: (RRC-2) UE needs to include appliedLTM-CandidateID in RRCReconfigurationComplete upon all cases of LTM execution, i.e.,

- Upon R18 intra-CU MCG/SCG LTM

- Upon inter-CU MCG LTM

- Upon inter-CU SCG LTM

- Upon intra-CU CLTM

Proposal 1b: (RRC-2) The appliedLTM-CandidateID is sent to the node which generated the candidate configuration to UE, i.e.,

- included in RRCReconfigurationComplete to MN upon inter-CU MCG LTM

- included in RRCReconfigurationComplete to MN upon inter-CU SCG LTM.

- included in RRCReconfigurationComplete to MN upon intra-CU MCG LTM

- included in RRCReconfigurationComplete to SN upon intra-CU SCG LTM

Proposal 1c: TP in Annex 1 is adopted if p1a and p1b are agreed.

Further details on fast recovery

Proposal 2: (RRC-8) Support the LTM fast recovery to a candidate cell that different R19 ID from the target cell after inter-CU LTM failure.

Proposal 3a: (RRC-8) If the selected cell has same R19 ID as the source cell after the inter-CU MCG LTM failure, UE perform LTM fast recovery to the selected cell with the key update using the received NCC included in the LTM cell switch command and the selected cell’s PCI and frequency.

Proposal 3b: If the selected cell has same R19 ID as the target cell after the inter-CU MCG LTM failure, UE performs LTM fast recovery to the selected cell with the security key update using the NCC in the LTM cell switch command and the selected cell’s PCI and frequency.

Proposal 4a: (RRC-6) Rely on legacy procedure text for the UE behaviour on reverting back to the source PCell configuration upon fast recovery failure, i.e., the related change in the running CR should be removed.

Proposal 4b: (RRC-6) Upon T304 expires caused by LTM cell switch execution procedure which requires a security key refresh, UE needs to revert back to the source PCell configuration but keeps NCC received in the LTM command.

Proposal 4c: (RRC-6) TP in Annex 2 is adopted if P4b is agreed.

On the interaction for the configuration of inter-CU SCG LTM candidate

Proposal 5a: The legacy field maxNumberLTM-CandidatesSCG in the inter-node RRC message can be used to indicate the maximum number of the candidate configuration for SCG LTM no matter intra-CU or inter-CU. Additional indication for inter-CU SCG LTM is not needed.

Proposal 5b: Send LS to RAN3 if P5a is agreed.

Clarification on the SK-counter list

Proposal 6: Confirm that the SK counter list is only applied to inter-CU SCG LTM.

Reference

[1] Draft_RAN2_129bis_Meeting_Report_v2

Annex 1

2>	if this RRCReconfiguration message is applied due to an LTM cell switch execution procedure according to clause 5.3.5.18.6:

3>if this RRCReconfiguration message is received via SRB3, or

3>if this RRCReconfiguration message is received via SRB1 and included in the ltm-Config within nr-SCG within mrdc-SecondaryCellGroup, or

3>if this RRCReconfiguration message is received via SRB1 and not within nr-SCG within mrdc-SecondaryCellGroup

4>	include in the appliedLTM-CandidateId the LTM-CandidateId of the applied LTM candidate configuration;

Annex 2

2>	else:

3>	if attemptLTM-Switch is configured and the T304 of the MCG expiry is caused by an LTM cell switch execution as described in 5.3.5.18.6 and 5.3.5.18.x:

4>	if the LTM cell switch execution procedure does not require a security key refresh:

5>	revert back to the UE configuration used in the source Pcell except for the PDCP state variables for SRB(s) associated to the MCG;

4>	else if the LTM cell switch execution procedure requires a security key refresh:

5>	revert back to the UE configuration used in the source PCell and keep the NCC value indicated from lower layer;

3>	else:

4>	revert back to the UE configuration used in the source Pcell;