3GPP TSG-RAN WG2 Meeting #130                                                               R2-2503414

St Julian, Malta, 19th – 23rd May, 2025

Source:	CATT

Title:	Consideration on on-demand SSB SCell operation

Agenda Item:	8.5.2

Document for:	Discussion and Decision

Introduction

In last RAN3#129bis meeting [1], on-demand SSB was discussed with the following agreements:

Agreements on OD-SSB

- For explicit activation/deactivation, the OD-SSB MAC-CE includes fixed sized bitmap to indicate whether OD-SSB is activated in each SCell (i.e., similar to legacy SCell A/D MAC-CE).

- A/D bit: “1” means activation, “0” means deactivation for explicit deactivation case.

- For explicit activation/deactivation, the OD-SSB MAC-CE supports two formats: one format indicates up to 7 SCells and the other format indicates up to 31 SCells.

- OD-SSB MAC-CE includes a configuration index for each SCell activating OD-SSB.

- RAN2 aims to define single MAC CE format for both explicit and implicit OD-SSB deactivations. Details are FFS.

- Working assumption: When A-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB. FFS if we allow multiple OD-SSBs with the different frequencies for a given SCell.

In this contribution, we share our view on OD-SSB L3 measurement and the coexistence of explicit and implicit OD-SSB deactivation mechanisms in Section 2. Our proposals are summarized in Section 3.

Discussion

OD-SSB measurement of SCell

Measurement configuration

At the last meeting, one remaining essential open issue was that if we allow multiple OD-SSBs with different frequencies for a given SCell. Current agreement allows that if always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB, which enables traditional UE to connect to SCell through AO-SSB. However, there is no clear motivation to configure multiple OD-SSBs with different frequencies for a given SCell. And it will introduce additional spec impact if we support this scenario. Therefore, multiple OD-SSBs with different frequencies for a given SCell should not be allowed.

Proposal 1: For a given SCell, multiple OD-SSBs with different frequencies are not allowed.

In the last RAN1#120bis meeting, the relationship of spatial domain of OD-SSB and AO-SSB were discussed with the following agreements.

Agreement of RAN1 For a cell supporting on-demand SSB SCell operation, - Frequency of the on-demand SSB (i.e., od-ssb-absoluteFrequency) can be absent for Case #2. - If absent, the center frequency of on-demand SSB is the same as that of always-on SSB. - od-ssb-PositionsInBurst, if provided, has the same bitmap structure as ssb-PositionsInBurst provided in ServingCellConfigCommon, that is od-ssb-PositionsInBurst is choice of 4-bit shortBitmap, 8-bit mediumBitmap, or 64-bit longBitmap. - od-ssb-PositionsInBurst can be absent for Case #2. - If absent, od-ssb-PositionsInBurst is the same as ssb-PositionsInBurst provided in ServingCellConfigCommon.

According to the above agreement, OD-SSB and AO-SSB have the same bitmap structure as ssb-PositionsInBurst provided in ServingCellConfigCommon, but the od-ssb-PositionsInBurst of OD-SSB and the ssb-PositionsInBurst of AO-SSB may be different. In the existing specification, ssb-ToMeasure is configured in MeasObjectiveNR, which indicates the set of SSBs to be measured within the SMTC measurement duration. For Case2, if the od-ssb-PositionsInBurst of OD-SSB is different from the ssb-PositionsInBurst of AO-SSB, the ssb-ToMeasure may not be applicable after OD-SSB is activated.

One solution is to introduce a new ssb-ToMeasure for OD-SSB. However, since the od-ssb-PositionsInBurst may change dynamically, there may be more than one new ssb-ToMeasure that needs to be introduced and the relationship between the new ssb-ToMeasures for OD-SSB and the received new MAC CE for OD-SSB transmission need to be considered. Another solution is that UE uses od-ssb-PositionsInBurst to measure OD-SSB directly, which is configured within the OD-SSB configuration. For simplicity, we think that OD-SSB measurements based on OD-SSB PositionsInBurst should be supported.

Proposal 2: For the measurement of OD-SSB, UE determines the set of OD-SSB to be measured based on od-ssb-PositionsInBurst (i.e., not based on ssb-ToMeasure).

L3 RRM framework

In the RAN2#127bis meeting, the details on L3 RRM framework were discussed and the following two options were summarized.

- Option1: Based on different measurement configuration when OD-SSB is transmitted

- Option2: Based on OD-SSB pattern ignoring SMTC when OD-SSB is transmitted

During the previous RAN4#112, 113 and 114bis meetings, the issue of OD-SSB based deactivated SCell measurement was discussed, and RAN4 have the following agreements:

Agreement of RAN4#112meeting: Issue 1-2-1: OD-SSB based deactivated SCell measurement(Case 1) Once UE receives the OD-SSB activation indication, UE performs the OD-SSB based fast L3 measurement for deactivated SCell within a time window. Agreement of RAN4#113 meeting: Issue 1-2-1: OD-SSB based deactivated SCell measurement – measurement periodicity in fast mode measurement For the case of CSSF = 1: - OD-SSB periodicity regardless of measCycleSCell and DRX cycle. - FFS whether a lower bound will be specified. FFS for the other cases of CSSF. Sub-issue 1 – How to perform measurement after time window within one time OD-SSB activation/deactivation? - After the fast time window, the UE shall meet the legacy deactivated SCell measurement requirement as long as OD-SSB is activated/transmitted from UE’s perspective. - The agreement can be revisited based on further RAN1 agreement about OD-SSB deactivation mechanism Agreement of RAN4#114bis meeting - The UE follows legacy requirements based on measCycleSCell if UE has sent a valid measurement report before OD-SSB activation indication within the time window as follow. - max(5*measCycleSCell, 5*DRX cycles) for FR1, or - 4s for UE supporting power class1 and 3s for UE supporting power class 2/3/4 in FR2. - Otherwise, the same approach in Case 1 can be reused. - Note: The agreement is applied when OD-SSB and AO-SSB are in the same frequency and with same spatial relation. - Note 2: T2 discussion is a separate issue.

According to the above agreement, a fast time window is defined by RAN4. Within the fast time window, UE performs the OD-SSB based fast L3 measurement for deactivated SCell, i.e., the measurement period in fast time window is related to OD-SSB period regardless of measCycleSCell and DRX cycle.

Observation 1: A fast time window is defined by RAN4, during which UE performs the OD-SSB based fast L3 measurements for deactivated SCell within a measurement period associated with the OD-SSB period.

For Option 1, the multiple introduced new SMTCs (SMTC-list) are related to the OD-SSB patterns. Furthermore, when implementing Option 1, it is essential to establish a corresponding relationship between the SMTC-list for OD-SSB and the received new MAC CE for OD-SSB transmission. While Option 1 achieves consistent performance with Option 2 in terms of functionality, it introduces higher implementation complexity due to the required coordination mechanisms.

In addition, according to our analysis on measurement configuration, for the set of OD-SSB to be measured, OD-SSB measurements based on od-ssb-PositionsInBurst is a more suitable approach. Therefore, Option 2 demonstrates greater suitability compared to Option 1.

Proposal 3: The RRM measurement of OD-SSB is performed based on OD-SSB pattern while ignoring SMTC when OD-SSB is transmitted.

Additional trigger of L3 measurement reporting

In the last RAN4#114 meeting, following agreements have been achieved on OD-SSB based deactivated SCell L3 measurement of Case#2.

Agreement of RAN4 - The UE follows legacy requirements based on measCycleSCell if UE has sent a valid measurement report before OD-SSB activation indication within the time window as follow. - max(5*measCycleSCell, 5*DRX cycles) for FR1, or - 4s for UE supporting power class1 and 3s for UE supporting power class 2/3/4 in FR2. - Otherwise, the same approach in Case 1 can be reused. - Note: The agreement is applied when OD-SSB and AO-SSB are in the same frequency and with same spatial relation. - Note 2: T2 discussion is a separate issue.

It can be seen that when there are no valid measurement report, after receiving the OD-SSB transmission indication, UE performs the OD-SSB based fast L3 measurement following OD-SSB periodicity for deactivated SCell within a time window.

Observation 2: For Case#2, if there is no valid measurement report, after receiving the OD-SSB transmission indication, UE performs the OD-SSB based fast L3 measurement following OD-SSB periodicity for deactivated SCell within a time window.

According to RAN4 [2], the activation delay of the unknown cell is larger than that of the known cell. And if UE has not sent a valid measurement report within the above-mentioned time window, the SCell is an unknown cell. Therefore, in order for UE to quickly report measurement results after receiving the OD-SSB transmission indication, to turn an unknown cell into a known cell, a new trigger condition of measurement reporting should be introduced, i.e., UE triggers measurement reporting after the OD-SSB is activated via MAC CE.

Proposal 4: In order for UE to quickly report measurement results after receiving the OD-SSB transmission indication, a L3 measurement reporting is triggered after the OD-SSB is activated via MAC CE.

OD-SSB measurement of neighbor cell

According to the agreement of “the UE expects to measure SSB when on-demand SSB transmission is activated”, there is a question about whether UE can measure the OD-SSB of a neighbor cell when the OD-SSB transmission of the neighbor cell is activated. According to the WID [3], OD-SSB transmission is only applicable to SCell. Indicating the OD-SSB transmission of the neighbor cell to the UE is not supported.

Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4]

- Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling)

- Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

In addition, during RAN4#114 meeting [5], following agreements have been achieved on neighbour cell measurement.

Issue 7-2: OD-SSB based Neighbour cell measurement - Scenario 1a: There is OD-SSB neighbor cell in the same MO with legacy serving cell Agreement: - For Scenario 1a: No RAN4 spec. change. - Scenario 1b: Neighbour cell with legacy SSB in the same MO with OD-SSB serving cell Agreement: - For FR1, follow legacy requirement for neighbour cell measurement with legacy SSB. - The agreement can be revisited if not aligned with the further progress in other WGs. - Further discuss for FR2. - Scenario 1c: There is OD-SSB neighbor cell in the same MO with OD-SSB serving cell Agreement: - For FR1, follow legacy requirement for neighbour cell measurement. - The agreement can be revisited if not aligned with the further progress in other WGs. - Further discuss for FR2.

Based on the agreement, RAN4 will not update the neighbour cell L3 measurement requirement due to the OD-SSB transmission. This means that UE will continue to measure SSB using the always-on SSB measurement configuration, even if OD-SSB transmission is activated. Therefore, OD-SSB SCell operation has no impacts to RRM measurement on neighbor cell.

Proposal 5: OD-SSB SCell operation has no impact to RRM measurement of neighbor cells.

Handling of OD-SSB measurement after SCell deactivation

In the previous RAN1 meeting, it was discussed whether OD-SSB still transmits when SCell is deactivated, but no consensus was reached.

Agreement RAN1#118b For a cell supporting on-demand SSB SCell operation, deactivation of on-demand SSB transmission is supported. In order to deactivate on-demand SSB transmission from a UE perspective, support at least one of the following options. - Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Option 1A: Explicit indication of deactivation for on-demand SSB via RRC for on-demand SSB transmission indication - Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated - Option 3: Configuration/indication of the duration of on-demand SSB transmission window - Option 4: On-demand SSB transmission, if any, is deactivated when UE receives SCell deactivation MAC-CE for the activated SCell - Option 4A: On-demand SSB transmission, if any, is deactivated when the timer for SCell deactivation is expired - Option 5: On-demand SSB transmission, if any, is deactivated when SCell activation is completed - Option 6: Explicit indication of deactivation for on-demand SSB via [group-common] DCI - FFS: Each option is applicable to which Cases or Scenarios - FFS: Details related to each of the above options

Regardless of whether OD-SSB is still transmitted, it is meaningless for UE to measure OD-SSB after SCell deactivation. Therefore, UE should stop measuring OD-SSB after SCell deactivation.

Proposal 6: After SCell deactivation, UE stops the OD-SSB measurement and considers the OD-SSB as deactivated.

Mixture of explicit and implicit OD-SSB deactivations

In the last RAN2 meeting, it was agreed that RAN2 aims to define single MAC CE format for both explicit and implicit OD-SSB deactivations.

RAN2 aims to define single MAC CE format for both explicit and implicit OD-SSB deactivations. Details are FFS.

To support the mixture of explicit and implicit OD-SSB deactivations, it is necessary to consider the impact on the SCell configured with implicit OD-SSB deactivation. The following cases need to be considered:

- Case 1: The OD-SSB of SCell A configured with implicit OD-SSB deactivation is still in an active state,  while the MAC CE activates/deactivates the OD-SSB of other cell(s) (e.g., SCell B)

- Case 2: The OD-SSB of SCell A configured with implicit OD-SSB deactivation is already in a deactivated state, while the MAC CE activates/deactivates the OD-SSB of other cell(s) (e.g.,SCell B)

- Case 3: The OD-SSB of SCell A configured with implicit OD-SSB deactivation is still in an active state, while the MAC CE deactivates the OD-SSB of SCell A

- Case 4: The OD-SSB of SCell A configured with implicit OD-SSB deactivation is still in an active state, while the MAC CE changes the associated configuration index of the OD-SSB of SCell A

- Case 5: The OD-SSB of SCell A configured with implicit OD-SSB deactivation is already in a deactivated state, while the MAC CE activates the OD-SSB of SCell A

Proposal 7a: The following cases are supported for the mixture of explicit and implicit OD-SSB deactivations:

- Case 1: The OD-SSB of SCell A configured with implicit OD-SSB deactivation is still in an active state,  while the MAC CE activates/deactivates the OD-SSB of other cell(s) (e.g., SCell B)

- Case 2: The OD-SSB of SCell A configured with implicit OD-SSB deactivation is already in a deactivated state, while the MAC CE activates/deactivates the OD-SSB of other cell(s) (e.g.,SCell B)

- Case 3: The OD-SSB of SCell A configured with implicit OD-SSB deactivation is still in an active state, while the MAC CE deactivates the OD-SSB of SCell A

- Case 4: The OD-SSB of SCell A configured with implicit OD-SSB deactivation is still in an active state, while the MAC CE changes the associated configuration index of the OD-SSB of SCell A

- Case 5: The OD-SSB of SCell A configured with implicit OD-SSB deactivation is already in a deactivated state, while the MAC CE activates the OD-SSB of SCell A

Considering the above cases, the indication content of MAC CE and the behavior of UE upon receiving the MAC CE can be:

| Cases | Content of MAC CE | UE behavior |
| --- | --- | --- |
| Case 1: The OD-SSB of SCell A configured with implicit OD-SSB deactivation is still in an active state, while the MAC CE activates/deactivates the OD-SSB of other cell(s) (e.g., SCell B) | Option1: The corresponding bit for SCell A in the bitmap is set to “1”, and the configuration index which is the same as the previous one for SCell A is included Option2: The corresponding bit for SCell A in the bitmap is “0” | Keep the OD-SSB transmission of the SCell A |
| Case 2: The OD-SSB of SCell A configured with implicit OD-SSB deactivation is already in a deactivated state, while the MAC CE activates/deactivates the OD-SSB of other cell(s) (e.g.,SCell B) | The corresponding bit for SCell A in the bitmap is set to “0” | Keep the OD-SSB in a deactivated state of the SCell A |
| Case 3: The OD-SSB of SCell A configured with implicit OD-SSB deactivation is still in an active state, while the MAC CE deactivates the OD-SSB of SCell A | The corresponding bit for SCell A in the bitmap is set to “0” | Deactivate the OD-SSB transmission of SCell A |
| Case 4: The OD-SSB of SCell A configured with implicit OD-SSB deactivation is still in an active state, while the MAC CE changes the associated configuration index of the OD-SSB of SCell A | The corresponding bit for SCell A in the bitmap is set to “1”, and the configuration index which is different form the previous one for SCell A is included | Deactivate the original OD-SSB transmission of SCell A, and activate a new OD-SSB configuration index |
| Case 5: The OD-SSB of SCell A configured with implicit OD-SSB deactivation is already in a deactivated state, while the MAC CE activates the OD-SSB of SCell A | The corresponding bit for SCell A in the bitmap is set to “1”, and the configuration index which is the same as or different from the previous one for SCell A is included | Activate the OD-SSB configuration index indicated by MAC CE |

Based on the above table, there are two options for Case1. However, supporting Option 2 would lead to a conflict between Case 1 and Case 3. Therefore, Option 1 should be adopted for Case 1.

As analyzed above, it can be seen that NW can indicate UE to keep or change the OD-SSB transmission in the OD-SSB transmission MAC CE. Specifically, when the content of the MAC CE indication is the same as previous and the indicated OD-SSB transmission state aligns with the state prior to receiving the MAC CE, the transmission of OD-SSB shall be kept. Otherwise, the UE shall change the transmission of OD-SSB according to the new MAC CE indication, such as changing the OD-SSB state or the configuration index.

Proposal 7b: For a SCell configured with implicit OD-SSB deactivation，NW can indicate UE to keep or change the OD-SSB transmission state in the OD-SSB MAC CE.

Proposal 7c: For a SCell configured with implicit OD-SSB deactivation, UE handles the OD-SSB state according to OD-SSB MAC CE if received. Otherwise, UE handles the OD-SSB state according to the implicit OD-SSB deactivation configuration.

Proposal 7d: If a SCell is configured with implicit OD-SSB deactivation and UE receives OD-SSB MAC CE, the UE shall treat the transmission of OD-SSB of the SCell as unchanged if the content of the MAC CE for the SCell (including the configuration index and activation/deactivation state) is the same as the current activated OD-SSB info for the SCell.

Conclusion

According to the analysis in section 2, our observations and proposals are summarized as follows:

Observation 1: A fast time window is defined by RAN4, during which UE performs the OD-SSB based fast L3 measurements for deactivated SCell within a measurement period associated with the OD-SSB period.

Observation 2: For Case#2, if there is no valid measurement report, after receiving the OD-SSB transmission indication, UE performs the OD-SSB based fast L3 measurement following OD-SSB periodicity for deactivated SCell within a time window.

OD-SSB measurement of Scell

Proposal 1: For a given SCell, multiple OD-SSBs with different frequencies are not allowed.

Proposal 2: For the measurement of OD-SSB, UE determines the set of OD-SSB to be measured based on od-ssb-PositionsInBurst (i.e., not based on ssb-ToMeasure).

Proposal 3: The RRM measurement of OD-SSB is performed based on OD-SSB pattern while ignoring SMTC when OD-SSB is transmitted.

Proposal 4: In order for UE to quickly report measurement results after receiving the OD-SSB transmission indication, a L3 measurement reporting is triggered after the OD-SSB is activated via MAC CE.

OD-SSB measurement of neighbor cell

Proposal 5: OD-SSB SCell operation has no impact to RRM measurement of neighbor cells.

Handling of OD-SSB measurement after SCell deactivation

Proposal 6: After SCell deactivation, UE stops the OD-SSB measurement and considers the OD-SSB as deactivated.

Mixture of explicit and implicit OD-SSB deactivations

Proposal 7a: The following cases are supported for the mixture of explicit and implicit OD-SSB deactivations:

- Case 1: The OD-SSB of SCell A configured with implicit OD-SSB deactivation is still in an active state,  while the MAC CE activates/deactivates the OD-SSB of other cell(s) (e.g., SCell B)

- Case 2: The OD-SSB of SCell A configured with implicit OD-SSB deactivation is already in a deactivated state, while the MAC CE activates/deactivates the OD-SSB of other cell(s) (e.g.,SCell B)

- Case 3: The OD-SSB of SCell A configured with implicit OD-SSB deactivation is still in an active state, while the MAC CE deactivates the OD-SSB of SCell A

- Case 4: The OD-SSB of SCell A configured with implicit OD-SSB deactivation is still in an active state, while the MAC CE changes the associated configuration index of the OD-SSB of SCell A

- Case 5: The OD-SSB of SCell A configured with implicit OD-SSB deactivation is already in a deactivated state, while the MAC CE activates the OD-SSB of SCell A

Proposal 7b: For a SCell configured with implicit OD-SSB deactivation，NW can indicate UE to keep or change the OD-SSB transmission state in the OD-SSB MAC CE.

Proposal 7c: For a SCell configured with implicit OD-SSB deactivation, UE handles the OD-SSB state according to OD-SSB MAC CE if received. Otherwise, UE handles the OD-SSB state according to the implicit OD-SSB deactivation configuration.

Proposal 7d: If a SCell is configured with implicit OD-SSB deactivation and UE receives OD-SSB MAC CE, the UE shall treat the transmission of OD-SSB of the SCell as unchanged if the content of the MAC CE for the SCell (including the configuration index and activation/deactivation state) is the same as the current activated OD-SSB info for the SCell.

Reference

- Draft Report of 3GPP TSG RAN WG2 meeting #RAN2 129bis

- 3GPP TS 38.133, “Requirements for support of radio resource management (Release 18)”, V18.4.0 (2023-12)

- RP-242354 Revised WID: Enhancements of network energy savings for NR