3GPP TSG-RAN WG2 Meeting #130	R2-2503435

# St Julian’s, Malta, 19 – 23 May 2025

Agenda item:		8.6.3

Source:	Xiaomi

Title:	Remaining issues of L1 event triggered measurement reporting

Document for:		Discussion and Decision

# Introduction

In RAN2#129bis meeting, following was agreed regarding L1 event triggered measurement reporting [1]:

Agreements on L1 event-triggered MR

- Introduce new MAC CE to activate/deactivate the semi-persistent CSI-RS resource.

- The new MAC CE includes: A/D indication, Target configuration id or {SP CSI-RS resource set id and/or candidate id}, TCI state id.

- In L1 reporting configuration, the UE can be configured with a list of candidate IDs. For each listed candidate ID, the network can configure an offset, this offset applies to all the RSs belong the candidate ID that associated with the reporting configuration (the offset value also includes 0dB); If a candidate ID is not provided in the reporting configuration, it means the UE is not required to measure/evaluate the RSs that belonging to the candidate ID for this event, even if these are configured within the L1 resource configuration (to which the L1 reporting configuration points to).

- In the MR MAC CE, adopt a uniform format for all the reported beams (i.e., using the same format for beams leaving the entering condition, beams met the entering condition, beams not satisfying the event). Introduce two bits per beam.

- RAN2 will consider only the following options:

- Option1: To introduce four codepoints to represent one of {type1, type2, type3, type4}

- Option3: To introduce three codepoints to represent one of {type1_or_type3, type2, type4}, i.e., type1 beam and type3 beam are not distinguished in reporting.

Type1 beam: a beam that has satisfied the entry condition of the event for TTT and thus triggers the MR MAC CE.

Type2 beam: a beam that has satisfied the leaving condition of the event for TTT and thus triggers the MR MAC CE.

Type3 beam: a beam that is neither type1 nor type2 but has remained in beamsTriggeredList (the beam was reported as type1 beam before).

Type4 beam: a beam that is neither type1 nor type2 and not included in beamsTriggeredList (i.e., just extra beam info)

- Working assumption: Option1: To introduce four codepoints to represent one of {type1, type2, type3, type4}

- Intention is that the UE should be able to report the event-triggered beam(s) that were not included in the truncated MR MAC CE by the following grant. Detailed wording can be further discussed as part of the running CR.

- For co-existence with mTRP, will be revisited in August. If one simple solution is not prepared / agreed until / in August meeting, we will not apply mTRP in Rel-19 event-triggered MR.

In this contribution, we discuss the remaining issues of the L1 event triggered measurement reporting.

# Discussion

## (MAC-5) MAC CE format for LTM2

In email discussion “[POST129b][117][MOB] (Vivo)” [3], one open issue is as follows:

| Index | Issue description | Rapporteur suggestion |
| --- | --- | --- |
| MAC-5 | The content of L1 MR MAC CE triggered by LTM2 Editor’s NOTE: FFS For MR triggered by LTM2, whether only include the current beam information in the MR MAC CE or the MR can include measurements for LTM candidates. | Issue Type: Essential How to address it: based on companies’ contribution |

LTM2 is the event where beam of serving cell becomes worse than threshold. Reporting of measurement results for LTM candidates can be helpful for the network to make informed decisions. Network can use field allowReportAnyBeam-r19 (with field description in RRC running CR [4] copied below) to control whether beams of candidate beams can be reported or not.

allowReportAnyBeam Indicates whether the UE can report the measurement results for the beams not satisfying the conditions of the events at least for the time durating configured by timeToTrigger as specified in TS 38.321 [3].

Proposal 1: For MR triggered by LTM2, whether to include measurement results of LTM candidates is configured by the network (e.g. via field allowReportAnyBeam).

## (MAC-6, MAC-7) Truncated Event Triggered L1 MR MAC CE and multiple Event Triggered L1 MR MAC CEs

In email discussion “[POST129b][117][MOB] (Vivo)” [3] , following open issue was identified:

| Index | Issue description | Rapporteur suggestion |
| --- | --- | --- |
| MAC-6 | Truncated L1 MR MAC CE details. E.g. Except the triggered beam, whether any beam should be prioritised to be included in the truncated L1 MR MAC CE? In case the UL grant could include at least 2 beams in truncated L1 MR MAC CE, which beam should be included as the first beam in the truncated L1 MR MAC CE? Based on following agreement: - For differential L1-RSRP reporting, the best quality beam among the beams included in L1 MR MAC CE is taken as the reference beam as the first one. The differential L1-RSRP value is derived based on the absolute L1-RSRP of the reference beam. FFS for truncated MAC CE. | Issue Type: Not essential How to address it: can be discussed based on companies’ contribution |

In RAN2#129 meeting, following was agreed: “Select the beam not satisfying the event condition, based on the measured quality (i.e. according to the best beam(s))”. This was reflected in MAC running CR [2]: “The RS(s) not satisfying the event for TTT is selected based on the descending order of measured quantity.” Therefore, for truncated MAC CE, UE includes the triggered beams first, and then include other beams in descending order of measured quantity. The best quality beam among all beams included in the truncated MAC CE is taken as the reference beam.

Proposal 2: (MAC-6) For truncated MR MAC CE, the same principle of regular MR MAC CE applies, i.e., in addition to triggered beams, the truncated MAC CE includes other beams in descending order of measured quantity. The best quality beam among all beams included in the truncated MAC CE is taken as the reference beam.

In RAN2#128 meeting, it was agreed that “LTM MR MAC CE only include one triggered measurement event and the LTM MR MAC CE can be multiplexed in any available UL grant.” In case that UL grant cannot accommodate all the LTM MR MAC CEs (similar to the case of truncated LTM MR MAC CE), discussion is needed on how to handle LTM MR MAC CE for different measurement events, e.g. whether to prioritize certain LTM MR MAC CEs, or the handling is left to UE implementation.

Proposal 3: RAN2 to discuss how to handle the case that UL grant cannot accommodate all the triggered Event Triggered L1 MR MAC CEs, e.g. whether to prioritize certain Event Triggered L1 MR MAC CEs, or the handling is left to UE implementation.

In email discussion “[POST129b][117][MOB] (Vivo)” [3], open issue [MAC-7] is as follows:

| MAC-7 | Whether one UL TB could include more than one truncated MAC CE or only one? For the case more than 1 triggering events are pending, if UL grant is sufficient for one regular L1 MR MAC CE or for two truncated L1 MR MAC CEs but insufficient for two regular L1 MR MAC CEs, the UE behaviour should be discussed. | Issue Type: Not essential How to address it: can be discussed based on companies’ contribution |
| --- | --- | --- |

The overhead of MAC subheader should be considered when evaluating one MAC CE vs. two MAC CEs. As from MAC running CR [2], (truncated) Event triggered L1 measurement report MAC CE is of variable size, and uses eLCID. Consequently, following 3-byte MAC subheader is used. Given that UL grant size is limited, it is preferable to transmit a single MAC CE to maximize the information to be transmitted.

Proposal 4: (MAC-7) If more than one triggering events are pending and UL grant is not sufficient for two regular L1 MR MAC CEs, UE transmits as much information as possible in one L1 MR MAC CE for one event, and transmits truncated L1 MR MAC CE for the other event if there is space available in the UL grant.

## (MAC-1) Event configuration

(MAC-1) Whether beam specific offset for serving cell is needed

In email discussion “[POST129b][117][MOB] (Vivo)” [3], following open issue was identified:

Open issue MAC-1 (essential): Whether the beam specific offset for serving cell Obs is needed for LTM event evaluation

Proposal 1: (4 vs. 4) RAN2 to discuss whether the beam offset for current beam of serving cell, i.e. Obs in the event evaluation inequality, is needed for the LTM event evaluation.

Proposal 2: (4/6) RAN2 to discuss whether the Obs offset is configured per-cell, while applied at the level of indidual beam, if it is agreed to be introduced.

Proposal 3: RAN2 to discuss the Obs offset is applied to which LTM event(s), e.g. LTM2, 3, 5, if it is agreed to be introduced.

This is related to offset Obs in the following inequalities in MAC running CR [2]:

Inequality LTM3-1 (Entering condition)

Mn + Obn – Hys > Ms + Obs + Off

Inequality LTM3-2 (Leaving condition)

Mn + Obn + Hys < Ms + Obs + Off

Currently, the report configuration for event LTM3 in RRC running CR is as follows, with ltm3-Offset-r19 corresponding to parameter Off.

eventLTM3-r19                                     SEQUENCE {

ltm3-Offset-r19                                   MeasTriggerQuantityOffset,

hysteresis-r19                                    Hysteresis,

timeToTrigger-r19                                 TimeToTrigger,

...

},

It is not clear what is the additional benefit of configuring Obs given that Off is configured, i.e. parameter Off can cover Obs. It is expected that if Obs is defined, it will be specified together with ltm3-Offset-r19, i.e. under eventLTM3-r19. This is different from L3 event A3, where cell specific offset (Ocp) of serving cell is configured in measObjectNR and a3-Offset is defined within reportConfigNR. For simplicity, it is proposed to not consider beam offset Obs of serving cell since it can be covered by parameter Off.

Proposal 5: (MAC-1) Beam offset for current beam of serving cell, i.e. Obs in the event evaluation inequality, is not needed for the LTM event evaluation.

SpCell or serving cell in event LTM2/3/5

In RRC running CR [4], the definition of event LTM2/3/5 are as follows:

Event LTM2:	Beam of SpCell becomes worse than absolute threshold;

Event LTM3:	Beam of candidate cell becomes amount of offset better than the beam of SpCell;

Event LTM5:	Beam of SpCell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

In MAC running CR [2], the definition of event LTM2/3/5 are as follows:

Event LTM2 (Beam of serving cell becomes worse than threshold)

Event LTM3 (Beam of candidate cell becomes offset better than beam of serving cell)

Event LTM5 (Beam of serving cell becomes worse than threshold1 and Beam of candidate cell becomes better than threshold2)

It can be seen that SpCell and “serving cell” are used in RRC CR and MAC CR, respectively. Layer 3 event A3 and A5 are both defined for SpCell. Although layer 3 event A2 is defined for serving cell, the main purpose of event A2 is for SCell management, which is not the case for Event LTM 2. It is suggested that RAN2 to clarify whether serving cell in Event LTM 2/3/5 refers to SpCell only.

Proposal 6: RAN2 to clarify whether serving cell in Event LTM 2/3/5 refers to SpCell only.

## (MAC-15, MAC-16) Event triggering / cancellation

In email discussion “[POST129b][117][MOB] (Vivo)” [3], following open issue was identified:

| MAC-15 | Whether the triggered MR should be cancelled in the below case: if all the triggered beam(s), i.e. Type#00 and Type#01, have been reported, no matter whether truncated MR MAC CE or regular MR MAC CE is used before, the trigger MR should be cancelled. Raised by Apple during the discussion. | Issue Type: Essential How to address it: based on companies’ contribution |
| --- | --- | --- |

Given that gNB can configure UE to report non-trigger beams (i.e. type 3 and type 4), it is inconsistent to cancel MR if all triggering beams have been reported. Triggering of MR should only be cancelled if all beams are reported according to configuration, i.e. the following configuration parameters as from running RRC CR [4]:

LTM-EventTriggeredReportContent-r19 ::=     SEQUENCE {

maxNumberOfReportedBeams-r19                   INTEGER (1..16),

allowReportAnyBeam-r19                         ENUMERATED {enabled}                OPTIONAL, -- Need R

reportCurrentBeam-r19                          ENUMERATED {enabled}                OPTIONAL, -- Need R

...}

Proposal 7: (MAC-15) Triggering of MR is only cancelled when all beams are reported according to configuration (i.e. maxNumberOfReportedBeams-r19, allowReportAnyBeam-r19, reportCurrentBeam-r19).

In MAC running CR [2], triggered MR is cancelled when a regular MR MAC CE is transmitted, as below:

2>	if UL-SCH resources are available for a new transmission in the serving cell and these UL-SCH resources can accommodate the L1 measurement report MAC CE plus its subheader as a result of logical channel prioritization:

…

3> cancel the triggered L1 measurement report;

After sending truncated MAC CE, if the subsequent UL grant can fit all the remaining beams, one question is whether the MAC CE is considered as regular MAC CE or truncated MAC CE. Given above cancellation policy, it is better to consider this MAC CE as a regular MAC CE. With the MAC CE considered as regular MAC CE, it also informs the gNB that all beam information is transmitted therefore no further UL grants are needed for MR MAC CE.

Proposal 8: A note is added to clarify that “Event Triggered L1 MR MAC CE is considered as regular MAC CE (i.e. not truncated MAC CE) if it contains information of all the remaining beams after truncated MAC CE is sent.”

In email discussion “[POST129b][117][MOB] (Vivo)” [3], following open issue was identified:

| MAC-16 | Whether the triggered MR should be cancelled in the below two cases: - Case 1: When a beam (or multiple beam) meets the entry condition for TTT (Type#00), a MR will be triggered. Before getting the UL grant, if this beam (or all these beams) meets the leaving condition, then, whether this MR is still triggered or should be cancelled? - Case 2: When a beam (or multiple beam) in the reporting list meets the leaving condition for TTT (Type#01), a MR will be triggered. Before getting the UL grant, if this beam (or all these beams) meets the entry condition again, then, whether this MR is still triggered or should be cancelled? Raised by Apple during the discussion. | Issue Type: Essential How to address it: based on companies’ contribution |
| --- | --- | --- |

MR MAC CE should reflect the latest information. If MR is triggered when a beam meets the entering condition, and later on the beam meets the leaving condition, then the corresponding MR should be cancelled to avoid potential inaccurate information to the network.

Proposal 9: (MAC-16) When a beam meets the entering condition and later meets the leaving condition before receiving UL grant, or vice versa, the triggered MR should be cancelled.

## (MAC-17) SR of Event Triggered L1 MR MAC CE

In email discussion “[POST129b][117][MOB] (Vivo)”, one open issue is “[MAC-17] FFS how to handle the case if the dedicated SR configuration for L1 measurement report MAC CE transmission is not configured, when MR is triggered”.

In MAC running CR [2], following is captured regarding SR for L1 measurement report MAC CE:

…For a logical channel or for SCell beam failure recovery (see clause 5.17), for consistent LBT failure recovery (see clause 5.21), and for event triggered L1 measurement report (see clause 5.x), at most one PUCCH resource for SR is configured per BWP…

1>	if at least one L1 measurement report has been triggered as specified in 5.x.3 and not cancelled:

2>	if UL-SCH resources are available for a new transmission in the serving cell and these UL-SCH resources can accommodate the L1 measurement report MAC CE plus its subheader as a result of logical channel prioritization:

…

2>	else if the UL-SCH resources are available for a new transmission in the serving cell and these UL-SCH resources can accommodate the Truncated L1 measurement report MAC CE plus its subheader as a result of logical channel prioritization:

…

2> else:

3>	if the dedicated SR configuration for L1 measurement report MAC CE transmission is configured:

4>	trigger the SR using the dedicated SR configuration for L1 measurement report.

In above highlighted part, SR is only triggered when dedicated SR is configured. The problem is that if dedicated SR is not configured for L1 measurement report, SR will not be triggered. This is not consistent with other use cases where dedicated SR is configured. Taken BFR (which can have dedicated SR) as an example, SR triggering is captured irrespective of whether dedicated SR is configured or not, as below:

2>	else:

3>	trigger the SR for beam failure recovery of each BFD-RS set for which BFR has been triggered, not cancelled, and for which evaluation of the candidate beams according to the requirements as specified in TS 38.133 [11] has been completed;

3>	trigger the SR for SCell beam failure recovery for each SCell for which BFR has been triggered, not cancelled, and for which evaluation of the candidate beams according to the requirements as specified in TS 38.133 [11] has been completed.

It is therefore suggested to change the above SR triggering part for L1 measurement report MAC CE as below:

2> else:

3>	trigger the SR for L1 measurement report.

Proposal 10: (MAC-17) When L1 measurement is triggered and UL-SCH resource is not available, SR is triggered irrespective of whether dedicated SR configuration is configured or not.

TS 38.321 has a general statement that if PUCCH resource is not available, RACH is used, as below:

1>	if the MAC entity has no valid PUCCH resource configured for the pending SR; and

1>	if there is no ongoing RACH-less LTM cell switch; and

1>	if rach-LessHO is not configured:

2>	initiate a Random Access procedure (see clause 5.1) on the SpCell and cancel the pending SR.

Therefore if the dedicated SR configuration for L1 measurement report MAC CE transmission is not configured, when MR is triggered and there is no UL-SCH resource available, SR is triggered, and RACH is triggered consequently. Note that in MAC running CR [2], it is already captured that RACH procedure due to SR for Event Triggered L1 MR can be cancelled when corresponding MAC CE is transmitted:

The MAC entity may stop, if any, ongoing Random Access procedure due to a pending SR for Event Triggered L1 Measurement Report, which has no valid PUCCH resources configured, if:

-	a MAC PDU is transmitted using a UL grant other than an UL grant provided by Random Access Response or a UL grant determined as specified in clause 5.1.2a for the transmission of the MSGA payload, and this PDU includes an Event Triggered L1 Measurement Report MAC CE (see clause 5.x).

Proposal 11: (MAC-17) If the dedicated SR configuration for L1 measurement report MAC CE is not configured, when L1 measurement is triggered and there is no UL-SCH resource available, SR is triggered, and RACH is triggered consequently. No specification change is needed.

In the RAN2#127bis meeting, RAN2 made the following agreements for the SR used for the Event Triggered L1 MR MAC CE:

11. The legacy SR procedure for resource allocation is the baseline to send the event-triggered L1 measurements MAC CE. 12. NW can configure a dedicated SR configuration for MR MAC CE transmission.

In RAN#129 meeting, RAN2 agreed that “The Measurement Report MAC CE is transmitted with the same LCH priority as MAC CE for (Enhanced) BFR.”

In general, we think that the triggering/cancellation/priority of the dedicated SR for the Event Triggered L1 MR MAC CE can reuse the procedure defined for the dedicated SR for BFR MAC CE. This means that the dedicated SR is triggered when a L1 measurement report is triggered. When the L1 measurement report is cancelled (e.g. due to the report/cancellation of the Event Triggered L1 MR MAC CE), the dedicated SR is also cancelled. The related note in TS 38.321 is copied below:

NOTE 3:	When the MAC entity has pending SR for SCell beam failure recovery and the MAC entity has one or more PUCCH resources (other than PUCCH resources of pending SR for beam failure recovery of a BFD-RS set) overlapping with PUCCH resource for SCell beam failure recovery for the SR transmission occasion, the MAC entity considers only the PUCCH resource for SCell beam failure recovery as valid. When the MAC entity has pending SR for beam failure recovery of a BFD-RS set of Serving Cell and the MAC entity has one or more PUCCH resources (other than PUCCH resources of pending SR for beam failure recovery) overlapping with PUCCH resource for beam failure recovery of that BFD-RS set for the SR transmission occasion, the MAC entity considers only the PUCCH resource for beam failure recovery of that BFD-RS set as valid.

According to TS 38.321 as quoted above, the BFR SR has higher priority than the BSR SR, because the beam failure recovery is more urgent than the data transmission. From our understanding, the transmission of Event Triggered L1 MR MAC CE should also have higher priority, so that the LTM cell switch can be triggered in time. This means that the priority of the SR for Event Triggered MR MAC CE should be higher than that of the SR for BSR.

Proposal 12: The dedicated SR for Event Triggered L1 Measurement Report MAC CE has higher priority than the SR for BSR.

# Conclusion

In this contribution, we discuss the remaining issues of the L1 event triggered measurement reporting, and propose the following:

(MAC-5) MAC CE format for LTM2

Proposal 1: For MR triggered by LTM2, whether to include measurement results of LTM candidates is configured by the network (e.g. via field allowReportAnyBeam).

(MAC-6, MAC-7) Truncated Event Triggered L1 MR MAC CE and multiple Event Triggered L1 MR MAC CEs

Proposal 2: (MAC-6) For truncated MR MAC CE, the same principle of regular MR MAC CE applies, i.e., in addition to triggered beams, the truncated MAC CE includes other beams in descending order of measured quantity. The best quality beam among all beams included in the truncated MAC CE is taken as the reference beam.

Proposal 3: RAN2 to discuss how to handle the case that UL grant cannot accommodate all the triggered Event Triggered L1 MR MAC CEs, e.g. whether to prioritize certain Event Triggered L1 MR MAC CEs, or the handling is left to UE implementation.

Proposal 4: (MAC-7) If more than one triggering events are pending and UL grant is not sufficient for two regular L1 MR MAC CEs, UE transmits as much information as possible in one L1 MR MAC CE for one event, and transmits truncated L1 MR MAC CE for the other event if there is space available in the UL grant.

(MAC-1) Event configuration

Proposal 5: (MAC-1) Beam offset for current beam of serving cell, i.e. Obs in the event evaluation inequality, is not needed for the LTM event evaluation.

Proposal 6: RAN2 to clarify whether serving cell in Event LTM 2/3/5 refers to SpCell only.

(MAC-15, MAC-16) Event triggering / cancellation

Proposal 7: (MAC-15) Triggering of MR is only cancelled when all beams are reported according to configuration (i.e. maxNumberOfReportedBeams-r19, allowReportAnyBeam-r19, reportCurrentBeam-r19).

Proposal 8: A note is added to clarify that “Event Triggered L1 MR MAC CE is considered as regular MAC CE (i.e. not truncated MAC CE) if it contains information of all the remaining beams after truncated MAC CE is sent.”

Proposal 9: (MAC-16) When a beam meets the entering condition and later meets the leaving condition before receiving UL grant, or vice versa, the triggered MR should be cancelled.

(MAC-17) SR of Event Triggered L1 MR MAC CE

Proposal 10: (MAC-17) When L1 measurement is triggered and UL-SCH resource is not available, SR is triggered irrespective of whether dedicated SR configuration is configured or not.

Proposal 11: (MAC-17) If the dedicated SR configuration for L1 measurement report MAC CE is not configured, when L1 measurement is triggered and there is no UL-SCH resource available, SR is triggered, and RACH is triggered consequently. No specification change is needed.

Proposal 12: The dedicated SR for Event Triggered L1 Measurement Report MAC CE has higher priority than the SR for BSR.

# References

[1] R2-2502981, Vice Chairman (Samsung), "Report from session on R18 SL, R18/19 MOB and R19 NES"

[2] R2-2503616, vivo (Rapporteur), "Running MAC CR for Mob Ph4"

[3] R2-2503766, vivo, "Discussion summary and list of MAC open issue for Mob Ph4"

[4] R2-2503785, Huawei, HiSilicon, "Introduction of event-triggered meas report for RRC spec"