3GPP TSG-RAN WG2 Meeting #130	R2-2503439

# St Julian’s, Malta, 19 – 23 May 2025

Agenda item:		8.7.5

Source:	Xiaomi

Title:	RLC AM retransmission enhancements

Document for:		Discussion and Decision

# Introduction

In RAN2#129bis meeting, following was agreed regarding RLC retransmission enhancements [1]:

Agreements on RLC enhancements - When the t-RxDiscard expires, the expiration of t-RxDiscard triggers an SR. FFS whether this is just usual SR or some changes are needed, or if UE implementation can decide (to be discussed during CR review) - For autonomous retransmission and polling, the remaining time is determined based on discardTimer at PDCP. FFS whether/what additional conditions are needed to prevent too early and/or unnecessary retransmission - Autonomous retransmission is triggered for an RLC SDU (segment) provided that the original RLC SDU has been submitted to lower layers. - Autonomous retransmission is not triggered if the RLC SDU (segment) is already pending for retransmission. FFS specifications impact.

In this contribution, we discus open issues of RLC AM retransmission enhancements.

# Discussion

## Avoid unnecessary RLC retransmissions

Open issue RLC-6

In “[POST129bis][505][XR] RLC running CR and open issues (vivo)” [2], open issue RLC-6 is related to the following Editor’s note: FFS whether there are any RLF detection impacts when avoiding unnecessary retransmissions is introduced. The potential issue is that with Tx side enhancement (discard RLC PDUs based on PDCP indication), maxRetxThreshold cannot be reached therefore RLF detection cannot be triggered.

The purpose of retransmission count based RLF detection is to handle RLC failure and to prevent UE from blindly retransmitting data when the channel quality is poor. With the Tx side enhanced to discard obsolete SDUs, RLC layer works fine without any problems although there can be less number of RLC transmissions. In addition, RLF detection can be still performed based on other RLC bearers not configured with stopReTxDiscardedSDU, or other mechanisms like RLM based detection. Therefore there is no impact on RLF detection when avoiding unnecessary retransmission at Tx side is introduced.

Proposal 1: (RLC-6) There is no specification impact on RLF detection due to avoiding unnecessary retransmission at Tx side.

Open issue RLC-10

In “[POST129bis][505][XR] RLC running CR and open issues (vivo)” [2], open issue RLC-10 is as follows: Whether/How to introduce any limitations or associations for the configuration of stopReTxObsoleteSDU and DL t-RxDiscard. The intention is to avoid Tx window stall, and the related proposal during email discussion is that if DL stopReTxDiscardedSDU is supported, DL t-RxDiscard should be configured; and if UL stopReTxDiscardedSDU is configured, UL timer based Rx discard should be activated in NW.

This issue is mainly up to network implementation. Taking DL as an example. Only if UE indicates it supports the Rx side operation (e.g. t-RxDiscard) and gNB supports the Tx side operation, gNB may configures UE with t-RxDiscard for some RLC bearers. This is similar to many other operations which require collaboration between UE and gNB. There is no need to specify such network implementation details.

Proposal 2: (RLC-10) It is up to network implementation on whether to configure stopReTxDiscardedSDU / t-RxDiscard and related network behavior.

## Timely RLC retransmission

Open issue RLC-7

In “[POST129bis][505][XR] RLC running CR and open issues (vivo)” [2], open issue RLC-7 is related to the following Editor’s note “It is still open how Autonomous Retransmission coexists with ARQ procedures, i.e. whether/how to increment the RETX_COUNT for Autonomous Retransmission.”

RETX_COUNT maintenance is related to RLC RLF. Related specification text in TS 38.322 is as follows:

When an RLC SDU or an RLC SDU segment is considered for retransmission, the transmitting side of the AM RLC entity shall:

-	if the RLC SDU or RLC SDU segment is considered for retransmission for the first time:

-	set the RETX_COUNT associated with the RLC SDU to zero.

-	else, if it (the RLC SDU or the RLC SDU segment that is considered for retransmission) is not pending for retransmission already and the RETX_COUNT associated with the RLC SDU has not been incremented due to another negative acknowledgment in the same STATUS PDU:

-	increment the RETX_COUNT.

-	if RETX_COUNT = maxRetxThreshold:

-	indicate to upper layers that max retransmission has been reached.

If RETX_COUNT reaches maxRetxThreshold, RLF is declared. The motivation of autonomous retransmission is that there is not sufficient time for RLC retransmission based on status report for XR traffic with short packet delay budget (PDB). In RAN2#129 meeting, it was agreed that “Only a single autonomous retransmission will be triggered per RLC SDU”. Therefore legacy behavior for RETX_COUNT is applicable, and special handling for autonomous retransmission is not needed.

Proposal 3: (RLC-7) Existing RLC procedure for RETX_COUNT is applicable for autonomous retransmission. No specification change is needed.

Open issue RLC-8

In “[POST129bis][505][XR] RLC running CR and open issues (vivo)” [2], open issue RLC-8 is related to the following Editor’s note: “It is still open on how to avoid excessive polling for the polling enhancement, e.g. only one polling or multiple.” Currently the triggering condition for enhanced polling is captured in running PDCP CR, as below:

For the purpose of [remaining-time-based RLC polling], if the remaining time till discardTimer expiry becomes less than the [RemainingTimeBasedRLCPollingThreshold] for the PDCP SDU for which the corresponding PDCP Data PDU has already been submitted to lower layers, the transmitting PDCP entity shall:

-	indicate to lower layers that condition for [remaining-time-based RLC polling] is met for the corresponding PDCP Data PDU.

To avoid excessive polling, polling can be triggered when the time duration of submitting a delay critical RLC SDU to MAC layer exceeds a configured threshold to allow some HARQ retransmissions. An example is shown in Figure 1 below. Polling is triggered when both conditions are satisfied: 1) A RLC SDU is delay critical (as the remaining time until expiry of discardTimer is less than enhancedPollingThreshold); and 2) time duration of submitting the RLC SDU to MAC layer (Δt) exceeds a configured threshold.

Figure 1: Polling enhancement considering transmission status

Proposal 4: (RLC-8) To avoid excessive polling, polling is triggered if the remaining time till discardTimer expiry becomes less than polling threshold for the PDCP SDU for which the corresponding PDCP Data PDU has already been submitted to lower layers for a time duration over a configured threshold.

Open issue RLC-9

In “[POST129bis][505][XR] RLC running CR and open issues (vivo)” [2], open issue RLC-9 is related to the following Editor’s note: “FFS whether/what additional conditions are needed to prevent too early and/or unnecessary retransmission due to polling enhancement.” After UE sending polling, how to generate status report is up to network implementation, therefore preventing too early and/or unnecessary retransmissions is up to network implementation.

Proposal 5: (RLC-9) No additional specification impacts are needed to prevent too early and/or unnecessary retransmissions due to polling enhancement.

Open issue PDCP-2

In email discussion “[POST129bis][504][XR] PDCP running CR (LGE)” [3], one open issue is “Issues 2: It is FFS for which PDCP SDU the UE shall trigger [remaining-time-based RLC retransmission and polling], if pdu-SetDiscard is configured.” This is related to the following running PDCP CR text:

For the purpose of [remaining-time-based RLC retransmission], if the remaining time till discardTimer expiry becomes less than the [RemainingTimeBasedRLCRetransmissionThreshold] for the PDCP SDU for which the corresponding PDCP Data PDU has already been submitted to lower layers, the transmitting PDCP entity shall:

-	indicate to lower layers that the condition for [remaining-time-based RLC retransmission] is met for the corresponding PDCP Data PDU.

For the purpose of [remaining-time-based RLC polling], if the remaining time till discardTimer expiry becomes less than the [RemainingTimeBasedRLCPollingThreshold] for the PDCP SDU for which the corresponding PDCP Data PDU has already been submitted to lower layers, the transmitting PDCP entity shall:

-	indicate to lower layers that condition for [remaining-time-based RLC polling] is met for the corresponding PDCP Data PDU.

When pdu-SetDiscard is configured, upon expiry of discard timer, all PDCP SDUs belonging to the PDU Set are discarded. The discard timer is still maintained per PDCP SDU. PDU set discard does not affect remaining-time-based RLC retransmission and polling which is operated in per PDCP SDU level. If one SDU meets the triggering condition in PDCP layer, it can be indicated to RLC layer. There is no need to indicate other PDCP SDUs belong to the same PDU Set to RLC layer.

Proposal 6: (PDCP-2) The UE shall trigger remaining-time-based RLC retransmission and polling in PDCP PDU level, no matter pdu-SetDiscard is configured or not. No specification change is needed.

Open issue RRC-3 and RRC-4

In “[POST129bis][503][XR] RRC running CR and open issues (Huawei)” [4], there are following open issues:

- Issue3: FFS whether the autonomous retransmission is also applicable for discard for PDUs with low importance, which uses a separate timer discardTimerForLowImportance

- Issue4: FFS whether enhanced polling is also applicable for discard for PDUs with low importance, which requires a separate timer discardTimerForLowImportance

For PDUs with low importance, timer discardTimerForLowImportance is started during congestion (i.e. when PSI based SDU discard is activated), as from TS 38.323 (copied below):

-	if discardTimerForLowImportance is configured and PSI based SDU discard is activated, and the PDCP SDU belongs to a low importance PDU Set:

-	start the discardTimerForLowImportance associated with this PDCP SDU;

-	else:

-	start the discardTimer associated with this PDCP SDU (if configured).

Timer discardTimerForLowImportance is always configured shorter than the corresponding discardTimer, as from the field description below. The motivation is that those low importance PDUs can be discarded during congestion.

discardTimerForLowImportance Value in ms of discardTimerForLowImportance specified in TS 38.323 [5]. Value ms0 corresponds to 0 ms, value ms2 corresponds to 2 ms and so on. The value of this timer for a PDCP entity is always configured shorter than discardTimer, discardTimerExt or discardTimerExt2, whichever is used for the PDCP entity. The presence of this field indicates that PSI-based SDU discard is configured for a DRB. The network configures at most 8 DRBs with discardTimerForLowImportance.

The motivation of autonomous retransmission and enhanced polling is to ensure timely RLC retransmissions for XR services with short packet delay budget. If these enhancements are also applicable to PDUs with low importance, low important PDUs can have more chances for RLC retransmissions than high importance PDUs due to shorter discard timer, which is not the intended behavior especially in the congestion situation. Therefore autonomous retransmission and enhanced polling are not applicable for discard for PDUs with low importance, which is also consistent with DSR (only discardTimer is considered).

Proposal 7: (RRC-3, RRC-4) Autonomous retransmission and enhanced polling are not applicable for discard for PDUs with low importance, which uses a separate timer discardTimerForLowImportance. No specification change is needed.

# Conclusion

In this contribution, we discuss open issues of RLC AM retransmission enhancements, and propose the following:

Avoid unnecessary RLC retransmissions

Proposal 1: (RLC-6) There is no specification impact on RLF detection due to avoiding unnecessary retransmission at Tx side.

Proposal 2: (RLC-10) It is up to network implementation on whether to configure stopReTxDiscardedSDU / t-RxDiscard and related network behavior.

Timely RLC retransmission

Proposal 3: (RLC-7) Existing RLC procedure for RETX_COUNT is applicable for autonomous retransmission. No specification change is needed.

Proposal 4: (RLC-8) To avoid excessive polling, polling is triggered if the remaining time till discardTimer expiry becomes less than polling threshold for the PDCP SDU for which the corresponding PDCP Data PDU has already been submitted to lower layers for a time duration over a configured threshold.

Proposal 5: (RLC-9) No additional specification impacts are needed to prevent too early and/or unnecessary retransmissions due to polling enhancement.

Proposal 6: (PDCP-2) The UE shall trigger remaining-time-based RLC retransmission and polling in PDCP PDU level, no matter pdu-SetDiscard is configured or not. No specification change is needed.

Proposal 7: (RRC-3, RRC-4) Autonomous retransmission and enhanced polling are not applicable for discard for PDUs with low importance, which uses a separate timer discardTimerForLowImportance. No specification change is needed.

# References

[1] R2-2502985, Session chair (Huawei), “Report from session on R19 XR”

[2] R2-2503767, vivo, “Discussion summary and list of RLC open issue for R19 XR”

[3] R2-2503697, LG Electronics Inc. (Rapporteur), “Discussion of [POST129bis][504][XR] PDCP running CR”

[4] R2-2503788, Huawei, HiSilicon, “Summary of [POST129bis][503][XR] RRC running CR (Huawei)”