3GPP TSG RAN WG2 #130	R2-2503450

St. Julians, Malta, May 19th–23rd, 2025

Agenda Item:	8.1.2.3

Source: 	Xiaomi

Title:  	Discussion on AI positioning case 1

Document for:	Discussion and Decision

# Introduction

After RAN2#129bis meetings, following post email discussions were used to discuss the running CRs and open issues for AI/ML positioning:

[POST129bis][014][AI PHY] 38.305 Running CR (CATT)

[POST129bis][015][AI PHY] 37.355 Running CR (Qualcomm)

In this contribution, we mainly focus on how to solve the remaining open issues for AI/ML positioning case 1.

# Discussion

## Error Cause (LPP-13/14)

As proposed in [POST129bis][015][AI PHY] 37.355 Running CR (Qualcomm), location server/target device error cause is listed as open issue LPP-13 and LPP-14, respectively.

Location Server Error Cause

Following assistance data for AI/ML positioning case 1 is captured in TS 37.355 running CR. It is observed that the assistance data used for AI/ML positioning case 1 is the same as NR DL-TDOA positioning, i.e., NR-DL-PRS-AssistanceData-r16, except NR-PeriodicAssistData.

#### 6.5.13.1 NR DL AI/ML Positioning Assistance Data #### – NR-DL-AIML-ProvideAssistanceData The IE NR-DL-AIML-ProvideAssistanceData is used by the location server to provide assistance data to enable UE-based DL AI/ML positioning. It may also be used to provide NR DL AI/ML positioning specific error reason. -- ASN1START NR-DL-AIML-ProvideAssistanceData-r19 ::= SEQUENCE { nr-DL-PRS-AssistanceData-r19 NR-DL-PRS-AssistanceData-r16 OPTIONAL, -- Need ON nr-SelectedDL-PRS-IndexList-r19 NR-SelectedDL-PRS-IndexList-r16 OPTIONAL, -- Need ON nr-On-Demand-DL-PRS-Configurations-r19 NR-On-Demand-DL-PRS-Configurations-r17 OPTIONAL, -- Need ON nr-On-Demand-DL-PRS-Configurations-Selected-IndexList-r19 NR-On-Demand-DL-PRS-Configurations-Selected-IndexList-r17 OPTIONAL, -- Need ON assistanceDataValidityArea-r19 AreaID-CellList-r17 OPTIONAL, -- Need ON nr-PositionCalculationAssistance-r19 NR-PositionCalculationAssistance-r16 OPTIONAL, -- Need ON nr-DL-AIML-Positioning-Error-r19 NR-DL-AIML-Positioning-Error-r19 OPTIONAL, -- Need ON ... } -- ASN1STOP Editor's Note: The above list of assistance data elements is FFS and depends on further RAN1/RAN2 agreements. The draft list above is the same as in IE NR-DL-TDOA-ProvideAssistanceData but without the NR-PeriodicAssistData.

Furthermore, following cause values are introduced for NR DL-TDOA positioning:

- assistanceDataNotSupportedByServer,

- assistanceDataSupportedButCurrentlyNotAvailableByServer,

- notProvidedAssistanceDataNotSupportedByServer,

- on-demand-dl-prs-NotSupportedByServer,

- on-demand-dl-prs-SupportedButCurrentlyNotAvailableByServer

It is observed that location server error cause is normally defined based on whether there’s any error cause for assistance data or on-demand DL PRS. As discussed above, there’s no new assistance data introduced for AI/ML positioning case 1. Even new assistance data is considered, it can be covered by existing error causes that are related to assistance data. There’s no need to introduce new error cause for AI/ML positioning case 1.

Proposal 1: (LPP-13) NR-DL-AI-ML-LocationServerErrorCauses uses error causes of NR-DL-TDOA-LocationServerErrorCauses as baseline. No new location server error cause is introduced for AI/ML positioning case 1.

Target device error causes

Recalling that RAN2 agreed in RAN2 #128 meeting that UE should inform LMF if its model for AI/ML based positioning becomes non-applicable:

2 As a baseline, If the AIML based positioning method becomes non-applicable when LMF requests UE location estimation, UE cannot perform the AIML based positioning, and reply with LPP Providelocationinformation message with error cause. FFS if other fallback options are considered

In NR DL-TDOA positioning method, following target device error cause is considered:

- assistance-data-missing,

- unableToMeasureAnyTRP,

- attemptedButUnableToMeasureSomeNeighbourTRPs,

- thereWereNotEnoughSignalsReceivedForUeBasedDL-TDOA,

- locationCalculationAssistanceDataMissing,

Different from above target device error causes, AI/ML positioning non-applicability is specifically for AI/ML, considering the applicability of certain AI/ML model/method is determined based on NW-side additional condition, UE-side additional condition and model availability. Therefore, non-applicability of AI/ML positioning method should be considered as a new target device error cause. Upon receiving the error message, LMF shall abort the ongoing procedure associated with the LPP-TransactionID if included in the received message. No new behavior is defined for this new target cause value.

Proposal 2: (LPP-14) Introduce ‘DL AIML positioning not applicable’ as new target device error cause for AI/ML positioning case 1.

## Performance Monitoring (LPP-16/17)

RAN1 reached following agreements on AI/ML positioning case 1 in terms of supporting performance monitoring Option A:

Agreement (RAN1#116bis) - Option A. The target UE side performs monitoring metric calculation. - Option A-1. At least information on ground truth label of the target UE is generated by LMF and provided to the target UE. - In one example, target UE and/or gNB sends measurement (e.g., legacy measurement) to LMF so that LMF can derive the information on ground truth label. - Option A-2. At least position calculation assistance data (e.g., existing information for UE-based positioning method) is provided from LMF to the target UE. - Option A-3. Reuse Rel-18 assistance data transfer framework from LMF to the target UE, where the PRU measurement (e.g., legacy measurement) and the corresponding PRU location are sent via LMF to the target UE. - Option A-4. PRU measurement (and the corresponding PRU location if not already known at the UE-side) are sent from PRU to the target UE side. - Note: Option A-4 can be realized by implementation in a manner transparent to specification if the PRU sends information to the target UE side in a proprietary method. Note: exact method to perform the monitoring metric calculation is up to implementation. Note: Other options are not precluded.

Conclusion (RAN1#118) For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, - Option A-4 can be realized by implementation in a manner transparent to specification specification if the PRU sends information to the target UE side in a proprietary method. No further discussion on Option A-4.

Agreement (RAN1#119) For model performance monitoring of AI/ML positioning Case 1, support at least: - Option A. The target UE side performs monitoring metric calculation. - The target UE may signal the monitoring outcome to the LMF. - FFS: content of monitoring outcome

Agreement (RAN1#120bis) For model performance monitoring of AI/ML positioning Case 1, “FFS: content of monitoring outcome” in RAN1#119 agreement is resolved by: - the content of monitoring outcome includes at least an indication that the target UE cannot perform the Case 1 positioning method.

In this section, we focus on the potential RAN2 impact of supporting performance monitoring of AI/ML positioning case 1.

Assistance Data and Report of Performance Monitoring

For Option A-2 and A-3, as mentioned in RAN1 agreements, existing assistance data supported by NR DL-TDOA positioning are transmitted from LMF to UE, supporting performance monitoring for AI/ML positioning case 1. Similar as NR DL-TDOA, NR-DL-AIML-ProvideAssistanceData can be used to provide assistance data for UE to calculate performance monitoring metrics.

Proposal 3: (LPP-16) ProvideAssistanceData is used to provide assistance data for UE to calculate performance monitoring metrics.

RAN1 further agreed that UE needs to inform LMF about the content of monitoring outcome (i.e., the target UE cannot perform case 1 positioning method). As discussed in above section, UE can report a new cause value ‘DL AIML positioning not applicable’ to LMF, indicating it cannot perform the requested AI/ML based positioning method. In our understanding, this new cause value can be reused for the purpose of monitoring outcome report. That is, if monitoring outcome indicating target UE cannot perform case 1 positioning method, UE reports an Error message with error cause ‘DL AIML positioning not applicable’ to LMF.

Proposal 4: (LPP-16) Monitoring outcome ‘target UE cannot perform case 1 positioning method’ is implicitly reported via UE reporting its target device error cause ‘DL AIML positioning not applicable’.

Signalling of "ground-truth label" information

It was agreed in RAN1 that the label data of UE location may be provided by LMF (i.e., Option A-1) as one of the options for performance monitoring. However, it is not clear whether Option A-1 is agreed to be supported by RAN1 and even it’s agreed, whether existing or new signaling is used.

Agreement (RAN1#120) For AI/ML based positioning Case 1, from RAN1 perspective, when the label data of location is generated by LMF and transferred from LMF to UE, label and quality indicator of label can be provided by reusing existing IEs. - From RAN1 perspective, the existing IE can use one of the geographic shapes defined in TS 23.032. The location estimate uncertainty and confidence (if included with the geographic shapes) can serve as quality indicator of the label.

First of all, for AI/ML positioning case 1, since AI/ML based positioning is performed at UE side, instead of requesting its own location from LMF, it is easier for a UE to use its own GNSS location as ground truth label. Hence, getting label data of location from LMF to UE is not mandatory.

Proposal 5: (LPP-17) For AI/ML positioning case 1, UE can use its own GNSS location as ground truth label. Requesting its own location information from LMF as label data is not mandatory.

Even if GNSS is not available at UE side and UE needs to get its own location information from LMF, the existing signalling can be reused to support such label data’s request and response.

As captured in TS 23.273[1], UE can request LMF to provide its own location related information via 5GC-MO-LR procedure.

## 6.2 5GC-MO-LR Procedure Figure 6.2-1 illustrates the general network positioning requested by the UE to the serving PLMN for obtaining the location related information of itself or just assistance data. <omitted unrelated context> 13) The AMF sends an MO-LR Response message included in a DL NAS TRANSPORT message. If the UE is requesting its own location, the response carries any location estimate requested by the UE and the timestamp of the location estimate (if available) including the indication received from LMF whether the obtained location estimate satisfies the requested accuracy or not, or an indicator whether a location estimate was successfully transferred to the identified LCS client or AF. If the location estimate was successfully transferred to the identified LCS Client or AF, the MO-LR Response message shall specify whether the location estimate of the UE has been handled successfully by the identified LCS Client or AF, and if not, the corresponding error cause obtained in step 13. In addition, AMF may record charging information.

It is further captured in TS 24.571 [2], MO-LR response message includes following information:

UE Network FACILITY <------------------------------------------------------------------------------------------------------------------------ Facility (Return result = lcs-MOLR (locationEstimate, velocityEstimate, add-LocationEstimate, decipheringKeys, referenceNumber, h-gmlc-address, mo-lrShortCircuit, reportingPLMNList, timestampOfLocationEstimate))

In our understanding, the above location estimate information can serve as the ground truth label for a UE. There’s no need to introduce a new message to transfer such location information from LMF to a UE.

Proposal 6: (LPP-17) Existing NAS signalling (i.e., MO-LR Request/Response message in UL/DL NAS TRANSPORT) is reused to request/provide ground truth label to the target UE, i.e., no further enhancement is needed from RAN2 point of view.

# Conclusions

In this contribution, we discussed LCM for AI positioning case 1 and provide the following observations and proposals:

Error Cause

Proposal 1: (LPP-13) NR-DL-AI-ML-LocationServerErrorCauses uses error causes of NR-DL-TDOA-LocationServerErrorCauses as baseline. No new location server error cause is introduced for AI/ML positioning case 1.

Proposal 2: (LPP-14) Introduce ‘DL AIML positioning not applicable’ as new target device error cause for AI/ML positioning case 1.

Proposal 3: (LPP-16) ProvideAssistanceData is used to provide assistance data for UE to calculate performance monitoring metrics.

Performance Monitoring

Proposal 4: (LPP-16) Monitoring outcome ‘target UE cannot perform case 1 positioning method’ is implicitly reported via UE reporting its target device error cause ‘DL AIML positioning not applicable’.

Proposal 5: (LPP-17) For AI/ML positioning case 1, UE can use its own GNSS location as ground truth label. Requesting its own location information from LMF as label data is not mandatory.

Proposal 6: (LPP-17) Existing NAS signalling (i.e., MO-LR Request/Response message in UL/DL NAS TRANSPORT) is reused to request/provide ground truth label to the target UE, i.e., no further enhancement is needed from RAN2 point of view.

# References

- TS 23.273, 5G System (5GS) Location Services (LCS); Stage 2

- TS 24.571, 5G System (5GS); Control plane Location Services (LCS) procedures; Stage 3