3GPP TSG-RAN WG2 Meeting #130	R2-2503452

St.Julians, Malta, 19th – 23rd May, 2025

Agenda Item:	8.7.4.1

Source: 	OPPO

Title:  	Discussion on LCH priority adjustment for XR

Document for:	Discussion and Decision

# Introduction

In Rel-19 XR WID [1] revised in RANP#105 meeting, the objectives of scheduling enhancements are updated as follows.

- Specify Enhancements for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs, as follows [RAN2]: - Specify additional Logical Channel priority handling using delay/deadline information of packets; - Specify enhanced DSR (Delay Status Report) reporting with multiple pairs of remaining time and buffer size for an LCG.

This contribution would like to provide our considerations on LCP enhancements for scheduling.

# Discussion on LCH priority adjustment

[MAC-01] SR priority adjustment

In legacy intra-UE prioritization, for the MAC entity configured with lch-basedPrioritization, the priority of an uplink grant is determined based on the priorities of the LCHs that are multiplexed or can be multiplexed. In detail, it is determined by the highest priority among priorities of the logical channels that are multiplexed (i.e. the MAC PDU to transmit is already stored in the HARQ buffer) or have data available that can be multiplexed (i.e. the MAC PDU to transmit is not stored in the HARQ buffer) in the MAC PDU, according to the mapping restrictions. Similarly, for the MAC entity configured with lch-basedPrioritization, the priority of an SR is also determined based on the priority of the LCH triggering the SR.

In the last meetings, the following agreements were made on impact on intra-UE prioritization, where SR priority determination is still FFS.

RAN2#129 Agreements on LCP enhancements There is no impact on BSR/SR/DSR triggering and reporting due to adjusted priority. Intra-UE prioritization shall also use the additional LCP priority for UL grant priority determination. FFS whether this has specifications impact

RAN2#129bis Agreements on LCP enhancements - We will check the impact of using additional LCP priority for SR priority determination during intra-UE prioritization in specifications and make a final decision on whether to have it based on that, i.e. it should be simple enough. - It should be explicitly specified which priority, e.g. “default” or additional priority, to use for a LCH in order to determine the priority of the corresponding UL grant, i.e. highest priority used/to be used during LCP. - For shared spectrum case, UE shall apply same rules for the LCH priority being used for determining the priority of a HARQ process as for determining the priority of an UL grant. - Understanding in RAN2 is that we will not address any other NR-U specific issues to support XR.

Furthermore, as shown below, in MAC open issue [MAC-01] SR priority adjustment, it is further clarified that companies should discuss based on the provided TP in this meeting, e.g., to discuss whether it is simple enough to capture, or whether there is further issue.

- [MAC-01] SR priority adjustment The current agreement is to evaluate the potential spec impact of SR priority adjustment and then decide whether to adopt it (i.e. only if it is simple enough to capture). Companies can use the following TP as the baseline for further discussion: For the MAC entity configured with lch-basedPrioritization, the MAC entity shall for a pending SR: 1> if the SR is triggered by a logical channel configured with additionalPriority; and - if the smallest remaining value of the running PDCP discardTimers among the data available for transmission in this logical channel, evaluated at the time of the first symbol of the next PUCCH transmission for the SR, is below or at priorityAdjustmentThreshold configured for the logical channel: 2> consider additionalPriority as the priority of the PUCCH transmission.

For the action of priority adjustment, we still hold the position that UE should only determine to use the additional LCP priority during LCP procedure when UL grant received, i.e., we are negative to allow additional priority adjustment for SR, which adds much UE complexity. Besides, it is reasonable to not use additional priority also for SR priority determination, considering that we have agreed that no impact on BSR/SR/DSR triggering and reporting due to additional priority.

However, if majority companies would like to introduce priority determination for SR, regarding the TP, we understand the issues needed to be discussed include

1/ For a single LCH, every SR transmission triggered by the LCH would have to evaluate the remaining time and the threshold to determine the priority, based on this TP. Considering not adding much load for UE, we tend to only perform priority adjustment for the first SR transmission, but it is unclear how to capture based on the current TP. So, RAN2 should further discuss whether every SR transmission triggered by the LCH should perform priority determination or not.

2/ Although there is motivation to use the additional LCP priority for SR priority determination, considering the same remaining time threshold used for priority adjustment in LCP procedure and SR priority determination in intra-UE prioritization, the benefit is doubted since the priority-adjusted data is likely to expire before UE receives UL grant requested by SR. So, RAN2 should further discuss whether a new remaining time threshold for priority determination in SR is needed or not.

Considering the late stage of this release, we propose not use additional LCP priority for SR priority determination, i.e., not adopt this TP, in order to avoid more open issues being discussed in RAN2.

- [MAC-01] The additional LCP priority is NOT used for SR priority determination in intra-UE prioritization.

[MAC-13] Priority fallback with consideration of PDU Set integrated handling

One open issue for MAC is the impact of PDU set on priority adjustment. When PDU set enabled, for a PDU set, if its PDU set remaining time (i.e., the shortest remaining time till discardTimer expiry among the PDCP SDUs belonging to the PDU Set) is shorter than priorityAdjustmentThreshold, but the remaining time of PDCP SDUs belonging to this PDU set and having not been transmitted in the LCH are larger than priorityAdjustmentThreshold, it is still unclear whether UE should apply the additional priority for the related LCH or not.

If considering the impact of PDU set on LCH priority adjustment, more negative impacts on the fairness of other LCHs would be caused. And additional specification impact is needed. Besides, notice that the data volume and remaining time for this kind of PDU sets could be reported to NW relying on Rel-19 DSR, which can already help NW to have the knowledge to schedule sufficient UL resources.

Therefore, we understand the simple approach is that MAC layer does not consider the impact of PDU set integrated handling.

- Considering the impact of PDU set on LCH priority adjustment would cause more negative impacts on the fairness of other LCHs.

- The Rel-19 DSR can already help NW to have the knowledge on the PDU set to schedule the sufficient UL resources.

- [MAC-13] LCH priority adjustment in MAC layer does not consider the impact of PDU set integrated handling. No additional specification impact is needed.

Impact on SRB

In RAN2#125bis meeting, the following agreement was made on SRB impact due to LCP enhancement, but no further conclusion on this.

RAN2 should consider potential impact on traffic from SRBs.

In our understanding, a proper network implementation would solve this issue easily. Networks can ensure the LCH of SRB always has higher LCH priority than the additional LCH priority used for the LCH with LCH priority-adjusted data.

- Up to network implementation to avoid SRB impact due to LCH prioritization-based LCP enhancement, i.e., configure higher priority for LCH of SRBs than LCH with LCH priority-adjusted data.

# 3. Conclusion

Based on the discussion above, we made the following observations and proposals:

[MAC-01] SR priority adjustment

- [MAC-01] The additional LCP priority is NOT used for SR priority determination in intra-UE prioritization.

[MAC-13] Priority fallback with consideration of PDU Set integrated handling

- Considering the impact of PDU set on LCH priority adjustment would cause more negative impacts on the fairness of other LCHs.

- The Rel-19 DSR can already help NW to have the knowledge on the PDU set to schedule the sufficient UL resources.

- [MAC-13] LCH priority adjustment in MAC layer does not consider the impact of PDU set integrated handling. No additional specification impact is needed.

Impact on SRB

- Up to network implementation to avoid SRB impact due to LCH prioritization-based LCP enhancement, i.e., configure higher priority for LCH of SRBs than LCH with LCH priority-adjusted data.

# 4. Reference

[1] RP-241771, Revised WID on XR (eXtended Reality) for NR Phase 3, Nokia