3GPP TSG-RAN WG2 Meeting #130	R2-2503458

St. Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	8.8.2

Source: 	CATT

Title: 	Discussion on downlink coverage enhancements

Document for: 	Discussion and Decision

Introduction

This contribution further discusses possible RAN2 impacts of DL coverage enhancements.

Discussion

2.1	Impact analysis due to extended SSB periodicity

2.1.1	SMTC enhancements

In RAN2#129bis [1], it was agreed that network can provide assistance information on the association between SMTC and location to help UE to perform SMTC selection for idle/inactive mode. And the details of location information are still FFS.

5.	Network can provide assistance information (for Rel-19 UEs, not necessarily supporting DL CE) on the association between SMTC and location to help UE to perform SMTC selection for idle/inactive mode. FFS on the details of location information, e.g. serving cell SSB index, reference location, etc. In any case it is up to UE implementation on how to utilize the assistance information for SMTC selection in idle/inactive mode.

Whereas the maximum number of the SMTCs that can be configured in the DL CE scenario depends on RAN4's reply, details of the location information can be further discussed by RAN2. In RAN2#129bis, there was an initial discussion regarding the FFS on the details of location information, but no consensus was reached. Companies mainly diverge on which of the following options to choose [2].

Option 2: reference location based SMTC selection, i.e. UE choose which SMTC to measure based distance between UE and the reference location of neighbour cell. Option 3: serving cell's SSBs based SMTC selection, i.e. UE choose which SMTC to measure based on the selected SSB of serving cell. For option 2, some companies think it is dependent on the availability of UE location information, may only be applicable for the case that location based neighbor cell measurement initiation is configured or low UE mobility. For option 3, it is only works for the case that different SSBs of a cell are transmitted in different beam footprints. And some company points out that the selected SSB may not reflect the UE location due to near-far effect and that UE may not select the SSB with the highest RSRP.

Between option 2 and option 3, we have a little preference on option 3, as it is simpler and can bring less signalling overhead. Some companies during last meeting offline discussion had concerns that option 3 may not work well as the UE may not select the SSB with the highest RSRP. However, this issue may not be a serious problem in NTN cell, as there should be obvious difference on RSRP between the NTN beams at the beam edge. That is, the detected SSB index can well reflect the position of the UE. RAN2 can thus consider support of per SSB configured SMTC list in SIB2 and SIB4 to help UE to perform SMTC selection for idle/inactive mode.

Proposal 1: RAN2 considers support of per SSB configured SMTC list in SIB2 and SIB4.

2.1.2	Impact on RRC procedure

The following is to discuss whether any other enhancements need to be introduced due to extended SSB periodicity of 160 ms. Note that since the extended SSB periodicity was introduced for the default SSB value, which impacts only UEs' access from RRC_IDLE/INACTIVE, the following will analyse the SI/paging/RACH related aspects.

1) System information

According to current spec [3], when the period of SSB is larger than 80 ms, the MIB is transmitted with the same periodicity as SSB, i.e., if the SSB periodicity is 160 ms, the MIB is transmitted with periodicity of 160 ms too. No spec impact is needed.

- the MIB is always transmitted on the BCH with a periodicity of 80 ms and repetitions made within 80 ms (TS 38.212 [17], clause 7.1) and it includes parameters that are needed to acquire SIB1 from the cell. The first transmission of the MIB is scheduled in subframes as defined in TS 38.213 [13], clause 4.1 and repetitions are scheduled according to the period of SSB; NOTE 1: If the period of SSB is larger than 80 ms, the MIB is transmitted with the same periodicity as that of SSB.

Observation 1: Current TS 38.331 supports the MIB broadcasted with the same periodicity as SSB when the periodicity of SSB is larger than 80 ms.

The scheduling of SIB1 relies on the SSB and CORESET multiplexing pattern, it is up to RAN1’s design to ensure that the SIB1 periodicity under different multiplexing patterns could match the SSB periodicity of 160 ms. RAN2 could wait for RAN1’s progress.

Observation 2: It is up to RAN1 whether any change is needed for the resource configuration used for SIB1 scheduling.

- the SIB1 is transmitted on the DL-SCH with a periodicity of 160 ms and variable transmission repetition periodicity within 160 ms as specified in TS 38.213 [13], clause 13. The default transmission repetition periodicity of SIB1 is 20 ms but the actual transmission repetition periodicity is up to network implementation. For SSB and CORESET multiplexing pattern 1, SIB1 repetition transmission period is 20 ms. For SSB and CORESET multiplexing pattern 2/3, SIB1 transmission repetition period is the same as the SSB period (TS 38.213 [13], clause 13).

Other SI is scheduled with the SI-window mechanism. The SI-window is determined with parameters of si-WindowLength and si-Periodicity. As a reference to the current parameter value range, the SI-window can be configured to adapt to the SSB periodicity by network implementation. For example: the si-Periodicity is configured as rf16 or multiples of rf16 and the si-WindowLength is configured with a suitable value to ensure the transmission of system information to be completed during the beam coverage time of the serving cell. As another possible configuration, the SI window length can be configured with 160 ms or multiples of 160 ms, which can be supported based on current configuration as well.

Hence based on current RRC spec [3], suitable configuration for SI scheduling can adapt to the 160 ms SSB periodicity, and no extra Spec impact is foreseen from RAN2 perspective, unless RAN1 requires any enhancements.

SI-SchedulingInfo ::= SEQUENCE { schedulingInfoList SEQUENCE (SIZE (1..maxSI-Message)) OF SchedulingInfo, si-WindowLength ENUMERATED {s5, s10, s20, s40, s80, s160, s320, s640, s1280, s2560-v1710, s5120-v1710 }, si-RequestConfig SI-RequestConfig OPTIONAL, -- Cond MSG-1 si-RequestConfigSUL SI-RequestConfig OPTIONAL, -- Cond SUL-MSG-1 systemInformationAreaID BIT STRING (SIZE (24)) OPTIONAL, -- Need R ... } SchedulingInfo ::= SEQUENCE { si-BroadcastStatus ENUMERATED {broadcasting, notBroadcasting}, si-Periodicity ENUMERATED {rf8, rf16, rf32, rf64, rf128, rf256, rf512}, sib-MappingInfo SIB-Mapping }

Observation 3: By network implementation, the SI scheduling in the current Spec can adapt to the SSB periodicity of 160 ms.

2) PCCH-Config

As specified in [4], the DRX cycle is determined by the shortest of the UE specific DRX value configured by RRC (if any), the UE specific DRX value configured by upper layers (if any) and a default DRX value broadcast in system information. The value range of all these DRX values is {rf32, rf64, rf128, rf256}. Furthermore, the paging frame and offset are configured with the parameter nAndPagingFrameOffset. When this parameter is set to oneSixteenthT, the PF period equals to 16rf. The paging configuration on paging cycle and PF can fulfil the needs for extended SSB periodicity of 160 ms, so that enhancement is not needed from RAN2 perspective.

nAndPagingFrameOffset CHOICE { oneT NULL, halfT INTEGER (0..1), quarterT INTEGER (0..3), oneEighthT INTEGER (0..7), oneSixteenthT INTEGER (0..15) },

Observation 4: The PCCH-Config in the current Spec can adapt to the extended SSB periodicity of 160 ms.

3) RACH configuration

The RACH occasion is configured per SSB with the parameter of ssb-perRACH-OccasionAndCB-PreamblesPerSSB and the mapping relationship of SS/PBCH block indexes to PRACH occasions is defined in [5]. It is up to RAN1 to check whether any enhancement is needed for PRACH related parameters due to extended SSB periodicity.

ssb-perRACH-OccasionAndCB-PreamblesPerSSB CHOICE { oneEighth ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64}, oneFourth ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64}, oneHalf ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64}, one ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64}, two ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32}, four INTEGER (1..16), eight INTEGER (1..8), sixteen INTEGER (1..4) } OPTIONAL, -- Need M

Observation 5: It is up to RAN1 to check whether any enhancement is needed for PRACH configuration.

4) Common SearchSpace configuration

The NW configures some common search spaces for UE (including searchSpaceOtherSystemInformation, pagingSearchSpace and ra-SearchSpace), so that the UE can determine the time domain resource of PDCCH monitoring occasions. When SearchSpaceID = 0 is configured for UE, the UE follows the predefined tables in [5], and it is up to RAN1 to check whether any enhancement is needed to for SSB periodicity 160 ms. When SearchSpaceID other than 0 is configured for these parameters, the UE applies the configurations provided via RRC signalling. After checking the current spec, we think the current value range can fulfil the possible requirement caused by the extended SSB periodicity of 160 ms (since the monitoring periodicity of search space can be up to 2560 ms for SCS 15 kHz and 320 ms for SCS 120 kHz). It seems there is no enhancement required on the configuration of search space from RAN2 perspective.

monitoringSlotPeriodicityAndOffset CHOICE { sl1 NULL, sl2 INTEGER (0..1), sl4 INTEGER (0..3), sl5 INTEGER (0..4), sl8 INTEGER (0..7), sl10 INTEGER (0..9), sl16 INTEGER (0..15), sl20 INTEGER (0..19), sl40 INTEGER (0..39), sl80 INTEGER (0..79), sl160 INTEGER (0..159), sl320 INTEGER (0..319), sl640 INTEGER (0..639), sl1280 INTEGER (0..1279), sl2560 INTEGER (0..2559) } OPTIONAL, -- Cond Setup4

Observation 6: The current common search space configuration provided via RRC signalling can adapt to the extended SSB periodicity of 160 ms.

Based on the above observations, we found no enhancement needed from RAN2 perspective due to SSB extension to 160 ms, w.r.t. SI, paging, RACH related aspects (unless any enhancement is required by RAN1). We propose this as follows:

Proposal 2: RAN2 confirms that the current RAN2 Spec can support the extended SSB periodicity of 160 ms. No enhancement on SI-scheduling, PCCH-Config, RACH configuration and Common SearchSpace is foreseen (unless required by RAN1).

2.2	Discussion on link level enhancement

For link level DL CE, RAN1 is discussing support of Type0 PDCCH CSS repetition, PDSCH repetition for SIB1 and Msg4 PDSCH repetition [6]. This section discusses possible RAN2 impacts based on latest RAN1 progress of RAN1#120bis.

- On access barring

In RAN1#120bis, the following agreement was achieved, based on which a legacy UE might decode the PDCCH and associated PDSCH in one slot, and this enables the legacy UE to acquire SIB1 and access the cell normally. But for M=1 and M=1/2, it is uncertain whether the legacy UE might also acquire SIB1 by decoding one of the repetitive PDCCH and associated PDSCH. So, it is still pending whether possible impacts on backward compatibility and access barring for link level enhancement is needed.

Agreement For PDCCH repetition for Type0 PDCCH CSS of searchSpaceZero configured within MIB pdcch-ConfigSIB1, support repeated PDCCH candidates in the two consecutive slots n0 and n0+1 associated with the same SSB index ( n0 as defined in section 13 of TS 38.213) at least for M=2. - Repeated PDCCH candidates share the same aggregation level (AL), coded bits and same candidate index - Note: if the network repeats the Type 0 PDCCH across two consecutive slots, a legacy UE might decode the PDCCH and associated PDSCH in one slot and skip PDCCH monitoring in the other slot. Note: further discuss the potential solution for M=1 and M=1/2.

Observation 7: Based on progress of RAN1, legacy UEs might successfully decode SIB1 when M=2. However, whether the legacy UE can decode SIB1 remains pending for M=1 and M=1/2.

- On Msg4 PDSCH repetition

This is related to whether the Msg4 PDSCH repetition is scheduled in a UE specific manner or not, which is further related to the down-selection on the following candidate options. Therefore, corresponding RAN2 discussions (e.g. whether any forms of early identification is needed as raised by some companies in last meeting) need be postponed till RAN1 make the down-selection first.

Agreement For enabling/disabling Msg4 PDSCH repetition, RAN1 to down-select among the following options: - Option 1: UE specific PDSCH with Msg4 repetition activation indicated via PDCCH- DCI Format 1_0 - FFS: indication details. - FFS: whether/how network is informed by the UE that certain conditions are met to trigger Msg4 PDSCH repetition (e.g. RSRP detected at UE is less than x dB) - Option 2: The enabling/disabling of Msg4 PDSCH repetition is implicitly indicated by the enabling/disabling of SIB1 PDSCH repetition. - Option 3: The enabling/disabling is indicated by SIB1 configuration - FFS: whether/how network is informed by the UE that certain conditions are met to trigger Msg4 PDSCH repetition (e.g. RSRP detected at UE is less than x dB) FFS: Whether UE reports its capability

Observation 8: RAN1 still needs to down select the options for informing UE about the Msg4 repetition and whether UE capability reporting is required.

Based on above situations, we think RAN2 still needs to wait for further firm RAN1 progress, in order to conclude possible RAN2 impacts on the link-level DL CE (e.g. barring, early identification, etc.).

Proposal 3: Postpone the discussion on link level enhancement, including access barring and early identification for Msg4 repetition, until RAN1 has clear progress.

Conclusion

In this contribution, DL coverage enhancement is further discussed with the corresponding observations and proposals listed below:

Observation 1: Current TS 38.331 supports the MIB broadcasted with the same periodicity as SSB when the periodicity of SSB is larger than 80 ms.

Observation 2: It is up to RAN1 whether any change is needed for the resource configuration used for SIB1 scheduling.

Observation 3: By network implementation, the SI scheduling in the current Spec can adapt to the SSB periodicity of 160 ms.

Observation 4: The PCCH-Config in the current Spec can adapt to the extended SSB periodicity of 160 ms.

Observation 5: It is up to RAN1 to check whether any enhancement is needed for PRACH configuration.

Observation 6: The current common search space configuration provided via RRC signalling can adapt to the extended SSB periodicity of 160 ms.

Observation 7: Based on progress of RAN1, legacy UEs might successfully decode SIB1 when M=2. However, whether the legacy UE can decode SIB1 remains pending for M=1 and M=1/2.

Observation 8: RAN1 still needs to down select the options for informing UE about the Msg4 repetition and whether UE capability reporting is required.

Proposal 1: RAN2 considers support of per SSB configured SMTC list in SIB2 and SIB4.

Proposal 2: RAN2 confirms that the current RAN2 Spec can support the extended SSB periodicity of 160 ms. No enhancement on SI-scheduling, PCCH-Config, RACH configuration and Common SearchSpace is foreseen (unless required by RAN1).

Proposal 3: Postpone the discussion on link level enhancement, including access barring and early identification for Msg4 repetition, until RAN1 has clear progress.

Reference

- RAN2_129bis_Meeting_Report.

- R2-2503055	Report of [AT129bis][303][R19 NR NTN] SMTC enhancements	Xiaomi	discussion	Rel-19	NR_NTN_Ph3-Core.

- 3GPP TS 38.331: "NR; Radio Resource Control (RRC) protocol specification"

- 3GPP TS 38.304: "User Equipment (UE) procedures in Idle mode and RRC Inactive state"

- 3GPP TS 38.213: "Physical layer procedures for control"

- RAN1#120bis Chair notes