3GPP TSG-RAN WG2 Meeting #130	R2-2503462

St. Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	8.17

Source: 	CATT

Title: 	Discussion on support of NB-IoT NTN TDD

Document for: 	Discussion and Decision

Introduction

In WID [1], RAN2 objectives are shown as follows:

This work item includes the following objectives: - Specify a new NB-IoT TDD NTN mode based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures, including: - Definition, configuration (if needed) and signaling (if needed) of the periodic pattern, necessary adaptation (if needed) and associated UE procedures [RAN1, RAN2] - Support a pattern with a period of 9 radio frames for the target MSS allocated band, where D=U=8 with a fixed guard period.RAN1 to consider whether there is a need for a mechanism to achieve an adjustable guard period for the purpose of allowing deployment with the TDD frame structure of the legacy system operating in the target MSS allocated band. - Other necessary impacts on higher layers [RAN2] - RRM and RF core requirements [RAN4]

This contribution analyzes potential RAN2 impacts to support NB-IoT TDD NTN on “other necessary impacts on higher layer” caused by the new frame structure of 90 ms period, based on the agreements of the RAN1 and RAN2 in the latest meeting.

Discussion

On MIB-NB and SIB1-NB

Whether there is any RAN2 impact on MIB-NB and SIB1-NB handling, based on RAN1 agreement, has not been concluded. This is discussed as follows.

Currently, the periodicity of MIB-TDD-NB is 640 ms, and repetition is made within 640 ms in the fixed subframe, as follows [2]:

The MasterInformationBlock-TDD-NB (MIB-TDD-NB) uses a fixed schedule with a periodicity of 640 ms and repetitions made within 640 ms. The first transmission of the MIB-TDD-NB is scheduled in subframe #9 of radio frames for which the SFN mod 64 = 0 and repetitions are scheduled in subframe #9 of all other radio frames. The transmissions are arranged in 8 independently decodable blocks of 80 ms duration.

Obviously, within one period, the MIB should be broadcast in subframe #9 every radio frame within 8 radio frames (i.e. 80 ms). However, in the 90 ms IoT NTN TDD frame structure, the DL channel is only kept for totally 8 ms as per RAN1 agreement. As a result, the subframes, which are determined by the current Spec to broadcast MIB with above periodicity and repetition pattern, are not ensured to be the DL channel with this 90 ms TDD configuration.

For SIB1-NB transmission, the transmission procedure for NB TDD in the current Spec is cited as follows: [2]

For TDD, SIB1-NB transmission on the anchor carrier occurs in either subframe #0 or subframe #4 of every other frame in 16 continuous frames and SIB1-NB transmission on a non-anchor carrier occurs in subframe #0 and next in subframe #5 of every other frame in 16 continuous frames. The starting frame for the first transmission of the SIB1-NB is derived from the cell PCID and the number of repetitions within the 2560 ms period and repetitions are made, equally spaced, within the 2560 ms period (see TS 36.213 [23]). TBS for SystemInformationBlockType1-NB, the repetitions made within the 2560 ms, and the subframe index (#0 or #4) are indicated by schedulingInfoSIB1 field in the MIB-TDD-NB

The SIB1 is broadcast with fixed period (2560 ms) and repetition with fixed subframe (e.g. either subframe #0 or subframe #4 of every other frame in 16 continuous frames). Similar as MIB, it cannot be ensured that the subframes needed for SIB1-NB transmission as per the current Spec are really the DL channel with this 90 ms TDD configuration. And RAN1 has left FFS on SIB1-NB in RAN1#120 meeting as shown the RAN1 agreements above.

The following agreement for MIB was achieved in RAN1#120 meeting [3]:

NPSS/NSSS/NPBCH transmissions are dropped in non-D NB-IoT subframes - NOTE: “non-D NB-IoT subframes” are subframes not contained within the set of D=8 usable consecutive downlink subframes in the TDD structure. - FFS: handling for SIB1-NB.

And the following agreement for SIB1-NB was further achieved in RAN1#120bis [4]:

Agreement SIB1-NB is dropped in non-D NB-IoT subframes - NOTE 1: “non-D NB-IoT subframes” are subframes not contained within the set of D=8 usable consecutive downlink subframes in the TDD structure. - NOTE 2: RAN1 observes that some combinations of {TBS, number of repetitions, additional SIB1 repetitions}, and depending on aspects such as minimum elevation angle or antenna gain, may not meet the link budget. It is up to the network to properly configure such values to ensure the link budget is met. No specification impact.

That is, the similar mechanism is adopted for MIB and SIB1-NB transmission in the new frame structure, according to RAN1 agreement. And for the potential link budget of SIB1-NB caused by some parameter combinations is left to network proper configuration. So for the MIB-NB and SIB1-NB transmission in the new TDD frame structure, RAN2 follows the agreement of RAN1 which may need to be captured in the system information procedure in TS 36.331 from the UE behavior point of view.

Proposal 1: RAN2 follows the agreement of RAN1 for the MIB-NB and SIB1-NB transmission in NB-IoT NTN TDD mode, and discusses whether to capture the UE behavior in TS 36.331 that the UE skips the reception of MIB/SIB1-NB in a non-D NB-IoT subframe, even if the subframe is determined as the subframe used to broadcast MIB-NB and SIB1-NB per the current Spec.

For the potential link budget issue caused by the dropped NPBSH and SIB1-NB, RAN2 can wait for the RAN1 input if they want to discuss this issue.

Other SI

SI is broadcast and repeated within SI window with the configured periodic and repetition pattern. In the current Spec, the length of SI window can be configured with the following values [2]:

si-WindowLength-r13					ENUMERATED {ms160, ms320, ms480, ms640,

ms960, ms1280, ms1600, spare1},

And the periodicity and the repetition pattern can be configured with following values:

si-Periodicity-r13				ENUMERATED {rf64, rf128, rf256, rf512,

rf1024, rf2048, rf4096, spare},

si-RepetitionPattern-r13		ENUMERATED {every2ndRF, every4thRF, every8thRF, every16thRF},

Within the SI-window, the corresponding SI message can be transmitted several times over 2 or 8 consecutive NB-IoT downlink subframes depending on TBS.

For example, the SI is broadcast within 8 consecutive NB-IoT downlink subframes every 2 frame. Similar to above discussion for MIB-NB and SIB1-NB, based on the 90 ms TDD configuration, other SI also suffer from the same problem that it is also difficult to ensure that the potential subframe and frame used to transmit SI as determined by the current Spec are really DL channel.

RAN1 has achieved some agreements for the NPDSCH transmission in RAN1#120 meeting [3], but with the SI case open:

Agreement The non-D NB-IoT subframes are not considered by the UE as “NB-IoT DL subframes” from the specifications - NPDCCH / NPDSCH (not containing SI) are mapped as per current specifications. - FFS: additional NB-IoT TDD-specific rules for handling non “NB-IoT DL subframes” - FFS: Introduction of new periodicities for NPDCCH monitoring (e.g. by the introduction of scaling factor) - FFS: whether all the configuration values for NPDCCH are valid for NB-IoT NTN TDD. - FFS: handling of DL transmission gap - NOTE: “non-D NB-IoT subframes” are subframes not contained within the set of D=8 usable consecutive downlink subframes in the TDD structure.

And RAN1 gave the following conclusion:

Conclusion It is RAN1’s understanding that the issue of scheduling and transmitting SI messages in the presence of non-D subframes will be solved mainly (or completely) by RAN2 specifications. RAN1 may discuss potential impact on RAN1 specifications, if any.

If the issue of scheduling and transmitting SI messages in the presence of non-D subframes is solved by RAN2, we think that, the simple solution is following the agreements of NPUSCH from RAN1 that, the non-D NB-IoT subframes are not considered by the UE as “NB-IoT DL subframes”, but the SI transmission should be postponed but not be dropped. This is important to guarantee the UE receive the SI successfully, especially for the UE needing coverage enhancement, because the UE may try to receive the SI during more than one successive SI windows.

Proposal 2: RAN2 confirms the understanding that the scheduling and transmission of SI (other than MIB/SIB1) should be postponed in the subframe which is determined for the SI transmission according to the current specification but is a non-D NB-IoT subframe.

Proposal 2a: RAN2 clarifies whether to capture the below UE behavior in the transmission procedure of other SIs in TS 36.331: UE skips the reception on a non-D NB-IoT subframe, even if the subframe is determined as the subframes used to broadcast an SI (other than MIB and SIB1) per the current Spec.

On PUR/UL SPS: clarification on RAN2 agreements

According to the current specification, if an uplink grant is indicated for a HARQ process for new transmission, the HARQ entity will obtain the MAC PDU to transmit from the "Multiplexing and assembly" entity, and then deliver the MAC PDU, the uplink grant and the HARQ information to the corresponding HARQ process for transmission.

The following agreements were achieved in RAN2#129bis for SPS and PUR:

4.	When PUR resource start subframe does not align with the UL subframes in the H-SFN, UE postpones the PUR resource start subframe to the next valid UL subframe

5.	When the UL SPS overlaps with non-U NB-IoT subframes UE postpones the UL SPS resource to the next valid UL subframe

That is, the UE will postpone the PUR resource start subframe or the UL SPS resource to the next valid UL subframe of the new TDD structure.

Then what RAN2 needs to further clarify is how to understand the "postpone" in the agreements, when an PUR/SPS transmission occasion is overlapped with non-U NB-IoT subframes, i.e. whether the UE really obtains the MAC PDU for new transmit from the "Multiplexing and assembly" entity or not for this PUR/SPS transmission occasion.

There can be the following two understandings. If a PUR/UL SPS occasion determined based on current Spec is overlapped with non-U NB-IoT subframe:

- Understanding 1: The PUR/UL SPS occasion is not considered as a valid UL grant at all, and no MAC PDU for new transmission is obtained from the MUX entity.

- Understanding 2: A MAC PDU for new transmission is still obtained and stored into the corresponding HARQ buffer for this PUR/UL SPS occasion, but the HARQ entity does not really perform the transmission of this MAC PDU on this specific occasion (so the transmission of the stored MAC PDU is delayed to later PUR/SPS UL grants with real UL NB-IoT subframe).

RAN2 may need to clarify which understanding is correct, w.r.t. the agreement of postponing PUR/UL SPS resources.

Proposal 3: If a PUR/UL SPS occasion determined as per current Spec is overlapped with non-U subframe, RAN2 confirms which understanding is correct w.r.t. the agreement on postponing the PUR/SPS resources:

- Understanding 1: The PUR/UL SPS occasion is not considered as a valid UL grant, and no MAC PDU for new transmission is obtained from the MUX entity for this PUR/UL SPS occasion.

- Understanding 2: A MAC PDU for new transmission is still obtained and stored in the corresponding HARQ buffer for this PUR/UL SPS occasion, but the HARQ entity does not really perform the transmission of this MAC PDU on this specific occasion (so the transmission of the stored MAC PDU is delayed to later PUR/SPS UL grants with real UL NB-IoT subframe).

Conclusion

In this contribution, the issue of RAN2 impact of TDD is discussed, with the following proposals:

Proposal 1: RAN2 follows the agreement of RAN1 for the MIB-NB and SIB1-NB transmission in NB-IoT NTN TDD mode, and discusses whether to capture the UE behavior in TS 36.331 that the UE skips the reception of MIB/SIB1-NB in a non-D NB-IoT subframe, even if the subframe is determined as the subframe used to broadcast MIB-NB and SIB1-NB per the current Spec.

Proposal 2: RAN2 confirms the understanding that the scheduling and transmission of SI (other than MIB/SIB1) should be postponed in the subframe which is determined for the SI transmission according to the current specification but is a non-D NB-IoT subframe.

Proposal 2a: RAN2 clarifies whether to capture the below UE behavior in the transmission procedure of other SIs in TS 36.331: UE skips the reception on a non-D NB-IoT subframe, even if the subframe is determined as the subframes used to broadcast an SI (other than MIB and SIB1) per the current Spec.

Proposal 3: If a PUR/UL SPS occasion determined as per current Spec is overlapped with non-U subframe, RAN2 confirms which understanding is correct w.r.t. the agreement on postponing the PUR/SPS resources:

- Understanding 1: The PUR/UL SPS occasion is not considered as a valid UL grant, and no MAC PDU for new transmission is obtained from the MUX entity for this PUR/UL SPS occasion.

- Understanding 2: A MAC PDU for new transmission is still obtained and stored in the corresponding HARQ buffer for this PUR/UL SPS occasion, but the HARQ entity does not really perform the transmission of this MAC PDU on this specific occasion (so the transmission of the stored MAC PDU is delayed to later PUR/SPS UL grants with real UL NB-IoT subframe).

Reference

- RP-243264 Revised WID on introduction of IoT-NTN TDD_cl

- 3GPP TS 36.331: "Universal Terrestrial Radio Access (UTRAN); Radio Resource Control (RRC)"

- Draft Report of 3GPP TSG RAN WG1 #120 v0.3.0

- Draft Report of 3GPP TSG RAN WG1 #120bis v0.1.0 (Wuhan, China, April 07th – 11th, 2025)