3GPP TSG-RAN WG2 Meeting #130	 R2-2503466

St Julian’s, Malta, May 19-23, 2025

Agenda Item:8.3.3

Source: NEC

Title: Discussion on AIML mobility Configuration and Report of Inference input to network sided model

Document for: Discussion and Decision

# 1 Introduction

In last RAN2#129b meeting, the following agreements are made:

=> The general LCM framework for beam management can be the baseline (where applicable) for AI mobility, such as the following aspects:

- Data collection for model training

- UE capability

- Applicability reporting

- Inference configuration and reporting

- Performance monitoring and management

= > Only functionality-based LCM is considered for AI/ML mobility (i.e. we don’t support model based LCM)

=> The UE applies inference configuration for the applicable RRM measurement prediction and measurement event prediction upon receiving configuration via RRC.  As baseline, no dynamic signaling (e.g. L1/2) is required.  FFS when the UE can perform inference

=> For RRM prediction, UE sided model, the UE can be configured with periodic or event triggered reporting of predicted and/or actual RRM measurements.  FFS details

=> For event prediction, UE-sided model, the UE can be configured with event-triggered reporting based on prediction and/or actual measurements.  FFS details

=> Baseline is that this applies to all A1-6 events, unless technical problems are identified.  Baseline quantity is RSRP.

=> The inference configuration and reporting for AI/ML mobility can be based on RRM measurement framework

=> For AI/ML for mobility, UE can report the applicable functionalities to NW via UAI or RRCReconfigurationComplete message.

FFS whether association ID is required and should provide motivation on per use case bases

=> As a baseline, use the AI ML PHY NW-side data collection procedures, configuration and reporting framework.

=> Study UE side data collection configuration, taking AI ML PHY relevant procedure as baseline. Postpone UE side data collection transfer discussion until further progress is made in AI ML PHY in R20.

=> Model transfer will not be discussed in this study item.

=> For UE sided model, the performance monitoring considered in AIML air interface can be studied as a baseline for AIML aided mobility, i.e. NW-side and UE-sided performance monitoring. FFS metrics per use case

In this contribution, we further discuss AIML mobility Configuration and Report of Inference input to network sided model.

# 2 Discussion

## 2.1 Configuration of Inference Inputs

In last RAN2#129b meeting, RAN2 has agreed that the inference configuration and reporting of input for AI/ML based network sided model needs to be considered. The network may configure periodic or event triggered inference reporting for the network sided AI/ML model. The data reported to the network for running the AI/ML model will merely not be enough. For dense scenarios where handover is more frequent, timely delivery for inference input is critical for efficient handover decision. The network may configure a maximum delay bound between the data collection and reporting by UE so that the validity of the inference input is maintained. In case of periodic reporting, it can be configured from the network that the inference input report must be sent to the network within some maximum time duration for ensuring timely mobility decision making. Similarly, in case of event triggered inference reporting, network should configure a maximum allowable delay from data collection to the reporting.

Proposal-1: Introduce periodic/event triggered inference reporting for network sided AI/ML models for mobility management based on delay thresholds in order to ensure timely mobility decisions.

## 2.2 Reporting of Inference Inputs

The UE may experience temporary poor quality inference inputs (for example, due to sudden deep fading). UE may indicate the confidence level of the inference inputs based on quality ( for example low, medium or high) while sending the report to the network. It may also supress the reporting based on certain threshold (either UE decision or sent in the inference configuration). Depending on the tag mentioned in the report, the network may treat those data separately.

Proposal-2: UE may indicate the confidence level of the inference inputs in terms of some reliability indicators to help network take some decision based on the quality of data received.

# 3 Conclusions

We have the following proposals for the configuration and reporting of inference inputs for the network sided model.

Proposal-1: Introduce periodic/event triggered inference reporting for network sided AI/ML models for mobility management based on delay thresholds in order to ensure timely mobility decisions.

Proposal-2: UE may indicate the confidence level of the inference inputs in terms of some reliability indicators to help network take some decision based on the quality of data received.