3GPP TSG-RAN2 Meeting #130	R2-2503471

# St.Julians, Malta, 19th May 2025 – 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.306 | CR | 1274 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on source band definition and UE reporting unsupported target band in pdcch-RACH-AffectedBandsList-r18 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Core |  |  |  |  |  | Date: |  |  | 2025-05-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The UE capability pdcch-RACH-AffectedBandsList-r18 is defined as whether the UE may cause an interruption on DL slot(s) on serving cells due to PDCCH-ordered RACH transmission, with the granularity of feature set. The UE reports a list with the length of the bands indicated in appliedFreqBandListFilter. Each element of the list has an ENUMERATED with two values {noInterruption, interruption}. In RAN2 #129 meeting, an LS (R2-2501392) was sent to RAN4 for the definition of the source band in source-target band pair for the granularity in pdcch-RACH-AffectedBandsList-r18. In the reply LS R2-2503325 (R4-2504940), RAN4 clairfies that the understanding 2 (The source band of the band pair represents the specific source band that have interruption or not) is the correct understanding and further explains that the band in which the PDCCH order is received has no impact on whether and on which band the interruption may occur. Therefore, the description of the capability should be updated accordingly for clarification. Additionally, if the UE does not support PDCCH-ordered RACH transmission for LTM towards certain target bands indicated in the appliedFreqBandListFilter, it is unclear whether the UE should report noInterruption or interruption for those elements. The related change was agreed in RAN2#129 meeting but was put on hold due to the above definition issue. Therefore, this change is also reintroduced to this CR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify the definition of source band in the source-target band pair represent the specific source band that have interruption or not (depends on the value “interruption” or “noInterruption”) in the current BC when UE performs PDCCH-ordered early RACH toward a target cell on target band. - Clarify that for those bands indicated in appliedFreqBandListFilter where the UE does not support PDCCH-ordered RACH towards target bands for LTM, it is up to UE implementation to select noInterruption or interruption for that element. Impact Analysis Impacted 5G architecture options: NR SA and NR-DC Impacted functionality: LTM Inter-operability analysis: - If the network is implemented according to the CR and the UE is not, there is no interoperability issue, but UE may not know how to indicate the serving band which UE may cause an interruption on DL slot when performing PDCCH-ordered early RACH. UE does not know how to select the values for those elements indicated in appliedFreqBandListFilter while UE does not support as target band for PDCCH-ordered RACH for LTM. - If the UE is implemented according to the CR and the network is not, network may misunderstand UE capability for the defination of the reported source band and may misunderstand those unsupported target bands of PDCCH-ordered RACH. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE may not know how to indicate the serving band which UE may cause an interruption on DL slot when performing PDCCH-ordered early RACH. UE does not know how to select the values for those elements indicated in appliedFreqBandListFilter while UE does not support as target band for PDCCH-ordered RACH for LTM. Network may misunderstand UE capability for the defination of the reported source band and may misunderstand those unsupported target bands of PDCCH-ordered RACH. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.7.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

4.2.7.5	FeatureSetDownlink parameters

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| pdcch-RACH-AffectedBandsList-r18 Indicates whether UE may cause interruption on DL slot(s) on serving cells due to PDCCH-ordered RACH transmission towards target bands. Each "source-target" pair indicates the band pair between the target band for RACH transmission and the source band under UE's current band combination. The source band of the band pair represents the specific source band that have interruption or not (depends on the value “interruption” or “noInterruption”) when UE performs PDCCH-ordered early RACH towards target band for LTM. The target bands only consist of the bands indicated in appliedFreqBandListFilter. They are listed in the same order as in appliedFreqBandListFilter and the first entry correspond to the first entry on appliedFreqBandListFilter and so on. For those bands indicated in appliedFreqBandListFilter where the UE does not support PDCCH-ordered RACH towards target bands for LTM, it is up to UE implementation to select noInterruption or interruption for that element. A UE supporting this feature shall also indicate support of rach-EarlyTA-Measurement-r18. | FS | No | N/A | N/A |