3GPP TSG-RAN2 Meeting #130	R2- 2503476

# St.Julian’s, Malta, May 19th – 23rd, 2025

Agenda item:		8.7.4.2 (NR_XR_Ph3)

Source:	LG Electronics Inc.

Title: 	Remaining issues on DSR enhancement for XR

Document for:	Discussion and Decision

1.	Introduction

According to WID of Rel-19 XR [1], it is agreed to specify the DSR enhancements with multiple pairs of remaining time and buffer size for an LCG.

- Specify Enhancements for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs, as follows [RAN2]: - Specify additional Logical Channel priority handling using delay/deadline information of packets; - Specify enhanced DSR (Delay Status Report) reporting with multiple pairs of remaining time and buffer size for an LCG.

During the PDCP running CR discussion [2], following open issue is left for DSR

- Issue 1: It is FFS which delay-reporting PDCP data volume shall consider PDCP Control PDUs, the PDCP SDUs to be retransmitted, and the PDCP Data PDUs to be retransmitted.

During the MAC running CR discussion [3], following open issue is left for DSR

- [MAC-03] DSR cancelation in DC configuration

In this documents, MAC and PDCP open issues for DSR enhancement are discussed.

2.	Discussion

Handling of PDCP/RLC control PDU and PDCP/RLC data to be retransmitted for delay-reporting data volume

In RAN2#129bis meeting[4], it was discussed how to handle the PDCP control PDUs and PDC SDUs to be retransmitted for delay-reporting data volume calculation with multiple reporting thresholds. Based on the discussion, it was agreed to calculated as either smallest “configured” threshold or smallest “reported” threshold.

During DSR data volume calculation the remaining time of retransmitted is not considered, i.e. it is always put either in the smallest configured or smallest reported threshold. FFS which one.

Based on the contributions submitted in RAN2#129bis meeting, there are different views on how to report the PDCP control PDUs and PDCP SDUs/PDCP Data PDUs to be retransmitted for AM DRBs as follows:

- Option 1: Reported with the shortest “configured” threshold for the corresponding LCG [5].

- Option 2: Reported with the shortest “reported” threshold for the corresponding LCG [6].

- Option 3: Reported with the shortest “reported” threshold for the corresponding PDCP/RLC entity [7].

For Option 1, it simplifies the UE behaviour on delay-reporting data volume calculation and follows the same approach as data volume calculation in Rel-18 DSR. In Rel-18 XR, it was agreed to include the PDCP/RLC control PDUs and retransmission data as delay-critical PDCP/RLC data volume, since these data are expected to be prioritised over other delay-critical PDCP SDUs. In other words, for each PDCP entity and each RLC entity, PDCP/RLC control PDUs and retransmission data is simply calculated as delay-critical data volume, without further consideration on delay status of other data buffered in PDCP/RLC entities. Similarly, with the multiple reporting thresholds, calculating the PDCP/RLC control PDUs and retransmission data as a shortest “configured” threshold for the corresponding LCG will simplifies the UE behaviour, as this option does not consider the buffer status of other data for the delay-reporting data volume calculation. Note that even though PDCP/RLC layer calculates the delay-reporting data for PDCP/RLC control PDUs and retransmission data, it may not be reported in R19 DSR MAC CE, if there is no pending DSR for the LCG [8].

6.1.3.72 Delay Status Report MAC CE A DSR MAC CE shall include delay status information of all LCGs which have pending DSRs when the MAC PDU containing this DSR MAC CE is to be built. (…omitted)

For Option 2, if the PDCP control PDU and PDCP SDUs/PDCP Data PDUs to be retransmitted for AM DRBs is reported with the shortest “reported” threshold for the LCG, it could avoid unnecessary overhead, when there is no delay-reporting data associated with the shortest configured threshold. In addition, if only one reporting threshold is configured for an LCG and it is same as triggering threshold, PDCP/RLC control PDU and retransmission data will be reported in Rel-18 DSR, i.e., the calculation PDCP/RLC control PDU and retransmission data is consistent with the existing DSR framework in Rel-18 DSR in this case.

However, given that the LCG may include multiple logical channels and the delay-reporting data volume calculation is performed in PDCP, it could cause additional UE complexity to consider inter-PDCP entity dependencies. For example, assume that LCG1 includes LCH1 and LCH2, and PDCP entity 1 is associated with LCH1 and PDCP entity 2 is associated with LCH2. If the reporting thresholds are configured as [10ms, 20ms, 30ms], then, for PDCP control PDU generated in PDCP entity 1, the delay status of data buffered in PDCP entity 2 should be checked in order to determine which reporting threshold should be considered for data volume calculation for PDCP control PDU (as depicted in Figure 1), considering that data volume calculation for each reporting threshold is performed in PDCP layer.

Figure 1.

If more than two PDCP entities are associated with an LCG, the UE complexity to consider the delay status of data in multiple PDCP entities would be further increased. In this sense, reporting PDCP/RLC control PDUs and retransmission data as shortest “reporting” threshold for the LGC (i.e., Option 2) is not desirable considering the UE complexity.

For Option 3, the inter-PDCP dependency can be avoided, and it reduces the overhead when there is no delay-reporting data associated with the shortest configured threshold, as in Option 2.

However, there is still additional UE complexity due to inter-layer interaction between PDCP layer and RLC layer if the same principle is applied for RLC control PDU and retransmission data. For example, assume that reporting thresholds are configured as [10ms, 20ms, 30ms]. Then, for RLC retransmission data, the delay status of data buffered in PDCP entity should be checked in order to determine which reporting threshold should be considered for data volume calculation for the RLC data to be retransmitted (as depicted in Figure 2), since the delay status of PDCP SDU is not indicated to the RLC layer unless the corresponding PDCP SDU is submitted to the RLC layer.

Figure 2.

In addition, even though there is delay-reporting data for another LCH in the same LCG, the PDCP/RLC control PDU and retransmission data will not be added as a delay-reporting data if the delay-reporting data for the LCH does not exist, which is different from the Rel-18 DSR. For example, assume that LCG1 includes LCH1 and LCH2, and PDCP entity 1 is associated with LCH1 and PDCP entity 2 is associated with LCH2. Then, if the PDCP entity 1 only has PDCP control PDU while PDCP entity 2 has delay-reporting data with the pending DSR, the PDCP control PDU 1 will not be calculated as delay-reporting data in Rel-19 DSR, while the PDCP control PDU in PDCP entity 1 is calculated as delay-critical data in Rel-18 DSR. In other words, for the same configuration, inconsistency between the Rel-18 DSR and Rel-19 DSR would occur without any clear motivation.

Given that the UE has tight processing time to generate DSR MAC CE after receiving UL grant, the UE complexity should be carefully considered for delay-reporting volume calculation with multiple reporting threshold. Therefore, in order to avoid any inter-layer or inter-PDCP entity dependencies and to simplify the UE operation for delay-reporting data volume calculation in Rel-19 DSR, the PDCP/RLC control PDUs and retransmission data should be reported with the shortest “configured” threshold (i.e., Option 1).

Proposal 1. The PDCP entity should consider followings for delay-reporting PDCP data volume calculation associated with the first configured reporting threshold:

-	the PDCP Control PDUs;

-	for AM DRBs, the PDCP SDUs to be retransmitted according to clause 5.1.2 and clause 5.13;

-	for AM DRBs, the PDCP Data PDUs to be retransmitted according to clause 5.5.

Proposal 2. The RLC entity should consider followings for delay-reporting RLC data volume calculation associated with the first configured reporting threshold:

- RLC data PDUs that are pending for retransmission (RLC AM).

- Estimated size of STATUS PDU.

For the case when only PDCP/RLC control PDUs and/or retransmission data is buffered for the LCG, it should not be reported in R19 DSR MAC CE, since it was not reported in R18 DSR MAC CE as well. However, as mentioned above, no further spec impact is needed, since the DSR MAC CE is required to report delay information only for LCG with pending DSR. In other words, if there is no PDCP SDU less than the triggering threshold for an LCG, there would be no pending DSR for the LCG, then the delay information is not needed to be reported for the LCG even though PDCP/RLC layer calculates the delay reporting data for PDCP/RLC control PDUs and retransmission data.

Proposal 3. If there is only PDCP/RLC control PDUs and/or retransmission data for an LCG, the delay information is not needed to be reported in R19 DSR MAC CE. No spec impact.

Meanwhile, if proposal 1 and proposal 2 are agreed, it could be ambiguous how to set the remaining time field, if there is no other data associated with the first configured reporting threshold, given that PDCP/RLC control PDU and retransmitted data are not associated with discard timer. In our view, for this specific case, the remaining time field in Rel-19 DSR MAC CE can be set to 0, which can explicitly indicate that the data volume associated with the shortest reporting time threshold only includes the PDCP/RLC control PDU and retransmitted data.

Proposal 4. If the delay-reporting data volume associated with shortest reporting threshold only includes the PDCP/RLC control PDU and retransmitted data, the remaining time field is set to 0.

DSR cancellation in DC case

In RAN2#129 meeting[9], it was discussed about the case that duplicated or empty DSR MAC CE may be transmitted in DC scenario, if DSR is triggered in both MAC entity for Rel-18 XR. However, no clarification agreed in Rel-18. Rather, it may be further discussed in Rel-19 DSR procedure.

R2-2500894 Discussion on DSR cancellation in DC Huawei, HiSilicon discussion Rel-18 NR_XR_enh-Core (…) [AT129][014][XR] DSR cancellation (Huawei) Intended outcome: Agree to intended behavior, including DSR cancelation when there is zero information. Deadline: Thursday No clarifications needed for Rel-18

As explained in [10], there are three conditions to cancel the pending DSR:

- Condition 1: when all the PDCP SDUs associated with the DSR have been discarded

- Condition 2: when a MAC PDU including DSR MAC CE is transmitted and this DSR MAC CE contains the delay information of all the PDCP SDUs associated with the DSR (as described in the clause 6.1.3.72).

- Condition 3: when MAC PDU is transmitted and this MAC PDU includes all the PDCP SDUs associated with the DSR

When the DSR procedure is triggered both in MAC entities, UE behaviour on DSR procedure for each cancellation condition is as follows:

- For condition 1, as indicated in [11], there would be no duplicated DSR MAC CE transmission, since the discard operation is performed in PDCP entity. In other words, as long as the PDCP SDUs associated with the DSR (based on remainingTimeThreshold for each MAC entity) is discarded, the DSR procedure will be cancelled in each MAC entity, and no further enhancement is needed.

- For condition 2, if both MAC entities trigger DSR procedure based on remainingTimeThreshold for each MAC entity, the UE will transmit the DSR MAC CE in both MAC entity, regardless of whether the other entity transmits DSR MAC CE or not. In other words, even though the delay information is reported via the other MAC entity, the DSR MAC CE will be transmitted in this MAC entity.

- For condition 3, if both MAC entities trigger DSR procedure based on remainingTimeThreshold for each MAC entity, the UE will transmit the DSR MAC CE in both MAC entity, regardless of whether the other entity transmits delay-critical data or not. In other words, even though the corresponding delay-critical data is transmitted via the other MAC entity, the DSR MAC CE will be transmitted in this MAC entity.

With this understanding, it should be discussed whether any enhancement is needed for condition 2 and condition 3 for DC case.

For condition 2, in our understanding, transmitting DSR MAC CE in both MAC entities would be helpful in some cases, even though all the delay information is reported in one MAC entity. Specifically, if MN and SN is not dynamically coordinated based on gNB implementation, even though all the delay information is transmitted to MN in MCG MAC entity, additional transmission of DSR MAC CE to SN is also needed in order to help the gNB scheduling for delay-critical data from SN side. Then, transmitting DSR MAC CE in both MAC entities would be helpful to receive the uplink grant for delay-critical data quickly via MN or SN in this case. Therefore, given that cancelling DSR procedure in this case is beneficial only in specific cases, no further enhancement of DSR cancellation for condition 2 is needed.

Proposal 5. For DC case, no further enhancements on DSR due to transmission of DSR MAC CE in the other MAC entity and this DSR MAC CE with the delay information of all the PDCP SDUs associated with the DSR.

For condition 3, it is true that an empty DSR MAC CE may be transmitted if the all the PDCP SDUs associated with the DSR is transmitted in the other MAC entity after the triggering DSR. In other words, if all the PDCP SDUs associated with the DSR are transmitted in the other MAC entity after triggering the DSR, there would be no ‘PDCP SDUs associated with the DSR’ exists at the time of the DSR MAC CE transmission in this MAC entity, so empty DSR MAC CE would be transmitted which causes resource waste. Therefore, it is desirable to cancel the DSR if all the ‘PDCP SDUs associated with the DSR’ are transmitted in the other MAC entity after triggering the DSR, since there is no information will be provided by transmitting empty DSR MAC CE.

On the other hand, during the discussion on [AT129][014][XR], companies agreed that the reasonable UE behaviour is to cancel the DSR in this case, but there were diverged views on whether the current spec cancels the DSR in Rel-18. Based on different understanding on the current behaviour, no further clarification was agreed in Rel-18 XR. However, for Rel-19 DSR, it would be better to have clearer text to cancel the DSR in this case.

Therefore, in order to clearly specify the desirable behaviour to cancel the DSR when all the PDCP SDUs associated with the DSR are transmitted by the other MAC entity, it should be clarified that DSR is cancelled when a MAC PDU is transmitted in any MAC entity and this MAC PDU includes all the PDCP SDUs associated with the DSR.

Proposal 6. For Rel-19, cancel the DSR when a MAC PDU is transmitted in any MAC entity and this MAC PDU includes all the PDCP SDUs associated with the DSR.

3.	Conclusion

In this contribution, it is discussed for the DSR enhancement for XR. This document includes following proposals.

Proposal 1. The PDCP entity should consider followings for delay-reporting PDCP data volume calculation associated with the first configured reporting threshold:

-	the PDCP Control PDUs;

-	for AM DRBs, the PDCP SDUs to be retransmitted according to clause 5.1.2 and clause 5.13;

-	for AM DRBs, the PDCP Data PDUs to be retransmitted according to clause 5.5.

Proposal 2. The RLC entity should consider followings for delay-reporting RLC data volume calculation associated with the first configured reporting threshold:

- RLC data PDUs that are pending for retransmission (RLC AM).

- Estimated size of STATUS PDU.

Proposal 3. If there is only PDCP/RLC control PDUs and/or retransmission data for an LCG, the delay information is not needed to be reported in R19 DSR MAC CE. No spec impact.

Proposal 4. If the delay-reporting data volume associated with shortest reporting threshold only includes the PDCP/RLC control PDU and retransmitted data, the remaining time field is set to 0.

Proposal 5. For DC case, no further enhancements on DSR due to transmission of DSR MAC CE in the other MAC entity and this DSR MAC CE with the delay information of all the PDCP SDUs associated with the DSR.

Proposal 6. For Rel-19, cancel the DSR when a MAC PDU is transmitted in any MAC entity and this MAC PDU includes all the PDCP SDUs associated with the DSR.

4.	Reference

[1] RP-241771 Revised WID on XR (eXtended Reality) for NR Phase 3

[2] R2-2503697 Discussion of [POST129bis][504][XR] PDCP running CR (LGE)

[3 R2-2503361 List of open issues in MAC

[4] Report of 3GPP TSG RAN WG2 meeting #129bis, Wuhan, China, 7 - 11 April, 2025

[5] R2-2502181 On Data Volume Calculations for Rel-19 DSR Apple, et. al.

[6] R2-2502301 Discussion on DSR enhancements in XR Huawei

[7] R2-2501947 Discussion on DSR Enhancements Sharp

[8] TS38.321 v.18.5.0

[9] Report of 3GPP TSG RAN WG2 meeting #129, Athens, Greece, 17-21 February, 2025

[10] R2-2500894 Discussion on DSR cancellation in DC	Huawei

[11] R2-2501763 Discussion on DSR enhancements Qualcomm Incorporated