3GPP TSG-RAN WG2 Meeting #130	R2-2503483

Malta, May 19th –23rd , 2025

Agenda Item:	8.2.4

Source: 	Xiaomi

Title:  	Remaining open issues on Data transmission

Document for:	Discussion and decision

# Introduction

In last RAN2 meeting, RAN2 discussed data transmission and the MAC PDU design. However following issues still need further discussion:

- Subgroup: Segmentation

  - Issue 3-1: Command for non-first segment

  - Issue 3-2: Offset for first segment

-	Subgroup: AS ID

  - Issue 3-3: AS ID release

-	Subgroup: D2R message content for data transmission

  - Issue 3-4: D2R padding indication

  - Issue 3-5: D2R message type

In this contribution, we continue the discussion on these remaining open issues and provide corresponding proposals.

# Discussion

## Segmentation

### 2.1.1 Issue 3-1: Command for non-first segment

As listed by Rapporteur, the issue is

| Issue 3-1: command for non-first segment | Whether upper layer command is included in the R2D message scheduling for non-first segment. - Relevant agreements: - FFS whether the reader always includes the command for retransmission of segments. - Status in running CR: captured as Editor’s Note in 5.4.2. |
| --- | --- |

During the online discussion, some companies raised the concern on the processing complexity of the device if command is always included. There are different device implementations:

Option 1: The device forwards both offset and command to upper layer, and upper layer generates the segment based on it;

Option 2: The device forwards the command to upper layer, and upper layer sends the result to AS layer, and AS layer generate the D2R message based on the offset.

Note, upper layer may read the memory based on the address indicated in the command message.

If the command is not included in the retransmission trigger message, i.e., only offset is contained:

Option 3: The device maintains the command response in AS layer, and cuts the data based on the offset.

Option 4: The device maintains the starting/ending point of the entire D2R response in AS layer, and forwards both offset and the starting/ending point of the entire D2R response to upper layer, rest see option 1/2.

Obviously, if only offset is indicated without the R2D command, from the device’s perspective, at least the starting/ending point of the entire D2R response still needs to be remembered, which will increase the requirements on the storage. Considering the motivation to rely on the reader to indicate the offset explicitly is to relax the memory of the device, always including the command is more aligned with the intention. Otherwise, the benefit of this method is not significant, i.e., only saving the memory of an offset. Another benefit to include the command for retransmission of non-first segment is to have unified format design since we agree to re-send the command for the retransmission of first segment/unsegment.

During the online discussion, another concern from companies is whether the command content will be changed or not? If it is the case, only option 3 can work. That means additional huge memory is needed which is not possible for A-IOT devices. In addition, that also means, the AIOTF cannot get the latest results if it can be changed in ms level.

Therefore, we think the R2D command should be included together with the offset for the retransmission of each non-first segment, i.e. option 1 and 2. But we do not need to specify the device behavior, i.e., how to handle the retransmission can be up to device implementation.

Proposal 1: (Issue 3-1) RAN2 to agree that for the retransmission of the non-first segment D2R message, the reader sends the R2D message by including the upper layer command again.

### 2.1.2 Issue 3-2: Offset for first segment

As listed by Rapporteur, the issue is

| Issue 3-2: offset for first segment | Whether offset is included in the R2D message scheduling for the first segment and unsegmented message - Relevant agreements: - For the retransmission of the first segment/unsegmented D2R message, the reader sends the R2D message by including the upper layer command again. FFS whether offset zero is always included. - Status in running CR: captured as Editor’s Note in 5.4.3. |
| --- | --- |

Actually, for the first segment and unsegmented message, the offset zero is not needed. However, always including the offset even for offset zero can achieve a unified format design and avoid additional signalling to indicate the presence of offset, which is also beneficial to reduce the processing complexity for the device. Therefore, we suggest to always include the offset zero when triggering the retransmission of the first segment/unsegmented D2R response.

Proposal 2: (Issue 3-2) RAN2 to agree that offset zero is always included for transmission/retransmission of first segment/unsegmented D2R response.

## AS ID

### 2.2.1 Issue 3-3: AS ID release

As listed by Rapporteur, the issue is

| Issue 3-3: AS ID release | Whether to specify any additional AS ID release method - Relevant agreements: - FFS other cases for release ASID to avoid keeping it indefinitely. - Status in running CR: captured as Editor’s Note in 5.2. |
| --- | --- |

In last meeting, RAN2 discussed the case whether the device shall release the AS ID upon receiving the paging with new transaction ID but not for itself. Some companies would like to consider parallel session/service request case. It is also related to the definition of “ongoing”. We do see the following issues if the device still keeps the AS ID upon receiving the paging with new transaction ID but not for itself:

- Issue 1: During the new paging procedure, the reader may need to allocate AS ID to the devices, if the AD ID in the previous paging round is still kept in use, collision may happen, especially when huge group of devices are handled in sequence.

- Issue 2: From Reader perspective, it has no idea which devices are addressed by the group ID. Therefore, the previous agreements does not work. It is useless for reader since the reader cannot know when the device will release the AS ID.

To address above issues, the device shall release the AS ID upon receiving the paging with new transaction ID, no matter whether the paging ID is for itself or not.

Proposal 3: (Issue 3-3) RAN2 to agree that AS ID should be released upon receiving paging message with a new transaction ID, no matter whether the paging is for that device or not.

## D2R message content for data transmission

### 2.3.1 Issue 3-4: D2R padding indication

As listed by Rapporteur, the issue is

| Issue 3-4: D2R padding indication | How to indicate padding and the Length filed for SDU or padding and its size - Relevant agreements: - In case where MAC PDU includes both MAC SDU and padding, for D2R a field to indicate how many SDU bits are present is required. FFS how this is provided (i.e. SDU length field or padding length field). The size of length field is FFS. - Status in running CR: captured as Editor’s Note in 6.2.2.2. |
| --- | --- |

To indicate the length of the MAC SDU is straightforward and the reader can get this information directly. If as large as 1000bits is transmitted, the required size of the length field is up to 7 bits.

While to indicate the length of the padding may reduce the size of the length field, however, the reader needs to calculate the size of the MAC SDU based on this information and the TBS by implementation. In addition, how much padding to be included also depends on the TBS, thus the maximum size of padding needs to be firstly concluded and then used to determine the size of the length field.

We think either to indicate the length of padding or the length of the MAC SDU can work, we slightly prefer to indicate the length of the MAC SDU directly considering it is a straightforward solution and the required size of the length field is determined, i.e., 7 bits to support the maximum 1000-bit message size.

Proposal 4: (Issue 3-4) RAN2 to agree to define a SDU length field to indicate the length of MAC SDU. The size of the SDU length field is 7 bits.

### 2.3.2 Issue 3-5: D2R message type

As listed by Rapporteur, the issue is

| Issue 3-5: D2R message type | Whether to support D2R message type - Relevant agreements: - FFS whether we introduce D2R message type. Discuss after looking at the overall MAC header design and space before deciding whether we introduce message type or reserved bits - Status in running CR: captured as Editor's Note in 6.1.1. |
| --- | --- |

Currently there are two types of D2R MAC PDU, i.e., D2R MAC PDU (MSG1) and D2R MAC PDU (MSG3 and data). The resource for the transmission of D2R MAC PDU (MSG1) is a common resource for RACH while the resource for the transmission of D2R MAC PDU (MSG3 and data) is a dedicated resource. Since both the common resource and dedicated resource are under the reader’s control, the reader should be able to separate them, then based on the resource where a D2R MAC PDU is received, and the transmitted R2D message, the reader is able to distinguish whether it is a Msg 1 or Msg3/data and which R2D message the D2R message responds to. From this perspective, there is no need to define the message type for D2R MAC PDU.

Proposal 5: (Issue 3-5) RAN2 to agree that D2R message type is not needed.

## R2D message content (Issue 2-7)

In last meeting, we confirmed AS ID only applies to inventory+command procedure. And according to the open issue list, there is an issue raised by the rapporteur on how to indicate the presence/absence of the AS ID.

| Issue 2-7: present/absent indication of assigned AS ID in Msg2 | How to indicate the AS ID presence in Msg2. - Based on the following agreement, the Rapp understands the assigned AS ID is an optional field in Msg2, but how to indicate the presence/absence has not been discussed yet. This can be considered as signaling design/stage3 issue which should be quite straightforward. Companies can check the proposal in 2.2. - For CBRA, it is up to Reader to decide whether to reuse the random ID as the AS ID or to assign a new AS ID. FFS how this is signalled, which message is used and size of AS ID. - For CBRA, Msg 2 is used for AS ID assignment. - Status in running CR: captured as Editor's Note in 6.2.1.3. |
| --- | --- |

In the email discussion, Rapporteur indicated their preference is

The Rapp understands the most straightforward method is to have a bit to indicate whether AS ID is assigned for each entry of the random ID list. ### Proposal for Issue 2-7: To have a bit to indicate AS ID presence/absence for each entry of the random ID included in Msg2.

During the email discussion, companies agreed to have a bit to indicate AS ID presence/absence for each entry of the random ID in Msg 2.

In our understanding, the AS ID assignment is only needed for Inventory +Command procedure, and it is not needed for Inventory only procedure. If we follow the proposal from Rapporteur “[Easy] [8/9]Proposal for Issue 2-7:	One bit indication is needed for each echoed random ID in Msg2 to indicate whether AS ID is present (i.e., assigned by reader) for this random ID.”, that means 1 bit indication per echoed random ID is needed in Msg2 even for Inventory only procedure.

From signaling design perspective, it increases additional signaling overhead especially when the Msg2 includes a number of RN16. Therefore, unified Msg2 format for Inventory only and Inventory + command is not a good choice, it is better to have different message for these two procedures, i.e., use msg type to distinguish the Msg2 for inventory only and the Msg2 for inventory+command. Based on the msg type, the device is able to identify whether AS ID is applicable or not and further determine whether to reserve the register memory for the storage of AS ID.

Proposal 6: (Issue 2-7) Introduce different R2D msg type for Msg2 used for inventory only procedure and Msg2 used for invenroty+command procedure.

## RN16 validity

For CBRA, the RN16 included in Msg1 should be stored as long as generated and used to determine if the contention resolution is successful or not. Even a different AS ID may be allocated in Msg2, the AS ID cannot replace the RN16 directly because the device may fail the transmission of Msg3 and the reader may resend the Msg2 to trigger a retransmission of Msg3. The reason why AS ID cannot be used in Msg 2 to trigger Msg3 retransmission is because the reader cannot distinguish between the case that device fails to receive Msg2 and the case that the reader fails to receive the Msg3. Therefore, to trigger a retransmission of Msg3 for a device, AS ID contained in Msg2 does not work for the case when the device fails to receive Msg2, RN16 is the only ID to identify the device before the successful transmission of Msg3. Therefore, RN16 should be stored and cannot be replaced by AS ID.

Observation 1: For CBRA, RN16 should be stored separately and cannot be replaced directly by AS ID.

Regarding the validity of RN16, as we analysed above, to support retransmission of Msg3, RN16 should be stored until the successful transmission of Msg3. Based on the following agreement from last meeting, for Msg3, the device considers no NACK until subsequent R2D message that triggers re-access, which means the device can consider the Msg3 transmission as successful if no NACK received after Msg3 transmission till the subsequent R2D message that triggers re-access and, in this case, RN16 can be released.

Agreements on NACK

- For CBRA, as a baseline, NACK based mechanism is applied only to the Msg3.   May come back for D2R data, if the NACK feedback indication is needed for the purpose to stop/terminate the “on-going procedure” and release the AS ID accordingly (depending on other later discussion).

- For msg3, we rely on whether the device receives NACK indication before subsequent R2D message to determine re-access.    No need for a timer.   FFS whether subsequent R2D message is trigger message or paging

- For CFRA, NACK feedback and re-access is not supported.  FFS how to achieve

- FFS on end of procedure

Another case to release the RN16 is that NACK for this device is received after Msg3 transmission. As we agreed before, NACK is used to trigger re-access. Upon receiving the NACK, the device considers the transmission of Msg3 as failed and re-access is triggered, during which a new RN16 will be generated and the existing RN16 is no longer valid. Therefore, upon receiving the NACK after Msg3 transmission, RN16 should be released.

In addition, similar as AS ID, if paging message with a new transaction ID is received, no matter whether the paging message is for the device or not, RN16 should be released. The reason to release the RN16 is as same as what we analysed in section 2.2.1.

Alternatively, if there is no requirement on the release of RN16, the device can store the RN16 until it is replaced by a new RN16, i.e., when it triggers new msg1 transmission as a result of receiving paging message (i.e., it has to generate a new RN16 for CBRA)

Proposal 7: RAN2 to agree that for CBRA, RN16 should be stored separately as long as generated and released/replaced upon:

- Successful transmission of Msg3, i.e., upon reception of subsequent R2D message triggering re-access with no NACK received

- Receiving the NACK for the device after the transmission of Msg3

- Receiving the paging message with a new transaction ID, i.e., different session/service

- Replacement by a new RN16, i.e., when it triggers new msg1 transmission as a result of receiving paging message (i.e., it has to generate a new RN16 for CBRA)

# Conclusions

Based on the above discussion, we propose following proposals:

Segmentation

Proposal 1: (Issue 3-1) RAN2 to agree that for the retransmission of the non-first segment D2R message, the reader sends the R2D message by including the upper layer command again.

Proposal 2: (Issue 3-2) RAN2 to agree that offset zero is always included for transmission/retransmission of first segment/unsegmented D2R response.

AS ID

Proposal 3: (Issue 3-3) RAN2 to agree that AS ID should be released upon receiving paging message with a new transaction ID, no matter whether the paging is for that device or not.

D2R message content

Proposal 4: (Issue 3-4) RAN2 to agree to define a SDU length field to indicate the length of MAC SDU. The size of the SDU length field is 7 bits.

Proposal 5: (Issue 3-5) RAN2 to agree that D2R message type is not needed.

R2D message content

Proposal 6: (Issue 2-7) Introduce different R2D msg type for Msg2 used for inventory only procedure and Msg2 used for invenroty+command procedure.

RN16 validity

Observation 1: For CBRA, RN16 should be stored separately and cannot be replaced directly by AS ID.

Proposal 7: RAN2 to agree that for CBRA, RN16 should be stored separately as long as generated and released/replaced upon:

- Successful transmission of Msg3, i.e., upon reception of subsequent R2D message triggering re-access with no NACK received

- Receiving the NACK for the device after the transmission of Msg3

- Receiving the paging message with a new transaction ID, i.e., different session/service

- Replacement by a new RN16, i.e., when it triggers new msg1 transmission as a result of receiving paging message (i.e., it has to generate a new RN16 for CBRA)

# Reference

[1] RAN2#129bis meeting agreement 
[2] R2-2503998 Summary of A-IoT MAC open issues