3GPP TSG-RAN WG2 Meeting #130	R2-2503484

St. Julians, Malta, May. 19th – 23rd, 2025

Agenda item:		8.6.3

Source:		Lenovo

Title:		Discussion on L1 measurement

Document for:		Discussion and Decision

# Introduction

The following has been agreed in the RAN2#129bis meeting [1]. This contribution aims to discuss the potential issues for the event-based measurement report.

# Discussion

SP CSI-RS resource for L1 measurement

Last meeting, RAN2 postponed the discussion for the above proposal because there was no consensus. Regarding the above proposal, UE determines whether to stop or continue measurement after cell switch is completed. Actually, UE can decide it already when the cell switch is triggered since it is obvious that UE does not need to measure the SP CSI-RS resource while T304 is running. For example, it can stop measurement on SP CSI-RS resource (if option 1 is selected) already as soon as the cell switch is triggered. Therefore, we update the same proposal as follows:

Proposal 1: RAN2 to discuss which option can be used to handle the activated SP CSI-RS resource for L1 measurement:

Option 1: UE changes the state for activated semi-persistent CSI-RS resources to deactivated state and stops measurement once cell switch is triggered.

Option 2: UE continues measuring the activated semi-persistent CSI-RS resources irrespective of if cell switch is triggered.

Option 3: UE changes the state for activated semi-persistent CSI-RS resources to deactivated state and stops measurement for the ‘remaining’ candidate cells (i.e. other than target cell) once cell switch is triggered. UE continues measuring the activated semi-persistent CSI-RS resources of only the target cell after cell switch is triggered.

SP CSI-RS resource for CSI acquisition

It was agreed in RAN1 meeting that the semi-persistent CSI-RS resource is supported for candidate cell CSI acquisition. We already agreed to introduce a new MAC CE to activate/deactivate the SP CSI-RS resource for L1 measurement purpose. UE can identify whether the CSI-RS resource set is associated with L1 measurement (to measure e.g., RSRP) or for CSI acquisition (for CSI reporting) based on resource configuration ID and repetition configuration. In legacy for the serving cell, one single MAC CE is specified for both purposes i.e., for activation/deactivation of SP CSI-RS resource for L1 measurement and for CSI acquisition purpose. Therefore, we propose:

Proposal 2: A single MAC CE is specified to activate/deactivate the semi-persistent CSI-RS resource from candidate cell for both L1 measurement and CSI acquisition purposes.

RAN1 has not decided when UE transmits the CSI report. The potential time is MSG3 message for RACH based LTM. Regarding RACHless LTM cell switch, it can be carried by the first transmission. Regardless of when the CSI report is transmitted, RAN2 can discuss how to handle the activated SP CSI-RS resource for CSI acquisition from target cell and other candidate cell after cell switch is triggered towards the target cell. On target cell, whether the UE will transmit CSI report during cell switch or after completed cell switch? Therefore, RAN2 can discuss which option can be used to handle the activated SP CSI-RS resource for CSI acquisition from target cell included in LTM command MAC CE after the first CSI information has been transmitted to target cell:

- Option 1: UE changes the state for activated semi-persistent CSI-RS resources to deactivated and stops CSI measurement after L1 CSI report has been transmitted to target cell.

- Option 2: UE continues measuring the activated semi-persistent CSI-RS resources after L1 CSI information has been transmitted to target cell.

Proposal 3: UE continues measuring the activated SP CSI-RS resource for CSI acquisition from target cell after L1 CSI information has been transmitted to target cell.

Regarding the activated semi-persistent CSI-RS resources from other candidate cells (not target cell), if UE stops measuring the SP CSI-RS resource, UE can stop when the cell switch is triggered. Therefore, we propose:

Proposal 4: RAN2 to discuss which option can be used to handle the activated SP CSI-RS resource for CSI acquisition from other candidate cells:

Option 1: UE changes the state for activated semi-persistent CSI-RS resources to deactivated and stops measurement after cell switch is triggered.

Option 2: UE continues measuring the activated semi-persistent CSI-RS resources after cell switch is triggered.

MR MAC CE

Based on the RAN1 LS, RAN1 assumes at maximum 16 beams can be reported by a single event triggered reporting regardless of whether or not the report includes the current beam. It is noted that the maximum number of beams that can be reported by a UE is subject to UE capability which is under discussion in RAN1. Whether current beam is counted in the maximum 16 beams is related to UE capability. Therefore, for the UE with high capability, UE can support max. 16 beams from the candidate cells and one current beam to be reported. If UE can not support max. 16 beams from the candidate cells, UE can report another value e.g., 8 beams from the candidate cells.

Proposal 5: UE can support maximum 16 beams from the candidate cells and one current beam in one MR MAC CE.

For structuring (=format) the MR MAC CE, following two possibilities can be considered:

Possibility 1: The beam with highest RSRP is included first in the report as an absolute value (which may or may not be Event triggered Type1/ 2/ 3 beam), as agreed in the previous meeting.

Agreement: - For differential L1-RSRP reporting, the best quality beam among the beams included in L1 MR MAC CE is taken as the reference beam as the first one. The differential L1-RSRP value is derived based on the absolute L1-RSRP of the reference beam. FFS for truncated MAC CE.

Remaining beams to be included using differential reporting. From the below agreement, it is not clear if the intention is to include only one triggered Beam or to maximize transmission of triggered Beams (i.e., any Type3/ 4 will only be included if all Type1/ 2 have been included):

- When UL grant is enough to accommodate only limited number of beams in MR MAC CE, the triggered beam should be included in the truncated MR MAC CE.

Possibility 1 requires UE to rearrange the full MR MAC to carve out a truncated MAC CE. But with this possibility, RAN2 will need to consider many other cases since “rearrangement” will be different depending on how much space is available for reporting the MAC CE. For example, should we include current Beam (so network configuration of including current beam also applies to truncated MR MAC CE) or rather other beams, as shown in Figure 1?

Figure 1

If RAN2 wants to stay with the agreements from last meeting, following additional proposals need to be discussed:

Proposal 6a: In a truncated MR MAC CE UE maximizes inclusion of Type1/ 2 beams (i.e., any Type3/ 4 will only be included if all Type1/ 2 have been included).

Proposal 6b-alt1: In a truncated MR MAC CE current beam is never included.

Proposal 6b-alt2: In a truncated MR MAC CE current beam is included if all Type1/ 2 beams are included.

Proposal 6b-alt3: In a truncated MR MAC CE current beam is included if at least one Type1/ 2 beam is included.

There might be some further permutations/ combination with possibility 1. A better option is available with the below possibility.

Possibility 2: The event triggering beam(s) i.e., Type1 or Type2 are included first in the report. This approach allows a “natural” truncation wherein the event triggering beam(s) will be included first instead of a non-triggering beam even if the quality (RSRP) of the latter may be better at the time of reporting. The differential reporting of measurement is used and is done with respect to the best quality (RSRP) being reported – which could be included after the event triggering beam(s) have been included. Among event triggering beam(s) and non-event triggering beam(s) the quality order is followed. One bit indication (AD) for each Quality value needs to be included indicating if the corresponding Quality is “Absolute” or “Differential”.

Possibility 2 above can support “natural” truncation which means the UE need not rearrange the MR MAC CE after determining that the complete MR MAC CE can’t be included in the remaining grant. This type of run-time change is complex to handle, causes processing overhead and may lead to slippage in timing to transmit the codeword. Based on the running CR, there is one ‘R’ bit for every included differential RSRI and as shown as an example below, this can come for free:

Figure 2

Proposal 7a: Beam with highest RSRP among the event triggering beam(s) (Type1/ 2) is included first in the report.

Proposal 7b: A 1-bit indication to differentiate Absolute or Differential RSRP (AD bit) is included.

With the possibility 2, the Truncated MR MAC CE includes starting from first Octet, as many as can be accommodated in the (remaining) grant. If inclusion of the ‘AD’ bit can come for free, as shown in Figure 2, we prefer RAN2 to agree on Proposal 7 (rather Proposal 6), but as proponent we can accept any of these Proposals.

Tail Reporting

As another issue, in an offline in the previous (Wuhan) meeting, it was briefly discussed if after truncation (once truncated MR MAC CE), the “tail” (i.e., remaining part of the complete MR MAC CE) should also be reported. The arguments in favor of reporting the tail could be that not all event triggering beams (Type1 or Type2) could be included in the already transmitted truncated MR MAC CE. However, this is not easy as it seems that therefore treating the not-yet-reported Type1/ 2 beams should be reported as “fresh” MR MAC CE if the beams still qualify for the even triggering. To cover each possibility will be overtly complex and therefore we suggest:

Proposal 8: No Tail reporting is done. The UE prepares a complete MR MAC CE if any of the ‘remaining’ Type1 or Type2 beams could not be included in the previous truncated MR MAC CE, if the remaining beams still fulfills the event triggering condition.

Useful optimizations

One MR MAC CE contains reporting of only one measurement event, and it is possible that there are more than one events to be reported for different cells. When the UL grant is limited and not all triggered/ generated measurement reports can be included, the following two possibilities exist:

- UE includes the most recent measurement report(s) when not all the triggered reports can fit in the available UL grant.

- Alternatively, the UE may include more than one truncated MR MAC CEs if there are more than one Measurement Events triggered. This ensures that the event triggering beams are prioritized over the non-event triggering ones.

We think that it is useful to maximize Type1/ 2 beam reporting over Type3/ 4 beam reporting. And therefore, we propose:

Proposal 9: When UL grant is not sufficient to include all Full MRs (each with many Type3/ 4 beams) and therefore can only include 'm' Full MRs, UE shall rather maximize reporting of triggered beams (Type1/ 2) by including 'n' truncated MRs in the TB (n < m). Exact implementation is left to the UE implementation.

It was agreed in RAN2 that the L1 MR MAC CEs can be multiplexed in one MAC PDU if multiple MR MAC CEs are triggered. That means some beams will be included in different MAC CEs. The UL resource can be limited and therefore it seems useful that UE includes a beam(s) only once and only in the latest triggering measurement report in a MAC CE if there are potentially multiple MRs with the same beam. Same applies to the current beam of the serving cell i.e., the current serving beam is included only once in the MAC TB.

Proposal 10: UE includes a beam(s) once and only in the latest triggering measurement report in a MAC PDU if there are potentially multiple MRs with the same beam. Same applies to the current beam of the serving cell i.e., the current serving beam is included only once in the MAC PDU.

Since the MR MAC CE can only include reporting for only one event and due to sheer number of beams across current and candidate cells (= 2*64 + 8*64), the reporting in field can go uncontrollably high. Someway to control such reporting could be useful. For example, the network configures a prohibit timer to a UE. Once the MR is sent, the prohibit timer is restarted and the UE may not report a MR MAC CE as long as the timer is running. Or, to restrict reporting, a serving network configures a delta value for beam quality such that the beam quality of a beam reported in the previous report changes more than by the configured delta value, only then the beam is reported again.

Proposal 11: RAN2 considers means to control amount of MR MAC CE reporting.

# Conclusion

In this contribution, the following proposals are given based on the discussion:

SP CSI-RS resource for L1 measurement

Proposal 1: RAN2 to discuss which option can be used to handle the activated SP CSI-RS resource for L1 measurement:

Option 1: UE changes the state for activated semi-persistent CSI-RS resources to deactivated state and stops measurement once cell switch is triggered.

Option 2: UE continues measuring the activated semi-persistent CSI-RS resources irrespective of if cell switch is triggered.

Option 3: UE changes the state for activated semi-persistent CSI-RS resources to deactivated state and stops measurement for the ‘remaining’ candidate cells (i.e. other than target cell) once cell switch is triggered. UE continues measuring the activated semi-persistent CSI-RS resources of only the target cell after cell switch is triggered.

SP CSI-RS resource for CSI acquisition

Proposal 2: A single MAC CE is specified to activate/deactivate the semi-persistent CSI-RS resource from candidate cell for both L1 measurement and CSI acquisition purposes.

Proposal 3: UE continues measuring the activated SP CSI-RS resource for CSI acquisition from target cell after L1 CSI information has been transmitted to target cell.

Proposal 4: RAN2 to discuss which option can be used to handle the activated SP CSI-RS resource for CSI acquisition from other candidate cells:

Option 1: UE changes the state for activated semi-persistent CSI-RS resources to deactivated and stops measurement after cell switch is triggered.

Option 2: UE continues measuring the activated semi-persistent CSI-RS resources after cell switch is triggered.

MR MAC CE

Proposal 5: UE can support maximum 16 beams from the candidate cells and one current beam in one MR MAC CE.

Proposal 6a: In a truncated MR MAC CE UE maximizes inclusion of Type1/ 2 beams (i.e., any Type3/ 4 will only be included if all Type1/ 2 have been included).

Proposal 6b-alt1: In a truncated MR MAC CE current beam is never included.

Proposal 6b-alt2: In a truncated MR MAC CE current beam is included if all Type1/ 2 beams are included.

Proposal 6b-alt3: In a truncated MR MAC CE current beam is included if at least one Type1/ 2 beam is included.

Proposal 7a: Beam with highest RSRP among the event triggering beam(s) (Type1/ 2) is included first in the report.

Proposal 7b: A 1-bit indication to differentiate Absolute or Differential RSRP (AD bit) is included.

Tail Reporting

Proposal 8: No Tail reporting is done. The UE prepares a complete MR MAC CE if any of the ‘remaining’ Type1 or Type2 beams could not be included in the previous truncated MR MAC CE, if the remaining beams still fulfills the event triggering condition.

Useful optimizations

Proposal 9: When UL grant is not sufficient to include all Full MRs (each with many Type3/ 4 beams) and therefore can only include 'm' Full MRs, UE shall rather maximize reporting of triggered beams (Type1/ 2) by including 'n' truncated MRs in the TB (n < m). Exact implementation is left to the UE implementation.

Proposal 10: UE includes a beam(s) once and only in the latest triggering measurement report in a MAC PDU if there are potentially multiple MRs with the same beam. Same applies to the current beam of the serving cell i.e., the current serving beam is included only once in the MAC PDU.

Proposal 11: RAN2 considers means to control amount of MR MAC CE reporting.

# References

[1] RAN2#129bis chair notes for Rel-19 mobility

[2] R1-2503077 Reply LS on number of beam measurements in the measurement report MAC CE