3GPP TSG-RAN WG2 Meeting #130	R2-2503486

St.Julians, Malta, May. 19th – 23rd, 2025

Agenda Item:	8.6.2

Source: 	OPPO

Title: Discussion on open issues for inter-CU LTM

Document for:	Discussion and Decision

Introduction

In this contribution, we would like to present our considerations on the open issues related to inter-CU LTM.

Discussion

RRC open issues

RRC-6

In current RRC CR, the UE behaviour upon T304 expiry is specified as follows.

As in the procedural text, if attemptLTM-Switch is configured and there is no security update during the failed LTM procedure, the UE will revert back to the UE configuration used in the source PCell except for PDCP state variables for SRB(s) associated with the MCG. This is what we have specified for R18 intra-CU LTM to avoid keystream reuse issue. While in case that there is security key update during the failed LTM and attemptLTM-Switch is configured, the UE will do nothing according to the text.

However, if the UE does not revert back to the source cell configuration in this case, it may not be able to perform fast failure recovery to an LTM candidate cell or transmit RRCReestablishmentRequest to the NW since the source cell configuration is required in either procedure. To be specific, if an LTM candidate cell is selected for fast failure recovery, the security context of the source cell should be reused or based on which the UE derives new security key. If transmission of RRCReestablishmentRequest is initiated when an LTM candidate cell is not selected, the UE needs to include the C-RNTI used in the source cell in the RRC message. Therefore, the UE should revert back to the UE configuration used in the source cell upon inter-CU LTM failure. Since there is security key update during the failed inter-CU LTM procedure, PDCP continuation is not required.

Upon T304 expiry, if attemptLTM-Switch is configured and the LTM procedure requires a security key refresh, the UE revert back to the UE configuration used in the source PCell.

RRC-7

In current RRC CR, in case of inter-CU MCG LTM with SCG configuration, the UE performs secondary key update by selecting the first sk-counter in sk-counter list.

While according to the agreement we have made, the sk-counter used for secondary key update in this case is configured in each LTM candidate cell configuration.

- NW configures the corresponding sk-Counter in all LTM candidate configurations, and UE uses the configured value for generating the SN key when security key update is performed in MCG.

In case of consecutive inter-CU MCG LTM (i.e., each LTM execution is inter-CU MCG LTM), one sk-counter is enough to derive a unique SKgNB due to the change of KgNB, i.e., there is no need to configure a list of sk-counter for UE selection.

A list of sk-counter is not needed in case of inter-CU MCG LTM with SCG configuration.

While according to previous agreements, a mixture of inter-CU LTM and intra-CU LTM can be configured to a UE and MN security key update is not required for each LTM attempt. With KgNB unchanged, the same SKgNB can be derived when the UE accesses to the same LTM candidate cell after several times of intra-CU LTM as the case shown in the figure below.

To be specific, in the first intra-CU LTM attempt, the UE moves from cell 1 to cell2 and derives the SKgNB#1 based on KgNB#1 and the sk-counter configured in cell2 RRC configuration. Then the UE leaves cell2 and accesses to cell3 in the second intra-CU LTM attempt. In the third LTM attempt, the UE switches back to cell2 and the same SKgNB#1 as in the first LTM attempt will be generated due to the use of the same KgNB #1 and sk-counter.

As we have discussed, the NW may configure the same sk-counter value (e.g., sk-counter =0) for all the LTM candidate cells, thus the same SKgNB can be generated when the UE moves to any intra-CU MCG LTM candidate cell with SCG configuration.

Furthermore, we have agreed that the exiting field mrdc-ReleaseAndAdd is used for SCG handling during MCG LTM execution, which means that the SKgNB generation is required at each MCG LTM execution if DC is configured in the target.

- The existing field mrdc-ReleaseAndAdd is always included when SCG configuration is included during an MCG LTM execution. Can be revisited if there is any issue.

Based on above discussion, it can be observed that during subsequent intra-CU MCG LTM with SCG configuration, there is COUNT reuse issue since the same SKgNB is used but the PDCP state variables for radio bearers (e,g,SRB3) are initialized during the LTM execution. To solve the problem, the NW can re-configure the sk-counter for subsequent intra-CU MCG LTM after the execution of MCG LTM with SCG configuration.

To avoid SkgNB reuse issue, the NW reconfigures the sk-counter for subsequent intra-CU MCG LTM after the execution of MCG LTM with SCG configuration.

RRC-8

In last RAN2 meeting, the following agreements were made for LTM-based fast failure recovery.

At RLF and reconfiguration with sync failure: - If RLF: If the selected candidate cell has the same Rel-19 ID as source (no security key change), the UE performs fast RLF recovery (same as in Rel-18). - if reconfiguration failure (inter-CU LTM): If the selected candidate cell has the same Rel-19 ID as target, the UE performs fast failure recovery. FFS if fast failure recovery with different Rel-19 IDs is allowed.

As agreed, candidate cells having the same Rel-19 ID as the target cell can be selected for fast recovery after inter-CU LTM failure. For candidate cells with Rel-19 ID different from the target cell, further discussion is needed on their feasibility of performing fast failure recovery.

When the Rel-19 ID of the selected candidate cell is different from the target cell, it can be same or different with respect to the source cell and the two cases should be discussed separately as follows.

Case1: The Rel-19 ID of the selected candidate cell is the same as the source. As discussed above, upon LTM failure, the UE will revert back to the source cell configuration including all the state variable and parameters used in the source. With this operation, we understand it is possible for the UE to recover to an intra-CU LTM candidate cell w.r.t the source cell and the security context of the source cell is reused.

Case2: The Rel-19 ID of the selected candidate cell is different from the source. For case2, whether fast failure recovery is allowed depends on how the source gNB distributes the security key among the candidate cells. As per RAN3 agreements, the KgNB*(s) for initial and subsequent LTM are distributed as follows from NW wise.

- For the initial inter-CU LTM preparation, the source gNB computes the KgNB*(s) (per candidate cell) for the candidate gNB, and forwards the {KgNB*, NCC} pair(s) to the candidate gNB via a Handover Request message as legacy. - The new source gNB needs to provide the new KgNB*(s) to the corresponding candidate gNB via a LTM configuration update procedure. FFS on the procedure design either per cell or per gNB.

In our understanding, the KgNB*(s) for all candidate cells are generated in the same way, i.e., either horizontal or vertical depends on whether the source gNB is available of an unused {NH, NCC} pair, thus the NCC value included in LTM cell switch command MAC CE can be applied for security key generation for other inter-CU LTM candidate cells in addition to the target cell. Therefore, fast failure recovery to LTM candidate cell having different Rel-19 ID from source and target should be allowed.

In case of inter-CU LTM failure, LTM candidate cell with different Rel-19 ID from the target cell can be selected for fast failure recovery. The Rel-19 ID of the selected candidate cell can be same or different with the source cell.

If the selected candidate cell has the same Rel-19 ID as source cell, the security key of the source cell is reused.

If the selected candidate cell has different Rel-19 ID from source cell and target cell, the NCC value received in LTM cell switch command MAC CE is used for security key update.

MAC open issues

MAC-19

In last RAN2 meeting, it was agreed to introduce a new R19 LTM cell switch MAC CE which is able to indicate NCC value.

- Introducing a new R19 LTM cell switch command MAC CE with indicating the NCC value for R19 inter-CU LTM cell switch. Details will be further discussed during CR phase.

During MAC CR discussion, it is unclear on when to use the new R19 LTM cell switch command MAC CE and how to include the NCC value.

Editor’s NOTE: FFS whether NCC is optional present in Enhanced LTM Cell Switch Command MAC CE. FFS for which cases should this Enhanced LTM Cell Switch Command MAC CE is used, e.g. whether only for inter-CU case with security key update.

We understand the new LTM cell switch command MAC CE is used to cover new LTM features, e.g., indicating NCC value. The NCC value field can be set as optional in which way the Enhanced LTM cell switch command CE can be extended to support other features by not affecting the legacy one. More R bits can be reserved for future use.

NCC value field is optional present in Enhanced LTM Cell Switch Command MAC CE.

The Enhanced LTM Cell Switch Command MAC CE is used to trigger R19 LTM, either with or without security key update.

In current MAC CR, the Enhanced LTM Cell Switch Command MAC CE is designed as follows.

According to filed description, the presence of Oct5-Oct7 is determined by the value of C fields and the presence of NCC value filed. For the presence of NCC value field, it depends on the check of R19 set ID according to the replies from MAC Rapp. The logic will make the procedure complex since the ID check is performed in RRC while the MAC CE decoding is performed in MAC. To make the new MAC CE clean and simple, it is suggested to have a separate Oct for NCC value.

In Enhanced LTM Cell Switch Command MAC CE, a separate Oct is introduced to include NCC value.

Conclusion

Based on the discussion in section2, we have following observation proposals:

Proposal 1	Upon T304 expiry, if attemptLTM-Switch is configured and the LTM procedure requires a security key refresh, the UE revert back to the UE configuration used in the source PCell.

Proposal 2	A list of sk-counter is not needed in case of inter-CU MCG LTM with SCG configuration.

Proposal 3	To avoid SkgNB reuse issue, the NW reconfigures the sk-counter for subsequent intra-CU MCG LTM after the execution of MCG LTM with SCG configuration.

Proposal 4	In case of inter-CU LTM failure, LTM candidate cell with different Rel-19 ID from the target cell can be selected for fast failure recovery. The Rel-19 ID of the selected candidate cell can be same or different with the source cell.

Proposal 5	If the selected candidate cell has the same Rel-19 ID as source cell, the security key of the source cell is reused.

Proposal 6	If the selected candidate cell has different Rel-19 ID from source cell and target cell, the NCC value received in LTM cell switch command MAC CE is used for security key update.

Proposal 7	NCC value field is optional present in Enhanced LTM Cell Switch Command MAC CE.

Proposal 8	The Enhanced LTM Cell Switch Command MAC CE is used to trigger R19 LTM, either with or without security key update.

Proposal 9	In Enhanced LTM Cell Switch Command MAC CE, a separate Oct is introduced to include NCC value.