3GPP TSG RAN WG2 #129-bis	R2-2503496

St. Julian’s, Malta, May 19th - 23rd, 2025

Source:	Panasonic

Title:	Store & Forward: Remaining open issues

Agenda Item:	8.9.2  Support of Store & Forward

Document for:	Discussion, decision

Introduction

Store & Forward operation of satellites is seen as a key enhancement of IoT-NTN in order to facilitate cost-effective deployment of IoT-NTN to allow for delay-tolerant, non-real-time IoT-NTN services [1].

- Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore: - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2, RAN4] - At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3] Note: Strive to minimise UE impact. Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required.

Furthermore, the NTN sessions of the last RAN2 meetings #129bis in Wuhan and #129 in Athens resulted in the following S&F-related results (see [3] and [4]):

Wuhan agreements (meeting #129-bis):

- For UEs supporting S&F, the UE AS indicates the information on transition time (if any) from current "S&F operation mode" to "normal mode" (if received) to the upper layers. Whether/how it is used by the upper layers is up to CT1.

- We introduce an indication in system information for the normal mode to S&F mode transition, at least for NAS use. FFS on the details (e.g. whether we can link this to other existing information). The information on transition time for the normal mode to S&F mode transition is sent from AS to NAS.

- We don’t introduce any new release cause related to S&F operation.

Athens agreements (meeting #129): - RAN2 currently assumes that a S&F mode changes does not necessarily imply an MME change. - The already agreed time information on transition from S&F operation mode to normal mode is provided in SIB31 (can work on signalling optimizations in case the new time information and T-service would carry the same information). - Use absolute UTC time to broadcast the (S&F-> normal mode) time information. When this information is broadcast and indicates a time in the future the UE can assume the NW is currently operating in S&F mode (until the indicated S&F-> normal mode transition time). - A change to the (S&F-> normal mode) time indication shall not result in system information change notification. Legacy rules for acquiring SIB31 apply.

The aim of this document is to propose solutions for the remaining open S&F issues.

Discussion on remaining open S&F issues

# 2.1 Indication of time instance of change from normal mode to S&F mode

  - That issue refers back to agreement 2 achieved during last meeting #129-bis in Wuhan where details of how to indicate the point of time the satellite changes from normal to S&F mode were left FFS (see clause 1 above).

  - In summary, we have agreed signalling changes in both directions – the one from S&F mode to normal mode (agreed to use absolute UTC time) and the one from normal mode back to S&F mode (see clause 1 above, as well).

  - As far as definitions in specifications are concerned, this is the current form of the definition of the Store and Forward Satellite operation mode in the RRC CR [2], i.e. draft TS 36.331 V19.x.y:

  - An operation mode that provides to the UE a communication service when the serving satellite has a discontinuous connection to the NTN gateway and connection to the NTN gateway is not available when the satellite is interacting with the UE.

  - Observation 1:	Is normal Satellite operation mode now implicitly defined as everything not being equivalent to Store and Forward Satellite operation mode? Even in that case a few explanatory words might be helpful in the RRC spec.
In the end, “normal mode” might better be replaced by “real-time mode” – to prevent any potential ambiguities.

  - Proposal 1:	Add explanation/definition for real-time Satellite operation mode to RRC CR.

  - The way the time instance of the change from normal/real-time to S&F mode is to be defined. The use of legacy field t-Service is not really appropriate, we believe.

  - t-Service is defined as follows in the RRC CR [2]:

  - Time information on when an NTN cell is going to stop serving the area it is currently covering, as specified in TS 36.304 [5]. This field applies for service link switches in NTN quasi-Earth fixed cells and feeder link switches for both NTN quasi-Earth fixed and earth-moving cells.

  - Observation 2:	Using legacy t-Service for the purpose in question – the indication of the time instance a satellite changes from normal/real-time mode operation to S&F mode operation – is inappropriate, because feeder link switches can still happen during normal/real-time mode operation. Using legacy t-Service also for the new purpose discussed here would hence introduce an unwanted ambiguity. It would also affect legacy devices in an unwanted manner.

  - Proposal 2:	RAN2 to introduce a new indication for the point of time a satellite changes from normal/real-time mode to S&F mode.

  - The change from S&F mode to normal/real-time mode is indicated with field t-ModeSwitching that is defined in the RRC CR [2] as a SIB31 field as follows:

  - Time information on when an NTN cell is going to switch from the Store and Forward Satellite operation mode to the normal mode. This field is excluded when determining changes in system information, i.e. changes to t-ModeSwitching do not result in system information change notification.

  - Our suggestion is to amend the aforementioned field t-ModeSwitching by an optional indication of the time period the related satellite will remain operating in normal/real-time mode.

  - Proposal 3:	Should proposal 2 above find consensus, amend the field t-ModeSwitching by an optional indication of the time period the related satellite is going to operate in real-time mode.

  - In the light of those mode change indications in both directions – they have principally been agreed already – there is a redundancy occurring in conjunction with the sf-OperationIndication flag agreed earlier – see [2] – i.e. that status flag becomes superfluous.

  - Proposal 4:	Drop recently agreed sf-OperationIndication flag, because the indication of the points of time a satellite changes between S&F and real-time operation modes provides the S&F status explicitly.

  - 2.2 Additional assistance information for neighbouring cells,
      e.g. mode of operation and/or mode change time instance

## The question is, if it would be useful for the UE, in particular, to know a priori, if a neighbouring cell is currently operating in normal/real-time or in S&F mode. A further open issue is, if an a priori information about an upcoming mode change in neighbouring cells is useful for the UE in its serving cell.

## We generally assume that what is part of MME-configured frequency lists would be prioritized by UEs over what is not part of these lists.

## Observation 3:	So far, there is neither an a priori indication provided within the currently serving cell regarding the mode of operation of a neighbouring or remote satellite nor is there an information in advance regarding the point of time the current mode of operation of the named neighbouring/remote satellite will change.

## We believe that a priori information about both attributes is useful. If it wouldn’t be provided, cell reselections would be done blindly. Only after changing to a neighbouring cell (including the remote cells that were coming closer), the UE could recognize the related situation and deal with it properly. Not an issue, if there is no alternative to a single neighbouring cell, i.e. the UE has to change to that neighbour anyway. But in case of multiple alternatives obtaining both information elements in advance would clearly be of advantage.

## Prior to a final decision here, the specification impact needs to be considered carefully.

## Proposal 5:	If specification impact is rated reasonable, RAN2 to adopt an indication for mode of operation and time instance of mode change for neighbouring and remote cells in SIB33(-NB) and SIB32(-NB) respectively.

  - 2.3 Restricted paging monitoring of S&F UEs for the sake of
      reduced energy consumption

It is obvious that during a longer term absence of any satellite in the vicinity of the UE it doesn’t make sense to watch out for paging requests of the network in RRC_IDLE or RRC_INACTIVE mode. So the question is, how to indicate the time period until a subsequent satellite will get into UE’s reach.

From our point of view, it would be straight forward to indicate the positions of satellites in same and neighbouring or crossing orbits. On that basis, the UE is able itself to determine the arrival of the next satellite. And then switch off widely – apart from the internal clock ticking down.

Proposal 6:	RAN2 to assure that during the absence periods of relevant satellites UEs are precluded from monitoring paging in vain.

Conclusions

In this document, we discussed the remaining open issues in conjunction with Store and Forward satellite operation mode.

The following observations and proposals were made:

  - Observation 1:	Is normal Satellite operation mode now implicitly defined as everything not being equivalent to Store and Forward Satellite operation mode? Even in that case a few explanatory words might be helpful in the RRC spec.
In the end, “normal mode” might better be replaced by “real-time mode” – to prevent any potential ambiguities.

  - Observation 2:	Using legacy t-Service for the purpose in question – the indication of the time instance a satellite changes from normal/real-time mode operation to S&F mode operation – is inappropriate, because feeder link switches can still happen during normal/real-time mode operation. Using legacy t-Service also for the new purpose discussed here would hence introduce an unwanted ambiguity. It would also affect legacy devices in an unwanted manner.

## Observation 3:	So far, there is neither an a priori indication provided within the currently serving cell regarding the mode of operation of a neighbouring or remote satellite nor is there an information in advance regarding the point of time the current mode of operation of the named neighbouring/remote satellite will change.

  - Proposal 1:	Add explanation/definition for real-time Satellite operation mode to RRC CR.

  - Proposal 2:	RAN2 to introduce a new indication for the point of time a satellite changes from normal/real-time mode to S&F mode.

  - Proposal 3:	Should proposal 2 above find consensus, amend the field t-ModeSwitching by an optional indication of the time period the related satellite is going to operate in real-time mode.

  - Proposal 4:	Drop recently agreed sf-OperationIndication flag, because the indication of the points of time a satellite changes between S&F and real-time operation modes provides the S&F status explicitly.

## Proposal 5:	If specification impact is rated reasonable, RAN2 to adopt an indication for mode of operation and time instance of mode change for neighbouring and remote cells in SIB33(-NB) and SIB32(-NB) respectively.

Proposal 6:	RAN2 to assure that during the absence periods of relevant satellites UEs are precluded from monitoring paging in vain.

References

[1]	RP-240776 Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3

[2]	R2-250xxxx RRC Runing CR for IoT NTN_v03_MTK (2025-05)
                       3rd Generation Partnership Project; Technical
                    	Specification Group Radio Access Network;
                    	Evolved Universal Terrestrial Radio Access (E-
                    	UTRA); Radio Resource Control (RRC);
                    	Protocol specification (Release 18)

[3]	R2-129bis NR-NTN-IoT-NTN (Sergio)_ EOM

[4]	R2-129 NR-NTN-IoT-NTN (Sergio)_ EOM

[5]	3GPP TS 36.304 V18.3.0 (2024-12)		3rd Generation Partnership Project; Technical
                                                                                  Specification Group Radio Access Network;
                                                                                  Evolved Universal Terrestrial Radio Access (E-
                                                                                  UTRA); User Equipment (UE) procedures in
                                                                                  idle mode (Release 18)