3GPP TSG-RAN WG2 Meeting #130											R2-2503508

St. Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	8.7.3

Source: 	Sharp

Title:  	UAI and Measurement Gap Enhancements

Document for: 	Discussion

# Introduction

RAN4 sent an LS to RAN2 on UE’s preference for measurement gap cancellation to ask RAN2 to implement the following agreements [1]:

## Issue 5-1: General on UAI <Agreement>: - Agree to define UAI based on the following assumptions, and further discuss the content of UAI - RAN4 will not specify any explicit or implicit requirement or expectation on gNB behaviour in response to any XR UAI (from RAN4 perspective) - Not define UE behaviour and the corresponding requirements based on UAI - Supporting of UAI is optional for UE with optional UE capability.

- The information in UAI: a ratio (R) of gap occasions that is recommended for cancellation during a time period of 1 s. - R ∈ {0, 20, 40, 60}%

Based on the LS, the RRC running CR [2] captures the following open issues:

- Issue5: FFS when the UE should trigger UAI for assistance information for measurement occasion - Issue6: FFS what are the configurations for controlling UAI reporting assistance information for measurement occasion ratio

This contribution discusses the open issues above.

# Discussion

The first issue is which information is included in the UAI message. We see that RAN2 can confirm RAN4 conclusion, as there is no serious issue foreseen from RAN2 point of view.

RAN2 to confirm the contents of the UAI as RAN4 agreed: a ratio (R) of gap occasions that is recommended for cancellation during a time period of 1s. The value range is {0%, 20%, 40%, 60%}.

A number of UAI configurations have been introduced in TS 38.331 so far. Although there are some differences between different UAI configurations, a basic framework of UAI is almost the same except a few detailed exceptions. Unless there is any critical issue, it would be desirable to keep the basic principle and framework as much as possible.

Most of UAI message is triggered only if it is configured. It means that gNB can configure whether the UE can initiate the corresponding UAI procedure. This principle can be applied for the measurement gap enhancement, and we do not see a reason to allow an exception.

An RRC message configures whether the UE can provide UAI on ratio of gap cancellation.

Two main triggering conditions of most of UAI message are (1) when the corresponding information is available upon being configured and (2) upon change of the UE’s preference. In measurement gap cancellation, UE’s preferred ratio depends on coverage condition and reliability of signals due to change of signal propagation paths. This factor could change in time, so change of the preference should trigger an update of the preference to the network. Hence, we can keep the principle.

A UE capable of providing its preference on ratio of gap cancellation shall initiate the UAI procedure when this information is available upon being configured to do so, and upon change of its preference.

Some UAIs have T346x prohibit timer not to allow frequent ping-pong updates. In measurement gap enhancement, the same issue may exist. Hence, it is reasonable to have a prohibit timer.

A prohibit timer (e.g. T346x) for UAI on measurement gap cancellation is introduced.

# Conclusions

Based on the discussion given above, RAN2 is requested to agree the following proposals:

- RAN2 to confirm the contents of the UAI as RAN4 agreed: a ratio (R) of gap occasions that is recommended for cancellation during a time period of 1s. The value range is {0%, 20%, 40%, 60%}.

- An RRC message configures whether the UE can provide UAI on ratio of gap cancellation.

- A UE capable of providing its preference on ratio of gap cancellation shall initiate the UAI procedure when this information is available upon being configured to do so, and upon change of its preference.

- A prohibit timer (e.g. T346x) for UAI on measurement gap cancellation is introduced.

# Reference

[1] R2-2503328	Reply LS on UE assistance information (R4-2504972; contact: Nokia)	SA4

[2] R2-2503787	Running RRC CR for R19 XR	 Huawei, HiSilicon