3GPP TSG RAN WG2 Meeting #130                                                            R2-2503518

# St Julian's, Malta, May 19th - 23rd, 2025

Agenda item:	8.2.2

Source:	Ofinno

Title:	Discussion on issues 1-1 and 1-3

Document for:	Discussion and decision

# Introduction

This document discusses the following issues identified during email discussion [POST129bis][017][AIoT] [1]: [issue 1-1] on Multi-Reader (addressing clarification of RAN2 agreement on parallel service, decision making upon detecting parallel services, potential controversial points to determine the end of A-IoT procedure and A-IoT service differentiation), and [issue 1-3] on the Paging ID, length, and content of the paging message. For reference, Rapp’s details captured on the issues here discussed are copied in Annex.

# Discussion

## [Issue 1-1] Multi-Reader scenario

For Issue 1-1 - how the device handles a new service request received while another one procedure is still ongoing,  companies were encouraged to work together in the last RAN2#129bis meeting and bring a joint document elaborating more concrete details for the mechanism based on option A) ignore and/or option B) terminate. We contributed and co-sourced the joint document describing the details how this option A) solution can work [2]. This section addresses potentially controversial topics and suggests other alternatives that might need to be considered.

### Clarification on RAN2 agreement for Parallel services

The following chair-meeting minutes and agreements were taken in RAN2#129 meeting:

Simultaneous Paging Procedures R2-2500493 Paging Aspects for Ambient IOT InterDigital discussion Rel-19 Ambient_IoT_Solutions Proposal 11: Parallel service requests by the same/different reader is not supported. Noted R2-2501069 Discussion on DL messages for Ambient IoT UEs Ericsson discussion Rel-19 FS_Ambient_IoT_solutions Proposal 8 Device ignores any other paging message received if it is participating in an ongoing paging round. Noted Discussions - Ericsson indicates that there may be coverage overlap so multiple reader can page the UE and there would be no coordination between the readers. For that case we can keep the design simple. - Qualcomm wonders if we are assuming no coordination between readers for multi reader scenario. Futurewei thinks that the real issue is that we don’t want the interleave the procedures. Interdigital indicates that even if there is no coordination the core network can ensure this doesn’t happen. Ericsson doesn’t think the CN can handle this, and there may be overlapping coverage and you don’t know. Vodafone thinks that readers are under gNB control so it is coordinated paging. - Xiaomi thinks that from device perspective the UE would stop current paging and start new one. Interdigital explains that this scenario shouldn’t happen. - Apple, Oppo and LG thinks that Ericsson’s proposal is simpler from device perspective. - CATT thinks that these proposals can co-exist, one is from UE and from device. - ZTE understands that the UE will not monitor paging and AS ID. Xiaomi and qualcomm think it can happen. - CMCC is concerned if the device ignores as there may be a re-access attempt. Parallel service requests by the same reader is not supported. - The device is expected to only perform one procedure at a time. FFS device behaviour if multiple requests are received in parallel (if needed).

From the discussion among companies, we wonder whether RAN2 should clarify/confirm the intention of the first agreement on whether it means from the device point of view (PoV) or from the reader PoV.

- From the reader PoV, the reader can only start one procedure with one device at any given time, i.e., if the reader has started a procedure with device A (and is ongoing), it cannot start a new procedure with other devices (e.g., including device A) until it finishes the current/ongoing procedure with device A.

- From the device PoV, the reader may have multiple procedures starting or ongoing with different devices at the time, i.e., a device can only have one ongoing procedure, but the reader may be allowed to have parallel procedures ongoing with different devices at any given time.

From the proponent document in RAN2#129 and the corresponding meeting notes (shown both above), it seems that device PoV was the original intention, however views expressed offline, companies might have assumed this agreement also applies from reader PoV although this will add certain limitation/restriction in Rel-19 network/operation. On the other hand, it may allow a simple operation to be defined, e.g., the usage of R2D trigger message to indicate the end of the A-IoT procedure [2]. This is a valid option if the reader PoV is confirmed although at the time it would require specific reader’s behaviour as they are explained by option A-1) and A-2).

[Issue 1-1] [Parallel Service] It is unclear whether RAN2 agreement “Parallel service requests by the same reader is not supported” is from the readers PoV or the device PoV:

  - Reader PoV: the reader can only start one procedure with a device at any given time (which prevents parallel interleaved operation of two or more procedures by the same reader and different devices).

  - Device PoV: the reader may start different procedures in parallel with difference devices while the device does not expect to receive parallel service requests by one reader.

[Issue 1-1] [Parallel Service] RAN2 agreement “The device is expected to only perform one procedure at a time” captures and is aligned to the interpretation from device PoV explained in Observation 1.

- [Issue 1-1] [Parallel Service] To clarify whether previous RAN2 agreement “Parallel service requests by the same reader is not supported” should be understood from device PoV (which means that the reader may start different procedures in parallel with difference devices while the device does not expect to receive parallel service requests by one reader) or from reader PoV (which means that the reader can only start one procedure with one device at any given time).

### Decission making upon detecting parallel services

As explained, RAN2 is discussing two prioritization options when a device receives a new service request while another is ongoing: A) ignore the new service request or B) terminate the ongoing procedure. PROS/CONS of both options were discussed during RAN2#129bis meeting and are also explained with large detail in the joint document [2]. Our preference is to enable option A) considering the concerns found for option B) which are well explained in the joint document [2] highlighting for example:

- Race conditions where a procedure initiated by one reader is terminated (repeatedly) by another reader(s), and the reader keeps re-initiating the terminated procedure to complete.

- Collision issues for Msg3 and NACK indication which would get worse towards the end of the paging round.

- Prolong the overall service latency, e.g. for the AIoT command service, a reader cannot receive from an AIoTF a command request and send to a device the command message until the reader completes the inventory procedure and sends the inventory report to the AIoTF.

- Interruption to command service is likely due to the extended duration between the inventory report and command with the 2-step command procedure being defined by SA2 in TS 23.369.

- Not forward compatible considering the multi-reader scenarios which would be even more problematic with the typical use cases in topology 2 as well as outdoor deployments.

- System performance might be worsened due to an increase in the energy consumption, waste of the radio resource and increase of the implementation complexity.

Considering those concerns, our preference is that option B) is not considered as the normal/default behaviour. However, there might be some cases or scenarios in which this option B) might be considered “good enough,” e.g., for very controlled or initial/early deployment of A-IoT. In addition, there could also be scenarios in which a network or an operator may need the device to handle with higher priority or urgency the new request regardless of the limitation or implications this might bring.

[Issue 1-1] [Parallel Service] When handling parallel services, option A) of ignoring the new service request (and continuing with the ongoing one) is preferable as default/normal considering all the concerns identified for option B). However, there might be specific scenarios where option B) of starting the new service (and terminating ongoing one) might be sufficient or required, e.g., for very controlled or initial/early deployment of A-IoT or for services that network or if/when operators may require to prioritize in case-by-case basis.

- [Issue 1-1] [Parallel Service] When a device has an ongoing A-IoT procedure and receives a new service request, default/normal operation is to ignore the new service request (option A) but when required, network can indicate to the device to prioritize this new service request instead (option B).

### End of A-IoT procedure

The joint document [2] suggests using the R2D trigger message as an end indicator for the ongoing A-IoT procedure of one device. This seems a preferable approach by majority of the supporting companies of this option A). However, we understand that there might be concerns that the immediately “sub-sequent” R2D trigger message might be restrictive from the network PoV. If so, other messages or mechanisms could be considered.

[Issue 1-1] [End of proc.] If the usage of immediately “sub-sequent” R2D trigger message by the device to determine the successful completion of an ongoing A-IoT procedure is restrictive from network point of view, other R2D messages and/or mechanisms could be considered.

#### Msg2-based end indication of A-IoT procedure

The companion document [3] explains the retransmitted Msg2 could be used to successful or failure indication of previously transmitted Msg3. Similarly, the (re-)transmitted Msg2 could be re-used to also indicate a successful/completion indication as a group ACK for the previously transmitted D2R message in the last round (e.g., for the inventory/command procedure. It is to note that the R2D trigger message targets any device (and would be decoded by all devices) vs potentially the retransmitted Msg2 might only target and be decoded by one or a limited group of devices, for example those with an ongoing A-IoT procedure (i.e., waiting for next R2D message and/or success/failure indication of the previously transmitted D2R message). This indication could potentially be defined per device or per group, as well as, as an explicit or implicit indication, as it is explained in [3].

- [Issue 1-1] [End of proc.] If there are concerns with using the “subsequent” R2D trigger message for the device to determine the successful completion of an ongoing A-IoT procedure, to discuss whether to use the retransmitted Msg2 for the that purpose.

#### Count-based end indication of A-IoT procedure

In some cases, devices may perform sub-sequent re-access or re-transmissions and even so, a device may still not be able to successfully complete the A-IoT procedure (e.g., when in poor coverage of the reader). This might end up being not optimum from power consumption PoV to the device. A counter-based mechanism may be used at the device (and/or the reader) side to determine end of A-IoT procedure after a certain number of (re-)tries.

- [Issue 1-1] [End of proc.] To discuss whether the device can determine the termination of the A-IoT procedure due to a “consistent failure” of the D2R (re)transmissions. If so, to discuss the meaning of “consistent failure” (e.g., by a maximum value/counter of D2R (re)transmissions, re-access/tries).

### Service differentiation (Transaction ID and R2D messages)

The following are agreements related to the transaction ID:

[RAN2#128 agreements] - An ID in paging message is beneficial to avoid duplicated response for same service request. FFS whether ID is generated by the reader or CN. FFS on the ID size. [RAN2#129 agreements] - The “transaction ID” can be generated by reader based on CN corelation ID. FFS how reader will generate “transaction ID”. FFS the size of transaction ID - RAN2 acknowledges that multi-reader scenario may exist but we will not specify something specific for this purpose. We can rely on transaction ID and implementation to handle it. - Re-use the subsequent paging message to trigger re-access. There is no need to differentiate msg1 resource for initial access vs re-access. - The service type of A-IoT (e.g., inventory only, inventory + command) is not included in paging message. [RAN2#129bis agreements] - The device releases the AS ID at least: - upon receiving Paging with new transaction id for that device, i.e. different session/service - when it triggers new msg1 transmission as a result of receiving Paging message (i.e. it has to generate a random ID for CBRA) FFS other cases for release ASID to avoid keeping it indefinitely.

The reader can perform inventory to one or multiple devices that are associated with the same A-IoT service (e.g., A-IoT inventory service or A-IoT command service) by sending one or more paging message(s). In our understanding, it is up to the reader’s implementation to decide how many paging messages the reader will send to perform inventory of all the devices associated with one specific A-IoT service. Reader’s decision may vary depending on multiple factors, for example, depending on the availability of the radio resource used for paging and random access, as well as a success or failure rate of inventorying the devices, etc.

The paging message could be followed by one or more subsequent R2D messages (e.g., subsequent paging messages) associated with the same A-IoT service/transaction. It may be unclear how this association (i.e., messages that belong to the same A-IoT service/transaction) could be distinguished. For this, the transaction ID in the paging message could be used to identify each specific A-IoT procedure. Moreover, this transaction ID could also be helpful for scenarios in which different readers send paging messages to the same UE (or group of UEs) associated with different A-IoT services. Summarizing, the paging message and one or more subsequent R2D messages (e.g., subsequent paging messages) that are associated with the same A-IoT service should include the same transaction ID.

In addition, in the context of handling/differentiating services, the joint document [2] also explains how the transaction ID is used to differentiate the specific ongoing procedure with a device. Moreover, it also proposes that the R2D trigger message includes the transaction ID either explicitly (e.g., via A-IoT MAC payload or L1 signaling) or implicitly (e.g., masking the CRC with the transaction ID).

[Issue 1-1] [Transaction ID] It is helpful if a device can recognize R2D messages (including first and subsequent paging messages) associated to the same A-IoT service or procedure considering e.g., readers may send multiple paging messages for the same A-IoT procedure (up to implementation), one A-IoT procedure may require multiple R2D messages, or the device receives a new A-IoT service request while another A-IoT procedure is ongoing in the device.

- [Issue 1-1] [Transaction ID] A device needs to differentiate any R2D message(s) associated with the same A-IoT service (or procedure). This is done by including the same transaction ID in the corresponding R2D message(s), which would also apply to paging message and subsequent paging messages.

## [Issue 1-3] Paging ID, length and content of paging message

Issue 1-3 focus on the content of paging message with focus on the field to indicate the paging ID length (e.g. value range, how many bits, format design). On this topic, in addition to the agreements shown above, the following ones were also agreed by RAN2 on this topic:

[RAN2#129 agreements] - The “one identifier” in the paging message includes both the case of “one single device identifier” and “one group identifier”/”filtering criteria”, while the exact format of latter is supposed to be designed by SA2. - The current assumption is that the paging identifier is transparent to the A-IoT MAC Layer and carried by upper layer. FFS if there is really a need for visibility in the MAC layer [RAN2#129bis agreements] - Introduce an explicit 1 bit indication to indicate whether it is CFRA or CBRA per paging message. A field indicating Paging ID length information is always included together with the paging ID field in the A-IoT paging message, except the case where no ID is included in the A-IoT paging message. The number of bits required for paging ID length field should be as small as possible. This would require the number of different Paging ID lengths to be small. Send an LS to SA2 to take this into account for their design.

Correspondingly the following TP is captured in the running CR to A-IoT MAC TS 38.391 [4]:

6.2.1.1 A-IoT Paging message The fields in this message are defined as follows: - R2D Message Type: This field indicates the message type. See the Table 6.1-1. - RA Type: This field indicates CBRA or CFRA. - Indication of Paging ID presence: This field indicates whether Paging ID and Length of Paging ID are present (when set to 1) or absent (when set to 0). - Length of Paging ID: This field indicates the Paging ID length information when Paging ID field is present. - Paging ID: xxx - Transaction ID: xxx - Number of Access Occasions: This field indicates the number of access occasions. - D2R Scheduling Info: This field indicates the physical layer parameters used for D2R scheduling. Editor’s Note: FFS if CFRA can omit the fields of transaction ID, Indication of Paging ID present/absence, Number of access occasions. Editor’s Note: FFS the length of transaction ID.

Concrete details of the paging ID length information will need to wait for SA2/CT4 input as requested in [5]. However, RAN2 can still work on the overall design of the A-IoT paging message.

RAN3 agreed the following information for the inventory request transfer [6] (which would trigger paging message):

9.2.x.1 INVENTORY REQUEST This message is sent by the A-IoT CN node to trigger the NG-RAN node to perform inventory over the A-IoT radio. Direction: A-IoT CN node NG-RAN node IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Message Type; M; 9.3.1.1; YES; reject. AIOTF Identifier; M; 9.3.3.xx2; YES; reject. AIoT Correlation Identifier; M; 9.3.3.xx1; YES; reject. AIOTF Identifier; M; 9.3.3.xx2; YES; reject. AIoT Correlation Identifier; M; 9.3.3.xx1; YES; reject. Inventory Response Transfer; M; OCTET STRING; Containing the Inventory Response Transfer IE specified in subclause 9.3.x.2; YES; reject. Criticality Diagnostics; O; 9.3.1.3; YES; ignore. 9.3.x.1 Inventory Request Transfer This IE provides the inventory request related information from the AIOTF to the NG-RAN node. In indirect communication, this IE is transparent to the AMF. IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. A-IoT Correlation Identifier; M; 9.3.3.xx1; YES; reject. A-IoT Device Identification Requested; M; 9.3.3.xx5; YES; reject. Requested Service Area Information; M; 9.3.1.xx3; YES; reject. Inventory Assistance Information; O; 9.3.1.xx4; YES; reject. Follow on Command Indication; O; ENUMERATED (true, ...); This IE indicates that there is command(s) to be transmitted.; YES; reject. 9.3.3.xx5 A-IoT Device Identification Requested This IE represents the Requested Identity of the A-IoT device(s). IE/Group Name; Presence; Range; IE type and reference; Semantics description. FFS. Editor’s Note: this IE enables identification of a single A-IoT device, a group of A-IoT devices, all A-IoT devices, detailed coding FFS.

The A-IoT paging message design should align, if no concerns are found, with upper layers design e.g., RAN3 is enabling an inventory request that triggers paging to a single A-IoT device, a group of A-IoT devices, all A-IoT devices. Even if in Rel-19, the A-IoT paging message only targets one paging ID, the design should allow or be at least forward compatible to include multiple paging IDs. Moreover, depending on SA2/CT1 input to [5], RAN2 might need to discuss account whether a single A-IoT paging message can include paging IDs with different lengths or not (in which case different A-IoT paging messages would be required).

[Issue 1-3] [Paging] The A-IoT paging message needs to align with SA2/RAN3 design which enables triggering inventory request to a single, group, or all A-IoT devices within the requested service area, providing a message that allows this in Rel-19 (at least from specification PoV) and/or is forward compatible with Rel-20 and beyond.

- [Issue 1-3] [Paging] The design of A-IoT paging message should allow triggering paging to one or more devices, as well as groups of devices (i.e., paging IDs). FFS on the paging ID length (dependent to SA2/CT4 response).

# Conclusion

The observations captured are the following:

Observation 1.	[Issue 1-1] [Parallel Service] It is unclear whether RAN2 agreement “Parallel service requests by the same reader is not supported” is from the readers PoV or the device PoV:

(a)	Reader PoV: the reader can only start one procedure with a device at any given time (which prevents parallel interleaved operation of two or more procedures by the same reader and different devices).

(b)	Device PoV: the reader may start different procedures in parallel with difference devices while the device does not expect to receive parallel service requests by one reader.

Observation 2.	[Issue 1-1] [Parallel Service] RAN2 agreement “The device is expected to only perform one procedure at a time” captures and is aligned to the interpretation from device PoV explained in Observation 1.

Observation 3.	[Issue 1-1] [Parallel Service] When handling parallel services, option A) of ignoring the new service request (and continuing with the ongoing one) is preferable as default/normal considering all the concerns identified for option B). However, there might be specific scenarios where option B) of starting the new service (and terminating ongoing one) might be sufficient or required, e.g., for very controlled or initial/early deployment of A-IoT or for services that network or if/when operators may require to prioritize in case-by-case basis.

Observation 4.	[Issue 1-1] [End of proc.] If the usage of immediately “sub-sequent” R2D trigger message by the device to determine the successful completion of an ongoing A-IoT procedure is restrictive from network point of view, other R2D messages and/or mechanisms could be considered.

Observation 5.	[Issue 1-1] [Transaction ID] It is helpful if a device can recognize R2D messages (including first and subsequent paging messages) associated to the same A-IoT service or procedure considering e.g., readers may send multiple paging messages for the same A-IoT procedure (up to implementation), one A-IoT procedure may require multiple R2D messages, or the device receives a new A-IoT service request while another A-IoT procedure is ongoing in the device.

Observation 6.	[Issue 1-3] [Paging] The A-IoT paging message needs to align with SA2/RAN3 design which enables triggering inventory request to a single, group, or all A-IoT devices within the requested service area, providing a message that allows this in Rel-19 (at least from specification PoV) and/or is forward compatible with Rel-20 and beyond.

The proposals captured are the following:

Proposal 1.	[Issue 1-1] [Parallel Service] To clarify whether previous RAN2 agreement “Parallel service requests by the same reader is not supported” should be understood from device PoV (which means that the reader may start different procedures in parallel with difference devices while the device does not expect to receive parallel service requests by one reader) or from reader PoV (which means that the reader can only start one procedure with one device at any given time).

Proposal 2.	[Issue 1-1] [Parallel Service] When a device has an ongoing A-IoT procedure and receives a new service request, default/normal operation is to ignore the new service request (option A) but when required, network can indicate to the device to prioritize this new service request instead (option B).

Proposal 3.	[Issue 1-1] [End of proc.] If there are concerns with using the “subsequent” R2D trigger message for the device to determine the successful completion of an ongoing A-IoT procedure, to discuss whether to use the retransmitted Msg2 for the that purpose.

Proposal 4.	[Issue 1-1] [End of proc.] To discuss whether the device can determine the termination of the A-IoT procedure due to a “consistent failure” of the D2R (re)transmissions. If so, to discuss the meaning of “consistent failure” (e.g., by a maximum value/counter of D2R (re)transmissions, re-access/tries).

Proposal 5.	[Issue 1-1] [Transaction ID] A device needs to differentiate any R2D message(s) associated with the same A-IoT service (or procedure). This is done by including the same transaction ID in the corresponding R2D message(s), which would also apply to paging message and subsequent paging messages.

Proposal 6.	[Issue 1-3] [Paging] The design of A-IoT paging message should allow triggering paging to one or more devices, as well as groups of devices (i.e., paging IDs). FFS on the paging ID length (dependent to SA2/CT4 response).

# Reference

- R2-2503998, Summary of A-IoT MAC open issues, Huawei, HiSilicon, May 2025.

- R2-2504112, Details of option A for overall A-IoT MAC procedure, ZTE Corporation, Sanechips, Continental Automotive, Tejas Networks, Nokia, Ofinno, Samsung, Fujitsu, Kyocera, Sony, ETRI, III, Ericsson, May 2025.

- R2-2503519, Discussion of issues 2-4, 2-6, 2-11, 1-4 and a new issue, Ofinno, May 2025..

- R2-2503997, A-IoT MAC running CR, Huawei, HiSilicon, May 2025.

- R2-2503197, LS on paging ID length, LS out, From RAN2, To SA2, CT4, Ccing RAN3, May 2025.

- R3-252480, (TP to TS 38.413 BL CR) Updates of the Inventory and command, Huawei, ZTE, Aprul 2025.

# Annex

Details captured by Rapporteur on the issues discussed on this document (as part of[POST129bis][017][AIoT] 38.391 Running CR [1]) are included also here for easier reference.

## Issue 1-1

| Subgroup: Multi-reader scenario |  |  |
| --- | --- | --- |
| Issue 1-1: multi-reader paging | If a device gets a new service request while one procedure is still ongoing, whether/how to specify device behaviour or leave it to implementation, and the end of procedure if needed. - Relevant agreements: - RAN2 acknowledges that multi-reader scenario may exist but we will not specify something specific for this purpose. We can rely on transaction ID and implementation to handle it. - FFS which solution if any for device behavior if it gets a new service request while one procedure is still ongoing or leave it to implementation. - For CBRA, as a baseline, NACK based mechanism is applied only to the Msg3. May come back for D2R data, if the NACK feedback indication is needed for the purpose to stop/terminate the “on-going procedure” and release the AS ID accordingly (depending on other later discussion). - FFS on end of procedure - Status in running CR: captured as Editor’s Note in 5.2. | To be discussed by company contributions |

## Issue 1-3

| Issue 1-3: Paging ID length field | The field to indicate the paging ID length, e.g. value range, how many bits, format design - Relevant agreements: - A field indicating Paging ID length information is always included together with the paging ID field in the A-IoT paging message, except the case where no ID is included in the A-IoT paging message. - The number of bits required for paging ID length field should be as small as possible. This would require the number of different Paging ID lengths to be small. - RAN2 sent LS to CT4 and SA2 in R2-2503197 asking for their feedback on the above agreement, for RAN2 to determine the field for paging ID length. - Note: SA2 already agreed the filtering information and captured it in clause 5.8 in 23.369, this may enable some extent of RAN2 discussion before their feedback. - Status in running CR: the field name is captured in 6.2.1.1 without the detailed format. | To be discussed by company contributions |
| --- | --- | --- |