3GPP TSG-RAN WG2 Meeting #130	R2-2503533

St.Julians, Malta, May 19th – 23rd , 2025

Source: 	Xiaomi

Title:  	Discussion on conditional LTM

Agenda Item:	8.6.4

Document for:	Discussion and Decision

# Introduction

A Revised WID on NR mobility enhancements Phase 4 [1] has been approved. In the WID, one of the objectives is to specify support for conditional Intra-CU LTM, as shown below.

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC

In this contribution, we provide our understandings on conditional intra-CU LTM.

# Discussion

  - Discussion for RRC/MAC open issue list

### (RRC-3 and MAC-10) Target cell and beam selection for CLTM execution

How to select the target cell and beam to perform CLTM needs to be discussed. In RAN2#129 meeting [2], RAN2 has reached the following agreements on beam selection for L1 based CLTM:

RAN2#129 meeting 5. For L1 based C-LTM (assuming single beam fulfils C-LTM execution condition), beam selection for RACH-less LTM with CG is based on the beam meets C-LTM execution condition. 6. For L1 based C-LTM events, the C-LTM execution is triggered when at least one beam fulfills the C-LTM event condition. When multiple candidate beams satisfy the C-LTM condition, it is up to UE implementation to select a beam and perform C-LTM.

For the target cell and beam selection of L3 based CLTM, two open issues have been provided in MAC and RRC open issue list [3][4]:

(RRC-3) Editor’s Note: FFS how to handle the case where more than one LTM candidate cell fulfil the LTM cell switch execution. (MAC-10) Beam selection for L3 based RACH-less CLTM determination.

(RRC-3) FFS how to handle the case where more than one LTM candidate cell fulfil the LTM cell switch execution.

In the legacy L3 CHO, if multiple NR cells are triggered in conditional reconfiguration execution, it is up to UE implementation which one to select. For L3 based CLTM, we can follow it.

Proposal 1: (RRC-3) When multiple candidate cells satisfy the L3 CLTM condition, it is up to UE implementation to select a candidate cell and perform CLTM.

(MAC-10) Beam selection for L3 based RACH-less CLTM determination.

To support RACH-less CLTM, the UE shall determine the target beam for cell switch from beams of the selected candidate cell which fulfils the L3 CLTM condition.

In L3 RACH-less HO, a RSRP threshold (cg-RRC-RSRP-ThresholdSSB) is configured for beam selection, and the UE selects an SSB with SS-RSRP above cg-RRC-RSRP-ThresholdSSB amongst the SSB(s) associated with the configured uplink grant [7].

In L1 based CLTM, the target beam selection for CG based RACH-less LTM is up to UE implementation.

Both of the above methods are feasible for L3 based CLTM. Following UE behaviours of L1 based CLTM can reduce the complexity, and the UE doesn’t need to distinguish between L1 based CLTM and L3 based CLTM for the target beam selection of CLTM.

Proposal 2: (MAC-10) For L3 based CLTM, it is up to UE implementation to select a beam and perform C-LTM.

### (MAC-9 and MAC-11) CG resource selection for RACH-less CLTM

For CG resource selection, two open issues have been provided in MAC open issue list [4]:

(MAC-9) In case UE considers RACH-less CLTM is ongoing, i.e., UE has valid TA for CLTM target cell, how UE selectes the valid CG resource for L3 based CLTM? (MAC-11) In case UE considers RACH-less CLTM is ongoing, i.e., UE has valid TA for LTM target cell, while UE couldn’t obtain valid CG resource, whether UE will fall back to RACH-based LTM? Or we need to change the condition to determine the RACH-less CLTM?

(MAC-9) In case UE considers RACH-less CLTM is ongoing, i.e., UE has valid TA for CLTM target cell, how UE selectes the valid CG resource for L3 based CLTM?

As captured in TS 38.331[5], the CG configuration can indicate SSB subset for SSB to CG PUSCH mapping within one CG configuration. In the Rel-18 RACH-less LTM, the UE selects the CG resource associated with the beam indicated in LTM CSC. Similarly, for the open issue (MAC-19), which CG resource is selected for L3 based CLTM is up to the target beam selected by UE.

As our view provided in P2, for L3 based CLTM, how to select the target beam is up to UE implementation. For the CG resource selection, the UE selects the CG resource associated with the selected target beam, if the selected target beam is associated with valid CG resource. And the principle also applies to L1 based CLTM.

Proposal 3: (MAC-9) If the selected target beam is associated with valid CG resource, the UE selects the CG resource associated with the selected target beam for CG based RACH-less CLTM.

(MAC-11) In case UE considers RACH-less CLTM is ongoing, i.e., UE has valid TA for LTM target cell, while UE couldn’t obtain valid CG resource, whether UE will fall back to RACH-based LTM? Or we need to change the condition to determine the RACH-less CLTM?

For the open issue (MAC-11), the simple method is that the UE falls back to RACH-based LTM. If the selected beam is not associated with valid CG resource, the UE shall perform RACH-based CLTM.

Proposal 4: (MAC-11) If the selected target beam for CLTM execution is not associated with valid CG resource, the UE performs RACH-based CLTM.

### Open issue: MAC-14

(MAC-14) FFS for the case when CLTM candidate TAT timer is running, an (Enhanced) LTM Cell switch Command MAC CE is received

For MAC-14, the key point is whether the valid TA stored by UE for the candidate cell can be used for MAC CE triggered LTM, and two options can be discussed:

- Option 1: The valid TA stored by UE can be used for MAC CE triggered LTM, e.g., if candidate LTM TAT timer is running; and TAC is set as FFF in (Enhanced) LTM Cell Switch Command MAC CE; and the UE has not measured the Timing Advance, the UE can perform RACH-less LTM Cell switch (using the TA value stored by UE).

- Option 2: The valid TA stored by UE cannot be used for MAC CE triggered LTM, e.g., if candidate LTM TAT timer is running; and TAC is set as FFF in (Enhanced) LTM Cell Switch Command MAC CE; and the UE has not measured the Timing Advance, the UE performs RACH-based LTM Cell switch.

For option 1, there are three methods (i.e., TA indicated in LTM CSC, UE measured TA, and TA stored for CLTM) for the candidate TA acquisition for LTM Cell switch. If we support option 1, RAN2 needs to discuss the priorities of these methods, which may result in additional work. Furthermore, both CLTM TA and TA in LTM CSC are indicated by the source cell. The source cell and the UE have the same understanding on the validity of candidate TA. It is simple to include the same TA value in LTM CSC as that maintained by the UE for the candidate cell. Hence, to support option 1 is not necessary.

Option 2 is aligned with the legacy UE behaviour of MAC CE triggered LTM. If an (Enhanced) LTM Cell switch Command MAC CE is received, the UE shall ignore the stored candidate TA. We prefer to support option 2, since it has no specification impact.

Proposal 5: (MAC-14) The valid candidate TA stored by UE cannot be used for MAC CE triggered LTM.

  - Other open issues

### CLTM TA

The issue when the number of TAs maintained by UE is up to the UE capability

In RAN2#129bis meeting [5], RAN2 has agreed to define a new per UE capability for UE support of early TA MAC CE reception for CLTM and also the max number of maintaining TA values. The value range is (1~8), which has been captured in the running CR of UE capability [9][10].

In Draft 306 [9]: In Draft 331[10]:

As captured in the current Running MAC CR [11], when an LTM Candidate Timing Advance Command MAC CE is received, the UE shall store the TA value and start or restart the associated ltm-Candidate-TimeAlignmentTimer.

The maximum number of CLTM candidate is 8 and CLTM with Rel-18 MIMO 2TA is supported. So, the most extreme case is the network would like to provide the TA value for each TAG id of each candidate via LTM Candidate Timing Advance Command MAC CE and the UE may need to store up to 16 TA values.

However, the maximum number of TA values that the UE can store is up to the UE capability cltm-EarlyTA-Indication-r19 and the value range of cltm-EarlyTA-Indication-r19 is 1~8. Moreover, C-LTM TAT for target cell and candidate cell is not stopped upon C-LTM execution. Therefore, it is highly possible that the number of TAs that the UE stores is up to the value of cltm-EarlyTA-Indication-r19 very soon.

In this case (when the number of TAs that the UE stores is up to cltm-EarlyTA-Indication-r19), if the network want to add a new CLTM TA to be maintained by UE (the UE needs to start a new CLTM TAT for the new TA), how to handle it. The issue can be solved from two different perspectives.

For the network-based method：

The network can avoid transmitting the LTM Candidate TAC to UE when the number of stored CLTM TAs is equal to cltm-EarlyTA-Indication-r19. Before transmitting the new TA value, the network can trigger CLTM TA release and then send the LTM Candidate TAC with the new TA value. Without the network triggered CLTM TA release procedure, if the network intends to send a new TA value for a better candidate, the network must wait for one CLTM TAT to expire.

In current Running MAC CR [11], the network can only add or modify the CLTM TA stored by UE, but the network can not release the CLTM TA stored by UE. For Rel-18 LTM, the TA value in LTM CSC can be set to FFF. If the TA value is set to FFF, this field indicates that no valid timing adjustment is available for the PTAG of the LTM target cell[7]. The similar principle can be used for the network triggered CLTM TA release, the network can set the TA value in LTM Candidate TAC to “FFF” to indicate that no valid TA is available for the indicated candidate cell (for the indicated TAG). If the UE receives the LTM Candidate Timing Advance Command MAC CE and the TA value in it is FFF, the UE shall release the TA value for the indicated candidate cell (for the indicated TAG) and stop associated CLTM TAT.

For the UE-based method:

In this case, the UE shall automatically release one maintained CLTM TA and stop associated TAT, and then maintain the new TA value indicated in the received LTM Candidate TAC.

For the UE based method, the key issue is which CLTM TA shall be released. One simple method is to release the oldest CLTM TA and the oldest TA is the one among all the TAs stored by UE that received the associated CLTM TAC earliest.

Based on the above analysis, both the network-based method and the UE-based method can be considered, and we provided the P6.

Proposal 6: When the number of CLTM TAs that the UE stores is up to cltm-EarlyTA-Indication-r19, to adding a new CLTM TA to be maintained by the UE, the following options can be considered.

- Option 1: (Network-based method) Before sending the LTM Candidate Timing Advance Command to add a new CLTM TA, the network can trigger one CLTM TA release by setting the TAC field to FFF in LTM Candidate Timing Advance Command MAC CE.

- Option 2: (UE-based method) Upon receiving an LTM Candidate Timing Advance Command MAC CE for adding a new CLTM TA, the UE shall release the oldest CLTM TA and stop associated CLTM TAT.

If the candidate TAT expires before CLTM trigger, how to handle the candidate TA.

Because the candidate TA is maintained in both UE and NW side, when the candidate TAT expires before CLTM trigger, the network can know it based on the network implementation. If needed, the network can trigger TA refresh by sending another PDCCH order to UE. (It is up to network implementation.)

Proposal 7: For PDCCH-ordered early TA acquisition, if the candidate TAT expires before CLTM trigger, it is up to network implementation to trigger TA refresh by sending another PDCCH order.

### The coexistence of CLTM and other mobility cases

In Rel-18 intra-CU LTM and Rel-19 inter-CU LTM, RAN2 has reached the following agreements for the coexistence of LTM and L3 Mobility：

For LTM and L3 HO/CHO: RAN2 #127 (Rel-19 MOB) agreement [12]: L3 mobility (including both the network triggered L3 HO and CHO) can be configured to UE, while the inter-CU LTM is configured (w/o DC), and the following items can be considered (follow Rel-18 intra-CU LTM): - When performing the L3 mobility (HO or CHO), the UE does not autonomously release inter-CU LTM configurations, unless these are explicitly released by the network. - The RRCReconfiguration message to execute an L3 mobility (HO or CHO) procedure may reconfigure inter-CU LTM configurations. - For the execution order between CHO and LTM, Rel-18 principle is applied. RAN2#128(Rel-18 MOB) agreement [13]: Clarify in RRC that the conditional reconfiguration cannot include the ltm-config. For DAPS: RAN2#125(Rel-18 MOB) agreement [14]: - The coexistence of LTM and DAPS HO is not supported.

For CLTM and L3 HO:

For the coexistence of CLTM and L3 HO, we can follow the Rel-19 inter-CU LTM agreement and support the coexistence of CLTM and L3 HO. When performing the L3 HO, the UE does not autonomously release CLTM configurations, unless these are explicitly released by the network and the L3 HO Command (RRCReconfiguration message) may reconfigure CLTM configurations.

Proposal 8: The coexistence of CLTM and L3 HO is supported in Rel-19.

For CLTM and CHO:

In Rel-18 and Rel-19, the coexistence of MAC CE triggered LTM and CHO is supported, and RAN2 has agreed “Network can send an LTM Cell Switch Command MAC CE indicating a CLTM candidate configuration” in [8]. So, the coexistence of CLTM and LTM is supported. To follow the regular LTM, the coexistence of CLTM and CHO can be considered.

But for the coexistence of LTM and CHO, the agreement in RAN2 #127 (Rel-19 MOB) and the agreement in RAN2#128(Rel-18 MOB) are in conflict.

Observation 1: For the coexistence of LTM/CLTM and CHO, the agreement in RAN2 #127 (Rel-19 MOB) and the agreement in RAN2#128(Rel-18 MOB) are in conflict:

- The agreement in RAN2 #127 (Rel-19 MOB): The RRCReconfiguration message to execute an L3 mobility (HO or CHO) procedure may reconfigure inter-CU LTM configurations.

- The agreement in RAN2#128(Rel-18 MOB): Clarify in RRC that the conditional reconfiguration cannot include the ltm-config.

Because the Rel-18 LTM agreement is newer than the Rel-19 LTM agreement, we need to follow the clarification for Rel-18 LTM that the conditional reconfiguration cannot include the ltm-config. Upon CHO execution, the serving cell changed and the Rel-18 ID of serving cell, Rel-19 ID of serving cell, UE based TA ID of serving cell and CLTM execution conditions need to be updated to associated with the CHO target cell. Without the LTM configuration update, the LTM and CLTM cannot work correctly. But the ltm-config cannot be updated during CHO execution based on the Rel-18 LTM agreement.

For the coexistence of CHO and MAC CE triggered LTM, there is no big problem. The network can send a new RRCReconfiguration to reconfigure the LTM configuration after CHO execution. Before the LTM configuration update, the network shall not trigger LTM Cell Switch.

But for the coexistence of CHO and CLTM, CLTM is triggered by UE. So, after CHO execution, the network may not be able to update the CLTM configuration before CLTM triggering based on the incorrect conditions. A simple method to solve the issue is that upon CHO execution, the UE shall stop the CLTM evaluation for all CLTM candidate cells. If the network reconfigures the CLTM execution condition(s), the UE can restart the evaluation of CLTM.

Proposal 9: The coexistence of CLTM and CHO is supported in Rel-19 and upon CHO execution, the UE shall stop the CLTM evaluation for all CLTM candidate cells.

For CLTM and DAPS:

For the coexistence case of CLTM and DAPS, we can follow the Rel-18 LTM.

Proposal 10: The coexistence of CLTM and DAPS HO is not supported.

### One issue for CLTM with Rel-18 MIMO 2TA

For the case without MIMO 2TA, RAN2 has reached the following agreements for the available TA value in RAN2#129 meeting [2]:

RAN2#129 meeting 4. When the C-LTM execution condition is satisfied with a candidate beam (any beam from the candidate RS set) / cell, UE performs RACH-less C-LTM (with that triggered beam for L1 based C-LTM case) if UE has a valid TA for the associated candidate cell. Otherwise, UE performs RACH-based C-LTM.

Based on the agreement, the UE can perform RACH-less CLTM if the UE has a valid TA for candidate cell. But in RAN2#129bis meeting [8], CLTM with Rel-18 MIMO 2TA is supported. There will be mapping between TCI-state and TAG id, and each candidate TCI state is mapping with a TAG id. If 2TAs are configured, when the UE doesn’t have a valid TA for the associated TAG id which is mapping with the selected candidate beam, the UE shall perform RACH-based CLTM.

Proposal 11: For CLTM, when 2TAGs are configured, if the UE has no valid TA for the TAG id of the selected candidate beam, the UE performs RACH-based CLTM.

# Conclusions

According to the analysis given above, we have the following observations and proposals:

2.1	Discussion for RRC/MAC open issue list:

2.1.1	(RRC-3 and MAC-10) Target cell and beam selection for CLTM execution

Proposal 1: (RRC-3) When multiple candidate cells satisfy the L3 CLTM condition, it is up to UE implementation to select a candidate cell and perform CLTM.

Proposal 2: (MAC-10) For L3 based CLTM, it is up to UE implementation to select a beam and perform C-LTM.

2.1.2	(MAC-9 and MAC-11) CG resource selection for RACH-less CLTM

Proposal 3: (MAC-9) If the selected target beam is associated with valid CG resource, the UE selects the CG resource associated with the selected target beam for CG based RACH-less CLTM.

Proposal 4: (MAC-11) If the selected target beam for CLTM execution is not associated with valid CG resource, the UE performs RACH-based CLTM.

2.1.3	 Open issue: MAC-14

Proposal 5: (MAC-14) The valid candidate TA stored by UE cannot be used for MAC CE triggered LTM.

2.2	Other open issues:

2.2.1	CLTM TA

Proposal 6: When the number of CLTM TAs that the UE stores is up to cltm-EarlyTA-Indication-r19, to adding a new CLTM TA to be maintained by the UE, the following options can be considered.

- Option 1: (Network-based method) Before sending the LTM Candidate Timing Advance Command to add a new CLTM TA, the network can trigger one CLTM TA release by setting the TAC field to FFF in LTM Candidate Timing Advance Command MAC CE.

- Option 2: (UE-based method) Upon receiving an LTM Candidate Timing Advance Command MAC CE for adding a new CLTM TA, the UE shall release the oldest CLTM TA and stop associated CLTM TAT.

Proposal 7: For PDCCH-ordered early TA acquisition, if the candidate TAT expires before CLTM trigger, it is up to network implementation to trigger TA refresh by sending another PDCCH order.

2.2.2 The coexistence of CLTM and other mobility cases

Proposal 8: The coexistence of CLTM and L3 HO is supported in Rel-19.

Observation 1: For the coexistence of LTM/CLTM and CHO, the agreement in RAN2 #127 (Rel-19 MOB) and the agreement in RAN2#128(Rel-18 MOB) are in conflict:

- The agreement in RAN2 #127 (Rel-19 MOB): The RRCReconfiguration message to execute an L3 mobility (HO or CHO) procedure may reconfigure inter-CU LTM configurations.

- The agreement in RAN2#128(Rel-18 MOB): Clarify in RRC that the conditional reconfiguration cannot include the ltm-config.

Proposal 9: The coexistence of CLTM and CHO is supported in Rel-19 and upon CHO execution, the UE shall stop the CLTM evaluation for all CLTM candidate cells.

Proposal 10: The coexistence of CLTM and DAPS HO is not supported.

2.2.3	 One issue for CLTM with Rel-18 MIMO 2TA

Proposal 11: For CLTM, when 2TAGs are configured, if the UE has no valid TA for the TAG id of the selected candidate beam, the UE performs RACH-based CLTM.

# Reference

- RP-250339, “Revised Work Item: NR mobility enhancements Phase 4”, Apple Inc, China Telecom

- R2-2501702, “Report of 3GPP TSG RAN WG2 meeting #129”

- R2-250xxxx, “List of RRC FFSs for mobility”, Ericsson

- R2-250xxxx, “Discussion summary and list of MAC open issues for mobility”, vivo

- 3GPP TS 38.331

- R2-250xxxx, “Running RRC CR for inter-CU and conditional LTM”, Ericsson

- 3GPP TS 38.321

- R2-250xxxx, “Report of 3GPP TSG RAN WG2 meeting #129bis”

- R2-250xxxx, “Draft 306 running CR for UE capability for Mob Ph4”, CATT

- R2-250xxxx, “Draft 331 running CR for UE capability for Mob Ph4”, CATT

- R2-250xxxx, “Running MAC CR for enhanced mobility Ph4”, vivo

- R2-2409210, “Report of 3GPP TSG RAN WG2 meeting #127”

- R2-2500002, “Report of 3GPP TSG RAN WG2 meeting #128”

- R2-2403994, “Report of 3GPP TSG RAN WG2 meeting #125”