3GPP TSG-RAN WG2 Meeting #130	R2-2503535

St. Julians, Malta, May 19th – 23rd, 2025

Source:	NTT DOCOMO, INC.

Title:	Discussions on data collection

Agenda Item:	8.3.4

Document for:	Discussion and Decisions

- Introduction

During the RAN#102 meeting, the SID on “Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR”[1] was approved. The SID describes the study scope of the AI/ML-aided mobility enhancements as below,

Study and evaluate potential benefits and gains of AI/ML aided mobility for network triggered L3-based handover, considering the following aspects:

- AI/ML based RRM measurement and event prediction,

  - Cell-level measurement prediction including intra and inter-frequency (UE sided and NW sided model) [RAN2]

    - Inter-cell Beam-level measurement prediction for L3 Mobility (UE sided and NW sided model) [RAN2]

  - HO failure/RLF prediction (UE sided model) [RAN2]

  - Measurement events prediction (UE sided model) [RAN2]

- Study the need/benefits of any other UE assistance information for the network side model [RAN2]

- The evaluation of the AI/ML aided mobility benefits should consider HO performance KPIs (e.g., Ping-pong HO, HOF/RLF, Time of stay, Handover interruption, prediction accuracy, and measurement reduction) etc.) and complexity tradeoffs [RAN2]

  - NOTE: Simulation assumption and methodology can leverage TR 38.901, 38.843 and 36.839. And leave the detail discussion to RAN2

- Potential AI mobility specific enhancement should be based on the Rel19 AI/ML-air interface WID general framework (e.g. LCM, performance monitoring etc) [RAN2]

  - NOTE: This would only be treated after sufficient progress is made in the Rel-19 AI/ML air interface WID

- Potential specification impacts of AI/ML aided mobility [RAN2]

- Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4]

This paper discusses the training data collection for both NW- and UE-sided model.

- Training data collection for the NW-sided model

At the RAN2 #129bis meeting, the baseline framework has been agreed for NW-side data collection [2],

As a baseline, use the AI ML PHY NW-side data collection procedures, configuration and reporting framework.

Figure 1: General procedure of NW side data collection from AI4PHY

Figure 1 shows the general procedure for NW-side data collection specified in the AI/ML for PHY session. Both gNB-centric and OAM-centric data collections are considered, while the immediate MDT framework is the baseline framework for OAM-centric data collections. The typical steps are listed in the following,

- UE receives configurations about the measurements and logging, where periodic or event-triggered data logging can be configured.

- UE starts the data logging based on the configurations and conditions.

- UE can report the availability indication due to reasons like full buffer being reached, buffer threshold being reached, low power.

- UEInformationRequest/UEInformationResponse are used for the on-demand reporting of logged data.

- UE reports the logged data via low priority (SRB4 or new SRB).

In our view, the procedure and container for configuring and reporting AI/ML for PHY could be reused. The contents of logged data and corresponding configuration, and the procedure affected by the content of logged data (e.g., the HO procedure) should be studied case by case.

Proposal 1

- The contents of logged data include the following,

  - For RRM measurement prediction and measurement event prediction,

    - UE should log the measurement results and related information (e.g., time stamp).

  - For measurement event prediction, UE can further log the following,

    - Entering, leaving, and/or triggering of the measurement events.

At the RAN2 #129bis meeting, the agreements about the data collection during HO are as follows[2],

Agreements on Idle/inactive and HO

- Introduce 1-bit indication on whether to release or retain un-retrieved data in RRCReconfiguration during/before HO.  Source gNB decides whether the data should be kept.  The indication is provided in RRCReconfiguration (i.e. not in RRC Reconfiguration from target cell).   FFS signaling details.

- Upon going to RRC_IDLE, RLF, or RRC_INACTIVE, UE discards any logged data

These agreements are reached considering use cases other than AI/ML for mobility, where the data collected during handover or RLF is not essential for the model training. Therefore, UE can discard them without impacting the model performance. However, for AI/ML for mobility use cases, the data collected when HO or RLF is about to happen is essential for the model performance and should be retained during the HO or RLF. Therefore, we have the following proposals,

Proposal 2

- UE should retain unretrieved data during/before HO, or RLF.

  - FFS: whether UE should retain unretrieved data upon going to RRC_IDLE/INACTIVE.

- Training data collection for the UE-sided model (issues other than data transfer)

At the RAN2 #129bis meeting, the baseline framework has been agreed for UE-side data collection [2],

Study UE side data collection configuration, taking AI ML PHY relevant procedure as baseline. Postpone UE side data collection transfer discussion until further progress is made in AI ML PHY in R20.

Figure 2: General procedure of UE data collection

Figure 2 shows the general procedure for UE-side data collection specified in the AI/ML for PHY session. The NW controls the UE-side data collection, and the UE/UE-side can send the data collection request to the NW if it is allowed by the NW. The NW can configure the UE for data collection configuration with or without the UE/UE-side request.

The main principle of UE-side data collection should be reused for AI/ML in mobility use cases. The NW controls the data collection by sending UE-related configurations. The UE/UE-side requests are usually helpful for the NW in judging whether UEs need to collect the data. Two approaches can be adopted to support this feature. One is the direct request from the UE. The other is from the UE-side servers, which occupy fewer air-interface resources. If the request from the UE-side server is supported, the NW should be able to disable the UE direct request.

For AI/ML for PHY use cases  (beam management and CSI prediction), what the UE to measure for the data collection is configured under the CSI framework. For this use case, RAN2 should study the detailed configuration for the UE to collect Layer 3 measurements.

Under this framework, we propose the following about reusing the existing features and further studying issues,

Proposal 3

- RAN2 reuses the principle that the NW controls both the UE data collection and the UE/UE-side request for the data collection.

  - The NW can configure the UE data collection with or without UE/UE-side request.

  - The NW configures whether the data collection request from the UE is allowed.

- Study how to configure UE to collect measurements for RRM measurement or event predictions.

- Conclusion

In this contribution, we have the following observations,

Proposal 1

- The contents of logged data include the following,

  - For RRM measurement prediction and measurement event prediction,

    - UE should log the measurement results and related information (e.g., time stamp).

  - For measurement event prediction, UE can further log the following,

    - Entering, leaving, and/or triggering of the measurement events.

Proposal 2

- UE should retain unretrieved data during/before HO, or RLF.

  - FFS: whether UE should retain unretrieved data upon going to RRC_IDLE/INACTIVE.

Proposal 3

- RAN2 reuses the principle that the NW controls both the UE data collection and the UE/UE-side request for the data collection.

  - The NW can configure the UE data collection with or without UE/UE-side request.

  - The NW configures whether the data collection request from the UE is allowed.

- Study how to configure UE to collect measurements for RRM measurement or event predictions.

References

- Moderator (CMCC), RP-234055, “Study on AI (Artificial Intelligence)/ML (Machine Learning) for mobility in NR,” Dec. 2023.

- RAN2 Chair (InterDigital), “RAN2 #129big Chair Notes”, Apr., 2025.