3GPP TSG-RAN WG2 Meeting #129bis	R2-2503538

St.Julians, Malta, May. 19th – 23rd, 2025

Agenda item:	8.1.3

Source:	Samsung

Title:	Discussion on NW side data collection

Document for:	Discussion & Decision

Introduction

This document discusses on NW side data collection for Rel-19 AI/ML air interface WI.

Discussion

Memory capability

The following agreement was made in RAN2#127, in order for UE to log training data for NW side data collection:

<Agreements (RAN2#127)> - UE stores the logged training data at AS layer with a minimum AS layer memory size supported by the UE. FFS on the memory size. This is across all use cases.

In legacy, there are existing memory requirements for UE to log measurement results. Firstly, for logged MDT, the fixed size (i.e., 64KB) of minimum memory requirement is defined. This is commonly mandatory for UEs who support logged MDT, and no separate capability is used for this memory requirement.

<TS 38.306> loggedMeasurements-r16 Indicates whether the UE supports logged measurements in RRC_IDLE and RRC_INACTIVE state. A UE that supports logged measurements shall support both periodical logging and event-triggered logging. The minimum memory size of MDT logged measurements is 64KB. For eRedCap UE supporting this feature, the minimum memory size of MDT logged measurements is 16KB.; UE; No; No; No.

Meanwhile, there are two separate memories defined for QoE measurement.

- Memory 1. This is the memory for UEs to log QoE reports in RRC_CONNECTED while QoE reporting is temporarily paused by NW. The mandatory minimum requirement for this memory is 64KB as highlighted below.

- Memory 2. This is the memory for UE to log QoE reports in RRC_IDLE and RRC_INACTIVE for MBS broadcast service. The mandatory minimum requirement for this memory is 64KB as highlighted below. Besides, UE optionally may support more than 64KB for this memory. In this case, UE indicates the size (i.e., larger than 64KB) it supports to NW via an explicit capability indication.

<TS 38.306> # 6 Conditionally mandatory features without UE radio access capability parameters Features; Condition. Acquisition of positioning SI messages with 80 milliseconds offset position compared to SI messages in schedulingInfoList; It is mandatory to support acquisition of positioning SI messages with 80 milliseconds offset position compared to SI messages in schedulingInfoList for UEs which support the acquisition of the posSIB types in posSchedulingInfoList as specified in TS 38.331 [9].. Acquisition of SI messages with explicit SI window positions; It is mandatory to support acquisition of SI messages with explicit SI window positions for UEs which support the SIB types in schedulingInfoList2 as specified in TS 38.331 [9].. AS layer memory size for QoE paused measurement reports; It is mandatory to support the minimum AS layer memory size of 64KB for QoE paused measurement reports for UEs which support qoe-Streaming-MeasReport-r17, qoe-MTSI-MeasReport-r17 or qoe-VR-MeasReport-r17.. AS layer memory size for QoE measurement reports in RRC_IDLE and RRC_INACTIVE; It is mandatory to support the minimum AS layer memory size of 64KB for QoE measurement reports stored in RRC_IDLE/RRC_INACTIVE for UEs which support qoe-IdleInactiveMeasReport-r18 and any of qoe-Streaming-MeasReport-r17 or qoe-MTSI-MeasReport-r17 or qoe-VR-MeasReport-r17. This memory size is additional to "AS layer memory size for QoE paused measurement reports". <SKIP> ## 4.2 UE Capability Parameters <SKIP> qoe-AdditionalMemoryMeasReport-r18 Indicates the minimum AS layer memory size the UE supports for QoE measurement in RRC_IDLE and RRC_INACTIVE in addition to the "AS layer memory size for QoE paused measurement reports". Value kB128 means the UE supports at least 128 kilobytes for this purpose, and so on. A UE supporting this feature shall also support qoe-IdleInactiveMeasReport-r18.; UE; No; No; No. <TS 38.331> #### – AppLayerMeasParameters The IE AppLayerMeasParameters is used to convey the capabilities supported by the UE for application layer measurements. AppLayerMeasParameters information element -- ASN1START -- TAG-APPLAYERMEASPARAMETERS-START AppLayerMeasParameters-r17 ::= SEQUENCE { qoe-Streaming-MeasReport-r17 ENUMERATED {supported} OPTIONAL, qoe-MTSI-MeasReport-r17 ENUMERATED {supported} OPTIONAL, qoe-VR-MeasReport-r17 ENUMERATED {supported} OPTIONAL, ran-VisibleQoE-Streaming-MeasReport-r17 ENUMERATED {supported} OPTIONAL, ran-VisibleQoE-VR-MeasReport-r17 ENUMERATED {supported} OPTIONAL, ul-MeasurementReportAppLayer-Seg-r17 ENUMERATED {supported} OPTIONAL, ..., [[ qoe-IdleInactiveMeasReport-r18 ENUMERATED {supported} OPTIONAL, qoe-NRDC-MeasReport-r18 ENUMERATED {supported} OPTIONAL, qoe-AdditionalMemoryMeasReport-r18 ENUMERATED {kB128, kB256, kB512, kB1024} OPTIONAL, qoe-PriorityBasedDiscarding-r18 ENUMERATED {supported} OPTIONAL, srb5-r18 ENUMERATED {supported} OPTIONAL ]] } -- TAG-APPLAYERMEASPARAMETERS-STOP -- ASN1STOP

If UE does not indicate this optional capability in UECapabilityInformation message, it means the UE only supports the mandatory 64 KB memory for QoE measurement in RRC_IDLE and RRC_INACTIVE. The motivation for this optional capability is UEs may have different memory capabilities and NW may want to select the UE who has big memory capability for more and consistent data collection.

We assume the same benefit can be expected, if we have similar capability for NW side data collection. i.e., UE mandatorily supports the fixed size of minimum AS layer memory without UE capability signalling, and optionally reports via UE capability a larger minimum AS layer memory size. i.e., Taking different sizes of buffer capability from UEs into account, NW can select/configure UEs to perform NW-side data collection. The exact numbers (e.g., 64KB) could be discussed after RAN1/2’s progress on data content for NW-side data collection.

Proposal 1. (Open issue RRC-38) UE mandatorily supports the fixed size of minimum AS layer memory without UE capability signalling, and optionally reports via UE capability a larger minimum AS layer memory size. FFS for the exact numbers.

UE assistance information

RAN2 agreed UE autonomously stops measurement for NW-side data collection in case of full buffer and it is indicated to NW via UAI.

<Agreement (RAN2#127)> - When UE reaches its buffer limitation the UE stops measurement for data collection purposes and logging. <Agreement (RAN2#129bis)> - Availability indication can be triggered due to: - Full buffer being reached (if configured) - Buffer threshold being reached (if configured). - Low power (if configured) - The UE send a UAI that indicates: - Data is available - Reason for trigger (full buffer, threshold) - Low power indication

RAN2 needs to discuss on when UE resumes suspended measurement due to full buffer. Obviously, UE can resume after any memory becomes available (i.e., after any data report is sent to NW after full buffer). In the perspective of NW, it can also implicitly know UE’s resumption based on UE’s data reporting after the full buffer indication. Therefore, we propose UE autonomously resumes measurement after any memory becomes available (i.e., after any data report is sent to NW after full buffer).

Proposal 2. (Open issue RRC-29) UE autonomously resumes measurement/logging paused due to full buffer, when memory becomes available (i.e., after logged data report is sent to NW).

RAN2 also agreed UE indicates low power status to NW via UAI. Based on this indication, network can de-configure UE’s data collection.

<Agreement (RAN2#127)> - The UE reports to the network when the power state is low. We will not specify how the UE determines low power state. The network should de-configure the data collection (this can be captured in stage 2). <Agreement (RAN2#129bis)> - Availability indication can be triggered due to: - Full buffer being reached (if configured) - Buffer threshold being reached (if configured). - Low power (if configured) - The UE send a UAI that indicates: - Data is available - Reason for trigger (full buffer, threshold) - Low power indication

However, there is no way for UE to indicate NW that it no longer suffers from low power issue. Without this information, NW could misunderstand the UE, who has sent a low power indication once, still experiences low power issue, even though it does not suffer any longer.

Proposal 3. (Open issue RRC-29) When UE is no longer in low-power state, UE triggers UAI transmission and indicates it to NW.

RAN2 also agreed UE triggers UAI transmission when buffer threshold is reached.

<Agreement (RAN2#129bis)> - Availability indication can be triggered due to: - Full buffer being reached (if configured) - Buffer threshold being reached (if configured). - Low power (if configured) - The UE send a UAI that indicates: - Data is available - Reason for trigger (full buffer, threshold) - Low power indication

In last meetings, companies proposed two different options for this buffer threshold.

- Option 1. Buffer threshold is configured as percentage (%) of logged data over UE’s whole memory size.

- Option 2. Buffer threshold is configured as the size of logged data (e.g., kilobytes)

As discussed in Proposal 1, NW can know the minimum requirement of UE’s memory (e.g., 256KB, 512KB, …). However, even though UE reports one of the minimum memory sizes (e.g., 256 KB), NW may not know the exact size of memory UE uses (e.g., 400KB). Thus, with Option 1, NW cannot understand how much UE stores data exactly. On the other hand, in Option 2, NW can know the exact size of logged i.e., same as the configured threshold, at least upon reception of UAI. So, based on this information, NW can decide the proper time to retrieve data.

Proposal 4. Buffer threshold is configured as the size of logged data (e.g., kilobytes). The exact numbers could be discussed after the decision on minimum memory requirement/capability.

However, even with Proposal 4 (i.e., Option 2), NW may not always track the exact size of logged data (i.e., NW only knows it only upon reception of UAI). For example, when UE’s buffer reaches the configured threshold (e.g., 128KB), UE triggers UAI transmission. However, NW may not retrieve data upon reception of UAI (e.g., NW may be overloaded temporarily). Meanwhile, UE can collect more data based on event-based logging, which means NW cannot track how more data is logged afterwards. After a while, NW can retrieve some data (e.g., 50KB). Now UE’s buffer could become lower than the configured threshold (e.g., 128KB). However, NW cannot understand the UE’s buffer is lower or still bigger than the configured threshold (e.g., 128KB), unless UE sends another UAI when the buffer becomes less than the threshold. So, we propose UE also triggers UAI when the buffer becomes less than the threshold. However, it could trigger too often UAI reporting, so we also propose to introduce a prohibit timer for buffer level threshold.

Proposal 5. (Open issue RRC-29) When UE's buffer level becomes less than the configured threshold, UE triggers UAI transmission and indicates it to NW.  Prohibit timer is introduced for buffer threshold based UAI transmission.

What we want to clarify further is NW-side data collection cannot work without UAI transmission (i.e., at least availability indication based on buffer threshold). In that sense, we propose UAI configuration (i.e., at least buffer-threshold) should always be configured when NW-side data collection is configured.

Proposal 6. When NW-side data collection is configured, UAI configuration (i.e., at least buffer-threshold) should be always configured together.

In the last meeting, RAN2 also have agreed the following agreements:

<Agreement (RAN2#129bis)> - Upon going to RRC_IDLE, RLF, or RRC_INACTIVE, UE discards any logged data

It means the continuity of data collection is not supported other than handover case. In that sense, we would like to clarify UE should release both NW-side data collection configuration and the corresponding UAI configuration in case of transition to RRC_IDLE and RRC_INACTIVE, and RRC Reestablishment.

Proposal 7.  (Open issue RRC-6) UE releases both NW-side data collection configuration and the corresponding UAI configuration in case of transition to RRC_IDLE and RRC_INACTIVE, and RRC Reestablishment.

Mobility

In the last meeting, RAN2 agreed to introduce a 1-bit indication on whether to retain data or not, during handover.

<Agreement (RAN2#129bis)> - Introduce 1-bit indication on whether to release or retain un-retrieved data in RRCReconfiguration during/before HO. Source gNB decides whether the data should be kept. The indication is provided in RRCReconfiguration (i.e. not in RRC Reconfiguration from target cell). FFS signaling details.

RAN2 also agreed this indication is decided by source gNB and the source includes this indication when it sends RRCReconfiguration message to UE. However, as legacy procedure, we understand the source gNB cannot modify the RRCReconfiguration message from target gNB. Therefore, we understand this agreement needs to be re-visited. The rapporteur for [POST129bis][016][AI PHY] also spotted the same issue as one of open issues and suggested two option [1]:

- Option 1. The source gNB sends the 1-bit indication to the UE before HO.

- Option 2. The source gNB decides if the 1-bit indication is needed and, if so, sends it to the target gNB, which includes it in the RRCReconfiguration sent to the UE during HO. No RAN3 impact is expected if the transmission of the 1-bit indication is limited to the case in which the target gNB is from the same vendor as the source gNB.

In our understanding, Option 1 has two drawbacks. Firstly, NW has to send a separate and additional RRCReconfiguration message only for this indication. Besides, we do not understand how the source can make the decision on the indication in advance (i.e., before handover) as it could differ depending on who would be the target node. Therefore, we prefer Option 2. However, we think the indication should be transferred from the source to the target within HandoverPreparationInformation message. i.e., We expect spec impact in RAN2 or RAN3.

Proposal 8. (Open issue RRC-18) During handover, source gNB provides target gNB with the 1-bit indication on whether to release or retain un-retrieved data in HandoverPreparationInformation message. Then, target gNB includes the indication in RRCReconfiguration message within HandoverCommand message.

We assume the target node can re-use the configuration for NW-side data collection configured by source node. For example, target and source nodes could have same PLMN and NW could want UE to continue data collection after handover. On the other hand, NW may not want to continue the data collection (e.g., if source and target nodes have different PLMN). So, we propose target node can keep or release the configuration for NW-side data collection configured by source node

Proposal 9. During handover, the target node can keep or release the configuration for NW-side data collection configured by the source node.

Data content

In the last meeting, RAN2 discussed on whether timestamp for logged data is need or not, and agreed NW should be known whether there is temporal gap more than configured periodicity.

<Agreement (RAN2#129bis)> - For temporal domain, the network is made aware whether there is a gap between two consecutive samples. FFS amount of gap and whether this is implicit or explicit

There were a few of proposals from companies to introduce timestamp in reports to indicate when the data is measured/logged. It is obvious any kind time information is needed for NW to train model at least for temporal prediction. However, we do not think the explicit timestamp is needed. i.e., NW can understand the time information implicitly, as discussed specifically below.

In case of periodic logging, for each CSI-RS or SSB resource, UE logs data (e.g., L1-RSRP) periodically. Given NW configures the periodicity of the resource, it is enough for UE to contain/report the data (e.g., L1-RSRP) in chronological order in a list for logging. Then, NW can understand the differential time between two consecutive entries in the list is same as the configured periodicity of the resource.

On the other hand, in case of event-based logging, data logging may not be uniformly periodic, as there could be the period during which UE does not measure the configured resources. For example,

- 1) When the event is fulfilled, UE starts logging data periodically (i.e., 1st logging instance).

- 2) When the event is not fulfilled any longer, UE stops measurement/logging.

- 3) When the event is fulfilled again, UE resumes measurement and logs data periodically again (i.e., 2nd logging instance).

There is a similar issue when we consider full buffer case, even if periodic logging is configured. For example,

- 1) When UE is configured with data collection (e.g., periodic logging), UE logs data periodically (i.e., 1st logging instance).

- 2) When UE’s buffer becomes full, UE stops measurement/logging.

- 3) After some data reporting to NW, some buffer becomes available and UE resumes measurement and logs data periodically again (i.e., 2nd logging instance).

In the two aforementioned examples, if UE contains the logged data for a common list sequentially, the time difference between the last data for the 1st logging instance and the first data of the 2nd logging instance may not be same as the configured periodicity of the resource. i.e., it would be same as the period in which the logging was suspended. However, NW cannot know about it, so it may bring out degradation of model training performance. To address this issue, one solution could be adding a timestamp or new indication to differentiate each logging instance. However, we propose the solution without explicit timestamp i.e., UE contains the data in a separate data list per each logging instance. For example,

- When UE performs measurement periodically (i.e., within a logging instance), UE contains the data sequentially in a common data list.

- Whenever UE suspends and resumes measurement (depending on whether event is fulfilled and whether the buffer is full), UE starts logging the subsequent data using a separate data list

Figure 1. Illustration for Proposal 9

Proposal 10. (Open issue RRC-21) No explicit timestamp is needed for logged data to indicate time gap between two consecutive samples. Instead, the following implicit way is introduced:

- When UE performs measurement periodically, UE contains the data sequentially in a common data list.

- Whenever UE suspends and resumes measurement (depending on whether event is fulfilled and whether the buffer is full), UE starts logging the subsequent data using a separate data list.

If NW wants to collect to sufficient labelled data for good quality of model training, the data collection could be configured to UE long enough (e.g., a few minutes or hours). However, the interval for logging or measurement could be short (e.g., a tens of millisecond). As a result, the numerous of data could be collected by UE. In that sense, we would like to note that signalling optimization is highly of importance to minimize the size of report. So, if UE is configured to collect L1-RSRP, we propose to include “absolute” L1-RSRP in the 1st entry of the data list, but “differential” L1-RSRPs for the other entries per each data list per each beam. The differential L1-RSRP can be calculated compared to L1-RSRP of the previous entry, as the two consecutive samples is likely to have similar L1-RSRPs, which requires less bits to represent differential L1-RSRP in ASN.1.

Figure 2. Illustration for Proposal 10

Proposal 11. When UE is configured to log L1-RSRP, UE includes L1-RSRPs sequentially in each data list for each beam, where the n-th entry of the data list is set as follows:

- If n=1, absolute L1-RSRP is set

- Otherwise (i.e., n>1), differential L1-RSRP is set compared to L1-RSRP of (n-1)th entry

Coexistence in report

Rapporteur has provided the list of RRC open issues [1]. As one of the issues, RAN2 needs to discuss the co-existence of two types of data in a common UEInformationResponse message, 1) AI/ML data (i.e., for NW-side data collection) and 2) the other existing data (i.e., for SON/MDT). There could be two options.

- Option 1. UEInformationResponse message cannot include both types of data.

- Option 2. UEInformationResponse message can include both types of data.

We assume Option 1 can be implemented by restricting NW behaviour. i.e., NW should not request both types of data when sending UEInformationRequest message. However, this would require two separate UEInformationRequest/UEInformationResponse exchange which is inefficient way of signalling. Thus, we support Option 2 and the new SRB could be used when the UEInformationResponse message includes at least AI/ML data.

Proposal 12. (Open issue RRC-26) UEInformationResponse message can include both types of data. 1) AI/ML data (i.e., for NW-side data collection) and 2) the other existing data (i.e., for SON/MDT). When UEInformationResponse message includes at least AI/ML data, the new SRB (e.g., SRBx) is used.

Conclusion

RAN2 is requested to discuss and agree the following proposals:

Proposal 1. (Open issue RRC-38) UE mandatorily supports the fixed size of minimum AS layer memory without UE capability signalling, and optionally reports via UE capability a larger minimum AS layer memory size. FFS for the exact numbers.

Proposal 2. (Open issue RRC-29) UE autonomously resumes measurement/logging paused due to full buffer, when memory becomes available (i.e., after logged data report is sent to NW).

Proposal 3. (Open issue RRC-29) When UE is no longer in low-power state, UE triggers UAI transmission and indicates it to NW.

Proposal 4. Buffer threshold is configured as the size of logged data (e.g., kilobytes). The exact numbers could be discussed after the decision on minimum memory requirement/capability.

Proposal 5. (Open issue RRC-29) When UE's buffer level becomes less than the configured threshold, UE triggers UAI transmission and indicates it to NW.  Prohibit timer is introduced for buffer threshold based UAI transmission.

Proposal 6. When NW-side data collection is configured, UAI configuration (i.e., at least buffer-threshold) should be always configured together.

Proposal 7.  (Open issue RRC-6) UE releases both NW-side data collection configuration and the corresponding UAI configuration in case of transition to RRC_IDLE and RRC_INACTIVE, and RRC Reestablishment.

Proposal 8. (Open issue RRC-18) During handover, source gNB provides target gNB with the 1-bit indication on whether to release or retain un-retrieved data in HandoverPreparationInformation message. Then, target gNB includes the indication in RRCReconfiguration message within HandoverCommand message.

Proposal 9. During handover, the target node can keep or release the configuration for NW-side data collection configured by the source node.

Proposal 10. (Open issue RRC-21) No explicit timestamp is needed for logged data to indicate time gap between two consecutive samples. Instead, the following implicit way is introduced:

- When UE performs measurement periodically, UE contains the data sequentially in a common data list.

- Whenever UE suspends and resumes measurement (depending on whether event is fulfilled and whether the buffer is full), UE starts logging the subsequent data using a separate data list.

Proposal 11. When UE is configured to log L1-RSRP, UE includes L1-RSRPs sequentially in each data list for each beam, where the n-th entry of the data list is set as follows:

- If n=1, absolute L1-RSRP is set

- Otherwise (i.e., n>1), differential L1-RSRP is set compared to L1-RSRP of (n-1)th entry

Proposal 12. (Open issue RRC-26) UEInformationResponse message can include both types of data. 1) AI/ML data (i.e., for NW-side data collection) and 2) the other existing data (i.e., for SON/MDT). When UEInformationResponse message includes at least AI/ML data, the new SRB (e.g., SRBx) is used.

References

[1] R2-2504350, Remaining RRC open issues in feature AIML PHY, Ericsson