3GPP TSG-RAN WG2 Meeting #130			           		R2-2503539

St.Julians, Malta, May 19th – 23rd, 2025

Agenda item:	8.1.2.2

Source:	Samsung

Title:	Discussion on LCM for UE-sided model for beam management

Document for:	Discussion and Decision

# Introduction

During post email discussion, RRC open issues are identified [1]. In this contribution, we would like to provide our view on some of RRC open issues related to LCM for beam management.

# RRC open issues listed in RAN2 email discussion

# Open issue RRC-1: Cause of inapplicability

Open issue RRC-1: Cause of inapplicability Issue description: It is FFS how to define the simple cause value of inapplicability related to model availability and how to capture it in the spec. This issue refers to the RAN2#129bis agreement: Together with inapplicability reporting, UE further indicates a simple cause value of inapplicability FFS how to define this simple cause related to model availability and how we capture it in the spec. The issue is captured as an editor’s note in the running CR, clause 5.3.5.3. Proposed resolution: It is suggested that companies provide contributions to the following meeting to resolve the issue.

In the last meeting, two different approaches were proposed by companies.

- Approach 1: indicate specific reason why a functionality is not applicable. For example, a cause is due to the lack of model meeting NW-side additional condition, UE side additional condition, the lack of trained model, etc.

- Approach 2: indicate a long term or short term that a functionality is non-applicable.

We have two reasons to prefer approach 2.

- It is not feasible to standardize these parameters clearly: although the cause considered in approach 1 should be factors that UE decide applicability but it doesn’t seem practical to standardize. RAN1 has not explicitly identified what is NW-side additional condition or UE side additional condition. In addition, the lack of trained model is also hard to define because we don’t define “model” in functionality-based LCM.

- The most important factor is how gNB utilize this information: from gNB point of view, it would be useful to know the cause of inapplicability to determine whether the functionality should be kept considering it can change applicability status in the near future or not. For example, if functionality is not applicable due to the lack of model, it is not likely that it becomes applicable in the near future because training would take significant time.

Proposal 1: [Open issue RRC-1] one bit to indicate two values (long term and short term) is introduced for cause of inapplicability.

# Open issue RRC-2: Content of otherConfig for enabling applicability reports in UAI

Open issue RRC-2: Content of otherConfig for enabling applicability reports in UAI Issue description: It is not yet clarified what the content (if any) of the UAI configuration should be, to enable the UE to report applicability in UAI, e.g. applicability updates/changes as agreed for option A. For instance, this configuration could be just a flag or could contain further information. The issue is captured as an editor’s note in the running CR, clause 6.3.4. Proposed resolution: It is suggested that companies provide contributions to the following meeting to resolve the issue.

We understand that this open issue is to define detailed parameters under ApplicabilityReportConfig-r19. As discussed in the last meeting, the detailed parameters are under discussion in RAN1. So, we could wait for the detailed parameters. For option A, a simple flag should be enough to enable applicability updates/changes.

Proposal 2: [Open issue RRC-2] for option B, RAN2 wait for RAN1 input on the detailed parameters.

Proposal 3: [Open issue RRC-2] for option A, one-bit simple indicator to enable applicability reporting is enough.

# Open issue RRC-3: UE data collection request

Open issue RRC-3: UE data collection request Issue description: For UE-side data collection, the details of UAI signaling for the UE start/stop request and especially the following aspects are not yet clarified: - how to refer in UAI to a preferred radio resource candidate configuration from a list of candidate configurations provided by NW - where/what the NW provides as candidate configurations - what the content of otherConfig for enabling UE data collection requests in UAI should be (e.g. just a flag, the list of candidate UE data collection configurations, etc.).

RAN1 agreed the following for data collection configurations.

Agreement For UE-sided model, for configuring the resource for data collection purpose, support - CSI-ReportConfig can used for configuring the resources for data collection purpose without CSI report. - One CSI-ResourceConfigId is configured for Set A. - One CSI-ResourceConfigId is configured for Set B. - Note: UE performs measurement on all resources - One or two associated IDs can be configured in CSI-ReportConfig - When Set B is equal or a subset of set A (i.e., NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set B is within the NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set A), one associated ID is configured, - Otherwise, one associated ID is configured for Set A and another one associated ID is configured for Set B - FFS: whether/how to support 'aperiodic' CSI RS Note: This is not related to whether/how to support delivery/transmission of the collected data for training for UE-sided model.

Observation 1: NW sends one or multiple CSI-ReportConfig without CSI report for data collection configuration.

Once NW provides candidate CSI-ReportConfig without CSI report, UE can determine which CSI-ReportConfig is preferred for own data collection.

Proposal 4: [Open issue RRC-3] UE refers the preferred candidate ID with CSI-ReportConfigId which is included in data collection configuration.

The next question is where/what the NW provides as candidate configurations. Given that UAI is used for UE to indicate the request of data collection configuration, it is natural that NW configuration is included in OtherConfig and only a list of CSI-ReportConfigId should be enough and actual CSI-ReportConfig can be configured by using existing structure.

DataCollectionPreferenceConfig-r19 :: = SEQUENCE {

candidateCSI-ReportConfigList SEQUENCE (SIZE (1..maxCandidate-r19)) OF CSI-ReportConfigId

}

# Open issue RRC-4: Activation of a periodic CSI report configuration upon change from inapplicable to applicable

Open issue RRC-4: Activation of a periodic CSI report configuration upon change from inapplicable to applicable Issue description: RAN2#129 agreed that for periodic CSI reporting, the UE autonomously activates the applicable functionality upon sending the applicability report via RRCReconfigurationComplete in Step 4: If option A is configured in Step 3, for periodic CSI reporting, the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5).. Further, RAN2#129bis agreed: 2 Upon receiving one or more full inference configuration(s) via RRCReconfiguration message, UE shall maintain all the full inference configuration(s) no matter the full inference configuration is applicable or inapplicable until the network releases it explicitly.. Thus, the UE will maintain a periodic CSI report configuration that is inapplicable, but it is unclear whether/how a periodic CSI report is activated upon a change of applicability, from inapplicable to applicable, since there is no legacy procedure to activate such a configuration (other than the initial sending of the RRC configuration). One way to solve this situation is by network implementation, namely the network can de-configure a periodic CSI report configuration after the UE initially reports it as inapplicable via RRCReconfigurationComplete.

In option A, if non-applicable periodic CSI report becomes applicable, UE would send UAI. However, it is not clear how NW “re”activate this periodic CSI report. There could be two possible approach to reactivate the periodic CSI report.

- Approach 1: NW release the periodic CSI report and add a new periodic CSI report. By the time UE receives RRC reconfiguration, UE would need to check the applicability as a new periodic CSI report is provided. The benefit of this approach would be based on existing mechanism.

- Approach 2: NW indicates to activate periodic CSI report. With this approach, the existing configuration can be maintained and what gNB configures is an additional bit to reactivate periodic CSI report.

Either approach is feasible but approach 1 causes unnecessary signaling overhead and UE operation to repeat applicability determination operation again. Therefore, approach 2 is more preferred.

Proposal 5: [Open issue RRC-4] A new indicator is defined to reactivate periodic CSI report.

# Open issue RRC-6: Handling of inference, applicability reporting and UE data collection preference configurations

Open issue RRC-6: Handling of inference, applicability reporting and UE data collection preference configurations when UE goes to RRC_IDLE/INACTIVE state and during RRCReestablishment Issue description: RAN2 made following agreements in RAN2#129bis meeting: Upon receiving one or more full inference configuration(s) via RRCReconfiguration message, UE shall maintain all the full inference configuration(s) no matter the full inference configuration is applicable or inapplicable until the network releases it explicitly.. It is not clear whether UE releases inference configurations when UE goes to RRC_IDLE/CONNECTED state or upon network configuration via RRCRelease, etc. Furthermore, handling of applicability configurations and UE data collection preference configuration (applicabilityReportConfig and dataCollectionPreferenceConfig in RRC running CR) during RRCReestablishment and in transition to RRC_INACTIVE state needs to be specified.

While transitioning to RRC_IDLE, UE normally releases all the configuration except the configurations related to some operations in the idle mode. As the inference configurations are not related to any idle mode operations, it is straight forward to release this configuration and no specific handling is needed.

Proposal 6: [Open issue RRC-6] UE releases inference configuration upon transitioning to RRC_IDLE (no specification impact).

During RRCReestablishment, new gNB and old gNB do not exchange the UE Assistance Information.  UE also normally releases the related configuration in the OtherConfig during RRCReestablishment procedure. A similar approach can be specified for the applicability configurations and UE data collection preference configuration.

Similarly, the UE normally releases configurations from OtherConfig while initiating RRC Resume procedure, and the applicability configurations and UE data collection preference configuration can also be handled in the same manner.

Proposal 7: [Open issue RRC-6] UE releases the applicability configurations and UE data collection preference configuration in the following cases:

- Initiation of RRC Resume Procedure.

- When RRC Reestablishment is transmitted.

# Open issue RRC-8: Coexistence between option A and option B

In order to merge or combine option A and option B, it is necessary to decide whether option A and option B can be simultaneously configured.

Technically, option A and option B can be used concurrently from signaling perspective. However, given that option A and option B would be decided considering the different network strategy/operation scenario e.g. multiple functionalities are supported or limited number of functionates is supported.

Therefore, there is not much motivation to configure option A and option B simultaneously. gNB configures CSI-ReportConfig to request applicability reporting (i.e. using option A) or gNB configures set(s) of inference related parameters to request applicability reporting (i.e. using option B). Mixed configuration should not be allowed e.g. gNB should not request some CSI-ReportConfigs and some sets of inference related parameters simultaneously for the same UE.

Proposal 8: [Open issue RRC-8] either option A or option B is configured for one UE i.e either option for all configured functionalities.

If we assume that set(s) of inference related parameters is configured in otherConfig. UE can assume that if set(s) of inference related parameters is configured, option B is used. Otherwise, option A is used.

Proposal 9: [Open issue RRC-8] option A or option B is implicitly indicated with the presence of a set of inference related parameters in otherConfig.

A remaining question is whether UE can also indicate applicability status for CSI-ReportConfig in option B in Step 5. One can consider that it is necessary for UE to indicate when a configured CSI-ReportConfig is non-applicable similar to option A. However, as long as the associated a set of inference related parameter is configured, the UE can still indicate the change of applicability status in UAI and gNB can be aware of changing of applicability status of the configured CSI-ReportConfig.

Proposal 10: [Open issue RRC-8] in option B, the UE doesn’t send applicability indication for a configured CSI-ReportConfig (i.e. UE sends applicability reporting only for a set of inference related parameters if configured).

The following shows an example of how option B is included based on the current applicabilityReportList provided by the CR rapporteur.

–	ApplicabilityReportList

The IE ApplicabilityReportList comprises information that the UE reports to gNB related to the applicability of the radio measurement prediction configurations at UE.

ApplicabilityReportList information element

-- ASN1START

-- TAG-APPLICABILITYREPORTLIST-START

ApplicabilityReportList-r19 ::=   SEQUENCE (SIZE (1..FFS)) OF ApplicabilityReport-r19

ApplicabilityReport-r19 ::=       SEQUENCE {

applicabilityCellId-r19               ServCellIndex,

applicabilityReportType  CHOICE {

applicabilityReportConfigIdList-r19      SEQUENCE (SIZE (1..maxNrofApplicabilityReports)) OF ApplicabilityReportConfigIdList-r19,

applicabilityReportConfigIdOptionBList-r19      SEQUENCE (SIZE (1..maxNrofApplicabilityReports)) OF ApplicabilityReportConfigIdBList-r19,

}

...

}

ApplicabilityReportConfigIdList-r19 ::=    SEQUENCE {

applicabilityReportConfigId-r19                CSI-ReportConfigId,

applicabilityStatus-r19                        ENUMERATED {true},

...

}

ApplicabilityReportConfigIdOptionBList-r19 ::=    SEQUENCE {

applicabilityReportConfigId-r19                inferenceParameterId,

applicabilityStatus-r19                        ENUMERATED {true},

...

}

-- TAG-APPLICABILITYREPORTLIST-STOP

-- ASN1STOP

# Open issue RRC-16: Associated ID

RAN1 introduced associated ID to ensure consistence between inference and training. RAN1 agreed about the need of associated ID and how to provide associate IDs as follows.

RAN1 #118 Working Assumption Regarding the associated ID for Rel-19, the UE assumes that NW-side additional conditions with the same associated ID are consistent at least within a cell - FFS: whether/how UE assumption can be applicable for multiple cells (including the feasibility study) RAN1 #120 For UE-sided model, in CSI-ReportConfig for inference - One or two associated IDs can be configured in CSI-ReportConfig - When Set B is equal or a subset of set A (i.e., NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set B is within the NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set A), one associated ID is configured, - Otherwise, one associated ID is configured for Set A and another one associated ID is configured for Set B - FFS: At least BM-Case 1, the applicability for 'aperiodic' CSI RS

One remaining issue is how NW determines associated IDs. It is our understanding that it should be discussed in RAN2 because it requires overall system operation in NW to decide the characteristics of associated ID.

First characteristic to decide regarding associated ID would be whether it is cell specific or multiple-cell specific. Main consideration would be the granularity of categorizing dataset for model training. If it is cell specific, it would mean that UE should categorize dataset per cell and per associated ID, which requires UE to accommodate a lot of models and consequently more complexity/burden in model training at UE side.

Observation 2: cell specific associated ID will require UE to train a huge number of models and consequently more complexity/burden in model training at UE side.

In addition, NW side additional conditions may not be different in each cell because it is more related to network deployment and antenna configuration. One model trained for a certain network deployment can be reused to multiple cells.

Observation 3: NW side addition condition is likely to be the same for multiple cells with the same NW configuration.

On the other hand, from NW point of view, cell specific ID may be more preferred if it is concerned that the limited number of associated IDs might reveal the network deployment configuration.

Observation 4: cell specific associated ID may be preferred if it is concerned that the limited number of associated IDs might reveal the network deployment configuration.

Considering pros and cons for each option, it would be desirable to provide flexibility to UE/NW implementation. That is, the signaling of associated ID should be defined to support both approaches i.e. cell specific ID and multiple cell-specific IDs.

Proposal 11: [Open issue RRC-16] RAN2 agree to support both cell specific ID and multiple cell-specific IDs for associated ID.

From signaling point of view, there could be two options to define associate ID to be applied for multiple cells.

- Area specific associated ID: associated ID(s) is provided with a certain area ID similar to system information area ID in NR. Different area ID can be used to differentiate the same associated ID. Therefore, the actual associated ID is the combination of associated ID and area ID. With this approach, the same associated ID can be applied across multiple cells with the same area ID. Allocation and mapping of area IDs to specific areas/cells should be left to implementation. At least, we can assume that area ID should be unique within a PLMN.

systemInformationAreaID Indicates the system information area that the cell belongs to, if any. Any SIB with areaScope within the SI is considered to belong to this systemInformationAreaID. The systemInformationAreaID is unique within a PLMN/SNPN.

- Globally unique associated IDs within PLMN: associated IDs are consistently unique as long as the cell belongs to the same PLMN. Allocation and mapping of associate IDs to specific NW sided additional conditions should be left to implementation.

The two options are quite similar from operation point of view i.e. requiring pre-allocation by operators to make it unique in PLMN. Area specific associated ID would require more specification efforts to update associated IDs with the received area ID. Nevertheless, both options are feasible from signaling point of view.

Proposal 12: [Open issue RRC-16] RAN2 discuss two options (area specific associated ID and globally unique associated IDs within PLMN) for associated IDs.

# Conclusion

In this document, we discussed some of RRC open issues from RAN2 post email discussion and propose the following points.

Proposal 1: [Open issue RRC-1] one bit to indicate two values (long term and short term) is introduced for cause of inapplicability.

Proposal 2: [Open issue RRC-2] for option B, RAN2 wait for RAN1 input on the detailed parameters.

Proposal 3: [Open issue RRC-2] for option A, one-bit simple indicator to enable applicability reporting is enough.

Proposal 4: [Open issue RRC-3] UE refers the preferred candidate ID with CSI-ReportConfigId which is included in data collection configuration.

Proposal 5: [Open issue RRC-4] A new indicator is defined to reactivate periodic CSI report.

Proposal 6: [Open issue RRC-6] UE releases inference configuration upon transitioning to RRC_IDLE (no specification impact).

Proposal 7: [Open issue RRC-6] UE releases the applicability configurations and UE data collection preference configuration in the following cases:

- Initiation of RRC Resume Procedure.

- When RRC Reestablishment is transmitted.

Proposal 8: [Open issue RRC-8] either option A or option B is configured for one UE i.e either option for all configured functionalities.

Proposal 9: [Open issue RRC-8] option A or option B is implicitly indicated with the presence of a set of inference related parameters in otherConfig.

Proposal 10: [Open issue RRC-8] in option B, the UE doesn’t send applicability indication for a configured CSI-ReportConfig (i.e. UE sends applicability reporting only for a set of inference related parameters if configured).

Proposal 11: [Open issue RRC-16] RAN2 agree to support both cell specific ID and multiple cell-specific IDs for associated ID.

Proposal 12: [Open issue RRC-16] RAN2 discuss two options (area specific associated ID and globally unique associated IDs within PLMN) for associated IDs.

# Reference

[1] R2-2504350, “RRC open issues AIML PHY”, Ericsson