3GPP TSG-RAN WG2 Meeting #130	R2-2503556

St.Julians, Malta, May 19th – 23th, 2025

Agenda item:	8.7.4.2

Source: 	Fujitsu

Title: 	Discussions on DSR enhancements

Document for:	Discussion and decision

Introduction

On DSR enhancements for XR, in RAN2#129 meeting, the following are agreed on DSR enhancements [1]:

Agreements on DSR enhancements - One extension bit (e.g. by redefining the reserved R bit) can be used to indicate whether a further pair of remaining time and buffer size information is present for the associated LCG in the enhanced DSR MAC CE. - FFS New DSR MAC CE will (always) be used when at least one LCG is configured with multiple thresholds. - We do not support truncated DSR nor fallback to legacy DSR in case of limited PUSCH grant size. - Different LCGs may be configured with different number of reporting thresholds. - If UE is configured to use R19 DSR, then any LCG with a triggering threshold shall be configured with at least one reporting threshold. - Triggering threshold is not used as a reporting threshold (but one of reporting thresholds can be configured to the same value as triggering threshold). - Do not support a configuration of an LCG without any triggering threshold but with DSR reporting threshold(s).

In RAN2#129bis meeting, there are following agreements [2] in RAN2#129bis meeting:

Agreements on DSR enhancements - To avoid the case that there is no delay-reporting data for DSR, at least one configured reporting threshold should be no lower than the DSR triggering threshold. This means we do not have to address “empty DSR” issue, but it is FFS whether we need to capture this restriction in specifications - During DSR data volume calculation the remaining time of retransmitted is not considered, i.e. it is always put either in the smallest configured or smallest reported threshold. FFS which one. - Clarify RAN2#128 agreement as “the UE may also support including non-delay-reporting data ahead of delay-reporting data for buffer size calculation of Rel-19 DSR, based on the capability indication” (the exact terminology to be discussed as part of CR review) - We will try to find a way to describe how the UE determines non-delay-reporting data ahead of delay-reporting data for delay-reporting data volume calculation. The aim is to have consistent UE behaviour to avoid fairness issues, but also consider different UE implementations.

In this contribution, we would like to discuss remaining open issues on DSR enhancements.

Discussion

Legacy DSR and enhanced DSR coexistence

In the RAN2#129 meeting, there is an FFS on whether the new DSR MAC CE will (always) be used when at least one LCG is configured with multiple thresholds.

In our understanding, we can always use a new DSR in case there is at least one LCG configured with multiple thresholds. For the LCG configured with multiple reporting thresholds but with only one pair of remaining time and buffer size be reported, or for the LCG configured with only one reporting threshold, the enhanced DSR format can include just one pair of remaining time information and buffer size information for this LCG. This is much simpler compared with determining whether legacy DSR or enhanced DSR to be reported based on buffer size or number of reporting threshold(s) of each LCG.

Observation 1: For the LCG configured with multiple reporting thresholds but with only one pair of remaining time and buffer size to be reported, or for the LCG configured with only one reporting threshold, the enhanced DSR format can include just one pair of remaining time information and buffer size information for this LCG.

Proposal 1: Enhanced DSR is always used in case there is at least one LCG configured with multiple reporting thresholds.

Control PDU and/or retransmission data

In RAN2#129 meeting, there are offline and online discussions on how to calculate the control PDU and retransmitted data in DSR reporting. 3 options are summarized as the result of offline discussions as following:

- Option 1. Both PDCP and RLC consider Control PDU and retransmitted data into the shortest reporting threshold included in the DSR, and discuss further about the issue of LCH only having the control PDU and retransmitted data at the next meeting.

- Option 2. Both PDCP and RLC consider Control PDU and retransmitted data into the shortest configured reporting threshold.

- Option 3. PDCP considers Control PDU and retransmitted data into the shortest reporting threshold included in the DSR, and discuss further about how RLC considers Control PDU and retransmitted data at the next meeting.

In RAN2#129bis meeting, how to calculate the remaining time of retransmitted data is discussed and RAN2 agrees that the remaining time of retransmitted is not considered during DSR data volume calculation, i.e. it is always put either in the smallest configured threshold or the smallest reported threshold. FFS which one.

That is to say, control PDUs and/or retransmitted data would be handled in the same way, i.e., always be considered into either the smallest reporting threshold in DSR (Option 1 above) or the smallest configured reporting threshold (Option 2 above).

In our understanding, Option 2 is simpler and is easier to be specified in PDCP and RLC specifications. However, Option 2 would consume two more bytes than Option 1 if there is no data to be reported in the smallest configured reporting threshold. From our perspective, we slightly prefer Option 2.

Proposal 2: Both PDCP and RLC consider Control PDU and/or retransmitted data into the shortest configured reporting threshold.

If Proposal 2 can be agreed, the following alternatives can be used to set the remaining time field for the shortest configured threshold in the DSR MAC CE:

o	Alt. 1: Set the remaining time field in DSR MAC CE as 0;

o	Alt. 2: Set the remaining time field in DSR MAC CE as the i-th dsr-ReportingThreshold with i=1 (i.e. the smallest reporting threshold);

o	Alt. 3: Leave it to UE implementation

In our understanding, Alt.1 using a reserved value of the remaining time field can indicate to the network about the case that there are only control PDUs and/or retransmitted data to be reported for the shortest configured reporting threshold, while Alt.2 or Alt.3 could not indicate this case. To be simple, we prefer Alt.1 if Proposal 2 can be agreed.

Proposal 3: The value of the remaining time field in the enhanced DSR MAC CE is set to 0, if there are only control PDUs and/or retransmitted data to be reported for the shortest configured reporting threshold of the LCG.

Conclusion

In this contribution, we have discussed the further details of the DSR enhancements. We have the following proposals:

Observation 1: For the LCG configured with multiple reporting thresholds but with only one pair of remaining time and buffer size to be reported, or for the LCG configured with only one reporting threshold, the enhanced DSR format can include just one pair of remaining time information and buffer size information for this LCG.

Proposal 1: Enhanced DSR is always used in case there is at least one LCG configured with multiple reporting thresholds.

Proposal 2: Both PDCP and RLC consider Control PDU and/or retransmitted data into the shortest configured reporting threshold.

Proposal 3: The value of the remaining time field in the enhanced DSR MAC CE is set to 0, if there are only control PDUs and/or retransmitted data to be reported for the shortest configured reporting threshold of the LCG.

References

- Report from session on R18 MBS, R18 QoE and R19 XR,	Session chair (Huawei), RAN2#129 meeting.

- R2-2502985, Report from session on R19 XR, Session chair (Huawei), RAN2#129bis meeting.