3GPP RAN WG2 Meeting #130	R2-2503562

Malta, Malta May 19th – 23rd, 2025

Agenda item:	8.1.3

Source:	LG Electronics Inc.

Title:	Remaining issue on NW side Data Collection

Document for:	Discussion and Decision

1. Introduction

During the RAN2 #129bis meeting, RAN2 reached the following agreements.

Agreements on Idle/Inactive and HO

1	Introduce 1-bit indication on whether to release or retain un-retrieved data in RRCReconfiguration during/before HO.  Source gNB decides whether the data should be kept.  The indication is provided in RRCReconfiguration (i.e. not in RRC Reconfiguration from target cell).   FFS signaling details.

2	Upon going to RRC_IDLE, RLF, or RRC_INACTIVE, UE discards any logged data

Agreements on availability indication

•Availability indication can be triggered due to:

- Full buffer being reached (if configured)

- Buffer threshold being reached (if configured).

- Low power (if configured)

•The UE send a UAI that indicates:

- Data is available

- Reason for trigger (full buffer, threshold)

- Low power indication

•The encoding of the data is available/UAI and the cause value is FFS

NOTE: it is up to UE Implementation how buffer threshold reached and low power is determined

Agreements on Data collection configuration

1	The measurement configuration of AI/ML data collection can configure measurements for multiple sets of resources and use cases (e.g. BM, Mobility, etc)

Agreements

1.	For temporal domain, the network is made aware whether there is a gap between two consecutive samples.   FFS amount of gap and whether this is implicit or explicit

2.	New SRB can be configured for NW-side data collection (with lower priority)

This contribution aims to specify the configuration and procedure involved in data collection for BM use case.

2. Discussion

Remaining open issues (First priority)

Open issue RRC-18: NW control on retaining logged data at HO

Issue description: The signaling details of the network control on how data should be retained at handover are FFS, based on the RAN2#129 bis agreement: Introduce 1-bit indication on whether to release or retain un-retrieved data in RRCReconfiguration during/before HO. Source gNB decides whether the data should be kept. The indication is provided in RRCReconfiguration (i.e. not in RRC Reconfiguration from target cell). FFS signaling details. The issue is captured as an editor’s note in the running CR, in clause 5.3.5.3 and 6.2.2. It the rapporteur’s view, it should be clarified Whether/how the 1-bit indication can be sent during or before HO, taking into account that the source gNB decides whether the data should be kept. From the rapporteur perspective, it is not possible for the source gNB to add the 1-bit indication during HO, in the same RRCReconfiguration that encapsulates the RRCReconfiguration from the target gNB. Thus, the rapporteur sees the following two possible solutions for this issue: - The source gNB sends the 1-bit indication to the UE before HO. - The source gNB decides if the 1-bit indication is needed and, if so, sends it to the target gNB, which includes it in the RRCReconfiguration sent to the UE during HO. No RAN3 impact is expected if the transmission of the 1-bit indication is limited to the case in which the target gNB is from the same vendor as the source gNB. Proposed resolution: It is suggested that companies provide contributions to the following meeting to resolve the issue.

Inter-gNB handover procedures (Figure 9.2.3.1-1 in TS 38.300)

In the legacy L3 handover procedure, the source gNB forwards the RRC configuration to the UE by forwarding the RRCReconfiguration message received from the target gNB, without adding any additional reconfiguration from the source gNB. As a result, the UE transitions its RRC connection to the target gNB and responds with the RRCReconfigurationComplete message. This process presents a challenge, as the 1-bit indication regarding whether to release or retain un-retrieved data cannot be included in the source gNB's configuration during the handover.

To address this issue, a solution proposed during an email discussion [1] involves the source gNB sending the 1-bit indication to the UE prior to the handover. In this scenario, the source gNB would transmit the 1-bit indication only after receiving the HANDOVER REQUEST ACKNOWLEDGE from the target gNB in step 2. This ensures that the indiation is set only when the handover to the target gNB is confirmed. However, this approach can lead to potential handover delay since the 1-bit indication must be sent before the handover command, potentially delaying handover execution.

Observation 1. [Open Issue RRC-18] In cases where the source gNB sends the 1-bit indication to the UE before HO, it can lead to handover delay since the 1-bit indication must be sent before the handover command, potentially delaying handover execution.

To avoid this issue, 1-bit indication should be sent during HO, i.e., 1-bit indication is included in the RRCReconfiguration message provided by the target gNB.

Proposal 1. [Open Issue RRC-18] 1-bit indication is sent during HO, i.e., 1-bit indication is included in the RRCReconfiguration message provided by target gNB

To set 1-bit indication in the RRCReconfiguration message provided by target gNB, there are three approaches:

- Option 1: Network implementation: This option may have constraints, such as requiring that the target gNB and source gNB be from the same vendor. Note that RAN2 has not agreed on the restriction that logged data is retained only for same-vendor scenarios, thus this solution can be excluded.

- Option 2: A blind request from the target gNB, if data collection is supported by the target gNB: This approach has the disadvantage that the 1-bit indication may be unnecessarily set for UEs that do not have logging configuration/logged data during the handover.

- Option 3: Including the indication within the HandoverPreparationInformation: In this case, during the handover preparation phase (e.g., in step 2), the source gNB can inform the target gNB of the logging-related information. Then, the 1-bit indication can then be set for the relevant UEs from the target gNB, thereby preventing unnecessary signaling compared to Option 2. Additionally, if the logging-related information can be included within the HandoverPreparationInformation, there is no impact on RAN3.

Observation 2. [Open Issue RRC-18] For the cases where the target gNB includes the 1-bit indication in the RRCReconfiguration message during HO, there are three approaches:

- Option 1: Network implementation: This option has constraints, such as requiring that the target gNB and source gNB be from the same vendor

- Option 2:3 A blind request from the target gNB, if data collection is supported by the target gNB: This option has the disadvantages that the 1-bit indication may be unnecessarily set for UEs that do not have logging configuration/logged data

- Option 3: Including the indication within the HandoverPreparationInformation: Since the source gNB can inform the target gNB of the logging-related information, the source gNB can set 1-bit indication for the relevant UEs without RAN3 impact

In conclusion, Option 3 is preferred as it effectively addresses the concerns associated with the other options while ensuring minimal impact on the network architecture. Additionally, it is essential to discuss the logging-related information pertinent to this option.

The source gNB cannot always guarantee whether the UE has available logged data. This uncertainty arises because the UE can indicate data availability based on its memory status, either when it is full or when it exceeds a certain threshold. Consequently, instead of providing direct information about the availability of logged data, the source gNB can inform whether it has set the logging configuration.

This approach allows the target gNB to set a 1-bit indication for relevant UEs that may potentially have logged data. If the UE does possess logged data, it can subsequently inform the target gNB of its availability through the RRCReconfigurationComplete message.

Proposal 2: [Open Issue RRC-18] The target gNB sets a 1-bit indication for the relevant UEs based on the HandoverPreparationInformation received from the source gNB (Option 3). This HandoverPreparationInformation includes an indication whether the source gNB has set a logging configuration, thereby maintaining awareness of potential logged data availability

Open issue RRC-19: Reporting assistance information related to logged measurements

Issue description: It is not yet clear what the otherConfig should contain to enable the UE to report assistance information via UAI, related to logging of radio measurements. For instance, should the low power, buffer full, and buffer threshold reached indications be all configured with a single bit, or should the configurations be separated? This issue is related to the RAN2#129bis agreement: Agreements on availability indication - Availability indication can be triggered due to: - Full buffer being reached (if configured) - Buffer threshold being reached (if configured). - Low power (if configured) - The UE send a UAI that indicates: - Data is available - Reason for trigger (full buffer, threshold) - Low power indication - The encoding of the data is available/UAI and the cause value is FFS NOTE: it is up to UE Implementation how buffer threshold reached and low power is determined. The issue is captured as an editor’s notes in the running CR, in clause 5.3.5.9, 5.7.4.3, 6.2.2, and 6.3.4. Proposed resolution: It is suggested that companies provide contributions to the following meeting to resolve the issue.

The expected network behavior differs depending on whether the indication relates to buffer status or to a low power state. For example, in the case of buffer-related indications, the network may decide whether or not to retrieve logged data from the UE. In contrast, for low power indications, the network may consider releasing the logging configuration.

Furthermore, even among buffer-related indications, different decisions may be made depending on whether the buffer is full or a predefined threshold has been reached—for instance, whether the network should retrieve the data immediately or wait.

Given these differences in network requirements and decision-making depending on the trigger, it is appropriate that each type of indication be configured separately.

Proposal 3. [Open issue RRC-19] The otherConfig configures separate bits for low power, buffer full, and buffer threshold reached indications

Open issue RRC-20: Further procedures for UE assistance information related to logging

Issue description: It has not yet been discussed whether further procedures for UE reporting assistance information related to logging is need. Such further procedures may be, e.g. prohibit timers, indication that battery state is not low any longer, indication that the memory is not full any longer, etc. The issue is captured as an editor’s note in the running CR, in clause 5.7.4.2 and 5.7.4.3. Proposed resolution: It is suggested that companies provide contributions to the following meeting to resolve the issue.

Regarding the need of a prohibit timer, the purpose of such a timer would be to prevent immediate re-triggering of assistance information reporting.

For buffer-related indications, the triggering condition is clearly defined (e.g., full buffer or threshold reached), and a one-shot indication when the condition is met is sufficient. Similarly, for low power state, the entry condition is determined based on well-defined criteria in UE implementation. As such, a one-time indication upon entering the low power state adequately reflects the UE’s status. Therefore, no further mechanism such as a prohibit timer is required.

Proposal 4. [Open issue RRC-20] UAI related to buffer status or low power state is triggered only once when specific conditions are met (e.g., buffer full/threshold, and low power state). A prohibit timer is not necessary for UAI related to buffer status or low power state

Regarding the need of an indication for recovery from low power state, there is no strong motivation for the UE to explicitly report that it has exited the low power state. The initial low power indication is intended to inform the network of the UE's constrained power condition, which may prompt the network to consider releasing the logging configuration.

Once the network receives this indication, any decision to resume or reconfigure logging should be left to network implementation, which may, for instance, be based on elapsed time or other internal policies. Moreover, the low power entry condition is determined by the UE implementation based on specific thresholds or criteria, and so is the recovery. Since these internal conditions are not standardized and can vary across UEs, requiring a standardized recovery indication from the UE would add unnecessary complexity.

Therefore, no additional signaling from the UE is required when the low power issue is resolved.

Proposal 5. [Open issue RRC-20] No additional signaling from the UE is required when the low power issue is resolved

Regarding the need of an indication for recovery from buffer issue, based on RAN2 agreement, when a UE reaches its logging buffer limit, the UE is allowed to stop logging activities. However, the logging configuration may be retained so that logging can resume later without additional configuration signaling.

A similar behavior is observed in legacy MDT operations, where logging can be paused due to IDC issues. In such cases, once the issue is resolved, the UE autonomously resumes logging without requiring explicit instruction or reconfiguration from the network.

By analogy, when the buffer issue is resolved, e.g., by memory being cleared or space becoming available, the UE may autonomously resume logging according to the retained configuration, at least for periodic CSI-RS logging. This approach avoids unnecessary signaling while maintaining continuity in data collection.

Therefore, an explicit indication from the UE about buffer recovery is not necessary, as the network does not require this information to re-initiate logging.

Proposal 6. [Open issue RRC-20] No additional signaling from the UE is required when the buffer full issue is resolved

Proposal 7. [Open issue RRC-20] When the buffer limit issue is resolved, UE resumes logging at least for periodic CSI-RS according to the retained logging configuration, if any

Open issue RRC-22: RAN1 involvement for logged data for NW-side and UE-side data collection

Issue description: Procedures for performing the L1 measurement results are captured in RAN1 specification, i.e. TS 38.214. Rapporteur assumes that the same should be applied for the case of radio measurements logging for the NW-side data collection and UE-side data collection. Hence RAN1 involvement is expected to capture procedures related to the radio measurements logging, e.g. in the UE variable VarCSI-LogMeasReport for the case of NW-side data collection. From the rapporteur’s perspective, an LS should be sent to RAN1. The issue is captured as an editor’s note in the running CR, in clause 5.7.10.3. Proposed resolution: It is suggested that companies provide contributions to the following meeting to resolve the issue.

We agree with the rapporteur’s assumption that the same framework should apply to radio measurements logging for both network-side and UE-side data collection. However, there are concerns about the placement of logging procedures in the physical layer specifications.

Logging is not limited to periodic physical layer measurements but may also include event-based triggers, such as those based on higher-layer metrics (e.g., L3 cell quality). These conditions are better suited for control and specification at the RRC layer, where network configuration and signaling are managed.

Observation 3. [Open issue RRC-22] Logging is not limited to periodic physical layer measurements but may also include event-based triggers, such as those based on higher-layer metrics (e.g., L3 cell quality). These conditions are better suited for control and specification at the RRC layer, where network configuration and signaling are managed.

Therefore, while physical measurement definitions should remain in the RAN1 specifications, logging conditions and control procedures, particularly those related to when and how logging is triggered, should be specified in the RRC layer.

Proposal 8. [Open issue RRC-22] Logging conditions and control procedures, particularly those related to when and how logging is triggered, should be specified in the RRC layer

Open issue RRC-23: Cell ID stored with logged data for NW-side data collection

Issue description: It has not been clarified what type of cell ID the UE needs to log along with the logged data, in order to unambiguously identify the cell in which the UE performed the data logging, e.g. CGI, PCI-ARFCN etc. The issue is captured as an editor’s note in the running CR, in clause 6.2.2. Proposed resolution: It is suggested that companies provide contributions to the following meeting to resolve the issue.

To ensure that logged data collected by the UE for NW sided data collection can be accurately associated with the serving cell in which the data was collected, the type of cell identifier to be logged needs to be clearly specified. Especially in scenarios such as OAM training, the ability to unambiguously identify the cell is essential. In legacy SON/MDT operations, this has typically been achieved using the CGI, which includes the PLMN ID and the cell identity and allows for a globally unique cell reference.

While other identifiers such as PCI and ARFCN are useful for measurement correlation, they do not uniquely identify a cell and may lead to ambiguity in certain deployments (e.g., PCI reuse).

Proposal 9. [Open issue RRC-23] Logged data includes the CGI along with the measurement results

Open issue RRC-24: Where to include the logging configuration from NW to UE

Issue description: It is FFS whether the logging configuration is included in the CSI framework (whether in CSI-ReportConfig or directly under CSI-MeasConfig) or at L3. This issue was discussed in RAN2#129bis and the following outcome was captured: Next meeting proponents should work together and bring complete proposals to show specification impact and consider future use cases.. The issue is captured as an editor’s note in the running CR, in clause 6.3.2. Configuration details of events for event-based logging configuration, e.g. whether we reuse configuration in reportConfig, can be included. Proposed resolution: It is suggested that companies provide contributions to the following meeting to resolve the issue.

According to RAN1 agreements, CSI-ReportConfig can be reused for UE-side model data collection without triggering CSI reports. If NW-side and UE-side data collection were to rely on separate CSI resource configurations, it would significantly increase both implementation complexity and configuration management overhead. This is especially true as CSI-related use cases expand, leading to scalability challenges. Moreover, in the case of semi-persistent CSI-RS, measurement can be activated or deactivated using existing MAC CE or DCI procedures. If logging configuration is implemented within the CSI framework, it can enable the network to dynamically manage semi-persistent CSI-RS measurements for data logging purposes without introducing new procedures.

Observation 4. [Open issue RRC-24] If NW-side and UE-side data collection were to rely on separate CSI resource configurations, it would significantly increase both implementation complexity and configuration management overhead. This is especially true as CSI-related use cases expand, leading to scalability challenges.

Therefore, implementing logging within the existing CSI framework minimizes specification impact and supports consistent system behavior.

Proposal 10. [Open issue RRC-24] Logging configuration is implemented within the CSI framework

To have a unified data collection solution for the UE and NW sided data collection, the CSI-ReportConfig without CSI reporting can be linked to a CSI logging config (i.e., CSI-LoggedMeasurementConfig) as follows.

CSI-LoggedMeasurementConfig-r19 ::=          SEQUENCE {

csi-LoggedMeasurementConfigId-r19         CSI-LoggedMeasurementConfigId-r19,

csi-LoggedResourceConfig-r19              CSI-ReportConfigIdCSI-ResourceConfigId,

eventTriggeredConfig-r19                  EventTriggeredConfig-r19                    OPTIONAL, --Need R

...

}

Proposal 11. [Open issue RRC-24] To set CSI-ReportConfigId associated with logging operation to the CSI logging config (i.e., CSI-LoggedMeasurementConfig)

For CSI-ReportConfig without CSI reporting, a way to prevent reporting is needed. To do so, a new reportConfigType-r19 with “none” can be introduced to ignore the legacy reportConfigType without suffix (mandatory config), as in Appendix 4.1. In this case, UE can proceed with logging, ignoring the related report resource settings (e.g. reportSlotConfig, pucch-CSI-ResourceList).

CSI-ReportConfig ::=                SEQUENCE {

reportConfigId                          CSI-ReportConfigId,

carrier                                 ServCellIndex                   OPTIONAL,   -- Need S

resourcesForChannelMeasurement          CSI-ResourceConfigId,

csi-IM-ResourcesForInterference         CSI-ResourceConfigId            OPTIONAL,   -- Need R

nzp-CSI-RS-ResourcesForInterference     CSI-ResourceConfigId            OPTIONAL,   -- Need R

reportConfigType                        CHOICE {

periodic                                SEQUENCE {

reportSlotConfig                        CSI-ReportPeriodicityAndOffset,

pucch-CSI-ResourceList                  SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource

},

semiPersistentOnPUCCH                   SEQUENCE {

reportSlotConfig                        CSI-ReportPeriodicityAndOffset,

pucch-CSI-ResourceList                  SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource

},

semiPersistentOnPUSCH                   SEQUENCE {

reportSlotConfig                        ENUMERATED {sl5, sl10, sl20, sl40, sl80, sl160, sl320},

reportSlotOffsetList                SEQUENCE (SIZE (1.. maxNrofUL-Allocations)) OF INTEGER(0..32),

p0alpha                                 P0-PUSCH-AlphaSetId

},

aperiodic                               SEQUENCE {

reportSlotOffsetList                SEQUENCE (SIZE (1..maxNrofUL-Allocations)) OF INTEGER(0..32)

}

},

…skipped…

[[

reportConfigType-v19                CHOICE {

none                                    NULL,

...,

}

]]

Proposal 12. [Open issue RRC-24] To introduce new reportConfigType-r19 with “none” in the CSI-ReportConfig to prevent CSI reporting

Regarding L3 measurement event configuration, the event-related configuration (i.e., eventTriggeredConfig) can link to a measId associated with a specific measurement object/report.

CSI-LoggedMeasurementConfig-r19 ::=          SEQUENCE {

csi-LoggedMeasurementConfigId-r19         CSI-LoggedMeasurementConfigId-r19,

csi-LoggedResourceConfig-r19              CSI-ResourceConfigId,

eventTriggeredConfig-r19                  MeasIdEventTriggeredConfig-r19                    OPTIONAL, --Need R

...

}

EventTriggeredConfig-r19 ::=     SEQUENCE {

FFS

}

Since the measurement result for the corresponding measId should not trigger an actual report, a separate report type for logging purposes needs to be considered. The separate report type should not include report related configuration (e.g., report interval, report amount, etc). When considering future extensions for mobility-related use cases, introducing a new event for event-based logging may be a suitable approach.

Observation 4. [Open issue RRC-24] When considering future extensions for mobility-related use cases, introducing a new event for event-based logging may be a suitable approach

ReportConfigNR ::=                          SEQUENCE {

reportType                                  CHOICE {

periodical                                  PeriodicalReportConfig,

eventTriggered                              EventTriggerConfig,

...,

reportCGI                                   ReportCGI,

reportSFTD                                  ReportSFTD-NR,

condTriggerConfig-r16                       CondTriggerConfig-r16,

cli-Periodical-r16                          CLI-PeriodicalReportConfig-r16,

cli-EventTriggered-r16                      CLI-EventTriggerConfig-r16,

rxTxPeriodical-r17                          RxTxPeriodical-r17,

reportOnScellActivation-r18                 ReportOnScellActivation-r18,

eventTriggeredLogging-r19                   EventTriggerConfigLogging-r19

}

}

EventTriggerConfigLogging-r19 ::=                      SEQUENCE {

logEventId                                     CHOICE {

logEventA1                                     SEQUENCE {

a1-Threshold                                MeasTriggerQuantity,

reportOnLeave                               BOOLEAN,

hysteresis                                  Hysteresis,

timeToTrigger                               TimeToTrigger

},

logEeventA2                                     SEQUENCE {

a2-Threshold                                MeasTriggerQuantity,

reportOnLeave                               BOOLEAN,

hysteresis                                  Hysteresis,

timeToTrigger                               TimeToTrigger

},

...

},

...

}

Proposal 13. [Open issue RRC-24] For event-based logging, MeasID is linked to event related configuration (i.e., eventTriggeredConfig)

Proposal 14. [Open issue RRC-24] If P18 is agreeable, new reportType is introduced for logging purpose

Open issue RRC-25: Dynamic activation/deactivation of data collection configurations for logging

Issue description: RAN2#127 made the following agreement regarding NW-side data collection: 1 As the baseline approach, the UE receives the measurement configuration for AI/ML-enabled features/FGs for data collection and logging of measurements. The network can explicitly configure the UE whether the corresponding data collection and logging (if supported) should be immediately started. FFS if multiple configurations can be provided to the UE. FFS if dynamic activation/deactivation is support.. The second FFS on dynamic activation/deactivation has not yet been addressed, as also pointed out in the stage-2 running CR draft. Proposed resolution: It is suggested that companies provide contributions to the following meeting to resolve the issue.

Open issue RRC-30: Semi-persistent resources for data collection

Issue description: RAN2 excluded usage of aperiodic CSI resource for data collection, but it is still unclear whether semi-persistent resources are needed for this. Proposed resolution: It is suggested that companies provide contributions to resolve this issue later.

In general, periodic CSI-RS may be cell-specific, while UE-dedicated CSI-RS, particularly in scenarios requiring beam refinement or per-UE adaptation, may rely on semi-persistent configurations. In such cases, supporting logging on semi-persistent CSI-RS may be necessary to enable data collection for training purposes.

Although the activation mechanism differs (i.e., semi-persistent CSI-RS requires explicit activation), once activated, the reporting behavior follows a periodic pattern. As a result, the logging or reporting burden imposed by semi-persistent CSI-RS is not significantly different from that of periodic CSI-RS when used for training data collection. Therefore, semi-persistent CSI-RS resource should be considered for inclusion in data collection frameworks.

Observation 5. [Open issue RRC-25/30] In general, periodic CSI-RS may be cell-specific, while UE-dedicated CSI-RS, particularly in scenarios requiring beam refinement or per-UE adaptation, may rely on semi-persistent configurations. In such cases, supporting logging on semi-persistent CSI-RS may be necessary to enable data collection for training purposes

Proposal 15. [Open issue RRC-25/30] For measurement/logging for semi-persistent CSI-RS, dynamic activation/deactivation mechanism is supported

Open issue RRC-27: How to set logging periodicity

Issue description: Regardless of the chosen logging framework, further discussion is needed on how to set the logging periodicity for each logging RS. To provide flexibility in the logging timing, two options could be considered for logging periodicity: (i) aligning with the RS transmission periodicity, or (ii) introducing an optional logging interval setting. Proposed resolution: It is suggested that companies provide contributions to the following meeting to resolve the issue.

In legacy MDT operations, when periodic reporting is configured, the UE logs data at specific logging intervals. Similarly, for CSI-RS-based logging, it is desirable to support a configurable logging interval that allows the network to control how frequently the UE logs the RS measurement results. If the logging interval is not configurable and logging is instead performed at every RS transmission, this could lead to unnecessarily frequent data collection, increasing UE processing burden and memory usage without proportional benefit.

To provide flexibility in logging behavior, two options can be considered:

- Logging aligned with the CSI-RS transmission periodicity;

- Logging based on an optionally configured logging interval, which may be longer than the RS transmission period.

If no logging interval is explicitly configured, the default behavior can fall back to RS transmission periodicity.

Proposal 16. [Open issue RRC-27] The UE performs logging at the logging interval defined in the logging configuration. If no logging interval is configured, the UE shall perform logging based on the RS resource transmission periodicity.

Open issue RRC-29: Whether data availability indication should be sent when the UE has data below the threshold and low power state is sent, and what cause should be included then

Whether data availability indication should be sent when the UE has data and low power state is sent and what cause should be included then. The UE may have some data when indicating low power state. Even though this data volume may be lower than full buffer/threshold, it is still useful to let the network know about that so that the data can be fetched. Proposed resolution: It is suggested that companies provide contributions to the following meeting to resolve the issue.

A issue has raised whether a data availability indication should be sent when the UE is in a low power state and has some logged data, even if the buffer is not full or a threshold has not been reached. This is to allow the network to retrieve any useful data before the logging configuration is released.

However, the primary purpose of the low power state indication is to support the network's decision on whether to release the logging configuration based on the UE's power conditions, rather than to signal data availability. Moreover, even without an explicit data availability indication, the network can still take informed actions: it may estimate the potential presence of logged data based on the elapsed time since logging was configured, or it may issue an on-demand request to retrieve data, to which the UE can respond if data is available. Given these considerations, it is concluded that a separate data availability indication beyond the existing buffer-related and low power state indications is unnecessary.

Proposal 17. [Open issue RRC-29] No separate data availability indication is included in UE assistance information related to logged data

Remaining Open issues (Second priority)

Open issue RRC-31: The release of logged AIML data in UE’s buffer

Issue description: RAN2 needs to discuss and capture in the spec when the logged data for AIML in UE buffer will be released. In addition to the retain/release of logged data during HO covered in Open issue RRC-7, the following can be also considered: - Power off or deregistration - 48 hrs after the release of logged measurement configuration - Explicit indication from the serving gNB. Proposed resolution: It is suggested that companies provide contributions to resolve this issue later.

If the power-off or deregistration happened or the valid time for the logged data has expired, the UE should consider the logged data as no longer valid and clear it for the next logging operation. The conditions for releasing logged data can be similar with the existing logged MDT operation.

Proposal 18. [Open issue RRC-31] UE releases logged data under the following conditions:

- upon power off

- upon deregistration

- upon a certain time later, FFS length of timer (like 48-hour timer in logged MDT)

- upon successful delivery of the UEInformationResponse message (w/ logged data)

However, when a new logging configuration is set, the behavior may differ from legacy logged MDT due to configuration differences.

In legacy logged MDT operation, the network sets logging configuration within a separate RRC message, i.e., LoggedMeasurementConfiguration message. Upon receiving the LoggedMeasurementConfiguration message, the UE shall discard the logged measurement configuration as well as the logged measurement information.

In contrast, logging configuration for AI/ML data collection is included in the RRCReconfiguration message and typically uses a list-based structure (ToAddModList and ToReleaseList). Since delta configuration is possible, it is inefficient to discard all logged data unconditionally.

Therefore, for AI/ML-oriented data collection, a differentiated approach can be applied to ensure efficient data retention and avoid unnecessary loss of valid training data. For example, the UE can retain any logged data that is not associated with configuration IDs in either the ToAddModList or the ToReleaseList.

Observation 6. [Open issue RRC-31] If the logging configuration is set with list-based structure (i.e., ToAddModList and ToReleaseList, delta configuration is possible. A differentiated approach can be applied to ensure efficient data retention and avoid unnecessary loss of valid training data.

Proposal 19. [Open issue RRC-31] Upon receiving a new configuration, the UE discards only the logged data that corresponds to the configuration IDs included in the ToAddModList or ToReleaseList

- e.g., If a configuration ID in the ToAddModList matches an existing logged data entry, the UE discards the corresponding logged data

- e.g., If a configuration ID in the ToReleaseList matches an existing logged data entry, the UE discards the associated logged data

- e.g., The UE retains any logged data that is not associated with configuration IDs in either the ToAddModList or the ToReleaseList

Open issue RRC-35: Release/retain un-retrieved data upon inter-RAT handover

RAN2 agreed to Introduce 1-bit indication on whether to release or retain un-retrieved data in RRCReconfiguration during/before HO. It needs to be discussed whether this is applicable for inter-RAT handover also. Since it is agreed that UE releases the logged L1 measurements for AI/ML while moving to RRC_IDLE/RRC_INACTIVE and RLF, if this agreement is applied for inter-RAT HO, there will be changes in LTE for releasing AI/ML measurements in LTE RLF, moving to LTE idle etc. Moreover, it is not possible to retrieve the data by eNB. So our preference is to always discard logged L1 measurements during inter-RAT handover without considering the indication.

A company proposed that, during inter-RAT handover, the UE should discard logged data regardless of the 1-bit indication. However, if the gNB releases the logging configuration prior to initiating an inter-RAT handover, the UE is allowed to discard the logged data accordingly. Therefore, the retention or release of logged data can be handled purely through network implementation without requiring any spec impact. This approach ensures consistent and simplified behavior for inter-RAT scenarios, while avoiding unnecessary procedural complexity.

Proposal 20. [Open issue RRC-35] UE releases the logged data before inter-RAT HO by NW implementation, i.e., the gNB releases logging configuration before inter-RAT HO

Open issue RRC-33: Whether separate user consent for gNB centric training is needed

Issue description: It can be discussed whether a separate user consent is required for gNB-centric training, as it may differ from the conventional MDT user consent. To address the potential impact on gNB and OAM regarding data collection, it may be necessary to send an LS to RAN3 and SA5, including agreements related to logging. Proposed resolution: It is suggested that companies provide contributions to resolve the issue later

Open issue RRC-36: How data is forwarded to OAM or source gNB after HO

Issue description: RAN2 agreed that target gNB can fetch data collected in the source gNB after HO. We need to decide how this data is forwarded to OAM or source gNB, e.g. via inter-node RRC message or in some other way. RAN3 involvement may be needed. Proposed resolution: It is suggested that companies provide contributions to resolve the issue later.

As identified by multiple companies, further coordination with RAN3 and SA5 may be required to ensure proper alignment of procedures and responsibilities in NW-sided logged data collection for AI/ML purposes. Therefore, the agreements reached in RAN2 regarding logged data collection, especially those that may impact UE consent handling, inter-gNB communication, or OAM integration, should be communicated to RAN3 and SA5 via an LS. This will help ensure that any procedural or architectural impacts are properly considered in the relevant WGs.

Proposal 21. [Open issue RRC-33/36] Send LS to RAN3 and SA5 to inform them of the agreements made regarding NW-sided logged data collection for AI/ML training

3. Conclusion

Remaining open issues (First priority)

Open issue RRC-18: NW control on retaining logged data at HO

Observation 1. [Open Issue RRC-18] In cases where the source gNB sends the 1-bit indication to the UE before HO, it can lead to handover delay since the 1-bit indication must be sent before the handover command, potentially delaying handover execution.

Proposal 1. [Open Issue RRC-18] 1-bit indication is sent during HO, i.e., 1-bit indication is included in the RRCReconfiguration message provided by target gNB

Observation 2. [Open Issue RRC-18] For the cases where the target gNB includes the 1-bit indication in the RRCReconfiguration message during HO, there are three approaches:

- Option 1: Network implementation: This option has constraints, such as requiring that the target gNB and source gNB be from the same vendor

- Option 2:3 A blind request from the target gNB, if data collection is supported by the target gNB: This option has the disadvantages that the 1-bit indication may be unnecessarily set for UEs that do not have logging configuration/logged data

- Option 3: Including the indication within the HandoverPreparationInformation: Since the source gNB can inform the target gNB of the logging-related information, the source gNB can set 1-bit indication for the relevant UEs without RAN3 impact

Proposal 2: [Open Issue RRC-18] The target gNB sets a 1-bit indication for the relevant UEs based on the HandoverPreparationInformation received from the source gNB (Option 3). This HandoverPreparationInformation includes an indication whether the source gNB has set a logging configuration, thereby maintaining awareness of potential logged data availability

Open issue RRC-19: Reporting assistance information related to logged measurements

Proposal 3. [Open issue RRC-19] The otherConfig configures separate bits for low power, buffer full, and buffer threshold reached indications

Open issue RRC-20: Further procedures for UE assistance information related to logging

Proposal 4. [Open issue RRC-20] UAI related to buffer status or low power state is triggered only once when specific conditions are met (e.g., buffer full/threshold, and low power state). A prohibit timer is not necessary for UAI related to buffer status or low power state

Proposal 5. [Open issue RRC-20] No additional signaling from the UE is required when the low power issue is resolved

Proposal 6. [Open issue RRC-20] No additional signaling from the UE is required when the buffer full issue is resolved

Proposal 7. [Open issue RRC-20] When the buffer limit issue is resolved, UE resumes logging at least for periodic CSI-RS according to the retained logging configuration, if any

Open issue RRC-22: RAN1 involvement for logged data for NW-side and UE-side data collection

Observation 3. [Open issue RRC-22] Logging is not limited to periodic physical layer measurements but may also include event-based triggers, such as those based on higher-layer metrics (e.g., L3 cell quality). These conditions are better suited for control and specification at the RRC layer, where network configuration and signaling are managed.

Proposal 8. [Open issue RRC-22] Logging conditions and control procedures, particularly those related to when and how logging is triggered, should be specified in the RRC layer

Open issue RRC-23: Cell ID stored with logged data for NW-side data collection

Proposal 9. [Open issue RRC-23] Logged data includes the CGI along with the measurement results

Open issue RRC-24: Where to include the logging configuration from NW to UE

Observation 4. [Open issue RRC-24] If NW-side and UE-side data collection were to rely on separate CSI resource configurations, it would significantly increase both implementation complexity and configuration management overhead. This is especially true as CSI-related use cases expand, leading to scalability challenges.

Proposal 10. [Open issue RRC-24] Logging configuration is implemented within the CSI framework

Proposal 11. [Open issue RRC-24] To set CSI-ReportConfigId associated with logging operation to the CSI logging config (i.e., CSI-LoggedMeasurementConfig)

Proposal 12. [Open issue RRC-24] To introduce new reportConfigType-r19 with “none” in the CSI-ReportConfig to prevent CSI reporting

Observation 4. [Open issue RRC-24] When considering future extensions for mobility-related use cases, introducing a new event for event-based logging may be a suitable approach

Proposal 13. [Open issue RRC-24] For event-based logging, MeasID is linked to event related configuration (i.e., eventTriggeredConfig)

Proposal 14. [Open issue RRC-24] If P18 is agreeable, new reportType is introduced for logging purpose

Open issue RRC-25: Dynamic activation/deactivation of data collection configurations for logging

Open issue RRC-30: Semi-persistent resources for data collection

Observation 5. [Open issue RRC-25/30] In general, periodic CSI-RS may be cell-specific, while UE-dedicated CSI-RS, particularly in scenarios requiring beam refinement or per-UE adaptation, may rely on semi-persistent configurations. In such cases, supporting logging on semi-persistent CSI-RS may be necessary to enable data collection for training purposes

Proposal 15. [Open issue RRC-25/30] For measurement/logging for semi-persistent CSI-RS, dynamic activation/deactivation mechanism is supported

Open issue RRC-27: How to set logging periodicity

Proposal 16. [Open issue RRC-27] The UE performs logging at the logging interval defined in the logging configuration. If no logging interval is configured, the UE shall perform logging based on the RS resource transmission periodicity.

Open issue RRC-29: Whether data availability indication should be sent when the UE has data below the threshold and low power state is sent, and what cause should be included then

Proposal 17. [Open issue RRC-29] No separate data availability indication is included in UE assistance information related to logged data

Remaining Open issues (Second priority)

Open issue RRC-31: The release of logged AIML data in UE’s buffer

Proposal 18. [Open issue RRC-31] UE releases logged data under the following conditions:

- upon power off

- upon deregistration

- upon a certain time later, FFS length of timer (like 48-hour timer in logged MDT)

- upon successful delivery of the UEInformationResponse message (w/ logged data)

Observation 6. [Open issue RRC-31] If the logging configuration is set with list-based structure (i.e., ToAddModList and ToReleaseList, delta configuration is possible. A differentiated approach can be applied to ensure efficient data retention and avoid unnecessary loss of valid training data.

Proposal 19. [Open issue RRC-31] Upon receiving a new configuration, the UE discards only the logged data that corresponds to the configuration IDs included in the ToAddModList or ToReleaseList

- e.g., If a configuration ID in the ToAddModList matches an existing logged data entry, the UE discards the corresponding logged data

- e.g., If a configuration ID in the ToReleaseList matches an existing logged data entry, the UE discards the associated logged data

- e.g., The UE retains any logged data that is not associated with configuration IDs in either the ToAddModList or the ToReleaseList

Open issue RRC-35: Release/retain un-retrieved data upon inter-RAT handover

Proposal 20. [Open issue RRC-35] UE releases the logged data before inter-RAT HO by NW implementation, i.e., the gNB releases logging configuration before inter-RAT HO

Open issue RRC-33: Whether separate user consent for gNB centric training is needed

Open issue RRC-36: How data is forwarded to OAM or source gNB after HO

Proposal 21. [Open issue RRC-33/36] Send LS to RAN3 and SA5 to inform them of the agreements made regarding NW-sided logged data collection for AI/ML training

4. Reference

[1] R2-2504350 - RRC open issues AIML PHY