3GPP TSG-RAN WG2 Meeting #130	 R2- 2503569

St Julian’s, Malta, May 19-23, 2025

Agenda Item:	8.4.3

Source: 	NEC

Title:  	Discussion on LP-WUS RRM

Document for:	Discussion and Decision

- 1 Introduction

In last RAN2#129b meeting, the following agreements are made:

RAN2 assumes for the entry/ exit conditions of serving cell measurement offloading and serving cell RRM measurement relaxation: separate MR thresholds (according to RAN1 agreement)/LR thresholds can be configured for different types of LP WUR if a cell supports both types of LRs (can revisit based on RAN1 and RAN 4 progress, if any).

RAN2 assumes the entry/exit thresholds for RRM relaxation/offloading for OFDM-based WUR measuring LP-SS only are the same as that for OOK-based WUR measuring LP-SS. It can be revisited based on RAN1/RAN4 process, if any. Network is allowed to provide either OOK based threshold or OFDM based WUR mesasuring SSB threhold or both.

It is up to NW to configure either serving cell relaxation or serving cell offloading or both in one cell.

The metrics for RRM measurement offloading/relaxation criteria include (LP-)RSRP and optional (LP-)RSRQ.

How to define LP-RSRP and LP-RSRQ is up to RAN1.

The duplication between RAN2 and RAN4 specification on RRM relaxation and offloading should be avoided, details up to running CR rapporteur and companies’ review.

Merge the entry/exit condition for Serving Cell RRM measurement relaxation and Rel-19 Neighboring Cell RRM measurement relaxation (higher priority frequency is separate discussion).

In this contribution, we further discuss remaining issue for both relaxation and offloading.

- 2 Discussion

- 2.1 MR measurement relaxation

The last RAN2#129 meeting, RAN2 agreed that merge the entry/exit condition for Serving Cell RRM measurement relaxation and Rel-19 Neighboring Cell RRM measurement relaxation (higher priority frequency is separate discussion). One good progress is that we now have only one measurement relaxation criterion for both serving cell and neighbour cell. But it seems like exit condition is already assumed for measurement relaxation, however, based on RAN2 discussion and running CR Editor’s NOTE, i.e., FFS on exit condition for serving cell RRM relaxation, e.g., whether a separate exit condition other than ‘not fulfilling the entry condition’ is needed, or whether exit condition include MR and/or LR-based measurements. we still need to discuss whether exit condition is needed or not.

Whether separate exit condition is needed (38304-3)

Before the entry condition of measurement relaxation is fulfilled, the UE should perform normal/legacy MR serving cell measurement. However after entry condition is fulfilled, the UE cannot get MR serving cell measurement results in time as the UE has started to perform relaxed MR measurement. In this regard, MR measurement results may not be accurate or sufficient for exit condition evaluation. We should consider LR measurement results instead. Note that for only one entry condition, LR measurement is optional.

Therefore, we suggest to have a separate exit condition for measurement relaxation, that is to say, after entry condition is fulfilled, the UE performs LR measurements to evaluate whether exit condition of measured relaxation is fulfilled or not.

Proposal-1 (38304-3): introduce separate entry/exit condition, i.e., the LP-WUS UE performs relaxed MR measurements when entry condition is fulfilled and stops performing relaxed MR measurements when exit condition is fulfilled.

Entry condition: based on MR measurement results and optional LR measurement results;

Exit condition: based on LR measurement results.

Suggestion on higher priority frequency (38304-7/RRC-9)

In legacy, we have threshold SnonIntraSearchP and SnonIntraSearchQ to control higher priority frequency measurement behaviour. If the threshold is fulfilled, the UE search for higher priority frequency at least every Thigher_priority_search (normally 60s for each layer), while the threshold is not fulfilled, the UE search for higher priority frequency based on normal requirement as equal or lower priority frequency, i.e., Tmeasure,NR_Inter (normally one DRX cycle, e.g., 1.28s).

Therefore, when entry condition of measurement relaxation is fulfilled, how to perform higher priority frequency measurements should also be based on whether SnonIntraSearchP and SnonIntraSearchQ is fulfilled or not. One example is that if entry condition of measurement relaxation is fulfilled but Srxlev ≤ SnonIntraSearchP, the UE performs same relaxed MR measurement (based on the relaxed factor K) for all higher, equal, lower priority frequency. If entry condition of measurement relaxation is fulfilled and Srxlev > SnonIntraSearchP, higher priority frequency can be further relaxed based on Thigher_priority_search. However, it is assumed that RAN4 will discuss how and whether to combine higher priority frequency measurement requirement and relaxation/offloading.

In this sense, in our understanding, how to perform measurement requirement is in RAN4 scope, we have agreed that The duplication between RAN2 and RAN4 specification on RRM relaxation and offloading should be avoided. What we RAN2 needs to do is to specify what kind of measurement criterion we can have, as for the measurement requirement, e.g., how to perform measurements on intra-frequency, inter-frequency of higher, equal, lower priority, we can leave it to RAN4 discussion.

Proposal-2(38304-7/RRC-9): leave the measurement requirement on higher priority frequency to RAN4 discussion, RAN2 only to specify measurement criterion.

- 2.2 MR measurement offloading

Background:

Case #1 have been agreed in RAN4

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #1 Fully offloading case | Off | Off: FFS the condition and the details | ON |

The reason why the UE needs to perform neighbour cell measurement is to ensure the mobility, i.e., cell re-selection in RRC_IDLE/INACTIVE. Our understanding is that once the UE has fulfilled serving cell measurement offloading criterion (i.e., case#1, MR serving cell measurement is offloaded to LR), it means the signal strength/quality is already good enough, in this scenario, there will be no mobility problem/issue (noted that the LR is still used for serving cell measurement). Since MR serving cell measurement has been stopped, let along neighbour cell MR measurement. Therefore neighbour cell MR measurement should not be performed (i.e., FFS in case#1 can be removed).

Proposal-3: once the entry condition of MR serving cell measurement offloading is met, both MR serving cell and neighbour cell measurements should be stopped.

Some companies are mentioning whether to merge the entry/exit conditions for Serving Cell offloading and the legacy threshold for stopping neighbour cell measurement, i.e., Measurement rules for cell re-selection for both intra-frequency and inter-frequency (SIntraSearchP,Q / SnonIntraSearchP,Q). To our understanding, it is not easy to couple them. One reason is that we already agreed that entry/exit conditions are introduced for offloading criterion, however there is no exit condition for the legacy threshold (SIntraSearchP,Q / SnonIntraSearchP,Q). Another reason is that for legacy threshold, we have intra-frequency case and inter-frequency case, i.e., which threshold should be merged with offloading case is also not clear. Therefore, we suggest to not merge the entry/exit condition of offloading and the legacy threshold for stopping neighbour cell measurement.

Proposal-4: do not merge the entry/exit condition of offloading and the legacy threshold for stopping neighbour cell measurement (i.e., SIntraSearchP,Q / SnonIntraSearchP,Q).

- 3 Conclusions

Proposal-1(38304-3): introduce separate entry/exit condition, i.e., the LP-WUS UE performs relaxed MR measurements when entry condition is fulfilled and stops performing relaxed MR measurements when exit condition is fulfilled.

Entry condition: based on MR measurement results and optional LR measurement results;

Exit condition: based on LR measurement results.

Proposal-2(38304-7/RRC-9): leave the measurement requirement on higher priority frequency to RAN4 discussion, RAN2 only to specify measurement criterion.

Proposal-3: once the entry condition of MR serving cell measurement offloading is met, both MR serving cell and neighbour cell measurements should be stopped.

Proposal-4: do not merge the entry/exit condition of offloading and the legacy threshold for stopping neighbour cell measurement (i.e., SIntraSearchP,Q / SnonIntraSearchP,Q).

- 4 References