3GPP TSG-RAN WG2 Meeting #130	R2-2503573

Malta, May 19th –23th , 2025

Agenda Item:	8.5.2

Source: 	Xiaomi

Title:  	Discussion on on-demand SSB

Document for:	Discussion and Decision

1	Introduction

In RAN2#129bis meeting, we had some further discussion on OD-SSB and achieved the following agreement.

Agreements on OD-SSB

- For explicit activation/deactivation, the OD-SSB MAC-CE includes fixed sized bitmap to indicate whether OD-SSB is activated in each SCell (i.e., similar to legacy SCell A/D MAC-CE).

- A/D bit: “1” means activation, “0” means deactivation for explicit deactivation case.

- For explicit activation/deactivation, the OD-SSB MAC-CE supports two formats: one format indicates up to 7 SCells and the other format indicates up to 31 SCells.

- OD-SSB MAC-CE includes a configuration index for each SCell activating OD-SSB.

- RAN2 aims to define single MAC CE format for both explicit and implicit OD-SSB deactivations. Details are FFS.

- Working assumption: When A-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB. FFS if we allow multiple OD-SSBs with the different frequencies for a given SCell.

In this contribution, we would like to further discuss the procedures and signalling to support on-demand SSB and have corresponding observations and proposals.

2	Discussion

2.1  MAC-1: Single MAC CE format for both explicit and implicit OD-SSB deactivation

In last meeting, we discussed how to support implicit deactivation and agreed to define a single MAC CE for both explicit and implicit deactivation. RAN1 agreed if value N is configured, the OD-SSB is deactivated based on the value N implicitly or via MAC CE explicitly. In addition, RAN1 agrees OD-SSB is applied to Scenairo#3B, i.e., after SCell activation, however the MAC CE is only used for updating the transmission parameters, e.g., periodicity and/or SSB position and/or number N, which means “re-activation” with updated parameters is supported.

Agreement For a cell supporting on-demand SSB SCell operation, for configuring the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb-nrofBurst), - Alt 2: The value range of od-ssb-nrofBurst is {N2 integer values} - N2= [8] - If od-ssb-nrofBurst for an on-demand SSB is NOT configured, the on-demand SSB is deactivated via MAC CE. - If od-ssb-nrofBurst for an on-demand SSB is configured, the on-demand SSB is deactivated based on the configured value for od-ssb-nrofBurst [or via MAC CE].

Agreement For a cell supporting on-demand SSB SCell operation, the following combinations of scenarios and cases are supported for indicating OD-SSB using a MAC-CE. - Scenario #3B and Case #1 - In the above combinations of scenarios and cases, the MAC-CE is used only for updating the transmission parameter of a transmitted OD-SSB for the cell since the OD-SSB has been transmitted according to NW indication. - Scenario #3B and Case #2 - In the above combinations of scenarios and cases, the MAC-CE is used only for updating the transmission parameter of a transmitted OD-SSB for the cell since the OD-SSB has been transmitted according to NW indication.

Actually besides “re-activation”, another status is “maintain” for which there is neither update on the “activation/deactivation” status nor on the corresponding transmission parameters. Whether/how to support the status of “maintain” should be firstly determined. In our understanding, there is such a case, for a certain SCell, OD-SSB is activated and there is no update on the OD-SSB transmissions but to activate/deactivate/re-activate OD-SSB on another SCell, the indication/corresponding parameters for this certain SCell still needs to be indicated. To support the status of “maintain”, we think the following alternatives can be considered.

- Alternative1: single MAC CE for OD-SSB (re)activation and deactivation with two bitmaps

For this alternative, single MAC CE with two bitmaps will be defined. The first bitmap is used to indicate the SCells should apply the MAC CE and the second bitmap is used to indicate the (re)activation/deactivation status of the SCell. If the corresponding bit in the first bitmap is set to “0”, UE just ignores the MAC CE, while if the corresponding bit in the first bitmap is set to “1”, UE further checks the second bitmap to determine the (re)activation/deactivation status of the OD-SSB of each SCell. One example with serving cell index less than 8 is shown below.

- Alternative2: NW updates the associated number N for the SCells of which OD-SSB is “maintain”

For this alternative, it is up to NW implementation to update the associated number N for the SCells of which OD-SSB is “maintain”, e.g., without update on the related SSB position and/or periodicity, only the number N is updated/reduced, via association to another configuration index corresponding to the same SSB position/periodicity but different number N. Actually, from UE’s perspective, this is the same as “re-activation” since the transmission parameter/related configuration index is updated, however, from NW’s perspective, this is some kind of “maintain” of the OD-SSB transmission. In this case, from UE’s perspective, there is no “maintain” status of OD-SSB transmission, UE always follows the latest configuration index/transmission parameters if the OD-SSB is indicated as activated. To be more detailed, each time UE receives a MAC CE indicating the activation of the OD-SSB on a SCell, UE considers it as a new activation and follows the related configuration index/transmission parameters.

Alternative 1 requires additional bitmap and has impact on UE behavior on how to interpret the two bitmaps. However, for alternative 1, to “maintain” a SCell, the corresponding configuration index/transmission parameter can be skipped while alternative 2 always has to indicate/update the corresponding configuration index/transmission parameter even for a SCell which is to “maintain” the OD-SSB transmission status. Considering signaling overhead may be not an issue since there may be not many OD-SSB SCells and to reduce the impact on the specification, we suggest RAN2 to adopt alternative 2.

Proposal 1: (MAC-1) it is up to NW implementation to update the associated number N if NW intends to maintain the OD-SSB transmission on the SCell.

Proposal 2: (MAC-1) Each time UE receives a MAC CE indicating the activation of the OD-SSB on a SCell, UE considers it as a new activation and follows the related configuration index/transmission parameters.

If proposal 3 and proposal 4 are agreed, then the detailed format of OD-SSB MAC CE is quite straightforward as shown below.

Figure 1: OD-SSB Activation/Deactivation MAC CE with one octet Ci field

Figure 2: OD-SSB Activation/Deactivation MAC CE with four octet Ci field

Proposal 3: (MAC-1) For both explicit and implicit deactivation, the OD-SSB MAC-CE includes fixed sized bitmap to indicate whether OD-SSB is activated in each SCell (i.e., similar to legacy SCell A/D MAC-CE) with “1” means activation, “0” means deactivation.

Proposal 4: (MAC-1)	For both explicit and implicit deactivation, the OD-SSB MAC-CE supports two formats: one format indicates up to 7 SCells and the other format indicates up to 31 SCells.

2.2 	RRC-1: OD-SSB configuration index

According to the following agreement from RAN1, similar as periodicity, number N and SSB positions can be configured with multiple values and MAC CE indicates one applicable value. In last meeting, we agreed to include a configuration index for each SCell activating OD-SSB, therefore, the configuration index should be associated with a set of parameters, i.e., periodicity, SSB positions and number N of OD-SSB bursts.

Agreement For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s) (in addition to agreed ones), multiple candidate values can be configured (includes the case where no candidate values are configured) by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell. - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst (i.e., od-ssb-PositionsInBurst) for the following cases - The case where center frequency of AO-SSB and OD-SSB are different - Case 1 - Number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb- nrofBurst) FFS: Additional restrictions

Proposal 5: (RRC-1) The configuration index is associated with a set of parameters, i.e., od-ssb-Periodicity, od-ssb-PositionsInBurst and od-ssb-nrofBurst.

2.3 	RRC-3: L3 measurement framework

In RAN2#129 meeting, we discussed the impact on RRM measurement for SSB adaptation and achieved the following agreement.

- RAN2 preference is to keep SMTC based L3 RRM framework and to introduce additional SMTC configuration according to SSB adaptation for L3 RRM measurement on SCell with SSB adaptation.

Regarding the impact on RRM measurement for OD-SSB, even RAN4 has agreed to a fast mode measurement during a window, i.e., the measurement requirement is defined based on SSB periodicity. However, after the window, how to perform RRM measurement should be further discussed. We prefer to adopt unified solution as SSB adaptation to simplify the workload of both RAN2 and RAN4. This is because

- Firstly, case 2 of OD-SSB is quite similar as SSB adaptation, there is no reason to adopt different measurement method for these two cases.

- Secondly, with multiple SMTC windows defined for SSB adaptation, RAN4 need to work on the related measurement requirement, multiple SMTC for OD-SSB can be supported for free.

Therefore, for OD-SSB, additional SMCT configurations are introduced according to the different OD-SSB periodicities. When performing RRM measurement, UE adjusts the SMCT accordingly based on the indicated OD-SSB periodicity.

Proposal 6: (RRC-3)	For L3 measurement, unified solution as SSB adaptation is adopted for OD-SSB, e.g., additional SMTC configurations are introduced according to different OD-SSB periodicities.

2.4 	RRC-4: Which MO to use for measurement

In last meeting, we discussed the MO for OD-SSB and reached the following WA. We see no motivation to revert the WA, thus we suggest to confirm the WA.

- Working assumption: When A-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB. FFS if we allow multiple OD-SSBs with the different frequencies for a given SCell.

Proposal 7: RAN2 to confirm the WA as agreement “When A-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB.”

Another issue is when both OD-SSB and AO-SSB are available, which MO to use for measurement since RAN1 agreed UE is not required to measure both AO-SSB and OD-SSB. We think the situation is not as same as that of Redcap, since for Redcap, there is no CD-SSB on the active BWP, that’s why NCD-SSB is defined for measurement to relieve the UE from switching the frequency to measure CD-SSB. However, for NES, two SSBs are both available on the same BWP, then the point is to select one from the two. We think either MO associated with AO-SSB or MO associated with OD-SSB is feasible, or we can leave the selection to UE implementation.

Agreement

Regarding the relation in terms of frequency location (i.e., center frequency) between the always-on SSB and on-demand SSB,

- Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB.

  - On-demand SSB is not on sync raster

  - AO-SSB and OD-SSB are located in the same BWP

  - FFS: Additional conditions

  - Subject to separate UE capability

  - Note: UE is not required to measure both AO-SSB and OD-SSB

Proposal 8: (RRC-4) For case#2, when OD-SSB and AO-SSB are on different frequencies, either MO associated with AO-SSB or MO associated with OD-SSB can be used for measurement.

2.5 	RRC-5: New measurement report trigger

In previous meeting, there were proposals to trigger measurement report when OD-SSB is activated. The motivation is to quickly measure and report the latest measurement results when OD-SSB is activated, which can change the SCell from unknown cell to known cell to accelerate the activation of SCell.

However, we think for scenario#2, when to trigger SCell activation is up to gNB, even a measurement report is triggered when OD-SSB is activated, it cannot ensure the SCell is always known, e.g., the duration between the triggered measurement report and the SCell activation may exceed the limitation. Therefore, this optimization is not needed for scenario#2. While for Scenario#2A, we think legacy principle can be reused here e.g., after receiving the SCell activation command, if UE has valid L3 measurement results available but not reported on the unkown SCell, the L3 measurement report will be initiated. Nothing new for OD-SSB case 1 and case 2.

Proposal 9: (RRC-5) No need to trigger measurement report when OD-SSB is activated.

2.6 Neighbour cell measurement

For neighbour cells measurements, we think according to the WID, the impact on neighbour cell measurement with OD-SSB is out of scope. From RAN2 perspective, there is no need to discuss this aspect and we can assume the SSB status of neighbour cell is transparent to UE and how to handle the reported neighbour cell measurement is up to gNB implementation.

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

Proposal 10: There is no impact on neighbour cell measurement with OD-SSB, e.g., the OD-SSB status of neighbour cell is transparent to UE.

2.7 Indication of OD-SSB SCell

UE should be informed of whether a SCell is a normal SCell or on-demand SSB SCell. This is because without this information, UE may derive incorrect measurement results with the assumption that the SCell is normal SCell and SSB is transmitted. It may cause problems on RRM procedure as the accuracy of measurement may be significantly degraded. Therefore, we think upon SCell addition, NW should indicate whether the SSB transmission on the SCell is on-demand manner.

Proposal 11: Upon SCell addition, NW indicates whether the SSB transmission on this SCell is on-demand manner via RRC signalling.

3	Conclusion

In this contribution, we discussed the procedures and signalling to support on-demand SSB and have corresponding observations and proposals:

MAC-1: Single MAC CE format for both explicit and implicit OD-SSB deactivation

Proposal 1: (MAC-1) it is up to NW implementation to update the associated number N if NW intends to maintain the OD-SSB transmission on the SCell.

Proposal 2: (MAC-1) Each time UE receives a MAC CE indicating the activation of the OD-SSB on a SCell, UE considers it as a new activation and follows the related configuration index/transmission parameters.

Proposal 3: (MAC-1) For both explicit and implicit deactivation, the OD-SSB MAC-CE includes fixed sized bitmap to indicate whether OD-SSB is activated in each SCell (i.e., similar to legacy SCell A/D MAC-CE) with “1” means activation, “0” means deactivation.

Proposal 4: (MAC-1)	For both explicit and implicit deactivation, the OD-SSB MAC-CE supports two formats: one format indicates up to 7 SCells and the other format indicates up to 31 SCells.

RRC-1: OD-SSB configuration index

Proposal 5: (RRC-1) The configuration index is associated with a set of parameters, i.e., od-ssb-Periodicity, od-ssb-PositionsInBurst and od-ssb-nrofBurst.

RRC-3: L3 measurement framework

Proposal 6: (RRC-3)	For L3 measurement, unified solution as SSB adaptation is adopted for OD-SSB, e.g., additional SMTC configurations are introduced according to different OD-SSB periodicities.

RRC-4: Which MO to use for measurement

Proposal 7: RAN2 to confirm the WA as agreement “When A-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB.”

Proposal 8: (RRC-4) For case#2, when OD-SSB and AO-SSB are on different frequencies, either MO associated with AO-SSB or MO associated with OD-SSB can be used for measurement.

RRC-5: New measurement report trigger

Proposal 9: (RRC-5) No need to trigger measurement report when OD-SSB is activated.

Neighbour cell measurement

Proposal 10: There is no impact on neighbour cell measurement with OD-SSB, e.g., the OD-SSB status of neighbour cell is transparent to UE.

Indication of OD-SSB SCell

Proposal 11: Upon SCell addition, NW indicates whether the SSB transmission on this SCell is on-demand manner via RRC signalling.

4	Reference

[1] RAN1#120bis Chair notes.

[2] RAN2#129bis meeting minutes