3GPP TSG-RAN WG2 Meeting #130                                                       R2-2503585

St. Julian’s, Malta, May 19th – 23rd, 2025

Source:	CATT

Title:	MRO Enhancements for Rel-18 Mobility

Agenda Item:	8.10.2

Document for:	Discussion and Decision

1. Introduction

In this contribution, we further discuss the potential MRO enhancements for LTM and CHO with candidate SCG(s).

2. Discussion

2.1 MRO Enhancements for LTM

2.1.1 MRO for SCG LTM

In RAN2#125bis meeting, RAN2 achieved the following agreements about MRO for LTM [1].

RAN2 considers SHR, RA report and RLF for MCG LTM SON. RAN2 will start work on MCG LTM.

Based on the agreements above, RAN2 mainly focused on MRO for MCG LTM in the previous RAN2 meetings. Considering the limited time of Rel-19 and there are still some leftover issues for MCG LTM, we suggest MRO enhancements for SCG LTM is not considered in Rel-19 SON/MDT.

Proposal 1: MRO enhancement for SCG LTM is not considered in Rel-19 SON/MDT.

2.1.2 Enhance RLF report for LTM MRO

In LTM, if the UE can acquire the valid TA value (included in LTM cell switch command or measured by UE itself) when receiving the LTM cell switch command, UE will perform RACH-less LTM execution. For RACH-less LTM, both dynamic grant and configured grant can be used, and the UE will apply the TCI state indicated in the LTM cell switch command for dynamic grant reception and configured grant resource selection. If RACH-less LTM fails, it may be because the TCI state indicated in LTM cell switch command is inappropriate, then the UE needs to log the TCI state ID indicated in the LTM cell switch command in RLF report to help the network to optimize and select a suitable TCI state to avoid the RACH-less LTM failure.

Proposal 2: Include TCI state ID from LTM cell switch command in RLF report for RACH-less LTM failure.

2.1.3 Enhance SHR for LTM MRO

In case of successful LTM execution but it’s still close to failure, whether legacy SHR trigger conditions (i.e., T310 threshold, T312 threshold, T304 threshold) can be applied to LTM should be discussed. From our perspective, the T310 threshold and T304 threshold trigger conditions can be reused for LTM, but for T312 threshold trigger condition, considering that T312 is configured in MeasObjectNR, i.e., L3 RRM framework, it is not sure whether it is applicable for DU triggered LTM.

Proposal 3: The legacy SHR trigger conditions based on T310 threshold and T304 threshold can be applied to LTM.

SHR should be enhanced to record the successful LTM information. Similar as L3 measurement of serving cell, target cell and other LTM candidate cell(s) are included in RLF report upon RLF or mobility failure, L3 measurement of serving cell, target cell and other LTM candidate cell(s) should also be recorded in SHR for successful LTM execution.

Proposal 4: Include L3 measurement results of serving cell, target cell and other LTM candidate cell(s) in SHR for LTM MRO.

2.1.4 Enhance RACH report for LTM

In RAN2#125bis meeting, it was agreed that RAN2 considers RACH report for MCG LTM SON [1].

RAN2 considers SHR, RA report and RLF for MCG LTM SON.

For RACH-based LTM, raPurpose should be enhanced to cover successful RACH-based LTM execution, the detail can be discussed further, e.g., whether legacy raPurpose can be reused or a new raPurpose for LTM execution needs to be introduced.

Proposal 5: Enhance raPurpose in RACH report to support successful RACH-based LTM execution. The detail can be discussed further, e.g., whether to reuse legacy raPurpose or introduce a new raPurpose.

For the TA value in LTM cell switch command (network-based TA acquisition), the early RACH procedure is needed. The source cell needs to send the PDCCH order command to UE, then the UE sends the preamble towards the indicated candidate cell. After the candidate cell acquires the TA value when receives the preamble from UE, it sends the TA value to the source cell and source cell includes the TA value in the LTM cell switch command to UE. The RACH procedure can be triggered multiple times towards same or different candidate cells before LTM execution. A new raPurpose can be introduced to indicate the network-based early TA acquisition.

Proposal 6: A new raPurpose in RACH report is introduced to indicate the network-based early TA acquisition.

Based on current spec, the UE may include up to 8 RACH report in RACH report list. If the RACH report is enhanced to support logging the RACH information of network-based early TA acquisition, the RACH report entry will reach max number (i.e., maxRAReport) soon as the frequent network-based early TA acquisition, this will affect the legacy RACH information logging, e.g., RACH information of handover as the UE will stop the RACH report logging if the entry number reaches max number, which will cause the legacy RACH information loss and this is not expected as the legacy RACH information is important for RACH optimization.

5.7.10.4 Actions for the Random Access report determination Upon successfully performing random-access procedure initialized with 4-step or 2-step RA type, or upon failed or successfully completed on-demand system information acquisition procedure in RRC_IDLE or RRC_INACTIVE state, or upon failed or successfully completed RA-SDT operation as specified in clause 5.3.13.5, the UE shall: 1> if the RPLMN or the PLMN selected by upper layers (see TS24.501 [23]) from the PLMN(s) included in the plmn-IdentityList in SIB1 is not included in plmn-IdentityList stored in a non-empty VarRA-Report; or 1> if the registered SNPN identity or the SNPN identity selected by upper layers (see TS24.501 [23]) from the list of SNPN(s) included in the NPN-IdentityInfoList in SIB1is not included in snpn-IdentityList stored in a non-empty VarRA-Report: 2> clear the information included in VarRA-Report; 1> if the UE is not in SNPN access mode and if the number of RA-Report entries stored in the ra-ReportList in VarRA-Report is less than maxRAReport: 2> if the number of PLMN entries in plmn-IdentityList in identityList stored in VarRA-Report is less than maxPLMN; or 2> if the number of PLMN entries in plmn-IdentityList in identityList stored in VarRA-Report is equal to maxPLMN and the list of EPLMNs is subset of or equal to the plmn-IdentityList stored in VarRA-Report: 3> append the following contents associated to the random-access procedure as a new entry in the VarRA-Report: <omit part>

RAN2 needs to discuss how to log the RACH information of network-based early TA acquisition and avoid the impact on the legacy RACH information logging. Some solutions can be considered to address the issue, e.g., one entry in RACH report can be used to log the RACH information of all network-based early TA acquisition before one LTM execution or one entry in RACH report can be used to log the RACH information of all network-based early TA acquisition before LTM configuration release or log the RACH information of network-based early TA acquisition per candidate cell in one or multiple entries.

Proposal 7: RAN2 to discuss how to log the RACH information of network-based early TA acquisition to avoid the impact on the legacy RACH information logging (e.g., RACH information for handover):

- Option 1: one entry in RACH report to log the RACH information of all network-based early TA acquisition before one LTM execution;

- Option 2: one entry in RACH report to log the RACH information of all network-based early TA acquisition before LTM configuration release;

- Option 3: log the RACH information of network-based early TA acquisition per candidate cell in one or multiple entries.

2.1.5 Other issues

The LTM configuration will not be released after successful LTM execution, which means subsequent LTM can continue to be performed. However, LTM execution is based on the L1 measurement results which vary very rapidly and may cause the frequent LTM execution. Considering that the legacy RLF report/SHR only includes the latest failure/successful handover information, for LTM, there is high probability that the old RLF report/SHR will be overrode by new RLF report/SHR due to the frequent LTM execution, this will cause the information loss for network optimization. In order to address the issue, recording multiple RLF report/SHR can be considered as a candidate solution.

Proposal 8: To address the issue of frequent overriding RLF report/SHR for subsequent LTM, recording multiple reports can be considered as a candidate solution.

In Rel-18 mobility WID, L3 measurement based LTM is supported, i.e., the CU can request the DU to send MAC CE to trigger LTM execution based on UE layer 3 measurements. If L3 measurement based LTM execution failure/near failure, it should be the source CU to analyze and optimize the failure/near failure instead of source DU. However, considering that the source CU cannot know whether it is L3 measurement based LTM or L1 measurement based LTM execution when the source CU receives the RLF report/SHR, the source CU can send the RLF report/SHR to source DU by F1 interface to optimize the failure/near failure, but, the handover decision mechanism of the source DU may not be problematic. In order to address the issue, the indication information can be needed to be included in RLF report/SHR to indicate L1 measurement based LTM or L3 measurement based LTM to avoid the error optimization occurred.

Proposal 9: Indication information is needed in RLF report/SHR to indicate whether it’s L1 measurement based LTM or L3 measurement based LTM.

2.2 MRO Enhancements for CHO with candidate SCG(s)

2.2.1 Enhance RLF report for CHO with candidate SCG(s)

In RAN2#129 meeting, RAN2 agreed that includes the following highlighted information in RLF report for CHO with candidate SCG(s) [2].

- Enhance RLF report for CHO with candidate SCGs to include the information for each CHO, i.e., first fulfilled event and time duration between two events fulfilled, if any. - Enhance RLF report for CHO with candidate SCGs to include at least the following information: - Identifier of candidate PCell(s) which met the configured CHO execution conditions when the RLF is encountered; - Identifier of candidate PSCell(s) which met the configured CPAC execution conditions when the RLF is encountered; - The Identifier of candidate PCell(s) or PSCell(s) that fulfilled execution conditions before the RLF is encountered.

For CHO with candidate SCG(s) mobility failure, it is with high possibility that the CHO and/or CPAC execution condition configurations are not properly configured, which needs to be optimized.

Based on the current spec and agreements, CHO execution condition configuration and CHO execution condition fulfilled situation (first fulfilled event and time duration between two events fulfilled) have been supported in RLF report, however, CPAC execution condition configuration and CPAC execution condition fulfilled situation (first fulfilled event and time duration between two events fulfilled) are missing in RLF report which are important for CHO with candidate SCG(s) failure optimization.

Furthermore, considering that it was agreed to log the identifier of candidate PSCell(s) which met the configured CPAC execution conditions when the RLF is encountered, logging CPAC execution condition fulfilled situation (e.g., the time duration between two events fulfilled) can indicate the information implicitly.

Proposal 10: Enhance RLF report for CHO with candidate SCGs to include the information for each candidate PSCell, i.e., execution condition configuration, first fulfilled event and time duration between two events fulfilled, if any.

2.2.2 Both SHR and SPR being generated in a CHO with candidate SCG

For CHO with candidate SCG(s) failure or near failure, there can be more than one report are generated, e.g., when execution CHO with candidate SCG(s), both SHR and SPR trigger condition are met (e.g., both T310 in PCell and T310 in PSCell exceed a configured threshold), UE can store two successful report (SHR and SPR) separately. Considering that the two reports are related to the same one handover procedure, the two reports should be associated to indicate the correlation information to network for optimization.

Proposal 11: (RRC-1) Correlation of the SHR and SPR is needed in case of both reports being generated in a CHO with candidate SCG execution scenario.

Another issue is how to associate the two reports, we propose the following two options can be considered.

Option 1: one report to include both SHR and SPR information;

Option 2: two reports for SHR and SPR separately, and the association information is added in the two reports.

Similar as correlation of the RLF report and SHR in Rel-17 SON/MDT (two reports for RLF report and SHR separately for the same one handover procedure), we prefer Option 2, and the association information in the two reports can be further discussed.

Proposal 12: (RRC-1) Two reports for SHR and SPR separately, and the association information is added in the two reports when both SHR and SPR being generated in a CHO with candidate SCG execution scenario. FFS detailed association information.

In addition, in RAN2#129bis meeting, the following issue was discussed and no consensus was achieved [3].

FFS whether it is needed to avoid duplication of information in case of two reports being generated CHO with candidate SCGs, any redundancy (e.g., measurements) are recorded in the reports for PCell (i.e., in SHR, SPR).

From our perspective, the redundant information in SHR and SPR (e.g., measurement results, time info) should be avoided, the redundant information can be logged only in the report for PCell, i.e., SHR.

Proposal 13: (RRC-1) UE logs the overlapped information only in SHR when both SHR and SPR being generated in a CHO with candidate SCG execution scenario.

3. Conclusion

According to the discussion in section 2, we propose:

LTM

MRO for SCG LTM

Proposal 1: MRO enhancement for SCG LTM is not considered in Rel-19 SON/MDT.

Enhance RLF report for LTM MRO

Proposal 2: Include TCI state ID from LTM cell switch command in RLF report for RACH-less LTM failure.

Enhance SHR for LTM MRO

Proposal 3: The legacy SHR trigger conditions based on T310 threshold and T304 threshold can be applied to LTM.

Proposal 4: Include L3 measurement results of serving cell, target cell and other LTM candidate cell(s) in SHR for LTM MRO.

Enhance RACH report for LTM

Proposal 5: Enhance raPurpose in RACH report to support successful RACH-based LTM execution. The detail can be discussed further, e.g., whether to reuse legacy raPurpose or introduce a new raPurpose.

Proposal 6: A new raPurpose in RACH report is introduced to indicate the network-based early TA acquisition.

Proposal 7: RAN2 to discuss how to log the RACH information of network-based early TA acquisition to avoid the impact on the legacy RACH information logging (e.g., RACH information for handover):

- Option 1: one entry in RACH report to log the RACH information of all network-based early TA acquisition before one LTM execution;

- Option 2: one entry in RACH report to log the RACH information of all network-based early TA acquisition before LTM configuration release;

- Option 3: log the RACH information of network-based early TA acquisition per candidate cell in one or multiple entries.

Other issues

Proposal 8: To address the issue of frequent overriding RLF report/SHR for subsequent LTM, recording multiple reports can be considered as a candidate solution.

Proposal 9: Indication information is needed in RLF report/SHR to indicate whether it’s L1 measurement based LTM or L3 measurement based LTM.

CHO with candidate SCG(s)

Enhance RLF report for CHO with candidate SCG(s)

Proposal 10: Enhance RLF report for CHO with candidate SCGs to include the information for each candidate PSCell, i.e., execution condition configuration, first fulfilled event and time duration between two events fulfilled, if any.

Both SHR and SPR being generated in a CHO with candidate SCG

Proposal 11: (RRC-1) Correlation of the SHR and SPR is needed in case of both reports being generated in a CHO with candidate SCG execution scenario.

Proposal 12: (RRC-1) Two reports for SHR and SPR separately, and the association information is added in the two reports when both SHR and SPR being generated in a CHO with candidate SCG execution scenario. FFS detailed association information.

Proposal 13: (RRC-1) UE logs the overlapped information only in SHR when both SHR and SPR being generated in a CHO with candidate SCG execution scenario.

4. Reference

- R2-2404102 Report of 3GPP TSG RAN WG2 meeting #125bis, Changsha, China

- R2-2501702 Report of 3GPP TSG RAN WG2 meeting #129, Athens, Greece

- R2-2503302, RAN2_129bis_Meeting_Report