3GPP TSG RAN WG2 #130	                                                                             R2-2503592

St. Julian’s, Malta, May 19th – 23rd, 2025

Source:	CATT, Turkcell

Title:	Functionality management of AIML mobility

Agenda Item:	8.3.2

Document for:	Discussion and Decision

- Introduction

In RAN2#129bis meeting, preliminary agreements were made for spec analysis of AI mobility [1]. In this contribution, we analyze the spec impact of RRM measurement prediction and measurement event prediction from the following aspects for UE-sided model:

- Applicability procedure;

- Inference procedure.

- Discussion

  - RRM measurement prediction

In this section, we discuss the functionality management of AI/ML mobility for RRM measurement prediction for UE sided model.

Applicability procedure

The following agreements were made in RAN2#129bis meeting for applicability procedure:

Agreements 1. The general LCM framework for beam management can be the baseline (where applicable) for AI mobility, such as the following aspects: - Data collection for model training - UE capability - Applicability reporting - Inference configuration and reporting - Performance monitoring and management 2 Only functionality-based LCM is considered for AI/ML mobility (i.e. we don’t support model based LCM)

- For AI/ML for mobility, UE can report the applicable functionalities to NW via UAI or RRCReconfigurationComplete message.

- FFS whether association ID is required and should provide motivation on per use case bases

Regarding network additional conditions for BM, they are used for the UE to understand specific conditions under which the AI/ML model is applicable, such as scenarios, sites, and datasets. Some network additional conditions are helpful to improve the prediction accuracy, e.g., the antenna configuration, scenarios, etc. For AI mobility, the RSRP can be predicted from measured RSRP which may be collected in various scenarios, such as different UE speeds, cell configurations, MRRTs, and etc. And it has been proved that generalization can be applied to different cell configurations with minimum performance impacts. Therefore, it is required to figure out what network additional conditions are required for inference, whether these network additional conditions are invisible to the UE and whether generalization is workable. If these conditions are required and invisible to the UE, and the generalization does not work efficiently, we think the associated ID is needed. Otherwise, visible additional network conditions can be directly configured to the UE without associated ID. Hence, the following options are provided:

- Option 1: Network additional condition is not needed, considering e.g., the good performance of model generalization;

- Option 2: Network additional condition is needed, which can be configured to the UE using associated ID or cell configuration.

Proposal 1: RAN2 discusses which option is adopted for network additional condition for RRM measurement prediction:

- Option 1: Network additional condition is not needed, considering the good performance of model generalization;

- Option 2: Network additional condition is needed, which can be configured to the UE using associated ID or specific cell configuration.

Inference configuration and report

For inference of AI mobility, it has been agreed in RAN2#129bis meeting that:

Agreements 1. The general LCM framework for beam management can be the baseline (where applicable) for AI mobility, such as the following aspects: - Data collection for model training - UE capability - Applicability reporting - Inference configuration and reporting - Performance monitoring and management

The UE applies inference configuration for the applicable RRM measurement prediction and measurement event prediction upon receiving configuration via RRC. As baseline, no dynamic signaling (e.g. L1/2) is required. FFS when the UE can perform inference

1 The inference configuration and reporting for AI/ML mobility can be based on RRM measurement framework

It has been agreed in RAN2#129bis that RRM measurement framework can be used for inference configuration and reporting. According to current specification, the RRM measurement framework is illustrated in Figure 1 where a measID links one measurement object ID and one report configuration ID. Regarding the agreed the cases, the measurement inputs are mainly based on the actual measurement results which can be configured referring to measurement objects, e.g. MeasObjectNR.

Proposal 2: For UE-sided model inference of RRM measurement prediction, the MO need to be configured as inference inputs for RRM framework.

Figure 1 Association of MeasConfig

Considering how to configure the reporting of the inference outputs, i.e. the predicted measurement results, the following are considered.

Intra-frequency temporal domain case A

For intra-frequency temporal domain case A, the UE uses the historical measurement results to predict the future measurement results. Hence, the inference configuration for predicted measurement report can the same as that of the actual measurement configuration except it is based on predication. However, the network needs to indicate the timing information, e.g., the minimum duration when the predicted measurement results should be obtained, for the predicted measurement results.

Proposal 3: For measurement results prediction, the inference configuration for intra-frequency temporal domain case A can include the timing information, e.g., the minimum duration when the predicted measurement results should be obtained.

Considering there may be multiple measurement results within the PW, RAN2 needs to discuss which results needs to be reported in the measurement results. Generally, there are 3 options:

- Option 1: The last predicted measurement result in PW;

- Option 2: The average measurement result in PW;

- Option 3: The sample measurement result in PW.

Option 1 can be used directly for handover decision but with the lowest accuracy. While in Option 2, the accuracy is averaged, but the averaged measurement result does not reflect the last value in the PW. While Option 3 may bring high signaling burden on reporting but more precise.

Proposal 4: For intra-frequency temporal domain case A, RAN2 to discuss which option is included in the measurement report of RRM measurement prediction:

- Option 1: The last predicted measurement result in PW;

- Option 2: The average measurement result in PW；

- Option 3: The sample measurement result in PW.

Intra-frequency temporal domain case B

In intra-frequency temporal domain case B, measurement results in PW are predicted by actual historical measurement result(s) in OW. Then OW and PW slide forward and measurement results in previous PW are skipped. The inference configuration for predicted measurement can the same as that of the actual measurement configuration except for the timing configuration for the predicted measurement results. Normally, the timing configuration on the predicted measurement results can be:

- Option 1: the network configures the MRRT and the UE derives the timing configuration according to MRRT;

- Option 2: the network configures the periodicity and the timing offset of the predicted measurement.

For Option 1, the skipping pattern also impacts the predicted measurement occasion. Hence, it can also be included in the configuration.

Proposal 5: For intra-frequency temporal domain case B, RAN2 to discuss what is configured for the inference RRM measurement prediction:

- Option 1: the network configures the MRRT and the UE derives the timing configuration according to MRRT which may include the skipping pattern;

- Option 2: the network configures the periodicity and the timing offset of the predicted measurement;

Inter-frequency (frequency domain)

For inter-frequency prediction, the measurement result of one frequency is predicted based on the measurement results of another frequency. Different from temporal domain, frequency information, e.g.,  ssbFrequency or refFreqCSI-RS, of the predicted measurement needs to be configured to the UE.

Proposal 6: For inter-frequency prediction, the frequency information of the predicted measurement needs to be configured for the UE for the inference of RRM measurement prediction.

  - Measurement event prediction

In this section, we discuss the spec impact for measurement event prediction for UE sided model. Both indirect measurement event prediction and direct measurement event prediction are considered, and for indirect measurement event prediction, all the following 4 listed will be considered which have been agreed in previous meeting.

Agreements 1 Agree to listed 4 definitions of indirect measurement event prediction Indirect measurement event prediction for temporal domain case A: In indirect measurement event prediction, future measurement result(s) is predicted by a RRM measurement prediction model for intra-frequency temporal domain case A at first. Afterwards, predicted and optionally actual historical measurement result(s) are used to derive whether a measurement event at one future time instance occurs, without further involvement of an AI/ML model. Indirect measurement event prediction for temporal domain case B: In indirect measurement event prediction, measurement result(s) is predicted by a RRM measurement prediction model for intra-frequency temporal domain case B at first. Afterwards, predicted and optionally actual historical measurement result(s) are used to derive whether a measurement event at one time instance occurs, without further involvement of an AI/ML model. Indirect measurement event prediction for frequency domain: In indirect measurement event prediction, measurement result(s) is predicted by a RRM measurement prediction model in frequency domain at first. Afterwards, predicted and optional actual historical measurement result(s) of serving cell are used to derive whether a measurement event at one time instance occurs, without further involvement of an AI/ML model. Indirect measurement event prediction for spatial domain: In indirect measurement event prediction, measurement result(s) is predicted by a RRM measurement prediction model in spatial domain at first. Afterwards, predicted measurement result(s) and actual measurement results are used to derive whether a measurement event at one time instance occurs, without further involvement of an AI/ML model

Applicability procedure

Similar as data collection for training for indirect measurement event prediction, considering that the RRM measurement prediction model is used for indirect measurement event prediction, the applicability reporting of indirect measurement event prediction is same as RRM measurement prediction.

Proposal 7: For indirect measurement event prediction, the applicability reporting is the same as that for RRM measurement prediction.

For direct measurement event prediction, the applicability reporting can take the applicability reporting of AI/ML based beam management as baseline. For applicability reporting for BM UE-sided model, the following steps were proposed.

Figure 1: signalling procedure on applicability reporting for BM UE-sided model

In direct measurement event prediction, for Step 1 and Step 2, new UE capability is needed, e.g., whether the UE supports direct measurement event predication.

For step 3, RAN2 has agreed to support two options in beam management use case, i.e., Option A (full inference configuration in Step 3) and Option B (inference related parameters in Step 3), both the two options can also be considered in direct measurement event prediction. Similar as BM use case, for Option A, one or more of ReportConfigNR for inference configuration can be configured in RRM framework, and for Option B, it can be further discussed whether UAI or RRM framework is used. In step 4, similar as BM use case, for Option A, the applicable/non-applicable functionalities are reported in RRCReconfigurationComplete for initial reporting; UAI is used for the change of applicability. For Option B, it is FFS.

For the activation of applicable functionalities, for Option A, the UE autonomously activates the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in Step 4 for both periodic and event trigger reporting. For Option B, it is FFS.

Proposal 8: For direct measurement event prediction, for applicability reporting,

- new UE capability is needed, e.g., whether the UE supports direct measurement event predication.

Inference configuration and report

As mentioned above, considering that the RRM measurement prediction model is used for indirect measurement event prediction, the model inference configuration e.g., input and output configuration of model inference for indirect measurement event prediction should be same as RRM measurement prediction.

Proposal 9: For indirect measurement event prediction, the model inference configuration e.g., input and output configuration of model inference is the same as that for RRM measurement prediction.

According to the current spec, the report configuration criteria consist of periodical reporting and event triggered reporting. For indirect measurement event predication, event triggered reporting should be supported.

Proposal 10: For indirect measurement event predication, event triggered reporting should be supported.

For temporal domain case B, considering L3 filtering can be applied to the measurement results, the measurements satisfying A3 entry condition and triggering condition are based on the filtered results of both actual and predicted measurements. Apart from this, the measurement event can be triggered considering all the measurement results within TTT which also include both actual and predicted measurement results. Hence, it is complex to indicate the measurement result which is actual or predicted. Consequently, the legacy RRM measurement report is sufficient for measurement report for indirect prediction of temporal domain case B.

For frequency domain and spatial domain prediction, the measurement results are predicted based on another frequency or RS resource set. And the measurement event can be predicted totally based on the predicted measurement results. Hence, there is no need to differentiate whether the measurement result is actual or predicted. Therefore, the legacy RRM measurement reporting can be reused for indirect measurement event predication of frequency domain and spatial domain.

Proposal 11: For temporal domain case B, frequency domain, and spatial domain prediction, the legacy RRM measurement reporting can be reused for indirect measurement event predication.

In indirect measurement event prediction for intra-frequency temporal domain case A, measurement result(s) is predicted by a RRM measurement prediction model at first. Afterwards, predicted and optionally actual historical measurement result(s) are used to derive whether a measurement event at one future time instance occurs, without further involvement of an AI/ML model. It is used for handover performance enhancement. The UE can report the predicted occurrence time for the indirect predicted measurement event.

Proposal 12: For temporal domain case A, the UE can report the predicted occurrence time for the indirect predicted measurement event.

For direct measurement event prediction, in previous RAN2 meeting, we agreed the following agreements .

Agreements on inputs/outputs and KPIs - For direct measurement event prediction, the model output is the probability of event occurrence within a time window.

For direct measurement event prediction, the model input can be measurement results of serving/neighboring cells and the event configuration parameters (e.g., TTT), the model output is the probability of event occurrence within a time window, the model input can also include other information, e.g., UE location etc., therefore, the input/output configuration can be configured from network to UE using RRM framework as mentioned in Proposal 8.

Proposal 13: For direct measurement event prediction, the input/output configuration of model can be configured from network to UE:

- Input configuration: measurement resources of serving/neighbouring cells, event configuration parameters (e.g., TTT), other configuration information is FFS;

- Output configuration: the probability of event occurrence within a time window.

Similar as indirect measurement event prediction for temporal domain case A, enhanced RRM measurement reporting mechanism is needed for the inference result reporting, e.g. reporting the probability of event occurrence within a time window.

Proposal 14: For direct measurement event prediction, enhanced RRM measurement reporting mechanism is needed for the inference result reporting, e.g. reporting the probability of event occurrence within a time window.

  - When to perform inference

It was FFS on when the UE can perform inference. In the following, we discuss this issue for both RRM measurement prediction measurement event prediction. Regarding RRM measurement framework, the measurement report can be reported periodically or based on event trigger. In the following, we analyze the report procedure of predicted measurement report for FR2-FR2 Intra-frequency temporal domain case A, FR1-FR1 Intra-frequency temporal domain case B, inter-frequency domain and spatial domain.

FR2-FR2 Intra-frequency temporal domain case A

In intra-frequency temporal domain case A, continuous measurement results in PW are predicted by continuous historical measurement result(s) in OW and the measurement results in the future can be predicted to improve the handover performance.

Regarding AI mobility, there are 2 options on when to trigger inference:

- Option A: the inference is performed as the real measurement result is obtained as shown in Figure 3;

Figure 3: Inference pattern of Option A

- Option B: the inference is performed when actual measurement event is triggered. For example, when the A3 entry condition is satisfied by actual measurement result, the inference is performed. And the prediction on the future result is performed which is shown in Figure 4.

Figure 4: Inference pattern of Option B

For Option A, predicted measurement results can be obtained periodically considering the inference is performed periodically based on the measurement period. While for Option B, the inference is only performed when TTT is started due to the actual measurement result satisfying the event entry condition. Hence, it can be seen that different trigger mechanisms of inference brings different impacts on measurement report of prediction. Thus, RAN2 needs to study the inference trigger for AI mobility.

For RRM measurement prediction, only Option A is suitable since the inference is performed periodically based on the measurement period. While for measurement event predication, both Option A and Option B are workable. But less inference work is brought in Option B.

FR1-FR1 Intra-frequency temporal domain case B

In intra-frequency temporal domain case B, the measurement is skipped to reduce the measurement overhead and measurement results in PW are predicted by historical measurement result(s) in OW. The UE needs to predict the measurement result for the skipped time instance based on the network configuration. In this case, the UE can report the predicted measurement result periodically or based on the event trigger.

Inter-frequency domain

For inter-frequency, the measurement result of one frequency are predicted by the actual measurement result of another frequency to reduce the measurement overhead of different frequencies. Since the measurement results are continuous predicted by the actual measurement results of the other frequency, the measurement results can either be reported periodically or triggered by measurement event.

Spatial domain

For Spatial domain, AI is applied to predict the skipped beams. As a consequence, the measurement overhead in the spatial domain is reduced. The skipped beams can be predicted continuously. In this case, the UE can report the predicted measurement result periodically or based on the event trigger.

Based on the analysis above, it can be seen that for Intra-frequency temporal domain case A, it is required that the inference is performed periodically for RRM measurement predication. Hence, it is proposed that:

Proposal 15: For Intra-frequency temporal domain case A, the inference is performed periodically for RRM measurement predication.

- Conclusion

In this contribution, we provide our views on the specification impact for AIML mobility, and the observations and proposals are summarized as follows:

RRM measurement prediction

Applicability report

Proposal 1: RAN2 discusses which option is adopted for network additional condition for RRM measurement prediction:

- Option 1: Network additional condition is not needed, considering the good performance of model generalization;

- Option 2: Network additional condition is needed, which can be configured to the UE using associated ID or specific cell configuration.

Inference configuration and report

Proposal 2: For UE-sided model inference of RRM measurement prediction, the MO need to be configured as inference inputs for RRM framework.

Proposal 3: For measurement results prediction, the inference configuration for intra-frequency temporal domain case A can include the timing information, e.g., the minimum duration when the predicted measurement results should be obtained.

Proposal 4: For intra-frequency temporal domain case A, RAN2 to discuss which option is included in the measurement report of RRM measurement prediction:

- Option 1: The last predicted measurement result in PW;

- Option 2: The average measurement result in PW；

- Option 3: The sample measurement result in PW.

Proposal 5: For intra-frequency temporal domain case B, RAN2 to discuss what is configured for the inference of RRM measurement prediction:

- Option 1: the network configures the MRRT and the UE derives the timing configuration according to MRRT which may include the skipping pattern;

- Option 2: the network configures the periodicity and the timing offset of the predicted measurement.

Proposal 6: For inter-frequency prediction, the frequency information of the predicted measurement needs to be configured for the UE for the inference of RRM measurement prediction.

Measurement event prediction

Applicability report

Proposal 7: For indirect measurement event prediction, the applicability reporting is the same as that for RRM measurement prediction.

Proposal 8: For direct measurement event prediction, for applicability reporting,

- new UE capability is needed, e.g., whether the UE supports direct measurement event predication.

Inference configuration and report

Proposal 9: For indirect measurement event prediction, the model inference configuration e.g., input and output configuration of model inference is the same as that for RRM measurement prediction.

Proposal 10: For indirect measurement event predication, event triggered reporting should be supported.

Proposal 11: For temporal domain case B, frequency domain, and spatial domain prediction, the legacy RRM measurement reporting can be reused for indirect measurement event predication.

Proposal 12: For temporal domain case A, the UE can report the predicted occurrence time for the indirect predicted measurement event.

Proposal 13: For direct measurement event prediction, the input/output configuration of model can be configured from network to UE:

- Input configuration: measurement resources of serving/neighbouring cells, event configuration parameters (e.g., TTT), other configuration information is FFS;

- Output configuration: the probability of event occurrence within a time window.

Proposal 14: For direct measurement event prediction, enhanced RRM measurement reporting mechanism is needed for the inference result reporting, e.g. reporting the probability of event occurrence within a time window.

When to perform inference

Proposal 15: For Intra-frequency temporal domain case A, the inference is performed periodically for RRM measurement predication.

- References

- R2-2503302, RAN2#129bis Meeting Report